| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4458 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Proposed to start the work on Telepresence system signaling |  |  |  |
| Purpose: | Proposal |  |  |  |

- Summary

This contribution proposes to discuss and start a new work item on Telepresence system signaling in ITU-T Q5/16.

- Introduction

Currently ITU-T Q5/16 has three work items on requirement, architecture and audio/video parameters of Telepresence system, which are now becoming more mature. But we still have some main problems need to be solved urgently in order to complete the technical implementable standard work in Q5/16.

One is about the telepresence capability negotiation and the other is about the multiplex transport of the media streams.

As has been noted in previous meetings, our work in Q5/16 has a dependency on the work in the IETF CLUE WG. Now, there has been much progress in IETF CLUE on framework, signalling and RTP mapping drafts. We believe that it’s time for us to discuss the corresponding signalling procedures works for H.323 based Telepresence system in ITU-T Q5/16.

- Discussion

2.1 Capability negotiation

The procedure of the capability negotiation in the legacy H.323 based video conference system has already defined in H.245. The legacy endpoint mostly has one main camera and one screen, so it normally only need one audio and one main video stream to be described before to be transport, and has no scene entry related concept.

But in the Telepresence system there are possibly several audio streams and several video streams from which grouped some scene entries needed to be negotiated between Telepresence endpoints or MCU. So how to describe and negotiate these capabilities for Telepresence system should be discussed and considered.

IETF CLUE has already started to discuss the signalling procedure including the capability negotiation based on SIP.  As discussed and noted before , ITU-T SG16 should incorporate the work of the CLUE WG , in fact the information contained in the framework defined in IETF CLUE has largely been adopted by Q5/16’s working item H.TPS-AV, but how to use these parameters is still need more technical recommendations. So it’s time for us to discuss and start some new work items to handle these issues about capability negotiation based on H.323 system.

2.2 Multiplex information transport

Since RTP is used for media transport in both H.323 and SIP based system, we can still reuse the multiplex transport protocol discussed in IETF CLUE or other WG. But we need to consider how to organize and transfer the RTP multiplex information during the signalling procedures for H.323 based Telepresence system in Q5/16.

- Conclusions

It’s proposed to start a new work item to discuss the signalling procedure of Telepresence system in Q5/16. The possible content may include the Telepresence capability negotiation and multiplex information transport based on some existed protocols such as ITU-T H.225 / H.245 and IETF CLUE messages. The tentative outline could come from the following items:

- Signalling procedures for Telepresence capability negotiation and exchange

- Signalling procedures for media channel creation

[End Proposal]

_________________