|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 12 – C 0043R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | March 2013 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 10/12 |  |  |  |
| STUDY GROUP 12 – CONTRIBUTION 43Rev.1 |  |  |  |  |  |
| Source: |  | Deutsche Telekom AG, Telefon AB – LM Ericsson |  |  |  |
| Title: |  | Initial Insights for P.SAM (Spatial Audio Meetings quality evaluation) |  |  |  |

This contribution gives some initial insights for the work item P.SAM (Spatial Audio Meetings Quality Evaluation) that shall serve as basis for discussions. First a brief state of the art is given which is summarizing a number of topics that are of interest for P.SAM, followed by some conclusions concerning the drafting of P.SAM.

We propose to consider the aspects and issues discussed in this contribution during the drafting process of P.SAM and we propose to include them into any upcoming draft text at appropriate sections.

# A. State of the art

A.1. Terms and definitions

Concerning the “Definitions” clause that an ITU-T recommendation usually has, we suggest to incorporate as much as possible those terms that are used in the spatial audio research community or in the literature, respectively.

The Audio Engineering Society is working on a glossary of terms concerning spatial audio reproduction. This is work in progress and there are no publications on this subject available yet. However, we are in close contact with one of the involved people (Prof. S. Spors, University of Rostock, Germany); hence we aim to incorporate any progress into the drafting process of P.SAM as early as possible.

In addition, we compiled a list of terms used in relevant literature that might be of interest for P.SAM, without providing a definition for them yet:

- Binaural sound reproduction (via headphones, or with crosstalk compensation via loudspeakers)

- Stereophony / stereophonic sound reproduction  (2 loudspeaker reproduction)

- Wave Field Synthesis

- Ambisonics (Panning)

- (Near field compensated) Higher Order Ambisonics

- Multichannel loudspeaker reproduction using stereo panning laws, such as 5.1 surround (remark: also considered  under the term stereophonic reproduction in the spatial audio community)

- Spatial sound attributes, such as Immersion,Envelopment, Localization Blur

- “monaural” sound attributes (a better term might be necessary)

- Sweet spot

- Localization / Lateralization

- Temporal audio-visual synchrony

- Spatial audio-visual synchrony

A.2. Background on spatial hearing

The different spatial audio rendering techniques are exploiting the spatial hearing capabilities of the human auditory system. A brief state of the art on those capabilities could provide useful background information for P.SAM.

The human ability to operate within and to interact with the real-life environment depends strongly on the spatial awareness, which humans obtain through the spatial perception derived from the visual and auditory system. Concerning the direction of direct sound sources, we rely more on our vision to perceive the spatial scene in front of us, while we rely more on our hearing for things that are not directly in front of us. In addition, our auditory system is able to extract additional information about the environment such as room characteristics, room size, background, social situation etc.

The human auditory system is equipped with a highly developed spatial hearing that is able to give us a fairly detailed picture of the spatial environment deduced only from the acoustical signals reaching the two ears. Three of the most important components are the ability to sense from which direction a sound wave is coming, to be able to separate several such simultaneous sounds, and to be able to group direct sound and later reflections coming from room surfaces into one auditory foreground object and a passive background object, which corresponds to the room.

The ability to identify the direction of a sound source is based on the evaluation of a number of so called spatial cues that the auditory system is extracting from the ear signals:

Direction in the horizontal plane, i.e. azimuth angle (Fig. 1): Interaural Level Difference (ILD), Interaural Time Difference (ITD), Signal coherence.

Direction in the median plane, i.e. elevation angle (Fig. 1): direction dependent filtering of the pinna & ear canal (Directional bands)

- Distance: 
small distances (r <50cm): evaluation of level dependency, interaural level differences through shadowing effect of the head if close to one ear

- medium distances (50 cm < r < 15 m): level dependency (1/r - law) from distance can be evaluated for a known source type

- large distances (r>15m): frequency dependent attenuation through absorption in air medium (lowpass characteristic, far sources sound dull)

Figure 1: Through our spatial hearing we are able to sense from which direction, i.e. azimuth and elevation angle, a sound wave is coming from.

The ability to separate several sound sources approaching us from different directions is much better than the ability to separate them if they were all coming from the same direction, also known as the cocktail party effect: in such a situation we are able to focus our attention on a single conversation of all the ones that are taking place around us.

The third ability, grouping direct sound and room reflections into one auditory object, enables us, to identify a sound source in a room or some other enclosed spatial environment. An emitted sound wave will be reflected several times by the walls, floor, ceiling and other reflective surfaces in the environment before dying out (Fig. 2). Hence we will receive several filtered and delayed versions of the same sound, which would make it difficult to identify – or in case of speech – to understand the source, if the human auditory system is not properly dealing with this situation. Apparently, the accumulated signal of these filtered and delayed sounds is usually perceived as the reverberation of the original sound. Furthermore, our spatial hearing extracts from this reverberation a sense of the approximate size of the room or enclosure we are in (i.e. a small room versus a concert hall) as well as the reflective nature of the surfaces in the environment (i.e. small office versus a tiled bathroom).

Figure 2: In a room a listener receives sound wave via the direct path (blue path) and with some delay via reflections from the walls, floor, ceiling and other reflective surfaces in the environment (red path).

For more detailed information on spatial hearing we refer to [Blauert1999, Begault1994, Rumsey2005].

A.3. Background on spatial audio reproduction

Similar to previous section, a brief overview about existing spatial audio rendering techniques could provide useful background information for P.SAM.

A.3.1.Stereophonic sound rendering via 2 loudspeakers or via headphones (Stereo)

Using two loudspeakers, these techniques apply level or time differences between the generated sound signals in order to create virtual sound sources that a listener can perceive as coming from a position between the loudspeakers. Usually, level differences are applied and the audio community is also referring to these techniques as “amplitude panning”.

In the case of headphone reproduction of these signals, the virtual source is again perceived in between the two speakers that is inside the head of the listener.

A.3.2.Multichannel loudspeaker reproduction using panning laws (5.1, 7.1, etc.)

The mechanisms to create virtual sources are very similar to classical stereophony: they use amplitude panning, but extended to more loudspeakers. Deviations from that are the usage of a mono channel for speech (center loudspeaker) and the enhanced possibility to play ambience sounds through all speakers.

A.3.3.Binaural sound reproduction (via headphones)

Synthesis of the ear signals by emulating the acoustic properties of the outer ear and upper torso. This is realized by convolving impulse responses representing the Head-Related filters (the z- or Fouriertransform of those filter responses are typically called HRTF) with the raw, i.e. non-spatial audio signals. Those HRTFs are usually measured with a head and torso simulator (HATS), while individual measurements of people’s own HRTFs are also possible. Note that HRTFs are functions of the direction, which essentially means, in these transfer functions or impulse responses, respectively, the most important aspects of spatial hearing are covered: ITD, ILD, and directional filtering. Figure 3 gives an example of an HRTF and the corresponding HR impule response. Note that there are approaches of binaural rendering using loudspeakers, but those require crosstalk cancellation (both loudspeaker signals are perceived by both ears), which requires rather unpractical constraints (acoustical room properties, known position of listener, one listener only).

Figure 3: Top: HRTF for the left and right ear for an azimuth angle of 45°. Bottom: Corresponding impulse response.

A.3.4.Wave Field Synthesis (via loudspeakers)

This technique uses a large number of closely spaced loudspeakers to generate by means of superposition of the individual sound waves of the loudspeakers that sound field that would be created if the virtual source (the rendered source) would be actually present.

A.3.5.Higher Order Ambisonics (via loudspeakers)

This technique applies the same idea as Wave Field Synthesis, but employs spherical harmonics for the description of the sound field. Therefore, it relies on special loudspeaker setup as a circle or a sphere in order to create the sound field of a virtual source.

A.3.6.Ambisonics (via loudspeakers)

Ambisonics uses the same mathematical principals as Higher Order Ambisonics, but employs only a few loudspeakers. Due to this restriction the virtual sources created with Ambisonics are very similar to the ones created by stereophonic panning.

A.4Potential influence of spatial audio reproduction method

Different spatial rendering techniquesare known to cause different artefacts in the reproduced sound field. Contribution [COM 12 – C zz – E] discusses various use cases and it addresses potential consequences of different sound rendering techniques on the evaluation method. Here we briefly summarize known problems in spatial sound reproduction that might be of interest for P.SAM. Note that P.SAM will probably focus on overall quality of spatial audio meetings and not specific artefacts; however, any methodologies recommended in P.SAM should be checked if they appropriately cover such artefacts.

- Stereophonic sound rendering via loudspeakers:
keyword: sweet spot

- Multichannel loudspeaker reproduction using panning laws:
keyword: sweet spot, center loudspeaker in telemeeting scenarios, problems in panning of virtual sources at the listener’s side

- Stereophonic sound rendering via headphones
keyword: in-head localization

- Binaural sound reproduction (via headphones)
keyword: front-back confusion, limited out-of-head localization

- Wave Field Synthesis (via loudspeakers)
keyword: coloration, strong artefacts in special cases such as reproduction of focused sources

- Ambisonics (via loudspeakers)
keyword: sweet spot

- Higher Order Ambisonics (via loudspeakers)
keyword: sweet spot, few results on perception when out of the sweet spot

A.5.Research in spatial audio perception

The research in the perception of virtual sound scenes generated by spatial audio reproduction systems has mainly focused on two aspects, namely the spatial fidelity and the timbral fidelity. The first one includes basic properties like localization, auditory source width, envelopement. The main attribute of the second one is coloration. For 5.1 surround systems [Rumsey2005] have shown, that the variance in the overall quality ratings of such systems can be explained with 30% by spatial fidelity and with 70%by timbral fidelity. For sound reproduction methods applying lots of loudspeakers like Wave Field Synthesis or Higher Order Ambisonics only a few results are available at the moment. [Wittek2007] has shown that Wave Field Synthesis was rated as colored compared to stereophony. [Wierstorf2012] showed that the localization in Wave Field Synthesis is good in the whole listening area in contrast to the sweet spot found in stereophonic techniques.In contrast to stereophony the research of the perception for methods like Wave Field Synthesis or Higher Order Ambisonics is still at an early stage. The overall quality has so far not been investigated for these methods.

A.6.Subjective Test Methods and Goals

One interesting questionfor P.SAM is how to deal with the fact that quality of a spatial telemeeting can refer to a combination of two types of quality: one would be the – so to say – monaural quality, i.e. quality of the speech signal as such; the second one would be the quality of the spatial reproduction, which would refer to the aspects plausibility (localization) and immersion (envelopment). In that context, it needs to be well defined which test goals and corresponding methodologies should be considered in P.SAM:

- Analytic methods

  - Ongoing research activities attempt to find an appropriate list of attributes that describe in detail the perception of spatial audio reproduction, as it is done for example in the SEACEN project [SEACEN2012]

  - This is work in progress, a well-agreed specific list of attributes is not available for the time being

- Utilitaristicmethods (MOS values)

  - When being asked to give MOS ratings of a telemeeting (using an ACR scale for instance), subjects may rate an arbitrary combination of two different aspects: speech quality in terms of artifacts and spatial impression in terms of localization or envelopment

  - An approach with one overall quality and one spatial quality scale was used in a test reported in [COM 12 – C 97 – E]. In that test the overall and spatial quality scales formed a 2D-diagram, but similar results were achieved also with two separate scales.

  - An interesting similar approach in another domain is P.835, concerning quality of speech communication systems including noise suppression algorithms: individual questions address speech signal, background noise, and overall quality. One could use similar questions for P.SAM: one question on the overall quality, one question on the “monaural” speech quality, one question on the spatial impression.

A.7.Multiparty specific questions

As P.1301 points out, multiparty telemeetings differ in two major aspects from two-party calls: the special communicative situation and asymmetry (of connections, capture and rendering devices etc.)

Concerning the communicative situation, various studies [Balids2001, Kilgore2003, Yankelovich2006, COM 12 – C 190 – E, ITU-T COM 12 – C 287 – E] discuss the added value of spatial audio reproduction in multiparty audio conferencing situations.The majority of those studies show that spatial audio helps in terms of the ease of communication. However, to our knowledge, it is unclear yet, how signal artefacts caused by a spatial audio reproduction technique (affecting the perceived signal quality) may be compensated with an increased ease of communication.[ITU-T COM 12 – C 287 – E] suggests that people can separate cognitive effort and quality, however [COM 12 – C 0035 – E] specifies this by showing that this holds if the specific question is addressing the “Quality of perceived connection”. However, if the specific question is addressing “Overall quality of telemeeting”, then both, technical quality and ease of communication can influence the results. Hence, such balancing between both aspects is possible.

Concerning asymmetry, P.1301 Annex E and Annex F discuss potential effects of having different rendering techniques included in the test setup.

A.8. Test stimuli / test tasks

In order to evaluate spatial audio meetings in a listening-only test, appropriate test stimuli needs to be selected. In case of a conversational test, both the technical test setup and the test tasks should be appropriately designed.

P.1301 gives detailed guidelines on the requirements of test stimuli / test tasks concerning the multiparty aspects, but does not elaborate on any specific requirements in terms of spatial audio reproduction.

[Berndtsson2008] discusses the requirements for creating appropriate test stimuli for a teleconference system that applies stereo capturing. [Berndtsson2008] points out that live recordings in the ordinary test environment leads to natural and correct reproduction of source locations, speaking behavior (e.g. loudness or Lombard-effect) and background noise, while they lack the flexibility to render other audio scenes. In contrast, anechoic recordings of speakers followed by a spatial rendering technique using room impulse responses and simulated background noise usually lack naturalness, but they enable a flexible creation of different audio scenes. An example of processing of test samples simulating an audio scene with two talkers from anechoic speech recordings can be found in [COM 12 – C 97 – E]  and in The Qualification Quality Assessment Test for the joint SWB extension of G.718 and G.729.1.

Concerning the spectral and temporal signal properties of test material, both artificial sounds (i.e. noise) and real-life sounds are used in the spatial audio community for developing and evaluating spatial audio rendering methods. In localization experiments, Gaussian or band-limited noise is a typical stimulus. For Wave-Field-Synthesis (WFS) systems, known artefacts are usually affecting the temporal properties; hence transient sounds like castanets are often used. Such sounds are known to be appropriate for the specific questions concerning the rendering techniques. Concerning a quality test of a spatial telemeeting system, however, we are not aware of any insights or evidence that requirespecial signal properties. Hence, normal speech material appears to be appropriate and P.SAM could simply refer to other recommendations such as ITU-T P.800.

A.9.Dynamic conditions

Concerning dynamic conditions, one aspect of spatial audio meetings has – to our knowledge – not been thoroughly investigated yet: how do people perceive the quality of a spatial audio meeting when people are leaving and entering the conference call at different moments in time? While P.1301 discusses the options of using existing continuous rating methods, no new insight appear to be available yet.
Another dynamic aspect refers to the rendering of moving sound sources. In WFS for instance, coloration is a function of position, hence moving sources yield in changing coloration. In addition, fast moving sources can cause unwanted Doppler effects, if the system is not compensating for it. The rendering of moving sources in the telemeeting context may be applied in at least two cases. First, a high-end system using speaker tracking (beam forming) techniques would enable an authentic reproduction of moving speakers. Second, when participants are entering or leaving the conference call, the spatial rendering system might move the positioning of the participants in order to optimize the available virtual acoustic space.

A.10.Spatial audio in audio-only vs. audiovisual telemeeting systems

As P.1301 Clause 7 points out, quality perception of an audio-only and an audiovisual meeting may differ, such as the importance of speaker separation and ease of communication. However, such aspects are, as discussed above, particularly important in the context of spatial audio meetings. Hence, distinctions need to be made between the quality of a spatial audio-only meeting and a spatial audiovisual meeting.

# B) Conclusions for drafting P.SAM

We observed that there are still many open questions when it comes to the quality assessment of spatial audio meetings. For that reasons, drafting an appropriate P.SAM recommendation requires a number of steps:

- Actively seek additional input from colleagues from inside and outside SG12, who have experience in spatial audio reproduction.

- If necessary, conduct an additional thorough literature review by a team of volunteers.

- If necessary, conduct new experimental studies in SG12 context.

Furthermore, we suggest to build P.SAM upon P.1301, while ensuring an appropriate differentiation between these two recommendations:

- Aim to fill P.SAM with more specific recommendations than we did in P.1301, in order to go beyond the aspects already mentioned there.

- Keep the spirit of P.1301, i.e. refer to existing recommendations where possible, but avoid to duplicate information already in P.1301, and – eventually – check for consistency with P.1301

# References

[COM 12 – C 50– E] Karlsson, E., Berndtsson, G., Skowronek, J., “Proposal for evaluation criterions for Spatial audio meetings quality evaluation (P.SAM)“, submitted to ITU-T SG12 meeting March 2013, Geneva.

[COM 12 – C 0035 – E] Skowronek, Raake “Update on Number of interlocutors and QoE in multiparty telephone conferences”, Geneva, Mar 2013.

[COM 12 – C 97 – E] Berndtsson, Folkesson“New subjective test method for evaluation of overall and spatial sound quality”, Geneva, Oct. 2009.

“Qualification Quality Assessment Test Plan for the joint superwideband extension of G.718 and G.729.1,” version 0.5 11 July 2008 (Tdoc AH-08-65Rev) (Q7 Rapporteurs’ meeting).

[Balids2001] J. J. Baldis, Effects of Spatial Audio on Memory, Comprehension, and Preference during Desktop Conferences, in: M. Beaudouin-Lafon, R. J. K. Jacob (Eds.), Proceedings of the ACM CHI 2001 Human Factors in Computing Systems Conference, vol. 3 (1), 166-173, 2001.

[Kilgore2003] R. Kilgore, M. Chignell, P. Smith, Spatialized audioconferencing: what are the benefits?, in: Proceedings of the 2003 conference of the Centre for Advanced Studies on Collaborative research CASCON '03, IBM Press, 135-144, 2003.

[Yankelovich2006] N. Yankelovich, J. Kaplan, J. Provino, M. Wessler, J. M. DiMicco, “Improving Audio Conferencing: Are Two Ears Better than One?”, Proceedings of CSCW 2006, ACM Press, 2006.

[COM 12 – C 190 – E] Raake, A., Hoeldtke, K., “Listening and conversational quality of spatial audio conferencing”, Geneva, Jan. 2011.

[ITU-T COM 12 – C 287 – E] Skowronek, J., Raake, A., “Number of interlocutors and QoE in multiparty telephone conferences”, Geneva, Oct. 2011.

[Berndtsson2008] GunillaBerndtsson, “Creation of test material that simulates the stereo capture of a teleconference site”, ITU-T Workshop on "From Speech to Audio: bandwidth extension, binaural perception", Lannion, France, 10-12 September 2008

[Blauert1999] J. Blauert, Spatial hearing : The phychophysics of human sound localization, The MIT Press, Cambridge, Massachusetts, USA, 1999.

[Begault1994] D.R. Begault, 3-D sound for virtual reality and multimedia, AP Professional, Cambridge, Massachussets, USA, 1994.

[Rumsey2005] F. Rumsey, Spatial Audio, Focal Press, Oxford, UK, 2005.

[Wittek2007] Wittek, H. Perceptual differences between wavefield synthesis and stereophony. University of Surrey, 2007.

[Rumsey2005] Rumsey, F., Zielinski, S., Kassier, R., &Bech, S. On the relative importance of spatial and timbral fidelities in judgments of degraded multichannel audio quality.JASA, 118(2), 968–976, 2005.

[Wierstorf2012] Wierstorf, H., Raake, A., & Spors, S. Localization of a virtual point source within the listening area for Wave Field Synthesis.AES133 (Vol. c), 2012.

[SEACEN2012] http://www.seacen.tu-berlin.de/menue/seacen/parameter/en/

_________________