| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4468 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposed architectural functions of MCU (F/H.TPS-Arch) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The contribution is to enhance the definition of MCU described in section 7.2 of F/H.TPS-Arch by aligning with the functional requirements related to Multipoint Control Unit (F.TPS- Reqs).

# Discussion

As defined in ITU-T H.323, MCU is responsible for managing multipoint conferences (two or more endpoints engaged in a conference).

For telepresence system, the functional requirements for MCU are defined in F.TPS-Reqs, and the functions of MCU are defined in F/H.TPS-Arch.

However, there are slight mismatch between functional requirements and architectural functions related to MCU. Therefore it is necessary to align the requirements defined with the architectural functions. To do this, there is a discussion issue written in the following table; sorting the functional requirements (section 8 of F.TPS-Reqs.) along with the functions of endpoint architecture (section 7.2 of F/H.TPS-Arch.).

| Functions in F/H.TPS-Arch |  | Requirements in F.TPS-Reqs |
| --- | --- | --- |
| Access functions: | - To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints. | Med003, Med006, Med007 Cal01, Int03 |
|  | - To support multi-image composition videoconferencing. |  |
|  | - To support single/multiple auxiliary media streams. | Med002b, Med002c |
|  | - To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode. | Int02 |
| Adaptation functions: | - Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc. | - |
|  | - Adaptation for media streams according to characteristics of endpoint, such as screen aspect ratio, etc. | Med001, , |
| Control functions: | - To support voice switch and manual switch mode. | Con01, Con03 |
|  | - To support switch based on endpoint or screens when an endpoint has multiple screens. | Con06, Con07 |
| Not defined yet in F/H.TPS-Arch | - To support non-mixing | Med008 |
|  | - To support conference chairman control | Con08, Con04, Con02 |
|  | - To support far-end camera control | Con05 |
|  | - To support non-telepresence systems | Con02 |
|  | - To support spatial arrangement | Med002, Med002a, Cal02 |
|  | - To support report | Other01 |
|  | - To support security | Sec001, Sec002, Sec003, Sec004 |
|  | - To support QoS | Net001, Net002 |
|  | - To support NAT, IPv4/v6 | {Net003, Net004} |

# Proposal

Based on the discussion above, we propose to modify architectural functions defined in F/H.TPS-Arch. as follows;

## 7.2	Multipoint Control Unit

The main functions of an MCU are basically identical to the definitions in [ITU-T H.323], as per the following descriptions.

Access functions:

- To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints.

- To support multi-image composition videoconferencing.

- To support single/multiple auxiliary media streams.

- To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode.

- To support network resource reservation for assuring QoS

- To support automatic NAT traversal and IPv4/v6

Adaptation functions:

- Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc.

- Adaptation for media streams according to characteristics of endpoint, such as screen aspect ratio, etc.

- Adaptation for media stream of non-mixed mode

Control functions:

- To support voice switch and manual switch mode.

- To support switch based on endpoint or screens when an endpoint has multiple screens.

- To support conference chairman control  such as media selection, adding/deleting an site(s)

- To support far-end  camera control

- To support non-telepresence systems for heterogeneous systems

- To support spatial arrangement

- To support report of various information

- Security functions:

- To support authenticate, optionally signal/media security

__________________