| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-14 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.TLS |  |  |  |
| Title: | H.248.TLS "Gateway control protocol: H.248 packages for control of transport security using TLS" (New): Output draft Ed. 0.9 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.TLS "H.248 packages for control of transport security using TLS" (Ed. 0.9).

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TLS Ed. 0.1 | Initial draft (TD-23) from WP 2/16 Geneva meeting, Feb. 2012 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.TLS Ed. 0.2 | Input (TD 749 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749 |
| H.248.TLS Ed. 0.3 | Output (TD 749R1/WP2) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749R1 |
| H.248.TLS Ed. 0.4 | Input (AVD-4232) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4232.zip |
| H.248.TLS Ed. 0.5 | Output (TD-23) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-23.zip |
| H.248.TLS Ed. 0.6 | Input (TD 20 (WP2/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.7 | Output (TD 20 R1 (WP2/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.8 | Input (AVD-4364) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4364.zip |
| H.248.TLS Ed. 0.9 | Output (TD-14) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-14.zip |
| H.248.TLS Ed. 0.10 |  |

CONTENTS

Page

1 Scope 11 1.1 Applicability statements 12 2 References 12 3 Definitions 13 3.1 Terms defined elsewhere 13 3.2 Terms defined in this Recommendation 13 4 Abbreviations and acronyms 14 5 Conventions 14 6 Use case descriptions 15 6.1 Use cases related to TLS transport modes 15 6.2 Bearer connection network use cases with H.248 IP-IP gateways 15 6.2.1 High-level use case categories 15 6.2.2 Use case variations 17 6.3 Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways 17 6.4 Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway) 17 7 Models 17 7.1 Network model from H.248 entity point of view 17 7.2 Bearer connection model 18 8 TLS basic session control package 19 8.1 Properties 20 8.2 Events 21 8.2.1 TLS session state change (“BNC change”) 21 8.3 Signals 22 8.3.1 Establish (BNC) 22 8.3.2 Release (BNC) 22 8.4 Statistics 23 8.5 Error Codes 23 8.6 Procedures 23 8.6.1 TLS endpoint creation 23 8.6.2 TLS endpoint role assignment 23 8.6.3 TLS security session establishment 24 8.6.4 TLS application data transfer 24 8.6.5 TLS session release 25 9 TLS extended session control package 27 9.1 Properties 27 9.1.1 Stream Endpoint Pair Interlinkage 27 9.1.2 Protocol Layers Interlinkage 29 9.2 Events 30 9.3 Signals 30 9.3 Statistics 31 9.6 Procedures 31 9.6.1 TLS security session establishment 31 9.6.2 TLS security session release 31 10 TLS capability negotiation package 32 10.1 Properties 32 10.1.1 TLS Domain Profile Identifier 32 10.1.2 TLS Versions 33 10.1.3 Cipher Suites 33 10.1.4 Compression Methods 34 10.1.5 Support for Session Resumption 34 10.1.6 Support for Renegotiation of the Security Context 35 10.1.7 Renegotiation Period 35 10.1.8 Client Authentication Required 36 10.2 Events 36 10.3 Signals 36 10.4 Statistics 36 10.5 Error codes 36 10.6 Procedures 36 10.6.1 General default behaviour (MGC does not influence TLS capability negotiations) 36 10.6.2 MGC-controlled TLS capability negotiations: Options 36 10.6.3 Determining TLS Related Capabilities 37 10.6.4 Indication of TLS profile(s) to MG by MGC 38 10.6.5 Indication of individual TLS protocol capabilities to MG by MGC 38 11 TLS session maintenance package 40 11.2 Events 40 11.2.1 TLS Error Alert 40 11.3 Signals 42 11.3.1 MGC triggered TLS error alert 42 11.4 Statistics 43 11.5 Error codes 43 11.6 Procedures 43 11.6.1 TLS warning error alerts 43 11.6.2 TLS fatal error alerts 43 11.6.3 Lower layer issues 44 12 TLS traffic volume metrics package 44 12.1 Properties 44 12.2 Events 44 12.3 Signals 44 12.4 Statistics 45 12.4.1 Number of Received TLS-Fragments 45 12.4.2 Number of Sent TLS-Fragments 45 12.4.3 TLS Octets Sent 45 12.4.4 TLS Octets Received 46 12.4.5 Number of Received Application Data Octets 46 12.4.6 Number of Sent Application Data Octets 47 12.5 Error codes 47 12.6 Procedures 47 12.6.1 Ingress TLS traffic – Statistic "TLS Packets Received" 47 12.6.2 Ingress TLS traffic – Statistic "TLS Octets Received" 47 12.6.3 Ingress TLS traffic – Statistic "TLS Application Data Octets Received" 47 12.6.4 Egress TLS traffic – Statistic "TLS Packets Sent" 47 12.6.5 Egress TLS traffic – Statistic "TLS Octets Sent" 47 12.6.6 Ingress TLS traffic – Statistic "TLS Application Data Octets Sent" 47 13 Package-less TLS control 48 13.1 Related to TLS bearer establishment 48 13.1.1 Overview – principal H.248 control steps 48 13.1.2 Step 1: Bearer endpoint creation 51 13.1.3 Step 2: Bearer type indication for IP transport “L4/L3” 51 13.1.4 Step 3: Block / Unblock L4 bearer control procedures at SEP 52 13.1.5 Step 4: Start IP transport connection establishment procedure 52 13.1.6 Step 5: Local L4/L3 NAT traversal support 52 13.1.7 Step 6: Reporting successfully established IP transport connection 52 13.1.8 Step 7: Bearer type indication for TLS 52 13.1.9 Step 8: Block / Unblock TLS security session control procedures at SEP 52 13.1.10 Step 9: Start TLS security session establishment procedure 52 13.1.11 Step 10: Local L4+ NAT traversal support 52 13.1.12 Step 11: Reporting successfully established TLS session 52 13.1.13 Step 12: Active communication phase 52 13.2 Related to TLS authentication 52 14 The TLS profile concept 53 14.1 TLS protocol profiles 53 14.2 Illustration of TLS profile concept 54 14.3 Example for the TLS MG profile concept 55 15 Security considerations 56 Annex A State modelling for TLS bearer connection endpoints 57 A.1 Introduction and purpose 57 A.2 Original state model for TLS session endpoints 57 A.2 Simplified state model for H.248-based TLS basic session control 57 Appendix I Sample use-cases of TLS bearer encryption 59 I.1 Use case “Terminal-to-MG TLS session, provisioned TLS service negotiation” 59 I.2 Use case “Terminal-to-MG TLS session, MGC-controlled TLS service negotiation” 60 I.3 Use case “WebRTC to NGN/IMS interworking function with DTLS-to-TLS support” 60 I.x Use-case “TLS-based transport security for facsimile packet relay service T.38” 60 Appendix II Example call flows 61 II.1 TLS to non-TLS interworking with TCP as example bearer type 61 II.1.1 Initial session setup using SDP security descriptions in application control signalling 61 II.1.2 Traffic flow examples for communication establishment phase 62 II.1.3 Traffic flow examples for communication release phase 66 Appendix III Example TLS profiles 69 III.1 Typical Internet TLS profile 69 III.2 3GPP TLS domain profile 70 III.3 OMA TLS domain profiles 72 III.3.1 OMA public key TLS domain profile 72 III.3.2 OMA pre-shared key TLS domain profile 73 III.4 IETF Minimum TLS domain profile 74 III.5 IETF example of a national TLS domain profile 74 III.6 ITU-T TLS domain profile for NGN signalling and management plane 74 Appendix IV Illustration of protocol semantics of the TLS basic session control package 74 IV.1 Overview 74 IV.2 Conventions 74 IV.3 Establishment of TLS security sessions 74 IV.3.1 Successful establishment – Terminating side 74 IV.3.2 Successful establishment – Originating side 75 IV.3.3 Unsuccessful establishment 76 IV.4 Release of TLS security sessions 76 IV.4.1 Successful release – Terminating side 76 IV.4.2 Successful release – Originating side 77 IV.4.3 Unsuccessful release 78 Appendix V Illustration of protocol semantics of the TLS extended session control package 78 V.1 Overview 78 V.2 Conventions 78 V.3 Usage of SEPP Interlinkage property 78 V.4 Usage of Protocol Layers Interlinkage property 78 V.4.1 Property „plslink = FALSE“ 79 V.4.2 Property „plslink = TRUE“ 79 Appendix IV Living List related to DTLS and DTLS-SRTP 84 IV.1 DTLS protocol 84 IV.2 DTLS related services 84 IV.3 DTLS in scope of H.248.TLS? 84 IV.4 DTLS use cases 84 IV.4.1 Use cases from TLS-to-DTLS interworking perspective 85 IV.4.2 Use cases from SRTP “key exchange” perspective 85 IV.4.3 Two interpretations for „DTLS-SRTP“? 86 IV.4.4 Use cases from WEBRTC perspective 87 IV.4.5 Use cases from Telepresence perspective 87 Bibliography 88

List of Tables

Page

Table 1 – Principal TLS-based interfaces of H.248 entities and their relevance for this Recommendation 12 Table 2 – Security session establishment preparedness (of TLS endpoints) 20 Table 3 – Parameter values 21 Table 4 – TLS endpoint role assignment (at establishment phase) 24 Table 5 – Impact of StreamMode on TLS bearer traffic at external MG interface 24 Table 6 – Semantic of property sepplink 28 Table 7 – Semantic of property plslink 30 Table 8 – Variants of MGC-controlled TLS capability negotiations 37 Table 9 – H.248 control steps and possible control elements 50 Table 10 – Analogon between H.248.80 media configurations and TLS profile types 54 Table 11 – Example of a TLS MG profile 55 Table 12 – Example of an intersection between a TLS domain profile and a TLS MG profile 56 Table III.1.a – Example of a TLS domain profile – Main part 69 Table III.1.b – Example of a TLS domain profile – Cipher Suites 70 Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part 71 Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites 72 Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part 72 Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites 73

List of Figures

Page

Figure 1 – TLS transport modes 15 Figure 2 – Bearer connection network use cases with H.248 IP-IP gateways 16 Figure 3 – Network model from H.248 entity point of view (“half call/bearer connection model”) 18 Figure 4 – TCP/IP protocol stack with TLS protocol 19 Figure 5 – Two-termination context with a TLS termination 19 Figure 6 – Principle stimuli for triggering TLS session release 25 Figure 7 – Principal H.248 control steps for establishing a TLS-over-L4 bearer 49 Figure 8 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile 54 Figure 9 – Illustration of TLS profile concept 55 Figure A.1 – Simplified state model for H.248-based TLS basic session control 58 Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG 60 Figure II.1 – Configuration for the example call flow 61 Figure II.2 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)) 63 Figure II.3 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment (decoupled end-to-end L4 connection)) 64 Figure II.4 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment (but interlinked end-to-end L4 connection establishment)) 65 Figure II.5 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment (plus interlinked end-to-end L4 connection establishment)) 66 Figure II.6 – Example TLS session release, initiated by remote TLS endpoint (MGC enforces immediate L4 connection release) 67 Figure II.7 – Example TLS session release, local TLS/L4 decoupled, but coupled end-to-end L4 connection release) 68 Figure II.8 – Example TLS session release, coupled TLS/L4 and end-to-end L4 connection release 69 Figure IV.1 – Successful establishment – Terminating side 75 Figure IV.2 – Successful establishment – Originating side 76 Figure IV.4 – Successful release – Terminating side 77 Figure IV.5 – Successful release – Originating side 77 Figure V.1 – Illustration of Protocol Layers Interlinkage property – Property „plslink = FALSE“ 79 Figure V.2 – Illustration of Protocol Layers Interlinkage property – Property „plslink = TRUE“ 79 Figure V.3 – Property „plslink = TRUE“ – L4 incoming bearer establishment request 80 Figure V.4 – Property „plslink = TRUE“ – TLS incoming bearer establishment request 80 Figure V.5 – Property „plslink = TRUE“ – L4 stimuli 81 Figure V.6 – Property „plslink = TRUE“ – TLS stimuli 81 Figure V.7 – Property „plslink = TRUE“ – L4 incoming bearer release request 82 Figure V.8 – Property „plslink = TRUE“ – TLS incoming bearer release request 82 Figure V.9 – Property „plslink = TRUE“ – L4 stimuli 83 Figure V.10 – Property „plslink = TRUE“ – TLS stimuli 83 Figure IV.1 – TLS/DTLS protocol historic 84 Figure IV.2 – Example TLS-to-DTLS interworking 85 Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248) 85 Figure IV.4 – Media-path coupled key exchange using DTLS 86 Figure IV.5 – Interpretation „DTLS only used for SRTP key exchange“ 86 Figure IV.6 – Interpretation „Secure RTP traffic using DTLS“ 87 Figure IV.7 – DTLS use cases: „WebRTC data“ & „WebRTC SRTP key exchange“ 87

Draft new ITU-T Recommendation H.248.TLS

Gateway control protocol: 
H.248 packages for control of transport security using TLS

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

Recommendation ITU-T H.248.TLS defines new ITU-T H.248 packages for support of secured IP transport bearer traffic according the Transport Layer Security (TLS) technology. In addition, this Recommendation identifies typical network use cases with different ITU-T H.248 media gateway variants, and provides example signalling flows.

# 1	Scope

{Editor’s note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two IP transport connection endpoints. This Recommendation defines in general H.248 signalling elements for support of TLS in various options.

This Recommendation provides in more detail information about

- basic exchange architecture of keying information between control plane (incl. H.248 interface) and bearer plane;

- negotiation aspects: indication and determination of cryptographic capabilities between TLS endpoints;

- minimum amount of information carried by H.248 for establishing H.248 TLS/L4 terminations (NOTE 1);

NOTE 1 – Some information could be provisioned via management and there is also TLS information exchanged via the (L4) bearer interface with the remote TLS endpoint.

- TLS procedures in detail at the various TLS sublayers, i.e. for support of the

  - TLS record protocol

  - TLS handshake protocol

  - TLS change cipher spec protocol

  - TLS alert protocol

  - TLS application data protocol

- profiling of TLS services;

- specify set of cipher suites;

- consideration of H.248 MG modes of operation and connection models;

- SDP- versus Property-based TLS endpoint control

- {list to be updated before approval/consent …}

- The scope of this Recommendation is limited to {… any limitations?  …}

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP)

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (Draft), Gateway control protocol: Usage of the revised SDP offer / answer model with H.248.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), TCP support packages.

[ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	TLS-client [IETF RFC 5246]: the TLS entity that initiates a TLS security session to a TLS server.  The primary operational difference between the TLS server and TLS client is that the TLS server is generally authenticated, while the TLS client is only optionally authenticated.

3.1.2	TLS-server [IETF RFC 5246]: the TLS entity that responds to requests for security sessions from TLS clients.

3.1.3	Transparent forwarding [ITU-T H.248.RTPTOPO]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	TLS MG profile: The TLS profile supported by the MG.

3.2.2	TLS Domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.3	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.4	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.5	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.6	TLS profile: A selection of options from a set of TLS related protocol parameters and procedures.

NOTE – Also known as (nonstandard term) TLS protocol profile (example: the TLS profile defined by Annex E/[ETSI TS 133.310]).

3.2.7	TLS transparent forwarding: MG packet forwarding behaviour with the characteristic of TLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a TLS-PDU flow.

NOTE 1 – A TLS PDU relates to a TLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.3, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.RTPTOPO]).

NOTE 3 – There would be then the characteristic of TLS message integrity in the context of "TLS transparent forwarding". The MG might be TLS aware; e.g. support of TLS related statistics or event detection would not violate transparent forwarding behaviour.

{Editor’s note (2012-09): the definition proposals in clause 3.2 are still under discussion .}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2B | Back to Back |
| --- | --- |
| B2BIH | Back to Back IP Host (see also [b-ITU-T H.248.64]) |
| B2BTE | Back-to-Back TCP Endpoint |
| B2BUA | Back to Back User Agent (SIP) |
| BGF | Border Gateway Function |
| FTP | File Transfer Protocol |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| TLS Est.req TLS Est.ack | Abstracted (TLS message/procedure independent) representationfor establishment requests/acknowledgements of new TLS security sessions. |
| TLS Rel.req TLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing TLS security sessions. |

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.1) or IP-to-non-IP gateway (see clause 6.2) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.3).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to TLS transport modes

The TLS protocol is independent of the underlying transport protocol stack. Multiple TLS transport modes are basically useful (Figure 1).

Figure 1 – TLS transport modes

This Recommendation is not dependent on any TLS transport mode, but uses TLS-over-TCP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion “IP-to-IP” (briefly “IP-IP”) indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 2):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as “non-TLS to non-TLS”);

- Use case #1.2: an end-to-end TCP connection traverses two network domains with different transport security policies (“TLS”, “no TLS”). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as “TLS to non-TLS”);

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as “TLS to TLS”);

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as “TLS to TLS*”);

Figure 2 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor’s note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

### 6.2.2	Use case variations

#### 6.2.2.1	“TLS-to-TLS transparent forwarding”

This scenario is a variation of use case #1.3 and requires the MG behaviour of TLS transparent forwarding according clause 3.2.7.

#### 6.2.2.2	“TLS-to-TLS application aware and decryption-encryption based forwarding”

This scenario is a variation of use case #1.3.

The MG may be enforced for application awareness in case of e.g. bearer-level ALG support (according [ITU-T H.248.78]). The MG is located within a single security domain, therefore not any change of the TLS profile. However, the MG is located at the edge of two network address domain, thus requested for NAT-T and NAT support.

Example use case:

MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the “TLS-to-TLS” interworking function is termed as “TLS B2BUA”. It may be noted that this use case does not belong to the “TLS-to-TLS*” category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

## 6.3	Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways

Theoretical scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer.

For further studies.

## 6.4	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

{Editor’s note (2012-02): see also discussion of clause 13.3 in H.248.NATTP2P.}

For further studies.

# 7	Models

## 7.1	Network model from H.248 entity point of view

Following network model (Figure 3) may illustrate the relevant areas covered by this Recommendation. The H.248 MG is basically peering a TLS-capable IP host as remote endpoint. Both TLS endpoints spanning a network transport security domain (here TLS domain).

Figure 3 – Network model from H.248 entity point of view 
(“half call/bearer connection model”)

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

{Editor’s note (2012-02): there might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution. This may imply a modified Fig. 2, as well as an extend list above.}

## 7.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a single ITU-T H.248 termination. In addition, no assumptions are made regarding either the lower layer protocols beneath the TLS level or the upper layer protocols being carried by the TLS/TCP transport. This allows the use of the Recommendation's procedures in various connection models and use-cases.

Figure 4 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 4 – TCP/IP protocol stack with TLS protocol

Figure 5 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 5 – Two-termination context with a TLS termination

# 8	TLS basic session control package

{Editor’s note (2013-01): package redesign according C.14 and alignment with H.248.TCP “TCP basic connection control package”.}

|  | Package Name: | TLS basic session control package |
| --- | --- | --- |
|  | Package ID: | tlsbsc (0x####) |
|  | Description: | This package provides the functionality to be able to establish and /release a TLS (or DTLS) based security session. Furthermore, the capability to block the start of TLS session establishment procedures at the MG bearer interface. NOTE 1 – The package design follows similar connection-oriented bearer (or media) protocols, such as the H.248 tcpbcc package [ITU-T H.248.TCP]. Terminology: “(D)TLS security session” correlates to “TCP bearer connection” and the “Q.1950 BNC” concept. NOTE 2 – The differences between DTLS and TLS are subject of [ITU-T H.248.DTLS]. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

## 8.1.1	Security session establishment blocking

|  | Property Name: | Security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1: Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG behaviour before and during TLS session establishment. NOTE 2: See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table 2 Table 2 – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour:. “blocked”, (0x0001); - Bearer resources are prepared (i.e., resources for TLS endpoint are reserved and allocated to H.248 termination/SEP); - TLS role assignment: still open, not yet decided; - TLS session still closed; - H.248 counterpart state = “Blocked” (Figure A.1); - any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall be not acknowledged).. “unblocked”, (0x0002); - MG shall be ready for incoming or/and outgoing TLS security session control procedures; - TLS role assignment: relates to an (initial) TLS server role effectively; - TLS session state: transitioning from closed to …; - H.248 counterpart state = “Idle” (Figure A.1); TLS security session control options: - Incoming TLS ClientHello: TLS server role confirmed; - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role assigned. |
|  | Default: | ”Unblocked” (due to be consistent with default behaviour as in [ITU-T H.248.84]). NOTE: H.248 profile specifications could redefine other default values. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Editor’s note (2013-06 meeting): following items (same as in H.248.TCP) were identified and need to be resolved:

C1: the property semantics are focusing on the ingress direction (tbc); i.e., when property not relevant in outgoing direction, then explicit text should be added.

C2: Signal EstBNC should implicitly overwrite the property value, i.e., replacing blocked by unblocked. Correspondent updates should be added.

}

## 8.2	Events

### 8.2.1	TLS session state change (“BNC change”)

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer (i.e., a TLS security session) has been established or a bearer has been released. The event is related to a simplified state transitioning model, which is described in Annex A. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Sublist of Enumeration |
|  | Optional: | No |
|  | Possible values: | Table 3 defines the possible values: Table 3 – Parameter values Value; Name; Final state (Annex A.2); Semantic. Est, [0x01]; Bearer Established; Established; notify MGC successful TLS security session establishment.. Rel, [0x05]; Bearer Released; ### Idle; notify MGC successful TLS security session release.. |
|  | Default: | Type=[Est,Rel] The MGC would be then automatically notified about all state transitions when subscribing to event tlsbsc/BNCChange. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est, [0x01] Bearer Established Rel, [0x05] Bearer Release |
|  | Default: | None |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).}

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0003) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TLS security session: the MG takes the TLS client role and initiates the start of a TLS session establishment by sending a TLS ClientHello request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

#### None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0004) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TLS security session: the MG sends a TLS alert close_notify request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination.

Other options?

None!?

Conclusion: a packageless semantic could be defined.}

#### 8.3.2.1	Additional parameters

#### None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	TLS endpoint creation

The MGC is responsible for creating a TLS session enabled SEP in the MG. This may be indicated on the H.248 interface through the use of the SDP “m=” element <proto>..

The MG shall reserve “TLS bearer resources” (such as a “TLS encryption(decryption function”) and allocates them to the SEP. The TLS role (client/server) is not (yet) determined by the SDP “m=” line information.

NOTE 1 – This functionality is synonym to the “bearer type indication” semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TLS endpoint role assignment

The TLS role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). Table 4 summarizes MGC actions and their results in TLS role assignments:

Table 4 – TLS endpoint role assignment (at establishment phase)

|  | TCP role | H.248 indication | MG behaviour: |
| --- | --- | --- | --- |
| 1 | not yet assigned | H.248 state “Blocked” of SEP, either due to explicit signalling of bceb property or correspondent default value | Any incoming TLS PDU shall be silently discarded. TBD |
| 2 | TLS server | H.248 state “Idle” by “unblocking” the SEP | Expect incoming (TLS) ClientHello, subsequent completion of the establishment related handshake procedure. |
| 2 | TLS client | H.248 state “Idle” of SEP plus signal EstBNC enabled. | Start of TLS session establishment related handshake procedure. |

### 8.6.3	TLS security session establishment

{Editor’s note (2013-06 meeting): align with § 8.6.3/H.248.TCP as soon as procedure stable …}

The MG will shall follow TLS security session establishment according to the indicated TLS.

### 8.6.4	TLS application data transfer

#### 8.6.4.1	MG externally

The TLS session is ready for application data transfer when remote and local TLS endpoints are both completed the TLS handshake procedures for session establishment.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TLS control information used to establish or close the TLS session. The requested MG behaviour is defined by Table 5:

{Editor’s note (2013-06 meeting): Consensus required on this behaviour. See H.248.TCP … }

Table 5 – Impact of StreamMode on TLS bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | all TLS PDUs sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | only TLS PDUs related to - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | all TLS PDUs sent |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | only TLS PDUs related to - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol. |

#### 8.6.4.2	MG internally

This clause is only relevant for H.248 contexts with TLS SEPP(s).

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the SEPP interlinkage property of the tlsesc package (see clause X.1.A.B) should be used.

### 8.6.5	TLS session release

The trigger for a TLS session termination might come from different sources, see Fig. 6:

Figure 6 – Principle stimuli for triggering TLS session release

- Bearer control protocol – Security session: The TLS session is terminated by the remote TLS endpoint by sending a (TLS) close_notify alert or a fatal alert;

- Bearer control protocol – Transport connection: The underlying L4 transport connection is terminated;

- TLS protocol: A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicates the closure of the TLS session by the signal tlsbsc/RelBNC;

- The release of the security session of the partner SEP triggers the release autonomously in the MG (Note: not supported by base package tlsbsc, related to inter-linkage property tlsesc/sepplink, see clause 9.x.y.z);

- In case the MG acts in an application aware mode, the application part in the MG may indicate the release of the TLS session;

#### 8.6.5.1	Bearer control protocol – Security session: 	TLS alert protocol

{Editor’s note (2013-06): the procedure overlapps with the TLS session maintenance package procedures. The text is not yet aligned with clause 10.6.x. Need to be fixed.}

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a received TLS close_notify the TLS endpoint shall reply a TLS close_notify on its own and the TLS session is closed.

Impact on underlying IP transport connection:

The MG shall not autonomously initiate the release of the (if connection-oriented) IP transport connection as well.

NOTE: extension package tlsesc allows to tightly couple bearer control procedures for L4 and TLS, see property tlsesc/plslink in clause 9.1.2.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}. Handling of the TLS-specific AlertDescription as received in the TLS alert message is for further studies.

Impact on H.248 SEP and possibly SEPP:

The SEP as such still exists after a successful TLS session release (state Idle). Removal of the SEP follows usual H.248 SUBtract.req or MODify.req commands.

Property tlsesc/sepplink (clause 9.1.1) allows to trigger a TLS session release at the partner SEPP.

#### 8.6.5.2		Bearer control protocol – Transport connection: L4 bearer release

In case the underlying IP transportconnection is released, e.g. due to a received TCP-FIN or TCP-RST in case of TCP transport, then

- default mode (base package): < description is missing! >

- enhanced mode (extension package): the MG may autonomously and immediately initiate the closure of the TLS session (dependent on property setting of tlsesc/plslink). The release procedures of the IP transport connection shall be delayed as long as the TLS session is successfully released (see also Appendix V.4.2.2.1).

NOTE: extension package tlsesc allows to couple bearer control procedures for L4 and TLS, see clause 9.1.2.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.3		TLS protocol: non recoverable error condition detected

{Editor’s note (2013-06 meeting): below information might overlap with the TLS session maintenance package … }

In case an error condition is detected locally in the TLS protocol layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to “fatal” and the AlertDescription is provided accordingly.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.4		H.248: MGC driven TLS session release

In case the H.248 signal tlsbsc/RelBNC is received, the TLS session closure shall be initiated by the MG by sending a TLS close_notify alert towards remote TLS entity..

#### 8.6.5.5	MG autonomous release initiated on partner SEP

This capability is not supported by the base package, subject of extenstion package tlsesc (property sepplink), see clause 9.

#### 8.6.5.6	Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session shall be closed autonomously by sending a TLS alert (close_notify).

The underlying IP transport shall be not affected (base package behaviour).

# 9	TLS extended session control package

|  | Package Name: | TLS extended session control package |
| --- | --- | --- |
|  | Package ID: | tlsesc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TLS bearer session control behaviour across a “horizontal” Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - TLS/L4 bearer control behaviour across “vertical” protocol layers, rather instead of decoupled, individual control of TLS security sessions and their underlying IP transport connection. |
|  | Version: | 1 |
|  | Extends: | tlsbsc |

## 9.1	Properties

### 9.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0005?) |
|  | Description: | The semantical scope of this property relates to a Stream Endpoint Pair (SEPP; see definition in (clause 3.2.11) [ITU-T H.248.1]). This property defines if a detected condition leading to the establishment or release of the TLS security session will trigger an MG-autonomous (see clause 3.2.2/H.248.TCP) establishment or release of the TLS security session of the partner SEP as well. Only TLS protocol control information is part of such conditions. Normal TLS data traffic is not affected by this property. Semantical scope limitations: - only the two phases of TLS bearer session establishment and release; - only incoming direction; - only the first stimuli of each TLS bearer session control procedure (i.e., first incoming (TLS) ClientHello; first incoming (TLS) close_notify); - SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries) {Editor’s note (2013-06 meeting): following items (as in H.248.TCP) were identified and need to be resolved: C1: the semantical scope needs to be more specific, particularily bullet items 2 to 4 (e.g., which bearer level procedure triggers in particular a correspondent procedure at the partner SEP). } |
|  | Type: | sublist of Boolean (for separated support of establishment and release phase) |
|  | Possible values: | ## Table 6 defines the protocol semantic per value and dependent on connection control phase. See model in Figure 5 concerning the used SEP names. Table 6 – Semantic of property sepplink Value; MG semantic for TLS bearer session. Establishment; Release. True; a) Incoming: An incoming TLS bearer session establishment request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session establishment request at the partner SEP T2(S1). b) Outgoing: An MGC-initiated outgoing TLS bearer session establishment at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).; a) Incoming: An incoming TLS bearer session release request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session release request at the partner SEP T2(S1). b) Outgoing: Any action (e.g. by MGC) in closing the outgoing TLS bearer session at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).. False; The state of TLS bearer session endpoint SEP T1(S1) shall not impact the state of TLS bearer session endpoint SEP T2(S1) (and vice versa).. |
|  | Default: | [False, False] (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”): TBD

}

### 9.1.2	Protocol Layers Interlinkage

|  | Property Name: | Protocol Layers Interlinkage |
| --- | --- | --- |
|  | Property ID: | plslink (0x0006?) |
|  | Description: | The semantical scope of this property relates to a single Stream Endpoint (SEP) with regards to a portion of the bearer specific protocol stack. This property defines whether bearer control procedures of the TLS security session shall or shall not impact as well the bearer control procedures of the underlying IP transport connection (e.g., TCP, SCTP). This property serves the two fundamental use cases of - Tightly coupled mode: the holding time of the TLS security session correlates directly with the underlying IP transport connection; e.g. a TLS session termination will always imply the release of the TCP or SCTP transport connection; - Decoupled mode: the TLS security session is disjoint from bearer control procedures of the underlying IP transport connection. For instance: a TCP-based communication service with alternate phases of encrypted and clear data, a TLS/TCP (or a DTLS/DCCP) session where the loss of the security session should not lead to the stop of data transfer, DTLS with UDP, DTLS-SRTP, different aimed behaviour between TLS/SCTP and DTLS/SCTP, etc. The default behaviour is aligned with the base package tlsbsc, which is the decoupled mode. The property takes only effect in case of connection-oriented IP transport protocols (such as TCP, SCTP, DCCP). The property shall be ignored in case of connectionless IP transport protocols (UDP). ## ToBeChecked |
|  | Type: | sublist of Boolean (for separated support of establishment and release phases) |
|  | Possible values: | Table 7 defines the protocol semantic per value and dependent on connection control phase. Table 7 – Semantic of property plslink Value; MG semantic for protocol stack “TLS-over-L4”. Establishment; Release. True; a) Incoming: An incoming L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. The SEP shall then sent an outgoing TLS bearer session establishment request (i.e., a TLS ClientHello). b) Outgoing: An outgoing L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. See above.; a) Incoming: An incoming TLS bearer session release request shall trigger the outgoing L4 transport connection release request, as soon as the TLS security session is successfully released.. b) Outgoing: An outging TLS bearer session release request shall … see above.. False; Any bearer control procedures related to TLS security sessions shall not impact the underlying L4 transport connection (and vice versa).. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”): TBD

}

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	TLS security session establishment

{Editor’s note (2013-06 meeting): following items (as in H.248.TCP) were identified and need to be resolved:

C1: properties are of type list, not  a single value, hence all procedural sections need to be updated from value False or True to [X,Y] combinations!).

}

#### 9.6.1.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session establishment request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session establishment request shall be propagated to the partner SEP, triggering there an outgoing TLS ClientHello message.

#### 9.6.1.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing (NOTE 2) L4 transport connection establishment request (such as a TCP SYN packet) shall immediately trigger an outgoing TLS security session establishment request, after a successfully established L4 bearer.

NOTE 1 – Appendix V.4.2.1.1 provides a signalling example.

NOTE 2 – E.g., either MGC driven or in case of e.g. TLS-over-TCP might be also the [ITU-T H.248.TCP] property tcpbsc/sepplink in use.

### 9.6.2	TLS security session release

#### 9.6.2.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session release request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session release request shall be propagated to the partner SEP, triggering there an outgoing TLS closure_notify alert message.

#### 9.6.2.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing TLS security session release request shall immediately trigger an outgoing L4 bearer connection release request, after a successfully release TLS session.

NOTE 1 – Appendix V.4.2.2 provides signalling examples.

# 10	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x####) |
|  | Description: | ## When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. This package provides control tools for the MGC for influencing bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. There are two options for negotiation of TLS protocol configurations: - MGC may indicate a TLS profile (or multiple) as such; - MGC may control individual TLS protocol parameters. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0001) |
|  | Description: | This property identifies the TLS domain profile that shall be ideally used by the MG to setup the TLS security session. The actually applied, so called TLS endpoint profile is the match result of locally supported TLS capabilities by the MG (termed as TLS MG profile) with requested TLS domain profiles. It should be noted that the normal case would be aligned TLS profile capabilities, i.e.: applied “TLS endpoint profile” = requested “TLS domain profile”. |
|  | Type: | sublist of String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.2	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0002) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS protocol version related IETF RFCs (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the “0x”, and if needed, a leading “0” is added for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) {Editor’s note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited. => (2013-06): might be addressed in clause 10.6.2} {Editor’s note (2013-06 meeting): following items were identified and need to be resolved: C1: clarify possible interaction between non-root and root termiantion usage, or define exclusive approach, etc). This comment is relevant for all package properties.} |
|  | Characteristics: | Read/Write |

### 10.1.3	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0003) |
|  | Description: | This property indicates the cipher suites and their precedence for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sublist of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA (http://www.iana.org/assignments/tls-parameters/tls-parameters.xml#tls-parameters-3 ). The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The “0x” shall be omitted and if needed, a “0” is used for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.4	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0004) |
|  | Description: | This property declares the list of compression methods and their preference for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers (see http://www.iana.org/assignments/comp-meth-ids/comp-meth-ids.xml ). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.5	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0005) |
|  | Description: | This property declares the support for the TLS session resumption. {Editor’s note (2013-06 meeting): the capability of TLS session resumption, - from perspective of H.248 entities -, is still not fully clear. Description should provide more info … )} |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported True Session Resumption shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.6	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0006) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported. |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported True Renegotiation shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.7	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0007) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. {Editor’s note (2013-01 meeting):: Do we need other criteria for a renegotiation trigger, like nbr fragments sent/rec., nbr octets sent/rec., etc?} |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.8	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0008) |
|  | Description: | This property declares if the MG – if acting as a TLS-server – shall authenticate the remote TLS client. |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS-server, the client shall not be authenticated. True In case the MG acts as a TLS-server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

### 10.6.1	General default behaviour (MGC does not influence TLS capability negotiations)

Package tlscn is optional for H.248 profile specifications with TLS support. When the package is not used, then the MG shall independently negotiate TLS configuration(s) with remote TLS endpoints.

Such MG-autonomous TLS capability negotiations shall be compliant to [IETF RFC 5246]. The negotiations would be based on the TLS MG profile and possibly preprovisioned negotiation policies.

### 10.6.2	MGC-controlled TLS capability negotiations: Options

There are two options for MGC-controlled TLS capability negotiations, with the aim to determine the TLS domain profile instance, which is to be used by the MG to form the TLS endpoint profile (by also taking into account the TLS MG profile), see Table 8:

Table 8 – Variants of MGC-controlled TLS capability negotiations

| Method | H.248 elements | Protocol procedures, see: |
| --- | --- | --- |
| a) TLS profile approach | Property: - tlscn/dpid | clause 9.6.4 |
| b) Individual TLS protocol parameter control | Properties: - tlscn/tlsv - tlscn/cs - tlscn/cm - tlscn/ssr - tlscn/srsc - tlscn/rp - tlscn/car | clause 9.6.5 |

{Editor’s note (2013-06 meeting): theoretically, a third Method c) might be added: 
Explicit signalling of tlscn/dpid and additionally explicit signalling of some other tlscn/x properties with x != dpid. Or should this be explicitly forbidden and thus be rejected by the MG? Contributions are solicited….}

### 10.6.3	Determining TLS Related Capabilities

#### 10.6.3.1	Auditing supported TLS protocol profiles

The MGC may access supported TLS protocol profile(s) by auditing property tlscn/dpid. The identifier of a particular TLS protocol profile relates to a reference on a TLS protocol profile description, which might be for instance explicitly part of an H.248 profile definition or documented anywhere else.

#### 10.6.3.2	Auditing individual supported TLS protocol capabilities

The MGC may determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr). Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS-versions supported by the MG without any specific sequence.

The Supported Cipher Suite property shall include the cipher suite TLS_NULL_WITH_NULL_NULL (“0000”), which is used as the initial cipher suite during the first handshake for the TLS-session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

{Editor’s note (2013-01 meeting): TLS-resumption use cases to be identified.}

### 10.6.4	Indication of TLS profile(s) to MG by MGC

The MGC may request TLS domain profile(s) support from the MG via property dpid, leading to following MG behaviour:

- Built intersection between locally supported TLS capabilities (the “TLS MG profile”) with indicated TLS domain profile(s), resulting in a TLS endpoint profile (see also clause 14);

- The TLS endpoint profile shall be the baseline TLS negotiations with the remote TLS entity (during TLS security session establishment/resumption procedures).

(To be checked again)

### 10.6.5	Indication of individual TLS protocol capabilities to MG by MGC

{Editor’s note (2013-01 meeting): In this case the “TLD Domain profile” instance to be used by the MG is (partially) defined by the MGC by the actually signalled tlscn/x properties with x != dpid, isn’t it? Those tlscn/x properties, which are not explicitly signalled by the MGC assume their values as provisioned in MG’s “MG TLD profile”, - to be cross-checked again.}

#### 10.6.5.1	TLS protocol parameter “TLS version”

The list of TLS versions used for the TLS procedures of the session is determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned (ie., the property value for dpid) on the MG as typically default configuration.

NOTE – The TLS endpoint profile again (see clause 10.x.y) relates to the intersection/match of TLS MG profile (as provisioned on MG) and individual explicitly signalled tlscn/x properties with “x != dpid”.

In case the MGC signals the property TLS Versions (tlsv), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG is acting as a TLS client, the highest TLS versions as defined by the specific TLS-endpoint profile are included into the initial TLS-handshake message (ClientHello). In case the MG acts as a TLS server, the MG shall select the highest TLS version that is present in the list of TLS-versions offered from the TLS client and in the TLS versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS-handshake is regarded as failed.

{Editor’s note (2012-09 meeting): Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

#### 10.6.5.2	TLS protocol parameter “cipher suite”

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Chipher Suites (cs), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS version.

#### 10.6.5.3	TLS protocol parameter “compression method”

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Compression Methods (cm), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle as the negotiation of the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS version.

#### 10.6.5.4	TLS protocol procedure “client authentication”

The MG’s TLS server implementation shall support the client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid).

In case the MGC signals property Client Authentication Required (car), the MG shall use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

#### 10.6.5.5	TLS protocol procedure “self-signed client certificates”

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute “a=fingerprint”. If present in the Remote Descriptor, the client certificate shall be self-signed and shall be verified against the fingerprint.

#### 10.6.5.6	TLS protocol procedure “signed client certificates”

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no SDP fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile. This list of root certificates is supposed to be configured per dpid on the MG.

#### 10.6.5.7	TLS protocol procedure “server authentication”

In case the TLS endpoint in the MG is configured as a TLS server, the MG shall provide the server certificate to the client as defined by the TLS domain profile.

#### 10.6.5.8	TLS protocol procedure “session resumption”

FIX THIS

#### 10.6.5.9	TLS protocol procedure “renegotiation support”

The TLS implementation in the MG shall provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile.

In case the MGC signals property Support of Renegotiation of the Security Context (srsc), the MG shall use this value instead.

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, taking into account property Renegotiation Period (rp).

In case the TLS endpoint profile disables renegotiation, the MG shall never initiate a renegotiation. If receiving a renegotiation request from the remote TLS-endpoint, the MG shall respond with a TLS no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS-version.

If renegotiation shall be supported, the MG shall accept and process a renegotiation request from the remote TLS endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to value “0”, no renegotiation shall be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS-handshake completed successfully. On expiration of the timer the MG shall trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS-version. In case the MG receives a trigger for the renegotiation from the remote TLS-endpoint while this timer is running, the MG shall cancel and restart this timer.

{Editor’s note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.
However, to be considered: “If the MGC provides the property tlscn/rp, then the MG will use this value instead of the provisioned default TLD Domain profile’s “rp” value }

# 11	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x####) |
|  | Description: | This package defines capabilities for support of TLS alerts, which are exchanged between the remote and MG local TLS endpoint, following the TLS alert protocol. This package version is limited to the so-called TLS error alerts, as defined by clause 7.2.2/[IETF RFC 5246]. The package provides capabilities for MGC triggered sending of TLS alerty by the MG, as well as a reporting mechanism to notify the MGC about received TLS alerts by the MG. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

### 11.2.1	TLS Error Alert

|  | Event Name: | TLS error alert |
| --- | --- | --- |
|  | Event ID: | swa (0x0003) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the termination of the TLS session. The abnormal condition may be detected on the local or on the remote TLS endpoint. The event is limited on TLS error alerts and excludes the TLS closure alert event. The event shall be set at stream level. |

#### 11.2.1.1	EventsDescriptor parameters

None

#### 11.2.1.2	ObservedEventsDescriptor parameters

##### 11.2.1.2.1	Bearer level Alert Initiator

|  | Parameter Name: | Bearer level alert initiator |
| --- | --- | --- |
|  | Parameter ID: | blai (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Abnormal condition detected by the MG local TLS endpoint “remote” (0x0002) Abnormal condition detected by the remote TLS endpoint |
|  | Default: | NA |

##### 11.2.1.2.2	Error Alert Type

|  | Parameter Name: | Error alert type |
| --- | --- | --- |
|  | Parameter ID: | eat (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition by referencing the TLS error alert type. The codepoints registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Default: | NA |

## 11.3	Signals

### 11.3.1	MGC triggered TLS error alert

|  | Signal Name: | MGC triggered TLS error alert |
| --- | --- | --- |
|  | Signal ID: | mgcea (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, the session shall be terminated. NOTE: the TLS “close_notify(0)” alert is excluded because it does not belong to the TLS error alert category. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

11.3.1.1	Additional parameters

11.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246] , i.e., either a warning or fatal alert severity. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

11.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml ). |
|  | Default: | NA |

## 11.4	Statistics

None.

## 11.5	Error codes

None.

## 11.6	Procedures

### 11.6.1	TLS warning error alerts

#### 11.6.1.1	General behaviour

The TLS session should keep active (and not released) when TLS alerts with warning severity are exchanged between local and remote TLS endpoints.

#### 11.6.1.2	MG received TLS error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at warning level, it shall be notified towards MGC by

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to “IANA registered codepoint”; and

- the tagging mgea/blaw equal to “remote”.

#### 11.6.1.3	MG locally generated TLS error alerts and sent to remote TLS endpoint

Same as previous clause 11.6.1.1, but a tagging mgea/blaw equal to “local”.

#### 11.6.1.4	MGC triggered TLS error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alerts towards the remote TLS endpoint.

### 11.6.2	TLS fatal error alerts

#### 11.6.2.1	General behaviour

Whenever a TLS fatal error alert is exchanged between local and remote TLS endpoints, there will be always a subsequent, immediate release procedure of the TLS session.

#### 11.6.2.2	MG received TLS fatal error alerts from remote TLS endpoint

As in clause 11.6.1.2. The subsequently expected TLS closure_notify alert indicates a TLS session release request.

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic session control package procedures of clause 8.6.5.1. The text is not yet aligned. Need to be fixed.}

#### 11.6.2.3	MG locally generated TLS fatal error alerts and sent to remote TLS endpoint

As in clause 11.6.1.3. The MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause x.y.z) or autonomously started by the MG itself (see clause x.y.z).

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic and extended session control package procedures. The text is not yet aligned. Need to be fixed.}

#### 11.6.2.4	MGC triggered TLS fatal error alert, sent by the MG to remote TLS endpoint

As in clause 11.6.1.4. The MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause x.y.z) or autonomously started by the MG itself (see clause x.y.z).

[…FIXTHIS]

{Editor’s note (2013-06): the procedure overlapps with the TLS basic and extended session control package procedures. The text is not yet aligned. Need to be fixed.}

### 11.6.3	Lower layer issues

There might be issues in the protocol layers below the TLS layer.

Example use case:

- Established IP bearer path, i.e. there is an active TLS session;

- The underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- Hence, there would be still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective;

- And therefore the TLS session would be interrupted, not any TLS alert could be sent to the remote side.

Procedures for the maintenance of lower layer issues are out of scope of this package, rather subject of exsting H.248 error handling capabilities.

# 12	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x####) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TLS protocol. The statistics are applicable as long as the MG is aware that the H.248 Stream carries TLS traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 12.1	Properties

None.

## 12.2	Events

None.

## 12.3	Signals

None.

## 12.4	Statistics

{Editor’s note (2013-06 meeting): the terminology (and hence semantics) is not yet consistent across all statistics defined by this package. E.g., terms “TLS fragment”, “TLS message”, “TLS application data”, “TLS-over-L4 packet”. We need a data model which is tightly linked to the data structures of RFC 5246 (“the TLS RFC is unfortunately lacking precise definitions”). The statistic definitions should be then updated again.}

### 12.4.1	Number of Received TLS-Fragments

|  | Statistic Name: | Number of Received TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recfrag (0x0001) |
|  | Description: | Provides the number of packets received on the Termination or Stream since the statistic has been set. The packets represent the ingress TLS-fragments of all TLS flows of an H.248 Stream. It is the total number of TLS-over-L4 packets, inclusive of TLS session establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.2	Number of Sent TLS-Fragments

|  | Statistic Name: | Number of Sent TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentfrag (0x0002) |
|  | Description: | Provides the number of packets sent from the Termination or Stream since the statistic has been set. The packets represent the egress TLS-fragments of all TLS flows of an H.248 Stream. It is the total number of TLS-over-L4 packets, inclusive of TLS session establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.3	TLS Octets Sent

|  | Statistic Name: | TLS Octets Sent |
| --- | --- | --- |
|  | Statistic ID: | tlsos (0x0003) |
|  | Description: | This Statistic provides the number of octets sent from the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress TLS messages of all TLS flows of an H.248 Stream, excluding all lower layer overhead. It is the total number of octets (i.e., including TLS header) transmitted in TLS messages. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.4	TLS Octets Received

|  | Statistic Name: | TLS Octets Received |
| --- | --- | --- |
|  | Statistic ID: | tlsor (0x0004) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress TLS messages of all TLS flows of an H.248 Stream excluding all lower layer overhead.. It is the total number of octets (i.e., including TLS header) received in TLS messages. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.5	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0005) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the layer above TLS. Received application data messages from the remote TLS endpoint are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.6	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0006) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). {Editor’s note (2013-06): further comment concerning applicability for use case according cl. 6.2.2.2?} |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 12.5	Error codes

None.

## 12.6	Procedures

### 12.6.1	Ingress TLS traffic – Statistic "TLS Packets Received"

Every incoming L4 packet, carrying a TLS fragment, which is successfully delivered to its H.248 Context and H.248 TLS Stream/Termination is counted by Statistic tlstv/recfrag.

### 12.6.2	Ingress TLS traffic – Statistic "TLS Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/tlsor.

### 12.6.3	Ingress TLS traffic – Statistic "TLS Application Data Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/recado.

### 12.6.4	Egress TLS traffic – Statistic "TLS Packets Sent"

Every outgoing TLS-over-L4 packet, sent from a H.248 TLS Stream/Termination, is counted by Statistic tlstv/sentado.

### 12.6.5	Egress TLS traffic – Statistic "TLS Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 according to Statistic tlstv/tlsos.

### 12.6.6	Ingress TLS traffic – Statistic "TLS Application Data Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 packets according Statistic tlstv/sentado.

# 13	Package-less TLS control

{Editor’s note (2013-01 meeting): dedicated chapter on SDP based control required, addressing the following

Purpose of this clause is to describe the SDP-based semantics for TLS control, which needs the consideration of

- SDP usage in H.248.69, H.248.84

- how the package-based elements/procedures would be realized via SDP?

- which functionality would be out of scope of SDP control?

- assumption of mixed Property- & SDP-based control

First comments (from C-20):

To requirement of “Originating/Terminating Bearer control protocol procedures”:

[other package elements]

Contributions are solicited.}

## 13.1	Related to TLS bearer establishment

### 13.1.1	Overview – principal H.248 control steps

{Editor’s note (2013-06 meeting): the text is this clause is still subject of discussion … }

Figure 7 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TLS-over-L4/IP bearer connections in environments demanding for NAT traversal support. The dashed boxes are L4-dependent (e.g., in case of TCP see [ITU-T H.248.TCP, H.248.69, H.248.84]):

Figure 7 – Principal H.248 control steps for establishing a TLS-over-L4 bearer

It is apparent that some steps might be combined (e.g., 1, 2 and 7) and that specific steps might be optional (e.g., 3, 5, 8, 10).

Table 9 provides a preliminary inventory of possible control elements from H.248 perspective (at the example of TCP as L4 protocol). The column “H.248/SDP element” refers to “package-less TLS control”.

Table 9 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase I: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property, e.g., TCP: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure (e.g. TCP active or passive OPEN) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „Established“) | Event, e.g., TCP: tcpbcc/BNCChange | — |  |
| Establishment phase II: „IP security session establishment“ |  |  |  |  |
| 7 | Bearer type (L4+ = ‘TLS’) indication to SEP | Indirect: package elements used from TLS-related H.248 packages | “m=”-line <proto> | NOTE 2. SDP may be ambiguous due to “protocol stack” concept (NOTE 1). |
| 8 | Block / Unblock L4+ Bearer Control Procedures at SEP | Property: tlsbsc/bceb | None (?) | i.e., handling of - TLS handshake protocol - TLS cipher change protocol - TLS alert protocol |
| 9 | Start IP Security Session Establishment Procedure („TLS negotiations“) | Event: tlsbsc/BNCChange Signal: tlsbsc/EstBNC | None (?) | above TLS protocols are enabled |
| 10 | Local L4+ NAT traversal support | H.248.78 | e.g., MSRP: “a=msrp-cema” (?) | NOTE: L4+ NAT-T may be also requested for non-TLS. |
| 11 | Successfully established IP Transport Security Session | Event: tlsbsc/BNCChange | — |  |
| Communication phase: |  |  |  |  |
| 12 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP “m=” line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well known issue of this element. NOTE 2 – Step 7 may be merged with step 1 in case of tightly coupled “TLS/L4” control. |  |  |  |  |

{Editor’s note (2013-06 meeting): possible conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

Steps 3 to 5 in more detail, dependent on TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table 9.

}

### 13.1.2	Step 1: Bearer endpoint creation

Not SDP related, based on H.248 Command request.

### 13.1.3	Step 2: Bearer type indication for IP transport “L4/L3”

Starting point should be the SDP-based approach by using the SDP “m=” line <proto> element as transport indicator. The SDP method shall work for all required TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]).

Conditional approach, dependent on:

C3.1: unambiguous indication of transport protocol “L4” (Issue: “m=”-line <proto> relates to a protocol stack based indicator, rather than a distinct identifier to a specific L4 protocol)

C3.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

### 13.1.4	Step 3: Block / Unblock L4 bearer control procedures at SEP

Out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 13.1.5	Step 4: Start IP transport connection establishment procedure

Relevant for connection-oriented IP transport protocols, which are used as TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]), but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 13.1.6	Step 5: Local L4/L3 NAT traversal support

Crucial step because TLS will fail without any successfully established IP transport connection. However, out of scope of this Recommendation. There are dedicated L4/L3 NAT-T support methods, such as [ITU-T H.248.37], [ITU-T H.248.50] or [ITU-T H.248.84].

It should be noted that above example NAT-T mechanisms are primarily enabled using H.248 packages (“and not via LD/RD-embedded SDP signalling”).

### 13.1.7	Step 6: Reporting successfully established IP transport connection

Not supported by SDP, but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 13.1.8	Step 7: Bearer type indication for TLS

If SDP based, then only same method as in step 2.

Conditional approach, dependent on:

C7.1: unambiguous indication, see C3.1.

### 13.1.9	Step 8: Block / Unblock TLS security session control procedures at SEP

Not supported by SDP.

### 13.1.10	Step 9: Start TLS security session establishment procedure

There is not any SDP as explicit indicator for outgoing and incoming TLS “handshake procedures” as part of TLS session establishment.

### 13.1.11	Step 10: Local L4+ NAT traversal support

FIXTHIS

### 13.1.12	Step 11: Reporting successfully established TLS session

Not supported by SDP.

### 13.1.13	Step 12: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TLS traffic) are out of scope of the SDP.

## 13.2	Related to TLS authentication

NOTE 1 – This clause satisfies requirement R-8.3.3.2.2/1//[ITU-T H.248.TLSPROF].

The SDP “a=fingerprint:” attribute (according [IETF RFC 4572]) shall be used under the condition of H.248 gateways involved in network architectures (given by trust model and SIP/SDP as application control protocol) as described by this RFC.

NOTE 2 – The SDP based method is exclusive, i.e., there are not any alternative H.248 signalling elements defined.

If the SDP “a=fingerprint:” element would be required by an H.248 profile, then

- the MGC would provide a fully specified SDP fingerprint attribute in the H.248 RD and

- the MGC would provide either a fully specified or wildcarded (see [ITU-T H.248.39]) SDP fingerprint attribute in the H.248 LD

to the MG.

Usage of the the SDP “a=fingerprint:” attribute at the H.248 interface validates the bearer-to-call binding between SIP call and TLS secured bearer (see e.g. Figure 6/[ITU-T H.248.TLSPROF]).

## 13.3	Related to TLS bearer release

FIXTHIS

# 14	The TLS profile concept

## 14.1	TLS protocol profiles

A MG may be requested to support more than one TLS domain. For each such TLS domain different “TLS services” may be required from the MG. For each TLS domain those required TLS behaviours are defined by a domain-sopecific TLS domain profile.

Generally the TLS domain profile may be provisioned on the MG. E.g., when using the tlscn package, then property TLS Domain Profile Identifier (dpid) shall be used to refer to a TLS domain profile. This property shall be then aligned between the MG and the MGC, as the MGC may use it to indicate to the MG which TLS domain profile should be applicable for the creation of a TLS endpoint in the MG.

Important to note: any concrete, detailed, normative specifications of TLS protocol profiles are out of scope of this Recommendation!

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as TLS MG profile”(see also clause 3.2.1).

{Editor’s note (2012-09 meeting): Thus, if the “dpid” property is not signalled, does the applied assumed “default” TLS Domain profile correspond to the TLS client/server role specific subset of this “MG TLS profile”?}

On creation of a TLS endpoint in the MG, the intersection of the selected TLS domain profile(s) and the TLS MG profile will be build. This intersection may be termed as the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the TLS-enabled endpoint (i.e., the MG H.248 SEP/termination).

Figure 8 illustrates the principle relation between the profiles.

Figure 8 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile

Normally, the result of intersection would be a TLS endpoint profile that exactly matches with the requested TLS domain profile(s). However, there might be partial matches in principle as well, as depicted by the example in Figure 8.

## 14.2	Illustration of TLS profile concept

The indication and negotiation of media configurations across the H.248 interface is subject of [ITU-T H.248.80]. A TLS configuration relates effectively to a subset of a media configuration (see clause 3.2.1.2/[ITU-T H.248.80]). Table 10 opposes correspondent terms and profile types:

Table 10 – Analogon between H.248.80 media configurations and TLS profile types

| H.248.80 generic media configuration | TLS-specific media configuration | H.248.TLS TLS profile term: |
| --- | --- | --- |
| Overall term: |  |  |
| Configuration (clause 3.2.1 / [ITU-T H.248.80]) | TLS configuration | TLS profile |
| Qualified term: |  |  |
| Prefered configuration (e.g., prefered codec list) | Prefered TLS configuration (NOTE 1) | TLS domain profile |
| Supported configuration (e.g., supported codec list) | Supported TLS configuration | TLS MG profile |
| Actual configuration (e.g., actual codec list) | Actual TLS configuration | TLS endpoint profile |
| NOTE 1 – There is a subtle difference between H.248.80 and this Recommendation: there are not any SIP-level SDP Offer/Answer negotiations concerning TLS configurations due to the fact that TLS provides itself an embedded TLS handshake protocol for TLS-level negotiations. Thus, there is neither a “Offered TLS configuration” nor “Answered TLS configuration” at application control protocol level. |  |  |

Figure 9 provides an illustration of the three TLS profile types, as in use during TLS security session establishment:

Figure 9 – Illustration of TLS profile concept

In general, the TLS MG profile reflect the status of implemented TLS capabilities, it may be therefore principally a subset, superset or equal to a requested TLS domain profile.

H.248 profiles could also simplify network solutions by mandating supported TLS capabilities, which could lead to the equation of “TLS MG profile equal to TLS domain profile”, resulting in “TLS endpoint profile also equal to TLS domain profile”.

{Editor’s note (2012-09 meeting): two clarifications are requested:

- notion of “overall TLS profle”( what exactly? required?)

- relation of “MG TLS profile” to “TLS domain profile(s)”: subset, equal, superset, - all options possible?

Contributions are solicited.}

## 14.3	Example for the TLS MG profile concept

Appendix III provides examples for TLS domain profiles .Table 11 depicts an example for a TLS MG profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 11 – Example of a TLS MG profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlscn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlscn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlscn/cm |
| Client authentication required | yes | tlscn/car |
| Resumption support | no | tlscn/ssr |
| Extensions |  |  |

The following Table 12 shows an example how the intersection of a TLS domain profile and the TLS MG profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. Table 3 does not indicate which properties can be provided by the MGC.

Table 12 – Example of an intersection between a TLS domain profile and a TLS MG profile

| Property | TLS Domain profile | TLS MG profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B Provisioned per TLS domain profile |
| MG certificate |  | List of certificates, one certificate per TLS domain profile | Provisioned per TLS domain profile |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |

{Editor’s note (2012-09 meeting): A clear view on the MG certificate is still missing. It is used during the TLS handshake, thus it is a part of the “set of TLS related parameter and procedures” (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the TLS MG profile. The MG certificate is network element specific and TLS-domain specific.}

# 15	Security considerations

…

Annex A

State modelling for 
TLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TLS basic session control package (clause 8) is tightly coupled to a state model, because the event (tlsbsc/BNCChange) and signal (tlsbsc/EstBNC) are related to state transitioning behaviour.

It has to be noted that the IETF TLS related RFCs do not define any explicit state model with regards to TLS security session endpoints. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tlsbsc is described by this Annex.

## A.2	Original state model for TLS session endpoints

There is not any standardized state model for TLS (in comparison to TCP).

## A.2	Simplified state model for H.248-based TLS basic session control

The TLS basic session control package makes a couple of assumptions, similar as outlined in Annex A/[H.248.TCP] for TCP.

The correspondent simplified model for TLS is depicted by Figure A.1:

Figure A.1 – Simplified state model for H.248-based TLS basic session control

[…]

Transitioning from Idle to Established:

The bearer establishment related TLS handshake procedures will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters.

- The TLS-Finish message has been received and has been successfully verified.

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event tlsbsc/BNCChange{type=Est} can be used to notify the MGC accordingly.

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use case “Terminal-to-MG TLS session, provisioned TLS service negotiation”

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, but the MGC is in any case aware of the applied TLS network domain profile for the secured bearer traffic.

- IP transport bearer type: ‘TCP’ in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS  (see also clause 6.2.1)

- Bearer path protocol layering: Application/TLS/TCP/IP (i.e., not any tunnelled transport).

- UE: TCPclient and TLS client, MG: TCP server and TLS server

- H.248 MG TCP-mode: TCP-proxy. 
NOTE – In more detail: L4/L3 NAT traversal support requirements during the establishment phase of TCP bearer connection may imply e.g. initially a TCP merge mode (see [ITU-T H.248.84]), which is later on transitioning to a TCP proxy mode (due to the TLS protocol termination here for TLS to non-TLS interworking). Another option is already the TCP proxy mode from the very beginning, e.g., in case of lacking NAT-T support.

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed.

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG (e.g., in case of an H.248 profile without support of the TLS capability negotiation package, see clause 10).

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case “Terminal-to-MG TLS session, MGC-controlled TLS service negotiation”

This use case is an extension of the first use case (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level via usage of the the TLS capability negotiation package (see clause XY). The first use case is based on preconditions which are partially subject of network provisioning. This use case is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

## I.3	Use case “WebRTC to NGN/IMS interworking function with DTLS-to-TLS support”

To be inserted.

{Editor’s note (2013-01): this use case is also considered from TCP perspective, thus covered by Appendix I.1/H.248.TCP. I.e., there might be a reference added later on in this clause …}

## I.x	Use-case “TLS-based transport security for facsimile packet relay service T.38”

Background:

[ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the “TLS to non-TLS” interworking case (see clause 6.2.1);

- UDP transport implies DTLS;

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation)

## II.1	TLS to non-TLS interworking with TCP as example bearer type

NOTE: TCP is used as exemplary L4 protocol for TLS. [ITU-T H.248.TCP] provides further details on TCP bearer connection control procedures and takes precedence in case of discrepancies between both Recommendations.

### II.1.1	Initial session setup using SDP security descriptions in application control signalling

This use case is presented in Appendix I.1. Figure II.1 outlines the network configuration as used in the message flow.

Figure II.1 – Configuration for the example call flow

The scenario shows a simplified SIP session establishment. The TCP bearer connection establishment direction is defined at SIP level by the SDP setup attribute (based on [IETF RFC 4145]), and the fingerprint of the UE’s certificate is included in the SDP (based on [IETF RFC 4572]).

The H.248 MG is triggered by the MGC to create two terminations, whereby the bearer for the first SEP/termination is secured by the use of TLS. In the RD of this termination the MG also receives the fingerprint of the UE’s certificate.

### II.1.2	Traffic flow examples for communication establishment phase

There are multiple options for establishing the communication path for the TLS to non-TLS interworking. Following estalishment examples are illustrated:

- Scenario: unclear situation whether remote NAT devices may demand for L4/L3 NAT-T support, leading to an initial establishment and connectivity check of the end-to-end IP L4 connection before the initiation of the TLS security session; the MGC explicitly want to trigger (or not) TLS establishment, dependent on a successfully established IP L4 connection (see clause II.1.2.1).

- Scenario: NAT-T support seems to be not required, however, the MGC separates the communication establishment between the TLS network domain and non-TLS network domain, e.g., due to separated capability negotiations in each domain at application control level (see clause II.1.2.2).

- Scenario: the end-to-end IP L4 connection should be autonomously established by the MG (e.g., because the MGC does not expect any blocking issues such as L4 protocol deadlocks). The MGC wants to be notified about L4 connectivity and then trigger explicitly TLS security session establishment (see clause II.1.2.3).

- Scenario: the MGC does not expect any issues at all and may optimize communication establishment by minimizing setup delay times, i.e., enabling full MG autonomous behaviour across the SEPP and vertical protocol layers (see clause II.1.2.4).

NOTE – This is a non-exhaustive list of variants, just a few examples.

### II.1.2.1	Example: TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)

The call flow (Figure II.2) shows the setup of the TCP bearer connection as well as the subsequent  TLS handshake on the first stream endpoint T1(S1). The MGC initially blocks any TLS security session establishment due to uncertainity whether the underlying TCP bearer connection could resolve all “NAT traversal challenges”. After the MG reporting of a sucessfully established TCP bearer connection, the MGC initiates TLS session establishment. The TLS server role is supposed to be located at MG side in this example.

The self-signed certificate received from the UE is verified by the use of the fingerprint. After the TLS handshake is finished, application data may flow between the UE and the remote TLS session endpoint. In the traffic flow presented here the MG will forward these data in the TCP proxy mode.

Figure II.2 – Example setup of a TLS to non-TLS IWF 
(TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection))

#### II.1.2.2	Example: combined TLS/L4 establishment (decoupled end-to-end L4 connection)

Similar scenario as in clause II.1.2.1, but the successful L4 connections establishment towards remote TLS endpoint (i.e., UE-X) shall immediately trigger the TLS session establishment without MGC involvement. Such a behaviour is achieved by setting property tlsesc/plslink to TRUE (at SEP T1(S1)), see Figure II.3.

Figure II.3 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment (decoupled end-to-end L4 connection))

It may be reminded again that the final MG notifications concerning a successfully established end-to-end IP bearer path are basically optional (dependent on MGC).

#### II.1.2.3	Example: TLS establishment decoupled from L4 establishment (but interlinked end-to-end L4 connection establishment)

Similar scenario as in clause II.1.2.1, but now providing an interlinked TCP bearer connection establishment. Such a behaviour is achieved by setting property tcpecc/sepplink to TRUE (at both SEPs, if the establishment direction would be unknown from MGC side), see Figure II.4.

Figure II.4 – Example setup of a TLS to non-TLS IWF
(TLS establishment decoupled from L4 establishment (but interlinked end-to-end L4 connection establishment))

#### II.1.2.4	Example: combined TLS/L4 establishment (and also interlinked end-to-end L4 connection establishment)

If possible, then the MGC could delegate the “entire” control of end-to-end IP bearer path establishment down to the MG, by enabling interlinkage at L4-SEPP level as well as between TLS and L4 at SEP T1(S1), see Figure II.5.

Figure II.5 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment (plus interlinked end-to-end L4 connection establishment))

### II.1.3	Traffic flow examples for communication release phase

There are again multiple options for release of the IP bearer path for the TLS to non-TLS interworking scenario. Following release examples are illustrated:

- Scenario: the MGC basically separates TLS and L4 bearer release, e.g. due to possible TLS session resume. Thus, any later IP L4 connection release will be explicitly triggered by the MGC (see clause II.1.3.1).

- Scenario: the MGC issues the TLS session release and wants the IP L4 connection release to be initiated by the remote TCP endpoints. E.g., if either UE-X or UE-Y initiates a TCP release activity, then the end-to-end TCP bearer connection should be “MG autonomously” released (see clause II.1.3.2).

- Scenario: similar as previous scenario, but now MG should immediately start the IP L4 connection release towards the remote TLS/L4 endpoint. (see clause II.1.3.3).

NOTE – This is a non-exhaustive list of variants, just a few examples.

Concerning MG autonomous support with respect to TLS session release procedures: it may be noted  that for all release examples it is assumed that the properties tlsesc/plslink, tlsesc/sepplink and tcpecc/sepplink are set to their defaults if not otherwise specified.

#### II.1.3.1	Example: TLS session release initiated by remote TLS endpoint and MGC enforced immediate L4 connection release

The MGC is expecting an incoming TLS session release (see Figure II.6). UE-X initiates the TLS session release by sending a TLS alert (close_notify) message.

Figure II.6 – Example TLS session release, initiated by remote TLS endpoint 
(MGC enforces immediate L4 connection release)

The MG reports a successfully released TLS session to the MGC, which then decides to release also the underlying TCP bearer connection, towards both remote UEs.

#### II.1.3.2	Example: outgoing TLS session release, local TLS/L4 decoupled, coupled end-to-end L4 connection release

Figure II.7 depicts a situation when the MGC initiates the TLS session release towards UE-X, and configures the MG to autonomously release any possible end-to-end TCP bearer connection release, which might be initiated by one (or both) remote UE(s).

Figure II.7 – Example TLS session release, local TLS/L4 decoupled, 
but coupled end-to-end L4 connection release)

#### II.1.3.3	Example: outgoing TLS session release, coupled TLS/L4 and end-to-end L4 connection release

Similar as previous example (clause II.1.3.3), but now the MGC wants explicitly start TCP bearer connection release (e.g., in order to be independent of remote UEs), see Figure II.8.

Figure II.8 – Example TLS session release, 
coupled TLS/L4 and end-to-end L4 connection release

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some exemplary TLS profiles.

## III.1	Typical Internet TLS profile

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | signed, self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

Network domain security is defined in [ETSI TS 133.310] with the normative TLS domain profile in Annex O. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | see client |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) “TLS_DH_anon_WITH_” (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [IETF RFC 4346]. NOTE 3 – Because the “the key exchange method shall not be anonymous”. |  |  |  |

{Editor’s note (2013-01): Preferred cipher suites (ordered list): negotiation aspect?}

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [OMA-TS-TLS-V1_0-20080902-A], a “public key TLS domain profile” and a “pre-shared key TLS domain profile”.

FIXTHIS

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE z1) |  |  |  |
| 3.6 | Extensions: | [IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – #. NOTE z1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | see client |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | see client |
| NOTE 1 – # |  |  |  |

### III.3.2	OMA pre-shared key TLS domain profile

see [OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

## III.5	IETF example of a national TLS domain profile

[b-IETF RFC 6460] represents an example of a national TLS domain profile. Inter alia, TLS protocol version and ciper suites are profiled.

## III.6	ITU-T TLS domain profile for NGN signalling and management plane

[b-ITU-T Y.2704], clause 9.1, defines a TLS domain profile, recommended for signalling and management interfaces in NGNs.

Appendix IV

Illustration of protocol semantics 
of the TLS basic session control package

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tlsbsc package (clause 8) defines the basic TLS security session control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## IV.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS session endpoint.

## IV.3	Establishment of TLS security sessions

### IV.3.1	Successful establishment – Terminating side

See Figure IV.1, termed as use case (E.1):

Figure IV.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of TLS session establishment (1).

- Early incoming TLS messages are discarded by the MG (2).

- The MGC unblocks the TLS SEP (3).

- TLS session establishment (5 to 7); MG acting as TLS server.

- Optional notification of MGC concerning available TLS session for application data transfer (8), if subscription to event by MGC (4).

### IV.3.2	Successful establishment – Originating side

See Figure IV.2, use case (E.2)

Figure IV.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause IV.3.1

- TLS session establishment (5 to 7), triggered by MGC (4); MG acting as TLS client.

- Optional notification of MGC concerning available TLS session for application data transfer (9), if subscription to event by MGC (8).

### IV.3.3	Unsuccessful establishment

## IV.4	Release of TLS security sessions

### IV.4.1	Successful release – Terminating side

See Figure IV.4, termed as use case (R.1):

Figure IV.4 – Successful release – Terminating side

More details for this example:

- MG receives an incoming TLS close_notify alert (2), the indication for TLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released TLS session (4), if subscription to event by MGC (1).

### IV.4.2	Successful release – Originating side

See Figure IV.5, use cases (R.2) …

Figure IV.5 – Successful release – Originating side

More details for this example:

- MGC triggers the TLS session release (1).

- MG sends an outgoing TLS close_notify alert (2), the indication for TLS session release to wards remote TLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released TLS session (5), if subscription to event by MGC (4).

### IV.4.3	Unsuccessful release

Appendix V

Illustration of protocol semantics 
of the TLS extended session control package

(This appendix does not form an integral part of this Recommendation.)

## V.1	Overview

The tlsesc package (clause 9) defines an augmented TLS security session control on top of the base package (tlsbsc, clause 8). The effect of each individual package property is illustrated in clauses V.3 (property tlsesc/sepplink) and V.4 (property tlsesc/plslink). Clause V.5 describes examples of combined usage of both properties.

The illustrated use cases are only examples. An exhaustive consideration of all possible combinations is out of scope.

## V.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS and L4 layers) are highlighted besides the H.248 interface.

Furthermore some Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS/L4 SEP.

## V.3	Usage of SEPP Interlinkage property

Property tlsesc/sepplink is defined in clause 9.1.1. The property affects TLS security session establishment and release procedures. The semantics are correspondent to the TCP layer. See e.g. the illustration and examples in [ITU-T H.248.TCP], Appendix ?.3.

## V.4	Usage of Protocol Layers Interlinkage property

Property tlsesc/plslink is defined in clause 9.1.2. The property impact is depicted in below two illustrations. Figure V.1 recalls again the default mode according base package (tlsbsc):

Figure V.1 – Illustration of Protocol Layers Interlinkage property 
– Property „plslink = FALSE“

The MGC is responsible for individual control of layer-specific bearer control procedures.

Figure V.2 illustrates the tightly coupled mode, which is conditional to the aimed communication service, but when applicable, it provides some benefits such as minimal MGC involvement and performance optimized bearer establishment and/or release.

Figure V.2 – Illustration of Protocol Layers Interlinkage property 
– Property „plslink = TRUE“

### V.4.1	Property „plslink = FALSE“

Semantic FALSE implies base package behaviour.

### V.4.2	Property „plslink = TRUE“

The property may be applied for the establishment or release phase or both.

### V.4.2.1	Bearer establishment phase

#### V.4.2.1.1	Incoming direction

Figure V.3 illustrates the normal case: the start of any TLS security session establishment precedes a successfully established IP transport connection. Thus, the MG receives an incoming L4 Est.req (2). When L4 is established, then immediately an outgoing TLS session establishment (4) is initiated by the MG.

Figure V.3 – Property „plslink = TRUE“ – L4 incoming bearer establishment request

The actually impossible case of TLS establishment request first, without yet successfully established IP transport, is indicated Figure V.4:

Figure V.4 – Property „plslink = TRUE“ – TLS incoming bearer establishment request

Theoretical example:

E.g., case of TLS/TCP bearer: the very first incoming TCP packet (with set SYN flag) would contain already a TLS ClientHello as TCP payload, but the local TCP connection state is still not Established!

##### V.4.2.1.2	Outgoing direction

SEP T1(S1) is triggered for an outgoing L4 bearer establishment procedure (see Figure V.5). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful IP transport connection establishment (3, 4), the MG initiates immediately an outgoing TLS session establishment (5).

Figure V.5 – Property „plslink = TRUE“ – L4 stimuli

Figure V.5 shows also the optional event notifications related to bearer specific state transitioning (here: L4 and TLS reporting).

There might be also a TLS-related stimuli first from SEP perspective (without a prior explicit L4 trigger). The scenario is almost identical as the previous case, see Figure V.6:

Figure V.6 – Property „plslink = TRUE“ – TLS stimuli

Example:

In case of TLS/TCP with SEPP interlinkage usage, and property settings 
a) TCP layer: tcpecc/sepplink = FALSE and  
b) TLS layer: tlsesc/sepplink = TRUE, 
then the SEP would receive a “TLS level stimuli” from his partner SEP.

#### V.4.2.2	Bearer release phase

##### V.4.2.2.1	Incoming direction

Figure V.7 illustrates the abnormal case: the start of any IP transport connection release precedes a successfully released TLS security session. The MG receives an incoming L4 Rel.req (2), but would suspend L4 release procedural steps in favour of a TLS session release (3) first. The L4 release procedure is resumed again (5) after successful TLS termination (4).

Figure V.7 – Property „plslink = TRUE“ – L4 incoming bearer release request

The standard case of TLS release request first is depicted in Figure V.8: the TLS session release (2, 3) triggers the subsequent IP transport connection release (4, 6).

Figure V.8 – Property „plslink = TRUE“ – TLS incoming bearer release request

##### V.4.2.2.2	Outgoing direction

SEP T1(S1) is triggered for an outgoing L4 bearer release procedure (see Figure V.9). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful TLS session release (3, 4), the MG initiates immediately an outgoing IP transport connection release (5).

Figure V.9 – Property „plslink = TRUE“ – L4 stimuli

Figure V.10 outlines the case of a TLS-related stimuli:

Figure V.10 – Property „plslink = TRUE“ – TLS stimuli

Appendix IV
Living List related to DTLS and DTLS-SRTP

(This appendix does not form an integral part of this Recommendation)

{Editor’s note (2013-01 meeting): This Appendix is related to “DTLS” and “DTLS related topics”

## IV.1	DTLS protocol

DTLS is derived from TLS, could be thus considered as a single security protocol family. DTLS versioning numbers are (somehow) linked to TLS protocol versions.

Figure IV.1 – TLS/DTLS protocol historic

## IV.2	DTLS related services

There are two slightly different applications:

- DTLS as “transport security” means for UDP/IP bearer traffic

- DTLS as “key exchange” means for RTP/UDP/IP bearer traffic using media security according SRTP

## IV.3	DTLS in scope of H.248.TLS?

There are basically following options:

- DTLS covered by H.248.TLS?

- DTLS in a self-contained, new H.248.DTLS?

- DTLS added to H.248.77 due to link with DTLS-SRTP?

- DTLS covered by H.248.WEBRTC due usage by WebRTC service.

Option #2 (or #1) seems to be preferable.

## IV.4	DTLS use cases

Initial list of possible use cases:

### IV.4.1	Use cases from TLS-to-DTLS interworking perspective

Figure IV.2 provides an interworking example (here: DTLS based secure WebRTC data service).

Figure IV.2 – Example TLS-to-DTLS interworking

### IV.4.2	Use cases from SRTP “key exchange” perspective

SRTP-based media encryption, - there are many key exchange options for SRTP. For example the two outlined in below subclauses.

#### IV.4.2.1	Media-path decoupled key exchange using SDP (via SIP & H.248)

Figure IV.3 recalls again the protocol stack for SDES-based key exchange for SRTP, as in scope of H.248.77 or as part of the 3GPP H.248 Iq profile (29.334):

Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248)

#### IV.4.2.2	Media-path coupled key exchange using DTLS

Figure IV.4 illustrates the alternative of a DTLS-based key exchange for SRTP:

Figure IV.4 – Media-path coupled key exchange using DTLS

### IV.4.3	Two interpretations for „DTLS-SRTP“?

RFC 5764:

This document describes DTLS-SRTP, a SRTP extension for DTLS that

combines the performance and encryption flexibility benefits of SRTP

with the flexibility and convenience of DTLS-integrated key and

association management.  DTLS-SRTP can be viewed in two equivalent

ways: as a new key management method for SRTP, and a new RTP-specific

data format for DTLS.

I.e., there are two possible interpretations from protocol stack point of view, see Figures IV.5 and IV.6.

Figure IV.5 – Interpretation „DTLS only used for SRTP key exchange“

Figure IV.6 – Interpretation „Secure RTP traffic using DTLS“

It has to be noted that these interpretations might be incorrect. To be confirmed.

### IV.4.4	Use cases from WEBRTC perspective

DTLS use cases from WebRTC perspective:

Figure IV.7 – DTLS use cases: „WebRTC data“ & „WebRTC SRTP key exchange“

In Figure IV.7:

1) WebRTC data traffic

2) WebRTC DTLS-based key exchange for SRTP

### IV.4.5	Use cases from Telepresence perspective

Are there any? TBD

Bibliography

[b-ITU-T Y.2704]	Recommendation ITU-T Y.2704 (01/2010), Security mechanisms and procedures for NGN

[b-IETF RFC 6460]	IETF RFC 6460 (2012), Suite B Profile for Transport Layer Security (TLS).

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-IETF tls-minimal]	IETF draft-tschofenig-lwig-tls-minimal (2012), A Minimal (Datagram) Transport Layer Security Implementation.
URL: http://tools.ietf.org/html/draft-tschofenig-lwig-tls-minimal-01

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

____________________

Defined as: “A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint.“ ([IETF RFC 6714])