| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-16 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.TCP |  |  |  |
| Title: | H.248.TCP "Gateway control protocol: TCP support packages" (New): Output draft Ed. 0.6 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.TCP "Gateway control protocol: TCP support packages" (Ed. 0.6).

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TCP Ed. 0.1 | Input (AVD-4297) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4297.zip |
| H.248.TCP Ed. 0.2 | Output (TD-25) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-25.zip |
| H.248.TCP Ed. 0.3 | Input (TD 19 (WP1/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0019 |
| H.248.TCP Ed. 0.4 | Output (TD 19R1 (WP1/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0019 |
| H.248.TCP Ed. 0.5 | Input (AVD-4366) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4366.zip |
| H.248.TCP Ed. 0.6 | Output (TD-16) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-16.zip |

CONTENTS

Page

1 Scope 9 1.1 Applicability statements 9 1.2 Package-less TCP connection control 10 2 References 11 3 Definitions 11 3.1 Terms defined elsewhere 11 3.2 Terms defined in this Recommendation 12 4 Abbreviations and acronyms 13 5 Conventions 14 5.1 Naming of stream endpoints and terminations 14 5.2 TCP bearer 14 5.3 TCP bearer control 14 6 Motivation use cases and models 15 6.1 Bearer connection network use cases 15 6.2 Bearer connection model 16 6.2.1 Model for single TCP termination scope only 16 6.2.2 Model for context-level TCP termination scope 17 6.2.3 Model with focus on context-internal interworking function 17 7 Relation to similar H.248 packages 19 7.1 Overview of TCP related packages 19 7.2 ITU-T Q.1950 19 7.2.1 Package ‘gb’ (generic bearer connection): 20 7.2.2 Package ‘bcp’ (bearer characteristics): 20 7.2.3 Package ‘bnct’ (bearer network connection cut through): 21 7.2.4 Package ‘ri’ (reuse idle): 21 7.3 ITU-T H.248.67 22 7.3.1 Package ‘trm’ (GCP transport mode indication): 22 7.4 ITU-T H.248.84 22 7.4.1 Package-independent procedures for NAT-T with TCP bearers: 23 7.4.2 Package ‘tcphp’ (TCP hole punching): 23 7.4.3 Package ‘tcptv’ (TCP traffic volume metrics): 23 7.4.4 Package ‘tcpccm’ (TCP connection control metrics): 24 7.4.5 Package ‘tcpcqm’ (TCP connection quality metrics): 24 7.4.6 Inventory of TCP functions and their possible impact on H.248 TCP gateways: 24 7.5 ITU-T H.248.43 and ITU-T H.248.79 25 8 TCP basic connection control package 25 8.1 Properties 26 8.2 Events 27 8.2.1 TCP connection state change (“BNC change”) 27 8.3 Signals 28 8.3.1 Establish (BNC) 28 8.3.2 Release (BNC) 29 8.4 Statistics 29 8.5 Error Codes 29 8.6 Procedures 29 8.6.1 TCP endpoint creation 30 8.6.3 TCP bearer connection establishment 30 8.6.4 TCP application data transfer 31 8.6.5 TCP bearer connection release 32 8.6.6 Path MTU Discovery 33 8.6.7 Interaction of TCP-retransmissions with traffic control functions 33 9 TCP extended connection control package 33 9.1 Properties 34 9.1.1 Stream Endpoint Pair Interlinkage 34 9.1.2 Oneway Release Indicator 35 9.2 Events 36 9.3 Signals 36 9.3 Statistics 36 9.5 Error codes 36 9.6 Procedures 36 10 TCP retransmission metrics package 38 10.1 Properties 38 10.2 Events 38 10.3 Signals 38 10.4 Statistics 38 10.4.1 Retransmitted TCP Octets Sent 38 10.5 Error codes 39 10.6 Procedures 39 11 Package-less TCP control 40 12 Security considerations 43 Annex A State modelling for TCP bearer connection endpoints 44 A.1 Introduction and purpose 44 A.2 Original state model for TCP bearer connection endpoints 44 A.3 Simplified state model for H.248-based TCP basic connection control 45 Appendix I Sample use-cases of TCP connection control 47 I.1 Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG 47 Appendix II Example call flows 49 II.1 Assignment of TCP mode of operation 49 II.1.1 Mode type: application agnostic TCP-proxy 49 II.2 Basic TCP bearer connection control 49 II.2.1 Network configuration 49 II.2.2 Establishment 50 II.2.3 Release 52 II.3 Extended TCP bearer connection control 53 II.3.1 Establishment 53 II.3.2 Release 54 Appendix III Illustration of protocol semantics of the TCP basic connection control package 55 III.1 Overview 55 III.2 Conventions 55 III.3 Establishment of TCP bearer connections 56 III.3.1 Successful establishment – Terminating side 56 III.3.2 Successful establishment – Originating side 56 III.3.3 Unsuccessful establishment 57 III.4 Release of TCP bearer connections 58 III.4.1 Successful release – Terminating side 58 III.4.2 Successful release – Originating side 59 III.4.3 Unsuccessful release 59 Appendix IV Illustration of protocol semantics of the TCP extended connection control package 60 IV.1 Overview 60 IV.2 Conventions 60 IV.3 Usage of SEPP Interlinkage property 60 IV.3.1 Property „sepplink = FALSE“ 60 IV.3.2 Property „sepplink = TRUE“ 62 IV.4 Usage of Oneway Release Indicator property 63 IV.4.1 Property „ori = FALSE“ 63 IV.4.2 Property „ori = TRUE“ 64 IV.5 Combined consideration of both properties 65 IV.5.1 Property settings: “ori = FALSE“ and “sepplink = TRUE“ 65 IV.5.2 Property settings: “ori = TRUE“ and “sepplink = TRUE“ 66 IV.5.3 Signalling examples 67 Bibliography 68

List of Tables

Page

Table 1 – Principal TCP-based interfaces of H.248 entities and their relevance for this Recommendation 10 Table 2 – Use cases and TCP connection control aspects 16 Table 3 – Example IWFs 18 Table 4 – Q.1950-defined H.248 packages – gb package 20 Table 5 – Q.1950-defined H.248 packages – bcp package 20 Table 6 – Q.1950-defined H.248 packages – bnct package 21 Table 7 – Q.1950-defined H.248 packages – ri package 22 Table 8 – H.248.67-defined H.248 package – bnct package 22 Table 9 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers 23 Table 10 – H.248.84-defined H.248 package – tcphp package 23 Table 11 – H.248.84-defined H.248 package – tcptv package 24 Table 12 – H.248.84-defined H.248 package – tcpccm package 24 Table 13 – H.248.84-defined H.248 package – tcpcqm package 24 Table 14 – H.248.84– Inventory of TCP functions 25 Table 15 – Bearer connection establishment preparedness (of TCP endpoints) 26 Table 16 – Semantic of parameter type 28 Table 17 – TCP endpoint role assignment (at establishment phase) 30 Table 18 – TCP bearer establishment - Results 30 Table 19 – Impact of StreamMode on TCP bearer traffic at external MG interface 31 Table 20 – MG bearer plane procedures (outgoing side) 32 Table 21 – MG bearer plane procedures (incoming side) 33 Table 22 – Semantic of property sepplink 34 Table 23 – Semantic of property ori 36

List of Figures

Page

Figure 1 – Overview: Methods for H.248-based TCP connection control 10 Figure 2 – Conventions for TCP bearer control 14 Figure 3 – Bearer connection network use cases with TCP stream endpoints 15 Figure 4 – Two-termination context with scope on a single TCP termination only 16 Figure 5 – Two-termination context with context-level TCP scope 17 Figure 6 – Two-termination context with context-internal interworking function 17 Figure 7 – Overview of TCP related packages 19 Figure A.1 – [IETF RFC 793] copy: TCP connection state diagram 45 Figure A.2 – Simplified state model for H.248-based TCP basic connection control 46 Figure A.3 – Extended state model for H.248-based TCP basic connection control 47 Figure I.1 – Use-case #1: WebRTC to IMS interworking function 48 Figure II.1 – Configuration “application agnostic TCP-proxy” 49 Figure II.2 – Traffic flow example for an MGC strictly controlled TCP bearer connection establishment using the TCP-proxy mode 51 Figure II.3 – Traffic flow example for an incoming TCP bearer connection release, explicitly propagated by the MGC in outgoing direction 52 Figure II.4 – Traffic flow example for outoing TCP bearer connection release procedures towards both remote TCP endpoints X and Y 53 Figure II.5 – Traffic flow example for an extended TCP bearer connection establishment 54 Figure II.6 – Traffic flow example for an extended TCP bearer connection release 55 Figure III.1 – Successful establishment – Terminating side 56 Figure III.2 – Successful establishment – Originating side 57 Figure III.4 – Successful release – Terminating side 58 Figure III.5 – Successful release – Originating side 59 Figure IV.1 – Property „sepplink = FALSE“ 61 Figure IV.2 – Signalling example for TCP bearer connection establishment – Property „sepplink = FALSE“ 61 Figure IV.3 – Property „sepplink = TRUE“ 62 Figure IV.4 – Signalling example for TCP bearer connection establishment – Property „sepplink = TRUE“ 63 Figure IV.5 – Property „ori = FALSE“ 64 Figure IV.6 – Property „ori = TRUE“ 65 Figure IV.7 – Property settings: “ori = FALSE“ and “sepplink = TRUE“ 66 Figure IV.8 – Property settings: “ori = TRUE“ and “sepplink = TRUE“ 67 Figure IV.9 – Signalling example for TCP bearer connection release and example value combinations for both properties 68

Draft new Recommendation ITU-T H.248.xx (ex H.248.TCP)

Gateway Control Protocol:
TCP support packages

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

{Editor’s note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

In scope of this Recommendation are TCP-based bearer interfaces of H.248 IP media gateways. Such an H.248 MG could provide various functions for processing of TCP/IP packets, TCP payload data and involvement in functions related to TCP protocol control information (based on TCP header flags). This Recommendation focuses primarily on the aspect of TCP connection control, which comprises

- establishment,

- modification (tbd) and

- release

of TCP bearer connections.

The Recommendation considers

- connection models with a single or multiple TCP bearer connection endpoints within an H.248 context;

- establishment, through-connection and release behaviour in case of end-to-end TCP bearer connections across two H.248 TCP terminations (“a so-called H.248 TCP media stream”);

- different H.248 control models

further, this Recommendation provides information

- about existing H.248 tools with respect to other  TCP support functions.

## 1.1	Applicability statements

Table 1 summarizes all possible TCP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TCP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TCP-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TCP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

## 1.2	Package-less TCP connection control

Figure 1 summarizes the methods for H.248-based TCP connection control (at the MG bearer interface). The general approach would be based on package-defined H.248 properties by this Recommendation, - besides SDP elements. However, in simple and limited network environments might be already the package-less option sufficient.

Figure 1 – Overview: Methods for H.248-based TCP connection control

{Edittor’s note: a mixed model of SDP-based and Property-based control might be also an option. To be cross checked again when the package designs get stable ...}

{Edittor’s note (2013-01): SDP-based … in scope   dedicated clause 11 inserted ...}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Ses.sion Description Protocol (SDP).

[IETF RFC 4976]	IETF RFC 4976 (2007), Relay Extensions for the Message Session Relay Protocol (MSRP).

[IETF RFC 6349]	IETF RFC 6349 (08/2011), Framework for TCP Throughput Testing.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.43]	Recommendation ITU-T H.248.43 (06/2008), Gateway control protocol: Packages for gate management and gate control.

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (02/2012), Gateway control protocol: Guidelines for packet-based streams.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services

[ITU-T Q.1950]	Recommendation ITU-T Q.1950 (12/2002), Bearer independent call bearer control protocol.

[ITU-T Q.Sup31]	Technical Report TRQ.2141.0 (12/2000): Signalling requirements for the support of narrow-band services over broadband transport technologies - Capability set 2 (CS-2).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	TCP packet [ITU-T H.248.84]: IP datagram (also known as IP packet) carrying a (single) TCP segment in the payload.

NOTE – Such a L4/L3 PDU is also known as TCP/IP packet (briefly TCP packet).

3.1.2	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): statefull forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a statefull TCP connection state machine.

NOTE 1 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

{Edittor’s note (2013-01): it is yet undecided whether terms 3.2.1, 3.2.2 and 3.2.3 remain in H.248.TCP due to the fact that they are more“H.248 generic”, may be thus rather subject of H.248.1. Examples: applicable as well for H.248.78, H.248.TLS, H.248 “T.38 control” (= T.38 Annex E.).}

3.2.1	Basic H.248 master-slave control mode: An H.248 decomposed gateway following an hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship. With MGC as master and MG as slave entity due to considered distribution models on the decomposed gateway entities

NOTE 1: [ITU-T Q.Sup31] Annexes C.2 or C.3 provides examples of functional-to-physical mapping models according basic H.248 master-slave control mode.

3.2.2	MG autonomous mode: For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited on decisions with MG available information only.

3.2.3	MGC-strictly controlled mode: Synonym to the basic H.248 master-slave control mode. All control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit.

3.2.4	TCP proxy variants: the basic TCP proxy mode is defined in clause 3.1.2. The following TCP proxy variants are useful for the various TCP services as supported by H.248 MGs:

There are two orthogonal attributes:

- Discrimination with respect to TCP payload data awareness:

- Application-aware TCP proxy (the MG knows the carried protocol (stack) by TCP packets)

- Application-agnostic TCP proxy (the MG is unaware of the TCP payload content)

- Discrimination with respect to statefull TCP connection control:

- Statefull TCP proxy (by support of the complete TCP connection state machine, see Figure 6 in [IETF RFC 793])

- Lightweight TCP proxy (by support of simplified TCP connection state models)

Example: “application-agnostic, statefull TCP proxy” mode.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2BTE | Back-to-Back TCP Endpoint |
| --- | --- |
| BCP | Bearer Control Protocol |
| BGF | Border Gateway Function |
| BICC | Bearer Independent Call Control |
| BNC | Backbone Network Connection |
| DTLS | Datagram Transport Layer Security |
| FFS | For further studies |
| FTP | File Transfer Protocol |
| GCP | Gateway Control Protocol |
| HTTP | Hypertext Transfer Protocol |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| MPTCP | Multipath TCP |
| MSRP | Message Session Relay Protocol |
| MTU | Maximum Transmission Unit |
| NAT | Network address translation |
| NAT-T | NAT traversal |
| NGN | Next generation network |
| PMTUD | Path MTU discovery |
| RD | Remote descriptor |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | SEP Pair |
| SIP | Session initiation protocol |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| UDP | User datagram protocol |
| UE | User equipment |
| W3C | World Wide Web Consortium |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

## 5.1	Naming of stream endpoints and terminations

The notations “Tx” and “Tx(Sy)” are used for naming of terminations and stream endpoints (SEP) respectively, with

- “Tx” as TerminationID value;

- “Sy” as StreamID value and

- numerical variables x an y.

## 5.2	TCP bearer

The notation of TCP bearer connection is synonym to a TCP connection in the network user plane. There might be also a control plane TCP connection (between MGC and MG) in case of a TCP-based H.248 transport mode [ITU-T H.248.67].

NOTE 1 – This relates to the so called backbone network connection (BNC) concept in [ITU-T Q.1950], which is referred by clause 8.

## 5.3	TCP bearer control

There are connection control information flows for TCP bearer connections in the network user plane (e.g., IP media/data/bearer plane) and the control plane (here H.248). Figure 2 outlines the used conventions for the differentiation of both control flows:

Figure 2 – Conventions for TCP bearer control

# 6	Motivation use cases and models

## 6.1	Bearer connection network use cases

There are three fundamental use cases from perspective of H.248 MGs and the TCP bearer connection control (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection; there are two interconnected H.248 stream endpoints (SEP), - a TCP stream endpoint pair (TCP SEPP) -, within the H.248 context; two sub-cases may be distinguished from an overall network perspective:

  - Use case #1.1.1: there is a single, continuous end-to-end TCP connection, any interim H.248 IP-IP MG is not fully terminating the TCP protocol; this use case relates to a TCP relay or merge mode behaviour of the MG (see [ITU-T H.248.84]);

  - Use case #1.1.2: the end-to-end TCP connection is divided in two separate TCP connection segments, the H.248 IP-IP MG provides complete TCP protocol terminations towards the remote TCP bearer connection endpoints; this use case relates to the TCP proxy mode behaviour of the MG;

- Use case #1.2: a TCP to non-TCP interworking scenario; the H.248 MG is acting as a single TCP endpoint.

Further, but less focus:

- Use case #2: multipath TCP (MPTCP) and H.248 MG involvement.

- Use case #3: multiple TCP bearer connection endpoint models such as applied by media servers (e.g., an [ITU-T H.248.69] based MSRP switch).

Figure 3 – Bearer connection network use cases with TCP stream endpoints

The TCP connection control aspect is a primary scope of this Recommendation, some general conclusions could be derived, see Table 2:

Table 2 – Use cases and TCP connection control aspects

| Use case: | TCP connection control aspects: |
| --- | --- |
| 1.1.1 | TCP connection establishment (and release) is true end-to-end, meaning that the TCP connection control procedures at the two H.248 SEPs are tightly coupled. E.g., an incoming TCP connection establishment request (i.e. a TCP SYN packet) from remote endpoint X would lead to an immediate forwarding of that “bearer connection control request” towards remote endpoint Y. This relates to a TCP relay and merge mode scenario. |
| 1.1.2 | TCP connection establishment (and release) is decoupled from end-to-end perspective. This looks like a back-to-back TCP endpoint as given by TCP proxy mode. The TCP bearer connection control procedures are independent for each SEP. |
| 1.2 | The H.248 MG provides an IP host function for a single TCP connection endpoint. |

## 6.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a connection model with up to two ITU-T H.248 terminations (note: there might be one or multiple stream endpoint pairs (SEPP) in case of two terminations). In addition, no assumptions are made regarding either the lower layer protocols beneath the TCP layer or the upper layer protocols being carried by the TCP payload. This allows the use of the Recommendation's procedures in various connection models and use cases such as illustrated in the sub-clauses.

### 6.2.1	Model for single TCP termination scope only

Figure 4 details the generic connection-model where a TCP-enabled termination is connected to a single other termination.

Figure 4 – Two-termination context with scope on a single TCP termination only

This model relates to use case #1.2.

### 6.2.2	Model for context-level TCP termination scope

Figure 5 represents the general model for use case category #1.1. The type of interworking function (IWF) determines the particular MG behaviour concerning TCP handling (see next sub-clause 6.2.3).

Figure 5 – Two-termination context with context-level TCP scope

### 6.2.3	Model with focus on context-internal interworking function

Figure 6 depicts a two termination model with scope on example interworking functions.

Figure 6 – Two-termination context with context-internal interworking function

The IWF is principally a context-level function, associated to a particular SEPP (here: SEPs T1(S1) and T2(S1)). Table 3 lists some example IWFs, which are relevant in scope of this Recommendation and TCP processing, classified from a layered protocol perspective:

Table 3 – Example IWFs

| IWF: | Example functions: | Comments from TCP perspective: |
| --- | --- | --- |
| L3 IWF | - Back-to-back IP host (B2BIH) | Conditional useful for all kind of TCP interworking (NOTE 1). |
|  | - B2BIH with: - Network address translation (NAT) - Network prefix translation (NPT in case of IPv6) |  |
|  | - IP protocol version interworking (IPv4-to-IPv6 or vice versa) |  |
|  | - NAT-T according [ITU-T H.248.37] | No impact (NOTE 2). |
|  | - IP router (IPR) [ITU-T H.248.64] - IPR with NAT | Conditional for all kind of TCP interworking: TCP relay and TCP merge mode would be inline with IPR mode, whereas the TCP proxy mode would be rather B2BIH (NOTE 4). |
| L4 IWF | - this Recommendation | There are two crucial aspects to be noted: - Cut-through characteristic (“when the two SEPs are interconnected for enabled traffic flow”); - BCP (for TCP) information of one SEP as possible stimuli for procedures at partner SEP, or not? (NOTE 3) |
|  | - NAT-T according [ITU-T H.248.84] | Impact, see clause 7.4. |
| L4+ IWF | - MSRP relay [ITU-T H.248.69] | For MSRP-over-TCP, bearer type indication is not explicitly specified, but supposed to be part of SDP “m=” line (see clause 6.2/[ITU-T H.248.69]). For MSRP-over-TLS: unspecified, indicated as FFS by [ITU-T H.248.69]. |
|  | - MSRP relay [IETF RFC 4976] | MSRP relays facilitate NAT-T. The MSRP relay function could be provided by an H.248 gateway, e.g. using a similar mapping model as [ITU-T H.248.69]. However, [ITU-T H.248.69] does not (yet) support a “MGC controlled MSRP relay”. |
|  | - Bearer-level ALG [ITU-T H.248.78] | Interactions to be considered. |
|  | - Transport level security [ITU-T H.248.TLS] | There are impacts, dependent on TLS-to-TLS or TLS-to-non-TLS type of interworking. |
| NOTE 1 – Exception: attention to ICMP (see clause 6.7/[ITU-T H.248.79], such as TCP import ICMP information with end-to-end significance. NOTE 2 – This is rather a SEP related function than SEPP scope. NOTE 3 – Example: an incoming TCP connection release triggers an outgoing TCP connection release, or not. NOTE 4 – The “full protocol termination” as part of TCP proxy mode implies two separate, addressable IP transport endpoints, which are only subject of B2BIH mode. |  |  |

{Editor’s note (2013-06 meeting): clause 6.2.3 may be revisited again …}}

# 7	Relation to similar H.248 packages

## 7.1	Overview of TCP related packages

There are some packages from the past which are either defined for TCP or could be theoretically reused for TCP, see Figure 7.

Figure 7 – Overview of TCP related packages

The specific relation to this Recommendation is described below.

## 7.2	ITU-T Q.1950

This Recommendation introduces 11 H.248 packages for usage in so-called Bearer-Independent Call Control type of network architectures. Four out of the eleven could be principally applied (with extensions) for TCP networks:

{Editor’s note (2013-01): whether the bnct and ri package (7.2.3 and 7.2.4) should be listed or not is still under discussion. because there couldn’t yet any strong requirements in that direction for TCP identified.}

### 7.2.1	Package ‘gb’ (generic bearer connection):

TCP is a connection-oriented protocol which implies a connection state model (see clause  in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table 4):

Table 4 – Q.1950-defined H.248 packages – gb package

| Package name: | Purpose: |
| --- | --- |
| Generic bearer connection package | The gb package provides control elements for such statefull bearer technologies, which implies basic stimuli for the bearer path coupled bearer control signalling protocol as well as a basic connection state concept. The bearer connection endpoint models supports also incoming and outgoing bearer control protocol procedures, which is e.g. required for the modelling of active/passive open (or close) procedures or client/server behaviour. {Editor’s note (2013-01): unclear is the usage of “cut-through” values?} |
| Possible relation to this Recommendation: |  |
| The TCP connection state model could be basically mapped on the underlying gb package state model. The gb package could be used for the majority of TCP connection control procedures. Open/differences: - TCP explicitly supports simultaneous bearer connection establishment and release, which would be so called collision scenarios in case of the gb package. - TCP bearer connection release: unidirectional concept (called half close), whereas the gb package considers bidirectionality only. - others? |  |

### 7.2.2	Package ‘bcp’ (bearer characteristics):

The notion of bearer characteristics relates to a particular (or multiple) protocol layer(s) in the bearer plane (see Table 5):

Table 5 – Q.1950-defined H.248 packages – bcp package

| Package name: | Purpose: |
| --- | --- |
| Bearer characteristics package | The existing bcp package supports an initial set of bearer technologies such as RTP/IP for IP-based networks. However, bearer type “TCP” is not supported. {Editor’s note (2013-01): assumption that the unassigned codepoints are reserved for additional bearer types?} |
| Possible relation to this Recommendation: |  |
| Usage of the gb package would imply the need for an explicit bearer type indication such as supported by the bcp package. Open/differences: - A new version or an extension package with additional codepoints would be required (at least bearer type “TCP”, but possibly also “TLS”, “TLS/TCP”, “DTLS”, “SCTP”, “DCCP”). - Instead of the Property-based bearer type indication (via bcp package) at LCD level exists the alternative of the SDP-based bearer type indication (via “m=” line element proto) at LD/RD level. - Use case TLS-over-TCP: the establishment/release of the TCP connection and the TLS session could be basically controlled via the the gb package. The bearer connection control procedures at TCP and TLS level could be basically temporarily decoupled, which would imply the modification of the bcp/BNCChar property value, a scenario originally out of scope of the bcp package. |  |

### 7.2.3	Package ‘bnct’ (bearer network connection cut through):

See Table 6:

Table 6 – Q.1950-defined H.248 packages – bnct package

| Package name: | Purpose: |
| --- | --- |
| Bearer network connection cut through package | The MG can indicate whether the cut through will occur "early" or "late", which are effectively related to bearer control protocol signalling events. Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. |
| Possible relation to this Recommendation: |  |
| The aspect of “cut-through” control is basically relevant for TCP as well, perhaps even more important due to a) the assured transport characteristic of TCP (e.g., handling of acknowledgments, loss, retransmission) and H.248 MG TCP modes (such as relay, merge or proxy). Open/differences: - The explicit cut-through semantic might be also an implicit capability of TCP modes (TBC). - Unclear is the interaction with StreamMode property settings? - others? |  |

### 7.2.4	Package ‘ri’ (reuse idle):

TCP is a connection-oriented protocol which implies a connection state model (see Figure 6 in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table 4):

Table 7 – Q.1950-defined H.248 packages – ri package

| Package name: | Purpose: |
| --- | --- |
| Reuse idle package | The ri/RII property is used by the MG to indicate to the MGC that an idle bearer is to be reused. {Editor’s note (2013-01): the semantics are not really clear: - What’s the supposed model of a “bearer”, - end-to-end or MG local scope of a bearer connection endpoint only? I.e., a question related to network level or MG local resource management. - Notion of “idle” implies a bearer connection state model, which? What’s the semantic of state “idle”?} |
| Possible relation to this Recommendation: |  |
| Reuse of existing TCP connections is a supported concept, such as in SIP networks by [IETF RFC 4145.] See also clause 13.6/[ITU-T H.248.84]. Open/differences: - Possible models of MG-level TCP resources, e.g.: - resource component “TCP local source/destination transport address”; - resource component “TCP local statefull bearer connection endpoint” which would imply the TCP connection state machine; - resource component “end-to-end TCP bearer connection”, based on the n-tuple of local and remote transport source/destination addresses and connection state; - others? - The semantic of reuse idle bearer would be then dependent on the agreed TCP resource model. It might be even transparent for the MG! |  |

## 7.3	ITU-T H.248.67

### 7.3.1	Package ‘trm’ (GCP transport mode indication):

See Table 8:

Table 8 – H.248.67-defined H.248 package – bnct package

| Package name: | Purpose: |
| --- | --- |
| GCP transport mode indication package | Related to the H.248 Control Association, which may use TCP-based transport modes. |
| Possible relation to this Recommendation: |  |
| None: [ITU-T H.248.67] and this Recommendation are both diametrically opposed. TCP usage for the H.248 signalling interface is out of scope of this Recommendation. |  |

## 7.4	ITU-T H.248.84

The TCP enabled SEP will be activated through the indication of the TCP-proxy mode as defined in [ITU-T H.248.84]. This Recommendation only defines properties that are needed on top of [ITU-T H.248.84] for the control of a TCP enabled SEP.

This Recommendation does not define TCP-related statistics and events which are already covered by [ITU-T H.248.84]. Whenever needed those statistics and events of the related packet as defined by [ITU-T H.248.84] must be used.

### 7.4.1	Package-independent procedures for NAT-T with TCP bearers:

See Table 9:

Table 9 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers

| Procedures: | Purpose: |
| --- | --- |
| Clause 13/[ITU-T H.248.84] | a) Indication of IP transport protocol 'TCP' b) Indication of 'TCP mode' for H.248 MG |
| Possible relation to this Recommendation: |  |
| To a): - Normative for SDP-based bearer type indication “TCP”. To b): - The TCP merge mode could be used as NAT traversal service. The TCP modes as such could interact with TCP connection control. E.g., the tcpcc package would be not required (and should be not used) for TCP connection establishment when NAT-T is used (because it provides already end-to-end TCP connection establishment (Note: TCP connection release is different). |  |

### 7.4.2	Package ‘tcphp’ (TCP hole punching):

See Table 10:

Table 10 – H.248.84-defined H.248 package – tcphp package

| Package name: | Purpose: |
| --- | --- |
| TCP hole punching package | NAT traversal support service for TCP. |
| Possible relation to this Recommendation: |  |
| Orthogonal to this Recommendation; the tcphp package could be basically used in conjunction with this Recommendation. |  |

### 7.4.3	Package ‘tcptv’ (TCP traffic volume metrics):

See Table 11:

Table 11 – H.248.84-defined H.248 package – tcptv package

| Package name: | Purpose: |
| --- | --- |
| TCP traffic volume metrics package | Support of octet and packet count statistics for TCP bearer protocol. |
| Possible relation to this Recommendation: |  |
| Performance monitoring is an orthogonal application; hence, the H.248 statistics could be basically used in conjunction with this Recommendation. |  |

### 7.4.4	Package ‘tcpccm’ (TCP connection control metrics):

See Table 12:

Table 12 – H.248.84-defined H.248 package – tcpccm package

| Package name: | Purpose: |
| --- | --- |
| TCP connection control metrics package | Statistics related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation: |  |
| Basically all three statistics could be used. |  |

### 7.4.5	Package ‘tcpcqm’ (TCP connection quality metrics):

See Table 13:

Table 13 – H.248.84-defined H.248 package – tcpcqm package

| Package name: | Purpose: |
| --- | --- |
| TCP connection quality metrics package | Additional statistic related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation: |  |
| The round-trip time based statistic may be used as well. |  |

### 7.4.6	Inventory of TCP functions and their possible impact on H.248 TCP gateways:

See Table 14:

Table 14 – H.248.84– Inventory of TCP functions

| Inventory in: | Purpose: |
| --- | --- |
| Appendix II/[ITU-T H.248.84] | TCP Functions versus H.248 TCP Modes of Operation, in ten areas: - Basic Capabilities - Topology Hiding - Protocol Encryption - Security - Application Data Inactivity Detection - Interactions with other policy rules - Performance measurements & statistics" - Connection keep alive support - Add-on's to TCP - IP layer operations |
| Possible relation to this Recommendation: |  |
| This informative analysis is principally valid as well. |  |

## 7.5	ITU-T H.248.43 and ITU-T H.248.79

Related to filtering of TCP traffic. See overview in clause 9/[ITU-T H.248.79].

# 8	TCP basic connection control package

|  | Package Name: | TCP basic connection control package |
| --- | --- | --- |
|  | Package ID: | tcpbcc (0x####) |
|  | Description: | This package provides the functionality to be able to establish and /release a TCP bearer connection. NOTE – The package design follows conceptually the generic bearer connection (gb) package according clause A.6/[ITU-T Q.1950] by using an event/signal-based approach for connection establishment/release control. Exceptions: a) bearer modification is not required for TCP; b) bearer release: using a package-defined codepoint instead of element g/cause. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

## 8.1.1	Bearer connection establishment blocking

|  | Property Name: | Bearer connection establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) |
|  | Description: | This property defines the MG behaviour before and during TCP bearer connection establishment. NOTE 1: See Annex A concerning a state model based illustration. NOTE 2: This capability might be relevant for the prevention of other TCP processing behaviour such as “TCP relay” or “TCP merge” modes (TBC). |
|  | Type: | Enumeration |
|  | Possible values: | See Table 15 Table 15 – Bearer connection establishment preparedness (of TCP endpoints) Value; MG behaviour:. “blocked”, (0x0001); - Bearer resources are prepared (i.e., resources for TCP endpoint are reserved and allocated to H.248 termination/SEP); - TCP role assignment: still open, not yet decided; - TCP connection state = “CLOSED” (Figure A.1); - H.248 counterpart state = “Blocked” (Figure A.3); - any incoming TCP packet shall be silently discarded (e.g., TCP-SYN requests shall be not acknowledged), not any state transitioning from “CLOSED” (NOTE 1).. “unblocked”, (0x0002); - MG shall be ready for incoming or/and outgoing TCP bearer connection control procedures; - TCP role assignment: relates to an (initial) TCP server role effectively (“TCP Passive Open”) (NOTE 2); - TCP connection state: transitioning from “CLOSED” to “LISTEN” (Figure A.1); - H.248 counterpart state = “Idle” (Figure A.3); TCP bearer connection control options: - Incoming TCP-SYN: TCP server role confirmed, 3-way handshake proceeded; - Signal …: start 3-way handshake by sending of TCP-SYN; TCP client role assigned. NOTE 1: This semantic is similar (identical?) to SDP codepoint “a=setup:holdconn”, see [IETF RFC 4145]. NOTE 2: This semantic could be correlated with SDP codepoint “a=setup:actpass”, see [IETF RFC 4145].. |
|  | Default: | ”Unblocked” (due to be consistent with default behaviour as in [ITU-T H.248.84]). NOTE: H.248 profile specifications could redefine other default values. Examples: H.248 profiles for - MGs with support of “TCP proxy” mode only may choose default value “blocked” due to … (TBC); - MGs with additional support of “TCP relay” and “TCP merge” modes may choose default value “unblocked” due to (TBC) |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Editor’s note (2013-06 meeting): following items were identified and need to be resolved:

C1: the property semantics are focusing on the ingress direction (tbc); i.e., when property not relevant in outgoing direction, then explicit text should be added.

C2: Signal EstBNC should implicitly overwrite the property value, i.e., replacing blocked by unblocked. Correspondent updates should be added.

NOTE – The semantics of this property needs to be considered from use cases with combined tcpbcc and tcpecc usage. (E.g., scenario Appendix IV.3.2 where SEP T2(S1) would be in “blocked” state, etc.)

}

## 8.2	Events

### 8.2.1	TCP connection state change (“BNC change”)

|  | Event Name: | TCP connection state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TCP) bearer network connection occurs. For example a bearer has been established or a bearer has been released. The event is related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A. NOTE – The event design is aligned with event gb/BNCChange according clause A.6.2.1/[ITU-T Q.1950] by using an event/signal-based approach for connection establishment/release control. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. |
|  | Type: | Sublist of Enumeration {Contributor’s note: in order to serve scenario Events=1{tcpbcc/BNCChange{Type=[Est,Rel]}} |
|  | Optional: | No |
|  | Possible values: | Table 16 defines the protocol semantic per value: Table 16 – Semantic of parameter type Value; Name; Final state (Annex A.2); Semantic. Est, [0x01]; Bearer Established; Established; notify MGC successful TCP bearer establishment.. Rel, [0x05]; Bearer Released; ### Idle; notify MGC successful TCP bearer release.. NOTE 3 – The numerical codepoint assignments are aligned with /[ITU-T Q.1950].. {Editor’s note (2013-06 meeting): following items were identified and need to be resolved: C1: the semantics should be more precisely tight to the TCP and H.248 state model (Annex A), i.e., relation to 3-way and 4-way handshakes for establishment and release. } |
|  | Default: | None |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TCP bearer connection. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est, [0x01] Bearer Established Rel, [0x05] Bearer Released |
|  | Default: | None |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).}

## 8.3	Signals

The signals are related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A.

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to start bearer establishment (i.e., this signal is used to establish the TCP bearer connection: the MG takes the TCP client role and sends a TCP SYN segment). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TCP connection: the MG sends a TCP FIN segment). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination. Any support of TCP connection reuse would be excluded.

Other options?

None!?

Conclusion: a packageless semantic could be defined.}

## 8.4	Statistics

None.

## 8.5	Error Codes

None

## 8.6	Procedures

{Editor’s note (2013-01): description of procedures needs to be updated …}

### 8.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection enabled SEP in the MG. This will be indicated on the H.248 interface through the use of the SDP “m=” line according the rules as specified in clause 13.4 in [ITU-T H.248.84].

The MG shall reserve “TCP bearer resources” (such as a “TCP connection state machine”) and allocates them to the SEP. The TCP role (client/server) is not (yet) determined by the SDP “m=” line information.

NOTE 1 – This functionality is synonym to the “bearer type indication” semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TCP endpoint role assignment

The TCP role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). Table 17 summarizes MGC actions and their results in TCP role assignments:

Table 17 – TCP endpoint role assignment (at establishment phase)

|  | TCP role | H.248 indication | MG behaviour: |
| --- | --- | --- | --- |
| 1 | not yet assigned | H.248 state “Blocked” of SEP, either due to explicit signalling of bceb property or correspondent default value | Any incoming TCP packet shall be silently discarded. TBD |
| 2 | TCP server | H.248 state “Idle” by “unblocking” the SEP | Expect incoming TCP SYN, subsequent completion of 3-way handshake. |
| 2 | TCP client | H.248 state “Idle” of SEP plus signal EstBNC enabled. | Start of 3-way handshake for TCP connection establishment. |
| 4 | both | Not possible in TCP. (TBC) |  |

### 8.6.3	TCP bearer connection establishment

The incoming and outgoing side of TCP bearer connection protocol procedures is given by the assigned TCP role (see above clause 8.6.2) by the MGC.

The MG shall initiate the TCP connection setup according to the indicated role and the TCP connection setup handshake is performed by the MG without any further involvement of the MGC. The end of the TCP bearer connection establishment phase may be summarized according Table 18:

Table 18 – TCP bearer establishment - Results

| TCP role | Result of TCP bearer connection establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tcpbcc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent error code or event g/cause. |
| Server (i.e., incoming establishment side) | Successful | As client side. |
|  | Unsuccessful | As client side. |

### 8.6.4	TCP application data transfer

#### 8.6.4.1	MG externally

The TCP bearer connection is ready for application data transfer when remote and local TCP endpoints are both transitioned to TCP connection state “ESTAB”.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TCP control information used to establish or close the TCP connection. The requested MG behaviour is defined by Table 19:

{Editor’s note (2012-09 meeting): Consensus required on this behaviour.
Comment (2013-06 meeting): the principle problem relates to the fact that TCP allows to piggyback control and data information, i.e., an individual TCP/IP packet may not always be classified solely as part of the TCP data flow (-> H.248 media flow) or TCP control flow (-> H.248 control flow).
Reminder: H.248.1/7.1.7.1.1 StreamMode affects only H.248 media flow, but not H.248 control flow traffic.}

Table 19 – Impact of StreamMode on TCP bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | all TCP packets sent |
|  | Sending disabled (i.e., values RecvOnly, Inactive) | only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are sent, independent of optional piggybacked data. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | all TCP packets accepted |
|  | Receiving disabled (i.e., values SendOnly, Inactive) | only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are accepted, independent of optional piggybacked data. |

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

#### 8.6.4.2	MG internally

This clause is only relevant for H.248 contexts with TCP SEPP(s), as outlined in clause 6.2.3.

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. . If application independent or unambiguous behaviour would be required, then the SEPP interlinkage property of the tcpecc package (see clause X.1.A.B) should be used.

### 8.6.5	TCP bearer connection release

#### 8.6.5.1	Characteristic of TCP bearer connection release

This package supports bidirectional TCP bearer connection release only. Consequently, two TCP half-close procedures shall be performed, resulting in a four-way handshake. The transitioning from H.248 state Established to Idle of the local TCP bearer connection endpoint is only done after the successful processing of both TCP half-close procedures.

#### 8.6.5.2	Assignment of incoming or outgoing TCP bearer connection release

##### 8.6.5.2.1	Outgoing side

The local TCP bearer connection endpoint shall be explicitly prepared for outgoing TCP bearer connection release via signal RelBNC by the MGC.

##### 8.6.5.2.2	Incoming side

The local TCP bearer connection endpoint shall be able to process incoming TCP bearer connection release requests (TCP-FIN packet) at any point in time.

#### 8.6.5.3	MG bearer plane procedures

##### 8.6.5.3.1	MGC initiated release (outgoing side)

The RelBNC signal is used to initiate Bearer Release signalling (Table 20). The MGC may optionally include a GeneralCause, FailureCause or Reset indication. If a MG receives the reset indication it shall release all resources associated with the bearer.

Table 20 – MG bearer plane procedures (outgoing side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Starts first TCP half-close by sending TCP-FIN. Awaits TCP-FIN-ACK and TCP-FIN of second TCP half-close. Subsequent confirmation by a TCP-FIN-ACK. Transition to connection state H.248 Idle (TCP ‘CLOSED’). | Successful | Optional, via event BNCChange, ObservedEventsDescriptor parameter codepoint “Type=Rel”. The MGC default assumption is a “successfully released bearer connection”. TBD |
|  | Unsuccessful | There are following options: - Via event g/cause (TBD) - Via error code. - Via event adid/ipstop (see [ITU-T H.248.40]) (TBD) |

##### 8.6.5.3.2	Bearer initiated release (incoming side)

The MG is awaiting an incoming TCP-FIN packet as start event for TCP bearer connection release (Table 21)

Table 21 – MG bearer plane procedures (incoming side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Awaits first TCP half-close by detection of incoming TCP-FIN. Subsequent acknowledgment as well as start of second TCP half-close procedures. Transition to connection state H.248 Idle (TCP ‘CLOSED’) after reception of TCP FIN-ACK. | Successful | see Table 20 |
|  | Unsuccessful | see Table 20 |

### 8.6.6	Path MTU Discovery

Path MTU discovery (PMTUD) is normally started after successful establishment of TCP bearer connections, see [IETF RFC 1191] (TCP-over-IPv4), [IETF RFC 1981] (TCP-over-IPv6) and [IETF RFC 2923] (TCP specific problems with PMTUD). It is expected that bearer plane PMTUD procedures are completely hidden to the MGC.

PMTUD procedures may result in error conditions, such as received ICMP errors from remote side. Whether such situations should lead to error notifications of the MGC by the MG is for further studies.

### 8.6.7	Interaction of TCP-retransmissions with traffic control functions

Retransmissions of unacknowledged TCP segment leads to an inherent increase of the consumed transport capacity. There might be therefore possible side effects and interactions with traffic control functions, such as traffic policing or traffic shaping.

#### 8.6.7.1	Interaction with traffic policing: IP byterate policing according [ITU-T H.248.53]

Such a traffic policer entity is enforced at the ingress side of a TCP SEP. The traffic policer function shall consider all incoming TCP packets, without distinction between first sent or retransmitted TCP data.

# 9	TCP extended connection control package

|  | Package Name: | TCP extended connection control package |
| --- | --- | --- |
|  | Package ID: | tcpecc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TCP bearer connection control behaviour across a Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - unidirectional TCP bearer connection release. |
|  | Version: | 1 |
|  | Extends: | tcpbcc |

## 9.1	Properties

### 9.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0005) |
|  | Description: | The semantical scope of this property relates to a Stream Endpoint Pair (SEPP; see definition in (clause 3.2.11) [ITU-T H.248.1]). This property defines if a detected condition leading to the establishment or closure of the TCP connection will trigger an MG-autonomous (see clause 3.2.2) establishment or release of the bearer connection of the partner SEP as well. Only TCP protocol control information (e.g., typically TCP header flag type of information or sequence/acknowledgement numbers) is part of such conditions. Normal TCP data traffic is not affected by this property. Semantical scope limitations: - only the two phases of TCP bearer connection establishment and release; - only stimuli at incoming direction may trigger autonomous procedures at the partner SEP; - only the first stimuli of each TCP bearer connection control procedure (i.e., first incoming TCP SYN of three-way handshake; first incoming TCP FIN of half-closure) may trigger autonomous procedures at the partner SEP; - SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries) {Editor’s note (2013-06 meeting): following items were identified and need to be resolved: C1: the background of the 4th bullet item should be elaborated in more detail (e.g., why not a ContextAttribute? TerminationState? etc). } |
|  | Type: | sublist of Boolean (for separated support of establishment and release phase) |
|  | Possible values: | Table 22 defines the protocol semantic per value and dependent on connection control phase. See model of Figure 5 concerning the used SEP names. Table 22 – Semantic of property sepplink Value; MG semantic for TCP bearer connection. Establishment; Release. True; a) Incoming: An incoming TCP bearer connection establishment request at SEP T1(S1) shall lead to an autonomous outgoing TCP bearer connection establishment request at the other related SEP T2(S1). b) Outgoing: An MGC-initiated outgoing TCP bearer connection establishment at SEP T1(S1), shall not impact the TCP connection state of SEP T2(S1).; a) Incoming: An incoming TCP bearer connection release request at SEP T1(S1) shall lead to an autonomous outgoing TCP bearer connection release request at the other related SEP T2(S1). b) Outgoing: Any action (e.g. by MGC) in closing the outgoing TCP bearer connection at SEP T1(S1), shall not impact the TCP connection state of SEP T2(S1).. False; The state of TCP bearer connection endpoint SEP T1(S1) shall not impact the state of TCP bearer connection endpoint SEP T2(S1) (and vice versa).. |
|  | Default: | [False, False] (because this relates to the MG behaviour according the base package (tcpbcc), see clause 8). |
|  | Defined in: | LocalControl {Editor’s note (2012-09): Its unclear how this property relates to the other Terminations/Streams in the Context. i.e. if a release is triggered as a result of this property are other terminations affected if they don't have this property set? Are events triggerred on these other terminations. Might be also a Context-level property? For further study. Comments (2013-06): the semantic is a) limited to a SEPP instance only; and b) would trigger event notification(s), if SEPP interlinkage would lead to valid event detection conditions at the paired SEP. Such events are primarily those in scope of the base package. } |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“False” semantic”, because the two SEPs within a SEPP are normally decoupled in H.248. Usually an explicit H.248 signalling request (such as a SUB.req for a SEP or termination) would trigger outgoing bearer control procedures at the MG bearer interface.

Option 2:

“True” semantic”, but justification?

Conclusion: a package-less semantic could be defined, any interaction with H.248.84 needs not to be considered because “H.248.84 NAT-T” is only for TCP connection establishment.
}

### 9.1.2	Oneway Release Indicator

|  | Property Name: | Oneway Release Indicator |
| --- | --- | --- |
|  | Property ID: | ori (0x0006) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | Table 23 defines the protocol semantic per value: Table 23 – Semantic of property ori Value; MG semantic for TCP bearer connection. False; Indicate the duplex closure of the TCP bearer connection by sending a TCP-FIN autonomously.. True; Keep the TCP bearer connection in the half-closed state, i.e. the MG will not generate the TCP-FIN autonomously. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tcpbcc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“True” semantic”, with the justification that the H.248 media stream concept is fundamentally bidirectional. Such a semantic would lead to a single solution for …

Option 2:

“False” semantic” in order to support this protocol specific characteristic of TCP.

Conclusion: a package-less semantic could be defined, but …. The semantic may be delegated to a profile specification (e.g., when all communication services across TCP would be of type bidirectional).
}

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

{Editor’s note (2013-06 meeting): following items were identified and need to be resolved:

C1: a general statement should be provided that the procedures are related to the extension package and are on top of the base package procedures according clause 8.).

}

### 9.6.1	TCP bearer connection establishment

Only relevant for incoming TCP bearer connection establishment requests from MG external bearer interface. Outgoing procedures are not affected.

{Editor’s note (2013-06 meeting): following items were identified (relevant as well for 9.6.2) and need to be resolved:

C1: The trigger is the incoming procedure, but the result is the outgoing procedure at the other side. The property is relevant thus for incoming and aoutgoing procedures, but in a different manner. 
=> 9.6.1 & 9.6.2 needs to be updated

}

#### 9.6.1.1	Stream endpoint pair interlinkage disabled

{Editor’s note (2013-06 meeting): following items were identified and need to be resolved:

C1: property sepplink is of type list, not  a single value, hence all procedural sections need to be updated from value False or True to [X,Y] combinations!).

}

If the property sepplink is set to False, then the incoming TCP bearer connection establishment procedure shall be identical to base package tcpbcc procedures.

NOTE – Appendix IV.3.1 provides signalling examples.

#### 9.6.1.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TCP bearer connection establishment request shall be propagated to the partner SEP, triggering there an outgoing TCP-SYN.

NOTE – Appendix IV.3.2 provides signalling examples.

### 9.6.2	TCP bearer connection release

Only relevant for incoming TCP bearer connection release requests from MG external bearer interface. Outgoing procedures are not affected.

#### 9.6.2.1	Stream endpoint pair interlinkage disabled and Oneway release indicator disabled

If the property sepplink is set to False, then the incoming TCP bearer connection release procedure shall be not propagated to the partner SEP.

If the property ori is set to False, then the incoming TCP bearer connection release procedure shall lead to a bidirectional TCP bearer connection release, hence identical to base package tcpbcc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled and Oneway release indicator disabled

If the property sepplink is set to True, then incoming TCP bearer connection release request shall be propagated to the partner SEP, triggering there an outgoing TCP-FIN.

If the property ori is set to False, then see clause 9.6.2.1.

NOTE – Appendix IV.5.3 provides signalling examples.

#### 9.6.2.3	Stream endpoint pair interlinkage disabled and Oneway release indicator enabled

If the property sepplink is set to False, then see clause 9.6.2.1.

If the property ori is set to True, then the incoming TCP bearer connection release procedure shall lead to a one TCP bearer connection release (so called TCP half closure).

#### 9.6.2.4	Stream endpoint pair interlinkage enabled and Oneway release indicator enabled

If the property sepplink is set to True, then see clause 9.6.2.2.

If the property ori is set to True, then see clause 9.6.2.3.

NOTE – Appendix IV.5.3 provides signalling examples.

# 10	TCP retransmission metrics package

|  | Package Name: | TCP retransmission metrics package |
| --- | --- | --- |
|  | Package ID: | tcprm (0x####) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TCP bearer protocol, related to TCP retransmission traffic. The statistics are applicable for all TCP modes of operation, as long as the MG is aware that the H.248 Stream carries TCP traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

None.

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

### 10.4.1	Retransmitted TCP Octets Sent

|  | Statistic Name: | Retransmitted TCP Octets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcpros (0x0001) |
|  | Description: | This Statistic provides the number of retransmitted octets sent on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of octets (i.e., including TCP header) retransmitted in TCP packets. At the Termination level, it is equal to the sum of the egress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpEStatsPerfOctetsRetrans of the TCP MIB [IETF RFC 4898]. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 10.4.2	Retransmitted TCP Packets Sent

|  | Statistic Name: | Retransmitted TCP Packets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcprps (0x0003) |
|  | Description: | Provides the number of retransmitted packets sent on the Termination or Stream since the statistic has been set. The packets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of TCP packets, inclusive of TCP connection establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the egress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpRetransSegs of the TCP MIB [IETF RFC 4022]. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 10.5	Error codes

None.

## 10.6	Procedures

FIXTHIS.

### 10.6.1	Egress TCP traffic – Statistic "Retransmitted TCP Packets Sent"

Every outgoing, retransmitted TCP packet, sent from a H.248 TCP Stream/Termination, is counted by Statistic tcprm/tcprps.

### 10.6.2	Egress TCP traffic – Statistic "Retransmitted TCP Octets Sent"

The measurement represents the volume of TCP retransmission of all TCP flows of an H.248 Stream, i.e. across all outgoing, retransmitted TCP packets according to Statistic tcprm/tcpros.

### 10.6.3	Derived statistics

The H.248 statistics of this package together with other TCP related statistics (such as from [ITU-T H.248.84] allow the derivation of other TCP performance metric. This clause provides one example.

#### 10.6.3.1	Performance metric “TCP efficiency”

This metric is defined in clause 4.2 in [IETF RFC 6349] according following equation:

TCP Efficiency % =Transmitted Bytes-Retransmitted BytesTransmitted Bytes×100

Metric TCP efficiency for an H.248 stream/termination could be calculated (by the MGC), using the two H.248 statistics TCP Octets Sent (tcptv/tcpos, see clause 10.4.1/[ITU-T H.248.84]) and Retransmitted TCP Octets Sent (tcprm/tcpros).

# 11	Package-less TCP control

{Editor’s note (2013-01 meeting): dedicated chapter on SDP based control required, addressing }

Purpose of this clause is to describe the SDP-based semantics for TCP control, which needs the consideration of

- SDP usage in H.248.69, H.248.84

- how the package-based elements/procedures would be realized via SDP?

- which functionality would be out of scope of SDP control?

- assumption of mixed Property- & SDP-based control

First comments (from C-11):

To requirement of “Bearer type indication”:

- still open (explicit via dedicated property or SDP? or implicit via using one or multiple of the TCP packages)

To requirement of “Originating/Terminating Bearer control protocol procedures”:

Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, - either in LD or RD or both -, represents 
a) an implicit TCP client role assignment and
b) a request for immediate start of TCP connection establishment procedure.
Any possible race condition of an incoming TCP connection establishment request (from remote TCP entity) at the MG bearer interface will be resolved as TCP simultaneous open procedure.

Option 2:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, but consideration of LD and RD scope with following logic:
Condition 1: IF LD only THEN “TCP server” role of stream endpoint/termination; Wait on incoming TCP connection establishment request (from remote TCP entity);
Condition 2: IF RD only THEN “TCP client” role of stream endpoint/termination; proceed as in option 1;
Condition 3: IF LD AND RD THEN EITHER “Hold connection establishment” OR “error case” OR “…”.

To requirement of “Originating/Terminating Bearer control protocol procedures”:

[other package elements]

Contributions are solicited.}

## 11.1	Overview – principal H.248 control steps

{Editor’s note (2013-06 meeting): the text is this clause is still subject of discussion … }

Figure x1 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TCP connections in environments demanding for NAT traversal support:

Figure x1 – Principal H.248 control steps for establishing a native TCP bearer

It is apparent that some steps might be combined (e.g., 1 and 2) and that specific steps might be optional (e.g., 3, 5, 7).

Table 24 provides a preliminary inventory of possible control elements from H.248 perspective. The column “H.248/SDP element” refers to “package-less TCP control”.

Table 24 – H.248 control steps and possible control elements

| H.248 control step: |  | H.248 property: | H.248/SDP element: | Comments: |
| --- | --- | --- | --- | --- |
| Establishment phase: „IP transport connection establishment“ |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | “m=”-line <proto> (clause 13.4 / H.248.84, H.248.69) | SDP may be ambiguous due to “protocol stack” concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property: tcpbcc/bceb | “a=setup:holdconn” (?) |  |
| 4 | Start IP Transport Connection Establishment Procedure („TCP Open“) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to H.248.84 “a=setup:” semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | H.248.37 | “a=setup:” for H.248.84 NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State „EstablishedTCP“) | Event: tcpbcc/BNCChange | — | MGC notification might be optional. |
| 7 | Local L4+ NAT traversal support | H.248.78 | e.g., “a=msrp-cema” (?) | NOTE 2 |
| Communication phase: |  |  |  |  |
| 8 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP “m=” line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well known issue of this element. NOTE 2 – The SDP based indication seems to be principally deficient (“application protocol specific SDP”), whereas H.248.78 supports the flexibility of different application protocols. |  |  |  |  |

{Editor’s note (2013-06 meeting): possible conclusions

The discussion about (non-)applicability of SDP is only relevant for steps 2, 3, 4, 5 and 7. The SDP method for step 2 may work for native TCP bearers (e.g., non-TLS scenario). Most critical are steps 3, 4 and 5 due to the single SDP attribute referred (“semantically overloaded”), which implies possible interactions between these control steps.

Steps 3 to 5 in more detail, dependent on TCP mode:

- TCP relay mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP merge mode: steps 3 to 5 are straightforward when H.248.84 based, i.e., no issue.

- TCP proxy mode: steps 3 to 5 see above Table y1.

}

## 11.2	Step 1: TCP endpoint creation

Not SDP related, based on H.248 Command request.

## 11.3	Step 2: Bearer type indication “TCP”

According the SDP-based method as defined by clause 9.6.1.

Conditional approach, dependent on:

C1: unambiguous indication of transport protocol “TCP” by protocol stack based SDP element “m=”-line <proto>.

C2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

## 11.4	Step 3: Block / Unblock TCP bearer control procedures at SEP

FIXTHIS

## 11.5	Step 4: Start TCP connection establishment procedure („TCP Open“)

FIXTHIS

## 11.6	Step 5: Local L4/L3 NAT traversal support

FIXTHIS

## 11.7	Step 6: Reporting successfully established TCP connection

FIXTHIS

## 11.8	Step 7: Local L4+ NAT traversal support

FIXTHIS

## 11.9	Step 8: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TCP) are out of scope of the SDP.

# 12	Security considerations

<placeholder from template>

Annex A

State modelling for 
TCP bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TCP basic connection control package (clause 8) is tightly coupled to a state model, because the event (tcpbcc/BNCChange) and signal (tcpbcc/EstBNC) are related to state transitioning behaviour. The detailed user plane state model for TCP is given by [IETF RFC 793]. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tcpbcc is described by this Annex.

## A.2	Original state model for TCP bearer connection endpoints

The principle model for TCP is comprised of 12 states and defined by Figure 6/[IETF RFC 793], see Figure A.1:

Figure A.1 – [IETF RFC 793] copy: TCP connection state diagram

## A.3	Simplified state model for H.248-based TCP basic connection control

The TCP basic connection control package makes a couple of assumptions, primarily:

- Bearer establishment:

  - neglects TCP simultaneous open;

  - interim states of 3-way handshake are not needed;

- Bearer release:

  - oneway closure not considered;

  - all interim states between the transitioning from TCP states ESTAB to CLOSED are not needed.

The resulting simplified model is depicted by Figure A.2:

Figure A.2 – Simplified state model for H.248-based TCP basic connection control

There are two remaining states, called Idle and Established in the H.248 state model. Signal tcpbcc/EstBNC is used for triggering state changes by the MGC, event tcpbcc/BNCChange prepares the MG for the reception of incoming bearer control information at user plane interface, and indicates successfully completed state transitions to the MGC.

The MGC may block the establishment of a TCP bearer connection, e.g., whilst TCP role assignment is yet unclear, ongoing call control signalling procedures, etc. Resulting in a third state Blocked from H.248 perspective, see Figure A.3.

Figure A.3 – Extended state model for H.248-based TCP basic connection control

Comment to the notion of “blocking”:

The concept of blocking/unblocking of bearer connection procedures is a known concept from BICC (see e.g. [b-ITU-T Q.2630.1]).

Appendix I

Sample use-cases of TCP connection control

(This appendix does not form an integral part of this Recommendation.)

## I.1	Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG

This use case shows an MG which provides a transport layer interworking function between a WebRTC-based data application and a UE located in a NGN or IMS network.

Background:

Web-based Real-Time Communication Services (WebRTC) include non-media (data) applications, which are using a datagram connection for data transport (see [b-IETF rtcweb-data]). The selected protocol solution results in a hierarchical protocol layering (data-over-SCTP-over-DTLS-over-ICE/UDP), driven by NAT traversal complexity, multiplexing and security related aspects.

The considered data applications (by WebRTC) would use legacy TCP transport in non-WebRTC IP network environments. [ITU-T H.248.WebRTC] considers several variants of H.248 WebRTC gateways which are positioned for WebRTC interworking scenarios.

The setup of termination T2 is controlled by the TCP connection control package while the setup of termination T1 is out of scope of this Recommendation.

Figure I.1 – Use-case #1: WebRTC to IMS interworking function

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation.)

## II.1	Assignment of TCP mode of operation

### II.1.1	Mode type: application agnostic TCP-proxy

The MGC needs to assign the TCP mode of operation to an H.248 context. This is fundamentally based on SDP information elements according [ITU-T H.248.84], clause 13.5. An example is provided by clause II.1.1.1.

The MGC could use also additional explicit H.248 signalling elements for the SEP individual assignment of TCP client/server roles, via the indication of incoming or outgoing TCP bearer connection establishment procedures, usingthe tcpbcc package capabilities as defined by this Recommendation, see example in clause II.1.1.2.

## II.2	Basic TCP bearer connection control

### II.2.1	Network configuration

The call flow is based on the network configuration as shown in Figure II.1:

Figure II.1 – Configuration “application agnostic TCP-proxy”

The use case shows an MG with two SEPs (labelled as T1(S1) and T2(S1)) configured in an application agnostic TCP-proxy mode. The configuration and the call flow are characterized as follows:

- MGC strictly control of the TCP bearer endpoints on both terminations

- Stream endpoint T1(S1): TCP-server, i.e., providing incoming TCP bearer establishment procedures (“TCP passive open” according [IETF RFC 793])

- Stream endpoint T2(S1): TCP-client, i.e., providing outgoing TCP bearer establishment procedures (“TCP active open” according [IETF RFC 793])

- TCP-connection for both terminations are setup in parallel

- TCP-connection released by remote TCP-endpoint of termination T1; application data may still flow from UE-Y to UE-X1.

### II.2.2	Establishment

Figure II.2 illustrates a traffic flow example concerning the establishment of an end-to-end TCP connection between UE-X and UE-Y, which relates to two TCP connection segments from H.248 MG perspective (due to TCP proxy mode).

The example traffic flow indicates parts of application control signalling (here SIP/SDP with scope on SDP Offer/Answer information), gateway control signalling (H.248) and bearer plane traffic (here TCP packets for connection establishment).

The MGC follows an early reservation strategy of MG resources, thus initiates the creation of a “TCP-TCP” context in the MG already during still ongoing capability negotiations at application control level. In order to prevent accidental TCP state transitioning in the MG due to “too early TCP media”, the MGC decides to block explicitly TCP bearer connection establishment (via property tcpbcc/bceb).

As soon as application control signalling allows, the MGC enforces incoming (at SEP T1(S1)) and outgoing (at SEP T2(S1)) TCP bearer connection establishment procedures.

The MGC wants to be notified about successfully established TCP bearer connection segments in this example.

The signalling scenario is termed as “MGC strictly controlled” due to the fact that only the TCP basic connection control package is used. The property sepplink of the TCP extension connection control package would tightly coupled both TCP connection control procedures, see Appendix IV.y.

Figure II.2 – Traffic flow example for an MGC strictly controlled TCP bearer connection establishment using the TCP-proxy mode

### II.2.3	Release

Figure II.3 illustrates the traffic flow for the use case when the MGC does not know which remote TCP endpoint initiates a TCP CLOSE procedures. Further, the default case of a bidirectional TCP bearer connection release is supposed. The MGC subscribes to event tcpbcc/BNCChange in order to be notified about release activities. It may be reminded that the notification is only issued when the local SEP is successfully transition to TCP connection state IDLE.

Figure II.3 – Traffic flow example for an incoming TCP bearer connection release, explicitly propagated by the MGC in outgoing direction

Figure II.4 provides another release example: the MGC enforces a TCP bearer release by initiating parallel TCP bearer connection release procedures towards both remote TCP endpoints X and Y:

Figure II.4 – Traffic flow example for outoing TCP bearer connection release procedures towards both remote TCP endpoints X and Y

## II.3	Extended TCP bearer connection control

### II.3.1	Establishment

This use case applies to the same configuration than the previous use case as depicted in Figure II.2. There is not any necessity for explicit TCP bearer establishment control by the MGC in this example, because e.g. of lacking NAT traversal support, or application specific, or security dedicated requirements. The direction of TCP bearer establishment is also unimportant from MGC point of view. The  MGC only want to wait with TCP establishment as long as application control level capability negotiations become stable, hence the MGC intially blocks possible establishment. Later on, TCP bearer establishment is unblocked and the interlinkage at the concerned stream endpoint pair is enabled, in both directions at the example of Figure II.5.

The notification of successful TCP bearer establishment is again an optional feature, up to the MGC level application/service control logic.

Figure II.5 – Traffic flow example for an extended TCP bearer connection establishment

### II.3.2	Release

Figure II.6 provides an example for an MG autonomous release of the TCP bearer connection. The MGC enables interlinkage in both traffic directions in this example, e.g. because the MGC does not know which remote TCP endpoint is initiating a TCP release.

Figure II.6 – Traffic flow example for an extended TCP bearer connection release

Appendix III

Illustration of protocol semantics 
of the TCP basic connection control package

(This appendix does not form an integral part of this Recommendation.)

## III.1	Overview

The tcpbcc package (clause 8) defines the basic TCP bearer connection control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## III.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TCP) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TCP bearer connection endpoint.

## III.3	Establishment of TCP bearer connections

### III.3.1	Successful establishment – Terminating side

See Figure III.1, termed as use case (E.1):

Figure III.1 – Successful establishment – Terminating side

### III.3.2	Successful establishment – Originating side

See Figure III.2, use case (E.2)

Figure III.2 – Successful establishment – Originating side

### III.3.3	Unsuccessful establishment

Figure III.3, use cases (E.3) summarizes the principle behaviour of unsuccessful TCP bearer connection establishment attempts:

Figure III.3 – Unsuccessful establishment

FIXTHIS: usage of g/cause still under discussion!

## III.4	Release of TCP bearer connections

### III.4.1	Successful release – Terminating side

See Figure III.4, termed as use case (R.1):

Figure III.4 – Successful release – Terminating side

FIXTHIS: usage of g/cause still under discussion!

### III.4.2	Successful release – Originating side

See Figure III.5, use cases (R.2) …

Figure III.5 – Successful release – Originating side

### III.4.3	Unsuccessful release

Figure III.6, use cases (R.3) summarizes the principle behaviour of unsuccessful TCP bearer connection establishment attempts:

Figure III.6 – Unsuccessful release

FIXTHIS: usage of g/cause still under discussion!

Appendix IV

Illustration of protocol semantics 
of the TCP extended connection control package

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tcpecc package (clause 9) defines an augmented TCP bearer connection control as the base package (tcpbcc, clause 8). The effect of each individual package property is illustrated in clauses IV.3 (property tcpecc/sepplink) and IV.4 (property tcpecc/ori). Clause IV.5 describes examples of combined usage of both properties.

The illustrated use cases are some examples. An exhaustive consideration of all possible combinations is out of scope.

## IV.2	Conventions

An H.248 context with a single SEPP, given by SEPs T1(S1) and T2(S1), is considered. The two tcpecc package properties are related to individual SEPs (as opposed to Context-level properties); however, the scope is on SEP T1(S1) property settings only.

Further, both properties provide only an extended behaviour (in contrast to the base package) for incoming TCP bearer connection control requests (i.e. the terminating side). The originating side is therefore out of scope of this Appendix.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) are not discussed, but could be for instance related to state changes of the local TCP bearer connection endpoint(s) (as defined by the base package).

## IV.3	Usage of SEPP Interlinkage property

Property tcpecc/sepplink is defined in clause 10.1.1. The property affects TCP bearer connection establishment and release procedures.

### IV.3.1	Property „sepplink = FALSE“

See Figure IV.1. Semantic FALSE implies decoupled SEPs, i.e., reflect the semantic of the base package. There are two separate TCP bearer connection segments, - the two segments related to TCP connections “X  MG (SEPT1(S1))” and “Y  MG (SEPT2(S1))” -, from perspective of the MG.

Figure IV.1 – Property „sepplink = FALSE“

Here: an incoming TCP bearer connection establishment request (via TCP-SYN) or release request (via TCP-FIN) at SEP T1(S1) does not affect SEP T2(S1).

Figure IV.2 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

Figure IV.2 – Signalling example for TCP bearer connection establishment 
– Property „sepplink = FALSE“

It may be emphasized that the sepplink setting at SEP T2(S1) does not have any impact in these scenarios.

If an incoming TCP bearer connection establishment request from remote X should also lead to establishment procedures towards remote TCP endpoint Y, then the MGC needs to be involved (see E.1 example).

### IV.3.2	Property „sepplink = TRUE“

See Figure IV.3. Semantic TRUE leads to a forwarding of the TCP bearer connection control request within a SEPP.

Figure IV.3 – Property „sepplink = TRUE“

The TCP open and closure procedures, when initiated by remote TCP endpoints (X or Y), could now have an end-to-end scope. E.g., the three-way handshake for TCP bearer connection establishment or a TCP half-closure for oneway TCP bearer connection release would be leaded by remote TCP endpoints (X, Y) and forwarded via the SEPP, under the condition that both sepplink properties (of SEPT1(S1) and SEPT2(S1)) would be set to TRUE.

Figure IV.4 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

Figure IV.4 – Signalling example for TCP bearer connection establishment 
– Property „sepplink = TRUE“

It may be emphasized that the sepplink setting at SEP T2(S1) does not have again any impact in these scenarios (because only considering incoming procedures at SEP T1(S1)).

## IV.4	Usage of Oneway Release Indicator property

Property tcpecc/ori is defined in clause 9.1.2. The property affects only TCP bearer connection release procedures.

### IV.4.1	Property „ori = FALSE“

See Figure IV.5. Semantic FALSE implies base package behaviour, i.e., a complete TCP bearer connection release.

Figure IV.5 – Property „ori = FALSE“

Here: an incoming TCP bearer connection release request (via TCP-FIN) at SEPT1(S1) triggers an immediate TCP half-closure of the reverse direction towards X, but does not affect SEPT2(S1).

### IV.4.2	Property „ori = TRUE“

Semantic TRUE results in a oneway TCP bearer connection release, see Figure IV.6..

Figure IV.6 – Property „ori = TRUE“

## IV.5	Combined consideration of both properties

Only TCP bearer connection release (due to ori property) needs to be considered.

### IV.5.1	Property settings: “ori = FALSE“ and “sepplink = TRUE“

See Figure IV.7. The ori property codepoint leads to a complete TCP bearer connection release between MG and remote TCP endpoint X, and the sepplink property codepoint results in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.7 – Property settings: “ori = FALSE“ and “sepplink = TRUE“

### IV.5.2	Property settings: “ori = TRUE“ and “sepplink = TRUE“

See Figure IV.8. The ori property codepoint leads to an oneway TCP bearer connection release only between MG and remote TCP endpoint X, and the sepplink property codepoint results again in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.8 – Property settings: “ori = TRUE“ and “sepplink = TRUE“

### IV.5.3	Signalling examples

Figure IV.9 summarizes the impact of both properties (Note: only sepplink = TRUE depicted; value FALSE does not provide any new information).

Figure IV.9 – Signalling example for TCP bearer connection release and example value combinations for both properties

Use case (C.1): the incoming TCP bearer connection release request from remote TCP endpoint X is propagated on the other side to remote TCP endpoint Y, and the base package behaviour of a bidirectional TCP connection release towards X.

Use case (C.2): Property ori enforces now a TCP half-closure in the direction to X, and sepplink the start of TCP bearer connection release towards Y.

Bibliography

[b-ITU-T Q.2630.1]	Recommendation ITU-T Q.2630.1 (1999), AAL type 2 signalling protocol

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

________________________