| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-17 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPTOPO |  |  |  |
| Title: | H.248.RTPTOPO "RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations" (New): Output draft Ed. 0.16 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.RTPTOPO "RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations" (Ed. 0.16).

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPTOPO Ed. 0.1 | Initial draft (TD-38) from WP 2/16 Research Triangle Park (USA) meeting (2010-11), based on AVD 4000 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/TD-38.zip |
| H.248.RTPTOPO Ed. 0.2 | Input (TD 458 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458 |
| H.248.RTPTOPO Ed. 0.3 | Input (TD 458R1 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458R1 |
| H.248.RTPTOPO Ed. 0.4 | Input (AVD-4051) to WP 2/16 Andover (USA) meeting (2011-07) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/AVD-4051.zip |
| H.248.RTPTOPO Ed. 0.5 | Output draft (TD-39) from WP 2/16 Andover (USA) meeting (2011-07) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/TD-39.zip |
| H.248.RTPTOPO Ed. 0.6 | Input (TD 620 (WP2/16)) from ITU-T SG16 Geneva meeting, Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620/ |
| H.248.RTPTOPO Ed. 0.7 | Output (TD 620R1 (WP2/16)) from ITU-T SG16 Geneva meet., Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620R1 |
| H.248.RTPTOPO Ed. 0.8 | Input (AVD-4162) to WP 2/16 Geneva (USA) meeting (2012-02) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/AVD-4162.zip |
| H.248.RTPTOPO Ed. 0.9 | Input (TD 748 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748 |
| H.248.RTPTOPO Ed. 0.10 | Output (TD 748R1 (WP2/16)) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748R1 |
| H.248.RTPTOPO Ed. 0.11 | Input (AVD-4231) to WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4231.zip |
| H.248.RTPTOPO Ed. 0.12 | Output (TD-21) from WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-21.zip |
| H.248.RTPTOPO Ed. 0.13 | Input (TD-18/WP1) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0018 |
| H.248.RTPTOPO Ed. 0.14 | Output (TD 18R1/WP1) from ITU-T SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0018 |
| H.248.RTPTOPO Ed. 0.15 | Input (AVD-4367) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4367.zip |
| H.248.RTPTOPO Ed. 0.16 | Output (TD-17) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-17.zip |

CONTENTS

Page

1 Scope 6 2 References 6 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 8 4 Abbreviations 9 5 Conventions 10 6 Background to RTCP handling by H.248 MGs 11 7 RTP topologies versus RTCP handling in H.248 MGs 12 7.1 Single H.248 "IP" Termination per Context 12 7.2 Two H.248 "IP" Termination per Context 12 7.2.1 RFC 5117 defined RTP topologies 12 7.2.2 Other RTP topologies 13 7.3 More than two H.248 "IP" Terminations per Context 14 7.4 Others 15 7.5 Notes to the H.248 Connection Model 15 7.6 Classes of RTCP related functions 16 7.7 Relation to modes of operations of IP-to-IP media gateways 16 8 RTCP services 17 8.1 RTCP packet structure 17 8.2 Important RTCP information elements (from H.248 MG perspective) 18 8.2.1 Information elements related to the identification of an RTP entity 18 8.2.2 Other relevant information elements 18 8.3 RTCP service levels 18 8.3.1 Basic RTCP services 18 8.3.2 RTP profile dependent RTCP services 18 8.3.3 Supplementary RTCP services 19 9 Scope of RTP topology control 19 10 RTP Topology Package 20 10.1 Properties 21 10.1.1 RTP Topology 21 10.2 Events 21 10.3 Signals 21 10.4 Statistics 21 10.5 Error Codes 21 10.6 Procedures 22 10.6.1 General 22 10.6.2 MGC level information for RTP topology control 22 10.6.3 Topology dependent procedures 22 11 Package-independent procedures for RTP topology control 24 11.1 Topology “RTP source translator” 24 11.1.1 Via SDP attribute "a=ssrc:" 24 11.1.2 Via MIME subtype “rtx” in SDP attributes 24 Bibliography 25 Appendix I Examples of transparent forwarding 26 I.1 Introduction 26 I.2 Convention 26 I.3 Examples for implicit signalling 27 I.3.1 Example 1 “UDP payload transparent forwarding” 27 I.3.2 Example 2 “fully RTP protocol unaware forwarding” 28 I.3.3 Example 3 “additional RTCP awareness” 28 I.3.4 Example 4 “fully specified SDP media description” 29 I.3.5 Example 5 “transport protocol translation for RTP bearer traffic” 30 I.3.6 Example 6 “transparent forwarding of encrypted RTP traffic” 30 Appendix II Living List related to RTP Topology Package 32 Appendix III Living List related to H.248.77 related RTP topologies 33

List of Tables

Page

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Single H.248 "IP" Termination per Context 12 Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Two H.248 "IP" Terminations per Context 13 Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models – More than two H.248 "IP" Terminations per Context 15 Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Others 15 Table 5 – Indication of topology “RTP source translator” 24 Table I.1 –Implicit signalling – abstracted H.248 command format with principal areas related to RTP 26 Table I.2 – Example 1 “UDP payload transparent forwarding” 27 Table I.3 – Example 2 28 Table I.4 – Example 4 29 Table I.5 – Example 5 30 Table I.6 – Example 6 30

List of Figures

Page

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): BGF modes - H.248 Local and Remote Descriptor: SDP "m="-line specification combinations 17 Figure 2 - RTCP packet structure (Compound RTCP packet) 18 Figure 3 – Level of RTCP services 20

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPTOPO)

Gateway control protocol: 
RTP Topology dependent RTCP Handling 
by H.248 Media Gateways with IP Terminations

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

The scope of this Recommendation is related to RTCP handling by H.248 Media Gateways (MG) [ITU-T H.248.1]. The required MG behaviour is fairly clear for the very majority of use cases. However, there are some scenarios which benefit from or even require explicit signalling indication for as an unambiguous control of MG RTCP (but also RTP) traffic handling behaviour.

This Recommendation covers

- information about history and motivation behind RTP topologies and RTCP handling and the available H.248 tool landscape (clause 6);

- overview tables concerning "RTP topology versus RTCP handling" aspects and relevant H.248 connection models (clause 7);

- RTCP service aspects (clause 8);

- scope and limitations of RTP topology control (clause 9); and

- an H.248 package for basic RTP topology control (clause 10).

This Recommendation may be useful in scenarios where an MG supports multiple RTP topologies and/or multiple RTP/RTCP services which may lead to ambiguous behaviour. In these scenarios a H.248 profile specification may use the package in this Recommendation to alleviate this behaviour.

# 2	References

{Editor's note: this is a preliminary list of documents related to RTP/RTCP. This is just a living list.}

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (06/2008), Gateway control Protocol: IP NAPT traversal package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control Protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control Protocol: NAT traversal toolkit packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (06/2010), Gateway control Protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control Protocol: RTCP support packages.

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (11/2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 4585]	IETF RFC 4585 (07/2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4588]	IETF RFC 4588 (07/2006), RTP Retransmission Payload Format.

[IETF RFC 4961]	IETF RFC 4961 (07/2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5093]	IETF RFC 5093 (12/2007), BT's eXtended Network Quality RTP Control Protocol Extended Reports (RTCP XR XNQ).

[IETF RFC 5117]	IETF RFC 5117 (01/2008), RTP Topologies.

[IETF RFC 5124]	IETF RFC 5124 (02/2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (04/2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5506]	IETF RFC 5576 (06/2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 5725]	IETF RFC 5725 (02/2010), Post-Repair Loss RLE Report Block Type for RTP Control Protocol (RTCP) Extended Reports (XRs)).

[IETF RFC 5760]	IETF RFC 5760 (02/2010), RTP Control Protocol (RTCP) Extensions for Single-Source Multicast Sessions with Unicast Feedback.

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port.

[IETF RFC 5968]	IETF RFC 5968 (09/2010), Guidelines for Extending the RTP Control Protocol (RTCP)).

[IETF RFC 6128]	IETF RFC 6128 (02/2011), RTP Control Protocol (RTCP) Port for Source-Specific Multicast (SSM) Sessions.

[IETF RFC 6792]	IETF RFC 6792 (11/2012), Guidelines for Use of the RTP Monitoring Framework.

# 3	Definitions

## 3.1	Terms defined elsewhere

3.1.1	Bit integrity [ITU-T Q.9]: Exists when the values of the bits in each octet of a digital bit stream at the output of a device or system are unchanged from those at the input.

NOTE – Digital processing devices such as A/mu law converters, echo suppressors and digital pads must be disabled to provide bit integrity.

3.1.2	Data integrity [ITU-T T.411]: The property that data has not been altered or destroyed.

## 3.2	Terms defined in this Recommendation

3.2.1	Basic RTCP service: the RTCP services based on RTCP packet types SR (200), RR (201), SDES (202) and BYE (203).

NOTE 1 – These are RTCP services are considered to be the minimum of any RTP implementation according [IETF RFC 3550].

3.2.2	Fully protocol unaware forwarding: Transparent forwarding, but the MG is unaware of the Lx protocol(s). The MG is thus not able to execute any kind of operation on the Lx-PDU.

NOTE 1 – Example for "fully RTP protocol unaware forwarding": the H.248 descriptor(s) does not contain any kind of signalling information (such as H.248 events, properties, signals, statistics or SDP elements) related to the indication of RTP traffic. There would be a fully L4+ agnostic stream setting.

3.2.3	Lx-PDU integrity: In the context of packet processing, it is the basic property of data integrity. The MG must guarantee bit integrity for forwarded protocol data units (PDU) at layer Lx. Lower layer protocol control information (PCI) may be modified.

{Editor's note #1 (2012-09):  this term would be better located in H.248.79, should we create a correspondent IG item?}

3.2.4	RTP domain: A network domain with RTP entities which share a common RTP/RTCP-related identifier space. A RTP domain is characterized therefore by a domain wide (and temporal limited, NOTE 1) uniqueness of such protocol identifiers, which typically constitute unambiguous RTP traffic flows (e.g., protocol elements SSRC, CNAME, PT).

NOTE 2 – Temporal horizon of identifier values: [b-IETF RFC 6222] differentiates the identifier lifetime in short-term persistent and long-term persistent identifier value assignments.

NOTE 3 – An end-to-end RTP traffic flow could traverse multiple RTP domains, leading to the following observations:

- The end-to-end communication association would be comprised of multiple, concatenated RTP sessions.

- The edge of RTP domains is given by RTP entities with RTP topologies of type "RTP Endsystem" or "Back-to-Back RTP Endsystem".

{Editor's note (2012-09): the term "RTP session", introduced by RFC 1889/3550,  is currently evolving due to ongoing work on "RTP multiplexing" in IETF WGs AVTCORE, MMUSIC, RTCWEB and CLUE. There are already different RTP session concepts floating around. It looks like that the RTP session is always a bidirectional concept. The RTP session is also dependent on RTP topologies. The RTP session is basically constituted by a n-tuple of identifiers. An RTP session includes usually the associated RTCP control flow to the RTP media flow.  […] The term "RTP session" is frequently used in this document and should be clarified in clause 3.1 or 3.2. A "RTP session" may become a local concept (related to an RTP endpoint), rather than a global concept as e.g. related to end-to-end communication.  Contributions are solicited.}

3.2.5	RTP packet integrity: Lx-PDU integrity for PDU type "RTP packet" [IETF RFC 3550]. Thus, RTP packet integrity implies RTCP packet integrity.

{Editor's note #2 (2012-09):  …inclusion of RTCP might be debatable …}

NOTE 4 – The embedded concept of bit integrity restricts this property to "input-output" related RTP topologies (e.g., the RTP end system topology would be excluded).

{Editor's note #3 (2012-09):  looks like that the characteristic of "RTP packet integrity" is limited to clause 7.2 topologies only …}

NOTE 5 – RTCP packet integrity is limited to end-to-end RTCP control flows, but not related to RTCP packets originating or terminating at the MG (such as possible RTCP XR packets).

3.2.6	RTP profile related RTCP service: the RTCP service given by a specific RTP profile.

3.2.7	RTP source translator: RTP topology related to the translation of RTP source specific parameters. This topology belongs to the high level category of “RTP translator” topology.

NOTE – This topology is not defined by [IETF RFC 5117].

{Editor’s note (2013-01 meeting): the name is still under discussion, might be e.g. replaced by “RTP source proxy” topology.
It was also observed that clause 3.3/RFC 5117 on translator topologies could benefit from some clarifications and/or more detailed information. Might be subject of a revised RFC 5117 in IETF …}

3.2.8	RTCP service: A MG service tied to a particular RTCP packet type codepoint. The RTCP PT value may be thus considered as RTCP service identifier.

NOTE 6 – This is the coarsest granularity of an RTCP service definition, but sufficient for this Recommendation. A finer grain RTCP service level could take into account further RTCP packet embedded identifiers such as the block type (BT) codepoint in case of RTCP XR or the feedback message type (FMT) codepoint in case of RTCP FB.

NOTE 7 – Another classification approach could derive generic (because usually PT-independent) RTCP service categories such as related to performance monitoring, media encryption, codec rate control, congestion control, generic feedback, keep alive, etc.

3.2.9	Supplementary RTCP service: any RTCP service in addition to basic RTCP services (clause 3.2.1) and/or RTP profile RTCP related services (clause 3.2.6).

3.2.10	Transparent forwarding: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

NOTE 8 – This is normally implicit, basic behaviour, when LD/RD settings are identical: "identical media descriptions" imply "transparent forwarding" by the MG by the MG under the condition that the MG could derive the intended protocol layer Lx of transparent forwarding.

NOTE 9 – There would be then the characteristic of RTP packet integrity in the context of "RTP transparent forwarding". The MG might be RTP aware; e.g. support of RTP related statistics or event detection would not violate transparent forwarding behaviour.

NOTE 10 – Delayed transparent forwarding is a variant of transparent forward from a temporal perspective, which is characterized by transparent forwarding, but Lx-PDUs might be intentionally delayed in the outgoing direction. The traffic characteristic of the Lx-PDU flow would be then modified.
Examples:

- The application of IP traffic shaping (see clause 7.1.5/[ITU-T Y.1221]) might be one example in case of Lx equal to 'IP'.

- The application of RTP jitter elimination, reduction or adjustment (see [ITU-T G.799.3]) might be one example in case of Lx equal to 'RTP'

# 4	Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| ASY | Asymmetric topologies |
| --- | --- |
| AVP | (RTP) Audio Visual Profile |
| AVPF | (RTP) Audio Visual Profile with Feedback |
| B2BRE | Back-to-Back RTP End system |
| BGF | Border Gateway Function NOTE – synonym to Border Gateway and IP-to-IP Gateway in this Recommendation |
| BT | (RTCP) Block Type |
| CNAME | Canonical Name |
| CSRC | Contributing Source |
| FB | (RTCP) Feedback |
| FMT | (RTCP FB) Feedback Message Type |
| IP | Internet Protocol |
| LD | Local Descriptor |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PT | (RTP) Payload Type; (RTCP) Packet Type |
| RD | Remote Descriptor |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| RTPASM | RTP Any Source Multicast |
| RTPE | RTP End system (or RTP endpoint) |
| RTPMU | RTP Multicast |
| RTPMT | RTP Media Translator |
| RTPRTM | RTP RTCP-Terminating-MCU |
| RTPSSM | RTP Source Specific Multicast |
| RTPTT | RTP Transport Translator |
| RTPVSM | RTP Video-Switch-MCU |
| SAVP | (RTP profile) Secure Audio Visual Profile |
| SDES | Source Description (RTCP Packet) |
| SR | (RTCP) Sender Report |
| SSRC | Synchronization Source |
| VoIP | Voice over Internet Protocol |
| XR | (RTCP) eXtension Report |

# 5	Conventions

None.

# 6	Background to RTCP handling by H.248 MGs

The handling of RTCP packets by H.248 media gateways is often not straightforward, sometimes not fully clear also from IETF RFC side. RTCP handling in general is tightly coupled to the RTP topology model, which drives the processing of RTP/RTCP traffic of a particular H.248 Context.

The aspect of RTP topologies is for many H.248 MG types not really relevant because the handling is obvious. For example: for all H.248 (IP, physical) connection models (as applied for VoIP MGs at the border towards circuit-switched networks). However, for H.248 (IP,IP) MGs the situation is slightly different.  In this case new RTCP services have been defined that increase the possible interactions between RTP topology and these services. This has led to possible ambiguity in the RTCP handling behaviour in the MG.

The problem also has an historical dimension: [IETF RFC 1889] and [IETF RFC 3550] were too vague on that point, which was the motivation to clarify on a more detailed level RTP topology models, published as [IETF RFC 5117]. [IETF RFC 5117] provides a more fine granular topology classification and detailed description.

NOTE 1- It may be noted that the problem is not specific to H.248, rather concerns also other gateway/policy control protocols, which are applied for IP-IP gateways or policy enforcement points. For example: [ITU-T H.248.48] considers different RTCP processing functions (e.g., local generation of RTCP reports, forwarding/filtering of RTCP reports). [ITU-T H.248.48] is actually applicable for multiple RTP topologies and could be categorized as a supplementary RTCP service (see clause 8.3.3). It has had to consider these interaction issues.

Generally RTP topology and RTCP handling may be summarized:

- Local RTCP handling by an H.248 MG is basically linked to the applied RTP topology, and the RTP topology is given by the Terminations/Streams in a H.248 Context.

- Without this Recommendation explicit H.248 means are not defined to control RTP topology. Thus, H.248 profiles that do not utilise this Recommendation assume implicit behaviour ("which may lead to misinterpretations").

- Existing H.248 protocol elements for RTP/RTCP control are for other, different purposes, i.e.:

  - [ITU-T H.248.1] Annex E.12 "RTP Package": event and statistics

  - [ITU-T H.248.30] "RTCP extended performance metrics packages":
- RTCP XR Base Package,
- RTCP XR Burst Metrics Package

  - [ITU-T H.248.48] "RTCP XR Block Reporting Package": statistics

  - [ITU-T H.248.50] "RTP Keep Alive Request Package": "RTP signals"

NOTE 2 – Discussion: H.248.50 signal parameter "Keep Alive Packet Type" (from signal "Send keepalive packet") indicates some RTP/RTCP related packet types, which represent the semantic of RTP topology "RTP endsystem" (sender only). There might be then following controversial situation if the H.248 Stream endpoint were enabled with signal e.g. kar/skap/kapt/cn (indicating topology RTP endsystem) and also part of an overall RTP transport translator (given by a property setting of "rtpt/rtptopo = TR"). Such a situation may be resolved by indicating the RTP keep alive mechanism as a supplementary service (see clause 9).

  - [ITU-T H.248.57] "RTP Control Protocol Package": L4 port allocation control

  - [ITU-T H.248.58] "RTP Application Data Package": statistics

  - [ITU-T H.248.71] "RTCP support packages":
- RTCP Source Description Package, 
- Received RTCP Package, 
- RTCP Feedback Message Package

The topic as such, correct RTCP handling by H.248 MGs, is important in order not to jeopardize end-to-end RTP-based services.

# 7	RTP topologies versus RTCP handling in H.248 MGs

This clause provides an attempt to structure RTP Topologies vs RTCP Handling vs H.248 MG Context configurations. The functions related to RTCP handling are labelled with "F-x" according clause 7.6. The RTP topologies are classified according H.248 connection models which are determined by the number of terminations per Context, see clauses 7.1 to 7.4.

## 7.1	Single H.248 "IP" Termination per Context

See Table 1.

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Single H.248 "IP" Termination per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| I.1 | RTP Endsystem (RTPE) (aka RTP Endpoint) | F-1,2,3,5 | (RTP, non-RTP) | VoIP Residential -, Access – or Trunking MG |

## 7.2	Two H.248 "IP" Termination per Context

### 7.2.1	RFC 5117 defined RTP topologies

See Table 2.

Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Two H.248 "IP" Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| II.1 | Back-to-Back RTP Endsystem (B2BRE) | F-1,2,3,4,5,6,7 | (RTP, RTP) | IP-to-IP MG located at the border between two RTP domains (e.g., SRTP-to-RTP interworking) |
| II.2 | RTP Transport Translator (RTPTT) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, without any media processing |
| II.3 | RTP Media Translator (RTPMT) in general (NOTE 1) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, with processing of the RTP payload |
| II.3.1 | RTP Media Translator without media format change (RTPMTlight) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., transpacketization (changing packetization times), jitter reduction between two IP networks |
| II.3.2 | RTP Media Translator with media format change (RTPMTfull) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., audio transcoding |
| NOTE 1 – The RTPMT topology is sometimes further sub-classified here with II.3.1 & II.3.2 (see e.g. [b-IETF RFC 6679]). |  |  |  |  |

NOTE - Some media-level functions may be achieved by more than one RTP topology. For example, audio transcoding may be done in B2BRE or RTPMT topologies. However, there would be different RTCP handling functions.

### 7.2.2	Other RTP topologies

The high-level RTP topology models according [IETF RFC 5117] did evolve since the publication of this RFC (in 2008) in terms of a few directions: benefit of more detailed information (semantical clarifications)?, evaluation whether the number of existing models might be reduced? and the identification of new RTP topology models. This subclause provides descriptions of such new/extended models.

#### 7.2.2.1	RTP source translator topology

RTP source translators (RTPST) do not modify the media stream itself, but are concerned with RTP source parameters. RTP source parameters comprise the

- RTP source identifier (parameter  SSRC),

- RTP source/endpoint persistent identifier (parameter CNAME),

- RTP source sequence number space (parameters SN, OSN) and

- RTP source time space (parameter TS).

- {others?}

RTP source translators could modify one or multiple RTP source related parameters.{Editor’s note (2013-01 meeting): RTCP handling should be described in more detail. Contributions are solicited.}

#### 7.2.2.2	IETF Media-aware network element (MANE)

{Editor’s note (2013-01 meeting): this is preliminary text on the “MANE function” and there relation to H.248 gateways and RTP topologies:

The concept of media-aware network elements (MANE) is loosely used in the IETF when looking at general IP traffic handling. MANE behaviour in context of RTP is defined in [b-IETF RFC 6184] (and referred e.g. by [b-IETF RFC 6184], [b-IETF rtp-howto]):

Media-aware network element (MANE): A network element, such as a middlebox or application layer gateway that is capable of parsing certain aspects of the RTP payload headers or the RTP payload and reacting to the contents.

Media-awareness implies signalling-awareness. H.248 IP-IP gateways could realize the IETF MANE behaviour. The MGC is responsible for the signalling path related MANE part (typically “SDP awareness”), the MG for RTP packet processing. The RTP functions provided by a MANE device could be considered to be similar as RTP media translator or RTP mixer topologies. However, “MANE packet policy rules” may contain additional policy actions such as packet discard in case.

MANEs for RTP traffic could be therefore considered as extended RTP topology models.

Discussion/observations:

The relevance of the MANE topology to this Recommendation is still unclear:

- Does it represent a totally disjoint value with the already defined rtpt/rtptopo values?

- Or is it a particular case of one the already existing values?

- Or is it overlapping with more than one value?

- How should it be signalled to the MG (implicit and/or explicit)?

- There is a rather specific behaviour expected from a MG that acts as a MANE in the sense of RFC 6184. It is questionable if this should be specified in this Recommendation.

- The generic MANE function is introduced by RFC 6184, but not tightly coupled to the RTP media format as in scope of RFC 6184, rather applicable for other RTP media format types as well.

- Discuss concrete MG behaviour for media-format specific examples. 
What would be the closest RTP topology?
How would such MG RTP traffic processing behaviour controlled via H.248 (see (4))?

Contributions are solicited}

## 7.3	More than two H.248 "IP" Terminations per Context

See Table 3.

Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models –
More than two H.248 "IP" Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| III.1 | RTP Mixer (RTPM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.2 | RTP Video-Switch-MCU (RTPVSM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.3 | RTP RTCP-Terminating-MCU (RTPRTM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server where each H.248 RTP Termination relates to the "RTP Endsystem" topology |
| III.4 | RTP Multicast (RTPMU) | TBD (see clause 7.6) | TBD |  |
| III.4.1 | RTP Source Specific Multicast (RTPSSM) | TBD (see clause 7.6) | TBD |  |
| III.4.2 | RTP Any Source Multicast (RTPASM) | TBD (see clause 7.6) | TBD |  |

## 7.4	Others

See Table 4.

Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models –
Others

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| IV.1 | Asymmetric topologies (RTPASY) | TBD (see clause 7.6) | (RTP, MUX, RTP … RTP) (TBD). | H.248 Contexts with internal H.248 MUX terminations … |
| IV.2 | Topology combinations | TBD (see clause 7.6) | (TBD). | Cascaded H.248 Contexts … |

## 7.5	Notes to the H.248 Connection Model

Names of terminations (TerminationID) may be arbitrarily chosen (see clause 6.2.2/[ITU-T H.248.1]), but termination naming schemes are usually given by H.248 profile specifications. Such H.248 profiles typically use the notation of "IP" for IP-based ephemeral terminations (e.g. in order not to exclude any possible IP applications, which might be concluded by a transport protocol or application protocol specific notation). This Recommendation uses "RTP" instead because there may be a difference between following connection models:

- (RTP, RTP)

  - MG is "RTP-aware", which enforces a transport-protocol aware mode of operation

  - MG may be media aware or media agnostic

- (IP, IP)

  - MG may be "RTP-agnostic", i.e., aware of the transport protocol (UDP), but unaware of higher protocol layers, thus RTP packets may transparently go through such an H.248 Context.

It could be noted that the termination name could carry an implicit semantic concerning configured RTP topologies and/or RTCP handling behaviour of a Context. However, it looks unfortunate to overload the termination name with such semantics (note: clause 7.1 would be an exception). This Recommendation therefore recommends that the H.248.TerminationID should not be used to indicate a particular RTP topology.

## 7.6	Classes of RTCP related functions

The MG could be requested to process following RTCP packets or RTCP related functions (Note: the list label is used in Tables 1 to 4):

F-1:	RTP node identifier: usage (or not) of MG-local CNAMEs (per MG?; per termination?);

F-2:	RTP source identifier: allocation (or not) of MG-local SSCRs per RTP session;

F-3:	RTCP protocol endpoint for basic services: origination/termination of RTCP packets (reports);

F-4:	RTCP packet forwarding: unmodified forwarding of RTCP packets (reports);

F-5:	RTCP packet modification: rewrite of RTCP packets;

F-6:	RTCP protocol endpoint for supplementary services: local generation of RTCP reports (such as for RTCP XR based applications);

F-7:	RTCP packet filtering: filtering of RTCP reports (e.g., either entire RTCP packet or just an individual RTCP report within a compound packet is filtered);

F-8:	others

## 7.7	Relation to modes of operations of IP-to-IP media gateways

The subject of "RTP topologies" affects all kinds of H.248 MGs, which are involved in RTP traffic processing, from PSTN residential/access/trunking MGs to media servers for large conferencing systems. Of particular interest are H.248 IP-to-IP media gateways (also known as border gateways, at the network level of access/edge, core or interconnect/peering) due to RTP traffic processing related to NAPT, NAT-T, QoS support, policy control and/or media conversion functions, based on RTP topologies according to clause 7.2.

The basic packet processing behaviour by such MGs is called "mode of operation", leading to the aspect of possible interactions between RTP topologies and such modes of operations as defined for IP-to-IP media gateways.

There are already a number of such modes discriminated for IP-to-IP media gateways, like e.g. "media-format agnostic, transport protocol type aware", etc. For instance, [b-ETSI TR 183 068] provides an established categorization which could be summarized according Figure 1.

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): 
BGF modes - H.248 Local and Remote Descriptor: 
SDP "m="-line specification combinations

The modes of operation are related to the media description in the local and remote descriptors. The classification in Figure 1 is based on the SDP "m="-line only.

Recommendations for H.248 profiles for IP-to-IP gateways:

A particular profile may explicitly require support of a number of RTP topologies and a number of different modes of operations. There are following two situations:

- There are not any further specification actions required in case of semantical unambiguity (e.g., no interactions concluded). The correct RTP/RTCP processing behaviour could be clearly concluded by the MG from Context specifications, without explicit RTP topology control

- In case of possible interactions, unclear semantics and/or explicit RTP topology related behaviour, then it is recommended to consider explicit RTP topology control according clause 10. The semantics of the RTP topology package takes then preceding over any other (implicit) semantics related to modes of operation.

NOTE  – the profile may also need to provide further clarification on the derived MG behaviour in the above instance.

# 8	RTCP services

Purpose of this clause is to describe the handling of RTCP packets, dependent on the RTP topology and used RTCP services.

## 8.1	RTCP packet structure

Figure 2 illustrates the basic RTCP packet structure at the example of a compound RTCP packet.

Figure 2 - RTCP packet structure (Compound RTCP packet)

The RTCP packet structures need to be considered to identify key information elements (see next subclause).

## 8.2	Important RTCP information elements (from H.248 MG perspective)

Not every information element of RTCP packets is relevant from a H.248 perspective. There is however some elements which demand careful consideration when processing RTP/RTCP traffic by H.248 MGs.

### 8.2.1	Information elements related to the identification of an RTP entity

There are primarily three information element types:

- SSRC (per originating RTP source),

- CSRC (for contributing RTP source), and

- SDES.

These elements define the spatial horizon of RTP media and RTCP control flows, i.e. the "end-to-end" significance of RTCP services. See for instance the detailed evaluation of example H.248 measurement architectures by [ITU-T H.248.48] and IETF monitoring architectures for RTP by [IETF draft- monarch].

### 8.2.2	Other relevant information elements

TBD

{Editor's note: there might be RTCP information elements of primary interests for RTP end-systems only …FFS.}

## 8.3	RTCP service levels

### 8.3.1	Basic RTCP services

Every RTP implementation must be compliant to [IETF RFC 3550] and thus basically support all the RTCP services defined by this RFC. They may be called basic RTCP services and cover the four RTCP packet types of SR (200), RR (201), SDES (202) and BYE (203).

NOTE - The codepoint for application defined RTCP packet types APP (204) is also introduced by this RFC, but the definition of application specific services is delegated to subsequent RFCs.

Basic RTCP services are indicated to the MG by e.g. RTCP port allocation (see [ITU-T H.248.57]) for processing RTCP control flow traffic by an H.248 context.

### 8.3.2	RTP profile dependent RTCP services

RTP profiles may add additional RTCP services like e.g. the RTP profiles RTP/AVPF and RTP/SAVPF.

The use of RTP profile dependent RTCP services are basically indicated to the MG by the protocol field value in the SDP "m=" line, as part of the LD-/RD-embedded SDP media description.

### 8.3.3	Supplementary RTCP services

RTP sessions may be supplemented by further RTCP-based services. [IETF RFC 5968] defines the generic extension principles for RTCP. Such kind of supplementary services are for instance the RTCP extension report (XR) based capabilities, as defined for measurement and monitoring architectures.

Supplementary RTCP services are indicated to the MG by e.g. RTCP report/packet type specific signalling elements like e.g. H.248 packages [ITU-T H.248.30] and [ITU-T H.248.48], or correspondent SDP attributes as part of the LD-/RD-embedded SDP media description.

{Editor's note: next step would be the completion of Tables 1 to 4 concerning RTCP handling input. The table input would have to address

- the guidelines from RFC 5117 for each topology,

- the key RTCP information elements, and

- the RTCP service levels.

One result might be that the RTP topology is basically just impacting the basic RTCP and RTP profile dependent services. And supplementary RTCP services would be an overlay on top of RTP topologies … For further studies.}

# 9	Scope of RTP topology control

There is a relation between RTP topology and RTCP services. However, the scope of RTP topology control needs to be limited due to the plethora of RTCP services and in order to handle complexity. Figure 3 summarizes again the main RTCP service categories (from clause 8).

Figure 3 – Level of RTCP services

The scope of RTP topology control, - based on the H.248 rtpt package defined by this Recommendation -, could be pragmatically limited on basic (I) and RTP profile dependent (II) RTCP services.

Example:

The H.248 RTP stream in an H.248 IP-IP MG could be configured for RTP topology "Topo-Translator", which implies RTCP services such as

- no allocation of local SSRC values,

- no generation of RTCP RR reports, and

- forwarding of RTCP BYE packets.

The same H.248 RTP stream could be additionally enforced for H.248.48 type performance monitoring capabilities, which could lead to RTCP services such as

- allocation of local SSRC values for measurement points, or/and

- generation of RTCP XR reports.

The H.248.48 related supplementary RTCP services are not in contradiction to the RTP topology associated ones.

# 10	RTP Topology Package

|  | Package Name: | RTP Topology Package |
| --- | --- | --- |
|  | Package ID: | rtpt (0x????) |
|  | Description: | The package allows a MGC to indicate to a MG which particular RTP topology an RTP stream is operating under. The RTP Topologies are fundamentally defined by [IETF RFC 5117] and complemented by this Recommendation. A single H.248 Stream allows the definition of two RTP streams thus the topology is defined at a Local and Remote Descriptor level. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	RTP Topology

|  | Property Name: | RTP Topology |
| --- | --- | --- |
|  | Property ID: | rtptopo (0x0001) |
|  | Description: | This property indicates which RTP topology (as primarily defined by [IETF RFC 5117]) that an RTP Stream is acting under. |
|  | Type: | Enumeration |
|  | Possible values: | MG (0x0000): MG Determined PP (0x0001): Topo-Point-to-Point (Note 1) MC (0x0002): Topo-Multicast TR (0x0003): Topo-Translator MX (0x0004): Topo-Mixer VS (0x0005): Topo-Video-Switch-MCU RT (0x0006): Topo-RTCP-terminating-MCU TA (0x0007): Topo-Asymmetric TF (0x0008): Topo-Transparent-Forwarding (Note 2) NOTE 1 – A pair of two connected Stream endpoints (between two RTP terminations within a context) and PP topology is synonymous with a Back-to-Back RTP End system (B2BRE) topology from the perspective of an end-to-end RTP session. NOTE 2 – An H.248 IP-IP MG will enforce transparent forwarding (see clause 3.2.10) behaviour. |
|  | Default: | MG |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error Codes

None.

## 10.6	Procedures

### 10.6.1	General

This package allows the MGC to indicate to the MG which particular RTP topology that an RTP stream is under acting under. The type of RTP topology may have impacts on various bearer level functions. It may have an impact on RTCP handling procedures.

In most cases a MG is able to determine itself the RTP topology that is used due to the H.248 Connection Model (i.e. the combination of Terminations/Streams and associated properties in a Context). For example: it can determine the difference between a point to point topology (i.e. a two Termination Context) versus a mixer (i.e. three or more Termination Context). Thus the MGC does not need to signal the "RTP Topology" (rtpt/topo) property.

However in some cases the MG is unable to distinguish between topologies. For example: in some network scenarios it may not be able to distinguish between point to point and translator topologies. This package may be used in scenarios where a more deterministic method is needed to indicate what the RTP topology of a RTP Stream is.

It is expected that Packages extending this package or profiles utilising this package specify what the function specific impact of the RTP Topology property would be.

### 10.6.2	MGC level information for RTP topology control

RTP topology control (at MGC level) could be based on

- call/session control signalling or/and

- local policies.

### 10.6.3	Topology dependent procedures

"MG Determined" indicates the MG shall determine RTP topology and subsequent RTCP handling based on the implicit information derived from the applicable H.248 descriptors. This behaviour may also be subject to profile specification.

#### 10.6.3.1	Topology "Point-to-Point"

The MG shall provide an own, local CNAME.

The MG shall allocate local SSRC values per RTP session endpoint (which corresponds to a H.248 flow pair of RTP media and RTCP control traffic; often identical to the H.248 Stream endpoint (SEP)).

The MG shall originate/terminate RTCP packets of type:

- at least: "SDES", "SR", "BR" and "BYE"

[…]

#### 10.6.3.2	Topology "Multicast"

Not used.

#### 10.6.3.3	Topology "Translator"

The MG may provide an own, local CNAME.

The MG shall not allocate local SSRC values for RTP traffic under this topology.

The MG may allocate local SSRC values for supplementary RTCP services.

The MG shall not originate/terminate RTCP packets of type:

- at least: "SR", "BR" and "BYE"

The MG may originate/terminate RTCP packets of type:

- e.g. "XR"

[…]

#### 10.6.3.4	Topology "Mixer"

Not used.

#### 10.6.3.5	Topology "Video-Switch-MCU"

Not used.

#### 10.6.3.6	Topology "RTCP-terminating-MCU"

Not used.

#### 10.6.3.7	Topology "Asymmetric"

Not used.

#### 10.6.3.8	Topology "Transparent Forwarding"

##### 10.6.3.8.1	Background to "Transparent Forwarding"

There are basically two H.248 signalling options in order to achieve a MG behaviour of "RTP transparent forwarding":

a)	Implicit signalling

Existing H.248 signalling using LD/RD settings with "identical media descriptions" (see examples in Appendix I). There are two terminations (and two associated stream endpoints), which implies synchronized settings between all four local and remote descriptors (in case of a bidirectional RTP session). The MG would provide a transparent forwarding behaviour (according clause 3.2.10) for all cases of unambiguous H.248 signalling.

b)	Explicit signalling

Explicit indication via rtpt/rtptopo property (see next sub-clause).

##### 10.6.3.8.2	Usage of rtpt/rtptopo property

The H.248 Stream Descriptor could contain RTP/RTCP related information (such as LD-/RD-embedded SDP media descriptions or other H.248 signalling elements related to RTP/RTCP; or LCD level information). Other descriptors could also contain RTP/RTCP related information (e.g. related to events, signals, statistics). Such kind of H.248 information could lead to the activation of RTP/RTCP processing functions in the MG or/and ambiguity concerning the aimed RTP topology behaviour.

Property value "TF" of (rtpt/topo) property shall be then used when the MG shall transparently forward RTP packets in order to guarantee deterministic traffic processing. The MG behaviour of RTP transparent forwarding (RTPTF) includes transparent forwarding (clause 3.2.10), but not fully protocol unaware forwarding (clause 3.2.2) due to basic RTP awareness (at least by usage of rtptopo property).

# 11	Package-independent procedures for RTP topology control

## 11.1	Topology “RTP source translator”

### 11.1.1	Via SDP attribute "a=ssrc:"

The MGC shall use the SDP attribute "a=ssrc:" (see [IETF RFC 5576]) for the RTP source translator assignment of a particular Stream endpoint. Table 5 defines the allowed roles (i.e. SDP a=ssrc: value settings), given by the 4-tuple {Ta(LD), Ta(RD), Tb(LD), Tb(RD)}.

Table 5 – Indication of topology “RTP source translator”

| H.248 SEP |  | SDP attribute | “RTP source translator” |
| --- | --- | --- | --- |
| Ta | LD | a= ssrc: | wildcarded |
|  | RD | a= ssrc: | not sent |
| Tb | LD | a= ssrc: | not sent |
|  | RD | a= ssrc: | not sent |
| NOTE 1 – The semantic of "not sent" means that the MGC shall not include this SDP attribute in the H.248 Descriptor, despite the fact whether it is used on call signalling level. |  |  |  |

The RTP source translator function is requested to translate the RTP source value with following semantic:

- direction Ta to Tb: not any translation of RTP source parameter values;

- direction Tb to Ta: the received SSRC value in any kind of RTP packets is replaced by the locally used SSRC value for this H.248 SEP;

The MG shall reply with error code 449 in case of incorrect value settings.

### 11.1.2	Via MIME subtype “rtx” in SDP attributes

The MIME subtype “rtx” is defined in [IETF RFC 4588]. The MGC may use this element in SDP attributes for triggering an RTP source translator function with regards to the translation of sequence number spaces.

However, this particular RTP source translator function is conditional, only required when the MG would provide parallel RTP interworking functions which are dependent on sequence number values (such as SRTP to non-SRTP interworking).

The RTP source translator function “sequence number translation” would be then enforced by a correspondent H.248 descriptor information.

Bibliography

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 6184]	IETF RFC 6184 (2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6190]	IETF RFC 6190 (2011), RTP Payload Format for Scalable Video Coding.

[b-IETF RFC 6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

[b-IETF RFC 6679]	IETF RFC 6679 (2012), Explicit Congestion Notification (ECN) for RTP over UDP.

[b-IETF rtp-howto]	IETF draft-ietf-payload-rtp-howto (2012), How to Write an RTP Payload Format.

[b-IETF rtp-topologies]	IETF draft-ietf-avtcore-rtp-topologies-update “RTP Topologies”

Appendix I
Examples of transparent forwarding

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction

The MG behaviour of “RTP transparent forwarding” may be enforced by either implicit or explicit H.248 signalling (see clause 10.6.3.8.1). The explicit method is semantically straightforward, whereas implicit signalling is often conditional. Some examples shall illustrate the uncontroversial and controversial usage of implicit signalling.

There is an (IP, IP) connection model in all examples. The signalling of a single stream endpoint is shown.

## I.2	Convention

The examples use an abstracted command notation, see Table I.1. There are three key areas of interest in such H.248 commands:

Table I.1 –Implicit signalling – abstracted H.248 command format 
with principal areas related to RTP

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …} | There are basically three areas in H.248 commands which could contain information indicating the existence of RTP bearer traffic: - SDP media descriptions embedded in LD/RD as part of the Stream Descriptor; relevant information would be primarily subject of the “m=” and “a=” lines; - H.248 properties in the LCD (again embedded in Stream Descriptor); - H.248 properties in other descriptors (e.g., Events Descriptor, Statistics Descriptor). |

## I.3	Examples for implicit signalling

The method of implicit signalling is described in clause 10.6.3.8.1 as option (a).

### I.3.1	Example 1 “UDP payload transparent forwarding”

Table I.2 illustrates “UDP payload transparent forwarding”:

Table I.2 – Example 1 “UDP payload transparent forwarding”

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/… Transaction = … { Context = … { Add = ip/$ ; Termination Ta, SEP S1 {Media { Stream = 1 { LocalControl { ... ; NOTE 1 } ... Local { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }…}, Add = ip/$ ; Termination Tb, SEP S1 {Media { Stream = 1 { LocalControl { ... } ... Local { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }…}, … | The RTP session shall be configured for transparent forwarding for both traffic directions. Prerequisites: - All four “m=” lines SHALL be identical. Example here: - Transport ‘UDP’ is indicated, which implies “RTP agnostic” to the MG. - The MG could be requested for local NAPT, which is a L4-dependent function (thus “UDP” indication). Conclusions: - This setting does not provide “UDP transparent forwarding” (which was also not requested) due to UDP checksum updates. - It leads rather to “UDP payload transparent forwarding” … - … which could be “RTP transparent forwarding” in case of RTP-over-UDP bearer traffic |

Discussion/conclusions of example 1:

- requested and actual forwarding mode (“UDP payload transparent forwarding”) are identical;

- implicit signalling is thus unambiguous.

### I.3.2	Example 2 “fully RTP protocol unaware forwarding”

Signalling according Table I.3 should lead to a MG behaviour of “fully RTP protocol unaware forwarding”:

Table I.3 – Example 2

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | The so called BGW mode of operation (in TISPAN BGF, 3GPP IMS-AGW, 3GPP TrGW, MSF SBG) with the characteristic of: - media type agnostic - media format agnostic - transport protocol type agnostic and - L4 port agnostic. The “-“ notation was introduced by TISPAN in border gateway profiles. |

Discussion/conclusions of example 2:

- The MG would implicitly provide “fully RTP protocol unaware forwarding” if there would be not any other signalling elements related to RTP.

- The MGC knows whether the implicit signalling would be unambiguous or not (due to the indicated condition). The MGC may therefore provide an additional explicit indication in case of guaranteed transparent forwarding service requirements.

### I.3.3	Example 3 “additional RTCP awareness”

Example 3: same as example 2, but with additional property “rtcph/rsb = ON” (see [ITU-T H.248.57]) in the LocalControl Descriptor (LCD)

Discussion/conclusions of example 3:

- That’s an explicit indication of RTP bearer traffic, the MG becomes RTP aware.

- However, there are basically three meaningful RTP translator options (PP, TR and TF) with different impact on RTP packets, thus pure implicit signalling would be controversial.

- Unambiguous behaviour might be achieved by explicit assignment of PP or TR topology (with different RTCP handling, but common SSRC/CSRC modifications) in case of non-transparent forwarding.

- Transparent forwarding however could not be achieved via implicit signalling, requires rather the explicit assignment of the TF topology.

### I.3.4	Example 4 “fully specified SDP media description”

Example 4: same semantic aimed as with example 2, but fully specified SDP media description, rather than the “-“ option, see Table I.4:

Table I.4 – Example 4

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …} | This option was added by 3GPP profiles (in comparison to TISPAN BGF). The MG has to compare all four SDP media descriptions, and if fully identical then to conclude a correspondent RTP topology behaviour. But which one exactly? |

Discussion/conclusions of example 4:

- This example should be functionally identical as example 2, i.e. leading to the same MG behaviour, but there are subtle difference between examples 2 and 4 when considering the dynamics of H.248 signalling.

- Problem: the final establishment of the H.248 stream might be subject of multiple signalling cycles (ADD, MOD, …). The MG becomes normally already RTP aware (even media aware) with the very first H.248 command request. But which topology? B2BRE, media translator, transport translator, transparent forwarding?

- The MG could of course wait till the stream is finally established, but that would be an unsatisfying situation for performance optimized implementations, which may want to achieve e.g. the minimization of call and bearer establishment delays.

- The explicit indication of the aimed RTP topology (MG, PP, TR or TF) in the first command request could solve the ambiguity.

### I.3.5	Example 5 “transport protocol translation for RTP bearer traffic”

Table I.5 illustrates the example RTP-over-UDP to RTP-over-TCP interworking (but could be also RTP-over-UDP to RTP-over-DCCP, etc.).

Table I.5 – Example 5

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> UDP - }, Remote { m=- <port> UDP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> TCP/RTP/AVP - }, Remote { m=- <port> TCP/RTP/AVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | Stream endpoint SEP Ta/S1 is L4+ agnostic, the MG would be RTP unaware. However, SEP Tb/S1 indicates RTP as L4+ protocol. The complete settings lead to the conclusion of transport protocol translation for RTP bearer traffic. |

Discussion/conclusions of example 5:

- That is a different signalling variant, but similar concerning conclusions as example 3.

- The MG would exclude the PP topology (because PP would be based on value “RTP/AVP” instead of “UDP” at Ta/S1 side.

- However, there are still two meaningful RTP translator options (TR and TF), thus, not unambiguous.

- Thus, guaranteed transparent forwarding would require rather the explicit assignment of the TF topology.

### I.3.6	Example 6 “transparent forwarding of encrypted RTP traffic”

This use case relates e.g. to so-called end-to-end media security scenarios where an IP-IP MG is located in the middle of the end-to-end SRTP-enabled RTP session. There are H.248 profiles which use the “m=” line transport codepoint “RTP/SAVP” for the indication of SRTP traffic to the MG, see Table I.6:

Table I.6 – Example 6

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | SRTP traffic is indicated by the “m=” line, in a media agnostic manner. The complete settings lead to the conclusion of transparent forwarding of SRTP bearer traffic. |

Discussion/conclusions of example 6:

- There could be an ambiguous situation without explicit RTP topology indication (e.g. MG could conclude B2BRE (= PP)), which would be controversial.

- Required would be a transparent forwarding behaviour, which would be satisfied by RTPTT and TF topologies.

- Solution: unambiguous semantic by explicit indication of rtpt/rtptopo = TF.

Appendix II
Living List related to RTP Topology Package

(This appendix does not form an integral part of this Recommendation)

{Editor's note: This Appendix is related to the RTP Topology Package (see clause 10):

- Level of RTP topology property: there might be following options

  - Context level (-> new Context Attribute)

          - Pro: RTP topology is a characteristic above the Stream level

          - Con: there might be different, Stream-dependent RTP topologies in case of multiple-Stream-per-Termination connection model

  - Topology descriptor  (-> syntactical or/and semantical extension)
=> not any concrete idea was presented

  - Stream level (-> new Property, see e.g. example below rtpt/rtptopo)

          - Pro: satisfies the requirement of  Stream level RTP topologies

          - Con: the MGC must ensure that all Stream endpoints, related to the RTP topology, must be configured for the same topology. Thus, any invalid signalling combination must rejected by an error code by the MG.

- Dynamic of RTP topology property value: there appears the question whether the RTP topology is a static characteristic over the lifetime of a Stream, or whether the RTP topology could even dynamically change? Some possible use cases:

          - Scenario 5.1: a Context is created with Terminations T1 (RTP) and T2 (Phy). T1 is thus in PP topology. Later on T2 is replaced (by SUBtract and MODify) by T3 (RTP), and T1 and T3 should now be connected in TR topology.
=> Transient RTP topology of T1 (PP -> TR).

          - Scenario 5.2: to a Context with two RTP Terminations (T1, T2)  in RTP Media Translator topology would be later a third RTP termination (T3) added, and the media processing should be done in "mixing mode", thus a MX topology. 
=> again transient RTP topology of T1 and  (TR -> MX).

- Impact of IETF draft-ietf-avtcore-rtp-topologies-update-00 “RTP Topologies” ([b-IETF rtp-topologies])? As revision of RFC 5117.

Contributions are solicited.}

Appendix III
Living List related to H.248.77 related RTP topologies

(This appendix does not form an integral part of this Recommendation)

{Editor's note (2013-01 meeting):

Purpose: this Appendix parks potential revision material for H.248.77 as long as evaluation still ongoing.

Ref: T13-SG16-C-0050

Initial proposal plus possible solution (in yellow highlighted), see below:

Annex A

Profile guidelines for (IP, IP) connection models

(This annex forms an integral part of this Recommendation.)

## A.1	Motivation

The main body of this Recommendation is considering a connection model with scope on a single termination only, see clause 1.1. However, some SRTP related interworking functions by H.248 IP-to-IP media gateways require the consideration of the overall stream endpoint pair (SEPP). Such use cases are outlined e.g. in Appendix II.2 and II.3.

Purpose of this Annex is the description of high level profile specification guidelines. Such guidelines are typically “SRTP interworking” specific, see following clauses.

## A.2	Profiles with support of "SRTP to non-SRTP interworking with local SSRC change"

This interworking scenario implies the MG local termination/origination of SRTP. The SRTP encryption/decryption function is dependent on RTP source parameters SSRC and SN values. The H.248 SRTP SEP must be therefore a local RTP source/sink entity with it’s own SSRC as identifier (due to SRTP cryptographic context). Following RTP topologies might be applicable.

## A.2.1	Back-to-Back RTP End system (B2BRE) topology

The resource management of RTP source related parameters would be an MG internal function; MG assigned RTP source parameter values would not be reported to the MGC.

## A.2.2	RTP source translator (RTPST) topology

The assignment of local SSRC values is either provided by the MGC or the MG. Such a MG level resource management function should be triggered via SDP wildcarding, see clause 6.15.9/[ITU-T H.248.39].

## A.3	Profiles with support of "SRTP to SRTP interworking"

This interworking scenario is outlined in Appendix II.3. The MG shall not change RTP source parameters SSRC and SN, which would violate SRTP encryption/decryption. Such behaviour would be achieved by “RTP transparent forwarding” topology, see [ITU-T H.248.RTPTOPO].

Discussion: reinserted again the conclusions from older H.248.RTPTOPO drafts plus additional comments:

## II.1	Use-case #1: ITU-T H.248 MG for peering IP and circuit-switched networks

[Comment (from H.248.RTPTOPO perspective]:

This use case is clear, related to the RTPE topology only.

## II.2	Use case #2: ITU-T H.248 MG for peering IP networks with SRTP to RTP

[Comments (from H.248.RTPTOPO perspective]:

H.248.77 indicates the B2BRE topology option. This topology model is believed to be the right one due to the SRTP concept of a cryptographic context (cl. 3.2.3/[IETF RFC 3611]). The 3-tuple identifier of the cryptographic context includes the SSRC element. This concludes that the SRTP-to-RTP interworking function by a H.248.77-capable MG implies also the change of the SSRC value. And the change of SSRC identifiers excludes other RTP topologies like e.g. RTPTT (due to maintained SSRC).

It should be also noted that the SSRC value is part of the key generation algorithm.

It may be concluded that the H.248 SRTP SEP shall be a RTP source/sink with it’s own SSRC as identifier. RTP topologies which does not provide that capability are therefore excluded.

## II.3	Use case #3: Transparent SRTP forwarding

[Comments (from H.248.RTPTOPO perspective]:

H.248.77 indicates a transparent forwarding topology option. 
=> this use case was solved (2012-09) with the transparent forwarding related text.

____________________

OSN: original sequence number