| Rapporteur Meeting of Questions 1,2, 3, 5 and 21/16 |  |  |  | TD-24 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2003-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPMUX |  |  |  |
| Title: | H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New): Output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Rapporteur’s output draft for draft new H.248. RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing”.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPMUX Ed. 0.1 | Initial baseline output (TD 59 (WP1/16)) from ITU-T SG16 Geneva meeting, Jan 2013. Based on C.34, C.35, C.41and C.53. Location: http://www.itu.int/md/T13-SG16-130114-TD-WP1-0056/en |
| H.248.RTPMUX Ed. 0.2 | Input draft to Oslo, 17 - 21 June 2013 meeting. Based on TD 56 (WP1/16). |
| H.248.RTPMUX Ed. 0.3 | Output (TD-24) from ITU-T SG16 Oslo meeting, Jun 2013. Based on AVD-4415, AVD-4450 and AVD-4459. Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-24.zip |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 7 6 Overview of RTP multiplexing 7 7 Use cases with RTP multiplexing 7 8 H.248 Native Control of RTP multiplexing 7 8.1 Assigning a multiplexed RTP stream 7 8.2 Modification of the multiplex 9 8.3 Scope of Signals / Events and Statistics 9 9 Enhanced control of RTP Multiplexing 9 10 Media Grouping package 10 10.1 Properties 10 10.1.1 Group Semantics 10 10.2 Events 11 10.3 Signals 11 10.4 Statistics 11 10.5 Error Codes 11 10.6 Procedures 11 10.6.1 Mapping of group attributes in H.248 11 10.6.2 Error handling 12 10.6.3 Usage in RTP Multiplexing 12 Appendix I Existing H.248 support for RTP multiplexing 14 I.1 3GPP-defined multiplexing for IP bearers with 3GPP framing protocol 14 Appendix II Living List related to IETF documents under observation 15 II.1 Important references 15 II.2 Important terminology 16

List of Tables

Page

Table 1 – Naming of multiplexing options related to transport and media multiplexing 5

List of Figures

Page

Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”) 12

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPMUX)

Gateway Control Protocol: 
H.248 support for RTP multiplexing

AAP Summary

<Mandatory material to be provided before Consent

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

1	Scope

{Editor’s note (2013-06): the package proposal isn’t yet complete, the current Media Grouping package in sub-clause 10 is only giving a general way to indicate the group characteristics of multiple media streams, however the following aspects are identified during the meeting for further investigation .

o RTCP handling needs to be considered and how it relates to bundling.

o Explicit port handling considering (i.e. H.248.57) needs to be considered.

o ReserveGroup do we allow or disallow grouping in grouped streams.

o IP endpoint transport multiplexing. Is it only IP or other transports?

o The general grouping package is contained in RTP-MUX however it may form its own Recommendation depending on progress.

Contributions are solicited.}

2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3

[IETF RFC 4566]	IETF RFC 4566 (07/2006), SDP: Session Description Protocol

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework

3	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.y1	(RTP) endpoint [IETF XYZ]: FIXTHIS

3.1.y2	RTP endsystem [IETF XYZ]: FIXTHIS

3.1.y3	media source [IETF XYZ]: FIXTHIS

+ media sink

3.1.y4	(H.248) media stream [ITU-T H.248.1]: FIXTHIS

3.1.y5	(RTP) media format [IETF XYZ]: FIXTHIS

3.1.y6	(RTP) media stream [IETF XYZ]: FIXTHIS

3.1.y7	(RTP) media type [IETF XYZ]: FIXTHIS

3.1.y8	media track [IETF XYZ]: FIXTHIS

3.1.y9	RTP source [IETF XYZ]: FIXTHIS

3.1.y10	RTP session [IETF XYZ]: FIXTHIS

3.1.y11	RTP session group [IETF XYZ]: FIXTHIS

3.1.y12	(RTP) endpoint [IETF XYZ]: FIXTHIS

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

{Editor’s note (2013-01 meeting): all below listed terminology proposals are just working assumptions and subject of change, primarily dependent on the work progress in IETF on RTP multiplexing models and RTP terminology.}

3.2.x1	RTP media multiplexing: for further studies

3.2.x2	RTP transport multiplexing: a single IP transport (L4) port for RTP and RTCP packets

3.3.x3	RTP multiplexing levels: protocol layer related multiplexing options transport (see clause 3.2.x2) and media (see clause 3.2.x1) multiplexing could be applied in each combination. Table 1 defines a naming scheme:

Table 1 – Naming of multiplexing options related to transport and media multiplexing

| Multiplexing options | Transport (NOTE 1) | Media (NOTE 2) | Name |
| --- | --- | --- | --- |
| a) | No | No | Fully unmultiplexed |
| b) | No | Yes | Media type multiplexed |
| c) | Yes | No | RTP/RTCP multiplexed |
| d) | Yes | Yes | Fully multiplexed |
| NOTE 1 – Single L4 port for RTP media flow(s) and RTCP control flow(s). NOTE 2 – Single L4 port for multiple RTP sources. There’s a single SSRC per RTP source. |  |  |  |

4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| IP | Internet protocol |
| --- | --- |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| RD | Remote descriptor |
| RTCP |  |
| RTP |  |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

5	Conventions

None.

6	Overview of RTP multiplexing

FIXTHIS

7	Use cases with RTP multiplexing

FIXTHIS

8	H.248 Native Control of RTP multiplexing

8.1	Assigning a multiplexed RTP stream

Typically a single RTP stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. To indicate the use of a multiplexed RTP stream the MGC shall request the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed RTP Stream.

For text based H.248 SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Add.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Table 1 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Table 1 / ITU-T H.248.RTPMUX– Example single and multiplexed RTP streams.

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RFC xxxx].

{Editor’s note: a specific reference to the RTP multiplexing procedures as defined by the IETF AVT and MMUSIC groups is expected to be added.}

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

If a MG does not support RTP multiplexing it should respond with error code “473- Conflicting property values”.

8.2	Modification of the multiplex

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed RTP stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own RTP stream.

8.3	Scope of Signals / Events and Statistics

Signals, Events and Statistics may be set at either a Termination or Stream level.  Generally when set at a Termination level the scope of this elements applies to all of the H.248 Streams on the Termination. For example: the octets sent statistic would be the sum of octets sent by each H.248 Stream and by corollary each RTP stream.

When these elements are set at a Stream level the scope of the element is given by the StreamID. In the case of the octets sent statistic it would contain only the octets sent by the H.248 Stream given by the StreamID.

When using multiplexed RTP Streams the above behaviour should be considered. Multiplexed RTP Streams may use a sub-set of the total H.248 Streams in a Termination. For example: StreamID(1) could be a single stream, StreamIDs(2&3) could denote a multiplexed RTP Stream. This leads to complications regarding the scope of how Signals, Events and Statistics are applied. For example: A Termination level octets sent Statistic would correctly give the number of octets sent by the Termination however there is no way to request an individual octets sent Statistic related to the multiplexed RTP stream. The MGC would have to request a statistic for StreamID(2) and another statistic for StreamID(3) and add them together.

This issue is also applicable to Signals and Events. This is no way to indicate that a Signal or Event is only applicable to a particular multiplexed RTP Stream. In order to be unambiguous as to the scope of the Signal or Event a specific StreamID in the bundled group should be used.

{Editor’s note:  It needs further analysis whether new information elements are needed to support such behaviour. This may also be applicable for the other grouping semantics, not only to bundle. Further contributions are solicited on the above issues.}

9	Enhanced control of RTP Multiplexing

As described above it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG support RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the packages descriptor in order to determine if the “Multiplexing package” (clause 10) is present. If present the MGC can request that multiple H.248 Streams use the same address/port information.

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The Multiplexing package contains the “association” property that allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the multiplexing package should check the “association” property when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by the “association” property then the same value should be returned for those H.248 Streams.

10	Media Grouping package

|  | Package Name: | Media Grouping |
| --- | --- | --- |
|  | Package ID: | mgroup, <0x????> > |
|  | Description: | This package provides a general way for the MGC to instruct the MG to group media streams defined in different stream descriptors within the same media descriptor. |
|  | Version: | 1 |
|  | Extends: | None |

10.1	Properties

10.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse, (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example: providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], with the index of media streams using type “StreamIDList”: StreamIDList = StreamID *1(LWSP StreamID) StreamID = UINT16 As defined by [ITU-T H.248.1 Annex B]. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

{Editor’s note:  It needs further analysis whether different groupings may apply for received and sent media. It may require a split of the property in two.}

10.2	Events

None.

10.3	Signals

None.

10.4	Statistics

None.

10.5	Error Codes

None.

10.6	Procedures

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping.

An MGC controlling a MG that supports the SDP Media Grouping package can instruct the MG to group two or more media streams. Each media stream is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which media streams to group

- For which purpose the media streams are grouped

10.6.1	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID is used for the index in the group

- The “a=group” attribute is mapped to the mgroup/groupse property and included in the TerminationState Descriptor. The content of the property, a sub-list of string, should be follow a copy of the “a=group” attribute that replacing the mid value with the corresponding StreamID.

NOTE: The identification tags included in the mgroup/groupse property should match the ones sent in the Local or Remote descriptor to the MG.

Example:

An MGC receives an SDP Offer with the following media description:

v=0

o=Alice 289083124 2890844526 IN IP4 124.124.124.222

c=IN IP4 124.124.124.222

a=group:LS 1 2

m=audio 30002 RTP/AVP 97

a=mid 1

a=rtpmap:97 AMR/8000

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \

mode-change-neighbor = 1; mode-change-capability=2; max-red=0

m=video 52738 RTP/AVP 99

a=mid 2

b=TIAS:256000

b=AS:270

a=rtpmap:99 H264/90000

a=fmtp:99 profile-level-id=428014;

a=sendrecv

It describes a session containing one video stream and one audio stream. They use the “LS” [IETF RF 5888] grouping semantics to indicate that the play-out of both media must be synchronized.

The MGC maps this description into the following Media Description

Media { 
  TerminationState { mgroup/groupse = "LS 1 2"},

Stream = 1 {

Remote {

c=IN IP4 124.124.124.222

m=audio 30002 RTP/AVP 97

a=rtpmap:97 AMR/8000

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \

mode-change-neighbor = 1; mode-change-capability=2; max-red=0

}

},

Stream = 2 {
     Remote {

c=IN IP4 124.124.124.222

m=video 52738 RTP/AVP 99

b=TIAS:256000

b=AS:270

a=rtpmap:99 H264/90000

a=fmtp:99 profile-level-id=428014;

a=sendrecv

}

}

}

Note: A similar mapping would apply to the Local Descriptor. The Local Descriptor is not shown but it is also part of the Media Descriptor and will be sent to the MG.

{Editor’s note: it is not excluded that some specific grouping semantics will require, or at least benefit, from further procedural descriptions, or even new protocol elements.}

10.6.2	Error handling

If there is a mismatch between the mgroup/groupse property and the Local or Remote Descriptor then error code 473 “Conflicting Property Values” should be returned. For example: if the MGC specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command).

{Editor’s note:  Error handling may be dependent on grouping semantics, i.e. an address error wouldn't be generated for Lip sync.}

10.6.3	Usage in RTP Multiplexing

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams (See clause 8).

Additionally the MGC may set the “association” (mgroup/groupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

For example:

An MGC may require a Termination (T1) with five Streams. StreamID(1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. It is assumed that the values of StreamID are already allocated before the following processing.

In the first example the mgroup/groupse property indicates that the video streams are associated:

mgroup/groupse = “BUNDLE 3 4 5”

In the second example the mgroup/groupse property indicates that the audio streams are associated together and the video streams are associated together.

mgroup/groupse = “BUNDLE 1 2”, “BUNDLE 3 4 5”

When adding or removing multiplexed H.248 Streams the MGC shall include an updated mgroup/groupse property in the same command to ensure consistency between Local and Remote Descriptor settings and the property.  For example: when adding a new H.248 Stream (StreamID 6) to an existing multiplex of StreamIDs 3, 4 & 5.

MEGACO/3 [123.123.123.4]:55555

Transaction = 10004 {

Context = 1 {

Add = T1 {

Media {

TermationState { mgroup/groupse = “BUNDLE 3 4 5 6”},

Stream = 6 {

Local {

v=0

c=IN IP4 $

m=video $ RTP/AVP 34

}

}

}

}

}

}

On reception of the mgroup/groupse property the MG shall assign resources in the knowledge that each of the StreamList element contains StreamIDs that are associated with one multiplexed stream. For example: if the MG receives a request to add a H.248 stream with a wildcarded address/port it should return the same as the other H.248 Streams associated with the multiplex.

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

Existing H.248 support for RTP multiplexing

(This appendix does not form an integral part of this Recommendation)

I.1	3GPP-defined multiplexing for IP bearers with 3GPP framing protocol

Reference: see clause 6.4/[ETSI TS 129.414].

Motivation: transport capacity reductions (“bandwidth efficiency”).

Multiplexing method: multiple RTP packets mapped into a single PDU augmented by a (3GPP defined) multiplex header (“similar to IETF SHIM approach").

H.248 control: not any signaling support, thus pure provisioning based approach.

Appendix II
Living List related to IETF documents under observation

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix would be again deleted as soon as the reference (I.1) and terminology (I.2) discussion becomes stable).

II.1	Important references

Following IETF documents (from four IETF working groups) are relevant to the RTP multiplexing discussion:

[1]	A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources
IETF “RAI Areaa” / WG AVTEXT? (still open)
Draft: draft-lennox-raiarea-rtp-grouping-taxonomy
URL: http://tools.ietf.org/html/draft-lennox-raiarea-rtp-grouping-taxonomy-00

[2]	Guidelines for using the Multiplexing Features of RTP
IETF AVTCORE
Draft: draft-westerlund-avtcore-multiplex-architecture 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multiplex-architecture-02.txt

[3]	Multiplexing RTP Data and Control Packets on a Single Port
IETF MMUSIC
RFC 5761

[4]	“BUNDLE”
= Multiplexing Negotiation Using SDP Port Numbers 
IETF MMUSIC
Draft: draft-ietf-mmusic-sdp-bundle-negotiation
URL: http://tools.ietf.org/wg/mmusic/draft-ietf-mmusic-sdp-bundle-negotiation/

[5]	“RTP for WebRTC”
 = Web Real-Time Communication (WebRTC): Media Transport and Use of RTP
IETF RTCWEB
Draft: draft-ietf-rtcweb-rtp-usage
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

[6]	“Multi-Media Concepts and Relations” 
IETF RTCWEB
Draft: draft-burman-rtcweb-mmusic-media-structure
URL: http://tools.ietf.org/id/draft-burman-rtcweb-mmusic-media-structure-00.txt

[7]	“RTP for Telepresence”
 = RTP Usage for Telepresence Sessions
IETF CLUE
Draft: draft-lennox-clue-rtp-usage
URL: http://tools.ietf.org/id/draft-lennox-clue-rtp-usage-04.txt /

Complementary:

[8]	Source-Specific Media Attributes
IETF MMUSIC
RFC 5576

9]	SDP Grouping Framework
IETF MMUSIC
RFC 5888

[10]	Media multiplexing with RTP subsessions
IETF AVTCORE
Draft: draft-ejzak-avtcore-rtp-subsessions
URL: http://tools.ietf.org/id/draft-ejzak-avtcore-rtp-subsessions-01.txt

[11]	Multiple RTP Sessions on a Single Lower-Layer Transport
IETF AVTCORE
Draft: draft-westerlund-avtcore-transport-multiplexing 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-transport-multiplexing-03.txt

[12]	Multiple Media Types in an RTP Session
IETF AVTCORE
Draft: draft-westerlund-avtcore-multi-media-rtp-session
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multi-media-rtp-session-00.txt

[13]	RTP Considerations for Endpoints Sending Multiple Media Streams
IETF AVTCORE
Draft: draft-lennox-avtcore-rtp-multi-stream
URL: http://tools.ietf.org/id/draft-lennox-avtcore-rtp-multi-stream-00.txt

[14]	Using Simulcast in RTP sessions
IETF AVTCORE
Draft: draft-westerlund-avtcore-rtp-simulcast 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-rtp-simulcast-01.txt

[15]	Cross Session Stream Identification in the SDP
IETF RTCWEB
Draft: draft-alvestrand-rtcweb-msid
URL: http://tools.ietf.org/id/draft-alvestrand-rtcweb-msid-02.txt

[16]	H.248.1v4 Living List: New Appendix V – H.248 Streams “Examples for valid media format suites”
ITU-T SG16
Draft: AVD-4241
URL: see http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/1209_Bri.html

II.2	Important terminology

Present work on multiplexed RTP did lead to a revision, modification, extension till the semantical overloading of some RTP related terms. Primarily the terms “media stream” and “RTP session” are used in many slightly different flavours. Following definitions from [1] might be quite useful (“despite the fact that they are not yet fully unambiguous”):

All that terms are either new or extensions/revisions of RFC 1889/3550 terms:

Endpoint:  A single entity sending or receiving RTP packets.  It may

be decomposed into several functional blocks, but as long as it

behaves a single RTP stack entity it is classified as a single

endpoint.

Comment: an endpoint relates to RTP topology “RTP endsystem” in our understanding (see RFC 5117, H.248.RTPTOPO).

Media Source:  The source of a stream of data of one Media Type, It

can either be a single media capturing device such as a video

camera, a microphone, or a specific output of a media production

function, such as an audio mixer, or some video editing function.

Sending data from a Media Source may cause multiple RTP sources to

send multiple Media Streams.

Media Stream:  A sequence of RTP packets using a single SSRC that

together carries part or all of the content of a specific Media

Type from a specific sender source within a given RTP session.

Comments:

- we need to distinguish between “IETF media stream” and “H.248 media stream”

- 1:1 relationship between “media stream” and “SSRC” due to “single SSRC”, right?

- May use different media streams the same SSRC value? The definition allows both interpretations …

RTP Source:  The originator or source of a particular Media Stream.

Identified using an SSRC in a particular RTP session.  An RTP

source is the source of a single media stream, and is associated

with a single endpoint and a single Media Source.  An RTP Source

is just called a Source in RFC 3550.

Comments:

- 1:1 relationship between “RTP source” and “SSRC”, right?

- May use different RTP sources the same SSRC value? The definition allows both interpretations …

Media Sink:  A recipient of a Media Stream.  The endpoint sinking

media are Identified using one or more SSRCs.  There may be more

than one Media Sink for one RTP source.

Multiplex:  The operation of taking multiple entities as input,

aggregating them onto some common resource while keeping the

individual entities addressable such that they can later be fully

and unambiguously separated (de-multiplexed) again.

RTP Session:  As defined by [RFC3550], the endpoints belonging to the

same RTP Session are those that share a single SSRC space.  That

is, those endpoints can see an SSRC identifier transmitted by any

one of the other endpoints.  An endpoint can receive an SSRC

either as SSRC or as CSRC in RTP and RTCP packets.  Thus, the RTP

Session scope is decided by the endpoints' network interconnection

topology, in combination with RTP and RTCP forwarding strategies

deployed by endpoints and any interconnecting middle nodes.

Comment: this is an extension of the RFC 3550 RTP session concept in our understanding. The RTP session is NOT any global concept across all involved RTP entities (such as endsystems, mixers, translators) of a particular communication application.
The RTP session is rather a local concept, given by the perspective of a particular endpoint.

Examples: a two-party call (= point to point topology) between A and B, and with different multiplexing models of each endpoint may lead to the effect that e.g. A perceives a single RTP session whereas B observes multiple RTP sessions for the same communication service.

Note: we may agree to such an “RTP session” concept (given that our interpretation is inline with [1]).

RTP Session Group:  One or more RTP sessions that are used together

to perform some function.  Examples are multiple RTP sessions used

to carry different layers of a layered encoding.  In an RTP

Session Group, CNAMEs are assumed to be valid across all RTP

sessions, and designate synchronisation contexts that can cross

RTP sessions.

Source:  Term that should not be used alone.  An RTP Source, as

identified by its SSRC, is the source of a single Media Stream; a

Media Source can be the source of mutiple Media Streams.

SSRC:  An RTP 32-bit unsigned integer used as identifier for a RTP

Source.

(tbd: The terms "SSRC multiplexing" and "session multiplexing" are

confusing, with unclear historical meanings; they need to be removed

from this document in the interests of clarity)

It might be beneficial to illustrate the multiplexing hierarchy of the defined terminology by [1] (Figure I.1 illustrates the traffic source side only):

Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”)

NOTE: The terminology in [1] should be further improved. There are  already a couple of different interpretations.

Importation clarifications required (by IETF):

- Exact relationship between entities (e.g., 1:1, 1:N, etc)

- Exact identifier of an entity („are there any identifiers, and if, which one, given by an n-tuple of …“)

It could be concluded that the evaluation of RTP multiplexing model demands for refined RTP terminology as originally introduced by RFC 1889, but the terminology update work by IETF is still not consistent and stable.

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

_______________________