| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28 Oct – 8 Nov 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): C |  |  |
| Source: | Huawei Technologies Co., Ltd |  |  |  |  |  |
| Title: | F/H.TPS-Arch: Updates with respect to furniture in Telepresence rooms |  |  |  |  |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Pty. Ltd. Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: christian.groves@nteczone.com |  |
| Contact: | Jiaoli Wu Huawei Technologies China |  |  |  | Tel: +86 755 36830154 Fax: +86 755 36830131 Email: wujiaoli@huawei.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

1.	Introduction

This Contribution proposes updated texts to F/H.TPS-Arch in order to address some concerns on the design for the furniture in the Telepresence rooms.

2.	Discussion

Q5/16 held an interim in Oslo to proceed with the current work items. In section 10.2 of FH.TPS-Arch, we talk about room characteristics. But right now, we still lack contents for the rest of this sub-section, the furniture part, which is currently noted by the Editor as for further contribution.

After studying the telepresence room, a couple of characteristics were found. As depicted through figure 17 to figure 19 in sub-section 10.1 of FH.TPS-Arch, the light paths from different cameras in the same room are not parallel with each other, nor are the screens in the same room. As a result, the tables and chairs are arranged in a way that in normal position each participant falls in the capture range of only one camera, and looks into that camera. The shape, or curve of the tables, should be in accordance with the arrangement of cameras and screens, to achieve a desirable experience.

It is suggested to place furniture that have a shape or curve like the table to have consistency in the whole Telepresence room. Also, heavy texture or complicated pattern of the furniture may make the video coding more complex and the system less effective. So it is suggested to have furniture beyond capture areas, and have clean and plain surfaced furniture with conforming curve within capture areas. Decorations are also suggested to be located out of capture areas.

Furniture affects the acoustic properties of the room, in terms of sound reflection and change of path. Furniture affects the lighting properties of the room, in terms of light reflection. To provide better experience, these factors should be considered to adjust changes introduced to the Telepresence room.

3.	Proposal

It is proposed to add the following texts to F/H.TPS-Arch. Changes are marked up against TD-22a from the ITU-T Q5/16 Oslo meeting.

Attachment 1

10.2.3	Furniture

In a Telepresence room, the tables and chairs are arranged in a way that each participant in normal seating position falls in the capture range of only one camera, and looks into that camera. The curve of the tables should be in accordance with the arrangement of cameras and screens, to achieve a desirable experience.

It is suggested furniture captured by cameras should have a shape or curve like the table to have consistency in the whole Telepresence room. Also, furniture with heavy texture or complicated pattern may make the video coding more complex and the system less effective. It is suggested to place any additional furniture beyond capture areas, and have clean and plain surfaced furniture with curves conforming to the capture areas. Decorations and adornments can be used to provide a sensation of depth, but their patterns and colours should be chosen carefully, to avoid problems as described in the above section.

Furniture affects the acoustic and lighting properties of the room, in terms of sound reflection and light reflection, respectively. To provide a better experience, these factors should be considered when introducing changes to a Telepresence room.

In order to provide a more natural experience, peripheral equipments, such as computer cases, wireless routers, server cabinets, are suggested to be concealed in large-sized furniture (desk, cupboard, etc.), or integrated into the framework for screens and cameras . It is suggested that cables be hidden from the cameras, and best embedded underneath the floor or rug. Table-mounted microphones are suggested to be flat (not with gooseneck), and it is suggested to hang ceiling-mounted microphones above the upper-boundary of captures.