| Question(s): | 3 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | H.248.RTPMUX: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to draft new H.248.RTPMUX “H.248 support for RTP multiplexing”.

# Discussion

At the Olso, June ITU-T Q.3/16 meeting a baseline was produced for draft new Recommendation ITU-T H.248.RTPMUX on “H.248 support for RTP multiplexing. An editor’s note was added to clause 1 “Scope” indicating that the proposal was not complete and there were further aspects for investigation. Additional editor’s notes were added throughout the text.

This contribution discusses a number of the editor’s notes and proposes updates to address them. Discussion is contained in contributor’s notes below.

# Proposal

It is proposed to accept the following updates. Changes are marked up against TD-24 from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

__________________

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPMUX)

Gateway Control Protocol: 
H.248 support for RTP multiplexing

…

# 1	Scope

{Editor’s note (2013-06): the package proposal isn’t yet complete, the current Media Grouping package in sub-clause 10 is only giving a general way to indicate the group characteristics of multiple media streams, however the following aspects are identified during the meeting for further investigation .

o RTCP handling needs to be considered and how it relates to bundling.

o Explicit port handling considering (i.e. H.248.57) needs to be considered.

o ReserveGroup do we allow or disallow grouping in grouped streams.

{Contributor’s note: In general this should be possible as the grouping concept is generic. For example: if the MG switches between media in a ReserveGroup it may or may not be able to apply lip-sync. With “bundle” a ReserveGroup should also be able to be used but each m= in the ReserveGroup should follow the bundle procedures. It isn’t be possible to have media (m= lines) in both applying to bundled and non-bundled transport. The media grouping property is at stream level not “Reserve group level (i.e. not in the m=line)”.It is proposed to add text to section 10.6 to address this.

o IP endpoint transport multiplexing. Is it only IP or other transports?

{Contributor’s note: The contributors have checked with BUNDLE author to confirm the scope of BUNDLE with respect to transports. The author indicated that BUNDLE should be a general mechanism but the scope of the description of the draft was only IP. If BUNDLE was to apply to other transports this would have to be documented as part of a separate RFC. It is proposed to add text to clause 10.6.2.9 indicating that the current scope of BUNDLE is for IP transports only.}

o The general grouping package is contained in RTP-MUX however it may form its own Recommendation depending on progress.

{Contributor’s note: The draft Recommendation may logically read better if the information regarding RTP multiplexing is moved to the end of the Recommendation. RTP multiplexing only relates to one of the possible semantics (i.e. bundle) associated with grouping. Thus it is proposed to move the clause “Usage in RTP Multiplexing” to a new clause 11.}

Contributions are solicited.}

…

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| IP | Internet protocol |
| --- | --- |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| RD | Remote descriptor |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

…

## 8.1	Assigning a multiplexed RTP stream

Typically a single RTP stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. To indicate the use of a multiplexed RTP stream the MGC shall request the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed RTP Stream.

For text based H.248 SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or {Contributor’s note: Mod.req commands are used to add additional streams to existing terminations.}

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

…

# 9	Enhanced control of RTP Multiplexing

{Contributor’s note: it is proposed to move this text into the grouping package where bundling is described.}

# 10	Media Grouping package

|  | Package Name: | Media Grouping |
| --- | --- | --- |
|  | Package ID: | mgroup, <0x????> > |
|  | Description: | This package provides a general way for the MGC to instruct the MG to group media streams defined in different stream descriptors within the same media descriptor. |
|  | Version: | 1 |
|  | Extends: | None |

## 10.1	Properties

### 10.1.1	Sent Stream Group Semantics

|  | Property Name: | Sent Group Semantics |
| --- | --- | --- |
|  | Property ID: | sgroupse, (0x0001) |
|  | Description: | This property allows a MGC to indicate which sent media streams (i.e. those described by the Remote Descriptor) to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example: providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. Several sets of sent stream groups are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics [LWSP StreamIDList] ; (NOTE 1) StreamIDList = StreamID *1(LWSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) As defined by [ITU-T H.248.1 Annex B]. Note 1: The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888]. {Contributor’s note: It is proposed to update the syntax to make the use of the semantic token clearer.} |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

{Contributor’s note: The contributors agree that the indication of the group semantics needs to be directional. The original specification of the group attribute [IETF RFC 5888] allowed both sides of a SDP exchange (i.e. send and received media) to specify the attribute. In order to provide the same functionality the group semantics properties need to be set for send and receive directions. It is proposed to split the existing property into two properties. One for the send and receive directions. This approach is proposed as it keep the Group from the send and received SIP Offers separate.}

### 10.1.2	Received Stream Group Semantics

|  | Property Name: | Received Stream Group Semantics |
| --- | --- | --- |
|  | Property ID: | rgroupse, (0x0002) |
|  | Description: | This property allows a MGC to indicate which received media streams (i.e. those described by the Local Descriptor) to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. Several sets of received stream groups are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | As per clause 10.1.1. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

…

## 10.6	Procedures

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the sgroupse and/or rgroupse properties to determine which semantics are supported by the MG. In this case the MG shall respond with the supported semantic tags and the StreamIDs set to 0.

{Contributor’s note: It is proposed to add the above text to clarify the use of the AuditCapability request.}

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to group

- For which purpose the media streams are grouped.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

### 10.6.1	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID is used for the index in the group

- The “a=group” attribute is mapped to the mgroup/sgroupse or the mgroup/rgroupse property and included in the TerminationState Descriptor depending whether the group relates to sent or received media. The content of the property, a sub-list of string, should follow the “a=group” attribute. The semantic is retained and the mid value is replaced with the corresponding StreamID.

NOTE: The identification tags included in the mgroup/sgroupse property should match the ones sent in the Remote descriptor and the mgroup/rgroupse should match the ones sent in the Local Descriptor to the MG.

Example:

An MGC wishes to send an SDP Offer with the following media description:

v=0

o=Alice 289083124 2890844526 IN IP4 124.124.124.222

c=IN IP4 124.124.124.222

a=group:LS 1 2

m=audio 30002 RTP/AVP 97

a=mid 1

a=rtpmap:97 AMR/8000

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \

mode-change-neighbor = 1; mode-change-capability=2; max-red=0

m=video 52738 RTP/AVP 99

a=mid 2

b=TIAS:256000

b=AS:270

a=rtpmap:99 H264/90000

a=fmtp:99 profile-level-id=428014;

a=sendrecv

It describes a session containing one video stream and one audio stream. They use the “LS” [IETF RF 5888] grouping semantics to indicate that the play-out of both media must be synchronized.

The MGC maps this description into the following Media Description

Media { 
  TerminationState { mgroup/sgroupse = "LS 1 2"},

Stream = 1 {

Remote {

c=IN IP4 124.124.124.222

m=audio 30002 RTP/AVP 97

a=rtpmap:97 AMR/8000

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \

mode-change-neighbor = 1; mode-change-capability=2; max-red=0

}

},

Stream = 2 {
     Remote {

c=IN IP4 124.124.124.222

m=video 52738 RTP/AVP 99

b=TIAS:256000

b=AS:270

a=rtpmap:99 H264/90000

a=fmtp:99 profile-level-id=428014;

a=sendrecv

}

}

}

Note: A similar mapping would apply to the Local Descriptor. The Local Descriptor is not shown but it is also part of the Media Descriptor and will be sent to the MG.

{Contributor’s note: The contributors have reviewed the currently IANA registered semantic tags. No need for additional H.248 protocol elements was identified. However several issues related to the implementation of these semantics via H.248 were noted. It is proposed to capture these issues as per the clauses below.}

### 10.6.2	MG Behaviour

### The behaviour of the MG with respect to the grouping is largely dependent on the semantic associated with the grouping (e.g. the value of semantic in the mgroup/rgroupse and mgroup/rgroupse properties.

### The SDP parameter IANA registry (http://www.iana.org/assignments/sdp-parameters/) contains a list of defined SDP group semantics. In general the MG should adopt the behaviour defined by the RFC (or other standard) that defines the semantic taking into consideration H.248 protocol behaviour. The following sub-clauses discuss several semantics.

### 10.6.2.1 Lip Synchronisation (LS)

### The “Lip Synchronisation” semantic is defined in [IETF RFC 5888]. As per clause 7[IETF RFC 5888], the MG must synchronise the play out of H.248 Streams listed together in the same “LS” marked group.

### 10.6.2.2 Flow Identifier (FID)

### The “Flow Identifier” semantic is defined in [IETF RFC 5888]. The use of the flow identifier with H.248 is largely redundant due to the H.248 connection model and H.248 usage of SDP. H.248 has the StreamID identifier which uniquely identifies media flows. Each H.248 stream may utilise the Reserve Group/Reserve Value (see 7.1.8.2.3/[ITU-T H.248.1]) construct which enables multiple configurations to be set for a H.248 stream. This allows a single H.248 stream to have multiple sets of characteristics of which one set is active at a particular point in time.

### 10.6.2.3 Single Reservation flow (SRF)

The “Single Reservation flow” semantic is defined in [IETF RFC 3524]. It indicates that media (described by m=lines) should be mapped to the same resource reservation flow. [ITU-T H.248.65] defines a package for the support of RSVP. The initiation of a resource reservation request is typically explicitly requested through the use of a H.248 signal. Any identification of flow is part of the parameters to the signal. Therefore the use of the SRF semantic may be redundant for H.248.

#### 10.6.2.4 Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [IETF RFC 4091]. The ANAT semantic allows a SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

#### 10.6.2.5 Forward Error Correction (FEC)

The “Forward Error Correction” semantic is defined in [IETF RFC 5956]. The use of “FEC” has been deprecated and should not be used with H.248.

#### 10.6.2.6 Forward Error Correction FR (FEC-FR)

The “Forward Error Correction FR” semantic is defined in [IETF RFC 5956]. This is used to associate source and repair flows for the purposes of forward error correction (FEC). If this semantic is received the MG should utilise the source and repairs H.248 Streams listed together in the same “FEC-FR” marked group for the purposes of FEC.

Note: The MGC should indicate whether a flow is a source or repair flow through the use of coding related parameters (i.e. a=rtpmap:111 1d-interleaved-parityfec/90000 and a=fmtp:111 L=10; D=10; repair-window=400000) rather than using the StreamID to indicate this.

#### 10.6.2.7 Composite Session (CS)

The “Composite Session” semantic is defined in clause 3.2/[IETF RFC ietf-rmt-flue-sdp]. It provides a mechanism to group together File Delivery over Unidirectional Transport (FLUTE) protocol sessions. This semantic may be used with H.248 to associate multiple FLUTE sessions together. However some relying on the behaviour outlined in the fore mentioned clause may lead to ambiguous MG behaviour. As H.248 Streams are distinct SDP is not shared across multiple SDP streams. Therefore it is not possible rely on the inheritance behaviour associated with session-level SDP attributes nor inheritance from the “primary media”. In order to avoid ambiguous behaviour a MGC should fully specify SDP associated with FLUTE sessions.

#### 10.6.2.8 Decoding Dependency (DDP)

The “Decoding Dependency” semantic is defined in [IETF RFC 5583]. This semantic may be used with H.248 as well as other SDP attributes (e.g. a=depend) to describe decoding dependencies for layered codings.

#### 10.6.2.9 Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF draft-ietf-mmusic-sdp-bundle-negotiation] only describes the use of BUNDLE for IP transports it shall not be used with Streams using non-IP transports.

As described in clause 8 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 10.6).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “sent group semantics” and “received group semantics” properties that in conjunction with the BUNDLE attribute allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “sent group semantics” and “received group semantics” properties when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by these properties then the same value should be returned for those H.248 Streams.

For more information the use of the BUNDLE and how to support RTP multiplexing in H.248 see clause 11.

### 10.6.3	Error handling

If there is a mismatch between the mgroup/sgroupse and/or mgroup/rgroupse property and the Local or Remote Descriptor in general error code 473 “Conflicting Property Values” should be returned. Although a more specific error code may be returned if available. The underlying reasons for generating an error may depend on the semantic associated with the group.

For example: An error may be generated if the MGC uses the “BUNDLE” semantic and specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command). An error may be generated if MGC uses the “LSP” semantic and specifies the StreamIDs of H.248 streams where the MG cannot maintain lip sync.

{Contributor’s note: The Contributors agree that the underlying reason for generating an error may be different depending on the semantic however errors typically would be as a result of a mismatch between the semantic and/or H.248 StreamIDs and the specific Termination/Stream properties/attributes. Therefore as a general rule error code 473 would be appropriate.}

### 11	Usage in RTP Multiplexing

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams (See clause 8).

Additionally the MGC may set the “association” (mgroup/sgroupse and/or mgroup/rgroupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

For example:

An MGC may require a Termination (T1) with five Streams. StreamID(1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. These streams are bi-directional. It is assumed that the values of StreamID are already allocated before the following processing.

In the first example the mgroup/sgroupse  and mgroup/rgroupse properties indicate that the sent and received video streams are associated:

mgroup/sgroupse = “BUNDLE 3 4 5”

mgroup/rgroupse = “BUNDLE 3 4 5”

In the second example the mgroup/groupse property indicates that the sent and received audio streams are associated together and the video streams are associated together.

mgroup/sgroupse = “BUNDLE 1 2”, “BUNDLE 3 4 5”

mgroup/rgroupse = “BUNDLE 1 2”, “BUNDLE 3 4 5”

When adding or removing multiplexed H.248 Streams the MGC shall include an updated mgroup/sgroupse and/or mgroup/rgroupse property in the same command to ensure consistency between Local and Remote Descriptor settings and the property. On reception of the properties the MG shall assign resources in the knowledge that each of the StreamList element contains StreamIDs that are associated with one multiplexed stream.

For example: when adding a new H.248 Stream (StreamID 6) to an existing multiplex of StreamIDs 3, 4 & 5.

MEGACO/3 [123.123.123.4]:55555

Transaction = 10004 {

Context = 1 {

Mod = T1 {

Media {

TermationState { mgroup/rgroupse = “BUNDLE 3 4 5 6”},

Stream = 6 {

Local {

v=0

c=IN IP4 $

m=video $ RTP/AVP 34

}

}

}

}

}

}

In the above example the MG receives a request to add a H.248 stream with a wildcarded address/port on the Local Descriptor as a result of the mgroup/rgroupse setting it returns the same address as the other H.248 Streams associated with the multiplex. The MGC may have also provided the mgroup/sgroupse property if it specified the Remote Descriptor.