| Question(s): | 3 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | H.Supp.Prio: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to H.Supp.Prio “Priority traffic treatment by H.248 gateways”.

# Discussion

H.Supp.Prio is scheduled to be put forward for Consent at the 28th October – 8th November ITU-T SG16 meeting. The Contributors have performed a review of the draft Recommendation and have identified a number of editorial corrections that should be addressed. There are also a number of technical issues that need to be addressed. These have been noted via MS Word “comments”.

# Proposed

It is proposed to accept the following updates. Changes are marked up against TD-32a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new ITU-T H.Sup.Prio

Gateway Control Protocol:
Priority traffic treatment by H.248 gateways

Summary

This Supplement provides complementary information on the handling of priority traffic (i.e., IEPS/ETS traffic) by the H.248 entities: Media Gateway Controller (MGC) and Media Gateway (MG).

# 1	Scope

This Supplement provides complementary information concerning the handling of priority traffic by the H.248 entities: Media Gateway Controller (MGC) and Media Gateway (MG).

The scope covers the following aspects:

- the notion of “priority traffic” from the perspective of the gateway control interface H.248 (clause 6);

- a comprehensive traffic model for priority services (as introduced by [ITU-T H.248.81]) with focus on the MG (clause 6.1);

- the specification aspects of H.248 profiles with respect to “generic” and “practical” profiles (clause 6.2);

- a status snapshot of some H.248 profiles and Context Attribute support for priority traffic handling (Appendix I);

- example use cases for different gateway types utilising priority traffic (such as with possible IEPS call indicator support) (Appendix II) and

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Supplement.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision; all users of this Supplement are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[ITU-T E.107]		Recommendation ITU-T E.107 (xxx), xxx.

[ITU-T I.130]		Recommendation ITU-T I.130 (11/1988), Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (xxxx), Gateway Control Protocol: xxx.

[ITU-T H.248.54]	Recommendation ITU-T H.248.54 (xxxx), Gateway Control Protocol: xxx.

[ITU-T H.248.56]	Recommendation ITU-T H.248.56 (xxxx), Gateway Control Protocol: xxx.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (02/2012), Gateway Control Protocol: Guidelines for Packet-Based Streams.

[ITU-T H.248.81]	Recommendation ITU-T H.248.81 (05/2011), Gateway Control Protocol: Guidelines on the use of the International Emergency Preference Scheme (IEPS) Call Indicator and Priority Indicator in H.248 Profiles.

[ITU-T Q.1950]		Recommendation ITU-T Q.1950 (2002), Bearer independent call bearer control protocol.

[ITU-T Y.2111]		Recommendation ITU-T Y.2111 (2011), Resource and admission control functions in next generation networks.

[ITU-T H.Sup9]		Supplement 9 to the ITU-T H-series (05/2008), Gateway Control Protocol: Operation of H.248 with H.225.0, SIP, and ISUP in Support of Emergency Telecommunications Service (ETS)/International Emergency Preference Scheme (IEPS).

[ETSI TS 129 238]		ETSI TS 129 238, Universal Mobile Telecommunications System (UMTS); LTE; Interconnection Border Control Functions (IBCF) - Transition Gateway (TrGW) interface; Ix interface; Stage 3.

[ETSI TS 129 334]		ETSI TS 129 334, xxx.

[ETSI TS 183 018]		ETSI TS 183 018, Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.

[3GPP TS 22.153]	3GPP TS 22.153, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects;  Multimedia priority service.

# 3	Definitions

## 3.1	Terms defined Elsewhere

This Supplement uses the following term defined elsewhere:

3.1.1	Emergency Telecommunications Service (ETS) [ITU-T E.107]: A national service providing priority telecommunications to the ETS authorized users in times of disaster and emergencies.

## 3.2	Terms defined in this Supplement

3.2.1	Bearer-dependent decision point: the MGC as the primary decision point with respect to the mapping of call control level information to gateway control signalling and the applicable H.248 context, termination and/or stream entity as bearer level elements. The MGC provides all necessary information about bearer plane protocol stack, supported QoS technologies, etc.

NOTE 1 – This is the usual model in the master-slave relationship of H.248 gateways.

NOTE 2 – The synonym of “technology-dependent …” / “technology-independent …” originates from the ITU-T Resource and Admission Control Function (RACF) architecture (see [ITU-T Y.2111]).

3.2.2	Bearer-independent decision point: this is in contrast to clause 3.2.1, the MGC is still the primary decision point with respect to the mapping of call control level information to gateway control signalling but lacks (partially or fully) bearer related information. The MGC applies generic signalling and delegates the final mapping to the MG.

NOTE 1 – The notion of “bearer-independent …” originates from the Bearer Independent Call Control (BICC) architecture (see ITU-T Q.19xx-series of Recommendation); applied e.g. in the H.248 profile “Bearer independent call bearer control protocol” according to [ITU-T Q.1950].

3.2.3	Mapping function: function, located in H.248 entities (MGC, MG), for mapping protocol objects between two interfaces (MGC: between call control and gateway control signalling; MG: between gateway control signalling and user plane protocols). The mapping function relates to the following, hierarchical information:

a) protocol type (“protocol mapping”) (e.g., H.248, H.323, SIP, IP; protocol type mapping: MGC level mapping: SIP to H.248, SIP(SDP) to H.248(SDP); MG level mapping: H.248 to IP);

NOTE 1 – The MGC located mapping function (MFMGC) could affect the two protocol levels (a1) application control protocol (e.g., H.323, SIP, BICC) and an optional embedded (a2) media description protocol (e.g., SDP).

b) information element type (“type mapping”) (e.g., signalling element type, IP header field) and

NOTE 2 – At a syntactic level the mapping of information element type usually implies a mapping of the information element value.

c) information element value (“value mapping”).

NOTE 3 – The combination of signalling element/type is also known as codepoint.

The mapping function may affect (or not) all three levels in any combination.

# 4	Abbreviations

| BD | Bearer Dependent |
| --- | --- |
| BDDP | Bearer-dependent Decision Point |
| BI | Bearer Independent |
| BICC | Bearer-Independent Call Control |
| BIDP | Bearer-Independent Decision Point |
| DPI | Deep Packet Inspection |
| DS | Differentiated Services |
| ETS | Emergency Telecommunications Service |
| IDS | Intrusion Detection System |
| IEPS | International Emergency Preference Scheme |
| IP | Internet Protocol |
| LxVPN | Layer X Virtual Private Network |
| MF | Mapping Function |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MPLS | Multi-path label switching |
| MPS | Multimedia Priority Service |
| QoS | Quality of Service |
| RACF | Resource and Admission Control Function |
| RPH | Resource Priority Header |
| TC | Traffic Class |
| TDF | Traffic Detection Function |
| SDO | Standards Development Organization |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| TDF | Traffic Detection Function |
| ToS | Type of Service |

…

## 6.1	Traffic model

Figure 1 illustratesthe overall traffic model from [ITU-T H.248.81]. Priority traffic handling as such affects multiple areas: “3”, “4” and “5” with regards to the MG entity.

Figure 1 – Example overall traffic model for priority services with scope on MG entity

The functional areas are (according [ITU-T H.248.81]):

- Part III”: MG Control Path – Priority command processing;

- Part IV: MG Data Path – Reservation, allocation and pre-emption of resources depending on national variation; and

- Part V: Bearer network – Support of dedicated "QoS and policy architectures".

Any kind of traffic priority indication sent from the MGC to the MG leads to correspondent actions in one or multiple functional areas. A generic MG processing rule concept for priority traffic handling may be introduced (see also Figure 2): the rule conditions are given by H.248 signalling elements and the rule actions are related to the above mentioned areas.

## 6.2	Conventions for abstracted H.248 signalling specifications

The following sub-clauses elaborate on specification aspects concerning priority traffic treatment. The generic method of “rule-based description of MG operations” (as e.g. outlined by [ITU-T H.248.79] is used due to the general approach utilised by this Supplement, the advantage of independence of specific H.248 Descriptors and the compact format.

## 6.3	Specification plane: Type of H.248 Profile

This Supplement introduces two H.248 profile types:

- Generic H.248 profile: characterized by a mix of mandatory, optional and conditional protocol capabilities, typically specified by SDOs (like e.g. international profiles);

- Practical H.248 profile: an instantiation of a generic profile, given by a concrete implementation and a given network operational environment. There are no optional capabilities: either something is useable or not.

MG rule actions may be linked to the MG rule conditions in case of a practical profile (because there are no uncertainties from network engineering and MG capability support point of view, and also from the fact the expected MG behaviour must be determined).

However, it’s rather difficult (often impossible) to specify the association between concrete MG actions and their underlying conditions in generic profiles (due to their generic nature, vagueness of operational environment, etc). Such a situation shall be called “semantic issue of MG actions” in the context of priority traffic handling.

Figure 2 – Generic profile view – Processing rule concept
(plus the three functional areas of possible MG actions)

Figure 3 indicates some principal issues related to generic profiles:

#1: 	generic vs practical profile type (see above);

#2: 	binding of actions to conditions (“the exact specified binding determines the final service logic and hence MG behaviour”);

#3:	dependencies on final operational environment of a deployed MG, concerning bearer protocol stacks, QoS architecture, over all traffic mix, etc.

Furthermore, each of the three Context attributes may lead to MG actions in the three areas, which could lead to some overlap (and then interaction problems) when multiple Context attributes are used in parallel (see Figure 4):

#4: 	isolated view (e.g. by defining individual call-dependent procedures for each ContextAttribute element) is not appropriate due to possible interaction problems of MG-level actions);

The isolated view is often given in specification roadmaps when an initial profile version supports one (or two) Context attributes for priority traffic indication, and subsequent profile versions add further support in this area. In order to avoid interaction issues, a reconsideration and comprehensive view would be then necessary (see Figure 5):

…

## 6.4	Specification plane: Codepoint space

The comprehensive view (Figure 5) alludes to the usage of a single parameter (instead of three (Context attributes)) for the control of correspondent MG action(s). This clause discusses the thought of such an approach. It does not revise existing H.248 protocol syntax.

Figure 6 illustrates such an approach: the left side provides legacy H.248 protocol syntax, the three Context attributes define a compound condition at Context level with regards to the required traffic handling. It’s a hierarchical scheme due to the three explicit signalling elements for action control.

The right hand side provides a flat scheme: the three Context attributes are mapped (and replaced) by a single variable, called TrafficPriorityParameter. Some aspects:

- It may be noted that the single parameter approach may be already applied today, implementation internally.

- There are 64 codepoints defined (required) by the three Context attributes (2  2  16).

- The three Context attributes could be thus unambiguously mapped on the TrafficPriorityParameter, defined by e.g. a data type of Integer and a value range of [0, 63] for this variable.

- The two signalling concepts (“three vs. a single parameter”) are functionally equal because the three H.248 Context attributes are generic in the sense that they do not specify concrete MG actions.

A future gateway control (or other signalling or management) protocol design may consider to define just a single signalling element with a future safe codepoint space.

Figure 6 – Hierarchical versus flat priority scheme

## 6.5	Specification plane: Further comments

The requirement on MG behaviour should normally drive the requirement(s) for the H.248 interface (but not vice versa). The resultant MG behaviour would be from the definition of an overall network solution.

The network element behaviour is firstly at the top level of a composed “MGC/MG” gateway entity, - according the overall traffic model of [ITU-T H.248.81]. Hence there’s a mapping of “service requirements/capabilities” on the “composed MGC/MG” and then “functional distribution on decomposed gateway model”.

# 7	Summary

This supplement illustrates the difficulty,  even impossibility of a generic specification  i.e. , a generic H.248 profile in justifying concrete, dedicated protocol solutions in case of available alternative for “emulation approaches”.

Appendix I

H.248 profiles and Context Attributes for priority traffic handling

The purpose of this Appendix is an analysis concerning signalling support for priority traffic handling.

## I.1	Status H.248 Profiles

H.248 profile definitions include Context Attribute support information (see the profile definition template in [ITU-T H.248.1]). There are many H.248 profile specifications published by standard development organizations (SDOs). Of particular interest in this Supplement are profiles for H.248 IP-IP gateways; such as the 3GPP-defined profiles for their H.248-based reference points Ix and Iq ( [ETSI TS 129 238] and [ETSI TS 129 334]; which are both based on the ETSI TISPAN Ia profile [ETSI TS 183 018]).

The following table summarizes the status for Ix and Iq interfaces:

Table II.1 – 3GPP Profiles and typical support of Context Attributes (status: Rel-11)

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| … |  |  |
| Priority Indicator | Optional (NOTE 1) | 0-15 (Note 2) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Call Indicator | No | NA |
| … |  |  |
| NOTE 1: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in [3GPP TS 22.153]. NOTE 2: Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

It may be noted that there’s support of emergency calls only in pure 3GPP environments, and additional priority level support in ETSI TISPAN and 3GPP instantiations.

## I.2	Discussion of the principal H.248 signalling options

H.248 supports the provisioning of protocol parameter values, which may sometimes be used as an alternative to the usual signalling of protocol values. There are further kinds of H.248 decomposed gateway models, such as bearer-independent or bearer-dependent style of operation (see clauses 3.2.2 and 3.2.1). The alternatives lead to various options which are outlined in this clause, using the concrete example of DiffServ-based QoS marking as the selected technology for priority traffic support. The different options are also related to a mapping function (MF; see clause 3.2.3) involved at MGC and/or MG level.

### I.2.1	Overview

The “principal H.248 signalling options” are given by the combination of following characteristics:

- Kind of mapping function (related to type and/or value mapping)

  - At MGC level: call control protocol related codepoints are mapped to H.248 codepoints (‘bearer dependent’ or ‘bearer independent’) format; or/and

  - At MG level: mapping of H.248 codepoints to final IP bearer path format ;

- H.248 signalling format

  - ‘bearer dependent’, i.e., QoS marking codepoints signalled already in the format as used in IP bearer packets  (hence, so-called type mapping not necessary);

  - ‘bearer independent’, i.e. (I.b) type mapping would be always required in the MG.

Three meaningful combinations are described subsequently.

### I.2.2	Bearer dependent H.248signalling

H.248 signalling (i.e., ADD and MODIFY request commands) carry the bearer dependent DiffServ codepoint (as e.g. assigned for priority traffic). The MG directly uses that DiffServ codepoint for QoS marking outgoing bearer packets. There are two variants from a MGC perspective, see Figure I.1:

Figure I.1 – Control of a MG-level QoS marking function – Variants 1 and 2

Details:

- Variant 1 could avoid type mapping and value mapping at MGC and MG level.

- Variant 2 is typically used when call control signalling either does not support signalling of DiffServ values or when a local DiffServ translation is applied. In this case, the QoS marking is provisioned at the MGC, and the MGC provides the DiffServ mapping.  Thus, there is type/value mapping at MGC level, but not any such mapping additionally necessary by the MG.

{Editor’s note: Further contribution is needed to document what the Variants are.}

### I.2.3	Bearer independent H.248 signalling

The DiffServ codepoint for a particular packet bearer traffic class might be also provisioned at MG level. This implies the necessity of a correspondent mapping function at MG level (MFMG, see Figure I.2) besides the MGC level MF.

Figure I.2 – Controlling of MG-level QoS marking function – Variant 3

Details:

- Variant 3 does not require the explicit signalling of DiffServ codepoints at the H.248 interface. Instead another H.248 protocol element could be signalled to trigger marking bearer packets with appropriate DiffServ codepoints.

- Variant 3 needs to take into account that an H.248 profile might support in addition direct DiffServ signalling, which may lead to a mix of both H.248 signalling options (i.e., ‘bearer deendent’ and ‘bearer independent’ formats used in parallel), and hence the careful consideration of possible interaction issues.

## I.3	IEPS/ETS traffic support: question of signalling (stage 3) solutions?

There are different signalling (stage 3) alternatives as outlined by the previous clause, in order to satisfy requirements (stage 2) for IEPS/ETS traffic support:

- Native approach: direct signalling (stage 3) method based on the explicit H.248 signalling element for that purpose, the ContextAttribute IEPS call indicator.

- Emulation approach: indirect signalling variant by reuse of existing signalling (stage 3) capabilities, which would relate here to the support of ContextAttribute Priority indicator for IEPS/ETS.

Discussion:

Option (A) looks straightforward, but there’s an issue with this approach: the IEPS call indicator ContextAttribute was introduced in H.248.1 Version 3, which is not yet supported by existing H.248 profile as published by SDOs.

For instance, there is reluctance (e.g., in case of the examples of clause I.1) to upgrade from H.248.1 Version 2 (as the core protocol baseline for these profiles) to Version 3, due to the functionality associated with a single additional protocol element. However, the introduction of H.248.1 V3 might be basically mitigated when a stepwise addition of V3 capabilities through profiles would be considered (“instead of the all or nothing approach”).

Appendix II

Potential use cases with IEPS call indicator support

## II.1	Overview

The following use cases may be identified concerning H.248 IP-IP gateway types (not an exhaustive list):

Table II.1 – Overview of use cases

Potential use cases with IEPS call indicator support

Use case #A: Support of multiple, different IP QoS architectures

Use case #B: IP bearer connection traversing different IP QoS architectures

Use case #C: Priority treatment with more than 16 priority levels

Use case #D: Network monitoring of signalling traffic (here: H.248)

Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Use case #F: Heterogeneous vendor landscape

A crucial location of such an H.248 gateway is the edge of an administrative domain, such as the peering point between different network operators. Such an interconnection scenario needs to consider a wide spectrum of possible network architectures and technology support due to different preferences and/or network status of individual operators.

Figure 1 (in clause 6) summarizes priority traffic handling by H.248 entities. Any kind of traffic priority indication sent from the MGC to the MG leads to correspondent actions in one or multiple functional areas.

## II.2	Use case #A: support of multiple, different IP QoS architectures

This use case focuses on bearer network characteristics (so called area “5” in Figure 1) by considering support of multiple, different IP QoS architectures (see Figure II.1):

Figure II.1 – Use case #A: support of multiple, different IP QoS architectures

The assumption is that the stage 2 requirement could be satisfied by dedicated QoS support for IEPS/ETS traffic as a stage 3 solution. This implies a mapping of IEPS/ETS information at call control signalling level to appropriate “H.248 means” (see also Figure. II.1).

There are two basic solutions, - according different mapping strategies -, outlined in Figure. II.1:

- Solution A.1:

  - the MGC is responsible for all kinds of QoS mappings, thus a so-called bearer-dependent decision point (see clause 3.2.1);

  - the stage 2 requirement could already be supported by H.248 signalling elements for “QoS technologies”;

  - the ContextAttribute IEPS call indicator may not be needed at all;

- Solution A.2:

- the BDDP responsibility is delegated to the slave MG entities;

- the MGC would be thus acting as a bearer-independent decision point (see clause 3.2.2);

- a generic mapping between call control signalling indicators and H.248 ContextAttributes would be possible.

Conclusions:

- Option A.2 looks attractive from MGC perspective, however, existing profile specifications assign the BDDP to the MGC (due to the “master” attribute of a MGC, see also clause 3.2.1);

- thus, the starting point for stage 3 would be the assumption of a “QoS technology aware” MGC (i.e. option A.1)

- any IEPS call indicator support isn’t straight forward to justify.

## II.3	Use case #B: IP bearer connection traversing different IP QoS architectures

The next use case is similar as scenario #A, but focuses on the interworking aspect: the end-to-end IP bearer connection for IEPS/ETS traffic traversing different IP QoS architectures (Fig. II.2). Thus, the two H.248 IP stream endpoints would then have different protocol stacks and/or different policy enforcements for IEPS/ETS support, leading to additional mapping complexity as in comparison to #A.

Figure II.2 – Use case #B: IP bearer connection traversing different IP QoS architectures

Conclusion:

- the discussion (for the indicated solutions B.1 and B.2 in Figure II.2) is fairly similar as in use case #A.

## II.4	Use case #C: Priority treatment with more than 16 priority levels

The codepoint space would be exhausted in network environments which request more than 16 priority levels (due to existing protocol design of [ITU-T H.248.1]). The “IEPS/ETS service” could not be emulated (“the option B in clause I.3”) by reserving a particular H.248 priority indicator codepoint exclusively for IEPS/ETS calls.

Figure II.3 – Use case #C: Priority treatment with more than 16 priority levels

Conclusions:

- there would be just one solution feasible (C.1) by support of the IEPS call indicator

- there is also not yet any stage 2 requirement announced requesting  more than 16 priority levels.

…

## II.6	Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Regulation authorities could explicitly mandate support of all ContextAttributes for national solutions.

Conclusion:

- No further consideration is needed here because a national (or local) H.248 profile would/could be derived from international (or global) H.248 profile specifications.

## II.7	Use case #F: heterogeneous vendor landscape

No further elaboration of this use case is needed, because this is rather a network operator topic and beyond the scope of SDOs.

…

_________________

Such a protocol engineering approach would avoid the side discussion here: upgrade of H.248 protocol version from 1 or 2 to 3 due to IEPS call indicator support.