|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 0267 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | October 2013 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 2/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 267 |  |  |  |  |  |
| Source: |  | Australia |  |  |  |
| Title: |  | H.asset work item updates |  |  |  |

# Summary

This document proposes changes to the H.asset work item created at the Rapporteurs Meeting in June 2013 in Oslo.

# Introduction

At the Rapporteurs Meeting in June 2013 in Oslo, AVD-4361 was presented and a new work item H.asset was agreed to be created. The proposed text was accepted as a baseline document. It was noted at that meeting

[TD-08a]

It was agreed to initiate a new work item (H.46x.0) to define the base specification for the series of ASSET specifications that will follow, with this document serving as the initial text.  Other ASSET specifications in the series will define vertical applications, such as file transfer, electronic whiteboard, etc.

A point raised was that if a Gatekeeper wants to terminate a call, it would not know that the main terminal device and a number of ASSET devices are participating in the same call, since each signalling connection looks like a distinct call to the Gatekeeper.  ASSET devices periodically poll the main terminal device to see that the call is still active.

There are potential security implications with this model that need to be considered.  Those security considerations need to be documented as a part of the specification.

It should be possible to discover, associate, and invoke applications in ASSETs even after the primary terminal initiates a call.

# Discussion

The expert’s review of the baseline text has been noted and this contribution developed to address concerns raised. After the meeting a number of informal discussions yielded other important concerns. They are also captured in this document.

# Summary of changes

- Include call flow diagrams to better illustrate ASSET pairing and invoking an application.

- How an endpoint discovers the aliasAddress of ASSET is beyond the scope of the document.

- Support an ASSET invoking applications.

- Simplify call flows.

- Add time to live and status update times to associations.

- Improve security by altering the application invocation logic so the invokeToken and aliasAddress is only available when that specific application needs to be invoked.

- Reformat and reorganise the ASN.1 code to be top down and more easily readable.

- Add security considerations

- Editorial Improvements

Due to the large number of changes/alterations, it is proposed to replace the baseline document of the work item with this new document.

Proposed Draft New Recommendation H.46x.0

H.323 Associative Set (ASSET) Feature

# 1	Scope

This Recommendation describes the procedures for using H.225.0 Call Signalling as a method for the association of H.323 systems for the purpose of remote querying and invoking of applications to realise functionality beyond the capability of one single device.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]	Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.225.0]	Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235]	Recommendation H.235 (2003), Security and encryption for H-Series 
(H.323 and other H.245-based) multimedia terminals.

[ITU-T H.245]	           Recommendation H.245 (2011), Control protocol for Multimedia Communications.

[ITU-T H.460.1]	Recommendation H.460.1 (2002), Guidelines for the Use of the Generic Extensible Framework.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

### 3.1.1	Application:  A real-time function which resides on an Associative Simple Endpoint Type.

## 3.2	Terms defined in this Recommendation

3.2.1 Pairing:  A unique association between an endpoint and an ASSET.

### 3.2.2 AssociationToken: A Globally Unique Identifier used to validate/authenticate ASSET pairings.

3.2.3 AssociationLife: The amount of time in seconds an AssociationToken shall remain active without any activity.

3.2.4 InvokeToken: A Globally Unique Identifier used to identity an Application in the context of a call to invoke/terminate.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ASSET 		    Associative Simple Endpoint Type as defined in H.323 Annex x.

| ASN.1 | Abstract Syntax Notation No. 1 |
| --- | --- |
| GEF | Generic Extensible Framework |
| OID | Object Identifier |
| UI | User Interface |

# 5	Feature description

This Recommendation defines a procedure wherein H.323 devices may associate with an ASSET for the purpose of discovering and utilising functionality on that device that otherwise would not be available to the H.323 device.

# 6	Informative description

This Recommendation outlines the mechanism to use the H.225.0 call signalling connection as a method for a H.323 device to associate with an ASSET for the purpose of discovering applications on that ASSET and invoking then during the context of a call on the H.323 device.

# 7	Capability advertisement

An ASSET shall have an H.225.0 endpointType of SET (bit 2 set) as defined in H.323 Annex x.

Endpoints and ASSETs capable of supporting this Recommendation shall advertise this capability via the generic extensibility framework (GEF) defined in [ITU-T H.323] and [ITU-T H.460.1].

An endpoint calling another endpoint shall advertise the ASSET feature via the supportedFeatures field in the Setup Message. An endpoint that supports this Recommendation shall respond via the supportedFeatures field in the Connect Message. All messaging shall be conveyed in the ASSETApplication parameter (refer Table 2)

An ASSET calling another ASSET within the context of an existing call shall advertise the ASSET feature via the neededFeatures field in the Setup Message. An endpoint that supports this Recommendation shall respond via the neededFeatures field in the Connect Message. All messaging shall be conveyed in the ASSETApplication parameter (refer Table 2)

All calls between endpoints and ASSETs shall set the conferenceGoal to callIndependentSupplementaryService. The ASSET feature shall not be advertised. One or more ASSETAssociate parameters (refer Table 3) shall be included in the genericData field of the H323-UU-PDU. All other optional fields shall be omitted.

Table 1 defines the ASSET feature in this Recommendation.

Table 1 – Indication of ASSET feature

| Feature name: | ASSET Feature |
| --- | --- |
| Feature description: | H.323 Associative Simple Endpoint (ASSET) Feature |
| Feature identifier type: | oid |
| Feature identifier value: | 0.0.8.46x.0 |

Table 2 – ASSETApplication parameter

| Parameter name: | ASSETApplication |
| --- | --- |
| Parameter description: | ASSET Application Message |
| Parameter identifier type: | oid |
| Parameter identifier value: | 0.0.8.46x.0.1 |
| Parameter type: | octetString ASN.1 Encoded ASSETPDU (SEQUENCE OF ASSETMessage) (Refer Annex A) |
| Parameter cardinality: | One and only one |

Table 3 – ASSETAssociate parameter

| Parameter name: | ASSETAssociate |
| --- | --- |
| Parameter description: | ASSET Associate Message |
| Parameter identifier type: | oid |
| Parameter identifier value: | 0.0.8.46x.0.0 |
| Parameter type: | octetString ASN.1 Encoded ASSETMessage (Refer Annex A) |
| Parameter cardinality: | One or more |

# 8	Pairing an endpoint with an ASSET

Endpoints wishing to pair shall place a call to the ASSET containing the ASSETMessage with the application field set to associateRequest. All other fields shall be omitted. The associateRequest shall contain a proposed AssociationLife.

How an endpoint discovers the H.225.0 AliasAddress of the ASSET to place the call and what value the AssociationLife to set to is beyond the scope of this Recommendation.

Upon receiving the associateRequest the ASSET may prompt the user to approve or decline the request. All subsequent messages shall be ignored until the pairing has been confirmed or declined.

Figures 1 and 2 below provide indicative call flows for endpoint and ASSET pairing

Figure 1 –Pairing rejection call flows

Figure 2 –Pairing confirmation and application listing call flows

Where pairing is rejected, the ReleaseComplete message shall be sent with reason set to genericDataReason.

Where the pairing has been approved, an AssociationToken (Globally Unique Identifier) shall be generated. This is transmitted back to the Endpoint via a Connect Message containing an ASSETMessage with the application set to associateResponse. All other fields of the ASSETMessage shall be omitted.

The associateResponse shall contain:-

associateToken: The locally generated AssociationToken for the pairing

timeToLive: AssociationTime for the AssociationToken.

and may include:-

statusInterval: The recommended time interval for the endpoint to send statusRequest.

The AssociationLife signalled shall be the lower of the endpoint and the ASSET. If there has been no activity for the given time, the AssociationToken shall no longer be valid for the ASSET.

The associationToken field in the ASSETMessage shall be presented in all future communication between the endpoint and the ASSET.

The endpoint may request an application listing. This is achieved by sending a Facility message containing a statusRequest.

Upon receiving a statusRequest, the ASSET shall reply with a Facility message containing an statusResponse..

Each statusResponse shall contain:-

applicationId: Unique object identifier (OID) for the application. (Refer Annex B).

and may include:-

display: An ApplicationDisplay contains human readable name of the application and the language.

avatar:  A URL path to an image to display for the application.

status: An ApplicationState choice indicating the current availability of the application.

From time to time or at the recommend statusInterval, the endpoint may send a statusRequest to the ASSET to update its listing. The ASSET may update the endpoint of any status change events by sending an unsolicited statusResponse.

# 9	Invoking Application

The callIdentifer field of the ASSETMessage shall be present for all messages that are within the context of a call.

Figure 3   – Call and application invocation call flows

## 9.1	Advertising and negotiating ASSET support during a call

The SETUP message shall contain a listRequest.  The listRequest shall contain the applications that are locally available for the call.

The remote endpoint shall reply with a listResponse.

Upon receiving a listResponse the endpoints shall send a callAppList to each ASSET supporting the applications available for the call.

The callAppList shall contain:-

applicationId: Unique object identifier (OID) for the application (Refer Annex B)

aliasAddress: AliasAddress of the original call.

During a call the endpoints may update the common application list by exchanging a replacement listRequest/listResponse.

## 9.2	Invoking an application during a call

An application may be invoked by the endpoint or the ASSET supporting the application.

An endpoint may invoke one or many applications concurrently.

An endpoint shall start an application by sending a preInvokeRequest to the local ASSET supporting the required application.

The ASSET shall respond with a preInvokeResponse. Where the application is available for use the state field shall be set to available. Where the application is not available the state field shall be set to invokeFail and include the reason.

Where the preInvokeResponse returns available, the endpoint shall send an invokeRequest to the remote endpoint which is then forwarded to the ASSET containing the application. The application shall respond with an invokeResponse. Where the request is declined the invokeResponse shall be set to declined and contain the reason. Where the request is approved, the invokeResponse shall be set to approved and contain the necessary information for the remote ASSET it initiate the call.

The approved field shall contain:-

applicationId: Unique object identifier (OID) for the application to invoke

aliasAddress: The SEQUENCE of H.225.0 AliasAddress the application can be contacted on.

invokeToken: The ASSET generated InvokeToken to be used to start the application.

Where both endpoints attempt to invoke the same application at the same time, the two InvokeToken shall be binary compared from the most significant to the least significant octet where the lower value shall hold ascendancy and used by the party to initiate the call.

## 9.3   Starting an application

An ASSET may start an application. To start an application the ASSET shall originate an invokeRequest to the local endpoint which is then forwarded on and the reply invokeResponse shall be forwarded back to the ASSET.

Where an endpoint invokes the application as per Clause 9.2, and after receiving the invokeResponse an invoke shall be sent to instruct the ASSET to start the application.

To start an application, the ASSET places a call to the H.225.0 Alias Address supplied in the aliasAddress field in either the invoke or invokeResponse. The SETUP message shall include an invokeStartList containing the applications to start.

### 9.3.1  Notifying an application has started

Upon successful call establishment the ASSET shall send to the endpoint an invokeNotify with the state field set to associated. Where the call is unsuccessful the state field shall be set to h2250Error.

## 9.4  Terminating an application

To terminate an application an ASSET simply ends the call. Where an endpoint terminates the application a stopRequest shall be sent to the local ASSET to end the call.

## The ASSET shall notify the endpoint that the application has terminated via a stopNotify.

## 9.5  Hanging up a call

When the original call terminates, all running applications associated with that call shall also terminate and resources released. This is done by sending a callRelease to all associated ASSETs.

# 10  Security considerations

All endpoint/ASSET pairing have a unique AssociationToken which may require approval from the ASSET before being issued. The AssociationToken must be present before an ASSET shall action any request from an endpoint.

An application shall not be invoked unless it has been issued a unique InvokeToken. The InvokeToken shall be a unique value linking an application to a call and shall be present in the SETUP Message. If a valid InvokeToken is not present the call shall be rejected with a ReleaseComplete.

The endpoint may receive an invokeRequest to invoke an application and prompt the user to allow the ASSET to receive that request. Likewise the ASSET may receive an invoke or SETUP PDU and prompt the user for approval before allowing an application to start.

For further study.

Annex A

ASN.1 Code for H.46x.0

ASSET {itu-t(0) recommendation(0) h(8) 46x (0) } DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AliasAddress,

GenericIdentifier,

GloballyUniqueID,

CallIdentifier,

TimeToLive

FROM H323-MESSAGES; -- See Rec. ITU-T H.225.0

--

-- ASSETPDU

--

ASSETPDU ::= SEQUENCE OF ASSETMessage

--

-- ASSETMessage

--

ASSETMessage ::= SEQUENCE

{

application      ApplicationIE,

associateToken   GloballyUniqueID              OPTIONAL,

callIdentifier   CallIdentifier                OPTIONAL,

...

}

--

-- Application IE

--

ApplicationIE ::= CHOICE

{

associateRequest    AssociateRequest,

associateResponse   AssociateResponse,

statusRequest       NULL,

statusResponse      SEQUENCE OF ApplicationStatus,

listRequest         SEQUENCE OF ApplicationStatus,

listResponse        SEQUENCE OF ApplicationStatus,

callApplist         SEQUENCE OF ApplicationAvailable,

preInvokeRequest    Application,

preInvokeResponse   ApplicationStatus,

invokeRequest       ApplicationInvokeRequest,

invokeResponse      ApplicationInvokeResponse,

invoke              ApplicationInvoke,

invokeStartList     SEQUENCE OF ApplicationStart,

invokeNotify        ApplicationStatus,

stopRequest         SEQUENCE OF Application,

stopNotify          SEQUENCE OF Application,

callRelease         NULL,

...

}

--

-- AssociateRequest

--

AssociateRequest ::= SEQUENCE

{

timeToLive      TimeToLive,   -- seconds

...

}

--

-- AssociateResponse

--

AssociateResponse ::= SEQUENCE

{

associateToken  GloballyUniqueID,

timeToLive      TimeToLive,   -- seconds

statusInterval  TimeToLive     OPTIONAL,   -- seconds

...

}

--

-- ApplicationStatus

--

ApplicationStatus ::= SEQUENCE

{

applicationId      GenericIdentifier,

display            SEQUENCE OF ApplicationDisplay   OPTIONAL,

avatar             IA5String                        OPTIONAL,

state              ApplicationState                 OPTIONAL,

...

}

--

-- ApplicationDisplay

--

ApplicationDisplay ::= SEQUENCE

{

language      IA5String                 OPTIONAL,    -- RFC4646 language tag

display       BMPString(SIZE(1..128))

}

--

-- ApplicationState

--

ApplicationState ::= CHOICE

{

available      NULL,

unavailable    NULL,

inuse          NULL,   -- In use by another call

associated     NULL,   -- In use by this call

invokeFail     InvokeFailReason,

...

}

--

-- InvokeFailReason

--

InvokeFailReason ::= CHOICE

{

unavailable    NULL,

inuse          NULL,   -- In use by another call

declined       NULL,

h2250Error     NULL,

...

}

--

-- Application

--

Application ::= SEQUENCE

{

applicationId      GenericIdentifier,

...

}

--

-- ApplicationAvailable

--

ApplicationAvailable ::= SEQUENCE

{

applicationId      GenericIdentifier,

aliasAddress       SEQUENCE OF AliasAddress,

...

}

--

-- ApplicationInvokeRequest

--

ApplicationInvokeRequest ::= CHOICE

{

applicationId      GenericIdentifier,

...

}

--

-- ApplicationInvokeResponse

--

ApplicationInvokeResponse ::= CHOICE

{

approved   ApplicationInvoke,

declined   InvokeFailReason,

...

}

--

-- ApplicationInvoke

--

ApplicationInvoke ::= SEQUENCE

{

applicationId      GenericIdentifier,

invokeToken        GloballyUniqueID,

aliasAddress       SEQUENCE OF AliasAddress,

...

}

--

-- ApplicationStart

--

ApplicationStart ::= SEQUENCE

{

applicationId      GenericIdentifier,

invokeToken        GloballyUniqueID,

...

}

END

Annex B

Table B.1 – OIDs Defined for this Recommendation

| OID | Clause Reference |
| --- | --- |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) } | 7 |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) associate(0) } | 7 |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) application(1) } | 7 |

_________________