| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4474 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.TLS |  |  |  |
| Title: | H.248.TLS "Gateway control protocol: H.248 packages for control of transport security using TLS" (New): Input draft Ed. 0.12 |  |  |  |
| Purpose: | Meeting input (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's input draft for draft new H.248.TLS "H.248 packages for control of transport security using TLS" (Ed. 0.12).

# Changes versus Ed. 0.11 (TD 84 R1 (WP 1/16) (2013-11)):

- editorial in Reference section

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TLS Ed. 0.1 | Initial draft (TD-23) from WP 2/16 Geneva meeting, Feb. 2012 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.TLS Ed. 0.2 | Input (TD 749 (WP2/16)) from SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749 |
| H.248.TLS Ed. 0.3 | Output (TD 749R1/WP2) from SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749R1 |
| H.248.TLS Ed. 0.4 | Input (AVD-4232) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4232.zip |
| H.248.TLS Ed. 0.5 | Output (TD-23) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-23.zip |
| H.248.TLS Ed. 0.6 | Input (TD 20 (WP2/16)) from SG16 Geneva meeting, Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.7 | Output (TD 20 R1 (WP2/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0020 |
| H.248.TLS Ed. 0.8 | Input (AVD-4364) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4364.zip |
| H.248.TLS Ed. 0.9 | Output (TD-14) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-14.zip |
| H.248.TLS Ed. 0.10 | Input (TD 84/WP1) to SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0084!MSW-E |
| H.248.TLS Ed. 0.11 | Output (TD 84R1/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0084!R1!MSW-E |
| H.248.TLS Ed. 0.12 | Input (AVD-4474) to WP 1/16 Geneva meeting (2014-03) Location: |
| H.248.TLS Ed. 0.13 | Output (TD-##) from WP 1/16 Geneva meeting (2014-03) Location: |
| H.248.TLS Ed. 0.14 |  |
| H.248.TLS Ed. 0.15 |  |

CONTENTS

Page

1 Scope 11 1.1 Applicability statements 12 2 References 12 3 Definitions 13 3.1 Terms defined elsewhere 13 3.2 Terms defined in this Recommendation 13 4 Abbreviations and acronyms 14 5 Conventions 16 5.1 Conventions used in signalling flows 16 5.2 TLS endpoint notations 16 5.3 Important notation derived from TLS presentation language 17 6 Use case descriptions 17 6.1 Use cases related to TLS transport modes 17 6.2 Bearer connection network use cases with H.248 IP-IP gateways 17 6.2.1 High-level use case categories 17 6.2.2 Use case variations 19 6.3 Bearer connection network use cases with H.248 "TCP to non-TCP" gateways 19 6.4 Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway) 19 7 Models 19 7.1 Network model from H.248 entity point of view 19 7.2 Bearer connection model 20 8 TLS basic session control package 21 8.1 Properties 21 8.1.1 Security session establishment blocking 21 8.2 Events 23 8.2.1 TLS session state change ("BNC change") 23 8.3 Signals 24 8.3.1 Establish (BNC) 24 8.3.2 Release (BNC) 24 8.4 Statistics 25 8.5 Error Codes 25 8.6 Procedures 25 8.6.1 TLS endpoint creation 25 8.6.2 TLS endpoint role assignment 25 8.6.3 TLS security session establishment 25 8.6.4 TLS application data transfer 26 8.6.5 TLS session release 27 9 TLS extended session control package 29 9.1 Properties 29 9.1.1 Stream Endpoint Pair Interlinkage 30 9.1.2 Protocol Layers Interlinkage 31 9.2 Events 33 9.3 Signals 33 9.3 Statistics 33 9.5 Error codes 33 9.6 Procedures 33 9.6.1 TLS security session establishment 33 9.6.2 TLS security session release 33 10 TLS capability negotiation package 34 10.1 Properties 35 10.1.1 TLS Domain Profile Identifier 35 10.1.2 TLS Versions 35 10.1.3 Cipher Suites 36 10.1.4 Compression Methods 37 10.1.5 Support for Session Resumption 37 10.1.6 Support for Renegotiation of the Security Context 37 10.1.7 Renegotiation Period 38 10.1.8 Client Authentication Required 38 10.2 Events 38 10.3 Signals 38 10.4 Statistics 38 10.5 Error codes 39 10.6 Procedures 39 10.6.1 General default behaviour (MGC does not influence TLS capability negotiations) 39 10.6.2 MGC-controlled TLS capability negotiations: Options 39 10.6.3 Determining TLS Related Capabilities 39 10.6.4 Indication of TLS profile(s) to MG by MGC 40 10.6.5 Indication of individual TLS protocol capabilities to MG by the MGC 40 11 TLS session maintenance package 42 11.1 Properties 43 11.2 Events 43 11.2.1 TLS Error Alert 43 11.3 Signals 44 11.3.1 MGC triggered TLS error alert 44 11.4 Statistics 45 11.5 Error codes 45 11.6 Procedures 45 11.6.1 TLS warning error alerts 45 11.6.2 TLS fatal error alerts 45 11.6.3 Lower layer issues 46 12 TLS traffic volume metrics package 46 12.1 Properties 46 12.2 Events 46 12.3 Signals 46 12.4 Statistics 47 12.4.1 Number of Received Application Data TLS-Fragments 47 12.4.2 Number of Sent Application Data TLS-Fragments 47 12.4.3 Number of Received TLS Message Fragments 47 12.4.4 Number of Sent TLS Message Fragments 48 12.4.5 Number of Received Application Data Octets 48 12.4.6 Number of Sent Application Data Octets 49 12.4.7 Number of Received TLS Message Octets 49 12.4.8 Number of Sent TLS Message Octets 49 12.4.9 Number of Sent Application Data TLS-PDU Octets 50 12.4.10 Number of Received Application Data TLS-PDU Octets 50 12.4.11 Number of Sent TLS Message PDU Octets 51 12.4.12 Number of Received TLS Message PDU Octets 51 12.4.13 Number of Sent Application Data Octets, Protected and Compressed 51 12.4.14 Number of Received Application Data Octets, Protected and Compressed 52 12.4.15 Number of Sent TLS Message Octets, Protected and Compressed 52 12.4.16 Number of Received TLS Message Octets, Protected and Compressed 53 12.4.17 Number of Octets of Compressed Application Data Fragments in egress Direction 53 12.4.18 Number of Octets of Compressed Application Data Fragments in ingress Direction 54 12.4.19 Number of Octets of Compressed TLS Message Fragments in egress Direction 54 12.4.20 Number of Octets of Compressed TLS Message Fragments in ingress Direction 55 12.5 Error codes 55 12.6 Procedures 55 12.6.1 Ingress TLS traffic – Statistic "TLS Packets Received" 55 12.6.2 Ingress TLS traffic – Statistic "TLS Octets Received" 55 12.6.3 Ingress TLS traffic – Statistic "TLS Application Data Octets Received" 55 12.6.4 Egress TLS traffic – Statistic "TLS Packets Sent" 56 12.6.5 Egress TLS traffic – Statistic "TLS Octets Sent" 56 12.6.6 Ingress TLS traffic – Statistic "TLS Application Data Octets Sent" 56 13 Package-less TLS control 56 13.1 Related to TLS session establishment 56 13.1.1 Overview – principal H.248 control steps 56 13.1.2 Step 1: Bearer endpoint creation 59 13.1.3 Step 2: Bearer type indication for IP transport "L4/L3" 59 13.1.4 Step 3: Block / Unblock L4 bearer control procedures at SEP 59 13.1.5 Step 4: Start IP transport connection establishment procedure 60 13.1.6 Step 5: Local L4/L3 NAT traversal support 60 13.1.7 Step 6: Reporting successfully established IP transport connection 60 13.1.8 Step 7: Bearer type indication for TLS 60 13.1.9 Step 8: Block / Unblock TLS security session control procedures at SEP 60 13.1.10 Step 9: Start TLS security session establishment procedure 60 13.1.11 Step 10: Local L4+ NAT traversal support 60 13.1.12 Step 11: Reporting successfully established TLS session 60 13.1.13 Step 12: Active communication phase 60 13.2 Related to TLS authentication 61 13.3 Related to TLS session release 61 14 The TLS profile concept 61 14.1 TLS protocol profiles 61 14.2 Illustration of TLS profile concept 62 14.3 Example for the TLS MG profile concept 63 15 Security considerations 64 Annex A State modelling for TLS bearer connection endpoints 65 A.1 Introduction and purpose 65 A.2 Original state model for TLS session endpoints 65 A.3 Simplified state model for H.248-based TLS basic session control 65 A.3.1 Transitioning from Idle to Established 66 A.3.2 Transitioning from Established to Idle 66 Annex B TLS protocol layer: Data model 66 B.1 Motivation 66 B.2 Data model 67 B.3 Terminology based on data model 69 Appendix I Sample use cases of TLS bearer encryption 70 I.1 Use case "Terminal-to-MG TLS session, provisioned TLS service negotiation" 70 I.2 Use case "Terminal-to-MG TLS session, MGC-controlled TLS service negotiation" 71 I.3 Use case "WebRTC to NGN/IMS interworking function with DTLS-to-TLS support" 71 I.4 Use-case "TLS-based transport security for facsimile packet relay service T.38" 71 Appendix II Example call flows 72 II.1 TLS to non-TLS interworking with TCP as example bearer type 72 II.1.1 Initial session setup using SDP security descriptions in application control signalling 72 II.1.2 Traffic flow examples for communication establishment phase 73 II.1.3 Traffic flow examples for communication release phase 77 Appendix III Example TLS profiles 79 III.1 Typical Internet TLS profile 79 III.2 3GPP TLS domain profile 80 III.3 OMA TLS domain profiles 82 III.3.1 OMA public key TLS domain profile 82 III.3.2 OMA pre-shared key TLS domain profile 83 III.4 IETF Minimum TLS domain profile 84 III.5 IETF example of a national TLS domain profile 84 III.6 ITU-T TLS domain profile for NGN signalling and management plane 84 Appendix IV Illustration of protocol semantics of the TLS basic session control package 84 IV.1 Overview 84 IV.2 Conventions 84 IV.3 Establishment of TLS security sessions 84 IV.3.1 Successful establishment – Terminating side 84 IV.3.2 Successful establishment – Originating side 85 IV.3.3 Unsuccessful establishment 86 IV.4 Release of TLS security sessions 86 IV.4.1 Successful release – Terminating side 86 IV.4.2 Successful release – Originating side 87 IV.4.3 Unsuccessful release 88 Appendix V Illustration of protocol semantics of the TLS extended session control package 88 V.1 Overview 88 V.2 Conventions 88 V.3 Usage of SEPP Interlinkage property 88 V.4 Usage of Protocol Layers Interlinkage property 88 V.4.1 Property "plslink = FALSE" 89 V.4.2 Property "plslink = TRUE" 89 Appendix V TLS alert protocol from H.248 gateway perspective 94 V.1 Background 94 V.2 H.248 gateway framework concerning TLS alert handling 94 V.3 TLS error alerts categorization 94 V.4 Summary 95 Bibliography 95

List of Tables

Page

Table 1 – Principal TLS-based interfaces of H.248 entities and their relevance for this Recommendation 12 Table 2 – Security session establishment preparedness (of TLS endpoints) 22 Table 3 – Parameter values 23 Table 4 – TLS endpoint role assignment (at establishment phase) 25 Table 5 – TLS security session establishment - Results 26 Table 6 – Impact of StreamMode on TLS bearer traffic at external MG interface 26 Table 7 – Semantic of property sepplink 30 Table 8 – Semantic of property plslink 32 Table 9 – Variants of MGC-controlled TLS capability negotiations 39 Table 10 – H.248 control steps and possible control elements 58 Table 11 – Analogous terms between H.248.80 media configurations and TLS profile types 62 Table 12 – Example of a TLS MG profile 63 Table 13 – Example of an intersection between a TLS domain profile and a TLS MG profile 64 Table B.1 – TLS presentation language and derived term 69 Table III.1.a – Example of a TLS domain profile – Main part 79 Table III.1.b – Example of a TLS domain profile – Cipher Suites 80 Table III.2.a – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Main part 81 Table III.2.b – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Cipher Suites 82 Table III.3.a – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Main part 82 Table III.3.b – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites 83

List of Figures

Page

Figure 1 – Conventions for TLS endpoint types 16 Figure 2 – TLS transport modes 17 Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways 18 Figure 4 – Network model from H.248 entity point of view ("half call/bearer connection model") 20 Figure 5 – TCP/IP protocol stack with TLS protocol 20 Figure 6 – Two-termination context with a TLS termination 21 Figure 7 – Principle stimuli for triggering TLS session release 27 Figure 8 – Principal H.248 control steps for establishing a TLS-over-L4 bearer 57 Figure 9 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile 62 Figure 10 – Illustration of TLS profile concept 63 Figure A.1 – Simplified state model for H.248-based TLS basic session control 66 Figure B.1 – Performance monitoring – Measurement points related to traffic volume specific performance metrics 67 Figure B.2 – Data processing in the TLS record layer 68 Figure B.3 – TLS-RL PDU: variable layout of a protected compressed TLS fragment 69 Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG 71 Figure II.1 – Configuration for the example call flow 72 Figure II.2 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)) 74 Figure II.3 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment (decoupled end-to-end L4 connection)) 75 Figure II.4 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment, but interlinked end-to-end L4 connection establishment) 76 Figure II.5 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment, plus interlinked end-to-end L4 connection establishment) 77 Figure II.6 – Example TLS session release, initiated by remote TLS endpoint (MGC enforces immediate L4 connection release) 77 Figure II.7 – Example TLS session release, local TLS/L4 decoupled, but coupled end-to-end L4 connection release) 78 Figure II.8 – Example TLS session release, coupled TLS/L4 and end-to-end L4 connection release 79 Figure IV.1 – Successful establishment – Terminating side 85 Figure IV.2 – Successful establishment – Originating side 86 Figure IV.4 – Successful release – Terminating side 87 Figure IV.5 – Successful release – Originating side 87 Figure V.1 – Illustration of Protocol Layers Interlinkage property – Property "plslink = FALSE" 89 Figure V.2 – Illustration of Protocol Layers Interlinkage property – Property "plslink = TRUE" 89 Figure V.3 – Property "plslink = TRUE" – L4 incoming bearer establishment request 90 Figure V.4 – Property "plslink = TRUE" – TLS incoming bearer establishment request 90 Figure V.5 – Property "plslink = TRUE" – L4 stimuli 91 Figure V.6 – Property "plslink = TRUE" – TLS stimuli 91 Figure V.7 – Property "plslink = TRUE" – L4 incoming bearer release request 92 Figure V.8 – Property "plslink = TRUE" – TLS incoming bearer release request 92 Figure V.9 – Property "plslink = TRUE" – L4 stimuli 93 Figure V.10 – Property "plslink = TRUE" – TLS stimuli 93

Draft new ITU-T Recommendation H.248.TLS

Gateway control protocol: 
H.248 packages for control of transport security using TLS

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

Recommendation ITU-T H.248.TLS defines new ITU-T H.248 packages for support of secured IP transport bearer traffic using the Transport Layer Security (TLS) technology. In addition, this Recommendation identifies typical network use cases with different ITU-T H.248 media gateway variants, and provides example signalling flows.

# 1	Scope

{Editor's note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two IP transport connection endpoints. This Recommendation defines in general H.248 signalling elements for support of TLS in various options.

This Recommendation provides in more detail information about

- basic exchange architecture of keying information between control plane (including H.248 interface) and bearer plane;

- negotiation aspects: indication and determination of cryptographic capabilities between TLS endpoints;

- minimum amount of information carried by H.248 for establishing H.248 TLS/L4 terminations (NOTE 1);

NOTE 1 – Some information could be provisioned via management and there is also TLS information exchanged via the (L4) bearer interface with the remote TLS endpoint.

- TLS procedures in detail at the various TLS sublayers, i.e. for support of the

  - TLS record protocol

  - TLS handshake protocol

  - TLS change cipher spec protocol

  - TLS alert protocol

  - TLS application data protocol

- profiling of TLS services;

- specifying a set of cipher suites;

- consideration of H.248 MG modes of operation and connection models;

- SDP- versus Property-based TLS endpoint control

- {list to be updated before approval/consent …}

- The scope of this Recommendation is limited to {… any limitations?  …}

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would typically bespecified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

{Editor's note: Refs in yellow are subject of deletion …}

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (2013), Gateway control protocol: Usage of the revised SDP offer / answer model with H.248.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (11/2013), Gateway control protocol: RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations.

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), Gateway control protocol: TCP support packages.

[ITU-T H.248.DTLS]	Draft Recommendation ITU-T H.248.DTLS (201X), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T H.248.TLSPROF]	Draft Recommendation ITU-T H.248.TLSPROF (201X), Gateway control protocol: Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles.

[ITU-T T.38]	Recommendation ITU-T T.38 (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP)

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 6714]	IETF RFC 6714 (08/2012), Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Application data (TLS): the application layer data, as processed and transported by the underlying TLS protocol layer. An application layer service data unit correlates to a TLS-RL user data unit (clause 3.2.12)

3.2.2	TLS client [IETF RFC 5246]: the TLS entity that initiates a TLS security session to a TLS server [IETF RFC 5246].  The primary operational difference between the TLS server and TLS client is that the TLS server is generally authenticated, while the TLS client is only optionally authenticated.

NOTE – The primary operational difference between the TLS server and TLS client is that the TLS server is generally authenticated, while the TLS client is only optionally authenticated.

3.2.3	TLS domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.4	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.5	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.6	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.7	TLS fragment: the protocol data unit given by data element "fragment" as defined within the superior TLS protocol data structure "TLSPlaintext" according clause 6.2.1/[IETF RFC 5246].

3.2.8	TLS message: a unit of TLS-RL user data (clause 3.2.12) as produced by the TLS handshake protocol, the TLS alert protocol or the TLS change cipher spec protocol.

3.2.9	TLS MG profile: The TLS profile supported by the MG.

3.2.10	TLS profile: A selection of options from a set of TLS related protocol parameters and procedures.

NOTE – Also known as (nonstandard term) TLS protocol profile (example: the TLS profile defined by Annex E/[b-ETSI TS 133 310]).

3.2.11	TLS server [IETF RFC 5246]: the TLS entity that responds to requests for security sessions from TLS clients [IETF RFC 5246].

3.2.12	TLS-RL user data: the block of information exchanged between the TLS Record Layer and the four higher layer protocols (NOTEs 1, 2). A unit TLS-RL user data represents therefore either a TLS message (clause 3.2.8) or a unit of (TLS) application data (clause 3.2.1).

NOTE 1 – Refers to the four protocols that use the record protocol according clause 6/[IETF RFC 5246]: "… the handshake protocol, the alert protocol, the change cipher spec protocol, and the application data protocol".

NOTE 2 – Clause 6.2.1/[IETF RFC 5246]: "The record layer fragments information blocks into TLSPlaintext…".

3.2.13	TLS-RL PDU: the protocol data unit (PDU) given by the TLS protocol data structure "TLSCiphertext" according clause 6.2.3/[IETF RFC 5246]. It is the PDU of the TLS record layer. The TLS-RL PDU correlates consequently to the payload data of the underlying IP transport protocol.

3.2.14	TLS transparent forwarding: MG packet forwarding behaviour with the characteristic of TLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a TLS-PDU flow.

NOTE 1 – A TLS PDU relates to a TLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.1, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of TLS message integrity in the context of "TLS transparent forwarding". The MG might be TLS aware; e.g. support of TLS related statistics or event detection would not violate transparent forwarding behaviour.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2BUA | Back to Back User Agent (SIP) |
| --- | --- |
| DCCP | Datagram Congestion Control Protocol |
| DTLS | Datagram Transport Layer Security |
| IP | Internet Protocol |
| IWF | Interworking Function |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| LCD | Local Control Descriptor |
| LD | Local Descriptor Local Destination (Address) |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MP | Measurement Point |
| MSRP | Message Session Relay Protocol |
| NAT | Network Address Translation |
| NAT-T | NAT Traversal |
| PDU | Protocol Data Unit |
| PSTN | Public Switched Telephony Network |
| RD | Remote Descriptor |
| RTCP | RTP Control Protocol |
| RTP | Real-Time Transport Protocol |
| SA | Security Association Source Address |
| SCTP | Stream Control Transmission Protocol |
| SDES | Source Description (RTP) |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SRTP | Secure Real Time Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TPKT | (T.38) Transport Protocol Data Unit Packet |
| UDP | User Datagram Protocol |
| UDPTL | (T.38) Facsimile UDP Transport Layer (protocol) |
| UE | User Equipment |

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| TLS Est.req TLS Est.ack | Abstracted (TLS message/procedure independent) representationfor establishment requests/acknowledgements of new TLS security sessions. |
| TLS Rel.req TLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing TLS security sessions. |

## 5.2	TLS endpoint notations

The notion of endpoint represents different concepts in each network plane (H.248 as control protocol belongs to the network control plane and the MG bearer interface to the network user plane), see Figure 1:

Figure 1 – Conventions for TLS endpoint types

Usage in

- control plane (here H.248): H.248 is a control protocol and is thus part of the control plane. H.248 terminations/stream endpoint with TLS processing are denoted as TLS-enabled termination or stream endpoint (SEP) respectively;

- user plane (TLS): a TLS bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., TLS protocol termination) plus a service access point (i.e., L4+ access).
Further, TLS is a client/server type of protocol, a "TLS endpoint" (clause 3.2.5) provides thus either a client (see clause 3.2.2) or server (see clause 3.2.11) role.

Whether the (H.248) TLS-enabled SEP provides a "TLS endpoint" is dependent on the H.248 connection model and enabled MG-internal interworking functions (see clauses 6 and 7).

## 5.3	Important notation derived from TLS presentation language

See summary in Annex B.3.

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to TLS transport modes

The TLS protocol is independent of the underlying transport protocol stack. Multiple TLS transport modes are possible (Figure 2).

Figure 2 – TLS transport modes

This Recommendation is not dependent on any TLS transport mode, but uses TLS-over-TCP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as "non-TLS to non-TLS");

- Use case #1.2: an end-to-end TCP connection traversing two network domains with different transport security policies ("TLS", "no TLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "TLS to non-TLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as "TLS to TLS");

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as "TLS to TLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor's note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

### 6.2.2	Use case variations

#### 6.2.2.1	"TLS-to-TLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of TLS transparent forwarding according clause 3.2.14.

#### 6.2.2.2	"TLS-to-TLS application aware and decryption-encryption based forwarding"

This scenario is a variation of use case #1.3.

The MG may be required to be application aware in case of e.g. bearer-level ALG support (according to [ITU-T H.248.78]). The MG is located within a single security domain, therefore no change of the TLS profile. However, the MG is located at the edge of two network address domains, thus is requested for to support NAT-T and NAT.

Example use case:

MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the "TLS-to-TLS" interworking function is termed as "TLS B2BUA". It may be noted that this use case does not belong to the "TLS-to-TLS*" category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

## 6.3	Bearer connection network use cases with H.248 "TCP to non-TCP" gateways

Theoretical scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer.

For further study.

## 6.4	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

E.g., an MSRP gateway function according [ITU-T H.248.69] would lead to connection models with more than two H.248 terminations (bearer type here: either MSRP/TCP/IP or MSRP/TLS/TCP/IP). There would be a TCP proxy mode at TCP level (see clause 13.3 in [ITU-T H.248.84]). Such gateway topologies provide normally application aware type of functions (e.g. as media server), thus TLS sessions would be terminated by the MG.

It results in a type of "TLS to TLS*" interworking, such as use case #1.4 (clause 6.2.1).

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 4) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a TLS-capable IP host remote endpoint. Both TLS endpoints span a network transport security domain (here TLS domain).

NOTE  – There might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution (such as key management systems). Such kind of interfaces are out of scope of this Recommendation.

Figure 4 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

Figure 5 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 5 – TCP/IP protocol stack with TLS protocol

Figure 6 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 6 – Two-termination context with a TLS termination

# 8	TLS basic session control package

|  | Package Name: | TLS basic session control package |
| --- | --- | --- |
|  | Package ID: | tlsbsc (0x####) |
|  | Description: | This package provides the functionality to establish and /release a TLS (or DTLS, see [ITU-T H.248.DTLS]) based security session. Furthermore it includes the capability to block the start of TLS session establishment procedures at the MG bearer interface. NOTE 1 – The package design follows similar connection-oriented bearer (or media) protocols, such as the H.248 tcpbcc package [ITU-T H.248.TCP]. Terminology: "(D)TLS security session" correlates to "TCP bearer connection" and the "Q.1950 BNC" concept. NOTE 2 – The differences between DTLS and TLS are subject of [ITU-T H.248.DTLS]. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Security session establishment blocking

|  | Property Name: | Security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1 - Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG behaviour before and during TLS session establishment. NOTE 2 - See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table 2 Table 2 – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources are prepared (i.e., resources for TLS endpoint are reserved and allocated to H.248 termination/SEP); - TLS role assignment: still open, not yet decided; - TLS session still closed; - H.248 counterpart state = "Blocked" (Figure A.1); - Any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall be not acknowledged). - Any outgoing TLS session control procedures are not affected. - Signal EstBNC (clause 8.3.1) implicitly overwrites the value to "unblocked".. "unblocked" (0x0002); - MG shall be ready for incoming or/and outgoing TLS security session control procedures; - TLS role assignment: relates to an (initial) TLS server role effectively; - TLS session state: transitioning from closed to …; - H.248 counterpart state = "Idle" (Figure A.1); - TLS security session control options: - Incoming TLS ClientHello: TLS server role confirmed; - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role assigned. |
|  | Default: | "Unblocked" NOTE - H.248 profile specifications could redefine other default values. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Editor's note (2013-06 meeting): following items (same as in H.248.TCP) were identified and need to be resolved:

C1: the property semantics are focusing on the ingress direction (tbc); i.e., when property not relevant in outgoing direction, then explicit text should be added.

C2: Signal EstBNC should implicitly overwrite the property value, i.e., replacing blocked by unblocked. Correspondent updates should be added. 
Comment (2013-10): You cannot have a signal that changes the value of a property.

Given that the Signal would have to be sent we don’t think it’s an issue to send a property “unblocking” the termination and sending the establishment.

The interaction between sending the Estab or Release signal whilst in state “blocked” would have to be described.

Also what is the effect if the SEP has a TLS connection state of Established and the blocking property is suddenly set to “blocked”.

}

## 8.2	Events

### 8.2.1	TLS session state change ("BNC change")

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer (i.e., a TLS security session) has been established or a bearer has been released. The event is related to a simplified state transitioning model, which is described in Annex A. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Sublist of Enumeration |
|  | Optional: | Yes |
|  | Possible values: | Table 3 defines the possible values. Table 3 – Parameter values Value; Name; Final state (Annex A.3); Semantic. Est, [0x01]; Bearer Established; Established; Notify MGC of successful TLS security session establishment.. Rel, [0x05]; Bearer Released; Idle; Notify MGC of successful TLS security session release.. |
|  | Default: | Type=[Est,Rel] The MGC would be then automatically notified about all state transitions when subscribing to event tlsbsc/BNCChange. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TLS bearer. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est [0x01] Bearer Established Rel [0x05] Bearer Release |
|  | Default: | None |

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TLS security session: the MG takes the TLS client role and initiates the start of a TLS session establishment by sending a TLS ClientHello request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TLS security session: the MG sends a TLS alert close_notify request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	TLS endpoint creation

The MGC is responsible for creating a TLS session enabled SEP in the MG. This may be indicated on the H.248 interface through the use of the SDP "m=" element <proto>.

The MG shall reserve "TLS bearer resources" (such as a "TLS encryption/decryption function") and allocates them to the SEP. The TLS role (client/server) is not (yet) determined.

NOTE 1 – This functionality is synonym to the "bearer type indication" semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TLS endpoint role assignment

{Editor's note (2013-10 meeting): the "TLS endpoint role" concept should be replaced by originating and terminating TLS session control procedures.}

The TLS role, client or server, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The TLS endpoint role assignment shall basically prepare (but not necessarily trigger) the MG for an incoming or outgoing TLS security session establishment (clause 8.6.3) procedure. Table 4 summarizes MGC actions and their results in TLS role assignments.

Table 4 – TLS endpoint role assignment (at establishment phase)

| No. | TCP role | H.248 indication | MG behaviour |
| --- | --- | --- | --- |
| 1 | Not yet assigned | H.248 SEP state "Blocked", either due to explicit signalling of bceb property or correspondent default value | Any incoming TLS PDU shall be silently discarded. |
| 2 | TLS server | H.248 SEP state "Idle" by "unblocking" the SEP | Expect incoming (TLS) ClientHello, subsequent completion of the establishment related handshake procedure. |
| 3 | TLS client | H.248 SEP state "Idle" of SEP plus signal EstBNC enabled. | Start of TLS session establishment related handshake procedure. |

### 8.6.3	TLS security session establishment

The incoming and outgoing side of TLS security session protocol procedures is given by the assigned TLS role (see above clause 8.6.2) by the MGC.

The MG shall initiate the TLS security session setup according to the indicated role and the TLS security session setup handshake is performed by the MG without any further involvement of the MGC. The end of the TLS security session establishment phase may be summarized according Table 5:

Table 5 – TLS security session establishment - Results

| TLS role | Result of TLS security session establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tlsbsc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent error code. {Editor's note (2013-11): error code is not possible … because bearer control procedures are still ongoing when H.248 command.reply is already sent. Either a Notify or ServiceChange based method …} |
| Server (i.e., incoming establishment side) | Successful | As client side. |
|  | Unsuccessful | As client side. |

### 8.6.4	TLS application data transfer

#### 8.6.4.1	MG external

The TLS session is ready for application data transfer when remote and local TLS endpoints have both completed the TLS handshake procedures for session establishment.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TLS control information used to establish or close the TLS session. The requested MG behaviour is defined by Table 6:

Table 6 – Impact of StreamMode on TLS bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All TLS PDUs sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All TLS PDUs received |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |

#### 8.6.4.2	MG internal

This clause is only relevant for H.248 contexts with TLS SEPP(s).

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the SEPP interlinkage property of the tlsesc package (see clause 9.1.1) should be used.

### 8.6.5	TLS session release

The trigger for a TLS session termination might come from different sources, see Figure 7:

Figure 7 – Principle stimuli for triggering TLS session release

- Bearer control protocol – Security session: The TLS session is terminated by the remote TLS endpoint by sending a (TLS) close_notify alert or a fatal alert;

- Bearer control protocol – Transport connection: The underlying L4 transport connection is terminated;

- TLS protocol: A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicates the closure of the TLS session by the signal tlsbsc/RelBNC;

- The release of the security session of the partner SEP triggers the release autonomously in the MG (Note: not supported by base package tlsbsc, related to inter-linkage property tlsesc/sepplink, see clause 9.1.1);

- In case the MG acts in an application aware mode, the application part in the MG may indicate the release of the TLS session.

#### 8.6.5.1	Bearer control protocol – Security session: TLS alert protocol

{Editor's note (2013-06): the procedure overlapps with the TLS session maintenance package procedures. The text is not yet aligned with clause 10.6.x. Need to be fixed.}

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a received TLS close_notify the TLS endpoint shall reply a TLS close_notify on its own and the TLS session is closed.

Impact on underlying IP transport connection:

- The MG shall not autonomously initiate the release of the (if connection-oriented) IP transport connection as well.

NOTE - extension package tlsesc allows to tightly couple bearer control procedures for L4 and TLS, see property tlsesc/plslink in clause 9.1.2.

- If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}. Handling of the TLS-specific AlertDescription as received in the TLS alert message is for further studies.

Impact on H.248 SEP and possibly SEPP:

- The SEP as such still exists after a successful TLS session release (state Idle). Removal of the SEP follows usual H.248 SUBtract.req or MODify.req commands.

- Property tlsesc/sepplink (clause 9.1.1) allows the trigger of a TLS session release at the partner SEPP.

#### 8.6.5.2	Bearer control protocol – Transport connection: L4 bearer release

In case the underlying IP transport connection is released, e.g. due to a received TCP-FIN or TCP-RST in case of TCP transport, then

- default mode (base package): the MGC will be notified (by the MG) about the L4 state change (e.g., either by subscription to a correspondent event, an error or ServiceChange indication) and provide instructions for further proceeding with the TLS layer.

- enhanced mode (extension package): the MG may autonomously and immediately initiate the closure of the TLS session (dependent on property setting of tlsesc/plslink). The release procedures of the IP transport connection shall be delayed in order for  the TLS session to be successfully released (see also clause V.4.2.2.1).

NOTE - extension package tlsesc allows to couple the bearer control procedures for L4 and TLS, see clause 9.1.2.

If requested, the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.3		TLS protocol: non recoverable error condition detected

{Editor's note (2013-06 meeting): below information might overlap with the TLS session maintenance package … }

In case an error condition is detected locally in the TLS protocol layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to "fatal" and the AlertDescription is provided accordingly.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.4	H.248: MGC driven TLS session release

In case the H.248 signal tlsbsc/RelBNC is received, the TLS session closure shall be initiated by the MG by sending a TLS close_notify alert towards remote TLS entity.

#### 8.6.5.5	MG autonomous release initiated on partner SEP

This capability is not supported by the base package, it is subject of extenstion package tlsesc (property sepplink), see clause 9.

#### 8.6.5.6	Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session shall be closed autonomously by sending a TLS alert (close_notify).

The underlying IP transport shall be not affected (base package behaviour).

# 9	TLS extended session control package

|  | Package Name: | TLS extended session control package |
| --- | --- | --- |
|  | Package ID: | tlsesc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TLS bearer session control behaviour across a "horizontal" Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - TLS/L4 bearer control behaviour across "vertical" protocol layers, rather instead of decoupled, individual control of TLS security sessions and their underlying IP transport connection. |
|  | Version: | 1 |
|  | Extends: | tlsbsc |

## 9.1	Properties

{Editor's note (2013-10 meeting): both properties ares dependent on the discussion the proposal for a generic "interlinkage" property, see  C-258 & C-264 ("H.248.SEPLINK"). If H.248.SEPLINK would be agreed, then following updates would be required for H.248.TLS:
a) clauses 9.1.1 & 9.1.2: deleted
b) new clause 9.5.1 concerning dedicated error code of TLS-specific interlinkage.
c) clause 9.6: updated by refererring to H.248.SEPLINK property
d) Appendix V: updated.).

### 9.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0002) |
|  | Description: | The scope of this property relates to a Stream Endpoint Pair (SEPP; see definition in (clause 3.2.11) [ITU-T H.248.1]). This property defines if a detected condition leading to the establishment or release of the TLS security session will trigger an MG-autonomous (see clause 3.2.2/H.248.TCP) establishment or release of the TLS security session of the partner SEP as well. Only TLS protocol control information is part of such conditions. Normal TLS data traffic is not affected by this property. Semantic scope limitations: - It only applies to the two phases of TLS bearer session establishment and release; - only incoming direction; - only the first stimuli of each TLS bearer session control procedure (i.e., first incoming (TLS) ClientHello; first incoming (TLS) close_notify); - SEP-related properties, i.e., there are four value combinations (per Property) from SEPP perspective (in order to address network asymmetries) {Editor's note (2013-06 meeting): following items (as in H.248.TCP) were identified and need to be resolved: C1: the semantical scope needs to be more specific, particularily bullet items 2 to 4 (e.g., which bearer level procedure triggers in particular a correspondent procedure at the partner SEP). } |
|  | Type: | Sub-list of Boolean (for separated support of establishment and release phase) |
|  | Possible values: | Table 7 defines the protocol semantic per value and dependent on connection control phase. See model in Figure 5 concerning the used SEP names. Table 7 – Semantic of property sepplink Value; MG semantic for TLS bearer session. Establishment; Release. True; a) Incoming: An incoming TLS bearer session establishment request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session establishment request at the partner SEP T2(S1). b) Outgoing: An MGC-initiated outgoing TLS bearer session establishment at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).; a) Incoming: An incoming TLS bearer session release request at SEP T1(S1) shall lead to an autonomous outgoing TLS bearer session release request at the partner SEP T2(S1). b) Outgoing: Any action (e.g. by MGC) in closing the outgoing TLS bearer session at SEP T1(S1), shall not impact the TLS session state of SEP T2(S1).. False; The state of TLS bearer session endpoint SEP T1(S1) shall not impact the state of TLS bearer session endpoint SEP T2(S1) (and vice versa).. |
|  | Default: | [False, False] (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic ("default MG behaviour"): TBD

}

### 9.1.2	Protocol Layers Interlinkage

|  | Property Name: | Protocol Layers Interlinkage |
| --- | --- | --- |
|  | Property ID: | plslink (0x0003) |
|  | Description: | The semantical scope of this property relates to a single Stream Endpoint (SEP) with regards to a portion of the bearer specific protocol stack. This property defines whether bearer control procedures of the TLS security session shall or shall not impact the bearer control procedures of the underlying IP transport connection (e.g., TCP, SCTP) as well. This property serves the two fundamental use cases of: - Tightly coupled mode: the holding time of the TLS security session correlates directly with the underlying IP transport connection; e.g. a TLS session termination will always imply the release of the TCP or SCTP transport connection; - Decoupled mode: the TLS security session is disjoint from bearer control procedures of the underlying IP transport connection. For instance: a TCP-based communication service with alternate phases of encrypted and clear data, a TLS/TCP (or a DTLS/DCCP) session where the loss of the security session should not lead to the stop of data transfer, DTLS with UDP, DTLS-SRTP, different aimed behaviour between TLS/SCTP and DTLS/SCTP, etc. The default behaviour is aligned with the base package tlsbsc, which is the decoupled mode. The property only takes effect in case of connection-oriented IP transport protocols (such as TCP, SCTP, DCCP). The property shall be ignored in case of connectionless IP transport protocols (UDP). ToBeChecked |
|  | Type: | Sub-list of Boolean (for separated support of establishment and release phases) |
|  | Possible values: | Table 8 defines the protocol semantic per value and dependent on connection control phase. Table 8 – Semantic of property plslink Value; MG semantic for protocol stack "TLS-over-L4". Establishment; Release. True; a) Incoming: An incoming L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. The SEP shall then sent an outgoing TLS bearer session establishment request (i.e., a TLS ClientHello). b) Outgoing: An outgoing L4 transport connection establishment shall trigger the establishment of TLS security session, as soon as the L4 transport connection is successfully established. See above.; a) Incoming: An incoming TLS bearer session release request shall trigger the outgoing L4 transport connection release request, as soon as the TLS security session is successfully released. b) Outgoing: An outging TLS bearer session release request shall … see above.. False; Any bearer control procedures related to TLS security sessions shall not impact the underlying L4 transport connection (and vice versa).. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tlsbsc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic ("default MG behaviour"): TBD

}

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	TLS security session establishment

{Editor's note (2013-06 meeting): following items (as in H.248.TCP) were identified and need to be resolved:

C1: properties are of type list, not  a single value, hence all procedural sections need to be updated from value False or True to [X,Y] combinations!).

}

#### 9.6.1.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session establishment request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session establishment request shall be propagated to the partner SEP, triggering there an outgoing TLS ClientHello message.

#### 9.6.1.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.1.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing (NOTE 2) L4 transport connection establishment request (such as a TCP SYN packet) shall immediately trigger an outgoing TLS security session establishment request, after a successfully established L4 bearer.

NOTE 1 – Appendix V.4.2.1.1 provides a signalling example.

NOTE 2 – E.g., either MGC driven or in case of e.g. TLS-over-TCP might be also the [ITU-T H.248.TCP] property tcpbsc/sepplink in use.

### 9.6.2	TLS security session release

#### 9.6.2.1	Stream endpoint pair interlinkage disabled

If the property sepplink is set to False, then the incoming TLS security session release request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled

If the property sepplink is set to True, then the incoming TLS security session release request shall be propagated to the partner SEP, triggering there an outgoing TLS closure_notify alert message.

#### 9.6.2.3	Protocol layers interlinkage disabled

If the property plslink is set to False, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.4	Protocol layers interlinkage enabled

If the property plslink is set to True, then an incoming or outgoing TLS security session release request shall immediately trigger an outgoing L4 bearer connection release request, after a successfully release TLS session.

NOTE 1 – Appendix V.4.2.2 provides signalling examples.

# 10	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x####) |
|  | Description: | When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. The aimed negotiation behaviour would be then provisioned in the MG (but the managed objects used does not necessarily correspond with the H.248 signalling objects of this package). This package provides tools for the MGC to control bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of: - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. There are two options for negotiation of TLS protocol configurations: - MGC may indicate a TLS profile (or multiple) as such; - MGC may control individual TLS protocol parameters. Both methods shall be mutually exclusive in this package version. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0001) |
|  | Description: | This property identifies the TLS domain profile that shall be ideally used by the MG to setup the TLS security session. The actually applied, so called TLS endpoint profile is the match result of locally supported TLS capabilities by the MG (termed as TLS MG profile) with requested TLS domain profiles. It should be noted that the normal case would be aligned TLS profile capabilities, i.e.: The applied "TLS endpoint profile" = requested "TLS domain profile". |
|  | Type: | Sub-list of String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.2	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0002) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS protocol version related IETF RFCs (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the "0x", and if needed, a leading "0" is added for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination){Editor's note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited. => (2013-06): might be addressed in clause 10.6.2} {Editor's note (2013-06 meeting): following items were identified and need to be resolved: C1: clarify possible interaction between non-root and root termiantion usage, or define exclusive approach, etc). This comment is relevant for all package properties.} {Comment to clause 10 as such (2013-06 meeting): there are - two negotiation methods ("profile" vs "individual") and - two termination levels ("Root" vs "non-Root") There are four combinations, and the mutual exclusion should be revisited from that perspective.} |
|  | Characteristics: | Read/Write |

### 10.1.3	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0003) |
|  | Description: | This property indicates the cipher suites and their precedence (in decending order of preference) for the negotiation with the remote TLS endpoint. |
|  | Type: | Sublist of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA (http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The "0x" shall be omitted and if needed, a "0" is used for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned {Editor's note (2013-11 meeting): discussion comment: "RFC5246 indicates that TLS_RSA_WITH_AES_128_CBC_SHA is mandatory to implement should we specify a default per version?" consideration "The package should be orthogonal to TLS versions. Otherwise: TLS version specific defaults?" } |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.4	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0004) |
|  | Description: | This property declares the list of compression methods and their preference (in descending order) for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers (see http://www.iana.org/assignments/comp-meth-ids/comp-meth-ids.xml). |
|  | Default: | “CompressionMethod.null” |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.5	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0005) |
|  | Description: | This property declares the support for the TLS session resumption. |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported True Session Resumption shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.6	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0006) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported. |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported True Renegotiation shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.7	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0007) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.8	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0008) |
|  | Description: | This property declares if the MG – if acting as a TLS server – shall authenticate the remote TLS client. |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS server, the client shall not be authenticated. True In case the MG acts as a TLS server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

### 10.6.1	General default behaviour (MGC does not influence TLS capability negotiations)

Package tlscn is optional for H.248 profile specifications with TLS support. When the package is not used, then the MG shall independently negotiate TLS configuration(s) with remote TLS endpoints.

Such MG-autonomous TLS capability negotiations shall be compliant to [IETF RFC 5246]. The negotiations would be based on the TLS MG profile and possibly preprovisioned negotiation policies.

### 10.6.2	MGC-controlled TLS capability negotiations: Options

There are two options for MGC-controlled TLS capability negotiations, with the aim to determine the TLS domain profile instance, which is to be used by the MG to form the TLS endpoint profile (by also taking into account the TLS MG profile), see Table 9.

Table 9 – Variants of MGC-controlled TLS capability negotiations

| Method | H.248 elements | Protocol procedures, see: |
| --- | --- | --- |
| a) TLS profile approach | Property: tlscn/dpid | clause 10.6.4 |
| b) Individual TLS protocol parameter control | Properties: tlscn/tlsv tlscn/cs tlscn/cm tlscn/ssr tlscn/srsc tlscn/rp tlscn/car | clause 10.6.5 |

Both methods shall be mutually exclusive in this package version. Thus, an H.248 profile with tlscn v1 package support should select one method.

### 10.6.3	Determining TLS Related Capabilities

#### 10.6.3.1	Auditing supported TLS protocol profiles

The MGC may access supported TLS protocol profile(s) by auditing property tlscn/dpid. The identifier of a particular TLS protocol profile relates to a reference on a TLS protocol profile description, which might be for instance explicitly part of an H.248 profile definition or documented anywhere else.

#### 10.6.3.2	Auditing individual supported TLS protocol capabilities

The MGC may determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr). Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS versions supported by the MG without any specific sequence.

The Supported Cipher Suite property shall include the cipher suite TLS_NULL_WITH_NULL_NULL ("0000"), which is used as the initial cipher suite during the first handshake for the TLS session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

### 10.6.4	Indication of TLS profile(s) to MG by MGC

The MGC may request TLS domain profile(s) support from the MG via property dpid, leading to following MG behaviour:

- Built intersection between locally supported TLS capabilities (the "TLS MG profile") with indicated TLS domain profile(s), resulting in a TLS endpoint profile (see also clause 14);

- The TLS endpoint profile shall be the baseline for TLS negotiations with the remote TLS entity (during TLS security session establishment/resumption procedures).

### 10.6.5	Indication of individual TLS protocol capabilities to MG by the MGC

In this case the TLD Domain profile instance to be used by the MG is (partially) defined by the MGC by the actually signalled tlscn/x properties with "x != dpid". Those tlscn/x properties, which are not explicitly signalled by the MGC assume their values as provisioned in MG's TLS MG profile.

#### 10.6.5.1	TLS protocol parameter "TLS version"

The list of TLS versions used for the TLS procedures of the session is determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned (ie., the property value for dpid) on the MG as typically default configuration.

NOTE – The TLS endpoint profile again (see also clause 14) relates to the intersection/match of TLS MG profile (as provisioned on MG) and individual explicitly signalled tlscn/x properties with "x != dpid".

In case the MGC signals the property TLS Versions (tlsv), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG acts as a TLS client, the highest TLS versions as defined by the specific TLS endpoint profile are included into the initial TLS handshake message (ClientHello). In case the MG acts as a TLS server, the MG shall select the highest TLS version that is present in the list of TLS versions offered from the TLS client and in the TLS versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS handshake is regarded as failed.

Unsuccessful TLS negotiations should be reported by error code #449 (Unsupported or Unknown Parameter or Property Value). (TBC)

#### 10.6.5.2	TLS protocol parameter "cipher suite"

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Chipher Suites (cs), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS version.

#### 10.6.5.3	TLS protocol parameter "compression method"

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Compression Methods (cm), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle as the negotiation of the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS version.

#### 10.6.5.4	TLS protocol procedure "client authentication"

The MG's TLS server implementation shall support client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid).

In case the MGC signals property Client Authentication Required (car), the MG shall use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

#### 10.6.5.5	TLS protocol procedure "self-signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute "a=fingerprint". If present in the Remote Descriptor, the client certificate shall be self-signed and shall be verified against the fingerprint.

#### 10.6.5.6	TLS protocol procedure "signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no SDP fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile. This list of root certificates is supposed to be configured per dpid on the MG.

#### 10.6.5.7	TLS protocol procedure "server authentication"

In case the TLS endpoint in the MG is configured as a TLS server, the MG shall provide the server certificate to the client as defined by the TLS domain profile.

#### 10.6.5.8	TLS protocol procedure "session resumption"

Use cases for TLS session resumption (with H.248 gateways) are for further studies.

#### 10.6.5.9	TLS protocol procedure "renegotiation support"

The TLS implementation in the MG shall provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile.

In case the MGC signals property Support of Renegotiation of the Security Context (srsc), the MG shall use this value instead.

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, taking into account property Renegotiation Period (rp).

In case the TLS endpoint profile disables renegotiation, the MG shall never initiate a renegotiation. If receiving a renegotiation request from the remote TLS endpoint, the MG shall respond with a TLS no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS version.

If renegotiation shall be supported, the MG shall accept and process a renegotiation request from the remote TLS endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to value "0", no renegotiation shall be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS handshake completed successfully. On expiration of the timer the MG shall trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS version. In case the MG receives a trigger for the renegotiation from the remote TLS endpoint while this timer is running, the MG shall cancel and restart this timer.

{Editor's note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.
However, to be considered: "If the MGC provides the property tlscn/rp, then the MG will use this value instead of the provisioned default TLD Domain profile's "rp" value }

# 11	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x####) |
|  | Description: | This package defines capabilities for support of TLS alerts, which are exchanged between the remote and MG local TLS endpoint, following the TLS alert protocol. NOTE: Appendix V provides some background information concerning TLS alert protocol handling by H.248 gateways. This package version is limited to the so-called TLS error alerts, as defined by clause 7.2.2/[IETF RFC 5246]. The package provides capabilities for MGC triggered sending of TLS alerts by the MG, as well as a reporting mechanism to notify the MGC about received TLS alerts by the MG. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

### 11.2.1	TLS Error Alert

|  | Event Name: | TLS error alert |
| --- | --- | --- |
|  | Event ID: | swa (0x0001) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the termination of the TLS session. The abnormal condition may be detected on the local or on the remote TLS endpoint. The event is limited to TLS error alerts and excludes the TLS closure alert event. The event shall be set at stream level. |

#### 11.2.1.1	EventsDescriptor parameters

None

#### 11.2.1.2	ObservedEventsDescriptor parameters

##### 11.2.1.2.1	Bearer level Alert Initiator

|  | Parameter Name: | Bearer level alert initiator |
| --- | --- | --- |
|  | Parameter ID: | blai (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | "local" (0x0001) Abnormal condition detected by the MG local TLS endpoint "remote" (0x0002) Abnormal condition detected by the remote TLS endpoint and reported to local TLS endpoint. |
|  | Default: | NA |

##### 11.2.1.2.2	Error Alert Type

|  | Parameter Name: | Error alert type |
| --- | --- | --- |
|  | Parameter ID: | eat (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition by referencing the TLS error alert type. The codepoints registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | N/A |

## 11.3	Signals

### 11.3.1	MGC triggered TLS error alert

|  | Signal Name: | MGC triggered TLS error alert |
| --- | --- | --- |
|  | Signal ID: | mgcea (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates "fatal", the session shall be terminated. NOTE - the TLS "close_notify(0)" alert is excluded because it does not belong to the TLS error alert category. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 11.3.1.1	Additional parameters

##### 11.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246] , i.e., either a warning or fatal alert severity. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for "AlertLevel", refer to [IETF RFC 5246]. |
|  | Default: | NA |

##### 11.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | NA |

## 11.4	Statistics

None.

## 11.5	Error codes

None.

## 11.6	Procedures

Generally, there are TLS alert specific H.248 procedures due to the nature of the TLS alert protocol, see Appendix V.

{Editor's note (2013-11): it was requested to expand the description by explicit text in all sub-clauses starting with "As per … clause …".}

### 11.6.1	TLS warning error alerts

#### 11.6.1.1	General behaviour

The TLS session should be kept active (and not released) when TLS alerts with warning severity are exchanged between local and remote TLS endpoints.

#### 11.6.1.2	MG received TLS error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at warning level, it shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai to equal "remote".

#### 11.6.1.3	MG locally generated TLS error alerts and sent to remote TLS endpoint

As per the previous clause 11.6.1.2, but a tagging mgea/blai equal "local".

#### 11.6.1.4	MGC triggered TLS error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alerts towards the remote TLS endpoint.

### 11.6.2	TLS fatal error alerts

#### 11.6.2.1	General behaviour

Whenever a TLS fatal error alert is exchanged between local and remote TLS endpoints, there will be always a subsequent, immediate release procedure of the TLS session (see also Appendix V.2).

#### 11.6.2.2	MG received TLS fatal error alerts from remote TLS endpoint

As per clause 11.6.1.2. The subsequently expected TLS closure_notify alert indicates a TLS session release request. The received TLS closure_notify alert would then trigger the TLS basic session control package (tlsbsc) procedures according clause 8.6.5.1.

#### 11.6.2.3	MG locally generated TLS fatal error alerts and sent to remote TLS endpoint

As per clause 11.6.1.3. The MG shall send a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.6.2).

#### 11.6.2.4	MGC triggered TLS fatal error alert, sent by the MG to remote TLS endpoint

As per clause 11.6.1.4. The MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure is either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.6.2).

### 11.6.3	Lower layer issues

There might be issues in the protocol layers below the TLS layer.

Example use case:

- Established IP bearer path, i.e. there is an active TLS session;

- The underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- Hence, there would be still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective;

- And therefore the TLS session would be interrupted, no TLS alert could be sent to the remote side.

Procedures for the maintenance of lower layer issues are out of scope of this package, rather subject of exsting H.248 error handling capabilities.

# 12	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x####) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TLS protocol. The statistics are applicable as long as the MG is aware that the H.248 Stream carries TLS traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 12.1	Properties

None.

## 12.2	Events

None.

## 12.3	Signals

None.

## 12.4	Statistics

### 12.4.1	Number of Received Application Data TLS-Fragments

|  | Statistic Name: | Number of Received Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recadfrag (0x0001) |
|  | Description: | Provides the number of application data TLS fragments received on the Termination or Stream since the statistic has been set. Only application data TLS fragments are counted which are passed from the TLS record layer to the application layer. A received TLS-PDU is regarded as application data if the TLS header field "type" of the received TLS-PDU has been set to "application_data(23)". At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.2	Number of Sent Application Data TLS-Fragments

|  | Statistic Name: | Number of Sent Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentadfrag (0x0002) |
|  | Description: | Provides the number of application data TLS fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as application data if the TLS header field "type" of the resulting TLS-PDU is set to "application_data(23)". At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.3	Number of Received TLS Message Fragments

|  | Statistic Name: | Number of Received TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | rectmfrag (0x0003) |
|  | Description: | Provides the number of TLS message fragments received on the Termination or Stream since the statistic has been set. Only TLS message fragments are counted which are passed from the TLS record layer to the change_cipher_spec protocol, the alert protocol or the handshake protocol. A received TLS-PDU is regarded as an TLS message if the TLS header field "type" of the received TLS-PDU has been set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.4	Number of Sent TLS Message Fragments

|  | Statistic Name: | Number of Sent TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | senttmfrag (0x0004) |
|  | Description: | Provides the number of TLS message fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as TLS message if the TLS header field "type" of the resulting TLS-PDU is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.5	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0005) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the application layer above the TLS record layer. Received application data messages from the remote TLS endpoint will have the TLS header field "type" set to "application_data(23)" and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.6	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0006) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.7	Number of Received TLS Message Octets

|  | Statistic Name: | Number of Received TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmo (0x0007) |
|  | Description: | This statistic logs the number of TLS message octets that are delivered to the change_cipher_spec protocol, the alert protocol or to the handshake protocol above the TLS record layer. Received TLS messages from the remote TLS endpoint will have the TLS header field "type" set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The TLS messages and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Application data messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.8	Number of Sent TLS Message Octets

|  | Statistic Name: | Number of Sent TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmo (0x0008) |
|  | Description: | This statistic logs the number of octets received from the change_cipher_spec protocol, the alert protocol or the handshake protocol. The octets are counted before the TLS messages are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.9	Number of Sent Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Sent Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | sentadpduo (0x0009) |
|  | Description: | This statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", sent from the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress Application Data TLS PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.10	Number of Received Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Received Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | recadpduo (0x000a) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress Application Data TLS PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.11	Number of Sent TLS Message PDU Octets

|  | Statistic Name: | Number of Sent TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmpduo (0x000b) |
|  | Description: | This statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets sent from the Termination or Stream since the Termination or Stream has existed and the statistic has been set. The octets represent the egress TLS Message PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.12	Number of Received TLS Message PDU Octets

|  | Statistic Name: | Number of Received TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmpduo (0x000c) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress TLS Message PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.13	Number of Sent Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | sentadpco (0x000d) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.14	Number of Received Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Received Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | recadpco (0x000d) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.15	Number of Sent TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | senttmpco (0x000f) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress protected and compressed TLS messags of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.16	Number of Received TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Received TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | rectmpco (0x0010) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress protected and compressed TLS message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.17	Number of Octets of Compressed Application Data Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | sentadcfo (0x0011) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.18	Number of Octets of Compressed Application Data Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | recadcfo (0x0012) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.19	Number of Octets of Compressed TLS Message Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | senttmcfo (0x0011) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress compressed TLS Messages of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.20	Number of Octets of Compressed TLS Message Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | rectmcfo (0x0012) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress compressed TLS Message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 12.5	Error codes

None.

## 12.6	Procedures

{Editor's note (2013-11 meeting): clause 12.6 is not yet updated}

### 12.6.1	Ingress TLS traffic – Statistic "TLS Packets Received"

Every incoming L4 packet, carrying a TLS fragment, which is successfully delivered to its H.248 Context and H.248 TLS Stream/Termination is counted by Statistic tlstv/recfrag.

### 12.6.2	Ingress TLS traffic – Statistic "TLS Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/tlsor.

### 12.6.3	Ingress TLS traffic – Statistic "TLS Application Data Octets Received"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all received TLS-over-L4 packets according Statistic tlstv/recado.

### 12.6.4	Egress TLS traffic – Statistic "TLS Packets Sent"

Every outgoing TLS-over-L4 packet, sent from a H.248 TLS Stream/Termination, is counted by Statistic tlstv/sentfrag.

### 12.6.5	Egress TLS traffic – Statistic "TLS Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 according to Statistic tlstv/tlsos.

### 12.6.6	Ingress TLS traffic – Statistic "TLS Application Data Octets Sent"

The measurement represents the volume of all TLS flows of an H.248 Stream, i.e. across all sent TLS-over-L4 packets according Statistic tlstv/sentado.

# 13	Package-less TLS control

Usage of dedicated SDP elements is principally not excluded in package-based TLS session control. However, such SDP elements shall be avoided which could semantically overlap or lead to ambiguities with H.248.

## 13.1	Related to TLS session establishment

### 13.1.1	Overview – principal H.248 control steps

Figure 8 indicates all possible control steps (from H.248 perspective) concerning the establishment of TLS-over-L4/IP bearer connections in environments demanding NAT traversal support. The dashed boxes are L4-dependent (e.g., in case of TCP see [ITU-T H.248.TCP, H.248.69, H.248.84]):

Figure 8 – Principal H.248 control steps for establishing a TLS-over-L4 bearer

It is apparent that some steps might be combined (e.g., 1, 2 and 7) and that specific steps might be optional (e.g., 3, 5, 8, 10).

Table 10 provides a preliminary inventory of possible control elements from H.248 perspective (at the example of TCP as L4 protocol). The column "H.248/SDP element" refers to "package-less TLS control".

Table 10 – H.248 control steps and possible control elements

| H.248 control step |  | H.248 property | H.248/SDP element | Comments |
| --- | --- | --- | --- | --- |
| Establishment phase I: "IP transport connection establishment" |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | "m="-line <proto> (clause 13.4 / [ITU-T H.248.84], [ITU-T H.248.69]) | SDP may be ambiguous due to "protocol stack" concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property, e.g., TCP: tcpbcc/bceb | "a=setup:holdconn" | NOTE 3 |
| 4 | Start IP Transport Connection Establishment Procedure (e.g. TCP active or passive OPEN) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to [ITU-T H.248.84] "a=setup:" semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | [ITU-T H.248.37] | "a=setup:" for [ITU-T H.248.84] NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State "EstablishedL4") | Event, e.g., TCP: tcpbcc/BNCChange | — | MGC notification might be optional. |
| Establishment phase II: "IP transport security session establishment" |  |  |  |  |
| 7 | Bearer type (L4+ = 'TLS') indication to SEP | Indirect: package elements used from TLS-related H.248 packages | "m="-line <proto> | NOTE 2. SDP may be ambiguous due to "protocol stack" concept (NOTE 1). |
| 8 | Block / Unblock L4+ Bearer Control Procedures at SEP | Property: tlsbsc/bceb | None | I.e., handling of TLS handshake protocol TLS cipher change protocol TLS alert protocol |
| 9 | Start IP Security Session Establishment Procedure ("TLS negotiations") | Event: tlsbsc/BNCChange Signal: tlsbsc/EstBNC | None | The above TLS protocols are enabled |
| 10 | Local L4+ NAT traversal support | [ITU-T H.248.78] | e.g., MSRP: "a=msrp-cema" | NOTE: L4+ NAT-T may be also requested for non-TLS. |
| 11 | Successfully established IP Transport Security Session (State "EstablishedTLS") | Event: tlsbsc/BNCChange | — |  |
| Communication phase: |  |  |  |  |
| 12 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP "m=" line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well known issue of this element. NOTE 2 – Step 7 may be merged with step 1 in case of tightly coupled "TLS/L4" control. NOTE 3 – See Table X/[H.248.TCP] due to example "L4 = 'TCP'" here. |  |  |  |  |

### 13.1.2	Step 1: Bearer endpoint creation

Not SDP related, based on H.248 Command request.

### 13.1.3	Step 2: Bearer type indication for IP transport "L4/L3"

The SDP-based approach of using the SDP "m=" line <proto> element as transport indicator should be used. The SDP method shall work for all required TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]).

Conditional approach, dependent on:

- C2.1: unambiguous indication of transport protocol "L4" (Issue: "m="-line <proto> relates to a protocol stack based indicator, rather than a distinct identifier to a specific L4 protocol).
Warning: possible semantical interaction with step 7!

- C2.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

- C2.3: could imply additional TCP bearer connection establishment (step 4) when using [ITU-T H.248.69].

### 13.1.4	Step 3: Block / Unblock L4 bearer control procedures at SEP

This is out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 13.1.5	Step 4: Start IP transport connection establishment procedure

Relevant for connection-oriented IP transport protocols, which are used as TLS transport modes (see clause 6.1/[ITU-T H.248.TLSPROF]), but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

Warning: possible semantical interaction with step 9 (see clause 11.2.5/[ITU-T H.248.TCP] in case of "L4 = 'TCP'")!

### 13.1.6	Step 5: Local L4/L3 NAT traversal support

This is a crucial step because TLS will fail without any successfully established IP transport connection. However, this is out of scope of this Recommendation. There are dedicated L4/L3 NAT-T support methods, such as [ITU-T H.248.37], [ITU-T H.248.50] or [ITU-T H.248.84].

It should be noted that the above example NAT-T mechanisms are primarily enabled using H.248 packages ("and not via LD/RD-embedded SDP signalling"). See also clause 11.2.6/[ITU-T H.248.TCP] in case of "L4 = 'TCP'".

### 13.1.7	Step 6: Reporting successfully established IP transport connection

Not supported by SDP, but out of scope of this Recommendation. E.g., see [ITU-T H.248.TCP] in case of TCP as TLS transport protocol.

### 13.1.8	Step 7: Bearer type indication for TLS

If SDP based, then only the same method as in step 2, i.e. using the SDP "m=" line <proto> element as security session indicator.

Conditional approach, dependent on:

- C7.1: unambiguous indication, see C2.1.

- C7.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic).

### 13.1.9	Step 8: Block / Unblock TLS security session control procedures at SEP

Not supported by SDP, thus property tlsbsc/bceb required.

### 13.1.10	Step 9: Start TLS security session establishment procedure

There is no explicit SDP indicator for outgoing and incoming TLS "handshake procedures" as part of TLS session establishment.

NOTE – Unconditional, general approach: signal tlsbsc/EstBNC or event tlsbsc/BNCChange{Type=Est} for triggering of outgoing or incoming TLS security session establishment procedures.

### 13.1.11	Step 10: Local L4+ NAT traversal support

Not possible via explicit SDP for general L4+ NAT-T, thus [ITU-T H.248.78] required.

NOTE – Some application-specific L4+ NAT-T support functions could be associated with application-specific SDP information, however, this is out of scope of this Recommendation.

### 13.1.12	Step 11: Reporting successfully established TLS session

Not supported by SDP, thus event tlsbsc/BNCChange required.

### 13.1.13	Step 12: Active communication phase

The control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TLS traffic) as such is out of scope of the SDP.

## 13.2	Related to TLS authentication

NOTE 1 – This clause satisfies requirement R-8.3.3.2.2/1 of [ITU-T H.248.TLSPROF].

The SDP "a=fingerprint:" attribute (according to [IETF RFC 4572]) shall be used under the condition of H.248 gateways involved in network architectures (given by trust model and SIP/SDP as application control protocol) as described by this RFC.

NOTE 2 – The SDP based method is exclusive, i.e., there are not any alternative H.248 signalling elements defined.

If the SDP "a=fingerprint:" element is required by an H.248 profile, then

- the MGC would provide a fully specified SDP fingerprint attribute in the H.248 RD and

- the MGC would provide either a fully specified or wildcarded (see [ITU-T H.248.39]) SDP fingerprint attribute in the H.248 LD

to the MG.

Usage of the the SDP "a=fingerprint:" attribute at the H.248 interface validates the bearer-to-call binding between SIP call and TLS secured bearer (see e.g. Figure 6/[ITU-T H.248.TLSPROF]).

## 13.3	Related to TLS session release

Not possible via SDP. The hard removal of a TLS-enabled stream endpoint via an H.248 MODify.req or SUBtract.req command would lead to a complete release of MG level TLS resources. Usage of signal tlsbsc/RelBNC allows to keep these resources as long as the H.248 SEP should be not removed.

# 14	The TLS profile concept

## 14.1	TLS protocol profiles

A MG may be requested to support more than one TLS domain. For each such TLS domain different "TLS services" may be required from the MG. For each TLS domain those required TLS behaviours are defined by a domain-specific TLS domain profile.

Generally the TLS domain profile may be controlled via H.248 by usage of the tlscn package (clause 10), either as an atomic entity (so called "TLS profile" approach in clause 10.6.1) or by "individual TLS protocol parameter control" (see clause 10.6.1).

Important to note: any concrete, detailed, normative specifications of TLS protocol profiles are out of scope of this Recommendation!

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as a "TLS MG profile (see also clause 3.2.9).

On creation of a TLS endpoint in the MG, the intersection of the selected (and possibly modified) TLS domain profile(s) and the TLS MG profile will be built. This intersection may be termed as the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the TLS-enabled endpoint (i.e., the MG H.248 SEP/termination, see also clause 5.2).

Figure 9 illustrates the principle relation between the profiles.

Figure 9 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile

Normally, the result of intersection is a TLS endpoint profile that exactly matches with the requested TLS domain profile(s). However, there might be partial matches in principle as well, as depicted by the example in Figure 8.

## 14.2	Illustration of TLS profile concept

The indication and negotiation of media configurations across the H.248 interface is subject of [ITU-T H.248.80]. A TLS configuration relates effectively to a subset of a media configuration (see clause 3.2.1.2/[ITU-T H.248.80]). Table 11 describe analogousterms and profile types.

Table 11 – Analogous terms between H.248.80 media configurations and TLS profile types

| H.248.80 generic media configuration | TLS-specific media configuration | H.248.TLS TLS profile term: |
| --- | --- | --- |
| Overall term: |  |  |
| Configuration (clause 3.2.1 / [ITU-T H.248.80]) | TLS configuration | TLS profile |
| Qualified term: |  |  |
| Prefered configuration (e.g., prefered codec list) | Prefered TLS configuration (NOTE 1) | TLS domain profile |
| Supported configuration (e.g., supported codec list) | Supported TLS configuration | TLS MG profile |
| Actual configuration (e.g., actual codec list) | Actual TLS configuration | TLS endpoint profile |
| NOTE 1 – There is a subtle difference between [ITU-T H.248.80] and this Recommendation: there are no SIP-level SDP Offer/Answer negotiations concerning TLS configurations due to the fact that TLS provides itself an embedded TLS handshake protocol for TLS-level negotiations. Thus, there is neither an "Offered TLS configuration" nor "Answered TLS configuration" at application control protocol level. |  |  |

Figure 10 provides an illustration of the three TLS profile types, as in use during TLS security session establishment.

Figure 10 – Illustration of TLS profile concept

In general, the TLS MG profile reflects the status of implemented TLS capabilities, it may be therefore principally a subset, superset or equal to a requested TLS domain profile.

H.248 profiles could also simplify network solutions by mandating supported TLS capabilities, which could lead to the equation of "TLS MG profile equal to TLS domain profile", resulting in "TLS endpoint profile also equal to TLS domain profile".

## 14.3	Example for the TLS MG profile concept

Appendix III provides examples for TLS domain profiles.Table 12 depicts an example for a TLS MG profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 12 – Example of a TLS MG profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlscn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlscn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlscn/cm |
| Client authentication required | yes | tlscn/car |
| Resumption support | no | tlscn/ssr |
| Extensions |  |  |

The following Table 12 shows an example how the intersection of a TLS domain profile and the TLS MG profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. Table 13 does not indicate which properties can be provided by the MGC.

Table 13 – Example of an intersection between a TLS domain profile and a TLS MG profile

| Property | TLS Domain profile | TLS MG profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B |
| MG certificate (NOTE 1) |  | List of certificates, one certificate per TLS domain profile (NOTE 2) |  |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |
| NOTE 1 – The MG certificate is used during the TLS handshake, thus it is a part of the "set of TLS related parameter and procedures" (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the TLS MG profile. The MG certificate is network element specific and TLS domain specific. Thus, at least one "default" TLD domain profile needs always to be provisioned on the MG, which is applicable if the MGC does not explicitly signal the tlscn/dpid property. In such a case this default provisioned TLS domain profile is merged with potentially signalled tldcn/x properties (x != dpid) to form the TLS endpoint specific TLD endpoint profile, which then also contains Root certificates and one MG certificate. NOTE 2 – Thus, the MG to "know" (e.g. via provisioning) a "default" TLD domain profile in case the MGC does not signal the tlscn/dpid property. |  |  |  |

# 15	Security considerations

It has to be noted that there are a number of known TLS protocol vulnerabilities, leading to ongoing TLS protocol improvement activities by the IETF.

The H.248 MG might become a point of potential, TLS related security attacks when terminating the TLS protocol (as e.g. in use cases #1.2 and #1.4 according clause 6.2.1). Appendix F "Security Analysis" of [IETF RFC 5246] should be then considered from security perspective.

Annex A

State modelling for 
TLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TLS basic session control package (clause 8) is tightly coupled to a state model, because the event (tlsbsc/BNCChange) and signal (tlsbsc/EstBNC) are related to state transitioning behaviour.

It has to be noted that the IETF TLS related RFCs do not define any explicit state model with regards to TLS security session endpoints. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tlsbsc is described by this Annex.

## A.2	Original state model for TLS session endpoints

There is no standardized state model for TLS (in comparison to e.g. L4 protocols TCP or SCTP).

## A.3	Simplified state model for H.248-based TLS basic session control

The TLS basic session control package makes a couple of assumptions, similar to those in Annex A/[H.248.TCP] for TCP.

The correspondent simplified model for TLS is depicted in Figure A.1:

Figure A.1 – Simplified state model for H.248-based TLS basic session control

There are two remaining states, called Idle and Established in the H.248 state model. Signal tlsbsc/EstBNC is used for triggering a state change from Idle to Established by the MGC, signal tlsbsc/RelBNC is used for the reverse transitioning and event tlsbsc/BNCChange indicates successfully completed state transitions to the MGC.

### A.3.1	Transitioning from Idle to Established

The bearer establishment related TLS handshake procedures, either a full handshake in case of new TLS sessions, or a abbreviated handshake in case of TLS session resumption, will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters.

- The TLS-Finish message has been received and has been successfully verified.

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event tlsbsc/BNCChange{type=Est} can be used to notify the MGC accordingly.

### A.3.2	Transitioning from Established to Idle

Transitioning to state Idle is completed after a complete TLS close_notify procedure.

{Editor's note (2013-10): in H.248 state Idle are not necessarily "all" TLS resources released due to the option of TLS session resumption. It should be clarified which TLS resource components exactly, and whether such resources are located at MG level (Root) or Stream/Termination level or both?}

Annex B

TLS protocol layer: Data model

(This annex forms an integral part of this Recommendation.)

## B.1	Motivation

TLS [IETF RFC 5246] is unusal from protocol specification perspective due to its idiosyncrasy in using a kind of proprietary presentation language concerning syntax definitions, see:

4. Presentation Language

This document deals with the formatting of data in an external representation. The following very basic and somewhat casually defined presentation syntax will be used. The syntax draws from several sources in its structure. Although it resembles the programming language "C" in its syntax and XDR in both its syntax and intent, it would be risky to draw too many parallels. The purpose of this presentation language is to document TLS only; it has no general application beyond that particular goal.

It is difficult to get a concise view about the structure of protocol data units within and at the interfaces of the TLS protocol layer. Such a data model is required for some of the defined H.248 signalling elements (such as statistics) by this Recommendation.

A data model is derived in this Annex, based on the TLS protocol data structures according Appendix A/[IETF RFC 5246].

## B.2	Data model

Figure B.1 (derived from Figure 4) outlines again the TLS protocol stack inclusive the four TLS sub-protocols. Four measurement points (MP) are highlighted, which are relevant for the several TLS traffic volume metrics according clause 12.

Figure B.1 – Performance monitoring – Measurement points related 
to traffic volume specific performance metrics

It is proposed to provide volume metrics on the following interfaces of the TLS record layer as depicted in Figure B.1:

- interface between the TLS application and the TLS record layer (measurement point "Application Data")

- interface between the TLS protocols (handshake protocol, change cipher spec protocol, alert protocol) and the TLS record layer (measurement point "TLS message")

- Interface between the TLS record protocol and the transport protocol (measurement point "TLS-RL PDU")

Figure B.2 indicates the major processing stages with the TLS Record Layer (TLS-RL). IT may be concluded that there will be no difference between the processing of application data and the processing of TLS messages. The term "TLS-RL User Data" comprises both, "TLS message" and "Application Data" (see also Figure B.1).

Figure B.2 – Data processing in the TLS record layer

In the sending direction the TLS record layer transforms a given amount of TLS-RL User Data into one or more TLS-RL PDUs.

Vice versa one or more received TLS-RL PDUs are transformed into a specific amount of TLS-RL User Data.

For a given amount of TLS-RL user data the resulting amount of TLS-RL PDUs depends on

- the current fragment size, as this dimension determines how many TLS-RL PDUs will be used;

- the compression method applicable for the current security session;

- the TLS version applicable for the current security session (e.g. TLS version 1.1 adds the initialization vector explicitly to the TLS-PDU to protect against CBC attacks);

- the cipher suite applicable for the current security session.

A protected, compressed TLS fragment is consequently of variable size, see layout in Figure B.3:

Figure B.3 – TLS-RL PDU: variable layout of a protected compressed TLS fragment

## B.3	Terminology based on data model

Table B.1 shows the relation between the terms as used in this Recommendation and the TLS presentation language [IETF RFC 5246]:

Table B.1 – TLS presentation language and derived term

| Term used in this Recommendation | TLS Presentation Language [IETF RFC 5246] |
| --- | --- |
| TLS fragment | TLSPlaintext.fragment (NOTE 1) |
| Compressed TLS fragment | TLSCompressed.fragment |
| Protected compressed TLS fragment | TLSCiphertext.fragment |
| TLS-RL PDU | TLSCiphertext |
| TLS-Header | TLSCiphertext.type + TLSCiphertext.version + TLSCiphertext.length |

NOTE 1 – It is equal to element "fragment" within data structure "TLSPlaintext", but [IETF RFC 5246] does not use explicitly the name "TLSPlaintext.fragment".

Appendix I

Sample use cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use case "Terminal-to-MG TLS session, provisioned TLS service negotiation"

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, in either case the MGC is aware of the applied TLS network domain profile for the secured bearer traffic.

- IP transport bearer type: 'TCP' is used in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [b-IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS (see also clause 6.2.1, use case #1.2).

- Bearer path protocol layering: "Application/TLS/TCP/IP" (i.e., no tunnelled transport).

- UE: TCP client and TLS client, MG: TCP server and TLS server

- H.248 MG TCP-mode: "TCP proxy".

NOTE – In more detail: L4/L3 NAT traversal support requirements during the establishment phase of TCP bearer connection may initially imply a TCP merge mode (see [ITU-T H.248.84]), which later transitions to a TCP proxy mode (due to the TLS protocol termination here for TLS to non-TLS interworking). Another option is to use the TCP proxy mode from the very beginning, e.g., in case of lacking NAT-T support.

- Mutual TLS-authentication via certificates, MG's certificate is signed, UE's certificate is self-signed.

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG (e.g., in case of an H.248 profile without support of the TLS capability negotiation package, see clause 10).

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case "Terminal-to-MG TLS session, MGC-controlled TLS service negotiation"

This use case is an extension of the first use case (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level via usage of the the TLS capability negotiation package (see clause 10). The first use case is based on preconditions which are partially subject of network provisioning. This use case is motivated by the fact that there are a number of reasons which demand MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

## I.3	Use case "WebRTC to NGN/IMS interworking function with DTLS-to-TLS support"

WebRTC-based data uses "SCTP/DTLS/UDP/IP" as transport (see also Appendix I.1/[ITU-T H.248.TCP]. An H.248 WebRTC gateway (see [b-ITU-T H.248.WebRTC]) could be requested for interworking to "TLS/TCP/IP" based transport, resulting in a DTLS-to-TLS scenario.

## I.4	Use-case "TLS-based transport security for facsimile packet relay service T.38"

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation)

{Editor's note (2013-10 meeting): work on H.248.SEPLINK would imply updates on the sections which are indicating "interlinkage" usage.}

## II.1	TLS to non-TLS interworking with TCP as example bearer type

NOTE: TCP is used as an example L4 protocol for TLS. [ITU-T H.248.TCP] provides further details on TCP bearer connection control procedures and takes precedence in case of discrepancies between both Recommendations.

### II.1.1	Initial session setup using SDP security descriptions in application control signalling

This use case is presented in Appendix I.1. Figure II.1 outlines the network configuration as used in the message flow.

Figure II.1 – Configuration for the example call flow

The scenario shows a simplified SIP session establishment. The TCP bearer connection establishment direction is defined at SIP level by the SDP setup attribute (based on [b-IETF RFC 4145]), and the fingerprint of the UE's certificate is included in the SDP (based on [IETF RFC 4572]).

The H.248 MG is triggered by the MGC to create two terminations, whereby the bearer for the first SEP/termination is secured by the use of TLS. In the RD of this termination the MG also receives the fingerprint of the UE's certificate.

### II.1.2	Traffic flow examples for communication establishment phase

There are multiple options for establishing the communication path for the TLS to non-TLS interworking. The following establishment examples are illustrated:

- Scenario 1: Unclear situation whether remote NAT devices may require L4/L3 NAT-T support, leading to an initial establishment and connectivity check of the end-to-end IP L4 connection before the initiation of the TLS security session. The MGC explicitly triggers (or not) TLS establishment, dependent on a successfully established IP L4 connection (see clause II.1.2.1).

- Scenario 2: NAT-T support seems to be not required, however, the MGC separates the communication establishment between the TLS network domain and non-TLS network domain, e.g., due to separated capability negotiations in each domain at application control level (see clause II.1.2.2).

- Scenario 3: the end-to-end IP L4 connection should be autonomously established by the MG (e.g., because the MGC does not expect any blocking issues such as L4 protocol deadlocks). The MGC requests to be notified about L4 connectivity and then explicitly triggers TLS security session establishment (see clause II.1.2.3).

- Scenario 4: the MGC does not expect any issues at all and may optimize communication establishment by minimizing setup delay times, i.e., enabling full MG autonomous behaviour across the SEPP and vertical protocol layers (see clause II.1.2.4).

NOTE – This is a non-exhaustive list of variants, just a few examples.

#### II.1.2.1	Example: TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)

The call flow (Figure II.2) shows the setup of the TCP bearer connection as well as the subsequent TLS handshake on the first stream endpoint T1(S1). The MGC initially blocks any TLS security session establishment due to uncertainity whether the underlying TCP bearer connection could resolve all "NAT traversal challenges". After the MG reports a sucessfully established TCP bearer connection, the MGC initiates TLS session establishment. The TLS server role is located at MG side in this example.

The self-signed certificate received from the UE is verified by the use of the fingerprint. After the TLS handshake is finished, application data may flow between the UE and the remote TLS session endpoint. In the traffic flow presented here the MG will forward these data in the TCP proxy mode.

Figure II.2 – Example setup of a TLS to non-TLS IWF 
(TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection))

#### II.1.2.2	Example: combined TLS/L4 establishment (decoupled end-to-end L4 connection)

This scenario is similar to clause II.1.2.1, but the successful L4 connections establishment towards the remote TLS endpoint (i.e., UE-X) shall immediately trigger the TLS session establishment without MGC involvement. Such a behaviour is achieved by setting property tlsesc/plslink to TRUE (at SEP T1(S1)), see Figure II.3.

Figure II.3 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment (decoupled end-to-end L4 connection))

The final MG notifications concerning a successfully established end-to-end IP bearer path are optional (and dependent on MGC requesting these notifications).

#### II.1.2.3	Example: TLS establishment decoupled from L4 establishment (but interlinked end-to-end L4 connection establishment)

This scenario is similar to clause II.1.2.1, but provides an interlinked TCP bearer connection establishment. Such behaviour is achieved by setting property tcpecc/sepplink to TRUE (at both SEPs, if the establishment direction is unknown from the MGC side), see Figure II.4.

Figure II.4 – Example setup of a TLS to non-TLS IWF
(TLS establishment decoupled from L4 establishment, but interlinked end-to-end L4 connection establishment)

#### II.1.2.4	Example: combined TLS/L4 establishment (and also interlinked end-to-end L4 connection establishment)

If possible, then the MGC could delegate the "entire" control of end-to-end IP bearer path establishment down to the MG, by enabling interlinkage at L4-SEPP level as well as between TLS and L4 at SEP T1(S1), see Figure II.5.

Figure II.5 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment, plus interlinked end-to-end L4 connection establishment)

### II.1.3	Traffic flow examples for communication release phase

There are multiple options for release of the IP bearer path for the TLS to non-TLS interworking scenario. The following release examples are illustrated:

- Scenario 1: The MGC separates TLS and L4 bearer release, e.g. due to possible TLS session resume. Thus, any later IP L4 connection release will be explicitly triggered by the MGC (see clause II.1.3.1).

- Scenario 2: The MGC issues the TLS session release and wants the IP L4 connection release to be initiated by the remote TCP endpoints. E.g., if either UE-X or UE-Y initiates a TCP release activity, then the end-to-end TCP bearer connection should be "MG autonomously" released (see clause II.1.3.2).

- Scenario 3: Similar to the previous scenario, but now MG should immediately start the IP L4 connection release towards the remote TLS/L4 endpoint. (see clause II.1.3.3).

NOTE – This is a non-exhaustive list of variants, just a few examples.

Concerning MG autonomous support with respect to TLS session release procedures: it may be noted that for all release examples it is assumed that the properties tlsesc/plslink, tlsesc/sepplink and tcpecc/sepplink are set to their defaults if not otherwise specified.

#### II.1.3.1	Example: TLS session release initiated by remote TLS endpoint and MGC enforced immediate L4 connection release

The MGC expects an incoming TLS session release (see Figure II.6). UE-X initiates the TLS session release by sending a TLS alert (close_notify) message.

Figure II.6 – Example TLS session release, initiated by remote TLS endpoint 
(MGC enforces immediate L4 connection release)

The MG reports a successfully released TLS session to the MGC, which then decides to release also the underlying TCP bearer connection, towards both remote UEs.

#### II.1.3.2	Example: outgoing TLS session release, local TLS/L4 decoupled, coupled end-to-end L4 connection release

Figure II.7 depicts a situation when the MGC initiates the TLS session release towards UE-X, and configures the MG to autonomously release any possible end-to-end TCP bearer connection release, which might be initiated by one (or both) remote UE(s).

Figure II.7 – Example TLS session release, local TLS/L4 decoupled, 
but coupled end-to-end L4 connection release)

#### II.1.3.3	Example: outgoing TLS session release, coupled TLS/L4 and end-to-end L4 connection release

This scenario is similar to the previous example (clause II.1.3.2), but now the MGC wants to explicitly start TCP bearer connection release (e.g., in order to be independent of remote UEs), see Figure II.8.

Figure II.8 – Example TLS session release, 
coupled TLS/L4 and end-to-end L4 connection release

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

The TLS profile concept is introduced in clause 14. This Appendix illustrates some exemple TLS profiles.

## III.1	Typical Internet TLS profile

TLS is used in many Internet applications. Tables III.1.a and III.1.b indicate an example TLS domain profile.

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | Signed, Self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

[b-ETSI TS 133 203], Annex O defines "Enhancements to the access security to enable TLS". When used, then the network domain security related "TLS domain profile", according to Annex E/[b-ETSI TS 133 310], is prescriptive.. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | Compression Method.null | - | methods of [IETF RFC 3749] | - |
| 2.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [b-IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) "TLS_DH_anon_WITH_" (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [b-IETF RFC 4346]. NOTE 3 – Because the "the key exchange method shall not be anonymous". |  |  |  |

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [b-OMA-TS-TLS-V1_0-20080902-A], a "public key TLS domain profile" and a "pre-shared key TLS domain profile".

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [b-IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [b-IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE 1) |  |  |  |
| 3.6 | Extensions: | [IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | None specified. |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | All others |

### III.3.2	OMA pre-shared key TLS domain profile

See [b-OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

## III.5	IETF example of a national TLS domain profile

[b-IETF RFC 6460] represents an example of a national TLS domain profile. Inter alia, TLS protocol version and ciper suites are profiled.

## III.6	ITU-T TLS domain profile for NGN signalling and management plane

[b-ITU-T Y.2704], clause 9.1, defines a TLS domain profile, recommended for signalling and management interfaces in NGNs.

Appendix IV

Illustration of protocol semantics 
of the TLS basic session control package

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tlsbsc package (clause 8) defines the basic TLS security session control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## IV.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS session endpoint.

## IV.3	Establishment of TLS security sessions

### IV.3.1	Successful establishment – Terminating side

See Figure IV.1, termed as use case (E.1):

Figure IV.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of TLS session establishment (1).

- Early incoming TLS messages are discarded by the MG (2).

- The MGC unblocks the TLS SEP (3).

- TLS session establishment (5 to 7); MG acting as TLS server.

- Optional notification of MGC concerning available TLS session for application data transfer (8), if subscription to event by MGC (4).

### IV.3.2	Successful establishment – Originating side

See Figure IV.2, use case (E.2).

Figure IV.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause IV.3.1

- TLS session establishment (5 to 7), triggered by MGC (4); MG acting as TLS client.

- Optional notification of MGC concerning available TLS session for application data transfer (9), if subscription to event by MGC (8).

### IV.3.3	Unsuccessful establishment

The TLS security session negotiation handshake is basically not completed.

## IV.4	Release of TLS security sessions

### IV.4.1	Successful release – Terminating side

See Figure IV.4, termed as use case (R.1).

Figure IV.4 – Successful release – Terminating side

More details for this example:

- MG receives an incoming TLS close_notify alert (2), the indication for TLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released TLS session (4), if subscription to event by MGC (1).

### IV.4.2	Successful release – Originating side

See Figure IV.5, use cases (R.2) illustrates an outgoing TLS session release:

Figure IV.5 – Successful release – Originating side

More details for this example:

- MGC triggers the TLS session release (1).

- MG sends an outgoing TLS close_notify alert (2), (the indication for TLS session release) towards remote TLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released TLS session (5), if subscribed to the event by MGC (4).

### IV.4.3	Unsuccessful release

An unsuccessful TLS session release procedure would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established TLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

Appendix V

Illustration of protocol semantics 
of the TLS extended session control package

(This appendix does not form an integral part of this Recommendation.)

{Editor's note (2013-10 meeting): work on H.248.SEPLINK would imply replacement of the reference to the sepplink & plslink property "tcpecc package (clause 9)".}

## V.1	Overview

The tlsesc package (clause 9) defines an augmented TLS security session control on top of the base package (tlsbsc, clause 8). The effect of each individual package property is illustrated in clauses V.3 (property tlsesc/sepplink) and V.4 (property tlsesc/plslink). Clause V.5 describes examples of combined usage of both properties.

The illustrated use cases are only examples. An exhaustive consideration of all possible combinations is out of scope.

## V.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS and L4 layers) are highlighted besides the H.248 interface.

Furthermore some Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS/L4 SEP.

## V.3	Usage of SEPP Interlinkage property

Property tlsesc/sepplink is defined in clause 9.1.1. The property affects TLS security session establishment and release procedures. The semantics are correspondent to the TCP layer. See e.g. the illustration and examples in [ITU-T H.248.TCP], Appendix ?.3.

## V.4	Usage of Protocol Layers Interlinkage property

Property tlsesc/plslink is defined in clause 9.1.2. The property impact is depicted in below two illustrations. Figure V.1 recalls again the default mode according base package (tlsbsc):

Figure V.1 – Illustration of Protocol Layers Interlinkage property 
– Property "plslink = FALSE"

The MGC is responsible for individual control of layer-specific bearer control procedures.

Figure V.2 illustrates the tightly coupled mode, which is conditional to the aimed communication service, but when applicable, it provides some benefits such as minimal MGC involvement and performance optimized bearer establishment and/or release.

Figure V.2 – Illustration of Protocol Layers Interlinkage property 
– Property "plslink = TRUE"

### V.4.1	Property "plslink = FALSE"

Semantic FALSE implies base package behaviour.

### V.4.2	Property "plslink = TRUE"

The property may be applied for the establishment or release phase or both.

#### V.4.2.1	Bearer establishment phase

##### V.4.2.1.1	Incoming direction

Figure V.3 illustrates the normal case: the start of any TLS security session establishment is preceded by a successfully established IP transport connection. Thus, the MG receives an incoming L4 Est.req (2). When L4 is established, then immediately an outgoing TLS session establishment (4) is initiated by the MG.

Figure V.3 – Property "plslink = TRUE" – L4 incoming bearer establishment request

The actually impossible case of TLS establishment request first, without yet successfully established IP transport, is indicated Figure V.4.

Figure V.4 – Property "plslink = TRUE" – TLS incoming bearer establishment request

Theoretical example:

E.g., case of TLS/TCP bearer: the very first incoming TCP packet (with set SYN flag) would contain already a TLS ClientHello as TCP payload, but the local TCP connection state is still not Established!

##### V.4.2.1.2	Outgoing direction

SEP T1(S1) initiatesan outgoing L4 bearer establishment procedure (see Figure V.5). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful IP transport connection establishment (3, 4), the MG immediately initiates an outgoing TLS session establishment (5).

Figure V.5 – Property "plslink = TRUE" – L4 stimuli

Figure V.5 shows also the optional event notifications related to bearer specific state transitioning (here: L4 and TLS reporting).

There might be also a TLS-related stimuli first from SEP perspective (without a prior explicit L4 trigger). The scenario is almost identical as the previous case, see Figure V.6:

Figure V.6 – Property "plslink = TRUE" – TLS stimuli

Example:

In case of TLS/TCP with SEPP interlinkage usage, and property settings 
a) TCP layer: tcpecc/sepplink = FALSE and  
b) TLS layer: tlsesc/sepplink = TRUE, 
then the SEP would receive a "TLS level stimuli" from his partner SEP.

#### V.4.2.2	Bearer release phase

##### V.4.2.2.1	Incoming direction

Figure V.7 illustrates the abnormal case: the start of any IP transport connection release precedes a successfully released TLS security session. The MG receives an incoming L4 Rel.req (2), but would suspend L4 release procedural steps in favour of a TLS session release (3) first. The L4 release procedure is resumed again (5) after successful TLS termination (4).

Figure V.7 – Property "plslink = TRUE" – L4 incoming bearer release request

The standard case of TLS release request first is depicted in Figure V.8: the TLS session release (2, 3) triggers the subsequent IP transport connection release (4, 6).

Figure V.8 – Property "plslink = TRUE" – TLS incoming bearer release request

##### V.4.2.2.2	Outgoing direction

SEP T1(S1) initiates an outgoing L4 bearer release procedure (see Figure V.9). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent sepplink settings). After successful TLS session release (3, 4), the MG immediately initiates an outgoing IP transport connection release (5).

Figure V.9 – Property "plslink = TRUE" – L4 stimuli

Figure V.10 outlines the case of a TLS-related stimuli:

Figure V.10 – Property "plslink = TRUE" – TLS stimuli

Appendix V

TLS alert protocol from H.248 gateway perspective

(This appendix does not form an integral part of this Recommendation.)

## V.1	Background

TLS [IETF RFC 5246] provides the specific protocol capability of an embedded alert information channel, defined as TLS alert protocol. The TLS alert protocol covers a variety of different alert services, which itself lead to "TLS alert"-specific handling in H.248 gateways.

## V.2	H.248 gateway framework concerning TLS alert handling

The TLS alert protocol defines two fundamental alert categories:

- Closure alert; used in normal service scenarios ("TLS session release"), thus covered by the TLS basic session control package (see clause 8.6.5.1); and

- Error alerts, used in abnormal situations, subject of the TLS session maintenance package (see clause 11).

There are arround 25 alert types within the error alert category. Furthermore, the TLS alert protocol defines two severity levels for further qualification of each alert type:

- Warning level: alert provides only a warning indication, the communication should/could continue; and

- Fatal level: continuation of communication not possible, the TLS session shall be released.

There are consequently three dimensions in structuring TLS alert activities: a) alert category, b) alert type and c) alert severity.

Further, H.248 media gateways may be involved in a variety of different TLS use cases (see clause 6) and interworking models (see clause 7), leading to the question concerning the likely location of the served user instance, responsible for processing the particular TLS alert information. There are three options in case of TLS protocol handling by a distributed architecture such as H.248 gateways:

- MG: the MG itself processes and terminates an incoming TLS alert or sents a TLS alert to the remote TLS endpoint, without any involvement of other remote control/management plane elements. Thus, a scenario with MG local TLS alert processing.

- MGC: either, the MGC may trigger the MG to sent a specific TLS alert to the remote TLS endpoint, e.g. due to security aspects, application logic, etc.; or the MGC would be notified by the MG by incoming TLS alert information (Note: which again may be mapped to an H.248.8 error code).

- NMS/EMS (here: element management system of MG): e.g., in case of particular TLS alerts resulting in fault or security management actions.

Above high level evaluation defines a framework for TLS alert handling by H.248 gateways.

## V.3	TLS error alerts categorization

TLS error alerts are defined in detail by clause 7.2.2/[IETF RFC 5246]. Following observations could be made:

- roughly 70 % of the (ca. 25) error alerts are of fatal severity, i.e., shall trigger the release of the TLS session;

- there are two error alerts which may appear only as warnings; 
and

- a few error alerts are related to "older TLS versions".

## V.4	Summary

The individual handling of specific TLS error alerts should be implementation specific and is out of scope of this Recommendation. However, as outlined in clause V.2, a general mechanism is supported by this Recommendation (by the tlsm package, clause 11) which allows to notify the MGC about received TLS alert information by the MG, - for instance the MGC could be notified about the fatal severity in order to initiate the release of a TLS session -, and the MGC would be able to trigger the MG in sending specific TLS alerts (e.g., due to application specific requirements).

Bibliography

[b-ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[b-ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[b-ITU-T V.153]	Recommendation ITU-T (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T Y.2704]	Recommendation ITU-T Y.2704 (01/2010), Security mechanisms and procedures for NGN.

[b-IETF RFC 3749]	IETF RFC 3749 (05/2004), Transport Layer Security Protocol Compression Methods

[b-IETF RFC 4145]	IETF RFC 4145 (09/2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[b-IETF RFC 4346]	IETF RFC 4346 (04/2006), The Transport Layer Security (TLS) Protocol Version 1.1.

[b-IETF RFC 4366]	IETF RFC 4366 (04/2006), Transport Layer Security (TLS) Extensions.

[b-IETF RFC 6460]	IETF RFC 6460 (2012), Suite B Profile for Transport Layer Security (TLS).

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-IETF tls-minimal]	IETF draft-tschofenig-lwig-tls-minimal (2012), A Minimal (Datagram) Transport Layer Security Implementation.
URL: http://tools.ietf.org/html/draft-tschofenig-lwig-tls-minimal-01

[b-ETSI TS 133 203]	ETSI TS 133 203 V11.2.0 (2012-11), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; 3G security; Access security for IP-based services (3GPP TS 33.203 version 11.2.0 Release 11).

[b-ETSI TS 133 310]	ETSI TS 133 310 V11.2.0 (2013-02), Universal Mobile Telecommunications System (UMTS); LTE; Network Domain Security (NDS); Authentication Framework (AF)(3GPP TS 33.310 version 11.2.0 Release 11).

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-OMA-TS-TLS-V1_0-20080902-A]	Open Mobile Alliance OMA-TS-TLS-V1_0-20080902-A (09/2008), OMA TLS Profile.

____________________

Defined as: "A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint." ([IETF RFC 6714]).