| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4477 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.SEPLINK |  |  |  |
| Title: | H.248.SEPLINK "Gateway control protocol: Stream endpoint interlinkage package" (New): Input draft Ed. 0.3 |  |  |  |
| Purpose: | Meeting input (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's input draft for draft new H.248.SEPLINK " Gateway control protocol: Stream endpoint interlinkage package" (Ed. 0.3).

# Changes versus Ed. 0.2 (TD 122 (WP 1/16) (2013-11)):

- not any editorial changes

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.SEPLINK Ed. 0.1 | 2013-10 (Geneva): contribution C-264 (and C-258) Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-C-0264 |
| H.248.SEPLINK Ed. 0.2 | Output (TD 122/WP1) from Geneva meeting (2013-10) Location: |
| H.248.SEPLINK Ed. 0.3 | Input (AVD-4477) to WP 1/16 Geneva meeting (2014-03) Location: |
| H.248.SEPLINK Ed. 0.4 | Output (TD-##) from WP 1/16 Geneva meeting (2014-03) Location: |
| H.248.SEPLINK Ed. 0.5 |  |
| H.248.SEPLINK Ed. 0.6 |  |
| H.248.SEPLINK Ed. 0.7 |  |
| H.248.SEPLINK Ed. 0.8 |  |
| H.248.SEPLINK Ed. 0.9 |  |
| H.248.SEPLINK Ed. 0.10 |  |

CONTENTS

Page

1 Scope 5 1.1 Applicability statements 5 2 References 5 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 6 6 Motivation, use cases and models 6 7 Stream endpoint interlinkage package 10 7.1 Properties 10 7.1.1 Interlinkage topology 10 7.2 Events 12 7.3 Signals 12 7.4 Statistics 12 7.5 Error Codes 12 7.6 Procedures 12 7.6.1 Configuration of "interlinkage topologies" 12 7.6.2 Relation to H.248 Stream Mode: 13 7.6.3 Relation to H.248 Topology descriptor 13 7.6.4 Error cases: 13 7.6.5 Examples: 13

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1 – Two-termination context with multiple connection-oriented protocols in the SEP-specific protocol stack (segments) 7 Figure 2 – Principle interlinkage options (here: two-termination context) from perspective of a single SEP (here: SEP-A) 7 Figure 3 – Interlinkage example for a three-termination context 9 Figure 4 – Example of a "TLS/TCP to SCTP" connection model – Vertical interlinkage configuration at SEP-A 14 Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP" 15 Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS" 16

Draft new Recommendation ITU-T H.248.xx (ex H.248.SEPLINK)

Gateway Control Protocol:
Stream endpoint interlinkage package

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

FIXTHIS

## 1.1	Applicability statements

The capability of "interlinkage" relates to the MG internal forwarding of protocol control information (PCI) between different SEPs and/or different protocol layers. A particular layered protocol architecture is thus associated with a SEP.

The interlinkage capability is basically limited on protocol stack segments, which may be unambiguously described via the mechanism according clause 7.1.1.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	stream endpoint (SEP) [clause 3.2.10/ITU-T H.248.1]: relates to a stream associated to a Termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.2	stream endpoint pair (SEPP) [clause 3.2.11/ITU-T H.248.1]: two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there is no further associated SEP on a third Termination etc).

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| EP | End Point |
| --- | --- |
| TCP |  |
| TLS |  |
| SCTP |  |
| SDP | Session Description Protocol |
| SEP | Stream End Point |
| SEPP | Stream End Point Pair |
| SEPT | Stream End Point Tuple |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# 6	Motivation, use cases and models

H.248 controlled Media Gateways may terminate multiple connection-oriented transport layer protocols, like TCP, TLS, SCTP.

This is outlined in the following Figure 1 where in a given H.248 context each Stream Endpoint (SEP) is supposed to host at least one or more connection-oriented transport layer endpoints.

Figure 1 – Two-termination context with multiple connection-oriented protocols 
in the SEP-specific protocol stack (segments)

The following Figure 2 outlines the various interlinkage options enabling an MG autonomous establishment/release of a connection/session-oriented partner transport protocol endpoint (again at the example of a stream endpoint pair (SEPP)):

Figure 2 – Principle interlinkage options (here: two-termination context) 
from perspective of a single SEP (here: SEP-A)

Options (in Figure 2):

- Within one SEP a lower layer connection oriented transport protocol endpoint (EP) triggers an upper layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Within one SEP an upper layer connection oriented transport protocol EP triggers a lower layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Given a SEP pair SEP-A triggers the connection oriented transport protocol of the peer SEP-B. Either the same or a different transport protocol can be interlinked, thus for example SEP-A TCP endpoint could trigger a SEP-B TCP endpoint, or a SEP-A TCP endpoint could trigger e.g. a SEP-B SCTP endpoint.

- Given a SEP pair SEP-A receives a trigger regarding a connection oriented transport protocol from its peer SEP-B. Either the same or different transport protocol can be interlinked, thus for example SEP-B TCP endpoint could trigger a SEP-A TCP endpoint, or a SEP-B TCP endpoint could trigger e.g. a SEP-A SCTP endpoint.

{Editor's note (2013-11):

The four examples of stimuli-based information flows are valid, but the structuring principles are unclear and should be revised: the protocol semantic of the seplink/linktopo property represents following information model:

The conditions indicate the supported logic by the seplink/linktopo property.

It may be observed that Figure 2 does neither describe the stimuli flow from source nor from sink perspective, rather from a particular SEP. However, the property allows only to define the stimuli flow in outgoing direction (due to the unidirectional nature) when the sink is located in a different SEP. Consequently, option 4 looks odd in above text.
A revision is solicited.}

There are following principal limitations:

- Horizontal protocol layer interlinkage (i.e., related to the stream endpoint pair (SEPP)) is only permitted between SEPs belong to the same H.248 stream.

- Vertical protocol layer interlinkage is only permitted within a SEP, i.e. between its supported transport protocol layers.

A H.248 context may contain more than two terminations; where more than two terminations host SEPs belonging to the same H.248 stream. In this case the SEP interlinkage capabilitiy needs to be able to address a dedicated stream endpoint tuple (SEPT).

For example, following interlinkage scheme should be supported in case of an H.248 context containing three SEPs (same H.248 streamID).

Figure 3 – Interlinkage example for a three-termination context

Whereas SEPP (SEP-A,SEP-B) can trigger each other regarding the identified transport protocol endpoints, the SEP-A connection oriented transport protocol EP may trigger SEP-C connection oriented transport protocol EP in addition. However SEPP (SEP-B,SEP-C) doesn’t have any interlinkage, whereas also SEP-C connection oriented transport protocol EP cannot trigger SEP-A connection oriented transport protocol EP in any way.

{Editor's note (2013-11):

In the current TCP, TLS package specific transport protocol layer coupling capabilities had been introduced (tcpecc/sepplink, tlsesc/sepplink, tlsesc/plslink), see H.248.TCP/H.248.TLS.

This new package is intended to provide more generic capabilities, i.e.

- enabling SEP pair identification in case of H.248 contexts with more than two SEPs,

- enabling the interlinkage of different transport protocols,

- re-usability of this generic mechanism for any other connection-oriented transport protocol, e.g. SCTP, thus avoiding to define transport layer coupling capabilities again and again.

This new package would then replace the correspondent properties in H.248.TCP/H.248.TLS.}

# 7	Stream endpoint interlinkage package

|  | Package Name: | Stream endpoint interlinkage package |
| --- | --- | --- |
|  | Package ID: | seplink (0x####) |
|  | Description: | This package provides the functionality to interlink connection/session oriented transport protocol endpoints. If two transport protocol endpoints are interlinked the establishment and/or release of a connection/session at the source transport protocol endpoint will trigger an MG-autonomous establishment and/or release of the interlinked transport protocol endpoint. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Interlinkage topology

|  | Property Name: | Interlinkage topology |
| --- | --- | --- |
|  | Property ID: | linktopo (0x0001) |
|  | Description: | This property defines the MG interlinkage behaviour regarding connection/session oriented transport endpoints. Interlinkage can be specified between the connection oriented transport layers of a given SEP or between any connection oriented transport layers of a given SEP pair. Interlinkage defines a undirectional information flow ("stimuli") from a "SEP-local transport EP" towards another "SEP-local or –remote transport EP". |
|  | Type: | Sub-list of Strings |
|  | Possible values: {Editor's note (2013-11): the syntax specification of values may benefit from an ABNF based format, e.g. the mode seems to be a sub-list also. Need to allow binary terminationIDs etc.} | The property consists of a sequence of interlinked transport protocol endpoints of the form "interlinked-SEP, source-transport-EPs, interlinked-transport EP, mode". Specification of string format of: interlinked-SEP "," source-transport-EPs "," interlinked-transport EP "," mode Where: - "," is a literal functioning as separator - interlinked-SEP is {Editor's note (2013-11): the "interlinked-SEP" is the local SEP. The identifier for a SEP is the 2-tuple of of StreamID and TerminationID values (see clause 3.2.10/H.248.1). The StreamID of the "interlinked-SEP" is implicitly given be the StreamDescriptor where the linktopo property is located, but not the TerminationID. Thus we need an explicit indication of the TerminationID value. Below description of the semantic should be enhanced} - semantic: specifies the interlinked stream endpoint by using the TerminationID, possibly using the ALL or CHOOSE wildcard. In case the interlinked TerminationID is the same TerminationID as of the SEP the property is belonging to vertical interlinkage is triggered. If they are not the same, horizontal interlinkage is triggered. {Editor's note (2013-11): TerminationID issue is that the semantic of interlinked-SEP is a terminationID but the syntax given is for a StreamID i.e. UINT16. Source-transport-EP and interlinked-transportEP have a similar syntax issue; further: binary encoding should be covered} - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. - source-transport-EPs is - semantic: specifies the source transport endpoint, e.g. TCP, TLS, SCTP or others. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. - interlinked-transport EP is - semantic: specifies the interlinked transport endpoint, e.g. TCP, TLS, SCTP or others. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. NOTE: The transport protocols as being used by the source/interlinked-transport-EP shall partially match with the particular SDP "m="-line transport protocol defintion (see clause 7.1.8.1.1/[ITU-T H.248.1]). Detailed agreements regarding the admitted SDP "m="-line transport protocol <proto> values shall be defined in the particular H.248 profile specifications, typically in the profile clauses on mandatory/optional support of SDP. - mode is - semantic: specifies whether the interlinkage is related to connection/session establishment and/or release procedure; Possible values: {Editor's note (2013-11): a sublist based value would be more future safe} - "est": establishment interlinkage only. - "rel": release interlinkage only. - "est-rel": establishment and release interlinkage. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. TBC: use case: do we need to support multiple transport layer EPs of the same type per SEP, e.g. TCPoTCP, IP-over-IP tunneling? |
|  | Default: | Empty list |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 7.2	Events

None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

None

## 7.6	Procedures

### 7.6.1	Configuration of "interlinkage topologies"

#### 7.6.1.1	SEP identification:

The source SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which triggers autonomous procedures at another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID and StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value.

- The transport protocol is explicitly specified in the seplink/linktopo property value.

The destination SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which receives a trigger to start autonomous procedures from another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID is explicitly specified in the seplink/linktopo property value.

- The StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value.

- The transport protocol is explicitly specified in the seplink/linktopo property value.

#### 7.6.1.2	Traffic directions:

The seplink/linktopo property defines a unidirectional behavior, that is it specifies the interlinkage behavior of a source (identified by the TerminationID and StreamID the LocalControl descriptor is related to) and an interlinked SEP (identified by the TerminationID; same StreamID as source).

In case a bidirectional interlinkage between a SEPP/SEPT is expected, each SEP’s Local Descriptor must contain a corresponding interlinkage topology.

#### 7.6.1.3	Wildcard usage:

The interlinked SEP can be specified using the ALL wildcard

"seplink/linktopo = ["*,TCP,TCP,est-rel"]".

In this case the specified interlinkage behavior is valid to all matching SEPPs (i.e. only horizontal interlinkage, vertical interlinkage is specifically excluded, that is ALL wildcard excludes own TerminationID). In case a new SEP is added into the H.248 context later on, the ALL wildcarded interlinkage topology is applied as well to the new SEP.

The interlinked SEP can be specified using the CHOOSE wildcard

"seplink/linktopo = ["$,TCP,TCP,est-rel"]".

The CHOOSE wildcard matches the TerminationID that the MG assigns in the first Add Command that uses a CHOOSE wildcard in the same action. This allows to add a new termination/SEP, and to modify the existing interlinkage topologies (using a modify command) within the same action.

Transaction = 1 {

Context = 1 {

Add = ip/1/$/$ {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["ip/1/1/1,TCP,TCP,est-rel"] }}}}

Modify = ip/1/1/1 {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["$,TCP,TCP,est-rel"] }}}}

### 7.6.2	Relation to H.248 Stream Mode:

A specified interlinkage topology is independent of the StreamMode property of the LocalControl Descriptor. The StreamMode property affects the application data flow rather than the transport protocol control information used to establish or close the corresponding transport connection.

### 7.6.3	Relation to H.248 Topology descriptor

The effectiveness or non-effectiveness of a specified horizontal interlinkage topology depends on the context level topology descriptor, i.e. whether the corresponding flow direction in the topology descriptor for a particular SEPP (or in case of wildcard ALL usage: for all matching SEPPs) has been enabled or not.

### 7.6.4	Error cases:

In case a new interlinkage topology is specified refering to an unknown termination identifier, then the MG shall reject this using Error #430 "Unknown TerminationID".

In case a new interlinkage topology is specified refering to a valid termination identifier but this termination doesn’t contain an SEP for this specific stream identifier, then the MG shall reject this using Error #581 "Does Not Exist".

In case a new interlinkage topology is specified refering to a transport protocol not being part of the SEP’s m-line transport protocol, then the MG shall reject this using Error #581 "Does Not Exist".

### 7.6.5	Examples:

#### 7.6.5.1	Example of a "TLS/TCP to SCTP" connection model

Given an SEP-A (T-A) uses transport layers TLS/TCP. A TCP connection setup shall autonomously trigger a setup of the TLS session and a TLS session release shall autonomously trigger a release of the TCP connection.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,TCP,TLS,est","T-A,TLS,TCP,rel"]
} } } }

The resulting interlinkage looks as follows (Fig. 4):

Figure 4 – Example of a "TLS/TCP to SCTP" connection model 
– Vertical interlinkage configuration at SEP-A

In addition SEP-A’s transport layer TCP can be interlinked with SEP-B’s transport layer SCTP (i.e. SEP-A’s TCP connection establishment/release shall autonomously trigger SEP-B’s SCTP connection establishment/release).

Note: SEP-A’s TCP connection establishment/release can be either stimulated by using the corresponding H.248 TCP package capabilities ( e.g., [ITU-T H.248.TCP] properties tcpbcc/EstBNC (outgoing establishment procedure (TCP client role)) or tcpbcc/RelBNC) or by receiving TCP connection establishment/release request from the far-end TCP endpoint.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,TCP,TLS,est", "T-A,TLS,TCP,rel",
                            "T-B,TCP,SCTP,est-rel"]
} } } }

Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP"

To make sure that SCTP connection establishment at SEP-B will also trigger SEP-A TCP connection establishment and that SCTP connection release at SEP-B will also trigger SEP-A TLS session release, the seplink/linktopo property has to be applied as follows in SEP-B’s LocalControl Descriptor:

H.248 command request:

Modify = T-B {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,SCTP,TCP,est", "T-A,SCTP,TLS,rel"]
} } } }

Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS"

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

____________________