| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4479 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.Sup.OpenFlow |  |  |  |
| Title: | Draft Technical Report TR H.Sup.OpenFlow: "Protocol evaluation – OpenFlow versus H.248" (New): Input draft Ed. 0.3 |  |  |  |
| Purpose: | Meeting input (Draft TR) |  |  |  |

Summary

This document contains the editor's input draft for draft new Draft Technical Report TR H.Sup.OpenFlow: "Protocol evaluation – OpenFlow versus H.248" (Ed. 0.3).

# Changes versus Ed. 0.2 (TD 115 (WP 1/16) (2013-11)):

- not any editorial changes

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.OpenFlow Ed. 0.1 | 2013-10 (Geneva): 1st input based on "COM 16 – C 216" Location: http://itu.int/md/T13-SG16-C-0216/en |
| H.Sup.OpenFlow Ed. 0.2 | TD 115/WP1 (2013-10) |
|  | Input (AVD-4479) to WP 1/16 Geneva meeting (2014-03) Location: |
|  | Output (TD-##) from WP 1/16 Geneva meeting (2014-03) Location: |

CONTENTS

Page

1 Scope 6 2 References 7 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 8 5 Conventions 9 6 Introduction 9 6.1 Brief protocol history 9 6.2 Terminology relations 10 6.3 Objectives in terms of network architecture 11 7 Packet forwarding model 13 7.1 OpenFlow Processing model 13 7.2 Policy rule based description of packet operations 13 7.2.1 OpenFlow: basic policy rule format 14 7.2.2 OpenFlow: staged process model – concatenation of multiple policy rules 15 7.2.3 OpenFlow: ### 15 8 Stage 1 – Services (use cases) 15 9 Stage 2 – Protocol requirements 15 9.1 Decomposed network element architecture 16 9.2 Operations on traffic flows 16 9.2.1 General operations 16 9.2.2 Regarding policy rule conditions (“flow identification”) 17 9.2.3 Regarding policy rule actions 18 9.3 High availability of services and networks 19 9.4 Public networks: support of regulation related requirements 19 10 Stage 3 – Protocol capabilities 19 10.1 Termination types: supported protocol stacks in forwarding plane 19 10.2 Connection models: IP mode of operation in forwarding plane 20 Appendix I Scope of OpenFlow – Use cases 23 I.1 Experimental prototyping (academic/research area) 23 I.2 Software defined networking (SDN) 23 I.3 Network functions virtualisation 23 I.4 Cloud oriented networking 23 I.5 Trill switching 23 I.6 Lx-VPNs (Layer x Virtual Private Network) 24 I.6.1 Layer 1 Virtual Private Network 24 I.6.2 Layer 2 Virtual Private Network 24 I.6.3 Layer 3 Virtual Private Network 24 I.7 Network element decomposition due to cost factor evolutions in network planes (user, control, management) 24 I.8 Network element decomposition due to high availability network solutions 24 I.9 Network performance management with respect to load balancing 24 I.10 Network element type “programmable Ethernet switch” 24 I.11 Network element type “programmable IP router” 24 I.12 Network element type “programmable firewall” 24 Appendix II Comparison H.248 and ForCES 25 II.1 ### 25 II.1.1 ### 25 Appendix III Comparison OpenFlow and ForCES 26 III.1 Scope of ForCES 26 III.2 Comparison OpenFlow and ForCES 26 III.3 Emulation of OpenFlow interface by ForCES 26 Bibliography 27

List of Tables

Page

Table 1 – Terminology relations between OpenFlow and H.248 10 Table 1 – Protocol requirements – H.248 vs OpenFlow – Relation controller to controlled network element 16 Table 2 – Protocol requirements – H.248 vs OpenFlow – Flow operations 17 Table 3 – Flow identification – OpenFlow – Flow identifier elements 18 Table 4 – Policy rule actions – OpenFlow – Action types 19 Table 5 – Protocol capabilities – H.248 vs OpenFlow – Basic Capabilities 19 Table 6 – Protocol capabilities – H.248 vs OpenFlow – Termination types for packet-oriented bearer technologies 20 Table 7 – Protocol capabilities – H.248 vs OpenFlow – Termination types for Circuit-oriented bearer technologies 20 Table 8 – Protocol capabilities – H.248 vs OpenFlow – Connection topologies 20 Table 9 – Protocol capabilities – H.248 vs OpenFlow – IP connection models 21 Table 10 – Protocol capabilities – H.248 vs OpenFlow – Policy rule actions – Generic view 21 Table I.1 – OpenFlow use case “prototyping” 23

List of Figures

Page

Figure 1 – Decomposed control models – OpenFlow vs H.248 12 Figure 2 – Decomposed control models – ForCES vs H.248 12 Figure 3 – Packet ﬂow through the processing pipeline (see Fig. 2/[OPENFLOW1.3]) 13 Figure 4 – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules 14 Figure 5 – OpenFlow: basic policy rule format (vs generic DPI policy rule format [ITU-T Y.2770]) 15 Figure II.1 – ForCES Architectural Diagram (see [b-IETF RFC 5810]/Figure 1) 25

Draft new Supplement H.Sup.OpenFlow

Technical report TR H.Sup.OpenFlow: 

Protocol evaluation – OpenFlow versus H.248

Summary

This Supplement ### to the H series of ITU-T Recommendations.

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The "vertical" decomposition of network elements (in packet based communication infrastructures), driven by network evolution scenarios which will demand for "open control" interfaces between the packet transport, forwarding and routing domain and the overlay domain of application/service control, demands for technologies for controlling of packet flows in the most general sense.

There are a number of candidate technologies for such network solutions. Purpose of this document is an attempt in comparing H.248 with OpenFlow (as defined by [OPENFLOW1.3]).

[…]

It has to be noted that an evaluation between H.248 and OpenFlow, - from overall protocol point of view -, is almost impossible due to the very narrowed scope of OpenFlow. OpenFlow was/is only considering on pure Ethernet switches with IP traffic as network elements, hence a packet technology dependent and tightly coupled protocol. H.248 is much more generic and covers all type of packet network entities related to packet switching, packet forwarding/routing, packet interworking, etc. H.248 as a protocol is thus much more powerful and may be considered as a superset with respect to OpenFlow. The capabilities of the OpenFlow protocol may be essentially also provided by H.248.

The results of any assessment are dependent on the considered context, which might be

- protocol Pi should satisfy the requirements given by network solution Nj (i.e., an evaluation against a specific, concrete set of required protocol capabilities);

- protocols Pi and Pj are compared with focus on the common set of capabilities (i.e., looking only at the intersection of Pi ∩ Pj);

- protocol Pi should be able to emulate the capabilities of protocol Pj (or vice versa) (i.e., an evaluation against a set of supported protocol capabilities); or

- any other objectives.

Architects of new networks would be interested in evaluation context 1. Technology replacement strategies would consider evaluation context 2. The scope of this technical report would be rather the third evaluation context.

[…]

The structure of this TR is as follows:

- protocol origins and histories (clause 6);

- packet forwarding models (clause 7);

- stage 1 (services, use cases) consideration (clause 8);

- stage 2 (protocol requirements) consideration (clause 9);

- stage 3 (protocol capabilities) consideration (clause 10);

and

- complementary information to OpenFlow (Appendix I);

- complementary, high-level assessment of H.248 vs ForCES (Appendix II) and

- reference to other work on OpenFlow vs ForCES.

{Editor's note (2013-11 meeting): clause 1 is not yet complete … }

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (2009), Gateway control protocol: IP router packages.

[IETF RFC 3654]	IETF RFC 3654 (2003), Requirements for Separation of IP Control and Forwarding.

[IETF RFC 5810]	IETF RFC 5810 (2010), Forwarding and Control Element Separation (ForCES) – Protocol Specification.

[IETF RFC 5812]	IETF RFC 5810 (2012), Forwarding and Control Element Separation (ForCES) – Forwarding Element Model.

[OPENFLOW1.3]	Open Networking Foundation (04/2013), OpenFlow Switch Specification Version 1.3.2 (Wire Protocol 0x04). 
URL: https://www.opennetworking.org/

{Editor's note (2013-11 meeting): just some initial references … check latest OpenFlow version in general …}

# 3	Definitions

{Editor's note (2013-11 meeting): just some initial terms, related to packet flows …}

## 3.1	Terms defined elsewhere

This supplement uses the following terms defined elsewhere:

3.1.1	flow [IETF RFC 5101]: A set of IP  packets passing an observation point in the network during a certain time interval. All packets belonging to a particular flow have a set of common properties. Each property is defined as the result of applying a function to the values of:

- one or more packet header fields (e.g., destination IP address), transport header fields (e.g., destination port number), or application header fields (e.g., RTP header fields [b-IETF RFC 3550]).

- one or more characteristics of the packet itself (e.g., number of MPLS labels, etc.).

- one or more of fields derived from packet treatment (e.g., next hop IP address, the output interface).

A packet is defined as belonging to a flow if it completely satisfies all the defined properties of the flow.

This definition covers the range from a flow containing all packets observed at a network interface to a flow consisting of just a single packet between two applications. It includes packets selected by a sampling mechanism.

NOTE – The above numbered listed items indicate flow properties in the categories of (1) “Protocol Control Information (PCI) of packets”, (2) “Protocol Data Unit (PDU) properties of packets” and (3) “Local packet forwarding information”.

3.1.2	flow descriptor (also known as flow level conditions) [ITU-T Y.2770]: Set of rule conditions that is used to identify a specific type of flow (according clause 3.1.3) from inspected traffic.

NOTE 1 – This definition of flow descriptor extends the definition in [b-ITU-T Y.2121] with additional elements as described in clause 3.

NOTE 2 – For further normative discussion of the flow descriptor as used in this Recommendation, see Annex A.

3.1.3	IPFIX flow identifier (IPFIX flow ID) [ITU-T Y.2770]: The set of values for the IPFIX flow keys, which is used in conjunction with the flow descriptor to identify a specific flow.

3.1.4	IPFIX flow key [ITU-T Y.2770]: Each of the information elements of the flow descriptor that is used in IPFIX-based flow identification processes (according to [IETF RFC 5101]).

NOTE – The IPFIX flow key definition is semantically consistent with the flow key definition specified in IPFIX [IETF RFC5101]. The only difference between the two terms is that the definition in this document is scoped to the flow descriptor.

## 3.2	Terms defined in this Recommendation

This supplement defines the following terms:

[TBD]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

{Editor's note (2013-11 meeting): some initial abbreviations …}

| CE | Controlling entity (OF controller; H.248 MGC) |
| --- | --- |
| CNE | Controlled network element (OF switch; H.248 MG) |
| ET | Ethernet |
| ForCES | Forwarding and Control Element Separation |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| HTTP | Hyper-text transfer protocol |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| NE | Network element |
| OF | OpenFlow |
| ONF | Open Networking Foundation (www.opennetworking.org) |
| PBB | Provider Backbone Bridges |
| RD | Remote descriptor |
| SDN | Software Defined Networking |
| SEP | (H.248) Stream endpoint |
| SIP | Session initiation protocol |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| UDP | User datagram protocol |

# 5	Conventions

Any?

# 6	Introduction

…

## 6.1	Brief protocol history

The basic idea of H.248 is the decomposition of a network element into control plane and user plane functions. The considered network element types cover circuit switches, frame, cell and packet switches, packet routers and gateways.

OpenFlow (and ForCES) share the same concept of control/user plane separation, but both are limited with respect to network element types in scope (Ethernet switches and IP routers respectively). Both protocols are also not that established and mature than H.248.

The H.248 technology is driven by the industry in terms of major manufactures and network operators, whereas OpenFlow (and ForCES) origin from academia. For instance, none of the major switch and router vendors offer OpenFlow or ForCES interfaces in their products (2012).

## 6.2	Terminology relations

{Editor's note (2013-11 meeting): terminology clarification should be at the very beginning of the TR  …}

Table 1 – Terminology relations between OpenFlow and H.248

| Term | OpenFlow | H.248 | Comments |
| --- | --- | --- | --- |
| Action: | an operation that forwards the packet to a port or modiﬁes the packet, such as decrementing the TTL ﬁeld. Actions may be speciﬁed as part of the instruction set associated with a ﬂow entry or in an action bucket associated with a group entry. Actions may be accumulated in the Action Set of the packet or applied immediately to the packet |  |  |
| Action Bucket: | a set of actions and associated parameters, deﬁned for groups. |  |  |
| Action Set | a set of actions associated with the packet that are accumulated while the packet is processed by each table and that are executed when the instruction set instructs the packet to exit the processing pipeline |  |  |
| Controller: | An entity interacting with the OpenFlow switch using the OpenFlow protocol. | MGC |  |
| Flow Entry: | an element in a ﬂow table used to match and process packets. It contains a set of match ﬁelds for matching packets, a priority for matching precedence, a set of counters to track packets, and a set of instructions to apply. |  |  |
| Flow Table: | A stage of the pipeline, contains ﬂow entries. |  |  |
| Group: | a list of action buckets and some means of choosing one or more of those buckets to apply on a per-packet basis. |  |  |
| Instruction: | Instructions are attached to a ﬂow entry and describe the OpenFlow processing that happen when a packet matches the ﬂow entry. An instruction either modiﬁes pipeline processing, such as direct the packet to another ﬂow table, or contains a set of actions to add to the action set, or contains a list of actions to apply immediately to the packet. | i.e., policy rule actions (see also H.248.79) |  |
| Match Field: | a ﬁeld against which a packet is matched, including packet headers, the ingress port, and the metadata value. A match ﬁeld may be wildcarded (match any value) and in some cases bitmasked | i.e., an information element in policy rule conditions (see also H.248.79) |  |
| Metadata: | a maskable register value that is used to carry information from one table to the next. |  |  |
| Meter: | a switch element that can measure and control the rate of packets. The meter trigger a meter band if the packet rate or byte rate passing through the meter exceed a predeﬁned threshold. If the meter band drops the packet, it is called a Rate Limiter. | a combination of - H.248.53 policer and - H.248 statistics |  |
| Outermost Tag: | the tag that appears closest to the beginning of a packet. |  |  |
| Packet: | an Ethernet frame, including header and payload. | Lx-PDU in general | OF is tightly coupled to ET! |
| Pipeline: | the set of linked ﬂow tables that provide matching, forwarding, and packet modiﬁcations in an OpenFlow switch. | could be modelled as a Context |  |
| Port: | where packets enter and exit the OpenFlow pipeline. May be a physical port, a logical port deﬁned by the switch, or a reserved port deﬁned by the OpenFlow protocol. | could be modelled as a Termination |  |
| Tag: | a header that can be inserted or removed from a packet via push and pop actions. | Lx-PCI in general |  |

## 6.3	Objectives in terms of network architecture

{Editor's note (2013-11 meeting): description of models is still missing …}

Figure 1 – Decomposed control models – OpenFlow vs H.248

Figure 2 – Decomposed control models – ForCES vs H.248

# 7	Packet forwarding model

## 7.1	OpenFlow Processing model

{Editor's note (2013-11 meeting): description still missing …}

Figure 3 – Packet ﬂow through the processing pipeline (see Fig. 2/[OPENFLOW1.3])

## 7.2	Policy rule based description of packet operations

This is a general description technique, applicable for all kind of network element types involved in any kind of packet processing (see also clause 7.3/[ITU-T H.248.79]).

Figure 4 – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules

### 7.2.1	OpenFlow: basic policy rule format

Figure 5 – OpenFlow: basic policy rule format 
(vs generic DPI policy rule format [ITU-T Y.2770])

### 7.2.2	OpenFlow: staged process model – concatenation of multiple policy rules

The general principle is described in ### .

### 7.2.3	OpenFlow: ###

# 8	Stage 1 – Services (use cases)

{Editor's note (2013-11 meeting): stage 1 information are a place holder for the time being; not sure whether we go into detailed considerations  …}

# 9	Stage 2 – Protocol requirements

{Editor's note (2013-11 meeting): stage 2 information may be elaborated in order to understand better the history of the protocols. E.g. ForCES made following agenda for their comparison

- Protocol Requirement: Configuration of Modeled Elements

- Protocol Requirement: Support for Secure Communication

- Protocol Requirement: Scalability

- Protocol Requirement: Multihop

- Protocol Requirement: Message Priority

- Protocol Requirement: Reliability

- Protocol Requirement: Interconnect Independence

- Protocol Requirement: CE Redundancy or CE Failover

- Protocol Requirement: Packet Redirection/Mirroring

- Protocol Requirement: Topology Exchange

- Protocol Requirement: Dynamic Association

- Protocol Requirement: Command Bundling

- Protocol Requirement: Asynchronous Event Notification

- Protocol Requirement: Query Statistics

Protocol Requirement: Protection Against Denial of Service Attacks

…}

## 9.1	Decomposed network element architecture

Table 1 – Protocol requirements – H.248 vs OpenFlow 
– Relation controller to controlled network element

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Single primary controller per switch/MG | Yes | Yes |  |
|  | Secondary Controller(s) as standby entities | Yes | Yes |  |
|  | Multiple primary controllers | Yes | Yes? (NOTE 1) |  |
|  | MG standalone mode | No | No | Temporary condition without any contact to a controller |
| NOTE 1 – The justification of that requirement is missing. It is also not clear how the parallel operation of multiple active controllers would be coordinated. |  |  |  |  |

## 9.2	Operations on traffic flows

The notion of traffic flow may be limited on packet flow, in which ‘packet’ represents generally a protocol data unit (PDU) at protocol layer Lx (i.e., ‘packet’ = Lx-PDU).

### 9.2.1	General operations

Table 2 – Protocol requirements – H.248 vs OpenFlow 
– Flow operations

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Flow termination | Yes | No | Protocol stack termination in CNE |
|  | Flow identification | Yes (NOTE 1) | No |  |
|  | Flow forwarding | Yes | Yes | Basic mode |
|  | Flow modification | Yes | Yes |  |
|  | Reporting of flow information | Yes | Yes |  |
| NOTE 1 – Scope of H.248.86 (by e.g. realizing IPFIX/Y.2770 flow identification service). |  |  |  |  |

### 9.2.2	Regarding policy rule conditions (“flow identification”)

The match field has been expanding as the ONF specifications evolve - from a

- 10-tuple [McKowen-2008], to

- 12-tuple [OFS-1.0],

- 15-tuple [OFS-1.1.0], to

- 39-tuple [OFS-1.3] (14 required and 25 optional).

Table 3 – Flow identification – OpenFlow – Flow identifier elements

[OPENFLOW1.3]: A.2.3.7 Flow Match Fields

/* OXM Flow match field types for OpenFlow basic class. */ enum oxm_ofb_match_fields { OFPXMT_OFB_IN_PORT = 0, /* Switch input port. */ OFPXMT_OFB_IN_PHY_PORT = 1, /* Switch physical input port. */ OFPXMT_OFB_METADATA = 2, /* Metadata passed between tables. */ OFPXMT_OFB_ETH_DST = 3, /* Ethernet destination address. */ OFPXMT_OFB_ETH_SRC = 4, /* Ethernet source address. */ OFPXMT_OFB_ETH_TYPE = 5, /* Ethernet frame type. */ OFPXMT_OFB_VLAN_VID = 6, /* VLAN id. */ OFPXMT_OFB_VLAN_PCP = 7, /* VLAN priority. */ OFPXMT_OFB_IP_DSCP = 8, /* IP DSCP (6 bits in ToS field). */ OFPXMT_OFB_IP_ECN = 9, /* IP ECN (2 bits in ToS field). */ OFPXMT_OFB_IP_PROTO = 10, /* IP protocol. */ OFPXMT_OFB_IPV4_SRC = 11, /* IPv4 source address. */ OFPXMT_OFB_IPV4_DST = 12, /* IPv4 destination address. */ OFPXMT_OFB_TCP_SRC = 13, /* TCP source port. */ OFPXMT_OFB_TCP_DST = 14, /* TCP destination port. */ OFPXMT_OFB_UDP_SRC = 15, /* UDP source port. */ OFPXMT_OFB_UDP_DST = 16, /* UDP destination port. */ OFPXMT_OFB_SCTP_SRC = 17, /* SCTP source port. */ OFPXMT_OFB_SCTP_DST = 18, /* SCTP destination port. */ OFPXMT_OFB_ICMPV4_TYPE = 19, /* ICMP type. */ OFPXMT_OFB_ICMPV4_CODE = 20, /* ICMP code. */ OFPXMT_OFB_ARP_OP = 21, /* ARP opcode. */ OFPXMT_OFB_ARP_SPA = 22, /* ARP source IPv4 address. */ OFPXMT_OFB_ARP_TPA = 23, /* ARP target IPv4 address. */ OFPXMT_OFB_ARP_SHA = 24, /* ARP source hardware address. */ OFPXMT_OFB_ARP_THA = 25, /* ARP target hardware address. */ OFPXMT_OFB_IPV6_SRC = 26, /* IPv6 source address. */ OFPXMT_OFB_IPV6_DST = 27, /* IPv6 destination address. */ OFPXMT_OFB_IPV6_FLABEL = 28, /* IPv6 Flow Label */ OFPXMT_OFB_ICMPV6_TYPE = 29, /* ICMPv6 type. */ OFPXMT_OFB_ICMPV6_CODE = 30, /* ICMPv6 code. */ OFPXMT_OFB_IPV6_ND_TARGET = 31, /* Target address for ND. */ OFPXMT_OFB_IPV6_ND_SLL = 32, /* Source link-layer for ND. */ OFPXMT_OFB_IPV6_ND_TLL = 33, /* Target link-layer for ND. */ OFPXMT_OFB_MPLS_LABEL = 34, /* MPLS label. */ OFPXMT_OFB_MPLS_TC = 35, /* MPLS TC. */ OFPXMT_OFP_MPLS_BOS = 36, /* MPLS BoS bit. */ OFPXMT_OFB_PBB_ISID = 37, /* PBB I-SID. */ OFPXMT_OFB_TUNNEL_ID = 38, /* Logical Port Metadata. */ OFPXMT_OFB_IPV6_EXTHDR = 39, /* IPv6 Extension Header pseudo-field */ };

### 9.2.3	Regarding policy rule actions

Table 4 – Policy rule actions – OpenFlow – Action types

[OPENFLOW1.3]: A A.2.5 Action Structure

enum ofp_action_type { OFPAT_OUTPUT = 0, /* Output to switch port. */ OFPAT_COPY_TTL_OUT = 11, /* Copy TTL "outwards" -- from next-to-outermost to outermost */ OFPAT_COPY_TTL_IN = 12, /* Copy TTL "inwards" -- from outermost to next-to-outermost */ OFPAT_SET_MPLS_TTL = 15, /* MPLS TTL */ OFPAT_DEC_MPLS_TTL = 16, /* Decrement MPLS TTL */ OFPAT_PUSH_VLAN = 17, /* Push a new VLAN tag */ OFPAT_POP_VLAN = 18, /* Pop the outer VLAN tag */ OFPAT_PUSH_MPLS = 19, /* Push a new MPLS tag */ OFPAT_POP_MPLS = 20, /* Pop the outer MPLS tag */ OFPAT_SET_QUEUE = 21, /* Set queue id when outputting to a port */ OFPAT_GROUP = 22, /* Apply group. */ OFPAT_SET_NW_TTL = 23, /* IP TTL. */ OFPAT_DEC_NW_TTL = 24, /* Decrement IP TTL. */ OFPAT_SET_FIELD = 25, /* Set a header field using OXM TLV format. */ OFPAT_PUSH_PBB = 26, /* Push a new PBB service tag (I-TAG) */ OFPAT_POP_PBB = 27, /* Pop the outer PBB service tag (I-TAG) */ OFPAT_EXPERIMENTER = 0xffff };

## 9.3	High availability of services and networks

## 9.4	Public networks: support of regulation related requirements

LI, emergency, …

# 10	Stage 3 – Protocol capabilities

{Editor's note (2013-11 meeting): just some initial areas  …}

Table 5 – Protocol capabilities – H.248 vs OpenFlow – Basic Capabilities

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  |  | Yes | Yes |  |

## 10.1	Termination types: supported protocol stacks in forwarding plane

Table 6 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for packet-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | L2: Ethernet | Yes | Yes (NOTE 1) |  |
|  | L2: other than Ethernet | Yes | No |  |
|  | L3: IP (V4 and V6) | Yes | Yes (NOTE 1) |  |
|  | L3: other than IP | Yes | No |  |
|  | L4: IP transport protocols TCP, UDP, SCTP | Yes | Yes |  |
|  | L4: IP transport protocols DCCP, SCTP, … | Yes | No |  |
|  | Tunneled IP stacks | Yes | No | OpenFlow just IETF-BRM |
| NOTE 1 – The OpenFlow basic network element is an Ethernet switch! Thus, the notion of ‘OpenFlow switch’, rather than IP router or gateway etc. The OpenFlow protocol stack is fundamentally IP-over-Ethernet. |  |  |  |  |

Table 7 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for Circuit-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | circuit-switched bearer | Yes | No (NOTE 1) |  |
| NOTE 1 – OpenFlow is not considering any interworking between IP or Ethernet network domains with circuit switched networks. |  |  |  |  |

## 10.2	Connection models: IP mode of operation in forwarding plane

Table 8 – Protocol capabilities – H.248 vs OpenFlow 
– Connection topologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | pure Ethernet termination | Yes | No |  |
|  | pure IP termination | Yes | No | H.248: (IP, non-IP) topology |
|  | ET-to-ET (switched Ethernet) | Yes | Yes |  |
|  | IP-to-IP (in IP router mode) | Yes | Yes |  |
|  | IP-to-IP (in IP router mode plus local NAT) | Yes | No |  |
|  | IP-to-IP (in B2BIH mode) | Yes | No |  |
|  | IP-to-IP (with protocol version translation) | Yes | No | such as V4-to-V6 |
|  | tunnelled to untunnelled IP interworkign | Yes | No |  |
|  | IP multipoint topologies | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 9 – Protocol capabilities – H.248 vs OpenFlow 
– IP connection models

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | IPR native | Yes | Yes |  |
|  | IPR with NAT | Yes | No |  |
|  | B2BIH native | Yes | No | V4-to-V4 ¦ V6-to-V6 |
|  | B2BIH V4-to-V6 | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 10 – Protocol capabilities – H.248 vs OpenFlow 
– Policy rule actions – Generic view

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Packet Forwarding (called Output in OF) | Yes | Yes |  |
|  | Packet Discarding (called Drop in OF) | Yes | Yes |  |
|  | Packet Scheduling (called Set-Queue in OF) | Yes | Yes | See clause 7.1.6/[ITU-T Y.1221] |
|  | Packet Marking | Yes | Yes (part of Set-Queue)? | See clause 7.1.4/[ITU-T Y.1221] |
|  | Traffic shaping | “Yes” (NOTE 1) | Yes (part of Set-Queue) | See clause 7.1.5/[ITU-T Y.1221] |
|  | Traffic policing | Yes | No | See clause 7.1.2/[ITU-T Y.1221] |
|  | Complete packet header removal or addition (called Push-Tag/Pop-Tag in OF) | Yes | Yes |  |
|  | Partial packet header modification (called Set-Field in OF) | Yes | Yes |  |
|  | Packet lifetime modification (called Change-TTL in OF) | No | Yes | Limited to IP and MPLS TTL. |
|  | Packet-, Flow- and Flow-aggregate-related Measurements |  |  |  |
| NOTE 1 – # |  |  |  |  |

Q1: how is NA(P)T realized?

Appendix I

Scope of OpenFlow – Use cases

(This appendix does not form an integral part of this Recommendation.)

This appendix provides example ##

## I.1	Experimental prototyping (academic/research area)

Table I.1 – OpenFlow use case “prototyping”

| Service description: |  |
| --- | --- |
| Reference(s): |  |
| Network architecture model: |  |
| Time scale of flow control/management: |  |
| Key protocol capabilities: |  |
| H.248 solution: |  |

## I.2	Software defined networking (SDN)

… network management level (traffic engineering)

automated management, self management

## I.3	Network functions virtualisation

## I.4	Cloud oriented networking

see article “OpenFlow as a Service”

## I.5	Trill switching

## I.6	Lx-VPNs (Layer x Virtual Private Network)

### I.6.1	Layer 1 Virtual Private Network

No

### I.6.2	Layer 2 Virtual Private Network

Ethernet based (e.g., PBB, VLAN)

### I.6.3	Layer 3 Virtual Private Network

MPLS based

## I.7	Network element decomposition due to cost factor evolutions in network planes (user, control, management)

## I.8	Network element decomposition due to high availability network solutions

## I.9	Network performance management with respect to load balancing

## I.10	Network element type “programmable Ethernet switch”

## I.11	Network element type “programmable IP router”

## I.12	Network element type “programmable firewall”

Appendix II

Comparison H.248 and ForCES

(This appendix does not form an integral part of this Recommendation.)

This appendix provides example ##

## II.1	###

### II.1.1	###

Figure II.1 – ForCES Architectural Diagram (see [b-IETF RFC 5810]/Figure 1)

Appendix III

Comparison OpenFlow and ForCES

(This appendix does not form an integral part of this Recommendation.)

The ForCES Fp interface could be principally realized also with H.248 (given by an H.248 Fp profile specification). Proponents of ForCES were invited for dispositions against OpenFlow. This appendix provides a brief summary.

## III.1	Scope of ForCES

See [b-IETF RFC 3654], [b-IETF RFC 3746], [b-IETF RFC 5810], [b-IETF RFC 5812].

## III.2	Comparison OpenFlow and ForCES

There is not yet any IETF approved evaluation available. [b-IETF hares] provides a private position.

## III.3	Emulation of OpenFlow interface by ForCES

[b-IETF haleplidis] provides an attempt in emulating OpenFlow by an extension to ForCES.

Bibliography

IETF working group ForCES (Forwarding and Control Element Separation):

[b-IETF RFC 3549]	IETF RFC 3549 (07/2003), Linux Netlink as an IP Services Protocol

[b-IETF RFC 3654]	IETF RFC 3654 (11/2003), Requirements for Separation of IP Control and Forwarding

[b-IETF RFC 3746]	IETF RFC 3746 (04/2004), Forwarding and Control Element Separation (ForCES) Framework

[b-IETF RFC 5810]	IETF RFC 5810 (03/2010), Forwarding and Control Element Separation (ForCES) Protocol Specification

[b-IETF RFC 5812]	IETF RFC 5812 (03/2010), Forwarding and Control Element Separation (ForCES) Forwarding Element Model

[b-IETF hares]	Private draft-hares-forces-vs-openflow (07/2012), Analysis of Comparisons between OpenFlow and ForCES
URL: http://tools.ietf.org/id/draft-hares-forces-vs-openflow-00.txt

[b-IETF haleplidis]	Private draft-haleplidis-forces-openflow-lib (07/2012), Forwarding and Control Element Separation (ForCES) OpenFlow Model Library
URL: http://tools.ietf.org/id/draft-haleplidis-forces-openflow-lib-01.txt

IETF working group IPFIX (IP Flow Information Export):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

IETF working group IRS (Interface to the Routing System):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

IETF working group TRILL (Transparent Interconnection of Lots of Links):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

[b-IETF RFC 6325]	IETF RFC 6325 (07/2011), Routing Bridges (RBridges): Base Protocol Specification

____________________