| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4482 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SEPLINK: seplink package (clause 7) - clarification of syntax and semantics |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides change proposals in order to fix the final open syntactical and semantical items of the seplink package.

NOTE – There's an overlapping with AVD-4541 (Huawei) concerning the ABNF update proposal. This contribution is not yet reflecting the latest discussed status.

1	Introduction

The interlinkage concept was introduced by work items H.248.TCP and H.248.TLS, but generalized later and therefore moved into a generic work item H.248.SEPLINK. A number of editor's notes were addressed against the first draft of the seplink package.

2	Discussion

2.1	Open editor's notes

Below table summarizes all editor's notes and their proposed resolution:

| No. | Editor's note: | Resolution: |
| --- | --- | --- |
| #1 (7.1.1) | {Editor's note (2013-11): the syntax specification of values may benefit from an ABNF based format, e.g. the mode seems to be a sub-list also. Need to allow binary terminationIDs etc.} | ABNF proposed, see change proposal |
| #2 (7.1.1) | {Editor's note (2013-11): the "interlinked-SEP" is the local SEP. The identifier for a SEP is the 2-tuple of of StreamID and TerminationID values (see clause 3.2.10/H.248.1). The StreamID of the "interlinked-SEP" is implicitly given be the StreamDescriptor where the linktopo property is located, but not the TerminationID. Thus we need an explicit indication of the TerminationID value. Below description of the semantic should be enhanced} | The concerned property paramenter interlinkedSEP is now tightly coupled (via ABNF) with the H.248.1 TerminationID. Note: binary encoding option is not considered. |
| #3 (7.1.1) | {Editor's note (2013-11): TerminationID issue is that the semantic of interlinked-SEP is a terminationID but the syntax given is for a StreamID i.e. UINT16. Source-transport-EP and interlinked-transportEP have a similar syntax issue; further: binary encoding should be covered} | TerminationID, see #2. ABNF added for sourceTransportEP and interlinkedTransportEP. |
| #4 (7.1.1) | to mode parameter: {Editor's note (2013-11): a sublist based value would be more future safe} | ABNF rule added, allowing sublists as well the addition of further values (via token). |
| #5 (7.1.1) | TBC: use case: do we need to support multiple transport layer EPs of the same type per SEP, e.g. TCPoTCP, IP-over-IP tunneling? | Topic discussed in parallel AVD-4483. |

2.2	Name of "transport protocol endpoints"

The linktopo property represents a list of one or multiple quadruples. The ABNF for such a quadruple is given by:

linkTopoTuple		= DQUOTE interlinkedSEP

COMMA  sourceTransportEP

COMMA  interlinkedTransportEP

COMMA  mode DQUOTE

The 2nd and 3rd parameter relate to a transport protocol endpoint. The previous value specification was rather open by allowing any kind of string. Such an open value specification option delegates the responsibility to H.248 profiles concerning the definition of used values.

We evaluated a number of alternatives how the property parameter value specification (for sourceTransportEP and interlinkedTransportEP) could be enhanced:

- reference of an existing IANA registry?

- creation of an own H.248.x registry?

- fix list of values defined by H.248.SEPLINK?

An ideal name registry would need to support

- all IP "transport" protocols of L4 and upper layers,

- single values,

- protocol reference models supporting hierarchical or tunnelling layering besides a basic reference model and

- preferably a tight coupling with the SDP "m=" line specification in H.248 LD/RD.

We finally concluded with solution 1 due the future effort in extending and maintaining a self-contained registry in H.248, which should be avoided, particularily with current focus on IETF (IP) protocols only.

Unfortunately, there isn't any IANA registry which fullfills above all requirements. The closest solution is the registry concerning the SDP m-line’s <proto> value (e.g., registry "IP protocol numbers" might be also worth of consideration). There are following issues with this protocol parameter:

- single transport protocol layer indication but also protocol stack segment indications;

- some protocol values are expressed indirectly by multiple values (e.g., 'DTLS' = "UDP/TLS"); and

- some important IP L4 protocols still missing in SDP.

Despite the fact of these deficiencies we'd like to refer to the IANA SDP <proto> registry. The first deficiency is tackled by a numbering scheme, which allows to set a pointer to a particular protocol in case of <proto> values of the form "…/…/…".

The selected approach does not support tunneled protocol stacks (such as IP-over-IP, TCP-over-TCP), but covers all existing use cases identified so far.

NOTE – The discussion of topic "IANA registry" for H.248.SEPLINK overlapps with the discussion of new proposed work item H.SUPP.IANA (see AVD-4569).

2.3	Package procedures

The procedural section is editorially updated.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4477).

Change proposal:

[Begin Proposed Correction]

# 7	Stream endpoint interlinkage package

|  | Package Name: | Stream endpoint interlinkage package |
| --- | --- | --- |
|  | Package ID: | seplink (0x####) |
|  | Description: | This package provides the functionality to interlink connection/session oriented transport protocol endpoints. If two transport protocol endpoints are interlinked the establishment and/or release of a connection/session at the source transport protocol endpoint will trigger an MG-autonomous establishment and/or release of the interlinked transport protocol endpoint. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Interlinkage topology

|  | Property Name: | Interlinkage topology |
| --- | --- | --- |
|  | Property ID: | linktopo (0x0001) |
|  | Description: | This property defines the MG interlinkage behaviour regarding connection/session oriented transport endpoints. Interlinkage can be specified between the connection oriented transport layers of a given SEP or between any connection oriented transport layers of a given SEP pair. Interlinkage defines a undirectional information flow ("stimuli") from a "SEP-local transport EP" towards another "SEP-local or –remote transport EP". |
|  | Type: | Sub-list of Strings |
|  | Possible values: | The property consists of a sequence of link topology quadruples, where each link topology quadruple consists of an interlinked H.248 stream endpoint, a source transport endpoint, an interlinked transport endpoint and a mode. Therefore the value of the linktopo property shall be of type linkTopoTupleList in following ABNF format: linkTopoTupleList = LSBRT linkTopoTuple *(COMMA linkTopoTuple) RSBRT linkTopoTuple = DQUOTE interlinkedSEP COMMA sourceTransportEP COMMA interlinkedTransportEP COMMA mode DQUOTE With LSBRT, RSBRT, COMMA and DQUOTE as defined in clause B.2 of [ITU-T H.248.1.] Where: - interlinkedSEP: - Semantic: specifies the interlinked stream endpoint by using the TerminationID, possibly using the ALL or CHOOSE wildcard. In case the interlinked TerminationID is the same TerminationID as of the SEP the property is belonging to vertical interlinkage is triggered. If they are not the same, horizontal interlinkage is triggered. - Syntax: As per following ABNF rule: interlinkedSEP = TerminationID with TerminationID as defined in clause B.2 of [ITU-T H.248.1]. - sourceTransportEP: - Semantic: Refers to the source transport endpoint in the source SEP m-line’s <proto> value, where the <proto> value’s individual <token> values are enumerated from left to right with positive integers starting with 1. E.g. if the m-line’s <proto> value was “TCP/TLS/MSRP”, then the sourceTransportEP value “1” would refer to “TCP”, whereas the value “2” would refer to “TLS”. An H.248 profile using this package may define additional sourceTransportEP values of type token. - Syntax: As per following ABNF rules: sourceTransportEP = transportEP transportEP = POS-DIGIT / token with POS-DIGIT and token as defined in clause 9 of [IETF RFC 4566]. - interlinkedTransportEP: - Semantic: Refers to the interlinked transport endpoint in the interlinked SEP m-line’s <proto> value in the same way as the sourceTransportEP value refers to the source transport endpoint as described above. As for sourceTransportEP an H.248 profile may define additional interlinkedTransportEP values of type token. - Syntax: As per following ABNF rule: interlinkedTransportEP = transportEP with transportEP as defined for sourceTransportEP. NOTE: Detailed agreements regarding the admitted SDP "m="-line transport protocol <proto> values shall be defined in the particular H.248 profile specifications, typically in the profile clauses on mandatory/optional support of SDP. - mode: - Semantic: Specifies whether the interlinkage is related to connection/session establishment and/or release procedure; Possible values: - "est": establishment interlinkage only. - "rel": release interlinkage only. - "est-rel": establishment and release interlinkage. - An H.248 profile using this package may define additional linkTopoMode values of type token. - Syntax: As per following ABNF rule: mode = LSBRKT linkTopoMode *(COMMA linkTopoMode) RSBRT linkTopoMode = "est" / "rel" / "est-rel" / token with LSBRT, RSBRT and COMMA as defined in clause B.2 of [ITU-T H.248.1.] and with token as defined in clause 9 of [IETF RFC 4566]. |
|  | Default: | Empty list |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 7.2	Events

None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

None

## 7.6	Procedures

### 7.6.1	Configuration of "interlinkage topologies"

#### 7.6.1.1	SEP identification:

a) Source of stimuli:

The source SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which triggers autonomous procedures at another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID and StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value. The name of the source SEP is thus implicitly given.

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "sourceTransportEP").

b) Sink of stimuli:

The destination SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which receives a trigger to start autonomous procedures from another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID is explicitly specified in the seplink/linktopo property value (parameter "interlinkedSEP").

- The StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value.

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "interlinkedTransportEP").

#### 7.6.1.2	Traffic directions:

The seplink/linktopo property defines a unidirectional behavior, that is it specifies the interlinkage behavior of a source (identified by the TerminationID and StreamID the LocalControl descriptor is related to) and an interlinked SEP (identified by the TerminationID; same StreamID as source).

In case a bidirectional interlinkage between a SEPP/SEPT is expected, each SEP’s Local Descriptor must contain a corresponding interlinkage topology.

#### 7.6.1.3	Wildcard usage:

The interlinked SEP can be specified using the ALL wildcard

"seplink/linktopo = ["*,TCP,TCP,est-rel"]".

In this case the specified interlinkage behavior is valid to all matching SEPPs (i.e. only horizontal interlinkage, vertical interlinkage is specifically excluded, that is ALL wildcard excludes own TerminationID). In case a new SEP is added into the H.248 context later on, the ALL wildcarded interlinkage topology is applied as well to the new SEP.

The interlinked SEP can be specified using the CHOOSE wildcard

"seplink/linktopo = ["$,TCP,TCP,est-rel"]".

The CHOOSE wildcard matches the TerminationID that the MG assigns in the first Add Command that uses a CHOOSE wildcard in the same action. This allows to add a new termination/SEP, and to modify the existing interlinkage topologies (using a modify command) within the same action.

Transaction = 1 {

Context = 1 {

Add = ip/1/$/$ {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["ip/1/1/1,TCP,TCP,est-rel"] }}}}

Modify = ip/1/1/1 {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["$,TCP,TCP,est-rel"] }}}}

### 7.6.2	Relation to H.248 Stream Mode:

A specified interlinkage topology is independent of the StreamMode property of the LocalControl Descriptor. The StreamMode property affects the application data flow rather than the transport protocol control information used to establish or close the corresponding transport connection.

### 7.6.3	Relation to H.248 Topology descriptor

The effectiveness or non-effectiveness of a specified horizontal interlinkage topology depends on the context level topology descriptor, i.e. whether the corresponding flow direction in the topology descriptor for a particular SEPP (or in case of wildcard ALL usage: for all matching SEPPs) has been enabled or not.

### 7.6.4	Error cases:

In case a new interlinkage topology is specified refering to an unknown termination identifier, then the MG shall reject this using Error #430 "Unknown TerminationID".

In case a new interlinkage topology is specified refering to a valid termination identifier but this termination doesn’t contain an SEP for this specific stream identifier, then the MG shall reject this using Error #581 "Does Not Exist".

In case a new interlinkage topology is specified refering to a transport protocol not being part of the SEP’s m-line transport protocol, then the MG shall reject this using Error #581 "Does Not Exist".

### 7.6.5	Examples:

#### 7.6.5.1	Example of a "TLS/TCP to SCTP" connection model

i) Example of intra-SEP interlinkage:

Given an SEP-A (T-A) uses transport layers TLS/TCP. A TCP connection setup shall autonomously trigger a setup of the TLS session and a TLS session release shall autonomously trigger a release of the TCP connection.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,TCP,TLS,est","T-A,TLS,TCP,rel"]
} } } }

The resulting interlinkage looks as follows (Fig. 4):

Figure 4 – Example of a "TLS/TCP to SCTP" connection model 
– Vertical interlinkage configuration at SEP-A

ii) Example of inter-SEP interlinkage:

In addition SEP-A’s transport layer TCP can be interlinked with SEP-B’s transport layer SCTP (i.e. SEP-A’s TCP connection establishment/release shall autonomously trigger SEP-B’s SCTP connection establishment/release).

Note: SEP-A’s TCP connection establishment/release can be either stimulated by using the corresponding H.248 TCP package capabilities ( e.g., [ITU-T H.248.TCP] properties tcpbcc/EstBNC (outgoing establishment procedure (TCP client role)) or tcpbcc/RelBNC) or by receiving TCP connection establishment/release request from the far-end TCP endpoint.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,TCP,TLS,est", "T-A,TLS,TCP,rel",
                            "T-B,TCP,SCTP,est-rel"]
} } } }

Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP"

iii) Example of bidirectional inter-SEP interlinkage:

To make sure that SCTP connection establishment at SEP-B will also trigger SEP-A TCP connection establishment and that SCTP connection release at SEP-B will also trigger SEP-A TLS session release, the seplink/linktopo property has to be applied as follows in SEP-B’s LocalControl Descriptor:

H.248 command request:

Modify = T-B {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A,SCTP,TCP,est", "T-A,SCTP,TLS,rel"]
} } } }

Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS"

[End Proposed Correction]

____________________