| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4492 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of clause 6.2 & Appendix I |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution intends the completion of clause 6.2 and Appendix I by fixing the remaining coments.

Discussion

- Comment [A…] "Propose to add further use case: WebRTC data channel: TLS/TCP to SCTP/DTLS/UDP." 
 already done (see Appendix I.3);

- Comment [A…] "Use case numbering? This list here does not seem to be related to Appendix I." 
 Use case numbering aligned ("bearer connection use cases" from clause 6.2 with "network application use cases" from Appendix I);

- {Editor's note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}
 that two year old comment should be obsoleted in the meanwhile by progress on H.248.TLSPROF;

- Comments in clause 6.2.3 concerning practical use cases for TLS in TCP-to-non-TCP gateways
 also obsolete due to recent addition of Appendix I.3

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal #1:

[Begin Proposed Correction]

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as "non-TLS to non-TLS");

- Use case #1.2: an end-to-end TCP connection traversing two network domains with different transport security policies ("TLS", "no TLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "TLS to non-TLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as "TLS to TLS");

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as "TLS to TLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.2	Use case variations

#### 6.2.2.1	"TLS-to-TLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of TLS transparent forwarding according clause 3.2.14.

NOTE  – That TLS transparent forwarding case is also described in the ETSI TISPAN Border Gateway Guidelines (see clause I.4.2.2/[b-ETSI TR 183 068]).

#### 6.2.2.2	"TLS-to-TLS application aware and decryption-encryption based forwarding"

This scenario is a variation of use case #1.3.

The MG may be required to be application aware in case of e.g. bearer-level ALG support (according to [ITU-T H.248.78]). The MG is located within a single security domain, therefore no change of the TLS profile. However, the MG is located at the edge of two network address domains, thus is requested for to support NAT-T and NAT.

Example use case:

MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the "TLS-to-TLS" interworking function is termed as "TLS B2BUA". It may be noted that this use case does not belong to the "TLS-to-TLS*" category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

## 6.3	Bearer connection network use cases with H.248 "TCP to non-TCP" gateways

Relates scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer. An example network scenario is described in Appendix I.3.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix I

Sample use cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use case #I.1 "Terminal-to-MG TLS session, provisioned TLS service negotiation"

This use case is based on the bearer connection use case #1.2 (from clause 6.2.1) and illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, in either case the MGC is aware of the applied TLS network domain profile for the secured bearer traffic.

- IP transport bearer type: 'TCP' is used in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [b-IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS (see also clause 6.2.1, use case #1.2).

- Bearer path protocol layering: "Application/TLS/TCP/IP" (i.e., no tunnelled transport).

- UE: TCP client and TLS client, MG: TCP server and TLS server

- H.248 MG TCP-mode: "TCP proxy".

NOTE – In more detail: L4/L3 NAT traversal support requirements during the establishment phase of TCP bearer connection may initially imply a TCP merge mode (see [ITU-T H.248.84]), which later transitions to a TCP proxy mode (due to the TLS protocol termination here for TLS to non-TLS interworking). Another option is to use the TCP proxy mode from the very beginning, e.g., in case of lacking NAT-T support.

- Mutual TLS-authentication via certificates, MG's certificate is signed, UE's certificate is self-signed.

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG (e.g., in case of an H.248 profile without support of the TLS capability negotiation package, see clause 10).

Figure I.1 – Use-case #I.1: Bearer path TLS security between UE and MG

## I.2	Use case #I.2 "Terminal-to-MG TLS session, MGC-controlled TLS service negotiation"

This use case is an extension of the first use case #I.1 (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level via usage of the the TLS capability negotiation package (see clause 10). The first use case is based on preconditions which are partially subject of network provisioning. This use case is motivated by the fact that there are a number of reasons which demand MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

## I.3	Use case #I.3 "WebRTC to NGN/IMS interworking function with DTLS-to-TLS support"

WebRTC-based data uses "SCTP/DTLS/UDP/IP" as transport (see also Appendix I.1/[ITU-T H.248.TCP]. An H.248 WebRTC gateway (see [b-ITU-T H.248.WebRTC]) could be requested for interworking to "TLS/TCP/IP" based transport, resulting in a DTLS-to-TLS scenario.

## I.4	Use-case #I.4 "TLS-based transport security for facsimile packet relay service T.38"

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

[End Proposed Correction]

____________________

Defined as: "A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint." ([IETF RFC 6714]).