| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4494 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of TLS extended session control package (clause 9 & Appendix V) (by reference to H.248.SEPLINK) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides a revision of clause 9 and Appendix V due to new reference to H.248.SEPLINK.

Discussion

Main editor's note to be resolved:

{Editor's note (2013-10 meeting): both properties ares dependent on the discussion the proposal for a generic "interlinkage" property, see  C-258 & C-264 ("H.248.SEPLINK"). If H.248.SEPLINK would be agreed, then following updates would be required for H.248.TLS:
a) clauses 9.1.1 & 9.1.2: deleted
b) new clause 9.5.1 concerning dedicated error code of TLS-specific interlinkage.
c) clause 9.6: updated by refererring to H.248.SEPLINK property
d) Appendix V: updated.).

Resolution:

We propose a similar proceeding as in case of H.248.TCP, see AVD-4487). Thus, we are suggesting to refer from clauses 9.1.1 and 9.1.2 to clause 7.1.1/H.248.SEPLINK, rather then deleting both. Because H.248.SEPLINK defines a generic, application-independent interlinkage capability, whereas the tlsesc package provides the usage and procedural information when applied on concrete bearer type "TLS" and "TLS/L4".

The focus of package tlsesc would be then on the specification on the concrete usage of interlinkage for TLS/L4 enabled stream endpoints, - with support of inter-SEP and intra-SEP interlinkage. There are some, technology-dependent limitations, which may be covered by a new error code (new clause 9.5.1).

Furthermore, Appendix V requires correspondent updates.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal #1:

[Begin Proposed Correction]

# 2	References

…

[ITU-T H.248.SEPLINK]	Recommendation ITU-T H.248.SEPLINK (2014), Gateway control protocol: Stream endpoint interlinkage package.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 9	TLS extended session control package

|  | Package Name: | TLS extended session control package |
| --- | --- | --- |
|  | Package ID: | tlsesc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TLS bearer session control behaviour across a "horizontal" Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - TLS/L4 bearer control behaviour across "vertical" protocol layers, rather instead of decoupled, individual control of TLS security sessions and their underlying IP transport connection. NOTE - Both capabilities are realized via the generic interlinkage capability according to [ITU-T H.248.SEPLINK]. This package is providing the application-specific and SEPP procedures for horizontal and vertical TLS interlinkage. |
|  | Version: | 1 |
|  | Extends: | tlsbsc |

## 9.1	Properties

### 9.1.1	Stream endpoint pair interlinkage

### Void clause. Property interlinkage topology (linktopo) from [ITU-T H.248.SEPLINK], clause 7.1.1 may be used for "horizontal" interlinkage TLS procedures at establishment or release phase.

### 9.1.2	Protocol layers interlinkage

### Void clause. Property interlinkage topology (linktopo) from [ITU-T H.248.SEPLINK], clause 7.1.1 may be used for "vertical" interlinkage TLS procedures at establishment or release phase.

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

### 9.5.1	Incorrect (TLS) interlinkage

|  | Error Code #: | <number> |  |
| --- | --- | --- | --- |
|  | Name: | Incorrect (TLS) interlinkage |  |
|  | Definition: | Vertical, protocol layers interlinkage (as based on a intra-SEP interlinkage configuration) is principally limited in case of "TLS-over-L4" protocol stack segments, particularily when the TLS used IP transport protocol would be connection-oriented (such as TCP). The MGC shall only use intra-SEP interlinkage configurations as defined by clauses 9.6.2.4 and 9.6.3.4. Other intra-SEP interlinkage configurations would violate protocol concepts and shall be replied by this error code. |  |
|  | Error text in the Error Descriptor: |  | Incorrect TLS interlinkage configuration. |
|  | Comment: | None. |  |

## 9.6	Procedures

### 9.6.1	Introduction

The procedures are related to the extension package tlsesc and are on top of the base package tlsbsc procedures according clause 8. The interlinkage property seplink/linktopo (according to [H.248.SEPLINK]) could be applied for bearer type "TLS" and the establishment and release procedures for self-contained TLS sessions, but also combined "TLS/L4" bearers.

There are two, self-contained interlinkage applications from perspective of this Recommendation (Table x.1):

Table x.1 – Interlinkage profiling by the TLS extension package tlsesc

| TLS interlinkage type | based on generic interlinkage type [ITU-T H.248.SEPLINK] |
| --- | --- |
| Stream endpoint pair interlinkage for TLS | Inter-SEP interlinkage configuration for a TLS-enabled SEP and a partner SEP with or without TLS. Interlinkage shall be limited on TLS layer or TLS and the L4 protocol at the partner SEP. |
| Protocol layers interlinkage for "TLS/L4" stack segments | Intra-SEP interlinkage configuration for TLS-enabled SEPs and interlinkage between TLS and the lower layer IP transport protocol. |

Both TLS interlinkage types are disjoint and may be combined. Bearer control related procedures are defined in following clauses.

### 9.6.2	TLS security session establishment

#### 9.6.2.1	Stream endpoint pair interlinkage disabled

When inter-SEP interlinkage is not applied, then the incoming TLS security session establishment request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS establishment across a TLS-enabled SEPP (see example H.248 syntax in Table x.2), then the incoming TLS security session establishment request shall be propagated to the partner SEP, triggering there an outgoing TLS ClientHello message. It has to be noted that such kind of interlinkage may be limited to bearer connection use case #1.4 (see clause 6.2.1).

Table x.2 – TLS security session establishment – SEPP interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TLS:TLS:est"] or seplink/linktopo = ["<interlinkedSEP>:<L4>/TLS: <L4>/TLS:est"] }}…} | Interlinkage (for stream S1) is enabled from termination T1 to termination T2 (given by property parameter value "interlinkedSEP"). |

#### 9.6.2.3	Protocol layers interlinkage disabled

When intra-SEP interlinkage is not applied, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.2.4	Protocol layers interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS establishment within a single TLS-enabled in order to interlink with the underlying IP transport protocol endpoint (see example H.248 syntax in Table x.3), then an incoming or outgoing (NOTE 2) L4 transport connection establishment request (such as a TCP SYN packet) shall immediately trigger an outgoing TLS security session establishment request, after a successfully established L4 bearer.

Table x.3 – TLS security session establishment – Protocol layers interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["T1:<L4>:TLS:est"] ???????? or seplink/linktopo = ["T1:<L4>:<L4>/TLS:est"] ???????? }}…} | Intra-SEP interlinkage (for stream S1) is enabled (given by self-referential property parameter value "interlinkedSEP = T1"). It should be noted that the interlinkage direction is only from lower (L4) to upper (TLS) protocol layer (and not vice versa for establishment). I.e., an interlinkage configuration with "<source transport EP> = L4" and "<interlinked tranport EP> = TLS". |

NOTE 1 – Appendix V.4.2.1.1 provides a signalling example.

NOTE 2 – E.g., either MGC driven or in case of e.g. TLS-over-TCP might be also inter-SEP interlinkage enabled at TCP layer, see  [ITU-T H.248.TCP].

### 9.6.3	TLS security session release

#### 9.6.3.1	Stream endpoint pair interlinkage disabled

When inter-SEP interlinkage is not applied, then the incoming TLS security session release request shall not be propagated to the partner SEP, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.3.2	Stream endpoint pair interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS release across a TLS-enabled SEPP (see example H.248 syntax in Table x.4), then the incoming TLS security session release request shall be propagated to the partner SEP, triggering there an outgoing TLS closure_notify alert message. It has to be noted that such kind of interlinkage may be limited to bearer connection use case #1.4 (see clause 6.2.1).

Table x.4 – TLS security session release – SEPP interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TLS:TLS:rel"] or seplink/linktopo = ["<interlinkedSEP>:<L4>/TLS: <L4>/TLS:rel"] }}…} |  |

#### 9.6.3.3	Protocol layers interlinkage disabled

When intra-SEP interlinkage is not applied, then any kind of TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in base package tlsbsc procedures.

#### 9.6.3.4	Protocol layers interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS release within a single TLS-enabled in order to interlink with the underlying IP transport protocol endpoint (see example H.248 syntax in Table x.5), then an incoming or outgoing TLS security session release request shall immediately trigger an outgoing L4 bearer connection release request, after a successfully released TLS session.

Table x.5 – TLS security session establishment – Protocol layers interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["T1:TLS:<L4>: rel"] ???????? or seplink/linktopo = ["T1:<L4>/TLS:<L4>:rel"] ???????? }}…} | It should be noted that the interlinkage direction is only from upper (TLS) to lower (L4) protocol layer (and not vice versa for release). I.e., an interlinkage configuration with "<source transport EP> = TLS" and "<interlinked tranport EP> = L4". |

NOTE 1 – Appendix V.4.2.2 provides signalling examples.

### 9.6.4	Unsuccessful interlinkage

#### 9.6.4.1	Incorrect intra-SEP interlinkage configurations

The MG shall reject incorrect interlinkage configuration requests using error code ### according to clause 9.5.1

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix V

Illustration of protocol semantics 
of the TLS extended session control package

(This appendix does not form an integral part of this Recommendation.)

## V.1	Overview

The tlsesc package (clause 9) defines an augmented TLS security session control on top of the base package (tlsbsc, clause 8) by reuse of [ITU-T H.248.SEPLINK] property seplink/linktopo applied on TLS.

The notion of stream endpoint pair interlinkage for TLS is based on the generic inter-SEP interlinkage configuration and protocol layers interlinkage for "TLS/L4" stack segments is based on the generic intra-SEP interlinkage configuration (see clause 9.6.1).

The effect of SEPP Interlinkage is illustrated in clause V.3  and of Protocol Layers Interlinkage in clause V.4. Clause V.5 describes examples of combined usage of inter- and intra-SEP interlinkage for TLS..

The illustrated use cases are only examples. An exhaustive consideration of all possible combinations is out of scope.

## V.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS and L4 layers) are highlighted besides the H.248 interface.

Furthermore some Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS/L4 SEP.

## V.3	Usage of SEPP Interlinkage property

Such an inter-SEP interlinkage configuration for TLS affects TLS security session establishment and release procedures. The semantics are correspondent to the TCP layer. See e.g. the illustration and examples in [ITU-T H.248.TCP], Appendix IV.3.

## V.4	Usage of Protocol Layers Interlinkage property

Property tlsesc/plslink is defined in clause 9.1.2. The property impact is depicted in below two illustrations. Figure V.1 recalls again the default mode according base package (tlsbsc):

Figure V.1 – Illustration of Protocol Layers Interlinkage 
– scenario without intra-SEP interlinkage

The MGC is responsible for individual control of layer-specific bearer control procedures.

Figure V.2 illustrates the tightly coupled mode, which is conditional to the aimed communication service, but when applicable, it provides some benefits such as minimal MGC involvement and performance optimized bearer establishment and/or release.

Figure V.2 – Illustration of Protocol Layers Interlinkage property 
– scenario with intra-SEP interlinkage between TLS and L4

### V.4.1	Without intra-SEP interlinkage

Without intra-SEP interlinkage between TLS and L4 implies base package behaviour.

### V.4.2	Using intra-SEP interlinkage

Intra-SEP interlinkage may be applied for the establishment or release phase or both (with some restrictions).

#### V.4.2.1	Bearer establishment phase

##### V.4.2.1.1	Incoming direction

Figure V.3 illustrates the normal case: the start of any TLS security session establishment is preceded by a successfully established IP transport connection. Thus, the MG receives an incoming L4 Est.req (2). When L4 is established, then immediately an outgoing TLS session establishment (4) is initiated by the MG.

Figure V.3 – Correct "TLS-L4" interlinkage configuration for establishment – L4 incoming bearer establishment request

The actually impossible case of TLS establishment request first, without yet successfully established IP transport, is indicated Figure V.4.

Figure V.4 – Correct "TLS-L4" interlinkage configuration for establishment, but TLS incoming bearer establishment request first

##### V.4.2.1.2	Outgoing direction

SEP T1(S1) initiates an outgoing L4 bearer establishment procedure (see Figure V.5). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent inter-SEP interlinkage settings). After successful IP transport connection establishment (3, 4), the MG immediately initiates an outgoing TLS session establishment (5).

Figure V.5 – Correct "TLS-L4" interlinkage configuration for establishment – L4 stimuli

Figure V.5 shows also the optional event notifications related to bearer specific state transitioning (here: L4 and TLS reporting).

There might be also a TLS-related stimuli first from SEP perspective (without a prior explicit L4 trigger). However, leading to an unsuccessful  scenario, replied by an error code, see Figure V.6:

Figure V.6 – Correct "TLS-L4" interlinkage configuration for establishment, but incorrect TLS stimuli

#### V.4.2.2	Bearer release phase

##### V.4.2.2.1	Incoming direction

Figure V.7 illustrates the abnormal case: the start of any IP transport connection release precedes a successfully released TLS security session. The MG receives an incoming L4 Rel.req (2), but would suspend L4 release procedural steps in favour of a TLS session release (3) first. The L4 release procedure is resumed again (5) after successful TLS termination (4). However, such a scenario is beyond the semantic of intra-SEP interlinkage, therefore not supported!

Figure V.7 – Correct "TLS-L4" interlinkage configuration for release, but L4 incoming bearer release request first

The standard case of TLS release request first is depicted in Figure V.8: the TLS session release (2, 3) triggers the subsequent IP transport connection release (4, 6).

Figure V.8 – Correct "TLS-L4" interlinkage configuration for release – TLS incoming bearer release request

##### V.4.2.2.2	Outgoing direction

SEP T1(S1) initiates an outgoing L4 bearer release procedure (see Figure V.9). Such a L4-related stimuli may be received the partner SEP (1) (due to correspondent inter-SEP interlinkage settings) (but not from the MGC!). After successful TLS session release (3, 4), the MG immediately initiates an outgoing IP transport connection release (5).

Figure V.9 – Correct "TLS-L4" interlinkage configuration for release, but L4 stimuli first

Above scenario in Figure V.9 is feasible when using a correct inter-SEP interlinkage configuration by connecting source "TCP at partner SEP" to destination "TLS at T1(S1)".

Figure V.10 outlines the case of a TLS-related stimuli (here originated from MGC, but might be also from partner SEP T2(S1)):

Figure V.10 – Correct "TLS-L4" interlinkage configuration for release – TLS stimuli

[End Proposed Correction]

____________________