| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4498 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Use cases for "TLS session resumption" and correspondent update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The capability of "TLS session resumption" and their support by H.248 entities was a pending action item. This contribution provides an analysis by consideration of two examples, and a proposal how we could document this capability in H.248.TLS.

1	Introduction

TLS session resumption is a feature allowing the reuse of a recently established TLS session. This results in an abbreviated TLS handshake for a new session by avoiding the computationally costly calculation of the TLS master secret.

TLS session resumption is only applicable for sessions between a given TLS-server/client pair.

This contribution presents use cases for which TLS session resumption support by a MG will reduce the utilization of network resources as well as TLS session setup time.

2	Discussion

In this contribution the term “resumable session" refers to a TLS session for which (after the handshake has been completed) the security context has been cached.

The term “resumed session" refers to TLS sessions that make use of the cached security context of resumable sessions, thus resumed sessions are utilizing an abbreviated handshake.

2.1	Use case: RCS: File transfer session initiated while a chat session is active

This use case is based on the document “RCC.07 Rich Communication Suite 5.1 Advanced Communication Services and Client Specification, Version 4.0" provided by the GSM Association.

Reference: http://www.gsma.com/network2020/rcs/specs-and-product-docs/

The following paragraph is quoted from this document:

"3.5.2  Interaction with other RCS features

A 1-to-1 File Transfer is not linked to other services (for example CS-voice call or ongoing chat session) and can be used either during or outside of other communication sessions.

The procedure for any file transfer within an ongoing 1-to-1 chat session is implemented as a separate session in parallel with the ongoing 1-to-1 chat and therefore is the same as the procedure for initiating a separate session for File Transfer."

The use case presented focuses on the interface between UE1 and the AGW (MG-1). It can be characterized as follows:

- E2ae security is applied between UE-1 and MG-1, i.e. the MSRP sessions between the UE1 and the MG-1 are secured by TLS.

- A 1-to-1 MSRP chat session is established between two UEs.

- During the chat session a file transfer session is established between the same participant.

- File transfer via MSRP is used.

Figure 1: established chat session between two UEs

Figure 1 shows a simplified network configuration after the chat session between UE-1 and UE-2 is established.

During the chat session both UEs decide to setup an additional session for a file transfer. Figure 2 shows the network configuration after file transfer session is established.

Figure 2: Parallel sessions for chat and file transfer between two UEs

In case the MSRP connections between the UE and the MG are secured by means of using TLS (e2ae security) TLS session resumption may be used to setup the TLS session for the file transfer.

Figure 3: E2ae security: potential TLS session resumption

To apply TLS session resumption at least the following conditions must be true:

- Both TLS sessions are established between the same TLS-client/server pair.

- TLS session resumption supported by the UE.

- TLS session resumption supported by the MG on root termination level.
For discussion: this is a rather flat statement. In which cases exactly session resumption is supported might be implementation dependent.

- TLS client/server role of the resumed session is the same than for the previous session.

- Etc?

Figure 4: Scenario for TLS session resumption for the file transfer

The scenario in figure 4 shows that the TLS security context for the chat session is saved locally at UE1 and/or at the MG-1.

When the new TLS session for the file transfer is setup, the security context from the chat session is retrieved at the UE1 and/or MG-1, thus session resumption can be used for the TLS handshake.

After the TLS handshake for the file transfer is finished, two TLS sessions between UE-1 and MG-1 are active in parallel.

2.2	Use case: RCS: Resumption of an interrupted file transfer

In the previous use case TLS session resumption was applied to resume a currently active TLS session, thus resulting into two established TLS sessions in parallel between the same endpoints.

In use case presented here TLS session resumption is used to resume a session which has been closed.

The use case being presented can be characterized as follows:

- A 1-to-1 file transfer is established between UE-1 and UE-2

- E2ae security is applied between UE-1 and MG-1, i.e. the MSRP sessions between the UE-1 and the MG-1 are secured by TLS.

- The file transfer between UE-1 and UE-2 fails for any reason, the media session as well as the signalling dialog is terminated.

- The file transfer is resumed by establishing a new signalling dialog and a new media session.

The scenario differs from the previous use case in the sense that the two TLS sessions are setup sequentially.

Figure 5: Scenario for resuming an interrupted file transfer

2.3	Constraints for session resumption

From MG perspective constraints apply to setup a resumable session and a resumed session.

Constraints for a resumable session:

- MG internal constraints
The capability for TLS session resumption must be supported in the MG. An implementation that does not support caching of a TLS security context can still support session resumption on the server side if session tickets as specified in RFC 5077 are used. Therefore the capability to support session resumption might be different for server- and client-side implementation and it may depend on the supported method used for session resumption (session Ids vs. session tickets).

- Constraints defined by the H.248 interface
Do we need a control mechanism via H.248? The demand for a H.248 property may depend on whether use cases can be identified where the MGC needs to disallow a session to be resumable. 
To be dicussed.

- Constraints enforced by the TLS protocol
Cases are defined in the base TLS RFCs which explicitly forbid a session to be resumable.

Constraints for resumed session:

- MG internal constraints
These are the same constraints as defined above for resumable sessions.

- Constraints defined by the H.248 interface
Is there a use case where the MGC might need to indicate that a TLS session to be created shall not be resumed from another TLS session?
To be discussed.

- Constraints enforced by the TLS protocol

  - A resumable session must exist (cached session ID available, session ticket available, resumable session not expired, certificate not expired, etc)

  - Both TLS endpoints must support the same method for session resumption (session ID vs. session tickets)

3	Conclusions

The TLS session resumption capability may be basically supported by H.248 entities. The usage as such isn't straightforward. Thus, we'd like to suggest a dedicated new Appendix on TLS session resumption in order to outline constraints, which may need to be considered by H.248 profile specifications. The proposed new text for such an Appendix (see below) provides answer proposals for the indicated questions for clarification in previous discussion section.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal #1:

[Begin Proposed Correction]

ToDo:... the capability of "TLS session resumption" needs to be linked from the main body of the Recommendation to the new proposed Appendix VII …

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix VII

TLS session resumption – Framework

(This appendix does not form an integral part of this Recommendation.)

{Editor's note (2014-03 meeting): we need to revisit again terminology – "resumable TLS session", "resumed TLS session", "resuming TLS session", "presumable TLS session" and "TLS security context", "TLS security parameters" …}

## VI.1	Introduction

Support of TLS sessione resumption is principally feasible in network solutions with H.248 entities, but not straightforward due to the typical nature of communication services supported by H.248 profiles (e.g., given by characteristics of call-dependent gateway control activities, call-to-bearer mapping models, limited call holding times, etc.).

H.248 profiles with support of TLS session resumption need to define carefully the conditions and procedures for this particular TLS capability.

Purpose of this Appendix is to provide example use cases and to identify typical operational constraints.

## VI.2	Brief summary of TLS session resumption

TLS session resumption is a feature allowing the reuse of a recently established TLS security session. This results in an abbreviated TLS handshake (instead of a full TLS handshake) for a new TLS session by avoiding the computationally costly calculation of the TLS master secret.

Hence, the principle benefit of TLS session resumption is twofold: a) reduced utilization of network resources as well as b) optimized TLS session setup times.

TLS session resumption is only applicable for TLS sessions between a given TLS server/client pair.

## VI.3	Example use cases

The notion of "resumable session" refers to a TLS security session for which (after the handshake has been completed) the security context has been cached. The notion of "resumed session" refers to TLS sessions that make use of the cached security context of resumable sessions, thus resumed sessions are utilizing an abbreviated handshake.

TLS session resumption may be applied in different directions:

- parallel usage: multiple TLS-enabled bearer connections in parallel (see clause VI.3.1); or

- serial usage: a single TLS-enabled bearer connection is interruped and resumed again (see clause VI.3.2).

NOTE to call control signalling in examples:

The SIP is used as example call control protocol. There is always a single end-to-end call between the two UEs X and Y. Resumed TLS sessions belong to the same, original call. A single SIP call may be given by a SIP dialog (as idendified by the SIP  Call-ID header value). Whenever multiple "SIP session" instances appear in the following use case examples, then they belong to a single SIP call.

### VI.3.1	Use case: "File transfer session initiated while a chat session is active"

This use case is part of the so called Rich Communication Suite (RCS), see [b-GSMA-RCS.07]. Following quote provides a brief summary:

"3.5.2  Interaction with other RCS features

A 1-to-1 File Transfer is not linked to other services (for example CS-voice call or ongoing chat session) and can be used either during or outside of other communication sessions.

The procedure for any file transfer within an ongoing 1-to-1 chat session is implemented as a separate session in parallel with the ongoing 1-to-1 chat and therefore is the same as the procedure for initiating a separate session for File Transfer."

The use case presented focuses on the interface between UE-X and the MG-1 ("which may serve as an access gateway"). It can be characterized as follows:

- E2ae security is applied between UE-X and MG-1, i.e. the MSRP sessions between the UE-X and the MG-1 are secured by TLS.

- Initially, a single end-to-end MSRP chat session is established between two UEs.

- During the active chat session a file transfer session is established between the same participants.

- File transfer via MSRP is used (see [b-IETF RFC 5547]).

Figure VI.1 shows a simplified network configuration after the chat session between UE-X and UE-Y is established.

Figure VI.1 – MSRP-based instant messageing – Established chat session between two UEs

During the chat session both UEs decide to setup an additional session for a file transfer. Figure VI.2 shows the network configuration after the file transfer session is established.

Figure VI.2 – Parallel MSRP/TLS sessions for chat and file transfer between two UEs

In case the MSRP sessions from the UE to the MG are secured by means of using TLS (such as the e2ae security model) TLS session resumption may be used to setup the TLS session for the file transfer. Figure VI.3 illustrates the correspondent half-call model:

Figure VI.3 – End-to-access edge (e2ae) security model: potential TLS session resumption

To apply TLS session resumption at least the following conditions must be true:

- Both TLS sessions are established between the same TLS client/server pair.

- TLS session resumption is supported by the UE.

- TLS session resumption is supported by the MG (note: the support level concerning root only or non-root in addition is implementation specific).

- TLS client/server role of the resumed session is the same than for the previous session.

The abstracted SIP and H.248 signalling flow and TLS handshake is illustrated in Figure VI.4:

Figure VI.4 – Scenario for TLS session resumption for the file transfer

Figure VI.4 underlines that the TLS security context for the chat session is saved locally at UE-X and/or at the MG-1.

When the new TLS session for the file transfer is setup, the security context from the chat session is retrieved at the UE-X and/or MG-1, thus session resumption can be used for the abbreviated TLS handshake. After the TLS handshake for the file transfer is finished, two TLS sessions between UE-X and MG-1 are active in parallel.

### VI.3.2	Use case: "Resumption of an interrupted file transfer"

In the previous use case TLS session resumption was applied to resume a currently active TLS session, thus resulting into two established TLS sessions in parallel between the same endpoints.

In the use case presented here TLS session resumption is used to resume a session which has beenclosed. The use case can be characterized as follows:

- A 1-to-1 file transfer is established between UE-X and UE-Y

- E2ae security is applied between UE-X and MG-1, i.e. the MSRP sessions between the UE-X and the MG-1 are secured by TLS.

- The file transfer between UE-X and UE-Y fails for any reason, the media session as well as the signalling dialog is terminated.

- The file transfer is resumed by establishing a new signalling dialog and a new media session.

The scenario (see Figure VI.5) differs from the previous use case in the sense that the two TLS sessions are setup sequentially.

Figure VI.5 – Scenario for resuming an interrupted file transfer

## VI.4	Conclusions and guidelines

From MG perspective constraints apply to setup a resumable session and a resumed session.

Constraints for a resumable session:

- MG internal constraints:
The capability for TLS session resumption must be supported in the MG. An implementation that does not support caching of a TLS security context can still support session resumption on the server side if session tickets as specified in [b-IETF RFC 5077] are used. Therefore the capability to support session resumption might be different for server- and client-side implementation and it may depend on the supported method used for session resumption (session Ids vs. session tickets).

- Constraints defined by network service control (via  H.248 interface):
Usage of TLS session resumption may be dependent on security policies, service level agreements, communication service type, etc. The MGC may consequently need to disallow a session to be resumable (either call-individually or in general).

- Constraints enforced by the TLS protocol:
Cases are defined in the base TLS RFCs which explicitly forbid a session to be resumable.

Constraints for resumed sessions:

- MG internal constraints:
These are the same constraints as defined above for resumable sessions.

- Constraints defined by network service control (via  H.248 interface):
As above, usage of TLS session resumption may be dependent on security policies, service level agreements, communication service type, etc. The MGC may consequently need to disable resumption.

- Constraints enforced by the TLS protocol:

  - A resumable session must exist (cached session identifier available, session ticket available, resumable session not expired, certificate not expired, etc)

  - Both TLS endpoints must support the same method for session resumption (session identifer vs. session tickets)

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[b-ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[b-ITU-T V.153]	Recommendation ITU-T (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T Y.2704]	Recommendation ITU-T Y.2704 (01/2010), Security mechanisms and procedures for NGN.

[b-IETF RFC 3749]	IETF RFC 3749 (05/2004), Transport Layer Security Protocol Compression Methods

[b-IETF RFC 4145]	IETF RFC 4145 (09/2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[b-IETF RFC 4346]	IETF RFC 4346 (04/2006), The Transport Layer Security (TLS) Protocol Version 1.1.

[b-IETF RFC 4366]	IETF RFC 4366 (04/2006), Transport Layer Security (TLS) Extensions.

[b-IETF RFC 5077]	IETF RFC 5077 (01/2008), Transport Layer Security (TLS) Session Resumption without Server-Side State.

[b-IETF RFC 5547]	IETF RFC 5547 (05/2009), A Session Description Protocol (SDP) Offer/Answer Mechanism to Enable File Transfer.

[b-IETF RFC 6460]	IETF RFC 6460 (2012), Suite B Profile for Transport Layer Security (TLS).

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-IETF tls-minimal]	IETF draft-ietf-lwig-tls-minimal (2014), A Hitchhiker's Guide to the (Datagram) Transport Layer Security Protocol for Smart Objects and Constrained Node Networks.
URL: http://tools.ietf.org/html/draft-ietf-lwig-tls-minimal-00

[b-ETSI TS 133 203]	ETSI TS 133 203 V11.2.0 (2012-11), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; 3G security; Access security for IP-based services (3GPP TS 33.203 version 11.2.0 Release 11).

[b-ETSI TS 133 310]	ETSI TS 133 310 V11.2.0 (2013-02), Universal Mobile Telecommunications System (UMTS); LTE; Network Domain Security (NDS); Authentication Framework (AF)(3GPP TS 33.310 version 11.2.0 Release 11).

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-OMA-TS-TLS-V1_0-20080902-A]	Open Mobile Alliance OMA-TS-TLS-V1_0-20080902-A (09/2008), OMA TLS Profile.

[b-GSMA-RCS.07]	GSM Association RCC.07 (09/2013), Rich Communication Suite 5.1 Advanced Communications Services and Client Specification, Version 4.0.

[End Proposed Correction]

____________________