| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4501 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DTLS: clauses 5 to 7 - update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes initial input to clauses 5 to 7.

Discussion

The input material is derived from H.248.TLS.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4476).

Change proposal #1:

[Begin Proposed Correction]

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| DTLS Est.req DTLS Est.ack | Abstracted (DTLS message/procedure independent) representationfor establishment requests/acknowledgements of new DTLS security sessions. |
| DTLS Rel.req DTLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing DTLS security sessions. |

## 5.2	DTLS endpoint notations

The notion of endpoint represents different concepts in each network plane (H.248 as control protocol belongs to the network control plane and the MG bearer interface to the network user plane), see Figure 1:

Figure 1 – Conventions for DTLS endpoint types

Usage in

- control plane (here H.248): H.248 terminations/stream endpoint with DTLS processing are denoted as DTLS-enabled termination or stream endpoint (SEP) respectively;

- user plane (DTLS): a DTLS bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., DTLS protocol termination) plus a service access point (i.e., L4+ access).
Further, DTLS is a client/server type of protocol, a "DTLS endpoint" provides thus either a client or server role.

Whether the (H.248) DTLS-enabled SEP provides a "DTLS endpoint" is dependent on the H.248 connection model and enabled MG-internal interworking functions (see clauses 6 and 7).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to DTLS transport modes

The DTLS protocol is independent of the underlying transport protocol stack. Multiple DTLS transport modes are possible (Figure 2).

Figure 2 – DTLS transport modes

This Recommendation is not dependent on any DTLS transport mode, but uses DTLS-over-UDP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-DTLS or different DTLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection without any applied transport security (abbreviated as "non-DTLS to non-DTLS");

- Use case #1.2: an end-to-end L4 connection traversing two network domains with different transport security policies ("DTLS", "no DTLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "DTLS to non-DTLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection with applied transport security (abbreviated as "DTLS to DTLS");

- Use case #1.4: there is end-to-end transport security, but different DTLS profiles (abbreviated as "DTLS to DTLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.2	Use case variations

#### 6.2.2.1	"DTLS-to-DTLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of DTLS transparent forwarding according clause 3.2.1.

## 6.3	Bearer connection network use cases with DTLS transport mode change

The L4 transport protocol could change, e.g., an H.248 MG enabled for DTLS/UDP to DTLS/DCCP interworking.

## 6.4	Bearer connection network use cases with multiparty services

A multipart communciation services leads to connection models with more than two H.248 terminations. Such gateway topologies provide normally application aware type of functions (e.g. as media server), thus DTLS sessions would be terminated by the MG. It results in a type of "DTLS to DTLS*" interworking, such as use case #1.4 (clause 6.2.1).

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 4) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a DTLS-capable IP host remote endpoint. Both DTLS endpoints span a network transport security domain (here DTLS domain).

NOTE  – There might be an additional network interface in case of dedicated, centralized network servers in the DTLS domain with respect to key distribution (such as key management systems). Such kind of interfaces are out of scope of this Recommendation.

Figure 4 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of DTLS level  mode(s) of operation in the H.248 MG; and

- configuration and procedures of the DTLS/L4/IP protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

Figure 5 provides the L4/IP protocol stack with the suite of DTLS protocols and their sub-layer organization.

Figure 5 – L4/IP protocol stack with DTLS protocol

Figure 6 details the generic connection-model where a DTLS-enabled termination is connected to a single other termination (either DTLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 6 – Two-termination context with a DTLS termination

[End Proposed Correction]

____________________