| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4503 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DTLS: Appendix I - Example of T.38-over-UDPTL/DTLS/UDP transport and other use cases |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a set of example use cases for Appendix I.

Discussion

Appendix I should provide a set of example use cases which demonstrate the variety of possible MG involvement in DTLS interworking scenarios. We'd like to add three use case areas:

Appendix I.1:

- generic consideration of DTLS/UDP data and interworking, keeping in mind that TCP is the usual data transport protocol in IP networks;

- there are some basic use cases concerning the combination of L4 transport protocol interworking and whether transport security is applied or not.

Appendix I.2:

- consideration of concrete application T.38;

- that use case area adds the specifics that T.38 again is a transport independent designed protocol which supports multiple T.38 transport modes.

Appendix I.3:

- consideration of concrete application WebRTC data traffic;

- that use case area adds the specifics of a hierarchical (tunneled) protocol stack layering.

It may be further concluded that the "DTLS transparent forwarding" would be a common MG behaviour in case of end-to-end DTLS sessions.

Proposal

Example use cases are added to Appendix I. Furthermore a definition for "DTLS transparent forwarding" (based on H.248.TLS) is added.

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4476).

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	DTLS transparent forwarding: MG packet forwarding behaviour with the characteristic of DTLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a DTLS-PDU flow.

NOTE 1 – A DTLS PDU relates to a DTLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.1, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of DTLS message integrity in the context of "DTLS transparent forwarding". The MG might be DTLS aware; e.g. support of DTLS related statistics or event detection would not violate transparent forwarding behaviour.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

... to be updated …

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix I

Sample use cases of DTLS bearer encryption

(This appendix does not form an integral part of this Recommendation.)

This Appendix illustrates some network level scenarios that employ DTLS bearer encryption.

## I.1	Use cases for "application protocol agnostic DTLS handling"

The MG is unaware of the IP application protocol carried by DTLS packets. There are some basic interworking modes (see Figure I.1), - from perspective of UDP- and TCP.based data transport:

- use case #I.1.1: MG provides interworking between "DTLS/UDP/IP" to "TCP/IP";

- use case #I.1.2: MG provides interworking between "DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.1.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

Figure I.1 – Use cases for "application protocol agnostic DTLS handling"

## I.2	Use cases for "DTLS-based transport security for facsimile packet relay service T.38"

## The IP application protocol is given by T.38 [b-ITU-T T.38], which relates to the Internet Facsimile Protocol (IFP) plus an underlying application level framing protocol such as UDPTL (Facsimile UDP Transport Layer).

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

There are two principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.2.1: MG provides interworking between "DTLS/UDP/IP" to "UDP/IP"; and

- use case #I.2.2: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode ("thus, similar as use case #I.1.3").

It should be noted that the MG is not aware of the IP application protocol, hence also not aware of the "UDPTL" protocol here. Hence, use cases #I.2.2 and #I.1.3 should be identical from H.248 control perspective.

Figure I.2 – Use cases for "DTLS-based transport security 
for facsimile packet relay service T.38"

## I.3	Use cases for "WebRTC data traffic"

Background:

WebRTC relates to a multimedia application inclusive a data component. The WebRTC data is using "SCTP/DTLS/UDP" transport.

The WebRTC data traffic related DTLS use cases provide further characteristics in addition to the use cases in clauses I.1 and I.2:

- hierarchical protocol stack layering by using a secured tunnel (given by SCTP/DTLS);

- multiplexing model because a single DTLS security session could be shared by multiple WebRTC data channels and

- communication between various WebRTC endpoint types leads to UDP-to-UDP and UDP-to-TCP interworking support by H.248 MGs.

There are three principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.3.1: MG provides interworking between "SCTP/DTLS/UDP/IP" to "TCP/IP";

- use case #I.3.2: MG provides interworking between " SCTP/DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.3.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

It should be noted that the MG is not aware again of the application protocol itself. However, the main difference is the embedded "SCTP stream/SCTP association" layering in comparison to clause I.1.

Figure I.3 – Use cases for "WebRTC data traffic"

## I.4	Use cases for "DTLS-based key exchange for SRTP"

TBD

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

[b-ITU-T V.153]	Recommendation ITU-T (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[End Proposed Correction]

____________________