| Rapporteur Meeting of Questions 1,2, 3, 5 and 21/16 |  |  |  | AVD-4519 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPMUX |  |  |  |
| Title: | H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New): input draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor’s input draft for H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New).

2.	Discussion

It is a copy of TD 97R1 (WP1/16) from the Geneva, 28 October – 8 November 2013. There are no changes.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPMUX Ed. 0.1 | Initial baseline output (TD 59 (WP1/16)) from ITU-T SG16 Geneva meeting, Jan 2013. Based on C.34, C.35, C.41and C.53. Location: http://www.itu.int/md/T13-SG16-130114-TD-WP1-0056/en |
| H.248.RTPMUX Ed. 0.2 | Input draft to Oslo, 17 - 21 June 2013 meeting. Based on TD 56 (WP1/16). |
| H.248.RTPMUX Ed. 0.3 | Output (TD-24) from ITU-T SG16 Oslo meeting, Jun 2013. Based on AVD-4415, AVD-4450 and AVD-4459. Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-24.zip |
| H.248.RTPMUX Ed. 0.4 | Input draft to Geneva, 28 October – 8 November 2013 meeting. Based on TD 24 (Oslo, June 2013). |
| H.248.RTPMUX Ed. 0.5 | Output draft to Geneva, 28 October – 8 November 2013 meeting. Changes due to: C.263, C.279, C.287, C.305, C.330, C.343 and C.359. |
| H.248.RTPMUX Ed. 0.6 | Input text to the Geneva, 10 – 14 March 2014 meeting. |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 7 4 Abbreviations and acronyms 8 5 Conventions 9 6 Overview of RTP multiplexing 9 6.1 Introduction 9 6.2 Protocol history: multiplexing framework defined for original RTP 10 6.3 RTP transport multiplexing 10 6.3.1 Limitations of RTP transport multiplexing 10 6.3.2 Control of RTP transport multiplexing 10 6.3.3 Relation to (RTP, non-RTP media) multiplexing in general 10 6.4 RTP media multiplexing 10 6.5 UDP payload multiplexing 10 7 Use cases with RTP multiplexing 11 8 H.248 Native Control of RTP multiplexing 11 8.1 Assigning a multiplexed RTP stream 11 8.2 Modification of the multiplex 12 8.3 Scope of Signals / Events and Statistics 12 8.4 Scope of Topology 13 9 Media Grouping package 14 9.1 Properties 14 9.1.1 Group Semantics 14 9.1.2 Group Representative 15 9.1.3 Group Representative for Topology 15 9.2 Events 16 9.3 Signals 16 9.4 Statistics 16 9.5 Error Codes 16 9.6 Procedures 16 9.6.1 Support of media grouping 16 9.6.2 Mapping of group attributes in H.248 16 9.6.3 MG Behaviour 19 9.6.4 Error handling 22 9.6.5 Usage of Group Representative 22 9.6.6 Usage of Group Representative for Topology 23 10 Usage in RTP Multiplexing 24 Annex A <Annex Title - Placeholder> 31 Appendix I Existing H.248 support for RTP multiplexing 32 I.1 3GPP-defined multiplexing for IP bearers with 3GPP framing protocol 32 Appendix II Living List related to IETF documents under observation 32 II.1 Important references 32 II.2 Important terminology 34

List of Tables

Page

Table 1 – Naming of multiplexing options related to transport and media multiplexing 8 Table 2 – Example command encoding– (SIP) SDP Offer 17 Table 3 – Example command encoding – MGC request 17 Table 4 – Example H.248 command request for multiplexed streams 25 Table 5 – Example reply to a H.248 command request for multiplexed streams 26 Table 6 – Example H.248 command request for multiple multiplexed streams 27 Table 7 – Example reply to a H.248 command request for multiple multiplexed streams 29 Table 8 – Example H.248 command request for an additional multiplexed stream 30 Table 9 – Example reply to a H.248 command request for an additional multiplexed stream 31

List of Figures

Page

Figure 1 – Different multiplexing methods for RTP traffic and their used RTP header fields 9 Figure 2 – Example single and multiplexed RTP streams. 12 Figure 3 – Example of a multiple stream aggragation into a single stream 24 Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”) 36

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPMUX)

Gateway Control Protocol: 
H.248 support for RTP multiplexing

AAP Summary

<Mandatory material to be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

{Editor’s note (2013-06): the package proposal isn’t yet complete, the current Media Grouping package in sub-clause 10 is only giving a general way to indicate the group characteristics of multiple media streams, however the following aspects are identified during the meeting for further investigation .

o RTCP handling needs to be considered and how it relates to bundling.

o Explicit port handling considering (i.e. H.248.57) needs to be considered.

o ReserveGroup do we allow or disallow grouping in grouped streams.

o IP endpoint transport multiplexing. Is it only IP or other transports?

o The general grouping package is contained in RTP-MUX however it may form its own Recommendation depending on progress.

Contributions are solicited.}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 4566]	IETF RFC 4566 (07/2006), SDP: Session Description Protocol

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework

[IETF draft taxonomy]	IETF draft-lennox-raiarea-rtp-grouping-taxonomy (201X), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1		media sink [IETF XYZ]: FIXTHIS

3.1.2		media source [IETF XYZ]: FIXTHIS

3.1.3		media track [IETF XYZ]: FIXTHIS

3.1.4		H.248 media stream [ITU-T H.248.1]: FIXTHIS

3.1.5		RTP end point [IETF XYZ]: FIXTHIS

3.1.6		RTP end system [IETF XYZ]: FIXTHIS

3.1.7 		RTP media format [IETF XYZ]: FIXTHIS

3.1.8		RTP media stream [IETF XYZ]: FIXTHIS

3.1.9		RTP media type [IETF XYZ]: FIXTHIS

3.1.10	RTP session [IETF XYZ]: FIXTHIS

3.1.11	RTP session group [IETF XYZ]: FIXTHIS

3.1.12	RTP source [IETF XYZ]: FIXTHIS

{Editor's note (2013-10 meeting): it should be noted that H.248.57 provides already a definition for "RTP session". However, this term is on the one side an "H.248 specific definition" (due to relation to H.248 flow concept), and on the other side lacking multiplexing considerations. See also contribution C-279.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

{Editor’s note (2013-01 meeting): all below listed terminology proposals are just working assumptions and subject of change, primarily dependent on the work progress in IETF on RTP multiplexing models and RTP terminology.}

3.2.1	MG RTP topology: The configuration of RTP termination(s) (NOTE 1) for an individual RTP session within an RTP node. There are multiple RTP terminations in general besides the single case. The RTP terminations are meshed for the "multiple case" and interconnected via an interworking function (IWF).  There are a number of protocol layer relateds IWFs; at the RTP layer itself and at lower or upper layers. A particular RTP topology is characterized by a specific IWF.

NOTE 1 – H.248 terminology: an RTP-enabled H.248 termination or stream endpoint.

Important MG RTP topologies are designated by a name.

Figure x1 – Illustration of "MG RTP topology"

NOTE 1 – Figure x1 illustrates the key aspects of the "MG RTP topology" concept. The term "RTP leg" is undefined but not decisive due to the relation to "RTP session".

{Editor's note #2 (2013-11):  term "MG RTP topology" is a work item, the explicit definition could be replaced when a definition for "(IETF) RTP topology" would be available, e.g., by

MG RTP topology: a configuration of RTP-enabled (H.248) stream endpoints (or terminations) in an (H.248) context realizing a specific (IETF) RTP topology.}

3.2.2	RTP media multiplexing: a single IP transport (L4) port for multiple RTP sources. There is a single unique SSRC per RTP source (NOTE 1).

NOTE 1 – Thus, also known as "SSRC multiplexing" (see clause 2.6.2/[IETF draft taxonomy]).3.2.a		RTP node:

<Proposal #1>:

Generic term used to designate a physical or logical network element with processing of RTP traffic, such as an IP host, terminal device, user equipment, gateway, media server, conference server, etc.

<Proposal #2>:

An RTP service endpoint or a network element with RTP processing.

3.2.3	RTP topology: expected to be defined by IETF.

3.2.4	RTP / RTCP transport multiplexing (briefly "RTP transport multiplexing"): a single IP transport (L4) port for RTP and RTCP packets

3.2.5	RTP multiplexing levels: protocol layer related multiplexing options transport (see clause 3.2.x2) and media (see clause 3.2.x1) multiplexing could be applied in each combination. Table 1 defines a naming scheme:

Table 1 – Naming of multiplexing options related to transport and media multiplexing

| Multiplexing options | Transport (NOTE 1) | Media (NOTE 2) | Name |
| --- | --- | --- | --- |
| a) | No | No | Fully unmultiplexed |
| b) | No | Yes | Media type multiplexed |
| c) | Yes | No | RTP/RTCP multiplexed |
| d) | Yes | Yes | Fully multiplexed |
| NOTE 1 – Single L4 port for RTP media flow(s) and RTCP control flow(s). NOTE 2 – Single L4 port for multiple RTP sources. There’s a single SSRC per RTP source. |  |  |  |

3.2.6	UDP payload multiplexing: a single IP transport (L4) port for multiple, UDP-based application (upper layer) protocols. The first octet of the UDP payload is used for multiplexing. The application protocols are used in their native syntax, which limits UDP payload multiplexing only on scenarios which could guarantee unambiguity.

NOTE 1 – Example: [IETF RFC 5764] defines UDP payload multiplexing for the three UDP packet based applications of RTP, STUN and DTLS.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| IP | Internet protocol |
| --- | --- |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| RD | Remote descriptor |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

None.

# 6	Overview of RTP multiplexing

## 6.1	Introduction

Multiple, RTP-based traffic flow components may be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture).

A RTP-based traffic flow is given by a packet flow with a common identifier (also known as flow identifier or traffic descriptor). A particular multiplexing method uses specific information elements of such a traffic descriptor, which are mandatory for de-multiplexing again. Figure 1 summarizes the three major multiplexing techniques for RTP traffic, as well as the used header elements.

Figure 1 – Different multiplexing methods for RTP traffic 
and their used RTP header fields

In more detail:

- RTP transport multiplexing (see clause 6.3) uses the 8-bit field of Marker bit (M) and 7-bit Payload Type (PT), which together comprise the 8-bit Packet Type (PT);

- RTP media multiplexing (see clause 6.4) uses the 32-bit SSRC (hence, also known as SSRC multiplexing);

- UDP payload multiplexing (see clause 6.5) is based on the first octet of the UDP payload, which relates to the RTP header field indicated in Figure 1.

## 6.2	Protocol history: multiplexing framework defined for original RTP

Clause 5.2/[IETF RFC 3550] "Multiplexing RTP Sessions" describes the general multiplexing principles for RTP.

## 6.3	RTP transport multiplexing

RTP transport multiplexing represents the case when a RTP media flow and its associated RTCP control flow share a common IP transport address endpoint. In a normal, non-multiplexed scenario, RTP and RTCP use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP transport multiplexing is achieved by the allocation of the same L4 port for RTCP as allocated for RTP.

Port allocation rules for RTCP at H.248 interfaces are subject of [ITU-T H.248.57]. It has to be noted that the 03/2013 edition of [ITU-T H.248.57] only considers non-multiplexed use cases.

RTP transport multiplexing is an extension to [IETF RFC 3550], motivated and defined by [IETF RFC 5761].

### 6.3.1	Limitations of RTP transport multiplexing

Clauses 4 and 5 in [IETF RFC 5761] describe a number of constraints, interactions and conditions to be considered when applying RTP transport multiplexing.

### 6.3.2	Control of RTP transport multiplexing

Control of RTP transport multiplexing:

- Using SDP: via SDP attribute "a=rtcp-mux" (according to clause 8/[IETF RFC 5761]).

- Others???

### 6.3.3	Relation to (RTP, non-RTP media) multiplexing in general

RTP transport multiplexing is orthogonal to other kinds of multiplexing mechanisms (such as application-specific media level multiplexing options).

RTP transport multiplexing is basically (RTP) application agnostic, only driven by NAT traversal aspects.

## 6.4	RTP media multiplexing

FIXTHIS

## 6.5	UDP payload multiplexing

FIXTHIS (…RFC 5764: UDP payload load types "RTP", "STUN" & "DTLS" are multiplexed)

# 7	Use cases with RTP multiplexing

FIXTHIS

# 8	H.248 Native Control of RTP multiplexing

## 8.1	Assigning a multiplexed RTP stream

Typically a single RTP stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. To indicate the use of a multiplexed RTP stream the MGC shall request the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed RTP Stream.

For text based H.248 SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Figure 2 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 2 – Example single and multiplexed RTP streams.

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RFC xxxx].

{Editor’s note: a specific reference to the RTP multiplexing procedures as defined by the IETF AVT and MMUSIC groups is expected to be added.}

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

If a MG does not support RTP multiplexing it should respond with error code “473- Conflicting property values”.

## 8.2	Modification of the multiplex

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed RTP stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own RTP stream.

## 8.3	Scope of Signals / Events and Statistics

Signals, Events and Statistics may be set at either a Termination or Stream level.  Generally when set at a Termination level the scope of this elements applies to all of the H.248 Streams on the Termination. For example: the octets sent statistic would be the sum of octets sent by each H.248 Stream and by corollary each RTP stream.

When these elements are set at a Stream level the scope of the element is given by the StreamID. In the case of the octets sent statistic it would contain only the octets sent by the H.248 Stream given by the StreamID.

When using multiplexed RTP Streams the above behaviour should be considered. Multiplexed RTP Streams may use a sub-set of the total H.248 Streams in a Termination. For example: StreamID(1) could be a single stream, StreamIDs(2&3) could denote a multiplexed RTP Stream. This leads to complications regarding the scope of how Signals, Events and Statistics are applied. For example: A Termination level octets sent Statistic would correctly give the number of octets sent by the Termination however there is no way to request an individual octets sent Statistic related to the multiplexed RTP stream. The MGC would have to request a statistic for StreamID(2) and another statistic for StreamID(3) and add them together.

This issue is also applicable to Signals and Events. This is no way to indicate that a Signal or Event is only applicable to a particular multiplexed RTP Stream. In order to be unambiguous as to the scope of the Signal or Event a specific StreamID in the bundled group should be used.

{Editor’s note:  It needs further analysis whether new information elements are needed to support such behaviour. This may also be applicable for the other grouping semantics, not only to bundle. Further contributions are solicited on the above issues. The media grouping package has been added to the contribution as a possible solution. Further description of the syntax is required. Alternative methods may also be considered.}

The Media Grouping package defined in this Recommendation introduces a mechanism to set the scope of signals, events and statistics at stream group level, i.e. all the streams belonging to the group.

## 8.4	Scope of Topology

For some of the media grouping semantics a MG may be required to interconnect a group of media streams at one interface with a single stream or a group of a different number of streams at another interface. This type of connectivity cannot be described with the existing topology descriptor.

The Media Grouping package defined in this Recommendation introduces the mechanism to establish topology connections between a termination handling a stream group and a termination handling a single stream or a different stream group.

# 9	Media Grouping package

|  | Package Name: | Media Grouping |
| --- | --- | --- |
|  | Package ID: | mgroup (0x????) |
|  | Description: | This package provides a general way for the MGC to instruct the MG to group media streams defined in different stream descriptors within the same media descriptor. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

### 9.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example: providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. The property allows a MGC to assign a group to a single stream directional or both directions via the optional “Direction” element. When not included a default of “bothway” is assumed. {Editor’s note: further contribution is needed regarding the directionality and to further align with the different semantics.} Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics 1*WSP StreamIDList[1*WSP Direction] ; (NOTE 1) WSP = SP/HTAB StreamIDList = StreamID *(WSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) Direction = Bothtoken / Sendtoken / Receivetoken Bothtoken = “B” Sendtoken = “S” Receivetoken = “R” As defined by [ITU-T H.248.1 Annex B]. Note 1: The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888]. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

### 9.1.2     Group Representative

|  | Property Name: | Group Representative |
| --- | --- | --- |
|  | Property ID: | grouprep (0x0002) |
|  | Description: | This property allows the MGC to assign a Stream ID as group representative. The defined stream identity must not be used by any of the termination streams. Using the Stream ID of the Group Representative when requesting signals, events and statistics determines that the scope of the signals, events and statistics is the complete group. |
|  | Type: | Sub-list of integer |
|  | Possible values: | Each value in the list must be a non-allocated stream ID for the corresponding group. Elements in the list are ordered according to the groupse list. |
|  | Default: | None |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

### 9.1.3    Group Representative for Topology

|  | Property Name: | Group Representative for Topology |
| --- | --- | --- |
|  | Property ID: | groupreptop (0x0003) |
|  | Description: | This property allows the MGC to assign a stream ID as group representative for the handling of context topology. Using the stream ID of the Group Representative for Topology when issuing a Topology command defines that the scope of the Topology is the complete media group. The defined stream identity must be the same as used in other terminations if a topology relationship has to be established to those terminations. |
|  | Type: | Sub-list of integer |
|  | Possible values: | Each value in the list is a Stream ID. Elements in the list are ordered according to the groupse list. |
|  | Default: | 1 |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.4	Statistics

None.

## 9.5	Error Codes

None.

## 9.6	Procedures

### 9.6.1	Support of media grouping

#### 9.6.1.1	Via profile

Usage and support of media grouping may be pre-agreed in a profile specification.

#### 9.6.1.2	Via auditing

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the sgroupse and/or rgroupse properties to determine which semantics are supported by the MG. In this case the MG shall respond with the supported semantic tags and the Stream IDs set to 0.

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to groupFor which purpose the media streams are grouped.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

### 9.6.2	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID is used for the index in the group

- The “a=group” attribute is mapped to the mgroup/groupse property and included in the TerminationState Descriptor. The content of the property, a sub-list of string, should be follow a copy of the “a=group” attribute. The semantic is retained and the mid value is replaced with the corresponding StreamID.

NOTE - The identification tags included in the mgroup/groupse property should match the ones sent in the Local or Remote descriptor to the MG.

Example:

An MGC sends an SDP Offer with the following media description (see Table 2).

Table 2 – Example command encoding– (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=Alice 289083124 2890844526 IN IP4 124.124.124.222 c=IN IP4 124.124.124.222 a=group:LS 1 2 m=audio 30002 RTP/AVP 97 a=mid 1 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 m=video 52738 RTP/AVP 99 a=mid 2 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv | It describes a session containing one video stream and one audio stream. They use the “LS” [IETF RF 5888] grouping semantics to indicate that the play-out of both media must be synchronized. |

The MGC maps this description into the following Media Description (Table 3).

Table 3 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Media { TerminationState { mgroup/groupse = ["LS 1 2"]}, Stream = 1 { Remote { c=IN IP4 124.124.124.222 m=audio 30002 RTP/AVP 97 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 } }, Stream = 2 { Remote { c=IN IP4 124.124.124.222 m=video 52738 RTP/AVP 99 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv } } } | NOTE - A similar mapping would apply to the Local Descriptor. The Local Descriptor is not shown but it is also part of the Media Descriptor and will be sent to the MG. |

### 9.6.3	MG Behaviour

The behaviour of the MG with respect to the grouping is largely dependent on the semantic associated with the grouping (e.g. the value of semantic in the mgroup/groupse properties.

The SDP parameter IANA registry (http://www.iana.org/assignments/sdp-parameters/) contains a list of defined SDP group semantics. In general the MG should adopt the behaviour defined by the RFC (or other standard) that defines the semantic taking into consideration H.248 protocol behaviour. The following sub-clauses discuss several semantics.

#### 9.6.3.1 Lip Synchronisation (LS)

The “Lip Synchronisation” semantic is defined in [IETF RFC 5888]. As per clause 7[IETF RFC 5888], the MG must synchronise the play out of H.248 Streams listed together in the same “LS” marked group.

In most scenario where LS is used both the video and audio would be bi-directional. However there are some scenarios such as synchronization of source audio and simultaneous translation. Where lip sync may only be applied in one direction.

#### 9.6.3.2 Flow Identifier (FID)

The “Flow Identifier” semantic is defined in [IETF RFC 5888]. The use of the flow identifier with H.248 is largely redundant due to the H.248 connection model and H.248 usage of SDP. H.248 has the StreamID identifier which uniquely identifies media flows. Each H.248 stream may utilise the Reserve Group/Reserve Value (see 7.1.8.2.3/[ITU-T H.248.1]) construct which enables multiple configurations to be set for a H.248 stream. This allows a single H.248 stream to have multiple sets of characteristics of which one set is active at a particular point in time.

Editor’s note: some of the applications of the FID semantics imply that the different grouped streams are simultaneously carrying data. It may not be possible to emulate this behaviour with Reserve Group

#### 9.6.3.3 Single Reservation flow (SRF)

The “Single Reservation flow” semantic is defined in [IETF RFC 3524]. It indicates that media (described by m=lines) should be mapped to the same resource reservation flow. [ITU-T H.248.65] defines a package for the support of RSVP. The initiation of a resource reservation request is typically explicitly requested through the use of a H.248 signal. Any identification of flow is part of the parameters to the signal. Therefore the use of the SRF semantic may be redundant for H.248.

Editor’s note: the allocation and subsequent policy enforcement of a predefined total bandwidth for a combined number of streams may however require the use of this semantic as opposed to allocate individual bandwidths for each stream.

RSVP is unidirectional in nature. The use of RSVP in a MGW requires [ITU-T H.248.65] which has the inherent mechanisms to determine the applicable direction.

#### 9.6.3.4 Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [IETF RFC 4091]. The ANAT semantic allows a SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

#### 9.6.3.5 Forward Error Correction (FEC)

The “Forward Error Correction” semantic is defined in [IETF RFC 5956]. The use of “FEC” has been deprecated and should not be used with H.248.

#### 9.6.3.6 Forward Error Correction FR (FEC-FR)

The “Forward Error Correction FR” semantic is defined in [IETF RFC 5956]. This is used to associate source and repair flows for the purposes of forward error correction (FEC). If this semantic is received the MG should utilise the source and repairs H.248 Streams listed together in the same “FEC-FR” marked group for the purposes of FEC.

Note: The MGC should indicate whether a flow is a source or repair flow through the use of coding related parameters (i.e. a=rtpmap:111 1d-interleaved-parityfec/90000 and a=fmtp:111 L=10; D=10; repair-window=400000) rather than using the StreamID to indicate this.

Editor’s note: However, if there are multiple source streams and/or multiple repair streams, this semantic may still be required to indicate the association between repair and source streams.

It may be possible to use forward error correction only in one direction. This behaviour can be specified with the use of StreamMode (SendOnly or RecvOnly) or the SDP direction attribute (a=sendonly or a=recvonly) in the repair stream(s).

{Editor’s note: the use of the SDP direction attribute needs further contribution.}

#### 9.6.3.7 Composite Session (CS)

The “Composite Session” semantic is defined in clause 3.2/[IETF RFC ietf-rmt-flue-sdp]. It provides a mechanism to group together File Delivery over Unidirectional Transport (FLUTE) protocol sessions. This semantic may be used with H.248 to associate multiple FLUTE sessions together. However some relying on the behaviour outlined in the fore mentioned clause may lead to ambiguous MG behaviour. As H.248 Streams are distinct SDP is not shared across multiple SDP streams. Therefore it is not possible rely on the inheritance behaviour associated with session-level SDP attributes nor inheritance from the “primary media”. In order to avoid ambiguous behaviour a MGC should fully specify SDP associated with FLUTE sessions.

#### 9.6.3.8 Decoding Dependency (DDP)

The “Decoding Dependency” semantic is defined in [IETF RFC 5583]. This semantic may be used with H.248 as well as other SDP attributes (e.g. a=depend) to describe decoding dependencies for layered codings.

#### 9.6.3.9 Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF draft-ietf-mmusic-sdp-bundle-negotiation] only describes the use of BUNDLE for IP transports it shall not be used with Streams using non-IP transports.

As described in clause 8 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 10.6).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “sent group semantics” and “received group semantics” properties that in conjunction with the BUNDLE attribute allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “sent group semantics” and “received group semantics” properties when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by these properties then the same value should be returned for those H.248 Streams.

BUNDLE is symmetric in nature. If it is used, it applies to source and destination.

NOTE - In practice, however, the required behaviour from the MG specific to this semantic, is related to the Local Descriptor. The use of the same address and port for multiple streams in the Remote Descriptor is usually not problematic in the MG, also when BUNDLE is not used.

See clause 10 for more information the use of the BUNDLE and how to support RTP multiplexing in H.248.

### 9.6.4	Error handling

In the following error cases the MG should return an error code 473 “Conflicting Property Values”:

- If any of the stream identities included in the groupse property does not exist in the termination.

- If the stream identity in the grouprep property exists already in the termination.

- Depending on the grouping semantics if there is a mismatch between the mgroup/groupse property and the Local or Remote Descriptor.

A more specific error code may be returned if available. The underlying reasons for generating an error may depend on the semantic associated with the group.

For example: An error may be generated if the MGC uses the BUNDLE media grouping and specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command). Another example is if an error may be generated if MGC uses the “LSP” semantic and specifies the StreamIDs of H.248 streams where the MG cannot maintain lip sync.

### 9.6.5 	Usage of Group Representative

{Editor’s note: The examples below are to illustrate the usage of the group representative property. Further contributions are expected as analysis is needed if other non-grouping methods may achieve the required functionality.}

An MGC may allocate a Stream ID as group representative for the handling of signals, events and statistics. The stream identity of the group representative must be different to the stream identities of all existing streams in the termination. Once allocated, the MGC may use the stream ID of the representative to define the scope of signals, events and statistics.

Usually the behaviour requested from the MG when the MGC defines stream group level scope for signals, events or statistics  is analogous to the case when the scope is the termination, with the difference that streams which are not part of the group are not considered for the signals, events or statistics. However, the behaviour may be different depending on the media grouping semantics.

#### 9.6.5.1 Usage of the Group Representative when requesting signals

Example 1:

A termination T1 has two audio streams with identities 1 and 2 which use the same codec. They are grouped with the FID (Flow Identification) semantics [IETD RFC 5888]. Thus they carry identical content, one is a replica of the other, usually sent to a different destination address. The termination has in addition other media stream with identity 3, not part of the media group. The MGC has defined a group representative: 8. Requesting an an/apv [H.248.7] signal on stream 8 to play an audio announcement should result in the announcement being sent synchronously to streams 1 and 2.

Example 2:

A termination T1 has two audio streams with identities 1 and 2 which use different codecs. They are grouped with the FID semantics [IETD RFC 5888].  At a time only one of the streams is carrying data in each direction while the other one is muted in that direction. This is dependent on the codec choice by the end user equipment which can change dynamically. The MGC has defined a group representative: 8. Requesting an an/apv [H.248.7] signal on stream 8 to play an audio announcement should result in the announcement being sent in the stream which is carrying data at the moment.

#### 9.6.5.2 Usage of the Group Representative when requesting events

Example 1:

A termination T1 has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus resource reservation using RSVP and policy enforcing is applied to the complete group. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined a group representative: 8. Requesting the rsvp/pathr [H.248.65] event on stream 8 is equivalent to requesting it in streams 1, 2 and 3.

Example 2:

A termination T1 has two streams with identities 1 and 2 carrying instant messaging data.  They are grouped with the ANAT (Alternative Network Address Type) semantics [IETF RFC 4091]. Thus, message data can be received at any time in one or the other stream. The MGC has defined a group representative: 8. The MGC requests the msrpstat/mquota [H.248.69] event on stream 8. The event occurs when the aggregated volume of data in streams 1 and 2 reaches the quota indicated in the corresponding event parameter, e.g. total number of received messages.

#### 9.6.5.3 Usage of the Group Representative when requesting statistics

Example 1:

A termination T1 handles three audio streams with identities 1, 2 and 3 grouped with the BUNDLE semantics. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined a group representative: 8. The MGC wishes to collect statistics on the data volumes sent and received in the bundled group at different levels (network, transport, application). It uses the Network Package os and or [H.248.1], the IP Layer Octets Counts Statistics Package ipos, ipor, ipps and ippr [H.248.61] and the RTP Application Data Package payloados and payloador [H.248.58] statistics. It requests these statistics by including them in the statisticsDescriptor of the streamDescriptor of stream 8. The MG should collect statistics in each of the three streams part of the group are sum up the values.

The statistics descriptor is the only descriptor that the MGC may send in the stream descriptor of a group representative, i.e. the local, remote and localControl should never be included.

### 9.6.6 	Usage of Group Representative for Topology

{Editor’s note: The examples below are to illustrate the usage of the group representative for Topology property. Further contributions are expected as analysis is needed if other non-grouping methods may achieve the required functionality.}

The Group Representative for Topology property allows the MG to transfer the media from/to several grouped media streams into/from one single stream or a different set of grouped media streams

If the MGC defines a group representative for topology for a group of streams, they can be handled as one single stream in a topology command and thus be connected to another termination with also one single stream –which may be a stream representative for topology as well-.

The requested behaviour from the MG depends on the grouping semantics but it will usually involve some dynamic switching of the connectivity between the terminations included in the topology command based on external conditions. This dynamic switching will not be controlled by the MGC.

Example 1:

Figure 3 illustrates a scenario where a context connects two terminations. One of them has three audio streams: stream with identity 1 (sendrecv) uses PCM A-law, stream with identity 2 (sendrecv) uses AMR and stream with identity 3 (sendonly) uses PCM A-law. The three streams are grouped with the FID [IETF RFC 5888] semantics. The other termination has one single stream with two payload types, PCM A-law and AMR.

Figure 3 – Example of a multiple stream aggragation into a single stream

NOTE - the direction attributes shown in the figure correspond to the MG terminations’ views.

P1 can change the codec dynamically to send data. According to the FID semantics, when PCM encoded audio is received externally in T1, T2 forwards it to P2 in stream 1 and as a replica in stream 3 –it may be with a different IP destination address if specified like that in the Remote Descriptor- and stream 2 remains muted. When AMR encoded audio is received in T1, T2 forwards it to P2 in stream 2 while stream 1 and stream 2 remain muted.

When PCM encoded audio is received externally in T2 in stream 1 it is forwarded to T1. When  AMR encoded audio is received externally in T2 stream 2 it is forwarded to T1.

The MGC defines stream identity 1 as group representative for topology in termination T2 and issues a topology command for stream 1 with a bothway connection between T1 and T2 to achieve the desired behaviour.

# 10	Usage in RTP Multiplexing

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams (See clause 8).

Additionally the MGC may set the “association” (mgroup/groupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

For example:

An MGC may require a Termination (T1) with five Streams. StreamID (1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. These streams are bi-directional.

In the first example the mgroup/groupse property indicates that the video streams are associated:

Table 4 – Example H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30001 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

The MG reply is:

Table 5 – Example reply to a H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=A4445{ Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50001 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50002 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30001 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

In the second example the mgroup/groupse property indicates that the audio streams are associated together and the video streams are associated together.

Table 6 – Example H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 1 2”, “BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

The MG reply is:

Table 7 – Example reply to a H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=A4445{ Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50001 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50001 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

When adding or removing multiplexed H.248 Streams the MGC shall include an updated mgroup/groupse property in the same command to ensure consistency between Local and Remote Descriptor settings and the property.  On reception of the properties the MG shall assign resources in the knowledge that each of the StreamList element contains StreamIDs that are associated with one multiplexed stream.

In the following example a new H.248 Stream (StreamID 6) is added to an existing multiplex of StreamIDs 3, 4 & 5, such as the one created in the previous example.

Table 8 – Example H.248 command request for an additional multiplexed stream

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Modify = A4445 { Media { TermationState { mgroup/groupse = [“BUNDLE 3 4 5 6”]}, Stream = 6 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 35 } } } } } } |  |

In the above example MG receives a request to add a H.248 stream with a wildcarded address/port returns the same address as the other H.248 Streams associated with the multiplex.

Table 9 – Example reply to a H.248 command request for an additional multiplexed stream

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10005 { Context = 1 { Modify = A4445 { Media { Stream = 6 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50005 RTP/AVP 35 } } } } } } |  |

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

Existing H.248 support for RTP multiplexing

(This appendix does not form an integral part of this Recommendation)

## I.1	3GPP-defined multiplexing for IP bearers with 3GPP framing protocol

Reference: see clause 6.4/[ETSI TS 129.414].

Motivation: transport capacity reductions (“bandwidth efficiency”).

Multiplexing method: multiple RTP packets mapped into a single PDU augmented by a (3GPP defined) multiplex header (“similar to IETF SHIM approach").

H.248 control: not any signaling support, thus pure provisioning based approach.

Appendix II

Living List related to IETF documents under observation

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix would be again deleted as soon as the reference (I.1) and terminology (I.2) discussion becomes stable).

## II.1	Important references

Following IETF documents (from four IETF working groups) are relevant to the RTP multiplexing discussion:

[1]	A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources
IETF “RAI Areaa” / WG AVTEXT? (still open)
Draft: draft-lennox-raiarea-rtp-grouping-taxonomy
URL: http://tools.ietf.org/html/draft-lennox-raiarea-rtp-grouping-taxonomy-00

[2]	Guidelines for using the Multiplexing Features of RTP
IETF AVTCORE
Draft: draft-westerlund-avtcore-multiplex-architecture 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multiplex-architecture-02.txt

[3]	Multiplexing RTP Data and Control Packets on a Single Port
IETF MMUSIC
RFC 5761

[4]	“BUNDLE”
= Multiplexing Negotiation Using SDP Port Numbers 
IETF MMUSIC
Draft: draft-ietf-mmusic-sdp-bundle-negotiation
URL: http://tools.ietf.org/wg/mmusic/draft-ietf-mmusic-sdp-bundle-negotiation/

[5]	“RTP for WebRTC”
 = Web Real-Time Communication (WebRTC): Media Transport and Use of RTP
IETF RTCWEB
Draft: draft-ietf-rtcweb-rtp-usage
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

[6]	“Multi-Media Concepts and Relations” 
IETF RTCWEB
Draft: draft-burman-rtcweb-mmusic-media-structure
URL: http://tools.ietf.org/id/draft-burman-rtcweb-mmusic-media-structure-00.txt

[7]	“RTP for Telepresence”
 = RTP Usage for Telepresence Sessions
IETF CLUE
Draft: draft-lennox-clue-rtp-usage
URL: http://tools.ietf.org/id/draft-lennox-clue-rtp-usage-04.txt /

Complementary:

[8]	Source-Specific Media Attributes
IETF MMUSIC
RFC 5576

9]	SDP Grouping Framework
IETF MMUSIC
RFC 5888

[10]	Media multiplexing with RTP subsessions
IETF AVTCORE
Draft: draft-ejzak-avtcore-rtp-subsessions
URL: http://tools.ietf.org/id/draft-ejzak-avtcore-rtp-subsessions-01.txt

[11]	Multiple RTP Sessions on a Single Lower-Layer Transport
IETF AVTCORE
Draft: draft-westerlund-avtcore-transport-multiplexing 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-transport-multiplexing-03.txt

[12]	Multiple Media Types in an RTP Session
IETF AVTCORE
Draft: draft-westerlund-avtcore-multi-media-rtp-session
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multi-media-rtp-session-00.txt

[13]	RTP Considerations for Endpoints Sending Multiple Media Streams
IETF AVTCORE
Draft: draft-lennox-avtcore-rtp-multi-stream
URL: http://tools.ietf.org/id/draft-lennox-avtcore-rtp-multi-stream-00.txt

[14]	Using Simulcast in RTP sessions
IETF AVTCORE
Draft: draft-westerlund-avtcore-rtp-simulcast 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-rtp-simulcast-01.txt

[15]	Cross Session Stream Identification in the SDP
IETF RTCWEB
Draft: draft-alvestrand-rtcweb-msid
URL: http://tools.ietf.org/id/draft-alvestrand-rtcweb-msid-02.txt

[16]	H.248.1v4 Living List: New Appendix V – H.248 Streams “Examples for valid media format suites”
ITU-T SG16
Draft: AVD-4241
URL: see http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/1209_Bri.html

## II.2	Important terminology

Present work on multiplexed RTP did lead to a revision, modification, extension till the semantical overloading of some RTP related terms. Primarily the terms “media stream” and “RTP session” are used in many slightly different flavours. Following definitions from [1] might be quite useful (“despite the fact that they are not yet fully unambiguous”):

All that terms are either new or extensions/revisions of RFC 1889/RFC 3550 terms:

Endpoint:  A single entity sending or receiving RTP packets.  It may

be decomposed into several functional blocks, but as long as it

behaves a single RTP stack entity it is classified as a single

endpoint.

Comment: an endpoint relates to RTP topology “RTP endsystem” in our understanding (see RFC 5117, H.248.RTPTOPO).

Media Source:  The source of a stream of data of one Media Type, It

can either be a single media capturing device such as a video

camera, a microphone, or a specific output of a media production

function, such as an audio mixer, or some video editing function.

Sending data from a Media Source may cause multiple RTP sources to

send multiple Media Streams.

Media Stream:  A sequence of RTP packets using a single SSRC that

together carries part or all of the content of a specific Media

Type from a specific sender source within a given RTP session.

Comments:

- we need to distinguish between “IETF media stream” and “H.248 media stream”

- 1:1 relationship between “media stream” and “SSRC” due to “single SSRC”, right?

- May use different media streams the same SSRC value? The definition allows both interpretations …

RTP Source:  The originator or source of a particular Media Stream.

Identified using an SSRC in a particular RTP session.  An RTP

source is the source of a single media stream, and is associated

with a single endpoint and a single Media Source.  An RTP Source

is just called a Source in RFC 3550.

Comments:

- 1:1 relationship between “RTP source” and “SSRC”, right?

- May use different RTP sources the same SSRC value? The definition allows both interpretations …

Media Sink:  A recipient of a Media Stream.  The endpoint sinking

media are Identified using one or more SSRCs.  There may be more

than one Media Sink for one RTP source.

Multiplex:  The operation of taking multiple entities as input,

aggregating them onto some common resource while keeping the

individual entities addressable such that they can later be fully

and unambiguously separated (de-multiplexed) again.

RTP Session:  As defined by [RFC3550], the endpoints belonging to the

same RTP Session are those that share a single SSRC space.  That

is, those endpoints can see an SSRC identifier transmitted by any

one of the other endpoints.  An endpoint can receive an SSRC

either as SSRC or as CSRC in RTP and RTCP packets.  Thus, the RTP

Session scope is decided by the endpoints' network interconnection

topology, in combination with RTP and RTCP forwarding strategies

deployed by endpoints and any interconnecting middle nodes.

Comment: this is an extension of the RFC 3550 RTP session concept in our understanding. The RTP session is NOT any global concept across all involved RTP entities (such as endsystems, mixers, translators) of a particular communication application.
The RTP session is rather a local concept, given by the perspective of a particular endpoint.

Examples: a two-party call (= point to point topology) between A and B, and with different multiplexing models of each endpoint may lead to the effect that e.g. A perceives a single RTP session whereas B observes multiple RTP sessions for the same communication service.

Note: we may agree to such an “RTP session” concept (given that our interpretation is inline with [1]).

RTP Session Group:  One or more RTP sessions that are used together

to perform some function.  Examples are multiple RTP sessions used

to carry different layers of a layered encoding.  In an RTP

Session Group, CNAMEs are assumed to be valid across all RTP

sessions, and designate synchronisation contexts that can cross

RTP sessions.

Source:  Term that should not be used alone.  An RTP Source, as

identified by its SSRC, is the source of a single Media Stream; a

Media Source can be the source of mutiple Media Streams.

SSRC:  An RTP 32-bit unsigned integer used as identifier for a RTP

Source.

(tbd: The terms "SSRC multiplexing" and "session multiplexing" are

confusing, with unclear historical meanings; they need to be removed

from this document in the interests of clarity)

It might be beneficial to illustrate the multiplexing hierarchy of the defined terminology by [1] (Figure I.1 illustrates the traffic source side only):

Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”)

NOTE: The terminology in [1] should be further improved. There are already a couple of different interpretations.

Importation clarifications required (by IETF):

- Exact relationship between entities (e.g., 1:1, 1:N, etc.)

- Exact identifier of an entity ("are there any identifiers, and if, which one, given by an n-tuple of …")

It could be concluded that the evaluation of RTP multiplexing model demands for refined RTP terminology as originally introduced by RFC 1889, but the terminology update work by IETF is still not consistent and stable.

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

_______________________

Reason: [ITU-T H.248.57] was developed in 2008, on the contrary, the option of RTP transport multiplexing was introduced in 2010 (by [IETF RFC 5761]).