| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4523a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Amendment to H.460.22 Negotiation of security protocols for H.225.0 call signalling |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This recommendation is difficult to read and vague in parts and may be subject to differing interpretations. The purpose of this contribution is to perform editorial clean ups and remove any ambiguity in interpretation. Also included are updates to document gatekeeper routed signaling, NAT traversal and to add DTLS signaling support.

Introduction

The current H.460.22 document is difficult to read and vague on when to use or not use parameters and causes some ambiguity in interpretation. The logic hints at the endpoint deciding the secure protocol to use but is not clear and could not be implemented as written. It also does not document gatekeeper routed signalling or use causes where NAT Traversal is required. The use of DTLS is not supported.

Discussion

When implementing this recommendation it was very difficult to read and there were a number of ambiguities which made implementation very difficult. There was a number of call scenarios that where not captured in the document. The recommendation requires revising.

Conclusion

Attached is a proposed revision of H.460.22.

The base document has been updated to include changes agreed to in H.460.22 cor1 and form the basis for change marks.

The following changes are proposed:

- Make the reasons to use signaling channel security much clearer especially with respect to H.235.6 and H.235.8 which is a prime reason to implement this recommendation.

- Add Support for signaling DTLS support.

- Advertising in the GRQ/GCF only needs to advertise the support for this feature, It does not require advertising all the parameters as some are not yet known such as connectionAddress.

- The RCF only needs to advertise the gatekeepers support, all other fields omitted, as the actual security protocols are negotiated when establishing a call as it is unclear whether the gatekeeper will route the H.225.0 channel or if it will go direct, that decision can only be made at the ARQ/ACF stage.

- The priority and connectionAddress are defined by the endpoint and advertised to the Gatekeeper in a full RRQ, it is not required to signal them in any other messages. The data is stored by the gatekeeper and used when establishing a call with a single gatekeeper or exchanged in the LRQ in two gatekeeper scenario. The gatekeeper is only required to supply a connectionAddress to the endpoint in the ACF to indicate where to establish the secure connection.

- The ACF shall advertise only one (or none) security protocol to be used for the call. It is the gatekeeper that makes the decision on which security protocol to use for the call.

- Improve instructions on using gatekeeper routed call signaling mode.

- Include section on NAT Traversal and interaction with H.460.18 and incompatibility with H.460.17.

Proposed Revised ITU-T Recommendation H.460.22

Negotiation of security protocols to protect H.225.0 call signalling messages

# Scope

This Recommendation specifies the security negotiation mechanism for H.225.0 call signalling message exchanges. The main goals include:

1)	Secure selection of the security mechanism. Otherwise, the procedure of negotiation is vulnerable to certain attacks such as malicious manipulation or bidding-down attacks. The entire RAS message shall be protected during the negotiation procedure.

2)	Involved H.323 entities shall determine mutually agreed security protocols without requiring additional round trips.

3)	Entities involved in the negotiation procedure shall be aware of the result of the negotiation, such as success or failure.

4)	The negotiation procedure should not cause any additional burden to the involved entities.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.225.0]	ITU-T Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235.0]	ITU-T Recommendation H.235.0 (2014), H.323 security: Framework for security in H-series (H.323 and other H.245-based) multimedia systems.

[ITU-T H.235.6]	ITU-T Recommendation H.235.6 (2014), H.323 security: Encryption profile with native H.235/H.245 key management.

[ITU-T H.235.8]	ITU-T Recommendation H.235.8 (2005), H.323 security: Key exchange for SRTP using secure signalling channels.

[ITU-T H.323]		ITU-T Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.460.1]	ITU-T Recommendation H.460.1 (2002), Guidelines for the use of the generic extensible framework.

[ITU-T H.460.17]    ITU-T Recommendation H.460.17 (2005), Using H.225.0 call signalling connection as transport for H.323 RAS messages

[ITU-T H.460.18]    ITU-T Recommendation H.460.18 (2013), Traversal of ITU-T H.323 signalling across network address translators and firewalls

[IETF RFC 4302]	IETF RFC 4302 (2005), IP Authentication Header.

[IETF RFC 4303]	IETF RFC 4303 (2005), IP Encapsulating Security Payload (ESP).

[IETF RFC 4346]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.1.

[IETF RFC 5246]	IETF RFC 5246 (2008), The Transport Layer Security (TLS) Protocol Version 1.2.

# Definitions

This Recommendation does not define any terms.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ACF 	Admission Confirmation

ARJ	Admission Reject

ARQ	Admission Request

ASN.1	Abstract Syntax Notation One

DTLS        Datagram Transport Layer Security

FW           Firewall

GCF	Gatekeeper Confirmation

GEF	Generic Extensible Framework

GRQ	Gatekeeper Request

IPSec	Internet Protocol Security

LCF	Location Confirmation

LRJ	Location Reject

LRQ	Location Request

MCU	Multipoint Control Unit

MiTM      Man in the Middle

NAT         Network Address Translator

RAS	Registration, Admission and Status

RCF	Registration Confirmation

RRJ	Registration Reject

RRQ	Registration Request

RTP	Real-time Transport Protocol

SCI           Service Control Indication

TCP	Transmission Control Protocol

TLS	Transport Layer Security

# Conventions

In this Recommendation, the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

# Negotiation description

The protection of [ITU-T H.225.0] call signalling messages is important for the security of [ITU-T H.323] systems. In small networks, network administrators can ensure that all H.323 entities use the same security protocol. However, in large networks, such as where endpoints are distributed in different network domains, a calling endpoint may not know in advance the security protocol supported by the called endpoint. Therefore, it is necessary for the two endpoints to negotiate the security mechanism for H.225.0 call signalling messages before initiating a call establishment procedure.

The negotiation of actual security parameters/algorithms is outside the scope of this Recommendation..

## Call establishment security

There are a number of important reasons to secure the call signalling channel (i.e., H.225.0). When using [ITU-T H.235.6] or [ITU-T H.235.8] to encrypt the media packets, the keys are exchanged in the signalling channel. These keys are sent either in the clear and require a secure signalling channel or are susceptible to MiTM attacks and should be protected by a secure call signalling channel. It also provides a very simple way to authenticate the endpoints to each other in direct call signalling mode or for each hop of the call in gatekeeper routed mode.

## Negotiation of security mechanism

The generic extensible framework (GEF) feature negotiation mechanism is used during the RAS communication. Whether or not this security negotiation mechanism is supported is negotiated between the endpoint and the gatekeeper. If neither the gatekeeper nor the endpoint supports this mechanism and no other security mechanism is used, the normal H.323 procedure will be followed.

The security negotiation procedure shall consist of the following steps:

- The endpoint, where enabled, discovers the gatekeeper. The endpoint shall advertise the                 securityProtocolNegotiation feature as a supportedFeature in the featureSet field of the GRQ. All parameters of the securityProtocolNegotiation feature shall be omitted. Where the gatekeeper supports this feature, it shall advertise as a supportedFeature with all parameters omitted in the reply GRQ. The absence of securityProtocolNegotiation feature indicates the gatekeeper does not support this feature.

- Where the endpoint has discovered support for this feature or is registering without discovery, the endpoint shall include the securityProtocolNegotiation feature as a supportedFeature in the featureSet field in the RRQ message. The parameters associated with the feature shall indicate all the endpoint’s supported security protocols. Every protocol is assigned a preference value where a smaller number signifies a higher preference. The gatekeeper stores this information for call establishment purposes. Lightweight RRQ shall only include the securityProtocolNegotiation feature advertisement and omit all parameters.

- The gatekeeper returns an RCF or RRJ. The gatekeeper shall indicate whether the negotiation mechanism is supported or not in the RCF. If this feature is supported, the gatekeeper shall include the securityProtocolNegotiation feature as a supportedFeature in the featureSet field. The absence of securityProtocolNegotiation shall indicate that the gatekeeper does not support this feature. There are no parameters present in the securityProtocolNegotiation feature in the RCF message.

- The endpoint initiates a call. The endpoint shall send an ARQ to the gatekeeper including securityProtocolNegotiation feature as a supportedFeature in the featureSet field of the ARQ. All parameters shall be omitted.

- The next step is determined based on whether the call is routed by the gatekeeper or not:

a)	If direct call model is used, the gatekeeper shall send an ARQ containing the highest priority matching  security protocol supported by both endpoints and shall include the stored connnectionAddress of the called endpoint where applicable . The priority field shall be omitted.

b)	If the gatekeeper-routed call model is used, the gatekeeper shall send an ARQ containing the highest priority matching security protocols that the calling endpoint and gatekeeper support and shall include the gatekeeper’s connectionAddress where applicable. .The priority field shall be omitted. The gatekeeper may send an ARJ message if there is no matching security protocol.

6)	If the calling endpoint receives an ACF, it shall check the returned supported security protocol in the ACF and use the security protocol to set up H.225.0 call signalling channel.

Figure 6-1 shows a general call flow scenario. In this scenario, it is assumed that there are at least two endpoints which belong to the same gatekeeper. These endpoints (e.g., endpoint A and endpoint B in Figure 6-1) may be H.323 terminals, MCUs, gateways etc., and are served by a single gatekeeper (named gatekeeper A). It is further assumed that H.323 endpoints communicate directly end-to-end for call establishment.

Figure 6-1 – Both endpoints are registered with the same gatekeeper – Direct call signalling

:

1)	Endpoints A and B send a GRQ message that includes, in the supportedFeatures in the featureSet field of the GRQ, the securityProtocolNegotiation feature.

2)	The gatekeeper returns a GCF message that includes, in the supportedFeatures in the featureSet field of the GCF, the securityProtocolNegotiation feature. The feature negotiation procedure is optional at the GRQ/GCF stage.

3)	Endpoints A and B send an RRQ that includes, in the supportedFeatures in the featureSet field, the securityProtocolNegotiation feature. This feature should include all of the security protocols including priority and connectionAddress (where applicable) they support.

4)	The gatekeeper returns an RCF message that includes, in the supportedFeatures of the featureSet field of the RCF, the securityProtocolNegotiation feature. The gatekeeper shall keep the endpoint's supported security protocols for future use.

5)	Before endpoint A initiates a call to endpoint B, endpoint A sends an ARQ message including the securityProtocolNegotiation feature to gatekeeper A.

6)	Depending on the security policy and whether the call signalling is gatekeeper routed, gatekeeper A in direct mode, may return an ACF message including endpoint B's securityProtocolNegotiation feature with the highest priority matching security protocol between endpoint A and endpoint B. , In routed mode, it may return the highest matching security protocol between endpoint A and the gatekeeper or due to security policy may return an ARJ to reject the call.

7)	In the case where there is at least one common supported security protocol, in direct mode, endpoint A establishes a secure call signalling channel to endpoint B. In routed mode, endpoint A established a secure call signalling channel with the gatekeeper and the gatekeeper in turn establishes a secure call signalling channel to endpoint B.

8)	In the case where there is no common supported security protocol between endpoint A and the gatekeeper and/or endpoint B and where gatekeeper security policy permits, endpoint A may proceed to set up a normal call.

A scenario with multiple gatekeepers is shown in Figure 6-2. It is assumed that there are at least two H.323 endpoints attached to different gatekeepers (e.g., endpoint A, endpoint B). It is further assumed that H.323 endpoints communicate directly end-to-end for call establishment.

Figure 6-2 – Both endpoints are registered with different gatekeepers – Direct call signalling

The negotiation procedure described for a single gatekeeper scenario can be extended to cover multiple, chained gatekeepers. Discovery of the far-end endpoint should be accomplished according to clause 8.1.6 of [ITU-T H.323], "Optional called endpoint signalling", using LRQ/LCF/LRJ procedure.

The corresponding message flows are shown in Figure 6-3 and are described as follows.

1-5)	The same as Steps 1-5 of the procedure for Figure 6-1.

6)	After receiving the ARQ from endpoint A, gatekeeper A may send an LRQ message conveying all the security protocols supported by endpoint A to called gatekeeper B via the intermediate Gatekeepers.

7)	Gatekeeper B receives the LRQ.

8)	Gatekeeper B returns an LCF including the highest priority matching security protocol between endpoint A and endpoint B and/or gatekeeper B depending on the call signalling routing mode. The appropriate connectionAddress parameter (where applicable) shall be included and the priority parameter shall be omitted.

9)	Gatekeeper A receives the LCF.

10)	Gatekeeper A returns an ACF including the security protocol information received in the LCF..

11)	Endpoint A establishes a secure call signalling channel to endpoint B with the common supported security protocol. If there is no common supported security protocol between A and B, endpoint A may proceed to set up a normal call.

Figure 6-3 – Example negotiation signalling flow

# Feature description securityProtocolNegotiation

H.225.0 RAS featureSet fields are used in this Recommendation.

The following securityProtocolNegotiation feature shall be presented as a supportedFeature in the featureSet field as shown in Table 7-1.

Table 7-1 – securityProtocolNegotiation feature

| Feature name: | securityProtocolNegotiation |
| --- | --- |
| Feature description: | Shall be indicated by an entity which supports the procedure defined by this Recommendation; may be present in GRQ/GCF, RRQ/RCF, ARQ/ACF, LRQ/LCF messages. |
| Feature identifier type: | Standard |
| Feature identifier value: | 22 |

Parameters associated with the securityProtocolNegotiation field are specified in the following clauses. In consideration of backward compatibility with further revisions to this Recommendation, the recipient shall simply ignore any parameters received other than those specified in this Recommendation.

## tlsSecurityProtocol

The description of the tlsSecurityProtocol parameter is shown in Table 7-2.

Table 7-2 – tlsSecurityProtocol

| Parameter name: | tlsSecurityProtocol |
| --- | --- |
| Parameter description: | It indicates that the TLS security protocol is supported by the entity. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | compound |
| Parameter cardinality: | Zero or one |

It contains the two parameters shown in Tables 7-3 and 7-4 respectively.

Table 7-3 – priority

| Parameter name: | priority |
| --- | --- |
| Parameter description: | It indicates the priority of the TLS security protocol. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | number8 |
| Parameter cardinality: | One |

Table 7-4 – connectionAddress

| Parameter name: | connectionAddress |
| --- | --- |
| Parameter description: | It indicates the transport address used by the TLS security protocol. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | transport |
| Parameter cardinality: | One |

## ipsecSecurityProtocol

The description of the ipsecSecurityProtocol parameter is shown in Table 7-5.

Table 7-5 – ipsecSecurityProtocol

| Parameter name: | ipsecSecurityProtocol |
| --- | --- |
| Parameter description: | It indicates that the IPsec security protocol is supported by the entity. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | compound |
| Parameter cardinality: | Zero or one |

It contains the parameter shown in Table 7-6.

Table 7-6 – priority

| Parameter name: | priority |
| --- | --- |
| Parameter description: | It indicates the priority of the IPsec security protocol. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | number8 |
| Parameter cardinality: | One |

## dtlsSecurityProtocol

The description of the dtlsSecurityProtocol parameter is shown in Table 7-7.

Table 7-7 – dtlsSecurityProtocol

| Parameter name: | dtlsSecurityProtocol |
| --- | --- |
| Parameter description: | It indicates that the DTLS security protocol is supported by the entity. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 3 |
| Parameter type: | compound |
| Parameter cardinality: | Zero or one |

It contains the two parameter shown in Table 7-8 and 7-9 respectively.

Table 7-8 – priority

| Parameter name: | priority |
| --- | --- |
| Parameter description: | It indicates the priority of the DTLS security protocol. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | number8 |
| Parameter cardinality: | One |

Table 7-9 – connectionAddress

| Parameter name: | connectionAddress |
| --- | --- |
| Parameter description: | It indicates the transport address used by the DTLS security protocol. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | transport |
| Parameter cardinality: | One |

# NAT Traversal considerations

When using this feature with [ITU-T H.460.18] consideration needs to be given to inbound calling to an endpoint behind a NAT/FW. In this scenario direct call signalling connection from outside of the NAT/FW to inside is impossible and a RAS pinhole needs to be maintained to allow the endpoint behind the NAT/FW to be notified of an inbound call. The call signalling connection needs to be establish from behind the NAT/FW to the gatekeeper. All call signalling is required to be gatekeeper routed.

[ITU-T H.460.18] clause 10 describes the method by which an endpoint behind a NAT/FW is notified of an inbound call by sending an RAS SCI message with a genericData field containing the IncomingCallIndication parameter. This notifies the endpoint of the call signalling address to establish an outbound TCP connection.

When using this recommendation in conjunction with [ITU-T H.460.18] the RAS SCI message specified in [ITU-T H.460.18]  clause 10 shall also contain this feature in the genericData field. It shall also contain the security protocol and the connectionAddress (where applicable) mof the gatekeeper to establish the secure connection. The priority field shall be omitted.

This recommendation is incompatible with [ITU-T H.460.17] and shall be omitted if the recommendation is in use.

_____________