| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4526 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Amendment to H.323 Annex O to support secure service discovery |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the amendment of H.323 Annex O to support SRV DNS secure service discovery.

# Introduction

Recommendations such as H.460.17 rely on the discovery of H.323 entities using SRV DNS records. H.323 defines the use of secure transports however H.323 Annex O does not define any secure transport SRV records to be used with these recommendations.

# Discussion

# H.460.17 it very useful when trying to register from behind a difficult NAT where most outbound ports are blocked and only a small number of TCP ports are available. To complicate things further some NAT devices also conduct deep packet inspection and will block unrecognised packets. The solution is to secure the signalling channel and use a widely recognised port such as TCP 443.

# H.323 defines the use of TLS to secure call signalling connections however the discovery of such services via SRV DNS is not defined. H.460.17 relies on the discovery of the gatekeeper using SRV DNS records so a service needs to be defined. .

# Conclusion

This contribution proposes to add a “-sec” suffix to the service portion of the SRV DNS records defined for h323ls, h323rs, h323cs to indicate the service is secure. Where the transport is “tcp” this shall indicate TLS and where the transport is “udp” shall indicate DTLS.

Proposed addition of secure transports to H.323 Annex O.

[Begin]

## O.9	Resolving an H.323 URL to IP Address using DNS

The host part of the H.323 URL can specify one of the following:

–	The IP numeric address of an H.323 entity.

–	The DNS name of a host which is an H.323 entity.

–	The remote DNS domain containing H.323 SRV RRs.

This clause defines the address resolution procedure covering these three cases.

When the host contains an IP numeric address, nothing needs to be resolved using DNS. The H.323 messages shall be sent directly to the specified IP address.

When the hostport part of the URL is present and contains a port number, it means that the host points to a specific H.323 entity (rather than specifying a DNS domain containing H.323 SRV RRs). This port value shall be assumed to be the port to which H.323 messages to be directed. Note that if the default port is to be used, the default port number shall be inserted in order to represent this case. The resolving entity shall attempt to retrieve Address Resource Record(s) ("A" RR or "AAAA" RR) for the domain name specified by the host. If more than a single record is retrieved, the resolving entity should select a single record based on its local policy (see also O.10.1). The H.323 messages shall be sent to the retrieved (and potentially selected) IP address and the port specified by the URL.

When the hostport part of the URL is present but does not contain a port number, it hints that the host most probably specifies a DNS domain containing H.323 SRV RRs. The resolving entity should attempt to locate the entity by performing a sequential SRV records retrieval within a subset of the possible H.323 services (i.e., h323ls, h323ls-sec, h323rs, h323rs-sec, h323be, h323cs and h323cs-sec) and their corresponding possible transport protocols (i.e., udp, tcp, and h323mux) according to the procedure specified in O.10.4. The subset shall match the resolving entity capabilities and the purpose of the procedure (i.e., locating a Gatekeeper vs. locating a foreign border element vs. locating a destination). If the H.323 URL service-parameter is present or the SRV service (e.g., h323rs) is specified, the order of the SRV lookups should start based on its value. In the case that the service-parameter is not specified, the resolving entity may search for any or all SRV record types in any order.

For each successful retrieval, an additional DNS lookup needs to be performed to retrieve the Address Resource Records. If successful, the H.323 messages shall be sent to the retrieved and selected IP address and a default port number (corresponding to the transport protocol).

If the SRV RR retrieval procedure is not implemented or fails, the resolving entity may attempt to retrieve the Address Resource Record(s) for the domain name specified by the hostport even if the port has not been specified. If more than a single record is retrieved, the resolving entity should select a single record based on its local policy (see also O.10.1). If successful, the H.323 messages shall be sent to the retrieved (and potentially selected) IP address and a corresponding default port number.

## O.10	Using DNS SRV Resource Records

### O.10.1	Applicability

By using DNS SRV RRs (RFC 2782 [O-3]) it is possible to publish an address (i.e., an URI) for a specific service (Service) that can be reached over a specific protocol (Proto). "The SRV RR allows administrators to use several servers for a single [DNS] domain, to move services from host to host with little fuss and to designate some hosts as primary servers for a service and others as backups."

In the following clauses, this annex defines symbolic names for H.323 services and H.323 transport protocols to be registered with IANA and required for using the DNS SRV RRs. This annex also defines normative procedures for the SRV RRs usage in H.323 systems.

### O.10.2	IANA registration

This specification defines the following symbolic names to be used in the Service field of the SRV record according to RFC 2782 [O-3].

| Service | Name | Meaning |
| --- | --- | --- |
| h323ls | Location Service | H.323 entity supporting H.225.0 LRQ procedure |
| h323ls-sec | Secure Location Service | H.323 entity supporting H.225.0 LRQ procedure with security |
| h323rs | Registration Service | H.323 entity supporting H.225.0 RRQ procedure (i.e., a Gatekeeper that accepts registration of endpoints) |
| h323rs-sec | Secure Registration Service | H.323 entity supporting H.225.0 RRQ procedure with security |
| h323cs | Call Signalling | H.323 entity that performs H.225.0 call signalling |
| h323cs-sec | Secure Call Signalling | H.323 entity that performs H.225.0 call signalling with security |
| h323be | Border Element | H.323 supporting communication as defined in Annex G/H.225.0 |

This specification defines the following symbolic names to be used in the Proto field of the SRV record according to RFC 2782 [O-3].

[End]

__________________