| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4545 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: TLS session negotiation - Data object "TLS profile" - Complementary presentation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The "TLS session negotiation package" provides a fairly powerful tool concerning a wide range of use case specific TLS service support. The data object "TLS profile" is the crucial key concept behind that capability. In order to cross-check the existing H.248.TLS protocol syntax/semantic

Discussion

There are some illustrations attached to this contribution. Some are new, some are copied from H.248.TLS.

A) What we did (or still doing): defining a data model for data object "TLS profile", which includes

- a structural data model (conceptual model; related to "static" aspects, classes, …) and

- a behavioral data model (computation model; related to "dynamic" aspects, instantiations, …).

Based on that evaluation we noticed a couple of observations:

B) Observations and aspects for clarification in H.248.TLS:

Table 1 – Observations and aspects for clarification in H.248.TLS

| No.: | Topic: | Comments: |
| --- | --- | --- |
| 1 | content of data object "TLS profile" | contains also "certificates" Yes/No? Status: included per term definition Proposal: should be excluded … because rather an orthogonal data object - separate data objects "TLS profile" and "Certificate chain" |
| 2 | structure of data object "TLS profile" | e.g., the "TLS MG profile" is a 7-tuple: 4-out-of--7 elements are single values (Boolean or Integer), but 3-out-of-7 represent a list structure Question: do we allow just a single value in case of a list, or the entire list? Status: we allow entire lists Proposal: we should proceed with the list option Example: a TLS MG profile could contain three supported TLS versions => there's a single TLSMG profile in case of list option, but there would be three (TLS protocol version related) TLS MG profiles |
| 3 | access via management plane | H.248 properties which are "provisionable" are inherently "managed objects" from management plane perspective. Thus, what's the view of the "TLS profile"object (as visible via H.248 tlscn package)? |
| 4 | Root vs non-Root operational level | Instantiation process of a "TLS endpoint profile"? During creation of the "Root termination", or always when new non-Root terminations are created? |
| 5 | Variant "profile" vs "individual" indication | The "profile" method implies a set of pre-provisioned "TLS domain profiles" at MGC and MG level, plus a naming scheme (which will be defined by an H.248 profile … see H.248.TLS). Hence, there are a set of "TLS endpoint profiles" as well (and not only a single one) due to "TLS endpoint profile" = intersection of "TLS MG profile" and "TLS domain profile(s)" |
| 6 | MGC auditing | Which data object exactly is accessed by an auditing procedure? Proposal: 1) TLS MG profile information should be part of AuditCapability ("because reflecting the supported capabilities") 2) TLS domain profile information: out of scope of auditing ("because such information already known at MGC level") 3) TLS endpoint profile information: should be part of AuditValue ("because reflecting the used capabilities") |
| 6a | AuditCapability | we need to cross-check H.248 procedures |
| 6b | AuditValue | we need to cross-check H.248 procedures |
| 7 | variants of data object "TLS endpoint profile" | Aspect: there are actually two instantiations, an initial object before TLS session negotiation and a subsequent one according the results of the TLS session negotion NOTE: there's no need to consider an entire data model of a TLS implementation in an H.248 MG, we need just to figure out the relevant data objects as visible from H.248.TLS perspective. |
| 8 | H.248 data object "TLS endpoint profile" vs TLS data object "TLS connection state" | See clause 6.1/RFC 5246. NOTE: both data objects are NOT identical, but overlapping. If we differentiate between "TLS endpoint profile (initial)" and "TLS endpoint profile (used)", then the "TLS endpoint profile (used)" reflects the "TLS connection state" (in more detail: only the three object elements "TLS version", "cipher suites" and "compression methods"). Consideration: explicit text in H.248.TLS concerning relation to "TLS connection state"? |
| 9 | TLS session resumption | The semantic of resumption in the TLS context is not exactly the same as the usual "suspend /resume connection" concepts in communication networks. More info in see AVD-4498. Here: which data object, e.g., "TLS endpoint profile", would be starting point for "resumption"? |
| 10 | H.248 bearer concepts: "TLS session" vs "TLS connection" | Status: we are just using loosely the notion of "TLS session", which is sufficient in H.248.TLS when we could equate "TLS-enabled H.248 SEP" represents a "single TLS session". Question: the "TLS session resumption" capability actually demands for a consideration of "TLS connections", too. If not, semantics are fairly difficult … Proposal for consideration: add definitions to H.248.TLS … e.g. something like - TLS session: "a TLS session is constituted by the 6-tuple of {session identifier, peer certificate, compression method, cipher spec, master secret, is resumable}" (based on clause 7/RFC 5246) - TLS connection: "a TLS connection is constituted by the 15/21-tuple of {…}" (based on clause 6.1/RFC 5246) Such better bearer concepts would facilitate a) TLS session resumption semantics and b) (perhaps) improve "TLS profile" definition |
| 11 | H.248.TLS terminology | We need to reconsider our "TLS profile definitions 3.2.4, 3.2.6, 3.2.9 and 3.2.10 |
| 12 | others? |  |

Conclusion

The majority of items could be considered as "implementation specific" from H.248.TLS perspective. Thus, the results of our data modeling exercise should not jeopardize the planed consent/approval date in June/July this year.

Further, the first version of the tlscn package (with its associated "TLS profile" concept) is uncontroversial, particular due to the assumption that the package usage needs to be detailed by a H.248 profile.

However, a more detailed package definition could benefit from conclusions from a data modelling task. This is of course not surprising: every good protocol specification needs to be a complementary formal specification ("SDL in the old days") besides the prose text, especially for control plane protocols with management plane interactions (as here).

Our plans:

- we are continueing in defining an entire (UML based) date model with regards to class/object "TLS profile"

- any identified change requests will be submitted to H.248.TLS at next meeting

- the data model (which represents a formal specification of a sub-set of the "H.248.TLS" GCP extension) may be documented (if support) in a future Supplement.

Proposal

There are NOT any changes proposed against H.248.TLS.

Attachments – Some illustrations

Figure 1: Background – TLS profile concept

Figure 2: Background – TLS endpoint profile 
= Intersection of the TLS domain profile 1 and the TLS MG profile

| H.248.80 generic media configuration | TLS-specific media configuration | H.248.TLS TLS profile term: |
| --- | --- | --- |
| Overall term: |  |  |
| Configuration (clause 3.2.1 / [ITU-T H.248.80]) | TLS configuration | TLS profile |
| Qualified term: |  |  |
| Prefered configuration (e.g., prefered codec list) | Prefered TLS configuration (NOTE 1) | TLS domain profile |
| Supported configuration (e.g., supported codec list) | Supported TLS configuration | TLS MG profile |
| Actual configuration (e.g., actual codec list) | Actual TLS configuration | TLS endpoint profile |
| NOTE 1 – There is a subtle difference between [ITU-T H.248.80] and this Recommendation: there are no SIP-level SDP Offer/Answer negotiations concerning TLS configurations due to the fact that TLS provides itself an embedded TLS handshake protocol for TLS-level negotiations. Thus, there is neither an "Offered TLS configuration" nor "Answered TLS configuration" at application control protocol level. |  |  |

Figure 3: Analogous terms between H.248.80 media configurations and TLS profile types

Figure 4: Illustration of TLS profile concept (Note: OAM view added for MG)

Figure 5: High level model of object "TLS MG profile"

Figure 6: H.248.TLS: object "TLS MG profile" in tlscn package

Figure 7: Object lifetime aspects

Figure 8: Overview data object "TLS profile" – Types and Instantiations 
(NOTE: this is just an example model, there might be better ones … particuarily concerning dynamic (instantiation) aspects)

________________________