| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4559 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Question(s): |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-SIG: High level framework introductions |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

Since the new work item H.TPS-SIG has been created at the last plenary meeting in Geneva we propose to contribute some substantial contents into H.TPS-SIG document. This contribution is relevant with framework in H.TPS-SIG.

# Discussion

The most significant software difference between the telepresence endpoint and the legacy endpoint is the newly defined parameters for the telepresence system including media capture, capture scene, individual encoding and encoding group etc. so the most important job in signalling for us to do is to negotiate these telepresence parameters and cooperate with other procedures smoothly based on H.323 series standards.

Since CLUE already gets some progresses in the singling of SIP-based telepresence system we can make the reference of the research form CLUE. The Basic Information Flow between the two SIP-based telepresence endpoints can be classified into two-round signalling exchange.

The first round exchange is the basic negotiation for ‘BASIC SDP MEDIA SESSION’ including the first offer/answer conveyed by ‘INVITE/200 OK’ to exchange the basic audio and video capabilities, the information to create the CLUE channel and maybe the individual encodings. For now there is an individual draft “draft-groves-clue-latent-config-00” in IETF CLUE discussing to use latent configuration to describe the individual encoding. We also think it is an appropriate method because it avoids allocating the actual resource before it is actually used and also makes the SDP and CLUE negotiation relatively independent. This first exchange establishes a basic media session which is optional and implementation dependent, for example audio-only, and also a CLUE channel between two Endpoints.

The second round is to ‘UPDATE SDP MEDIA SESSION’ for the final media transmission including the bidirectional of CLUE messages (ADVERTISEMENT/CONFIGURE etc.) via the CLUE channel and ‘REINVITE/ 200 0K’ (UPDATED SDP) to update the media channels according to the result of the telepresence capability negotiation.

Figure below is the signalling flow between the SIP-based telepresence endpoints concentrated from the figure 2 basic information flow in the CLUE-FRAMEWORK draft.

The signalling flow between the SIP-based telepresence endpoints

From the above the two-round exchange method adds the secondary negotiation to separate the new telepresence capability negotiation from the existing basic capability negotiation. This method solves the negotiation for the new telepresence features without breaking the existing protocol stack and also considers the compatibility with the legacy endpoint.

We can also use the two-round exchange method in H.323-based telepresence system.

It is better to keep the Registration and Call Setup procedures unchanged, since these two procedures need Gatekeeper to support the changes if they have any. If there are changes in the Registration and Call Setup it is not easy to interwork with the legacy device. It may need to restart the Call Setup procedures after the telepresence-enabled endpoint was rejected by the legacy device because the incomprehensible telepresence associated attribute.

We can use the H.245-based terminalCapabilitySet and openLogicalChannel message to do the first round exchange to open the basic media channel and also the CLUE channel.

In the terminalCapabilitySet syntactic structure the receive capability can be used to carry the basic media capability for the basic capability negotiation with no changes. The GenericCapability can be used to carry the indication information to indicate that the endpoint is the telepresence endpoint and CLUE-capable (support the CLUE channel and CLUE protocol). The transmit capability can be used to carry the individual encoding. The transmit capability is in some way like the latent configuration in SDP which was discussed in IETF draft “draft-groves-clue-latent-config-00”. The Transmit capabilities serve to offer receivers a choice of possible modes of operation so that the receiver may request the mode which it prefers to receive. The transmit capability and the latent configuration they both need the receiver to select what it wants before the actual resource is allocated for the media channels.

The openLogicalChannel message is subsequently used to open the basic media channel and also the CLUE channel. The benefit to choose CLUE channel to convey the CLUE message we can refer another contribution.

The second round exchange is the CLUE message exchange via the CLUE channel and H.245 messages including requestMode and openLogicalChannel message. The CLUE message is to exchange the newly added CLUE capabilities. The H.245 requestMode is for the receiver to select the individual encodings conveyed in the transmit capability from the transmitter. The H.245 openLogicalChannel message is used to update the media channels for the transmission of the telepresence media streams selected by the receiver.

# Proposal

It is proposed to add the following text into draft H.TPS-SIG. Changes are marked up against H.TPS-SIG Ed. 0.1(T13-SG16-131028-TD-WP1-0120) from the last plenary meeting.

Start proposed change

# 6	Framework

{Editor’s note: This section is to provide a linkage to architecture document and to indicate what signalling interfaces are being covered by this draft Recommendation. It will also provide a high level introduction of how telepresence support is introduced into H.series Recommendations.}

6.x High level introductions

The most significant difference between the telepresence endpoint and the legacy one is the newly defined parameters for the telepresence system including the descriptions of the media capture, capture scene, individual encoding attributes, encoding group and simultaneous transmission set constraints etc. We need consider how to negotiate these telepresence capabilities and cooperate with other procedures smoothly based on H.323 series standards.

Figure 1 High level information flow

The figure 1 is the high level information flow between the two H.323-based telepresence endpoints.

The registration and the call setup procedures follow the H.323 definitions. After these there is two-round signalling exchange to introduce the telepresence support into H.323-based signaling.

The first round exchange uses the H.245-based the terminalCapabilitySet message to exchange the codec information and telepresence indication information and the openLogicalChannel message to open the basic media channel and also the CLUE channel.

The terminalCapabilitySet message is to negotiate the basic media capability. The receive capability in the terminalCapabilitySet message can be used to carry the basic media capability for the basic capability negotiation. The transmit capability in the terminalCapabilitySet message can be used to carry the individual encoding information. The terminalCapabilitySet message also carries the telepresence indication information to indicate that the endpoint is the telepresence endpoint and CLUE-capable. The GenericCapability can be used to carry the indication information.

The openLogicalChannel and openLogicalChannelACK messages are to open the basic media channel according to the result of the basic media capability negotiation. Whether or not to transmit the basic media, for example audio-only, is optional and implementation dependent. The openLogicalChannel message can be used to open the CLUE channel after the exchange of the telepresence indication information. The CLUE channel should be opened according to the collection of the information like transmission protocol, channel address (IP/port) to create the CLUE channel.

The result of the first round exchange establishes the basic media channel and the CLUE channel.

The second round exchange uses the CLUE messages to negotiate the telepresence capabilities, the H.245 request Mode messages to request the media data that the receiver needs and the H.245 open Logical Channel operations to update the media channels used to transmit the real telepresence media streams selected by the receiver.

The CLUE message is used to negotiate the telepresence capabilities via the established CLUE channel. How to use the CLUE message should follow the CLUE protocol defined by IETF.

The H.245 request Mode messages are used to request the media data the receiver would like to receive offered in the transmit capabilities from the transmitter.

Once again the H.245 open logical channel operations are used to open the media logical channels bidirectionally according to the result of the second round negotiation. Then the telepresence media streams can be transmitted through these media logical channels.

End proposed change

__________________