| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4560 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Question(s): |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-SIG: Carriage of CLUE messages in H.323 systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Introduction

Since the new work item H.TPS-SIG has been created at the last plenary meeting in Geneva we propose to contribute some substantial contents into H.TPS-SIG document. This contribution is relevant with section 7.2 Carriage of CLUE messages in H.323 systems.

Discussion

Since in last meeting the experts in Q5/16 agreed that re-use of the CLUE signalling is highly desirable, fist of all we need find which channel we can use to convey CLUE messages in H.323 system. There are three primary methods to solve this issue. First method is to use H.225.0 channel to convey CLUE messages. Second is to use H.245 channel to convey CLUE messages. Third is to use CLUE channel to convey CLUE messages.

The three different methods will be discussed below and also compared from five aspects.

- Use H.225.0 channel to convey CLUE messages

H.225.0 channel is the reliable channel to convey the call signalling messages. It is also defined to convey other signalling messages such as Tunneling of signalling protocols (10.4/H.323) which provides a generic means of tunneling signalling messages in any H.225.0 call control message. This can be used to transfer CLUE messages use H.225.0 channel in H.225.0 type messages.

The pros and cons from 5 aspects of the first method are listed below:

  - Not easy to process the messages

Using H.225.0 channel to covey CLUE messages is not easy to process the messages. Comparing with using CLUE messages alone it needs to pack and unpack CLUE messages into and from the H.225.0 messages.

  - Involve gatekeeper in H.323 system

H.255.0 messages may involve some intermediate entities like GK to support this method that will increase the difficulty of the implementation.

  - Not easy to interwork with H.323-based non-CLUE-capable endpoint

Using H.225.0 channel to covey CLUE messages is not easy to interwork with H.323 non- CLUE-capable endpoint. It may need to resend the call setup messages. If one side is the CLUE-capable endpoint A and the other side is the non-CLUE-capable H.323 legacy endpoint B, endpoint B will reject the establishment of the call including the CLUE attributes from endpoint A. It may require endpoint A to send the non-CLUE-call establishment messages again after A received the rejection from B. All of these will increase the complexity of the procedure and the time to establish a call.

  - Not easy to interwork with SIP-based telepresence endpoint

Using H.225.0 channel to covey CLUE messages is also not easy to interwork with SIP-based telepresence endpoint. The CLUE messages are packaged in H.225.0 messages using H.225.0 to transfer so it will cost extra resource for GWs to unpack CLUE messages from H.225.0 call signalling messages and also pack CLUE messages into H.225.0 messages.

  - Without increasing the number of the ports

H.225.0 channel always need to be opened. Reuse H.225.0 channel and message elements to convey CLUE messages will reuse the ports already existing. This will keep the number of the port which will be good for Firewall/NAT.

- Use H.245 channel to convey CLUE messages

H.245 channel is the reliable channel used to carry the H.245 control information messages between two H.323-based endpoints. We can also find the way using H.245 channel to convey CLUE messages in H.245 type messages such as Generic messages (B.15/H.245).

The pros and cons from 5 aspects of the second method are listed below:

  - Not easy to process the messages

Using H.245 channel to covey CLUE messages is not easy to process the messages. It needs to pack and unpack CLUE messages into and from H.245 type messages. CLUE messages are not the only kind of message needed to be conveyed in H.245 channel. Except CLUE messages the original H.245 messages like MC, TCS, OLC also need this H.245 channel to convey. In this situation the progress needs to distinguish the messages from H.245 and CLUE. After the two kinds of messages are distinguished the progress still needs to separate CLUE messages from H.245 message structure.

  - Not involve gatekeeper in H.323 system

H.245 message is an endpoint-endpoint message. It doesn’t need the intermediate entities like gatekeeper to support this method.

  - Easy to interwork with H.323-based non-CLUE-capable endpoint

This method only needs one-round call setup procedure no matter the endpoints are CLUE-capable or not. Whether or not to send CLUE messages depends on the result of the CLUE-capable indicator exchange. So using this method is no need to resend any message repeatedly.

  - Not easy to interwork with SIP-based CLUE-capable endpoint

Using H.245 channel to covey CLUE messages is also not easy to interwork with SIP-based telepresence endpoint. The CLUE messages are packaged in the H.245 messages so it will need gateway to analyze the messages and unpack CLUE messages from H.245 messages.

  - Without increasing the number of the ports

H.245 channel always need to be opened. Reuse H.245 channel and message elements to convey CLUE messages will reuse the ports already existing. This will keep the number of the port which will be good for NAT.

- Use CLUE channel to convey CLUE messages

Since CLUE message is originally conveyed via CLUE channel defined in IETF CLUE it is easiest way for us to re-use the CLUE messages in H.323-based system.

The pros and cons from 5 aspects of the third method are listed below:

  - Easy to process the messages

CLUE channel is independent from H.225.0 and H.245 channel. Using CLUE channel to convey the CLUE messages is easy for the progress to listening and parsing the CLUE messages because of the independence of the CLUE channel and the CLUE messages are not packed into any structure of the other messages.

  - Not involve gatekeeper in H.323 system

The CLUE channel should be established and convey the CLUE messages after the call setup procedure and the exchange of the CLUE-capable indicator between the two endpoints. It doesn’t need the intermediate entities like gatekeeper to support this method.

  - Easy to interwork with H.323-based non-CLUE-capable endpoint

To use the CLUE channel to convey the CLUE messages should after the exchange of the CLUE-capable indicator. After get the result of the CLUE-capable indicator the CLUE-capable endpoint will know whether or not to establish the CLUE channel and convey the CLUE messages or just use the non-CLUE-capable method to do the ordinary H.245 negotiation procedures.

  - Easy to interwork with SIP-based CLUE-capable endpoint

Since the SIP-based CLUE-capable endpoint will certainly use the CLUE channel to convey the CLUE messages it’s easy for the gateway to translate and convert.

  - Increasing the number of the ports

The third method needs to create a new channel that means it will increase the number of the ports. But if we choose to use this method it should be executed after the basic H.245 TCS, MC and OLC procedures for the setting up the media transmission itself, involving potentially security exchanges, ICE and whatnot.

The table below lists the brief of the pros and cons mentioned above.

| Aspect Channel | Easy to process the messages | Not involve gatekeeper in H.323 system | Easy to interwork with H.323-based non-CLUE-capable endpoint | Easy to interwork with SIP-based CLUE-capable endpoint | Without increasing the number of the ports |
| --- | --- | --- | --- | --- | --- |
| H.225.0 | N | N | N | N | Y |
| H.245 | N | Y | Y | N | Y |
| CLUE | Y | Y | Y | Y | N |

As a conclusion it’s recommended to use the CLUE channel to convey the CLUE messages since there are much more advantages than disadvantages.

Proposal

It is proposed to add the following text into H.TPS-SIG. Changes are marked up against H.TPS-SIG Ed. 0.1(T13-SG16-131028-TD-WP1-0120) from the last plenary meeting.

Start proposed change

- 7.2	Carriage of CLUE messages in H.323 systems

{Editor’s note: This section shall detail how CLUE messages are sent and received by H.323 systems}

Consider of the aspects like the complexity of the program and the implementation, the compatibility with the SIP-based telepresence system and H.323-based legacy endpoint, it’s recommended to use the CLUE channel to convey the CLUE messages.

Before the CLUE channel is established the telepresence endpoint should exchange the telepresence indication information. The telepresence indication information indicates that if the endpoint is telepresence-enabled supporting the telepresence capabilities and CLUE-capable supporting the CLUE channel and the CLUE protocol. The telepresence endpoint can exchange the telepresence indication information during the first round basic capability exchange through H.245 terminalCapabilitySet message using GenericCapability. Based on the result of the telepresence indication information exchange the telepresence endpoint knows that the opposite endpoint is telepresence-enabled. And then the two telepresence endpoints can establish the CLUE channel and negotiate the telepresence capability via the CLUE channel.

End proposed change

__________________