| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4562 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.57: Rules for port allocation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes a port allocation rule to reconcile the use of the rsb property and the a=rtcp-mux attribute.

# Introduction

The existence of an RTCP flow in an RTP session requires the use of the rtcph/rsb property set to ON. This guarantees that the MG reserves the resources to handle the flow. In addition, the  rtcph/rsb = ON indication requests the MG to allocate one port for the RTCP flow. This is usually the consecutive port to the one allocated for the RTP flow (PORTRTP = PORTRTP+1). However, the MGC may indicate explicitly the port number to be used for RTCP using the a=rtcp attribute.

RFC 5761 specifies the multiplexing of the RTP and RTCP flows in one single port. The current H.248.57 does not support a mechanism that allows an MGC to request the MG to use the same port for RTP and RTCP.

# Discussion

RTP-RTCP multiplexing should require the use of rtcph/rsb = ON, as this property value is primarily an indication that an RTCP flow exists and resources should be reserved. In addition, the a=rtcp-mux must be included in the local descriptor as an indication of RTP-RTCP multiplexing and consequently that no extra port needs to be allocated for RTCP.

# Conclusion

It is propose to introduce the following changes in a new revision of H.248.57:

[Begin Correction]

#### 6.6.1.2	Specific behaviour "activated"

If the MGC sets the rtcph/rsb property to "ON", it expects an RTCP flow to exist for the stream and requests the MG to allocate corresponding resources. When the MG is requested to allocate/de-allocate a local destination port for an RTP stream, a consecutive port for the reception of the associated RTCP flow is automatically allocated/de-allocated with the exceptions described in 6.6.1.4.

[End Correction]

[Begin Correction]

#### 6.6.1.4	Precedence of rtcph/rsb property over other port allocation mechanisms

The setting of the rtcph/rsb property in general takes precedence over any values set in the local and remote descriptors with regard to port allocation for RTP streams. However, there are some exceptions:

- If the rtcph/rsb property is set to ON and a specific RTCP transport address is provided in the local and/or remote descriptor, e.g., using SDP "a=rtcp:" attribute as defined in [IETF RFC 3605], the RTCP packets should use the transport addresses indicated by the local/remote descriptors.

- If the rtcph/rsb property is set to ON and the a=rtcp-mux attribute is provided in the local and/or remote descriptor, the RTCP packets shall use the same port as the one allocated for the RTP stream.

The MGC should avoid using both the a=rtcp and the a=rtcp-mux attributes at the same time, as this leads to conflicting behaviours and may be replied with an error by the MG. However, a MG may also ignore the a=rtcp attribute if the a=rtcp-mux is included at the same time and use the same port for RTP and RTCP.

[End Correction]

[Begin Correction]

#### 6.6.1.6	Allocation rules in case of RTP-RTCP multiplexing [IETF RFC 5761]

Table 3 – Allocation rules for RTCP ports with RTP-RTCP multiplexing [IETF RFC 5761]

| H.248 property value | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" | No | Consecutive port to the one indicated by the local descript. | MG (Note 1) | Consecutive port to the one indicated by the remote descript. | Out of scope |
|  | Yes in H.248 LD | Same port as for RTP | MG (Note 1) | – | – |
|  | Yes in H.248 RD | – | – | Same port as for RTP (Note 3) | Out of scope |
| rsb = "OFF" | No | No port allocated | No port allocated | No port allocated | Out of scope |
|  | Yes in H.248 LD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
|  | Yes in H.248 RD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
| NOTE 1 – The management of LS(A) and LS(P) resources is under control of the media gateway. The MG shall apply the "consecutive" port allocation rule. NOTE 2 – This is a semantic contradiction, however the MG shall ignore the "a=rtcp-mux:" attribute. NOTE 3- Although RFC 5761 only specifies the bidirectional use of RTP-RTCP multiplexing, the MG is not required to return an error if a=rtcp-mux is only present in the LD. |  |  |  |  |  |

[End Correction]

__________________