| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4564 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.RTPMUX: Usage examples of stream representative |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides further examples and justification on the use of the stream representative in different grouping semantics scenarios and removes one example no longer valid.

# Discussion

Several examples are already included in the H.248.RTPMUX draft to illustrate the need of a stream representative for the use of H.248 signals, events, statistics and topology in connection with grouped media streams. One of them related to the ANAT semantics is no longer valid as it has been shown that the ANAT semantics is superfluous in H.248.

With regards to the SRF example and the use of the stream representative to activate an event from the rsvp package, some modifications are needed, since the use of the stream representative does not only serve the purpose of saving signalling, but it is indeed needed, as the received RSVP message refers to all grouped streams.

A new example illustrating the use of the stream representative for the setting of signals in relation to the SRF semantics is beneficial.

# Proposals

[Begin Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (07/2008), Gateway Control Protocol: Statistic conditional reporting package

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 4566]	IETF RFC 4566 (07/2006), SDP: Session Description Protocol

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework

[IETF draft taxonomy]	IETF draft-lennox-raiarea-rtp-grouping-taxonomy (201X), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources

[End Correction]

[Begin Correction]

#### 9.6.5.1 Usage of the Group Representative when requesting signals

Example 1:

A termination T1 has two audio streams with identities 1 and 2 which use the same codec. They are grouped with the FID (Flow Identification) semantics [IETD RFC 5888]. Thus they carry identical content, one is a replica of the other, usually sent to a different destination address. The termination has in addition other media stream with identity 3, not part of the media group. The MGC has defined a group representative: 8. Requesting an an/apv [H.248.7] signal on stream 8 to play an audio announcement should result in the announcement being sent synchronously to streams 1 and 2.

Example 2:

A termination T1 has two audio streams with identities 1 and 2 which use different codecs. They are grouped with the FID semantics [IETD RFC 5888].  At a time only one of the streams is carrying data in each direction while the other one is muted in that direction. This is dependent on the codec choice by the end user equipment which can change dynamically. The MGC has defined a group representative: 8. Requesting an an/apv [H.248.7] signal on stream 8 to play an audio announcement should result in the announcement being sent in the stream which is carrying data at the moment.

Example 3:

A termination T1 has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus resource reservation using RSVP and policy enforcing is applied to the complete group. The MGC has defined a group representative: 8. In order to send RSVP messages related to all grouped streams, the MGC should set the signals of the rsvp package [H.248.65], e.g. rsvp/path in the group representative, i.e. stream 8.

Note: RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media see [IETF RFC 3524].

[End Correction]

[Begin Correction]

#### 9.6.5.2 Usage of the Group Representative when requesting events

Example 1:

A termination T1 has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus resource reservation using RSVP and policy enforcing is applied to the complete group. The MGC has defined a group representative: 8. In order to receive RSVP messages related to all grouped streams, the MGC should request the events of the rsvp package [H.248.65], e.g. rsvp/pathr in the group representative, i.e. stream stream 8.

Note: RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media see [IETF RFC 3524].

Example 2:

A termination T1 has two audio streams which use different codecs. They are grouped with the FID (Flow Identification) semantics [IETD RFC 5888]. Thus, at any point in time, only one of them carries data while the other one is muted. The termination has in addition other media stream with identity 3, not part of the media group. The MGC has defined a group representative: 8 and it orders the MG to collect data volume related statistics on the stream representative. The MGC uses also the conditional reporting functionality [H.248.47] and activates therefore the scr/cr event on the stream representative and refers to the relevant statistics in the si parameter.

[End Correction]

__________________