| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4565 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.SEPLINK: Directionality aspects |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes further specification of the interlinkage semantics and syntax to take into account directionality of the triggering procedure.

# Introduction

H.248.SEPLINK provides mechanisms to link connection oriented establishment or release procedures at one SEP with those of the associated SEP in the SEPP, or between two protocol layers within the same SEP.

Some aspects of the directionality of those procedures need further specification.

# Discussion

If an interlinkage association is established so that a connection establishment or release procedure in one SEP may trigger a connection establishment or release procedure at the other SEP, it is assumed that the trigger is always an incoming procedure, i.e. H.248.SEPLINK can not be used to initiate an establishment procedure triggered by an outgoing establishment procedure at the other SEP.

However when interlinkage is established between two protocol layers of the same SEP, the MGC may want to restrict the trigger to incoming procedures only, to outgoing procedures only, or allow both. For example, in a TLS establishment procedure triggered by a TCP establishment procedure, if the SEP should take the same role in TLS as in TCP, the MGC would limit the trigger to outgoing TCP procedures.

Note: it is assumed that the resulting connection establishment or release procedure is always a procedure initiated by the SEP, i.e. it is an outgoing procedure. Theoretically H.248.SEPLINK could be used also to trigger acceptance of incoming procedures. However this may turn out to be problematic as it may involve autonomous change of property values, like changing the TLS establishment blocking property tlsbsc/bceb to unblocked.

# Proposals

[Begin Correction]

# 6	Motivation, use cases and models

H.248 controlled Media Gateways may terminate multiple connection-oriented transport layer protocols, like TCP, TLS, SCTP.

This is outlined in the following Figure 1 where in a given H.248 context each Stream Endpoint (SEP) is supposed to host at least one or more connection-oriented transport layer endpoints.

Figure 1 – Two-termination context with multiple connection-oriented protocols 
in the SEP-specific protocol stack (segments)

The following Figure 2 outlines the various interlinkage options enabling an MG autonomous establishment/release of a connection/session-oriented partner transport protocol endpoint (again at the example of a stream endpoint pair (SEPP)):

Figure 2 – Principle interlinkage options (here: two-termination context) 
from perspective of a single SEP (here: SEP-A)

Options (in Figure 2):

- Within one SEP a lower layer connection oriented transport protocol endpoint (EP) triggers an upper layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Within one SEP an upper layer connection oriented transport protocol EP triggers a lower layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Given a SEP pair SEP-A triggers the connection oriented transport protocol of the peer SEP-B. Either the same or a different transport protocol can be interlinked, thus for example SEP-A TCP endpoint could trigger a SEP-B TCP endpoint, or a SEP-A TCP endpoint could trigger e.g. a SEP-B SCTP endpoint.

- Given a SEP pair SEP-A receives a trigger regarding a connection oriented transport protocol from its peer SEP-B. Either the same or different transport protocol can be interlinked, thus for example SEP-B TCP endpoint could trigger a SEP-A TCP endpoint, or a SEP-B TCP endpoint could trigger e.g. a SEP-A SCTP endpoint.

{Editor's note (2013-11):

The four examples of stimuli-based information flows are valid, but the structuring principles are unclear and should be revised: the protocol semantic of the seplink/linktopo property represents following information model:

The conditions indicate the supported logic by the seplink/linktopo property.

It may be observed that Figure 2 does neither describe the stimuli flow from source nor from sink perspective, rather from a particular SEP. However, the property allows only to define the stimuli flow in outgoing direction (due to the unidirectional nature) when the sink is located in a different SEP. Consequently, option 4 looks odd in above text.
A revision is solicited.}

There are following principal limitations:

- Horizontal protocol layer interlinkage (i.e., related to the stream endpoint pair (SEPP)) is only permitted between SEPs belong to the same H.248 stream.

- Vertical protocol layer interlinkage is only permitted within a SEP, i.e. between its supported transport protocol layers.

- In horizontal protocol layer interlinkage only incoming procedures are permitted as stimuli.

- In vertical protocol layer interlinkage both incoming and outgoing procedures are permitted as stimuli. The MGC may optionally restrict the stimuli to one or the other.

- The triggered procedure is always an outgoing procedure

Contributor’s note: theoretically H.248.SEPLINK could be used also to trigger acceptance of incoming procedures. However this may turn out to be problematic as it may involve autonomous change of property values, like changing the TLS establishment blocking property tlsbsc/bceb to unblocked.

- Establishment procedures within one SEP shall follow an upwards order, i.e the initiation of an establishment procedure shall only start when the establishment procedure at the lower layer is completed

- Release procedures within one SEP should generally follow a downwards order, i.e. the inititation of a release procedure should only start when the release procedure at the upper later is completed. However, depending on the protocol rules, it may be possible to omit an explicit release procedure at the upper layer altogether.

A H.248 context may contain more than two terminations; where more than two terminations host SEPs belonging to the same H.248 stream. In this case the SEP interlinkage capabilitiy needs to be able to address a dedicated stream endpoint tuple (SEPT).

For example, following interlinkage scheme should be supported in case of an H.248 context containing three SEPs (same H.248 streamID).

Figure 3 – Interlinkage example for a three-termination context

Whereas SEPP (SEP-A,SEP-B) can trigger each other regarding the identified transport protocol endpoints, the SEP-A connection oriented transport protocol EP may trigger SEP-C connection oriented transport protocol EP in addition. However SEPP (SEP-B,SEP-C) doesn’t have any interlinkage, whereas also SEP-C connection oriented transport protocol EP cannot trigger SEP-A connection oriented transport protocol EP in any way.

{Editor's note (2013-11):

In the current TCP, TLS package specific transport protocol layer coupling capabilities had been introduced (tcpecc/sepplink, tlsesc/sepplink, tlsesc/plslink), see H.248.TCP/H.248.TLS.

This new package is intended to provide more generic capabilities, i.e.

- enabling SEP pair identification in case of H.248 contexts with more than two SEPs,

- enabling the interlinkage of different transport protocols,

- re-usability of this generic mechanism for any other connection-oriented transport protocol, e.g. SCTP, thus avoiding to define transport layer coupling capabilities again and again.

This new package would then replace the correspondent properties in H.248.TCP/H.248.TLS.}

[End Correction]

[Begin Correction]

### 7.1.1	Interlinkage topology

|  | Property Name: | Interlinkage topology |
| --- | --- | --- |
|  | Property ID: | linktopo (0x0001) |
|  | Description: | This property defines the MG interlinkage behaviour regarding connection/session oriented transport endpoints. Interlinkage can be specified between the connection oriented transport layers of a given SEP or between any connection oriented transport layers of a given SEP pair. Interlinkage defines a undirectional information flow ("stimuli") from a "SEP-local transport EP" towards another "SEP-local or –remote transport EP". |
|  | Type: | Sub-list of Strings |
|  | Possible values: {Editor's note (2013-11): the syntax specification of values may benefit from an ABNF based format, e.g. the mode seems to be a sub-list also. Need to allow binary terminationIDs etc.} | The property consists of a sequence of interlinked transport protocol endpoints of the form "interlinked-SEP, source-transport-EPs, interlinked-transport EP, mode". Specification of string format of: interlinked-SEP "," source-direction source-transport-EPs "," interlinked-transport EP "," mode Where: - "," is a literal functioning as separator - interlinked-SEP is {Editor's note (2013-11): the "interlinked-SEP" is the local SEP. The identifier for a SEP is the 2-tuple of of StreamID and TerminationID values (see clause 3.2.10/H.248.1). The StreamID of the "interlinked-SEP" is implicitly given be the StreamDescriptor where the linktopo property is located, but not the TerminationID. Thus we need an explicit indication of the TerminationID value. Below description of the semantic should be enhanced} - semantic: specifies the interlinked stream endpoint by using the TerminationID, possibly using the ALL or CHOOSE wildcard. In case the interlinked TerminationID is the same TerminationID as of the SEP the property is belonging to vertical interlinkage is triggered. If they are not the same, horizontal interlinkage is triggered. {Editor's note (2013-11): TerminationID issue is that the semantic of interlinked-SEP is a terminationID but the syntax given is for a StreamID i.e. UINT16. Source-transport-EP and interlinked-transportEP have a similar syntax issue; further: binary encoding should be covered} - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. - source-direction is optional and is only significant for vertical interlinkage. - semantic: specifies the direction incoming or outgoing of the triggering procedure . - source-direction = Bothtoken / Incomingtoken / Outgoingtoken - Bothtoken = “IO-” - Incomingtoken = “I-” - Outgoingtoken = “O-” - Default is bothtoken - source-transport-EPs is - semantic: specifies the source transport endpoint, e.g. TCP, TLS, SCTP or others. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. - interlinked-transport EP is - semantic: specifies the interlinked transport endpoint, e.g. TCP, TLS, SCTP or others. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. Contributor’s note: UINT16 does not seem the correct data type. NAME seems more suitable to convey the transport type. NOTE: The transport protocols as being used by the source/interlinked-transport-EP shall partially match with the particular SDP "m="-line transport protocol defintion (see clause 7.1.8.1.1/[ITU-T H.248.1]). Detailed agreements regarding the admitted SDP "m="-line transport protocol <proto> values shall be defined in the particular H.248 profile specifications, typically in the profile clauses on mandatory/optional support of SDP. - mode is - semantic: specifies whether the interlinkage is related to connection/session establishment and/or release procedure; Possible values: {Editor's note (2013-11): a sublist based value would be more future safe} - "est": establishment interlinkage only. - "rel": release interlinkage only. - "est-rel": establishment and release interlinkage. - syntax: as per UINT16 in clause B.2 of [ITU-T H.248.1]. TBC: use case: do we need to support multiple transport layer EPs of the same type per SEP, e.g. TCPoTCP, IP-over-IP tunneling? |
|  | Default: | Empty list |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

[End Correction]

__________________