| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4569 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.IANA (New): "SDP codepoints for gateway control"– Discussion |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the start of a new work item on the registry of H.248 specific SDP codepoints with IANA.

# Motivation

Problem statement:

Gateway control protocols may demand additional SDP codepoints, - beyond e.g. SDP information used by call control protocols -, for specific interworking functions supported at bearer plane level. This is a well-known fact, at least since the introduction of border gateways back in 2004/2005 (see e.g., Annex G in ETSI TR 183 068).

The problem could be mitigated by use of the "-" value in SDP, which was sufficient when the number of supported interworking functions at MG level were fairly limited to a few scenarios per H.248 profile. We've now reached the point where such "patches" are not sufficient anymore, particularily also due to the fact when alternative signalling (wrt the "-") was introduced when ETSI TISPAN transferred their border gateway profiles to 3GPP.

Prefered proceeding would be the registration of protocol codepoints with IANA, as identified and required by H.248 packages (and H.248 profiles).

See email "[Megaco] SDP codepoints for gateway control; FW: IANA "proto" codepoints;,"

Furthermore: the topic as such is also relevant and subject of H.248.SEPLINK (in case that such SDP codepoints would be used as property parameter values ("which is currently the assumption, see AVD-4482 and AVD-4483").

General considerations:

- IANA registration SHOULD be based on an Standards-Track RFC.

- Technology owner of SDP is IETF WG MMUSIC.

- ITU-T SHOULD NOT bypass IETF concerning SDP codepoint registrations (as already done (unfortunately) in the past, see e.g., T.38).

- IETF does NOT develop gateway control protocols (for the time being): WG MEGACOP is closed.

- IETF WG MMUSIC does NOT consider use cases with "middleboxes", "intermediaries", etc from SDP point of view. The MMUSIC products just consider simple client/server models with limited focus on SDP OFFERER and SDP ANSWERER roles.

- IANA SDP registry requests MUST be motivated by use cases.

- IETF WG MMUSIC does NOT consider use cases for decomposed gateway control using H.248 (or MGCP, or …).

There are (at least) three options:

- Private initiated reqistration without any ITU-T involvement: individuals propose for individual SDP codepoints correspondent IETF MMUSIC draft/RFCs.

- Non-ITU-T SDOs (such as ETSI NTECH, 3GPP, etc) prepare IETF input for H.248 specific SDP codepoints.

- ITU-T itself collects and coordinates the input for IETF MMUSIC.

Alcatel-Lucent is in favour of option (3), primarily due to the aspects that ITU-T is technology owner for protocol "H.248" and the correspodent review process of a critical mass of experts. Figure A outlines the principal proceeding for option (3).

NOTE – We neglect the fact of the different member types in ITU-T (states, organizations) and IETF (individuals), which imply that IETF drafts could NOT authored by e.g., "ITU-T Q.3/16", rather needs to be mapped on individuals.

Figure A – IANA registry - Proposed process for SDP registrations

We expect continuous updates, dependent on the development of new H.248 packages, procedures or/and profiels. It's thus more a living document, similar as our H.Sup2 (ITU-T H.248.x sub-series packages guide).

The proposed registration process is described in more detail in parallel AVD-4570.

# Proposals

Start a new work item on a new Supplement as central document concerning the registration of H.248-specific SDP codepoints with IANA.
A work item description is attached to this contribution.

Parallel AVD-4570 provides an initial draft for such a Supplement. See also Figure B:

Figure B – Overview of AVD contributions at this meeting related to H.Supp.IANA

Annex Q3.X
Proposed new ITU-T Recommendation H.Supp.IANA

| Question: | 3/16 | Proposed new ITU-T Supplement | Geneva (CH), 10 – 14 March 2014 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Supplement ITU-T H.Supp.IANA "H.Supp.IANA: "SDP codepoints for gateway control" - Discussion" |  |  |  |  |
| Base text: | Contribution AVD-4569 |  | Timing: | 2014 |  |
| Editor(s): | Nomen nescio |  |  |  |  |
| Scope: Single, and thus central document concerning the |  |  |  |  |  |
| - collection of candidate H.248-specific SDP codepoints for registration with IANA and - documentation of already registered IANA codepoints. |  |  |  |  |  |
| Summary |  |  |  |  |  |
| This Supplement provides a central place of H.248-specific SDP codepoints. |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| principally to the entire H.248.x-series of Recommendations plus H.248 related profiles, Supplements, etc |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| 3GPP, IETF |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Alcatel-Lucent, ?, ? |  |  |  |  |  |

_______________________