| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4583 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SCTP: clause 8 "SCTP basic connection control package" - Discussion of "StreamMode vs SCTP" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes input material related to StreamMode handling, similar as for bearer types "TCP" and "TLS" (see H.248.TCP, H.248.TLS).

Discussion

Purpose of this contribution is to start early thinking about StreamMode handling. This topic is a well-known issue concerning IP data path processing by H.248 MGs. The situation is somehow simpler as in case of TCP and TLS, but on the other side more tricky due to the "SCTP chunk" concept.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the output draft from last meeting (see TD 113 (WP 1/16)).

Change proposal:

[Begin Proposed Correction]

# 8	SCTP basic connection control package

|  | Package Name: | SCTP basic connection control package |
| --- | --- | --- |
|  | Package ID: | sctpbcc (0x####) |
|  | Description: | This package provides the functionality to establish and release a SCTP bearer connection. |
|  | Version: | 1 |
|  | Extends: | None |

[…]

## 8.6	Procedures

[…]

### 8.6.x	SCTP application data transfer

#### 8.6.x.1	MG external

The SCTP bearer connection is ready for application data transfer when remote and local SCTP endpoints are both transitioned to SCTP association state "ESTABLISHED" (see Figure 3/[IETF RFC 4960]).

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the SCTP control information used to establish or close the SCTP association. The requested MG behaviour is defined by Table x1:

Table x1 – Impact of StreamMode on SCTP bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All SCTP packets sent |
|  | Sending disabled (i.e., values RecvOnly, Inactive) | Only SCTP packets with SCTP control chunks. SCTP packets with SCTP data chunks only are blocked. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All SCTP packets accepted |
|  | Receiving disabled (i.e., values SendOnly, Inactive) | Only SCTP packets with SCTP control chunks (optional piggybacked SCTP data chunks would be removed???). SCTP packets with SCTP data chunks only are blocked. |

{Editor's note (2014-03): above table provides initial input for discussion. The situation is slightly different as in case of TCP and DTLS:

- SCTP: multiple SCTP chunks are multiplexed on a single SCTP packet

  - control information: in SCTP header

  - control information: explicit SCTP control chunks (ca. 10 types)

- TCP: control information solely part of TCP header

- TLS: dedicatel sub-protocol for control information; i.e., data relates to TLS messages with application data, and control to TLS negotiation/cipher/alert protocol

We need to consider (at least) following two aspects:

- SCTP control chunks: any types which could be similar handled as "data"?

- draft-ietf-tsvwg-sctp-ndata (http://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-sctp-ndata/ )

). }

[End Proposed Correction]

____________________