| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-16 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.DTLS |  |  |  |
| Title: | H.248.DTLS "Gateway control protocol: H.248 support for control of transport security using DTLS" (New): Output draft Ed. 0.6 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.DTLS "Gateway control protocol: H.248 support for control of transport security using DTLS" (Ed. 0.6).

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.DTLS Ed. 0.1 | Initial draft (AVD-4419) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.DTLS Ed. 0.2 | Output (TD-34) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-34.zip |
| H.248.DTLS Ed. 0.3 | Input (TD 80/WP1) to SG16 Geneva meeting, Oct/Nov 2013 Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0080/en |
| H.248.DTLS Ed. 0.4 | Output (TD 80R1/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0080/en |
| H.248.DTLS Ed. 0.5 | Input (AVD-4476) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4476.zip |
| H.248.DTLS Ed. 0.6 | Output (TD-16) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-16.zip |
| H.248.DTLS Ed. 0.7 |  |
| H.248.DTLS Ed. 0.8 |  |
| H.248.DTLS Ed. 0.9 |  |
| H.248.DTLS Ed. 0.10 |  |

CONTENTS

Page

1 Scope 6 2 References 6 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 7 4 Abbreviations and acronyms 7 5 Conventions 8 5.1 Conventions used in signalling flows 8 5.2 DTLS endpoint notations 8 6 Use case descriptions 9 6.1 Use cases related to DTLS transport modes 9 6.2 Bearer connection network use cases with H.248 IP-IP gateways 10 6.2.1 High-level use case categories 10 6.2.2 Use case variations 11 6.3 Bearer connection network use cases with DTLS transport mode change 11 6.4 Bearer connection network use cases with multiparty services 11 7 Models 12 7.1 Network model from H.248 entity point of view 12 7.2 Bearer connection model 12 8 Basic session control package (for DTLS) 13 9 DTLS-specific stream endpoint interlinkage procedures 14 10 Capability negotiation package (for DTLS) 15 11 Session maintenance package (for DTLS) 15 12 Traffic volume metrics package (for DTLS) 15 Annex A State modelling for DTLS bearer connection endpoints 16 Annex B DTLS protocol layer: Data model 16 Appendix I Sample use cases of DTLS bearer encryption 16 I.1 Use cases for "application protocol agnostic DTLS handling" 16 I.2 Use cases for "DTLS-based transport security for facsimile packet relay service T.38" 18 I.3 Use cases for "WebRTC data traffic" 19 I.4 Use cases for "DTLS-based key exchange for SRTP" 21 Appendix II Signalling flows for basic DTLS session establishment and release 21 II.1 Overview 21 II.2 Conventions 21 II.3 Establishment of DTLS security sessions 21 II.3.1 Successful establishment – Terminating side 21 II.3.2 Successful establishment – Originating side 22 II.3.3 Unsuccessful establishment 23 II.4 Release of DTLS security sessions 23 II.4.1 Successful release – Terminating side 23 II.4.2 Successful release – Originating side 24 II.4.3 Unsuccessful release 25 Appendix III SDP for gateway control - IANA considerations 25 III.1 Purpose 25 III.2 SDP codepoints related to SDP "m=" line <proto> element 25 III.3 SDP codepoints related to SDP attributes 26 Bibliography 26 Appendix L Living List related to DTLS and DTLS-SRTP 27 L.1 DTLS protocol 27 L.2 DTLS related services 27 L.3 DTLS in scope of H.248.TLS? 27 L.4 DTLS use cases 27 L.4.1 Use cases from TLS-to-DTLS interworking perspective 27 L.4.2 Use cases from SRTP "key exchange" perspective 28 L.4.3 Two interpretations for "DTLS-SRTP"? 29 L.4.4 Use cases from WEBRTC perspective 30 L.4.5 Use cases from Telepresence perspective 30

List of Tables

Page

Table 1 – Basic session control package (for DTLS) 14 Table 2 – DTLS-specific stream endpoint interlinkage procedures 14 Table 3 – Capability negotiation package (for DTLS) 15 Table 4 – Session maintenance package (for DTLS) 15 Table 5 – Traffic volume metrics package (for DTLS) 16 Table III.1 – SDP codepoints related to SDP "m=" line <proto> element 25

List of Figures

Page

Figure 1 – Conventions for DTLS endpoint types 9 Figure 2 – DTLS transport modes 10 Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways 11 Figure 4 – Network model from H.248 entity point of view ("half call/bearer connection model") 12 Figure 5 – L4/IP protocol stack with DTLS protocol 13 Figure 6 – Two-termination context with a DTLS termination 13 Figure I.1 – Use cases for "application protocol agnostic DTLS handling" 17 Figure I.2 – Use cases for "DTLS-based transport security for facsimile packet relay service T.38" 19 Figure I.3 – Use cases for "WebRTC data traffic" 20 Figure II.1 – Successful establishment – Terminating side 22 Figure II.2 – Successful establishment – Originating side 23 Figure II.3 – Successful release – Terminating side 24 Figure II.4 – Successful release – Originating side 24 Figure L.1 – TLS/DTLS protocol historic 27 Figure L.2 – Example TLS-to-DTLS interworking 28 Figure L.3 – Media-path decoupled key exchange using SDP (via SIP & H.248) 28 Figure L.4 – Media-path coupled key exchange using DTLS 29 Figure L.5 – Interpretation "DTLS only used for SRTP key exchange" 29 Figure L.6 – Interpretation "Secure RTP traffic using DTLS" 30 Figure L.7 – DTLS use cases: "WebRTC data" & "WebRTC SRTP key exchange" 30

Draft new ITU-T Recommendation H.248.DTLS

Gateway control protocol: 
H.248 support for control of transport security using DTLS

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

Keywords

H.248, TLS, DTLS

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The DTLS protocol [b-IETF RFC 4347] and [IETF RFC 6347] is derived and thus aligned with the TLS protocol [IETF RFC 5246]. There are consequently many commonalities between the control of DTLS bearers and TLS bearers in H.248 gateways.

H.248-controlled TLS bearers are subject of [ITU-T H.248.TLS].

The purpose of this Recommendation is to define usage of [ITU-T H.248.TLS] for DTLS bearers.

In scope of this Recommendation are in particular

- description of DTLS specific use cases;

- description of MG bearer plane differences between DTLS and TLS;

- usage of TLS-defined H.248 packages for DTLS bearer types; and

- procedures for the specific application of DTLS-SRTP [IETF RFC 5764].

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.TLS]	Recommendation ITU-T H.248.TLS (2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.TLSPROF]	Recommendation ITU-T H.248.TLSPROF (2014), Gateway control protocol: Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles.

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 6347]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.2.

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	DTLS transparent forwarding: MG packet forwarding behaviour with the characteristic of DTLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a DTLS-PDU flow.

NOTE 1 – A DTLS PDU relates to a DTLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.1, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of DTLS message integrity in the context of "DTLS transparent forwarding". The MG might be DTLS aware; e.g. support of DTLS related statistics or event detection would not violate transparent forwarding behaviour.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| UDP | User Datagram Protocol |

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| DTLS Est.req DTLS Est.ack | Abstracted (DTLS message/procedure independent) representationfor establishment requests/acknowledgements of new DTLS security sessions. |
| DTLS Rel.req DTLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing DTLS security sessions. |

## 5.2	DTLS endpoint notations

The notion of endpoint represents different concepts in each network plane (H.248 as control protocol belongs to the network control plane and the MG bearer interface to the network user plane), see Figure 1:

Figure 1 – Conventions for DTLS endpoint types

Usage in

- control plane (here H.248): H.248 terminations/stream endpoint with DTLS processing are denoted as DTLS-enabled termination or stream endpoint (SEP) respectively;

- user plane (DTLS): a DTLS bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., DTLS protocol termination) plus a service access point (i.e., L4+ access).
Further, DTLS is a client/server type of protocol, a "DTLS endpoint" provides thus either a client or server role.

Whether the (H.248) DTLS-enabled SEP provides a "DTLS endpoint" is dependent on the H.248 connection model and enabled MG-internal interworking functions (see clauses 6 and 7).

{Editor's note (2014-03 meeting): it was commented that the "endpoint notations" may be simplified.}

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to DTLS transport modes

The DTLS protocol is independent of the underlying transport protocol stack. Multiple DTLS transport modes are possible (Figure 2).

Figure 2 – DTLS transport modes

This Recommendation is not dependent on any DTLS transport mode, but uses DTLS-over-UDP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-DTLS or different DTLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection without any applied transport security (abbreviated as "non-DTLS to non-DTLS");

- Use case #1.2: an end-to-end L4 connection traversing two network domains with different transport security policies ("DTLS", "no DTLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "DTLS to non-DTLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection with applied transport security (abbreviated as "DTLS to DTLS");

- Use case #1.4: there is end-to-end transport security, but different DTLS profiles (abbreviated as "DTLS to DTLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.2	Use case variations

#### 6.2.2.1	"DTLS-to-DTLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of DTLS transparent forwarding according clause 3.2.1.

## 6.3	Bearer connection network use cases with DTLS transport mode change

The L4 transport protocol could change, e.g., an H.248 MG enabled for DTLS/UDP to DTLS/DCCP interworking.

## 6.4	Bearer connection network use cases with multiparty services

A multipart communciation services leads to connection models with more than two H.248 terminations. Such gateway topologies provide normally application aware type of functions (e.g. as media server), thus DTLS sessions would be terminated by the MG. It results in a type of "DTLS to DTLS*" interworking, such as use case #1.4 (clause 6.2.1).

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 4) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a DTLS-capable IP host remote endpoint. Both DTLS endpoints span a network transport security domain (here DTLS domain).

NOTE  – There might be an additional network interface in case of dedicated, centralized network servers in the DTLS domain with respect to key distribution (such as key management systems). Such kind of interfaces are out of scope of this Recommendation.

Figure 4 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of DTLS level  mode(s) of operation in the H.248 MG; and

- configuration and procedures of the DTLS/L4/IP protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

Figure 5 provides the L4/IP protocol stack with the suite of DTLS protocols and their sub-layer organization.

Figure 5 – L4/IP protocol stack with DTLS protocol

Figure 6 details the generic connection-model where a DTLS-enabled termination is connected to a single other termination (either DTLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 6 – Two-termination context with a DTLS termination

# 8	Basic session control package (for DTLS)

See Table 1:

Table 1 – Basic session control package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS basic session control package" (tlsbsc) according to [ITU-T H.248.TLS] shall be used for DTLS. |  |
| --- | --- | --- |
| H.248: DTLS specific differences versus TLS? | No. The same state model applies, see Annex A. The DTLS protocol internal procedural differences are not visible from H.248 interface perspective, with regards to the basic blocking, establishment and release of a DTLS session. |  |
| Limitations, modifications, special considerations? |  | None. |  |
| Further comments? |  | FIXTHIS … we should cross-check possible final unsuccessful DTLS session establishment in IP networks with high IPLR (thus, DTLS retransmissions, timer expirations, …) |  |

# 9	DTLS-specific stream endpoint interlinkage procedures

See Table 2:

Table 2 – DTLS-specific stream endpoint interlinkage procedures

| Reusable H.248 package? | Yes, the "Stream endpoint interlinkage package" (seplink) according to [ITU-T H.248.SEPLINK] shall be basically used for DTLS, too. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | Yes. - DTLS transport modes (see clause 6.1): The interlinkage between DTLS and the underlying L4 protocol is limited, actually L4 protocol dependent (due to support of connectionless (UDP) and connection-oriented (SCTP, DCCP) L4 protocols). - FIXTHIS … any other differences? … |
| Limitations, modifications, special considerations? | FIXTHIS. |
| Further comments? | None. |

{Editor's note (2014-03 meeting): there were comments concerning interlinkage => there are two basic "intra-SEP interlinkage" variants, i.e.,following  four direction-specific interlinkage options:

| Intra-SEP interlinkage options for DTLS: |  |  |
| --- | --- | --- |
| No. |  |  |  | <source transport EP> |  |  | <interlinked tranport EP> |  |  |
| 1a |  |  |  | DTLS |  |  | lower layer EP |  |  |
| 1b |  | lower layer EP |  |  | DTLS |  |  |
| 2a |  | DTLS |  |  | higer layer EP |  |  |
| 2b | higer layer EP | DTLS |

Above Table should take this into consideration.}

# 10	Capability negotiation package (for DTLS)

See Table 3:

Table 3 – Capability negotiation package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS capability negotiation package" (tlscn) according to [ITU-T H.248.TLS] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. The DTLS protocol internal procedural differences are not visible from H.248 interface perspective. |
| Limitations, modifications, special considerations? | FIXTHIS. |
| Further comments? | FIXTHIS … we still need to cross-check the DTLS extensions for DTLS-SRTP…) |

# 11	Session maintenance package (for DTLS)

See Table 4:

Table 4 – Session maintenance package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS session maintenance package" (tlsm) according to [ITU-T H.248.TLS] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. There might be different alerts, but the DTLS protocol internal procedural differences does not impact the H.248 package design. |
| Limitations, modifications, special considerations? | FIXTHIS |
| Further comments? | FIXTHIS … |

# 12	Traffic volume metrics package (for DTLS)

See Table 5:

Table 5 – Traffic volume metrics package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS traffic volume metrics package" (tlstv) according to [ITU-T H.248.TLS] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. The TLS data model could be reused for DTLS (see Annex B). |
| Limitations, modifications, special considerations? | None. |
| Further comments? | None. |

Annex A

State modelling for 
DTLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

The same state model as for TLS applies for DTLS, see Annex A.3/[ITU-T H.248.TLS].

Annex B

DTLS protocol layer: Data model

(This annex forms an integral part of this Recommendation.)

The same data model as for TLS applies for DTLS, see Annex B/[ITU-T H.248.TLS].

Appendix I

Sample use cases of DTLS bearer encryption

(This appendix does not form an integral part of this Recommendation.)

This Appendix illustrates some network level scenarios that employ DTLS bearer encryption.

## I.1	Use cases for "application protocol agnostic DTLS handling"

The MG is unaware of the IP application protocol carried by DTLS packets. There are some basic interworking modes (see Figure I.1), - from perspective of UDP- and TCP.based data transport:

- use case #I.1.1: MG provides interworking between "DTLS/UDP/IP" to "TCP/IP";

- use case #I.1.2: MG provides interworking between "DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.1.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

Figure I.1 – Use cases for "application protocol agnostic DTLS handling"

## I.2	Use cases for "DTLS-based transport security for facsimile packet relay service T.38"

## The IP application protocol is given by T.38 [b-ITU-T T.38], which relates to the Internet Facsimile Protocol (IFP) plus an underlying application level framing protocol such as UDPTL (Facsimile UDP Transport Layer).

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

There are two principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.2.1: MG provides interworking between "DTLS/UDP/IP" to "UDP/IP"; and

- use case #I.2.2: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode ("thus, similar as use case #I.1.3").

It should be noted that the MG is not aware of the IP application protocol, hence also not aware of the "UDPTL" protocol here. Hence, use cases #I.2.2 and #I.1.3 should be identical from H.248 control perspective.

Figure I.2 – Use cases for "DTLS-based transport security 
for facsimile packet relay service T.38"

## I.3	Use cases for "WebRTC data traffic"

Background:

WebRTC relates to a multimedia application inclusive a data component. The WebRTC data is using "SCTP/DTLS/UDP" transport.

The WebRTC data traffic related DTLS use cases provide further characteristics in addition to the use cases in clauses I.1 and I.2:

- hierarchical protocol stack layering by using a secured tunnel (given by SCTP/DTLS);

- multiplexing model because a single DTLS security session could be shared by multiple WebRTC data channels and

- communication between various WebRTC endpoint types leads to UDP-to-UDP and UDP-to-TCP interworking support by H.248 MGs.

There are three principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.3.1: MG provides interworking between "SCTP/DTLS/UDP/IP" to "TCP/IP";

- use case #I.3.2: MG provides interworking between " SCTP/DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.3.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

It should be noted that the MG is not aware again of the application protocol itself. However, the main difference is the embedded "SCTP stream/SCTP association" layering in comparison to clause I.1.

Figure I.3 – Use cases for "WebRTC data traffic"

## I.4	Use cases for "DTLS-based key exchange for SRTP"

TBD

Appendix II

Signalling flows for 
basic DTLS session establishment and release

(This appendix does not form an integral part of this Recommendation.)

## II.1	Overview

The tlsbsc package (clause 8/[ITU-T H.248.TCP]) defines the basic TLS security session control with scope on support for establishment and release. Basic DTLS session control reuses the solution defined by this package.

## II.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (DTLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local DTLS session endpoint.

It has to be noted that some DTLS specifics are not visible at the considered level of abstraction, such as

- Bearer type indication (via SDP "m=" line) when creating a DTLS-enabled SEP / Termination; and

- DTLS timeouts and retransmissions (during e.g. establishment handshake procedures) (i.e., related to [IETF RFC 6347]/Figure 3 "DTLS Timeout and Retransmission State Machine").

## II.3	Establishment of DTLS security sessions

### II.3.1	Successful establishment – Terminating side

See Figure II.1, termed as use case (E.1):

Figure II.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of DTLS session establishment (1).

- Early incoming DTLS messages are discarded by the MG (2).

- The MGC unblocks the DTLS SEP (3).

- DTLS session establishment (5 to 7); MG acting as DTLS server.

- Optional notification of MGC concerning available DTLS session for application data transfer (8), if subscription to event by MGC (4).

### II.3.2	Successful establishment – Originating side

See Figure II.2, use case (E.2).

Figure II.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause II.3.1

- DTLS session establishment (5 to 7), triggered by MGC (4); MG acting as DTLS client.

- Optional notification of MGC concerning available DTLS session for application data transfer (9), if subscription to event by MGC (8).

### II.3.3	Unsuccessful establishment

The DTLS security session negotiation handshake is basically not completed.

## II.4	Release of DTLS security sessions

### II.4.1	Successful release – Terminating side

See Figure II.3, termed as use case (R.1).

Figure II.3 – Successful release – Terminating side

More details for this example:

- MG receives an incoming DTLS close_notify alert (2), the indication for DTLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released DTLS session (4), if subscription to event by MGC (1).

### II.4.2	Successful release – Originating side

See Figure II.4, use cases (R.2) illustrates an outgoing DTLS session release:

Figure II.4 – Successful release – Originating side

More details for this example:

- MGC initiates the DTLS session release (1).

- MG sends an outgoing DTLS close_notify alert (2), the indication for DTLS session release to wards remote DTLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released DTLS session (5), if subscription to event by MGC (4).

### II.4.3	Unsuccessful release

An unsuccessful DTLS session release procedure would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established DTLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

Appendix III

SDP for gateway control - IANA considerations

(This appendix does not form an integral part of this Recommendation.)

## III.1	Purpose

The H.248 gateway control protocol reuses normally the SDP codepoints as defined by the IETF or other organizations. However, gateway control may demand for additional SDP codepoints. The ones idendified by this Recommendation are collected in this clause and are subject of registration with IANA.

## III.2	SDP codepoints related to SDP "m=" line <proto> element

This Recommendation identifies following bearer type indication requirements (Table III.1):

Table III.1 – SDP codepoints related to SDP "m=" line <proto> element

| Type | SDP name (value) | Use case | IANA status |
| --- | --- | --- | --- |
| proto | "DTLS" | application agnostic, transport protocol independent bearer type indication | not yet registered, but use case on scope of this Recommendation related to "SEP interlinkage" |
| proto | "UDP/DTLS" "DCCP/DTLS" | application agnostic, transport protocol specific bearer type indication | not yet registered, but use cases in scope of this Recommendation |
| proto | "SCTP/DTLS" |  | already available |
| proto | "DTLS/SCTP" | application agnostic, "DTLS tunnel" mode | already available |
| proto | "UDP/TLS/<application>" "DCCP/TLS/<application>" (NOTE 1) | application aware bearer type indication | some application specific codepoints are already supported |
| NOTE 1 – The (unfortunate) notion of "TLS" should not confuse: the combination of TLS with a specific L4 protocol type implies "DTLS-over-L4". |  |  |  |

There are additional SDP codepoints, which need to be registered with IANA.

## III.3	SDP codepoints related to SDP attributes

None.

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

[b-ITU-T V.153]	Recommendation ITU-T (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

Appendix L
Living List related to DTLS and DTLS-SRTP

(This appendix does not form an integral part of this Recommendation)

## L.1	DTLS protocol

DTLS is derived from TLS, could be thus considered as a single security protocol family. DTLS versioning numbers are (somehow) linked to TLS protocol versions.

Figure L.1 – TLS/DTLS protocol historic

## L.2	DTLS related services

There are two slightly different applications:

- DTLS as "transport security” means for UDP/IP bearer traffic

- DTLS as "key exchange” means for RTP/UDP/IP bearer traffic using media security according SRTP

## L.3	DTLS in scope of H.248.TLS?

There are basically following options:

- DTLS covered by H.248.TLS?

- DTLS in a self-contained, new H.248.DTLS?

- DTLS added to H.248.77 due to link with DTLS-SRTP?

- DTLS covered by H.248.WEBRTC due usage by WebRTC service.

Option #2 (or #1) seems to be preferable.

## L.4	DTLS use cases

Initial list of possible use cases:

### L.4.1	Use cases from TLS-to-DTLS interworking perspective

Figure L.2 provides an interworking example (here: DTLS based secure WebRTC data service).

Figure L.2 – Example TLS-to-DTLS interworking

### L.4.2	Use cases from SRTP "key exchange" perspective

SRTP-based media encryption, - there are many key exchange options for SRTP. For example the two outlined in the sub-clauses below.

#### L.4.2.1	Media-path decoupled key exchange using SDP (via SIP & H.248)

Figure L.3 recalls again the protocol stack for SDES-based key exchange for SRTP, as in scope of [b-ITU-T H.248.77] or as part of the 3GPP H.248 Iq profile ([b-ETSI TS 129 334]):

Figure L.3 – Media-path decoupled key exchange using SDP (via SIP & H.248)

#### L.4.2.2	Media-path coupled key exchange using DTLS

Figure L.4 illustrates the alternative of a DTLS-based key exchange for SRTP:

Figure L.4 – Media-path coupled key exchange using DTLS

### L.4.3	Two interpretations for "DTLS-SRTP"?

[b-IETF RFC 5764]:

This document describes DTLS-SRTP, a SRTP extension for DTLS that

combines the performance and encryption flexibility benefits of SRTP

with the flexibility and convenience of DTLS-integrated key and

association management.  DTLS-SRTP can be viewed in two equivalent

ways: as a new key management method for SRTP, and a new RTP-specific

data format for DTLS.

I.e., there are two possible interpretations from protocol stack point of view, see Figures L.5 and L.6.

Figure L.5 – Interpretation "DTLS only used for SRTP key exchange"

Figure L.6 – Interpretation "Secure RTP traffic using DTLS"

It has to be noted that these interpretations might be incorrect. To be confirmed.

### L.4.4	Use cases from WEBRTC perspective

DTLS use cases from WebRTC perspective.

Figure L.7 – DTLS use cases: "WebRTC data" & "WebRTC SRTP key exchange"

In Figure L.7:

- WebRTC data traffic

- WebRTC DTLS-based key exchange for SRTP

### L.4.5	Use cases from Telepresence perspective

Are there any? TBD

__________________