| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-24 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur Q2/16 |  |  |  |
| Title: | Rec. ITU-T X.1303bis Necessitates Revision to H.323 Annex M5 |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

ITU-T SG17 and OASIS revised the CAP specification from version 1.1 to 1.2.  The current H.323 Annex M5 can transport the newly approved text, but needs to be modified in order to allow proper identification of the new version.

Attached to this TD is the Rec. ITU-T X.1303 specification that was just approved.

NOTE – Rec. ITU-T X.1303 (2007), Common alerting protocol (CAP 1.1), and Rec. ITU-T X.1303bis (2014), Common alerting protocol (CAP 1.2), will coexist.

On the following pages are changes believed to be necessary to support ITU-T X.1303bis.  These changes should be discussed and a formal contribution submitted to the next SG16 meeting.

Annex M5

Tunnelling of common alerting protocol (CAP) messages in H.323

(This annex forms an integral part of this Recommendation)

## M5.1	Scope

The purpose of this annex is to give guidance on how the generic tunnelling mechanism described in clause 10.4 can be used to tunnel common alerting protocol (CAP) messages [M5-1 and M5-2] over H.323 networks. This annex does not define the actions a receiving device should take after receiving a CAP message since such actions will be dependent on the type of message received, physical characteristics of the H.323 device, the intended recipients, and other considerations which are outside the scope of a signalling specification.

## M5.2	Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[M5-1]		Recommendation ITU-T X.1303 (2007), Common alerting protocol (CAP 1.1).

[M5-2]		Recommendation ITU-T X.1303bis (2014), Common alerting protocol (CAP 1.2).

## M5.3	Identifying CAP messages

Endpoints that support tunnelling CAP messages shall use the procedures defined in clause 10.4. Endpoints shall identify CAP messages by using as the tunnelledProtocolID the OBJECT IDENTIFIER (OID) with one of the values as shown below:

{itu-t(0) recommendation(0) x(24) 1303 version1-1(1)}

{itu-t(0) recommendation(0) x(24) 1303 version1-2(2)}

The value of the OID shall properly identify the version of the CAP protocol transmitted, as determined by the last digit in the OID.

Since CAP is defined in both ASN.1 and XML, the subIdentifier field shall be used to distinguish between the two encoding methods. When CAP is present in XML format, the subIdentifier field shall be set to all uppercase letters "XML". When CAP is present in PER unaligned format as described in [M5-1 and M5-2], the subIdentifier field shall be set to all uppercase letters "PER".

NOTE – H.225.0 utilizes the aligned variant of PER, while CAP utilizes the unaligned variant.

## M5.4	Signalling support for CAP

An endpoint shall indicate which encoding formats it supports via the supportedTunnelledProtocols field of endpointType. This shall be included both in RAS and H.225.0 call signalling messages where endpointType is present.

Since PER encoding is more efficient than XML encoding, devices should utilize PER encoding of CAP messages, especially within network environments where bandwidth constraints might inhibit or reduce the quality of transmission of CAP messages.

## M5.5	Transporting CAP within H.225.0 messages

The content of a single CAP message shall be wholly contained in a single OCTET STRING and placed in the messageContent element of tunnelledSignallingMessage. Multiple CAP messages may be transmitted within a single H.225.0 message, with each CAP message stored in a separate OCTET STRING within the messageContent field.

## M5.6	Transmitting and processing CAP messages

Devices that support CAP shall accept and process CAP messages received in any H.225.0 message. When a call is initiated for the primary or sole purpose of delivering a CAP message, the CAP message should be placed in the Setup message. Since only a single protocol may be tunnelled within a single H.225.0 message, it may be necessary for an intermediary device to hold the CAP message for later transmission. For this reason, a called device (such as an automated alerting device that only responds to CAP messages) should provide reasonable time for the calling device to deliver a CAP message and shall not assume CAP messages will only be present in Setup.

CAP messages should not be transmitted in a Release Complete message, as they would be subject to being discarded. Further, CAP messages should not be placed in H.225.0 messages that do not have end-to-end significance, such as Call Proceeding.

Intermediary devices that support CAP shall ensure that CAP messages are transmitted in the order in which they were received and should transmit CAP messages as quickly as possible, utilizing Facility messages as necessary in order to reduce the time required to deliver the CAP message.

## M5.7	Requesting CAP support when initiating calls

An endpoint may request calls to be routed to CAP-enabled devices by including the CAP protocol identifiers within the desiredTunnelledProtocol field of the ARQ or LRQ message. For example, this could enable a Gatekeeper to select a CAP-enabled text-to-speech Gateway for delivering important messages to users on the PSTN.

## M5.8	Gatekeeper procedures

When requested by an endpoint via the desiredTunnelledProtocol field in an ARQ or LRQ, a Gatekeeper should make every effort to identify a terminating endpoint that supports CAP. When a Gatekeeper transmits an LRQ message to another Gatekeeper, the desiredTunnelledProtocol field shall be conveyed to the other Gatekeeper. The receiving Gatekeeper shall likewise try to identify an endpoint that supports CAP.

If possible, Gatekeepers that route call signalling shall forward received CAP messages within the H.225 message that would normally be transmitted in response to the H.225 message that contained the CAP message. For example, if a CAP message is received in a Setup message, the Gatekeeper should transmit the CAP message in an outgoing Setup message. Since this is not always possible, Gatekeepers may find it necessary to hold the CAP message for a period of time and transmit it via a Facility message.

Gatekeepers shall ensure that received CAP messages are forwarded in the order in which they were received and should transmit CAP messages as quickly as possible, utilizing Facility messages as necessary in order to reduce the time required to deliver the CAP message.

__________________