| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 217 (PLEN) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June - 11 July 2014 |
| TD |  |  |
| Source: | Editor H.248.90 |  |
| Title: | Consent: H.248.90 (ex H.248.TLS) "Gateway control protocol: H.248 packages for control of transport security using TLS" (New) |  |

This document contains draft new H.248.90 (ex H.248.TLS) "Gateway control protocol: H.248 packages for control of transport security using TLS" for Consent.

NOTE – A.5 justification in TD 207/Plen.

AAP Summary

This Recommendation provides multiple H.248 packages for support of TLS, complemented by models, considerations of package mode operations and signalling flows.

CONTENTS

Page

1 Scope 11 1.1 Applicability statements 12 2 References 12 3 Definitions 13 3.1 Terms defined elsewhere 13 3.2 Terms defined in this Recommendation 13 4 Abbreviations and acronyms 14 5 Conventions 16 5.1 Conventions used in signalling flows 16 5.2 TLS endpoint notations 16 5.3 Important notation derived from TLS presentation language 17 6 Use case descriptions 17 6.1 Use cases related to TLS transport modes 17 6.2 Bearer connection network use cases with H.248 IP-IP gateways 18 6.2.1 High-level use case categories 18 6.2.2 Use case variations 19 6.3 Bearer connection network use cases with H.248 "TCP to non-TCP" gateways 20 6.4 Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway) 20 7 Models 20 7.1 Network model from H.248 entity point of view 20 7.2 Bearer connection model 21 8 TLS basic session control package 22 8.1 Properties 22 8.1.1 Incoming security session establishment blocking 22 8.2 Events 24 8.2.1 TLS session state change ("BNC change") 24 8.3 Signals 25 8.3.1 Establish (BNC) 25 8.3.2 Release (BNC) 25 8.4 Statistics 25 8.5 Error Codes 26 8.6 Procedures 26 8.6.1 TLS endpoint creation 26 8.6.2 TLS endpoint role assumption 26 8.6.3 TLS security session establishment 27 8.6.4 TLS application data transfer 27 8.6.5 TLS session release 28 9 TLS-specific stream endpoint interlinkage procedures 31 9.1 Introduction 31 9.2 Procedures 31 9.2.1 Introduction 31 9.2.2 TLS security session establishment 32 9.2.3 TLS security session release 33 9.2.4 Unsuccessful interlinkage 34 10 TLS capability negotiation package 34 10.1 Properties 35 10.1.1 TLS Domain Profile Identifier 35 10.1.2 TLS Versions 35 10.1.3 Cipher Suites 36 10.1.4 Compression Methods 36 10.1.5 Support for Renegotiation of the Security Context 37 10.1.6 Renegotiation Period 37 10.1.7 Client Authentication Required 37 10.2 Events 38 10.3 Signals 38 10.4 Statistics 38 10.5 Error codes 38 10.6 Procedures 38 10.6.1 Principles of TLS capability negotiations – overview and scope 38 10.6.2 General default behaviour (MGC does not influence TLS capability negotiations) 38 10.6.3 MGC-controlled TLS capability negotiations: Options 39 10.6.4 Determining TLS related capabilities (TLS MG profile) 39 10.6.5 Indication of TLS profile(s) to MG by MGC 40 10.6.6 Indication of individual TLS protocol capabilities to MG by the MGC 40 10.6.7 Auditing the actual TLS configuration (TLS endpoint profile) 42 11 TLS session maintenance package 42 11.1 Properties 42 11.2 Events 42 11.2.1 TLS Error Alert 42 11.3 Signals 44 11.3.1 MGC triggered TLS error alert 44 11.4 Statistics 44 11.5 Error codes 45 11.6 Procedures 45 11.6.1 TLS warning error alerts 45 11.6.2 TLS fatal error alerts 45 11.6.3 Lower layer issues 46 12 TLS traffic volume metrics package 46 12.1 Properties 47 12.2 Events 47 12.3 Signals 47 12.4 Statistics 47 12.4.1 Number of Received Application Data TLS-Fragments 47 12.4.2 Number of Sent Application Data TLS-Fragments 47 12.4.3 Number of Received TLS Message Fragments 48 12.4.4 Number of Sent TLS Message Fragments 48 12.4.5 Number of Received Application Data Octets 48 12.4.6 Number of Sent Application Data Octets 49 12.4.7 Number of Received TLS Message Octets 49 12.4.8 Number of Sent TLS Message Octets 50 12.4.9 Number of Received Application Data TLS-PDU Octets 50 12.4.10 Number of Sent Application Data TLS-PDU Octets 50 12.4.11 Number of Received TLS Message PDU Octets 51 12.4.12 Number of Sent TLS Message PDU Octets 51 12.4.13 Number of Received Application Data Octets, Protected and Compressed 52 12.4.14 Number of Sent Application Data Octets, Protected and Compressed 52 12.4.15 Number of Received TLS Message Octets, Protected and Compressed 53 12.4.16 Number of Sent TLS Message Octets, Protected and Compressed 53 12.4.17 Number of Octets of Compressed Application Data Fragments in ingress Direction 53 12.4.18 Number of Octets of Compressed Application Data Fragments in egress Direction 54 12.4.19 Number of Octets of Compressed TLS Message Fragments in ingress Direction 54 12.4.20 Number of Octets of Compressed TLS Message Fragments in egress Direction 55 12.5 Error codes 55 12.6 Procedures 56 12.6.1 General information 56 12.6.2 Number of Application Data TLS-Fragments 58 12.6.3 Number of TLS Message Fragments 58 12.6.4 Number of Application Data Octets 58 12.6.5 Number of TLS Message Octets 58 12.6.6 Number of Application Data TLS-PDU Octets 58 12.6.7 Number of TLS Message PDU Octets 58 12.6.8 Number of Application Data Octets, Protected and Compressed 58 12.6.9 Number of TLS Message Octets, Protected and Compressed 58 12.6.10 Number of Octets of Compressed Application Data Fragments 58 12.6.11 Number of Octets of Compressed TLS Message Fragments 58 13 Package-less TLS control 58 13.1 Related to TLS session establishment 59 13.1.1 Overview – principal H.248 control steps 59 13.1.2 Step 1: Bearer endpoint creation 62 13.1.3 Step 2: Bearer type indication for IP transport "L4/L3" 62 13.1.4 Step 3: Block / Unblock L4 bearer control procedures at SEP 62 13.1.5 Step 4: Start IP transport connection establishment procedure 62 13.1.6 Step 5: Local L4/L3 NAT traversal support 63 13.1.7 Step 6: Reporting successfully established IP transport connection 63 13.1.8 Step 7: Bearer type indication for TLS 63 13.1.9 Step 8: Block / Unblock TLS security session control procedures at SEP 63 13.1.10 Step 9: Start TLS security session establishment procedure 63 13.1.11 Step 10: Local L4+ NAT traversal support 63 13.1.12 Step 11: Reporting successfully established TLS session 63 13.1.13 Step 12: Active communication phase 63 13.2 Related to TLS authentication 63 13.3 Related to TLS session release 64 14 The TLS profile concept 64 14.1 TLS protocol profiles 64 14.2 Illustration of TLS profile concept 65 14.3 Example for the TLS MG profile concept 66 15 Security considerations 67 A.1 Introduction and purpose 68 A.2 Original state model for TLS session endpoints 68 A.3 Simplified state model for H.248-based TLS basic session control 68 A.3.1 Transitioning from Idle to Established 69 A.3.2 Transitioning from Established to Idle 69 B.1 Motivation 70 B.2 Data model 70 B.3 Terminology based on data model 73 I.1 Use case #I.1 "Terminal-to-MG TLS session, provisioned TLS service negotiation" 74 I.2 Use case #I.2 "Terminal-to-MG TLS session, MGC-controlled TLS service negotiation" 75 I.3 Use case #I.3 "WebRTC to NGN/IMS interworking function with DTLS-to-TLS support" 75 I.4 Use case #I.4 "TLS-based transport security for facsimile packet relay service T.38" 75 II.1 TLS to non-TLS interworking with TCP as example bearer type 76 II.1.1 Initial session setup using SDP security descriptions in application control signalling 76 II.1.2 Traffic flow examples for communication establishment phase 77 II.1.3 Traffic flow examples for communication release phase 81 III.1 Typical Internet TLS profile 84 III.2 3GPP TLS domain profile 85 III.3 OMA TLS domain profiles 87 III.3.1 OMA public key TLS domain profile 87 III.3.2 OMA pre-shared key TLS domain profile 88 III.4 IETF Minimum TLS domain profile 88 III.5 IETF example of a national TLS domain profile 89 III.6 ITU-T TLS domain profile for NGN signalling and management plane 89 IV.1 Overview 89 IV.2 Conventions 89 IV.3 Establishment of TLS security sessions 89 IV.3.1 Successful establishment – Terminating side 89 IV.3.2 Successful establishment – Originating side 90 IV.3.3 Unsuccessful establishment 91 IV.4 Release of TLS security sessions 91 IV.4.1 Successful release – Terminating side 91 IV.4.2 Successful release – Originating side 92 IV.4.3 Unsuccessful release 93 V.1 Overview 93 V.2 Conventions 93 V.3 Usage of SEPP Interlinkage 93 V.4 Usage of Protocol Layers Interlinkage 93 V.4.1 Without intra-SEP interlinkage 94 V.4.2 Using intra-SEP interlinkage 94 VI.1 Background 99 VI.2 H.248 gateway framework concerning TLS alert handling 99 VI.3 TLS error alerts categorization 100 VI.4 Summary 100 VII.1 Introduction 100 VII.2 Brief summary of TLS session resumption 100 VII.3 Example use cases 101 VII.3.1 Use case: "File transfer session initiated while a chat session is active" 101 VII.3.2 Use case: "Resumption of an interrupted file transfer" 104 VII.4 Conclusions and guidelines 106

List of Tables

Page

Table 1 – Principal TLS-based interfaces of H.248 entities and their relevance for this Recommendation 12 Table 2 – Security session establishment preparedness (of TLS endpoints) 23 Table 3 – Parameter values 24 Table 4 – TLS endpoint role assumption (at establishment phase) 26 Table 5 – TLS security session establishment - Results 27 Table 6 – Impact of StreamMode on TLS bearer traffic at external MG interface 27 Table 7 – Interlinkage profiling for TLS bearer 31 Table 8 – TLS security session establishment – SEPP interlinkage enabled 32 Table 9 – TLS security session establishment – Protocol layers interlinkage enabled 32 Table 10 – TLS security session release – SEPP interlinkage enabled 33 Table 11 – TLS security session establishment – Protocol layers interlinkage enabled 33 Table 12 – Variants of MGC-controlled TLS capability negotiations 39 Table 13 – Overview of performance metrics – All H.248 statistics 56 Table 14 – Internal measurement points (MP), traffic types and conditional usage 57 Table 15 – H.248 control steps and possible control elements 61 Table 16 – Analogous terms between H.248.80 media configurations and TLS profile types 65 Table 17 – Example of a TLS MG profile 66 Table 18 – Example of an intersection between a TLS domain profile and a TLS MG profile 67 Table B.1 – TLS presentation language and derived term 73 Table III.1.a – Example of a TLS domain profile – Main part 84 Table III.1.b – Example of a TLS domain profile – Cipher Suites 85 Table III.2.a – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Main part 86 Table III.2.b – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Cipher Suites 87 Table III.3.a – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Main part 87 Table III.3.b – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites 88

List of Figures

Page

Figure 1 – Conventions for TLS endpoint types 16 Figure 2 – TLS transport modes 17 Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways 19 Figure 4 – Network model from H.248 entity point of view ("half call/bearer connection model") 21 Figure 5 – TCP/IP protocol stack with TLS protocol 21 Figure 6 – Two-termination context with a TLS termination 22 Figure 7 – Principle stimuli for triggering TLS session release 29 Figure 8 – Principal H.248 control steps for establishing a TLS-over-L4 bearer 60 Figure 9 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile 65 Figure 10 – Illustration of TLS profile concept 66 Figure A.1 – Simplified state model for H.248-based TLS basic session control 69 Figure B.1 – Performance monitoring – Measurement points related to traffic volume specific performance metrics 71 Figure B.2 – Data processing in the TLS record layer 72 Figure B.3 – TLS-RL PDU: variable layout of a protected compressed TLS fragment 73 Figure I.1 – Use-case # I.1: Bearer path TLS security between UE and MG 75 Figure II.1 – Configuration for the example call flow 76 Figure II.2 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)) 78 Figure II.3 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment (decoupled end-to-end L4 connection)) 79 Figure II.4 – Example setup of a TLS to non-TLS IWF (TLS establishment decoupled from L4 establishment, but interlinked end-to-end L4 connection establishment) 80 Figure II.5 – Example setup of a TLS to non-TLS IWF (combined TLS/L4 establishment, plus interlinked end-to-end L4 connection establishment) 81 Figure II.6 – Example TLS session release, initiated by remote TLS endpoint (MGC enforces immediate L4 connection release) 82 Figure II.7 – Example TLS session release, local TLS/L4 decoupled, but coupled end-to-end L4 connection release) 83 Figure II.8 – Example TLS session release, coupled TLS/L4 and end-to-end L4 connection release 84 Figure IV.1 – Successful establishment – Terminating side 90 Figure IV.2 – Successful establishment – Originating side 91 Figure IV.4 – Successful release – Terminating side 92 Figure IV.5 – Successful release – Originating side 92 Figure V.1 – Illustration of Protocol Layers Interlinkage scenario without intra-SEP interlinkage 94 Figure V.2 – Illustration of Protocol Layers Interlinkage property scenario with intra-SEP interlinkage between TLS and L4 94 Figure V.3 – Correct "TLS-L4" interlinkage configuration for establishment – L4 incoming bearer establishment request 95 Figure V.4 – Correct "TLS-L4" interlinkage configuration for establishment, butTLS incoming bearer establishment request first 95 Figure V.5 – Correct "TLS-L4" interlinkage configuration for establishment – L4 stimuli 96 Figure V.6 – Correct "TLS-L4" interlinkage configuration for establishment, but incorrect – TLS stimuli 96 Figure V.7 – Correct "TLS-L4" interlinkage configuration for release, but L4 incoming bearer release request first 97 Figure V.8 – Correct "TLS-L4" interlinkage configuration for release – TLS incoming bearer release request 97 Figure V.9 – Correct "TLS-L4" interlinkage configuration for release, but L4 stimuli first 98 Figure V.10 – Correct "TLS-L4" interlinkage configuration for release TLS stimuli 98 Figure VII.1 – MSRP-based instant messageing – Established chat session between two UEs 102 Figure VII.2 – Parallel MSRP/TLS sessions for chat and file transfer between two UEs 102 Figure VII.3 – End-to-access edge (e2ae) security model: potential TLS session resumption 103 Figure VII.4 – Scenario for TLS session resumption for the file transfer 104 Figure VII.5 – Scenario for resuming an interrupted file transfer 105

Draft new ITU-T Recommendation H.248.TLS

Gateway control protocol: 
H.248 packages for control of transport security using TLS

Summary

Transport Layer Security (TLS) is a session layer protocol for securing IP transport protocols. TLS bearer plane traffic could be terminated or forwarded by ITU-T H.248 media gateways. This Recommendation provides multiple H.248 packages for support of TLS, including the establishment, negotiation, release and performance monitoring of TLS sessions, complemented by models, considerations of package mode operations and signalling flows.

# 1	Scope

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two IP transport connection endpoints. This Recommendation defines in general H.248 signalling elements for support of TLS in various options.

This Recommendation provides in more detail information about:

- basic exchange architecture of keying information between control plane (including H.248 interface) and bearer plane;

- negotiation aspects: indication and determination of cryptographic capabilities between TLS endpoints;

- minimum amount of information carried by H.248 for establishing H.248 TLS/L4 terminations (NOTE 1);

NOTE 1 – Some information could be provisioned via management and there is also TLS information exchanged via the (L4) bearer interface with the remote TLS endpoint.

- TLS procedures in detail at the various TLS sublayers, i.e. for support of the

  - TLS record protocol

  - TLS handshake protocol

  - TLS change cipher spec protocol

  - TLS alert protocol

  - TLS application data protocol

- profiling of TLS services;

- specifying a set of cipher suites;

- consideration of H.248 MG modes of operation and connection models; and

- SDP- versus Property-based TLS endpoint control;

- The scope of this Recommendation is limited to the TLS protocol (the DTLS partner protocol is subject of another Recommendation).

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would typically bespecified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.8]	Recommendation ITU-T H.248.8 (03/2013), Gateway control protocol: Error code and service change reason description.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (2013), Gateway control protocol: Usage of the revised SDP offer / answer model with H.248.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (11/2013), Gateway control protocol: RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations.

[ITU-T H.248.89]	Recommendation ITU-T H.248.89 (07/2014), Gateway control protocol: TCP support packages.

[ITU-T H.248.91]	Recommendation ITU-T H.248.91 (07/2014), Gateway control protocol: Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles.

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T X.200]	Recommendation ITU-T X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol.

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP).

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 6714]	IETF RFC 6714 (08/2012), Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Application data (TLS): the application layer data, as processed and transported by the underlying TLS protocol layer. An application layer service data unit correlates to a TLS-RL user data unit (clause 3.2.12)

3.2.2	TLS client [IETF RFC 5246]: the TLS entity that initiates a TLS security session to a TLS server [IETF RFC 5246].

NOTE – The primary operational difference between the TLS server and TLS client is that the TLS server is generally authenticated, while the TLS client is only optionally authenticated.

3.2.3	TLS domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.4	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.5	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.6	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.7	TLS fragment: the protocol data unit given by data element "fragment" as defined within the superior TLS protocol data structure "TLSPlaintext" according clause 6.2.1/[IETF RFC 5246].

3.2.8	TLS message: a unit of TLS-RL user data (clause 3.2.12) as produced by the TLS handshake protocol, the TLS alert protocol or the TLS change cipher spec protocol.

3.2.9	TLS MG profile: The TLS profile supported by the MG.

3.2.10	TLS profile: A selection of options from a set of TLS related protocol parameters and procedures.

NOTE – Also known as (nonstandard term) TLS protocol profile (example: the TLS profile defined by Annex E/[b-ETSI TS 133 310]).

3.2.11	TLS server [IETF RFC 5246]: the TLS entity that responds to requests for security sessions from TLS clients [IETF RFC 5246].

3.2.12	TLS-RL user data: the block of information exchanged between the TLS Record Layer and the four higher layer protocols (NOTEs 1, 2). A unit TLS-RL user data represents therefore either a TLS message (clause 3.2.8) or a unit of (TLS) application data (clause 3.2.1).

NOTE 1 – Refers to the four protocols that use the record protocol according clause 6/[IETF RFC 5246]: "… the handshake protocol, the alert protocol, the change cipher spec protocol, and the application data protocol".

NOTE 2 – Clause 6.2.1/[IETF RFC 5246]: "The record layer fragments information blocks into TLSPlaintext…".

3.2.13	TLS-RL PDU: the protocol data unit (PDU) given by the TLS protocol data structure "TLSCiphertext" according clause 6.2.3/[IETF RFC 5246]. It is the PDU of the TLS record layer. The TLS-RL PDU correlates consequently to the payload data of the underlying IP transport protocol.

3.2.14	TLS transparent forwarding: MG packet forwarding behaviour with the characteristic of TLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a TLS-PDU flow.

NOTE 1 – A TLS PDU relates to a TLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.1, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of TLS message integrity in the context of "TLS transparent forwarding". The MG might be TLS aware; e.g. support of TLS related statistics or event detection would not violate transparent forwarding behaviour.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ALG | Application Level Gateway |
| --- | --- |
| B2BUA | Back to Back User Agent (SIP) |
| BNC | Bearer Network Connection |
| DCCP | Datagram Congestion Control Protocol |
| DTLS | Datagram Transport Layer Security |
| e2ae | End-to-Access-Edge (security model) |
| EMS | Element Management System |
| IP | Internet Protocol |
| IWF | Interworking Function |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| LCD | Local Control Descriptor |
| LD | Local Descriptor Local Destination (Address) |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MP | Measurement Point |
| MSRP | Message Session Relay Protocol |
| NAT | Network Address Translation |
| NAT-T | NAT Traversal |
| NMS | Network Management System |
| PDU | Protocol Data Unit |
| PSTN | Public Switched Telephony Network |
| RCS | Rich Communications Suite |
| RD | Remote Descriptor |
| RTCP | RTP Control Protocol |
| RTP | Real-Time Transport Protocol |
| SA | Security Association Source Address |
| SCTP | Stream Control Transmission Protocol |
| SDES | Source Description (RTP) |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SRTP | Secure Real Time Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TPKT | (T.38) Transport Protocol Data Unit Packet |
| UDP | User Datagram Protocol |
| UDPTL | (T.38) Facsimile UDP Transport Layer (protocol) |
| UE | User Equipment |

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| TLS Est.req TLS Est.ack | Abstracted (TLS message/procedure independent) representationfor establishment requests/acknowledgements of new TLS security sessions. |
| TLS Rel.req TLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing TLS security sessions. |

## 5.2	TLS endpoint notations

The notion of endpoint represents different concepts, see Figure 1:

Figure 1 – Conventions for TLS endpoint types

Usage in

- H.248 control: H.248 terminations/stream endpoint with TLS processing are denoted as TLS-enabled termination or stream endpoint (SEP) respectively;

- user plane (TLS): a TLS bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., TLS protocol termination) plus a service access point (i.e., L4+ access).
Furthermore, TLS is a client/server type of protocol, a "TLS endpoint" (clause 3.2.5) provides thus either a client (see clause 3.2.2) or server (see clause 3.2.11) role.

The Stream Endpoints (SEPs) subject of the packages and procedures of this Recommendation may act as a TLS bearer connection endpoint or not:

- The TLS basic session control package (clause 8), the TLS capability negotiation package (clause 10) and the TLS session maintenance package (clause 11) assumes a SEP that is a TLS bearer connection endpoint.

- The interlinkage procedures described in clause 9 using tools specified in [ITU-T H.248.92] assume a SEP or SEP pair that is (are) TLS bearer connection endpoint(s).

- The use of TLS traffic volume metrics package (clause 12) is applicable both for SEPs that are TLS bearer connection endpoints and SEPs which are not.

## 5.3	Important notation derived from TLS presentation language

See summary in Annex B.3.

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to TLS transport modes

The TLS protocol is independent of the underlying transport protocol stack. Multiple TLS transport modes are possible (Figure 2).

Figure 2 – TLS transport modes

This Recommendation is not dependent on any TLS transport mode, but uses TLS-over-TCP/IP as example (TCP see [IETF RFC 793]).

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as "non-TLS to non-TLS");

- Use case #1.2: an end-to-end TCP connection traversing two network domains with different transport security policies ("TLS", "no TLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "TLS to non-TLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as "TLS to TLS");

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as "TLS to TLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.2	Use case variations

#### 6.2.2.1	"TLS-to-TLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of TLS transparent forwarding according clause 3.2.14.

NOTE  – That TLS transparent forwarding case is also described in the ETSI TISPAN Border Gateway Guidelines (see clause I.4.2.2/[b-ETSI TR 183 068]).

#### 6.2.2.2	"TLS-to-TLS application aware and decryption-encryption based forwarding"

This scenario is a variation of use case #1.3.

The MG may be required to be application aware in case of e.g. bearer-level ALG support (according to [ITU-T H.248.78]). The MG is located within a single security domain, therefore no change of the TLS profile. However, the MG is located at the edge of two network address domains, thus is requested to support NAT.

Example use case:

An MG processing MSRP (Message Session Relay Protocol, [IETF RFC 4975]) traffic: MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the "TLS-to-TLS" interworking function is termed as "TLS B2BUA". It may be noted that this use case does not belong to the "TLS-to-TLS*" category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

## 6.3	Bearer connection network use cases with H.248 "TCP to non-TCP" gateways

This scenario relates to H.248 MGs with a TCP endpoint, with forwarding of application data via a non-TCP bearer. An example network scenario is described in Appendix I.3.

## 6.4	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

For example an MSRP gateway function according to [ITU-T H.248.69] would lead to connection models with more than two H.248 terminations (bearer type here: either MSRP/TCP/IP or MSRP/TLS/TCP/IP). There would be a TCP proxy mode at TCP level (see clause 13.3 in [ITU-T H.248.84]). Such gateway topologies normally provide application aware type of functions (e.g. as media server), thus TLS sessions would be terminated by the MG.

It results in a type of "TLS to TLS*" interworking, such as use case #1.4 (clause 6.2.1).

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 4) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a TLS-capable IP host remote endpoint. Both TLS endpoints span a network transport security domain (here TLS domain).

NOTE – There might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution (such as key management systems). Such kinds of interfaces are out of scope of this Recommendation.

Figure 4 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

Figure 5 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 5 – TCP/IP protocol stack with TLS protocol

Figure 6 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 6 – Two-termination context with a TLS termination

# 8	TLS basic session control package

|  | Package Name: | TLS basic session control package |
| --- | --- | --- |
|  | Package ID: | tlsbsc (0x0117) |
|  | Description: | This package provides the functionality to establish and release a TLS based security session. Furthermore it includes the capability to block the start of TLS session establishment procedures at the MG bearer interface. NOTE 1 – The package design follows similar connection-oriented bearer (or media) protocols, such as the H.248 tcpbcc package [ITU-T H.248.89]. Terminology: "TLS security session" correlates to "TCP bearer connection" and the [b-ITU-T Q.1950] "BNC" concept. NOTE 2 – The differences between DTLS and TLS are subject of [ITU-T H.248.93]. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Incoming security session establishment blocking

|  | Property Name: | Incoming security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1 - Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG availability to accept or reject incoming TLS session establishment procedures. NOTE 2 - See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table 2 Table 2 – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources are prepared (i.e., resources for TLS endpoint are reserved and allocated to H.248 termination/SEP); - TLS role: not determined; prevents assumption of server role; - TLS session still closed; - H.248 counterpart state = "Blocked" (Figure A.1); - Any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall not be acknowledged). - Any outgoing TLS session control procedures are not affected: - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role is effectively assigned - Start TLS session establishment by sending of TLS ClientHello triggered by interlinkage procedures [ITU-T H.248.92]; SEP adopts TLS client role.. "unblocked" (0x0002); - MG shall be ready for incoming TLS security session control procedures; - TLS role: not determined but prepares for a TLS server role effectively: - Incoming TLS ClientHello: TLS server role is confirmed; - Any outgoing TLS session control procedures are not affected - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role is effectively assigned - Start TLS session establishment by sending of TLS ClientHello triggered by interlinkage procedures [ITU-T H.248.92]; SEP adopts TLS client role. - TLS session state: transitioning from closed to another state; - H.248 counterpart state = "Idle" (Figure A.1);. |
|  | Default: | "Unblocked" NOTE - H.248 profile specifications could redefine other default values. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	TLS session state change ("BNC change")

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer (i.e., a TLS security session) has been established or a bearer has been released. The event is related to a simplified state transitioning model, which is described in Annex A. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Sublist of Enumeration |
|  | Optional: | Yes |
|  | Possible values: | Table 3 defines the possible values. Table 3 – Parameter values Value; Name; Final state (Annex A.3); Semantic. Est [0x01]; Bearer Established; Established; Notify MGC of successful TLS security session establishment.. Rel [0x05]; Bearer Released; Idle; Notify MGC of successful TLS security session release.. |
|  | Default: | Type=[Est,Rel] The MGC would be then automatically notified about all state transitions when subscribing to event tlsbsc/BNCChange. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TLS bearer. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est [0x01] Bearer Established Rel [0x05] Bearer Release |
|  | Default: | None |

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TLS security session: the MG takes the TLS client role and initiates the start of a TLS session establishment by sending a TLS ClientHello request). This signal takes effect when the underlying protocol layer is ready for data transfer (NOTE 1) and the stream endpoint is in connection state Idle. If the stream endpoint is in connection states Blocked or Established an error code is returned. NOTE 1 – If there is no establishment request pending in case of connection-oriented layer 4 protocols then an error code should be returned. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TLS security session: the MG sends a TLS alert close_notify request). This signal takes only effect in H.248 connection state Established (and would be replied by an error code when in H.248 connection states Blocked or Idle). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	TLS endpoint creation

The MGC is responsible for creating a TLS session enabled SEP in the MG. This may be indicated on the H.248 interface through the use of the SDP "m=" element <proto>.

The MG shall reserve "TLS bearer resources" (such as a "TLS encryption/decryption function") and allocates them to the SEP. The TLS endpoint role (client/server) is not (yet) determined.

NOTE 1 – This functionality is synonym to the "bearer type indication" semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[b-ITU-T Q.1950].

### 8.6.2	TLS endpoint role assumption

The TLS endpoint role, client or server, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The MGC does not explicitly assign the SEP a TLS role. It is an incoming or outgoing TLS session establishment procedure which leads to the assumption of a TLS role. The SEP is basically prepared for an incoming or an outgoing TLS session establishment procedure since the moment it is created with an appropriate media descriptor. The MGC may prevent the assumption of a server role by setting the bceb property to "blocked". The MGC may effectively determine the SEP to assume the client role by sending the EstBNC signal. The SEP may also assume the client role as a consequence of interlinkage procedures [ITU-T H.248.92]. If these procedures are allowed the SEP may assume a client role as a consequence of e.g. an incoming procedure in the associated SEP. Table 4 summarizes MGC actions and their results in TLS role assignments.

Table 4 – TLS endpoint role assumption (at establishment phase)

| No. | H.248 indication | MG behaviour | TLS role assumed by SEP |
| --- | --- | --- | --- |
| 1 | 1) H.248 SEP state "blocked", due to explicit signalling of bceb property | Any incoming TLS PDU shall be silently discarded. | Not yet determined |
| 2 | 2) H.248 SEP state "unblocked" either due to explicit signalling of bceb property or correspondent default value | Incoming (TLS) ClientHello leads to subsequent completion of the establishment related handshake procedure. | Server role |
| 3 | H.248 signal EstBNC. Value of bceb property not relevant. | Start of TLS session establishment related handshake procedure | Client role |
| 4 | Interlinkage procedures allowed according to [ITU-T H.248.92] Value of bcep property not relevant (although this or similar properties may be relevant at the incoming side of the interlinkage). | An interlinkage establishment procedure [ITU-T H.248.92], e.g. an outgoing TCP connection establishment procedure in the same SEP leads to the start of a handshake for TLS session establishment. | Client role |

The MGC may arm events tlsbsc/BNCChange and g/cause (in H.248 connection state Idle) in order to be notified about successful or unsuccessful TLS security session establishment.

### 8.6.3	TLS security session establishment

The incoming and outgoing side of TLS security session protocol procedures is given by the assigned TLS role (see above clause 8.6.2) by the MGC.

If the SEP assumes the TLS client role as a result of an EstBNC signal or of an interlinkage procedure (see clause 9), the MG shall initiate and complete the TLS security session setup ("handshake") without any further involvement of the MGC. The start of the outgoing TLS security session establishment procedure depends on the readiness for data transfer of the underlying protocol layer.

If the SEP assumes the server role, as a result of an incoming TLS ClientHello message, and being allowed to do so by the MGC by having the bceb property set to “unblocked”, the MG shall complete the TLS security session setup handshake without any further involvement of the MGC.

The end of the TLS security session establishment phase may be summarized according Table 5:

Table 5 – TLS security session establishment - Results

| TLS role | Result of TLS security session establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tlsbsc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent ServiceChange procedure or event g/cause. |
| Server (i.e., incoming establishment side) | Successful | As per client side. |
|  | Unsuccessful | As per client side. |

### 8.6.4	TLS application data transfer

#### 8.6.4.1	MG external

The TLS session is ready for application data transfer when remote and local TLS endpoints have both completed the TLS handshake procedures for session establishment.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TLS control information used to establish or close the TLS session. The requested MG behaviour is defined by Table 6:

Table 6 – Impact of StreamMode on TLS bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All TLS PDUs sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All TLS PDUs received |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |

#### 8.6.4.2	MG internal

This clause is only relevant for H.248 contexts with TLS SEPP(s).

Application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. The interlinkage capability as profiled by clause 9 may be used to align establishment procedures in both SEPs. This may increase the likelihood that application data received in one SEP is not lost.

### 8.6.5	TLS session release

The trigger for a TLS session termination might come from different sources, see Figure 7:

Figure 7 – Principle stimuli for triggering TLS session release

- Bearer control protocol – Security session: The TLS session is terminated by the remote TLS endpoint by sending a (TLS) close_notify alert or a fatal alert;

- Bearer control protocol – Transport connection: The underlying L4 transport connection is terminated;

- TLS protocol: A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicates the closure of the TLS session by the signal tlsbsc/RelBNC;

- The release of the security session of the partner SEP triggers the release autonomously in the MG (Note: not supported by base package tlsbsc, related to interlinkage property seplink/linktopo, see clause 9);

- In case the MG acts in an application aware mode, the application part in the MG may indicate the release of the TLS session.

#### 8.6.5.1	Bearer control protocol – Security session: TLS alert protocol

NOTE – The behaviour defined in this clause is consistent with the TLS session maintenance package according to clause 10. It is further noted that there might be H.248 profiles with and without this TLS session maintenance package.

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a received TLS close_notify the TLS endpoint shall reply a TLS close_notify on its own and the TLS session is closed.

Impact on underlying IP transport connection:

- The MG shall not autonomously initiate the release of the (if connection-oriented) IP transport connection as well.

NOTE - the interlinkage capability (see clause 9) allows to tightly couple bearer control procedures for L4 and TLS.

- If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}. Handling of the TLS-specific AlertDescription as received in the TLS alert message is for further studies.

Impact on H.248 SEP and possibly SEPP:

- The SEP as such still exists after a successful TLS session release (state Idle). Removal of the SEP follows usual H.248 SUBtract.req or MODify.req commands.

- The interlinkage capability (see clause 9) allows the trigger of a TLS session release at the partner SEP.

#### 8.6.5.2	Bearer control protocol – Transport connection: L4 bearer release

In case the underlying IP transport connection is released, e.g. due to a received TCP-FIN or TCP-RST in case of TCP transport, then

- default mode (base package): the MGC shall be notified (by the MG) about the L4 state change (e.g., either by subscription to a correspondent event, an error or ServiceChange indication) and provide instructions for further proceeding with the TLS layer.

- enhanced mode: the MG may autonomously and immediately initiate the closure of the TLS session (dependent used interlinkage capability, as described in clause 9). The release procedures of the IP transport connection shall be delayed in order to complete firstly successfully aTLS session release procedure (see also clause V.4.2.2.1).

NOTE - the interlinkage capability (see clause 9) allows to couple the bearer control procedures for L4 and TLS.

If requested, the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.3	TLS protocol: non recoverable error condition detected

NOTE – The behaviour defined in this clause is consistent with the TLS session maintenance package according to clause 10. It has to be further noted that there might be H.248 profiles with and without this TLS session maintenance package.

In case an error condition is detected locally in the TLS protocol layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert shall be sent to the remote TLS endpoint with the AlertLevel set to "fatal" and the AlertDescription is provided accordingly.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}, when H.248 TLS state has transitioned from Established to Idle.

#### 8.6.5.4	H.248: MGC driven TLS session release

In case the H.248 signal tlsbsc/RelBNC is received, the TLS session closure shall be initiated by the MG by sending a TLS close_notify alert towards remote TLS entity.

#### 8.6.5.5	MG autonomous release initiated on partner SEP

This capability is not supported by the base package, it is subject of the interlinkage capability, see clause 9.

#### 8.6.5.6	Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session shall be closed autonomously by sending a TLS alert (close_notify).

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}, when H.248 TLS state has transitioned from Established to Idle.

The underlying IP transport shall be not affected (base package behaviour).

# 9	TLS-specific stream endpoint interlinkage procedures

## 9.1	Introduction

Property "interlinkage topology" (linktopo) from [ITU-T H.248.92], clause 7.1.1 may be used for extended TLS session control procedures at the establishment and/or release phases, where there is:

- TLS bearer session control behaviour across a "horizontal" Stream Endpoint Pair (SEPP), instead of SEP-only scope; and

- TLS/L4 bearer control behaviour across "vertical" protocol layers, instead of decoupled, individual control of TLS security sessions and their underlying IP transport connection.

This clause defines TLS-specific stream endpoint interlinkage procedures.

## 9.2	Procedures

### 9.2.1	Introduction

The interlinkage procedures could be used on top of the base package tlsbsc procedures according to clause 8. The interlinkage property seplink/linktopo (according to [ITU-T H.248.92]) could be applied for bearer type "TLS" and the establishment and release procedures for self-contained TLS sessions, but also combined "TLS/L4" bearers.

There are two, self-contained interlinkage applications from perspective of this Recommendation (Table 7):

Table 7 – Interlinkage profiling for TLS bearer

| TLS interlinkage type | based on generic interlinkage type [ITU-T H.248.92] |
| --- | --- |
| Stream endpoint pair interlinkage for TLS | Inter-SEP interlinkage configuration for a TLS-enabled SEP and a partner SEP with or without TLS. Interlinkage shall be limited on TLS layer or TLS and the L4 protocol at the partner SEP. |
| Protocol layers interlinkage for "TLS/L4" stack segments | Intra-SEP interlinkage configuration for TLS-enabled SEPs and interlinkage between TLS and the lower layer IP transport protocol. |

Both TLS interlinkage types are disjoint and may be combined. Bearer control related procedures are defined in following clauses.

### 9.2.2	TLS security session establishment

#### 9.2.2.1	Stream endpoint pair interlinkage disabled

When inter-SEP interlinkage is not applied, then the incoming TLS security session establishment request shall not be propagated to the partner SEP.

#### 9.2.2.2	Stream endpoint pair interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS establishment across a TLS-enabled SEPP (see example H.248 syntax in Table 8). The incoming TLS security session establishment request shall then be propagated to the partner SEP, triggering an outgoing TLS ClientHello message. Such kind of interlinkage may be limited to bearer connection use case #1.4 (see clause 6.2.1).

Table 8 – TLS security session establishment – SEPP interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TLS:TLS:est"] }}…} | Interlinkage (for stream S1) is enabled from termination T1 to termination T2 (given by property parameter value "interlinkedSEP"). |

#### 9.2.2.3	Protocol layers interlinkage disabled

If intra-SEP interlinkage is not applied, then TLS control procedures shall not affect the control procedures of the underlying transport layer and any kind of control procedures in the underlying transport layer shall not affect the TLS control procedures

#### 9.2.2.4	Protocol layers interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TLS establishment within a single TLS-enabled SEP in order to interlink with the underlying IP transport protocol endpoint (see example H.248 syntax in Table 9). An incoming or outgoing (NOTE 2) L4 transport connection establishment request (such as a TCP SYN packet) shall then immediately trigger an outgoing TLS security session establishment request, after a successfully established L4 bearer.

Table 9 – TLS security session establishment – Protocol layers interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["ip/1/$/$:<L4>:TLS:est"] ; here <L4>='TCP' }}…} | Intra-SEP interlinkage (for stream S1) is enabled (given by self-referential property parameter value "interlinkedSEP = T1"). It should be noted that the interlinkage direction is only from lower (L4) to upper (TLS) protocol layer (and not vice versa for establishment). I.e., an interlinkage configuration with "<source transport EP> = L4" and "<interlinked tranport EP> = TLS". |

NOTE 1 – Appendix V.4.2.1.1 provides a signalling example.

NOTE 2 – Either MGC driven or in case of e.g. TLS-over-TCP might be also inter-SEP interlinkage enabled at TCP layer, see [ITU-T H.248.89].

### 9.2.3	TLS security session release

#### 9.2.3.1	Stream endpoint pair interlinkage disabled

When inter-SEP interlinkage is disabled, the incoming TLS security session release request shall not be propagated to the partner SEP, hence identical behaviour as in the tlsbsc package procedures.

#### 9.2.3.2	Stream endpoint pair interlinkage enabled

Property seplink/linktopo may be enabled for TLS release across a TLS-enabled SEPP (see example H.248 syntax in Table 10). The incoming TLS security session release request shall then be propagated to the partner SEP, triggering an outgoing TLS closure_notify alert message. Such kind of interlinkage may be limited to bearer connection use case #1.4 (see clause 6.2.1).

Table 10 – TLS security session release – SEPP interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TLS:TLS:rel"] }}…} |  |

#### 9.2.3.3	Protocol layers interlinkage disabled

When intra-SEP interlinkage is disabled, then TLS control procedures shall not affect the underlying transport layer, hence identical behaviour as in the tlsbsc package procedures.

#### 9.2.3.4	Protocol layers interlinkage enabled

Property seplink/linktopo may be enabled for TLS release within a single TLS-enabled in order to interlink with the underlying IP transport protocol endpoint (see example H.248 syntax in Table 11). An incoming or outgoing TLS security session release request shall then immediately trigger an outgoing L4 bearer connection release request, after a successfully released TLS session.

Table 11 – TLS security session establishment – Protocol layers interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["T1:TLS:<L4>: rel"]; here <L4>='TCP' }}…} | It should be noted that the interlinkage direction is only from upper (TLS) to lower (L4) protocol layer (and not vice versa for release). I.e., an interlinkage configuration with "<source transport EP> = TLS" and "<interlinked tranport EP> = L4". |

NOTE 1 – Appendix V.4.2.2 provides signalling examples.

### 9.2.4	Unsuccessful interlinkage

#### 9.2.4.1	Incorrect intra-SEP interlinkage configurations

The MG shall reject incorrect interlinkage configuration requests using error code 488 according to clause 7.5.1/[ITU-T H.248.92].

# 10	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x0118) |
|  | Description: | When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. The aimed negotiation behaviour would be then provisioned in the MG (but the managed objects used do not necessarily correspond with the H.248 signalling objects of this package). This package provides tools for the MGC to control bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of: - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. There are two options for negotiation of TLS protocol configurations: - MGC may indicate a TLS profile (or multiple) as such; - MGC may control individual TLS protocol parameters. Both methods shall be mutually exclusive in this package version. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0001) |
|  | Description: | This property identifies the TLS domain profile that shall be ideally used by the MG to setup the TLS security session. The actually applied, so called TLS endpoint profile is the match result of locally supported TLS capabilities by the MG (termed as TLS MG profile) with requested TLS domain profiles. It should be noted that the normal case would be aligned TLS profile capabilities, i.e.: The applied "TLS endpoint profile" = requested "TLS domain profile". |
|  | Type: | Sub-list of String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.2	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0002) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS protocol version related IETF RFCs (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the "0x", and if needed, a leading "0" is added for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.3	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0003) |
|  | Description: | This property indicates the cipher suites and their precedence (in decending order of preference) for the negotiation with the remote TLS endpoint. |
|  | Type: | Sublist of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA (http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The "0x" shall be omitted and if needed, a "0" is used for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned NOTE – A particular TLS protocol version may define default value(s). Thus, an H.248 profile which defines a default version for TLS could also specify default value(s) for this property. |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.4	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0004) |
|  | Description: | This property declares the list of compression methods and their preference (in descending order) for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers (see http://www.iana.org/assignments/comp-meth-ids/comp-meth-ids.xml). |
|  | Default: | “CompressionMethod.null” |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.5	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0005) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported. |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported True Renegotiation shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.6	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0006) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.7	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0007) |
|  | Description: | This property declares if the MG – if acting as a TLS server – shall authenticate the remote TLS client. |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS server, the client shall not be authenticated. True In case the MG acts as a TLS server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

### 10.6.1	Principles of TLS capability negotiations – overview and scope

The behaviour of TLS capability negotiations is principally influenced by the following dimensions:

- Package usage or package-less operation (see clause 10.6.2);

When package is used, then there are:

- two methods ("(TLS) profile" vs "individual"), and

- two termination levels ("Root" vs "non-Root")

concerning the indication of a requested TLS domain profile by the MGC to the MG (see clause 10.6.3).

There is consequently a high flexibility in supporting many TLS capability negotiation variants (from H.248 perspective), however, the allowed applicable procedures are limited in tlscn package version 1 in order to simplify the operation of an H.248 profile using this package. Such limitations may be reopened again in a future version of this package.

### 10.6.2	General default behaviour (MGC does not influence TLS capability negotiations)

Package tlscn is optional for H.248 profile specifications with TLS support. When the package is not used, then the MG shall independently negotiate TLS configuration(s) with remote TLS endpoints.

Such MG-autonomous TLS capability negotiations shall be compliant to [IETF RFC 5246]. The negotiations would be based on the TLS MG profile and possibly preprovisioned negotiation policies.

### 10.6.3	MGC-controlled TLS capability negotiations: Options

There are two options for MGC-controlled TLS capability negotiations, with the aim to determine the TLS domain profile instance, which is to be used by the MG to form the TLS endpoint profile (by also taking into account the TLS MG profile), see Table 12.

Table 12 – Variants of MGC-controlled TLS capability negotiations

| Method | H.248 elements | Protocol procedures |
| --- | --- | --- |
| a) TLS profile approach | Property: tlscn/dpid | See clause 10.6.4 |
| b) Individual TLS protocol parameter control | Properties: tlscn/tlsv tlscn/cs tlscn/cm tlscn/srsc tlscn/rp tlscn/car | See clause 10.6.5 |

Both methods shall be mutually exclusive in this package version. Thus, an H.248 profile with tlscn v1 package support should select one method.

### 10.6.4	Determining TLS related capabilities (TLS MG profile)

The supported TLS capabilities are subject of the "TLS MG profile". The TLS MG profile is a characteristic at MG level. The auditing of a TLS MG profile shall be therefore limited to the Root termination only.

#### 10.6.4.1	Auditing supported TLS protocol profiles

The MGC may access supported TLS protocol profile(s) by auditing property tlscn/dpid. The identifier of a particular TLS protocol profile relates to a reference on a TLS protocol profile description, which might be for instance explicitly part of an H.248 profile definition or documented anywhere else.

#### 10.6.4.2	Auditing individual supported TLS protocol capabilities

The MGC may determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), and Supported Compression Methods (scm).

The Supported TLS Versions property provides the TLS versions supported by the MG without any specific sequence.

The Supported Cipher Suite property shall include the cipher suite TLS_NULL_WITH_NULL_NULL ("0000"), which is used as the initial cipher suite during the first handshake for the TLS session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

### 10.6.5	Indication of TLS profile(s) to MG by MGC

The MGC may request TLS domain profile(s) support from the MG via property dpid, leading to following MG behaviour:

- Built intersection between locally supported TLS capabilities (the "TLS MG profile") with indicated TLS domain profile(s), resulting in a TLS endpoint profile (see also clause 14);

- The TLS endpoint profile shall be the baseline for TLS negotiations with the remote TLS entity (during TLS security session establishment procedures).

### 10.6.6	Indication of individual TLS protocol capabilities to MG by the MGC

In this case the TLD Domain profile instance to be used by the MG is (partially) defined by the MGC by the actually signalled tlscn/x properties with "x != dpid". Those tlscn/x properties, which are not explicitly signalled by the MGC assume their values as provisioned in MG's TLS MG profile.

#### 10.6.6.1	TLS protocol parameter "TLS version"

The list of TLS versions used for the TLS procedures of the session is determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned (i.e., the property value for dpid) on the MG as typically default configuration.

NOTE – The TLS endpoint profile again (see also clause 14) relates to the intersection/match of TLS MG profile (as provisioned on MG) and individual explicitly signalled tlscn/x properties with "x != dpid".

In case the MGC signals the property TLS Versions (tlsv), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG acts as a TLS client, the highest TLS versions as defined by the specific TLS endpoint profile are included into the initial TLS handshake message (ClientHello). In case the MG acts as a TLS server, the MG shall select the highest TLS version that is present in the list of TLS versions offered from the TLS client and in the TLS versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS handshake is regarded as failed.

Unsuccessful TLS negotiations should be reported by error code #449 (Unsupported or Unknown Parameter or Property Value).

#### 10.6.6.2	TLS protocol parameter "cipher suite"

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Chipher Suites (cs), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS version.

#### 10.6.6.3	TLS protocol parameter "compression method"

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Compression Methods (cm), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle as the negotiation of the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS version.

#### 10.6.6.4	TLS protocol procedure "client authentication"

The MG's TLS server implementation shall support client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid).

In case the MGC signals property Client Authentication Required (car), the MG shall use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

#### 10.6.6.5	TLS protocol procedure "self-signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute "a=fingerprint". If present in the Remote Descriptor, the client certificate shall be self-signed and shall be verified against the fingerprint.

#### 10.6.6.6	TLS protocol procedure "signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no SDP fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile. This list of root certificates is supposed to be configured per dpid on the MG.

#### 10.6.6.7	TLS protocol procedure "server authentication"

In case the TLS endpoint in the MG is configured as a TLS server, the MG shall provide the server certificate to the client as defined by the TLS domain profile.

#### 10.6.6.8	TLS protocol procedure "session resumption"

A framework for TLS session resumption is outlined in Appendix VII. However, specific use cases for TLS session resumption (with H.248 gateways) are for further studies.

#### 10.6.6.9	TLS protocol procedure "renegotiation support"

The TLS implementation in the MG shall provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile. When the MGC provides the property tlscn/rp, then the MG shall use this value instead of the provisioned default TLD Domain profile's "rp" value.

In case the MGC signals property Support of Renegotiation of the Security Context (srsc), the MG shall use this value instead.

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, taking into account property Renegotiation Period (rp).

In case the TLS endpoint profile disables renegotiation, the MG shall never initiate a renegotiation. If receiving a renegotiation request from the remote TLS endpoint, the MG shall respond with a TLS no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS version.

If renegotiation shall be supported, the MG shall accept and process a renegotiation request from the remote TLS endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to value "0", no renegotiation shall be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS handshake completed successfully. On expiration of the timer the MG shall trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS version. In case the MG receives a trigger for the renegotiation from the remote TLS endpoint while this timer is running, the MG shall cancel and restart this timer.

### 10.6.7	Auditing the actual TLS configuration (TLS endpoint profile)

The actual agreed TLS configuration between the local and remote TLS endpoint referes to the current value settings of a TLS endpoint profile and may be audited by the MGC via an AuditValue procedure. Such kind of auditing shall be limited on non-Root termination level.

# 11	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x0119) |
|  | Description: | This package defines capabilities for support of TLS alerts, which are exchanged between the remote and MG local TLS endpoint, following the TLS alert protocol. NOTE - Appendix V provides some background information concerning TLS alert protocol handling by H.248 gateways. This package version is limited to the so-called TLS error alerts, as defined by clause 7.2.2/[IETF RFC 5246]. The package provides capabilities for MGC triggered sending of TLS alerts by the MG, as well as a reporting mechanism to notify the MGC about received TLS alerts by the MG. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

### 11.2.1	TLS Error Alert

|  | Event Name: | TLS error alert |
| --- | --- | --- |
|  | Event ID: | mgea (0x0001) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the immediate termination of the TLS session. This is typically an TLS alert with severity level "warning", but could be also an received "fatal" alert (see also Appendix V.2).The abnormal condition may be detected on the local or on the remote TLS endpoint. The event is limited to TLS error alerts and excludes the TLS closure alert event. The event shall be set at stream level. |

#### 11.2.1.1	EventsDescriptor parameters

None

#### 11.2.1.2	ObservedEventsDescriptor parameters

##### 11.2.1.2.1	Bearer level Alert Initiator

|  | Parameter Name: | Bearer level alert initiator |
| --- | --- | --- |
|  | Parameter ID: | blai (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | "local" (0x0001) Abnormal condition originated at and detected by the MG local TLS endpoint. "remote" (0x0002) Abnormal condition originated at remote TLS endpoint side, then reported to the the MG local TLS endpoint via TLS alert protocol by the remoteTLS endpoint. |
|  | Default: | NA |

##### 11.2.1.2.2	Error Alert Type

|  | Parameter Name: | Error alert type |
| --- | --- | --- |
|  | Parameter ID: | eat (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition by referencing the TLS error alert type. The codepoints registered by IANA within the TLS Alert Registry (see http://www.iana.org/‌assignments/tls-parameters/tls-parameters.xml). |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | N/A |

## 11.3	Signals

### 11.3.1	MGC triggered TLS error alert

|  | Signal Name: | MGC triggered TLS error alert |
| --- | --- | --- |
|  | Signal ID: | mgcea (0x0001) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates "fatal", the session shall be terminated. NOTE - the TLS "close_notify(0)" alert is excluded because it does not belong to the TLS error alert category. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 11.3.1.1	Additional parameters

##### 11.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246], i.e., either a warning or fatal alert severity. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for "AlertLevel", refer to [IETF RFC 5246]. |
|  | Default: | Not applicable |

##### 11.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | Not applicable |

## 11.4	Statistics

None.

## 11.5	Error codes

None.

## 11.6	Procedures

Generally, there are TLS alert specific H.248 procedures due to the nature of the TLS alert protocol, see Appendix V.

### 11.6.1	TLS warning error alerts

#### 11.6.1.1	General behaviour

The TLS session should be kept active (and not released) when TLS alerts with warning severity are exchanged between local and remote TLS endpoints.

#### 11.6.1.2	MG received TLS error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at warning level, it shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "remote".

#### 11.6.1.3	MG locally generated TLS error alerts and sent to remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever there are such MG locally generated TLS error alerts, an outgoing TLS error alert at warning level is sent and the MG shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "local".

#### 11.6.1.4	MGC triggered TLS error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alert towards the remote TLS endpoint.

### 11.6.2	TLS fatal error alerts

#### 11.6.2.1	General behaviour

Whenever a TLS fatal error alert is exchanged between local and remote TLS endpoints, then there will be always a subsequent, immediate release procedure of the TLS session (see also Appendix V.2).

#### 11.6.2.2	MG received TLS fatal error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS fatal error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at fatal level, it shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "remote".

The subsequently expected TLS closure_notify alert indicates a TLS session release request. The received TLS closure_notify alert would then trigger the TLS basic session control package (tlsbsc) procedures according to clause 8.6.5.1.

#### 11.6.2.3	MG locally generated TLS fatal error alerts and sent to remote TLS endpoint

The MGC may subscribe to be notified about TLS fatal error alerts via event mgea. Whenever there are such MG locally generated TLS error alerts, an outgoing TLS error alert at fatal level is sent and the MG shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "local".

The MG shall send a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure shall be either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.2.3).

#### 11.6.2.4	MGC triggered TLS fatal error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alert towards the remote TLS endpoint.

Furthermore, the MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure shall be either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.2.3).

### 11.6.3	Lower layer issues

There might be issues in the protocol layers below the TLS layer.

Example use case:

- Established IP bearer path, i.e. there is an active TLS session;

- The underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- Hence, there would be still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective;

- And therefore the TLS session would be interrupted, no TLS alert could be sent to the remote side.

Procedures for the maintenance of lower layer issues are out of scope of this package, rather subject of exsting H.248 error handling capabilities.

# 12	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x011a) |
|  | Description: | This package is used to support statistics for the TLS protocol related to various protocol data unit types (such as octets, TLS fragments, etc.). The data model used is defined by Annex B, which is consistent with the TLS protocol itself. The statistics are applicable as long as the MG is aware that the H.248 Stream carries TLS traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 12.1	Properties

None.

## 12.2	Events

None.

## 12.3	Signals

None.

## 12.4	Statistics

### 12.4.1	Number of Received Application Data TLS-Fragments

|  | Statistic Name: | Number of Received Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recadfrag (0x0001) |
|  | Description: | Provides the number of application data TLS fragments received on the Termination or Stream since the statistic has been set. Only application data TLS fragments are counted which are passed from the TLS record layer to the application layer. A received TLS-PDU is regarded as application data if the TLS header field "type" of the received TLS-PDU has been set to "application_data(23)". At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.2	Number of Sent Application Data TLS-Fragments

|  | Statistic Name: | Number of Sent Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentadfrag (0x0002) |
|  | Description: | Provides the number of application data TLS fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as application data if the TLS header field "type" of the resulting TLS-PDU is set to "application_data(23)". At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.3	Number of Received TLS Message Fragments

|  | Statistic Name: | Number of Received TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | rectmfrag (0x0003) |
|  | Description: | Provides the number of TLS message fragments received on the Termination or Stream since the statistic has been set. Only TLS message fragments are counted which are passed from the TLS record layer to the change_cipher_spec protocol, the alert protocol or the handshake protocol. A received TLS-PDU is regarded as an TLS message if the TLS header field "type" of the received TLS-PDU has been set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.4	Number of Sent TLS Message Fragments

|  | Statistic Name: | Number of Sent TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | senttmfrag (0x0004) |
|  | Description: | Provides the number of TLS message fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as TLS message if the TLS header field "type" of the resulting TLS-PDU is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.5	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0005) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the application layer above the TLS record layer. Received application data messages from the remote TLS endpoint will have the TLS header field "type" set to "application_data(23)" and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.6	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0006) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.7	Number of Received TLS Message Octets

|  | Statistic Name: | Number of Received TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmo (0x0007) |
|  | Description: | This statistic logs the number of TLS message octets that are delivered to the change_cipher_spec protocol, the alert protocol or to the handshake protocol above the TLS record layer. Received TLS messages from the remote TLS endpoint will have the TLS header field "type" set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The TLS messages and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Application data messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.8	Number of Sent TLS Message Octets

|  | Statistic Name: | Number of Sent TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmo (0x0008) |
|  | Description: | This statistic logs the number of octets received from the change_cipher_spec protocol, the alert protocol or the handshake protocol. The octets are counted before the TLS messages are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.9	Number of Received Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Received Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | recadpduo (0x0009) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress Application Data TLS PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.10	Number of Sent Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Sent Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | sentadpduo (0x000a) |
|  | Description: | This statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", sent from the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress Application Data TLS PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.11	Number of Received TLS Message PDU Octets

|  | Statistic Name: | Number of Received TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmpduo (0x000b) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress TLS Message PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.12	Number of Sent TLS Message PDU Octets

|  | Statistic Name: | Number of Sent TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmpduo (0x000c) |
|  | Description: | This statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets sent from the Termination or Stream since the Termination or Stream has existed and the statistic has been set. The octets represent the egress TLS Message PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.13	Number of Received Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Received Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | recadpco (0x000d) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.14	Number of Sent Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | sentadpco (0x000e) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.15	Number of Received TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Received TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | rectmpco (0x000f) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress protected and compressed TLS message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.16	Number of Sent TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | senttmpco (0x0010) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress protected and compressed TLS messags of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.17	Number of Octets of Compressed Application Data Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | recadcfo (0x0011) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.18	Number of Octets of Compressed Application Data Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | sentadcfo (0x0012) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.19	Number of Octets of Compressed TLS Message Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | rectmcfo (0x0013) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress compressed TLS Message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.20	Number of Octets of Compressed TLS Message Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | senttmcfo (0x0014) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress compressed TLS Messages of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 12.5	Error codes

None.

## 12.6	Procedures

### 12.6.1	General information

#### 12.6.1.1	TLS data model

The underlying data model is contained in Annex B, which defines relevant measurement points (MP).

#### 12.6.1.2	Overview of performance metrics

Table 13 summarizes all ten categories of performance metrics which result in twenty statistics due to support in both traffic directions:

Table 13 – Overview of performance metrics – All H.248 statistics

| No. | Category of H.248 statistics | Direction | Clause | StatisticsID |
| --- | --- | --- | --- | --- |
| 1 | Number of Application Data TLS-Fragments | Received | 12.4.1 | recadfrag |
| 2 |  | Sent | 12.4.2 | sentadfrag |
| 3 | Number of TLS Message Fragments | Received | 12.4.3 | rectmfrag |
| 4 |  | Sent | 12.4.4 | senttmfrag |
| 5 | Number of Application Data Octets | Received | 12.4.5 | recado |
| 6 |  | Sent | 12.4.6 | sentado |
| 7 | Number of TLS Message Octets | Received | 12.4.7 | rectmo |
| 8 |  | Sent | 12.4.8 | senttmo |
| 9 | Number of Application Data TLS-PDU Octets | Received | 12.4.9 | recadpduo |
| 10 |  | Sent | 12.4.10 | sentadpduo |
| 11 | Number of TLS Message PDU Octets | Received | 12.4.11 | rectmpduo |
| 12 |  | Sent | 12.4.12 | senttmpduo |
| 13 | Number of Application Data Octets, Protected and Compressed | Received | 12.4.13 | recadpco |
| 14 |  | Sent | 12.4.14 | sentadpco |
| 15 | Number of TLS Message Octets, Protected and Compressed | Received | 12.4.15 | rectmpco |
| 16 |  | Sent | 12.4.16 | senttmpco |
| 17 | Number of Octets of Compressed Application Data Fragments | Received | 12.4.17 | recadcfo |
| 18 |  | Sent | 12.4.18 | sentadcfo |
| 19 | Number of Octets of Compressed TLS Message Fragments | Received | 12.4.19 | rectmcfo |
| 20 |  | Sent | 12.4.20 | senttmcfo |

Table 14 indicates the internal measurement points (given by Figure B.1) and whether a TLS protocol termination would be required:

Table 14 – Internal measurement points (MP), traffic types and conditional usage

| Category of H.248 statistics | StatisticsID | Relates to | Measurement point | Presentation Language of RFC 5246 | TLS protocol termination at SEP |
| --- | --- | --- | --- | --- | --- |
| Number of Application Data TLS-Fragments | recadfrag | Application Data | Application Data | TLSPlaintext. fragment | Yes |
|  | sentadfrag |  |  |  |  |
| Number of TLS Message Fragments | rectmfrag | TLS message | TLS message | TLSPlaintext. fragment | Yes |
|  | senttmfrag |  |  |  |  |
| Number of Application Data Octets | recado | Application Data | Application Data | - | No |
|  | sentado |  |  |  |  |
| Number of TLS Message Octets | rectmo | TLS message | TLS-RL PDU | - | No |
|  | senttmo |  |  |  |  |
| Number of Application Data TLS-PDU Octets | recadpduo | Application Data | TLS-RL PDU | TLSCiphertext | No |
|  | sentadpduo |  |  |  |  |
| Number of TLS Message PDU Octets | rectmpduo | TLS message | TLS-RL PDU | TLSCiphertext | No |
|  | senttmpduo |  |  |  |  |
| Number of Application Data Octets, Protected and Compressed | recadpco | Application Data | TLS-RL PDU | TLSCiphertext.fragment | No |
|  | sentadpco |  |  |  |  |
| Number of TLS Message Octets, Protected and Compressed | rectmpco | TLS message | TLS-RL PDU | TLSCiphertext.fragment | No |
|  | senttmpco |  |  |  |  |
| Number of Octets of Compressed Application Data Fragments | recadcfo | Application Data | TLS-RL PDU | TLSCompressed.fragment | Yes |
|  | sentadcfo |  |  |  |  |
| Number of Octets of Compressed TLS Message Fragments | rectmcfo | TLS message | TLS-RL PDU | TLSCompressed.fragment | Yes |
|  | senttmcfo |  |  |  |  |

#### 12.6.1.3	Profile selection guidelines

It is expected that a particular H.248 profile with support of the tlstv package will only support a subset from the defined set of statistics. Example:

- A profile interested in application level measurements only would just use the statistics related to measurement point "Application Data".

- A profile with support of "TLS transparent forwarding" only may support just the statistics which does not need a TLS protocol termination in the MG.

- A profile interested in "bitrate"-based traffic volume measurement data would just use the octet-based H.248 statistics.

- A profile interested in TCP traffic volume with and without TLS-based transport security (e.g., comparing data-over-TLS-over-TCP with data-over-TCP) may need to gather measurements at measurment point "TLS-RL PDU" for detailed performance evaluations.

#### 12.6.1.4	Measurement principles

The usual H.248 principles apply:

- consideration of all successfully received/sent data units to/from its H.248 TLS Stream/Termination;

- aggregation of all TLS flows of an H.248 Stream/termination.

### 12.6.2	Number of Application Data TLS-Fragments

Statistics tlstv/recadfrag and tlstv/sentadfrag provide a fragment-oriented based volume measurements between TLS and application protocol layer.

### 12.6.3	Number of TLS Message Fragments

Statistics tlstv/rectmfrag and tlstv/senttmfrag provide a fragment-oriented based volume measurements between TLS record protocol and TLS handshake/cipher/alert protocol layer.

### 12.6.4	Number of Application Data Octets

Statistics tlstv/recado and tlstv/sentado provide octet-oriented based volume measurements between TLS and application protocol layer.

### 12.6.5	Number of TLS Message Octets

Statistics tlstv/recmo and tlstv/sentmo provide octet-oriented based volume measurements between TLS record protocol and TLS handshake/cipher/alert protocol layer.

### 12.6.6	Number of Application Data TLS-PDU Octets

Statistics tlstv/recadpduo and tlstv/sentadpduo provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.7	Number of TLS Message PDU Octets

Statistics tlstv/rectmpduo and tlstv/senttmpduo provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.8	Number of Application Data Octets, Protected and Compressed

Statistics tlstv/recadpco and tlstv/sentadpco provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.9	Number of TLS Message Octets, Protected and Compressed

Statistics tlstv/rectmpco and tlstv/senttmpco provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.10	Number of Octets of Compressed Application Data Fragments

Statistics tlstv/recadcfo and tlstv/sentadcfo provide fragment-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.11	Number of Octets of Compressed TLS Message Fragments

Statistics tlstv/rectmcfo and tlstv/senttmcfo provide fragment-oriented based volume measurements between L4 transport and TLS protocol layer.

# 13	Package-less TLS control

Usage of dedicated SDP elements is principally not excluded in package-based TLS session control. However, such SDP elements shall be avoided which could semantically overlap or lead to ambiguities with H.248.

## 13.1	Related to TLS session establishment

### 13.1.1	Overview – principal H.248 control steps

Figure 8 indicates all possible control steps (from H.248 perspective) concerning the establishment of TLS-over-L4/IP bearer connections in environments demanding NAT traversal support. The dashed boxes are L4-dependent (e.g., in case of TCP see [ITU-T H.248.89, H.248.69, H.248.84]):

Figure 8 – Principal H.248 control steps for establishing a TLS-over-L4 bearer

It is apparent that some steps might be combined (e.g., 1, 2 and 7) and that specific steps might be optional (e.g., 3, 5, 8, 10).

Table 15 provides a preliminary inventory of possible control elements from H.248 perspective (at the example of TCP as L4 protocol). The column "H.248/SDP element" refers to "package-less TLS control".

Table 15 – H.248 control steps and possible control elements

| H.248 control step |  | H.248 property | H.248/SDP element | Comments |
| --- | --- | --- | --- | --- |
| Establishment phase I: "IP transport connection establishment" |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | "m="-line <proto> (clause 13.4 / [ITU-T H.248.84], [ITU-T H.248.69]) | SDP may be ambiguous due to "protocol stack" concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property, e.g., TCP: tcpbcc/bceb | "a=setup:holdconn" | NOTE 3 |
| 4 | Start IP Transport Connection Establishment Procedure (e.g. TCP active or passive OPEN) | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to [ITU-T H.248.84] "a=setup:" semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | [ITU-T H.248.37] | "a=setup:" for [ITU-T H.248.84] NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State "EstablishedL4") | Event, e.g., TCP: tcpbcc/BNCChange | — | MGC notification might be optional. |
| Establishment phase II: "IP transport security session establishment" |  |  |  |  |
| 7 | Bearer type (L4+ = 'TLS') indication to SEP | Indirect: package elements used from TLS-related H.248 packages | "m="-line <proto> | NOTE 2. SDP may be ambiguous due to "protocol stack" concept (NOTE 1). |
| 8 | Block / Unblock L4+ Bearer Control Procedures at SEP | Property: tlsbsc/bceb | None | I.e., handling of TLS handshake protocol TLS cipher change protocol TLS alert protocol |
| 9 | Start IP Security Session Establishment Procedure ("TLS negotiations") | Event: tlsbsc/BNCChange Signal: tlsbsc/EstBNC | None | The above TLS protocols are enabled |
| 10 | Local L4+ NAT traversal support | [ITU-T H.248.78] | e.g., MSRP: "a=msrp-cema" | NOTE: L4+ NAT-T may be also requested for non-TLS. |
| 11 | Successfully established IP Transport Security Session (State "EstablishedTLS") | Event: tlsbsc/BNCChange | — |  |
| Communication phase: |  |  |  |  |
| 12 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP "m=" line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well known issue of this element. NOTE 2 – Step 7 may be merged with step 1 in case of tightly coupled "TLS/L4" control. NOTE 3 – See Table 4/[H.248.89] due to example "L4 = 'TCP'" here. |  |  |  |  |

### 13.1.2	Step 1: Bearer endpoint creation

Not SDP related, based on H.248 Command request.

### 13.1.3	Step 2: Bearer type indication for IP transport "L4/L3"

The SDP-based approach of using the SDP "m=" line <proto> element as transport indicator should be used. The SDP method shall work for all required TLS transport modes (see clause 6.1/[ITU-T H.248.91]).

Conditional approach, dependent on:

- C2.1: unambiguous indication of transport protocol "L4" (Issue: "m="-line <proto> relates to a protocol stack based indicator, rather than a distinct identifier to a specific L4 protocol).
Warning: possible semantical interaction with step 7!

- C2.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic)

- C2.3: could imply additional TCP bearer connection establishment (step 4) when using [ITU-T H.248.69].

### 13.1.4	Step 3: Block / Unblock L4 bearer control procedures at SEP

This is out of scope of this Recommendation. E.g., see [ITU-T H.248.89] in case of TCP as TLS transport protocol.

### 13.1.5	Step 4: Start IP transport connection establishment procedure

Relevant for connection-oriented IP transport protocols, which are used as TLS transport modes (see clause 6.1/[ITU-T H.248.91]), but out of scope of this Recommendation. E.g., see [ITU-T H.248.89] in case of TCP as TLS transport protocol.

Warning: possible semantical interaction with step 9 (see clause 11.2.5/[ITU-T H.248.89] in case of "L4 = 'TCP'")!

### 13.1.6	Step 5: Local L4/L3 NAT traversal support

This is a crucial step because TLS will fail without any successfully established IP transport connection. However, this is out of scope of this Recommendation. There are dedicated L4/L3 NAT-T support methods, such as [ITU-T H.248.37], [ITU-T H.248.50] or [ITU-T H.248.84].

It should be noted that the above example NAT-T mechanisms are primarily enabled using H.248 packages ("and not via LD/RD-embedded SDP signalling"). See also clause 11.2.6/[ITU-T H.248.89] in case of "L4 = 'TCP'".

### 13.1.7	Step 6: Reporting successfully established IP transport connection

Not supported by SDP, but out of scope of this Recommendation. E.g., see [ITU-T H.248.89] in case of TCP as TLS transport protocol.

### 13.1.8	Step 7: Bearer type indication for TLS

If SDP based, then only the same method as in step 2, i.e. using the SDP "m=" line <proto> element as security session indicator.

Conditional approach, dependent on:

- C7.1: unambiguous indication, see C2.1.

- C7.2: consistent LD/RD signalling (due to the fact of a LCD level characteristic).

### 13.1.9	Step 8: Block / Unblock TLS security session control procedures at SEP

Not supported by SDP, thus property tlsbsc/bceb required.

### 13.1.10	Step 9: Start TLS security session establishment procedure

There is no explicit SDP indicator for outgoing and incoming TLS "handshake procedures" as part of TLS session establishment.

NOTE – Unconditional, general approach: signal tlsbsc/EstBNC or event tlsbsc/BNCChange{Type=Est} for triggering of outgoing or incoming TLS security session establishment procedures.

### 13.1.11	Step 10: Local L4+ NAT traversal support

Not possible via explicit SDP for general L4+ NAT-T, thus [ITU-T H.248.78] required.

NOTE – Some application-specific L4+ NAT-T support functions could be associated with application-specific SDP information, however, this is out of scope of this Recommendation.

### 13.1.12	Step 11: Reporting successfully established TLS session

Not supported by SDP, thus event tlsbsc/BNCChange required.

### 13.1.13	Step 12: Active communication phase

The control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TLS traffic) as such is out of scope of the SDP.

## 13.2	Related to TLS authentication

NOTE 1 – This clause satisfies requirement R-8.3.3.2.2/1 of [ITU-T H.248.91].

The SDP "a=fingerprint:" attribute (according to [IETF RFC 4572]) shall be used under the condition of H.248 gateways involved in network architectures (given by trust model and SIP/SDP as application control protocol) as described by this RFC.

NOTE 2 – The SDP based method is exclusive, i.e., there are not any alternative H.248 signalling elements defined.

If the SDP "a=fingerprint:" element is required by an H.248 profile, then

- the MGC would provide a fully specified SDP fingerprint attribute in the H.248 RD and

- the MGC would provide either a fully specified or wildcarded (see [ITU-T H.248.39]) SDP fingerprint attribute in the H.248 LD

to the MG.

Usage of the the SDP "a=fingerprint:" attribute at the H.248 interface validates the bearer-to-call binding between SIP call and TLS secured bearer (see e.g. Figure 6/[ITU-T H.248.91]).

## 13.3	Related to TLS session release

Not possible via SDP. The hard removal of a TLS-enabled stream endpoint via an H.248 MODify.req or SUBtract.req command would lead to a complete release of MG level TLS resources. Usage of signal tlsbsc/RelBNC allows to keep these resources as long as the H.248 SEP should be not removed.

# 14	The TLS profile concept

## 14.1	TLS protocol profiles

A MG may be requested to support more than one TLS domain. For each such TLS domain different "TLS services" may be required from the MG. For each TLS domain those required TLS behaviours are defined by a domain-specific TLS domain profile.

Generally the TLS domain profile may be controlled via H.248 by usage of the tlscn package (clause 10), either as an atomic entity (so called "TLS profile" approach in clause 10.6.1) or by "individual TLS protocol parameter control" (see clause 10.6.1).

Important to note: any concrete, detailed, normative specifications of TLS protocol profiles are out of scope of this Recommendation!

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as a "TLS MG profile (see also clause 3.2.9).

On creation of a TLS endpoint in the MG, the intersection of the selected (and possibly modified) TLS domain profile(s) and the TLS MG profile will be built. This intersection may be termed as the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the TLS-enabled endpoint (i.e., the MG H.248 SEP/termination, see also clause 5.2).

Figure 9 illustrates the principle relation between the profiles.

Figure 9 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile

Normally, the result of intersection is a TLS endpoint profile that exactly matches with the requested TLS domain profile(s). However, there might be partial matches in principle as well, as depicted by the example in Figure 8.

## 14.2	Illustration of TLS profile concept

The indication and negotiation of media configurations across the H.248 interface is subject of [ITU-T H.248.80]. A TLS configuration relates effectively to a subset of a media configuration (see clause 3.2.1.2/[ITU-T H.248.80]). Table 16 describe analogous terms and profile types.

Table 16 – Analogous terms between H.248.80 media configurations and TLS profile types

| H.248.80 generic media configuration | TLS-specific media configuration | H.248.TLS TLS profile term: |
| --- | --- | --- |
| Overall term: |  |  |
| Configuration (clause 3.2.1 / [ITU-T H.248.80]) | TLS configuration | TLS profile |
| Qualified term: |  |  |
| Prefered configuration (e.g., prefered codec list) | Prefered TLS configuration (NOTE 1) | TLS domain profile |
| Supported configuration (e.g., supported codec list) | Supported TLS configuration | TLS MG profile |
| Actual configuration (e.g., actual codec list) | Actual TLS configuration | TLS endpoint profile |
| NOTE 1 – There is a subtle difference between [ITU-T H.248.80] and this Recommendation: there are no SIP-level SDP Offer/Answer negotiations concerning TLS configurations due to the fact that TLS provides itself an embedded TLS handshake protocol for TLS-level negotiations. Thus, there is neither an "Offered TLS configuration" nor "Answered TLS configuration" at application control protocol level. |  |  |

Figure 10 provides an illustration of the three TLS profile types, as in use during TLS security session establishment.

Figure 10 – Illustration of TLS profile concept

In general, the TLS MG profile reflects the status of implemented TLS capabilities, it may be therefore principally a subset, superset or equal to a requested TLS domain profile.

H.248 profiles could also simplify network solutions by mandating supported TLS capabilities, which could lead to the equation of "TLS MG profile equal to TLS domain profile", resulting in "TLS endpoint profile also equal to TLS domain profile".

## 14.3	Example for the TLS MG profile concept

Appendix III provides examples for TLS domain profiles.Table 17 depicts an example for a TLS MG profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 17 – Example of a TLS MG profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlscn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlscn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlscn/cm |
| Client authentication required | yes | tlscn/car |
| Resumption support | no | For further studies. |
| Extensions | For further studies. |  |

The following Table 12 shows an example how the intersection of a TLS domain profile and the TLS MG profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. Table 18 does not indicate which properties can be provided by the MGC.

Table 18 – Example of an intersection between a TLS domain profile and a TLS MG profile

| Property | TLS Domain profile | TLS MG profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B |
| MG certificate (NOTE 1) |  | List of certificates, one certificate per TLS domain profile (NOTE 2) |  |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |
| NOTE 1 – The MG certificate is used during the TLS handshake, thus it is a part of the "set of TLS related parameter and procedures" (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the TLS MG profile. The MG certificate is network element specific and TLS domain specific. Thus, at least one "default" TLD domain profile needs always to be provisioned on the MG, which is applicable if the MGC does not explicitly signal the tlscn/dpid property. In such a case this default provisioned TLS domain profile is merged with potentially signalled tldcn/x properties (x != dpid) to form the TLS endpoint specific TLD endpoint profile, which then also contains Root certificates and one MG certificate. NOTE 2 – Thus, the MG to "know" (e.g. via provisioning) a "default" TLD domain profile in case the MGC does not signal the tlscn/dpid property. |  |  |  |

# 15	Security considerations

It has to be noted that there are a number of known TLS protocol vulnerabilities, leading to ongoing TLS protocol improvement activities by the IETF.

The H.248 MG might become a point of potential, TLS related security attacks when terminating the TLS protocol (as e.g. in use cases #1.2 and #1.4 according clause 6.2.1). Appendix F "Security Analysis" of [IETF RFC 5246] and related material such as [b-IETF tls-attacks] should be then considered from security perspective.

Annex A

State modelling for 
TLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TLS basic session control package (clause 8) is tightly coupled to a state model, because the event (tlsbsc/BNCChange) and signal (tlsbsc/EstBNC) are related to state transitioning behaviour.

It has to be noted that the IETF TLS related RFCs do not define any explicit state model with regards to TLS security session endpoints. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tlsbsc is described by this Annex.

NOTE – This Recommendation release does not describe an (H.248) TLS bearer connection (or TLS security session) as separate (IETF RFC) TLS session and (IETF RFC) TLS connection entities. Such a difference is not relevant for the first package versions because e.g. use case TLS session resumption is for further studies. Any future refinement may indicate TLS resource components in detail which are in scope of the H.248 state model below.

## A.2	Original state model for TLS session endpoints

There is no standardized state model for TLS (in comparison to e.g. L4 protocols TCP or SCTP).

## A.3	Simplified state model for H.248-based TLS basic session control

The TLS basic session control package makes a couple of assumptions, similar to those in Annex A/[H.248.89] for TCP.

The correspondent simplified model for TLS is depicted in Figure A.1.

Figure A.1 – Simplified state model for H.248-based TLS basic session control

There are two remaining states, called Idle and Established in the H.248 state model. Signal tlsbsc/EstBNC is used for triggering a state change from Idle to Established by the MGC, signal tlsbsc/RelBNC is used for the reverse transitioning and event tlsbsc/BNCChange indicates successfully completed state transitions to the MGC.

### A.3.1	Transitioning from Idle to Established

The bearer establishment related TLS handshake procedures, either a full or abbreviated handshake, will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters.

- The TLS-Finish message has been received and has been successfully verified.

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event tlsbsc/BNCChange{type=Est} can be used to notify the MGC accordingly.

### A.3.2	Transitioning from Established to Idle

Transitioning to state Idle is completed after a complete TLS close_notify procedure.

Annex B

TLS protocol layer: Data model

(This annex forms an integral part of this Recommendation.)

## B.1	Motivation

TLS [IETF RFC 5246] is unusal from protocol specification perspective due to its idiosyncrasy in using a kind of proprietary presentation language concerning syntax definitions, see:

4. Presentation Language

This document deals with the formatting of data in an external representation. The following very basic and somewhat casually defined presentation syntax will be used. The syntax draws from several sources in its structure. Although it resembles the programming language "C" in its syntax and XDR in both its syntax and intent, it would be risky to draw too many parallels. The purpose of this presentation language is to document TLS only; it has no general application beyond that particular goal.

It is difficult to get a concise view about the structure of protocol data units within and at the interfaces of the TLS protocol layer. Such a data model is required for some of the defined H.248 signalling elements (such as statistics) by this Recommendation.

A data model is derived in this Annex, based on the TLS protocol data structures according Appendix A/[IETF RFC 5246].

## B.2	Data model

Figure B.1 (derived from Figure 4) outlines again the TLS protocol stack inclusive of the four TLS sub-protocols. Four measurement points (MP) are highlighted, which are relevant for the several TLS traffic volume metrics according to clause 12.

Figure B.1 – Performance monitoring – Measurement points related 
to traffic volume specific performance metrics

Monitoring of traffic volume related performance metrics could be realized at the following interfaces of the TLS record layer as depicted in Figure B.1:

- interface between the TLS application and the TLS record layer (measurement point "Application Data")

- interface between the TLS protocols (handshake protocol, change cipher spec protocol, alert protocol) and the TLS record layer (measurement point "TLS message")

- Interface between the TLS record protocol and the transport protocol (measurement point "TLS-RL PDU")

Figure B.2 indicates the major processing stages with the TLS Record Layer (TLS-RL). IT may be concluded that there will be no difference between the processing of application data and the processing of TLS messages. The term "TLS-RL User Data" comprises both, "TLS message" and "Application Data" (see also Figure B.1).

Figure B.2 – Data processing in the TLS record layer

In the sending direction the TLS record layer transforms a given amount of TLS-RL User Data into one or more TLS-RL PDUs.

Vice versa one or more received TLS-RL PDUs are transformed into a specific amount of TLS-RL User Data.

For a given amount of TLS-RL user data the resulting amount of TLS-RL PDUs depends on

- the current fragment size, as this dimension determines how many TLS-RL PDUs will be used;

- the compression method applicable for the current security session;

- the TLS version applicable for the current security session (e.g. TLS version 1.1 adds the initialization vector explicitly to the TLS-PDU to protect against CBC attacks);

- the cipher suite applicable for the current security session.

A protected, compressed TLS fragment is consequently of variable size, see layout in Figure B.3:

Figure B.3 – TLS-RL PDU: variable layout of a protected compressed TLS fragment

## B.3	Terminology based on data model

Table B.1 shows the relation between the terms as used in this Recommendation and the TLS presentation language [IETF RFC 5246]:

Table B.1 – TLS presentation language and derived term

| Term used in this Recommendation | TLS Presentation Language [IETF RFC 5246] |
| --- | --- |
| TLS fragment | TLSPlaintext.fragment (NOTE 1) |
| Compressed TLS fragment | TLSCompressed.fragment |
| Protected compressed TLS fragment | TLSCiphertext.fragment |
| TLS-RL PDU | TLSCiphertext |
| TLS-Header | TLSCiphertext.type + TLSCiphertext.version + TLSCiphertext.length |
| NOTE 1 – It is equal to element "fragment" within data structure "TLSPlaintext", but [IETF RFC 5246] does not use explicitly the name "TLSPlaintext.fragment". |  |

Appendix I

Sample use cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use case #I.1 "Terminal-to-MG TLS session, provisioned TLS service negotiation"

This use case is based on the bearer connection use case #1.2 (from clause 6.2.1) and illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, in either case the MGC is aware of the applied TLS network domain profile for the secured bearer traffic.

- IP transport bearer type: 'TCP' is used in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [b-IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS (see also clause 6.2.1, use case #1.2).

- Bearer path protocol layering: "Application/TLS/TCP/IP" (i.e., no tunnelled transport).

- UE: TCP client and TLS client, MG: TCP server and TLS server

- H.248 MG TCP-mode: "TCP proxy".

NOTE – In more detail: L4/L3 NAT traversal support requirements during the establishment phase of TCP bearer connection may initially imply a TCP merge mode (see [ITU-T H.248.84]), which later transitions to a TCP proxy mode (due to the TLS protocol termination here for TLS to non-TLS interworking). Another option is to use the TCP proxy mode from the very beginning, e.g., in case of lacking NAT-T support.

- Mutual TLS-authentication via certificates, MG's certificate is signed, UE's certificate is self-signed.

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG (e.g., in case of an H.248 profile without support of the TLS capability negotiation package, see clause 10).

Figure I.1 – Use-case # I.1: Bearer path TLS security between UE and MG

## I.2	Use case #I.2 "Terminal-to-MG TLS session, MGC-controlled TLS service negotiation"

This use case is an extension of the first use case  #I.1 (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level via usage of the the TLS capability negotiation package (see clause 10). The first use case is based on preconditions which are partially subject of network provisioning. This use case is motivated by the fact that there are a number of reasons which demand MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

## I.3	Use case #I.3 "WebRTC to NGN/IMS interworking function with DTLS-to-TLS support"

WebRTC-based data uses "SCTP/DTLS/UDP/IP" as transport (see also Appendix I.1/[ITU-T H.248.89]. An H.248 WebRTC gateway (see [b-ITU-T H.248.WEBRTC]) could be requested for interworking to "TLS/TCP/IP" based transport, resulting in a DTLS-to-TLS scenario.

## I.4	Use case #I.4 "TLS-based transport security for facsimile packet relay service T.38"

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, - for both connection models -, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation)

## II.1	TLS to non-TLS interworking with TCP as example bearer type

NOTE: TCP is used as an example L4 protocol for TLS. [ITU-T H.248.89] provides further details on TCP bearer connection control procedures and takes precedence in case of discrepancies between both Recommendations.

### II.1.1	Initial session setup using SDP security descriptions in application control signalling

This use case is presented in Appendix I.1. Figure II.1 outlines the network configuration as used in the message flow.

Figure II.1 – Configuration for the example call flow

The scenario shows a simplified SIP session establishment. The TCP bearer connection establishment direction is defined at SIP level by the SDP setup attribute (based on [b-IETF RFC 4145]), and the fingerprint of the UE's certificate is included in the SDP (based on [IETF RFC 4572]).

The H.248 MG is triggered by the MGC to create two terminations, whereby the bearer for the first SEP/termination is secured by the use of TLS. In the RD of this termination the MG also receives the fingerprint of the UE's certificate.

### II.1.2	Traffic flow examples for communication establishment phase

There are multiple options for establishing the communication path for the TLS to non-TLS interworking. The following establishment examples are illustrated:

- Scenario 1: Unclear situation whether remote NAT devices may require L4/L3 NAT-T support, leading to an initial establishment and connectivity check of the end-to-end IP L4 connection before the initiation of the TLS security session. The MGC explicitly triggers (or not) TLS establishment, dependent on a successfully established IP L4 connection (see clause II.1.2.1).

- Scenario 2: NAT-T support seems to be not required, however, the MGC separates the communication establishment between the TLS network domain and non-TLS network domain, e.g., due to separated capability negotiations in each domain at application control level (see clause II.1.2.2).

- Scenario 3: the end-to-end IP L4 connection should be autonomously established by the MG (e.g., because the MGC does not expect any blocking issues such as L4 protocol deadlocks). The MGC requests to be notified about L4 connectivity and then explicitly triggers TLS security session establishment (see clause II.1.2.3).

- Scenario 4: the MGC does not expect any issues at all and may optimize communication establishment by minimizing setup delay times, i.e., enabling full MG autonomous behaviour across the SEPP and vertical protocol layers (see clause II.1.2.4).

NOTE – This is a non-exhaustive list of variants, just a few examples.

#### II.1.2.1	Example: TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection)

The call flow (Figure II.2) shows the setup of the TCP bearer connection as well as the subsequent TLS handshake on the first stream endpoint T1(S1). The MGC initially blocks any TLS security session establishment due to uncertainity whether the underlying TCP bearer connection could resolve all "NAT traversal challenges". After the MG reports a sucessfully established TCP bearer connection, the MGC initiates TLS session establishment. The TLS server role is located at MG side in this example.

The self-signed certificate received from the UE is verified by the use of the fingerprint. After the TLS handshake is finished, application data may flow between the UE and the remote TLS session endpoint. In the traffic flow presented here the MG will forward these data in the TCP proxy mode.

Figure II.2 – Example setup of a TLS to non-TLS IWF 
(TLS establishment decoupled from L4 establishment (decoupled end-to-end L4 connection))

#### II.1.2.2	Example: combined TLS/L4 establishment (decoupled end-to-end L4 connection)

This scenario is similar to clause II.1.2.1, but the successful L4 connections establishment towards the remote TLS endpoint (i.e., UE-X) shall immediately trigger the TLS session establishment without MGC involvement. Such a behaviour is achieved by the interlinkage capability by setting property seplink/linktopo (at SEP T1(S1)), see Figure II.3.

Figure II.3 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment (decoupled end-to-end L4 connection))

The final MG notifications concerning a successfully established end-to-end IP bearer path are optional (and dependent on MGC requesting these notifications).

#### II.1.2.3	Example: TLS establishment decoupled from L4 establishment (but interlinked end-to-end L4 connection establishment)

This scenario is similar to clause II.1.2.1, but provides an interlinked TCP bearer connection establishment. Such behaviour is achieved by using again the interlinkage capability (at both SEPs, if the establishment direction is unknown from the MGC side), see Figure II.4.

Figure II.4 – Example setup of a TLS to non-TLS IWF
(TLS establishment decoupled from L4 establishment, but interlinked end-to-end L4 connection establishment)

#### II.1.2.4	Example: combined TLS/L4 establishment (and also interlinked end-to-end L4 connection establishment)

If possible, then the MGC could delegate the "entire" control of end-to-end IP bearer path establishment down to the MG, by enabling interlinkage at L4-SEPP level as well as between TLS and L4 at SEP T1(S1), see Figure II.5.

Figure II.5 – Example setup of a TLS to non-TLS IWF
(combined TLS/L4 establishment, plus interlinked end-to-end L4 connection establishment)

### II.1.3	Traffic flow examples for communication release phase

There are multiple options for release of the IP bearer path for the TLS to non-TLS interworking scenario. The following release examples are illustrated:

- Scenario 1: The MGC separates TLS and L4 bearer release, e.g. due to possible TLS session resume. Thus, any later IP L4 connection release will be explicitly triggered by the MGC (see clause II.1.3.1).

- Scenario 2: The MGC issues the TLS session release and wants the IP L4 connection release to be initiated by the remote TCP endpoints. E.g., if either UE-X or UE-Y initiates a TCP release activity, then the end-to-end TCP bearer connection should be "MG autonomously" released (see clause II.1.3.2).

- Scenario 3: Similar to the previous scenario, but now MG should immediately start the IP L4 connection release towards the remote TLS/L4 endpoint. (see clause II.1.3.3).

NOTE – This is a non-exhaustive list of variants, just a few examples.

Concerning MG autonomous support with respect to TLS session release procedures, it may be noted that for all release examples it is assumed that any interlinkage is not enabled if not otherwise specified.

#### II.1.3.1	Example: TLS session release initiated by remote TLS endpoint and MGC enforced immediate L4 connection release

The MGC expects an incoming TLS session release (see Figure II.6). UE-X initiates the TLS session release by sending a TLS alert (close_notify) message.

Figure II.6 – Example TLS session release, initiated by remote TLS endpoint 
(MGC enforces immediate L4 connection release)

The MG reports a successfully released TLS session to the MGC, which then decides to release also the underlying TCP bearer connection, towards both remote UEs.

#### II.1.3.2	Example: outgoing TLS session release, local TLS/L4 decoupled, coupled end-to-end L4 connection release

Figure II.7 depicts a situation when the MGC initiates the TLS session release towards UE-X, and configures the MG to autonomously release any possible end-to-end TCP bearer connection release, which might be initiated by one (or both) remote UE(s).

Figure II.7 – Example TLS session release, local TLS/L4 decoupled, 
but coupled end-to-end L4 connection release)

#### II.1.3.3	Example: outgoing TLS session release, coupled TLS/L4 and end-to-end L4 connection release

This scenario is similar to the previous example (clause II.1.3.2), but now the MGC wants to explicitly start TCP bearer connection release (e.g., in order to be independent of remote UEs), see Figure II.8.

Figure II.8 – Example TLS session release, 
coupled TLS/L4 and end-to-end L4 connection release

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

The TLS profile concept is introduced in clause 14. This Appendix illustrates some exemple TLS profiles.

## III.1	Typical Internet TLS profile

TLS is used in many Internet applications. Tables III.1.a and III.1.b indicate an example TLS domain profile.

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | Signed, Self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

[b-ETSI TS 133 203], Annex O defines "Enhancements to the access security to enable TLS". When used, then the network domain security related "TLS domain profile", according to Annex E/[b-ETSI TS 133 310], is prescriptive. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [b-IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | Compression Method.null | - | methods of [b-IETF RFC 3749] | - |
| 2.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [b-IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex E/[b-ETSI TS 133 310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) "TLS_DH_anon_WITH_" (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [b-IETF RFC 4346]. NOTE 3 – Because the "the key exchange method shall not be anonymous". |  |  |  |

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [b-OMA-TS-TLS-V1_0-20080902-A], a "public key TLS domain profile" and a "pre-shared key TLS domain profile".

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [b-IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [b-IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE 1) |  |  |  |
| 3.6 | Extensions: | [b-IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [b-OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | None specified. |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | All others |

### III.3.2	OMA pre-shared key TLS domain profile

See [b-OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

## III.5	IETF example of a national TLS domain profile

[b-IETF RFC 6460] represents an example of a national TLS domain profile. Inter alia, TLS protocol version and ciper suites are profiled.

## III.6	ITU-T TLS domain profile for NGN signalling and management plane

[b-ITU-T Y.2704], clause 9.1, defines a TLS domain profile, recommended for signalling and management interfaces in NGNs.

Appendix IV

Illustration of protocol semantics 
of the TLS basic session control package

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tlsbsc package (clause 8) defines the basic TLS security session control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## IV.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS session endpoint.

## IV.3	Establishment of TLS security sessions

### IV.3.1	Successful establishment – Terminating side

See Figure IV.1, termed as use case (E.1):

Figure IV.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of TLS session establishment (1).

- Early incoming TLS messages are discarded by the MG (2).

- The MGC unblocks the TLS SEP (3).

- TLS session establishment (5 to 7); MG acting as TLS server.

- Optional notification of MGC concerning available TLS session for application data transfer (8), if subscription to event by MGC (4).

### IV.3.2	Successful establishment – Originating side

See Figure IV.2, use case (E.2).

Figure IV.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause IV.3.1

- TLS session establishment (5 to 7), triggered by MGC (4) (please note previous explicit "unblocking" step 3); MG acting as TLS client.

- Optional notification of MGC concerning available TLS session for application data transfer (9), if subscription to event by MGC (8).

### IV.3.3	Unsuccessful establishment

The TLS security session negotiation handshake is basically not completed.

## IV.4	Release of TLS security sessions

### IV.4.1	Successful release – Terminating side

See Figure IV.4, termed as use case (R.1).

Figure IV.4 – Successful release – Terminating side

More details for this example:

- MG receives an incoming TLS close_notify alert (2), the indication for TLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released TLS session (4), if subscription to event by MGC (1).

### IV.4.2	Successful release – Originating side

See Figure IV.5, use cases (R.2) illustrates an outgoing TLS session release:

Figure IV.5 – Successful release – Originating side

More details for this example:

- MGC triggers the TLS session release (1).

- MG sends an outgoing TLS close_notify alert (2), (the indication for TLS session release) towards remote TLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released TLS session (5), if subscribed to the event by MGC (4).

### IV.4.3	Unsuccessful release

An unsuccessful TLS session release procedure would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established TLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

Appendix V

Illustration of the TLS-specific interlinkage procedures

(This appendix does not form an integral part of this Recommendation.)

## V.1	Overview

Clause 9 defines an augmented TLS security session control on top of the base package (tlsbsc, clause 8). The effect of the interlinkage capability is illustrated in clauses V.3 ("inter-SEP interlinkage") and V.4 ("intra-SEP interlinkage"). Clause V.5 describes examples of combined usage of both interlinkage configurations.

The illustrated use cases are only examples. An exhaustive consideration of all possible combinations is out of scope.

## V.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS and L4 layers) are highlighted besides the H.248 interface.

Furthermore some Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS/L4 SEP.

## V.3	Usage of SEPP Interlinkage

Such an inter-SEP interlinkage configuration for TLS affects TLS security session establishment and release procedures. The semantics are correspondent to the TCP layer. See e.g. the illustration and examples in [ITU-T H.248.89], Appendix IV.3.

## V.4	Usage of Protocol Layers Interlinkage

Usage of protocol layers interlinkage impact is depicted in below two illustrations. Figure V.1 recalls again the default mode according base package (tlsbsc):

Figure V.1 – Illustration of Protocol Layers Interlinkage scenario without intra-SEP interlinkage

The MGC is responsible for individual control of layer-specific bearer control procedures.

Figure V.2 illustrates the tightly coupled mode, which is conditional to the aimed communication service, but when applicable, it provides some benefits such as minimal MGC involvement and performance optimized bearer establishment and/or release.

Figure V.2 – Illustration of Protocol Layers Interlinkage property scenario with intra-SEP interlinkage between TLS and L4

### V.4.1	Without intra-SEP interlinkage

Without intra-SEP interlinkage between TLS and L4 implies base package behaviour.

### V.4.2	Using intra-SEP interlinkage

Intra-SEP interlinkage may be applied for the establishment or release phase or both (with some restrictions).

#### V.4.2.1	Bearer establishment phase

##### V.4.2.1.1	Incoming direction

Figure V.3 illustrates the normal case: the start of any TLS security session establishment is preceded by a successfully established IP transport connection. Thus, the MG receives an incoming L4 Est.req (2). When L4 is established, then immediately an outgoing TLS session establishment (4) is initiated by the MG.

Figure V.3 – Correct "TLS-L4" interlinkage configuration for establishment – L4 incoming bearer establishment request

The actually impossible case of TLS establishment request first, without yet successfully established IP transport, is indicated Figure V.4.

Figure V.4 – Correct "TLS-L4" interlinkage configuration for establishment, butTLS incoming bearer establishment request first

##### V.4.2.1.2	Outgoing direction

SEP T1(S1) initiatesan outgoing L4 bearer establishment procedure (see Figure V.5). Such a L4-related stimuli may be received from the MGC (1) or from the partner SEP (due to correspondent inter-SEP interlinkage settings). After successful IP transport connection establishment (3, 4), the MG immediately initiates an outgoing TLS session establishment (5).

Figure V.5 – Correct "TLS-L4" interlinkage configuration for establishment – L4 stimuli

Figure V.5 shows also the optional event notifications related to bearer specific state transitioning (here: L4 and TLS reporting).

There might be also a TLS-related stimuli first from SEP perspective (without a prior explicit L4 trigger). However, leading to an unsuccessful scenario, replied by an error code, see Figure V.6:

Figure V.6 – Correct "TLS-L4" interlinkage configuration for establishment, but incorrect – TLS stimuli

#### V.4.2.2	Bearer release phase

##### V.4.2.2.1	Incoming direction

Figure V.7 illustrates the abnormal case: the start of any IP transport connection release precedes a successfully released TLS security session. The MG receives an incoming L4 Rel.req (2), but would suspend L4 release procedural steps in favour of a TLS session release (3) first. The L4 release procedure is resumed again (5) after successful TLS termination (4). However, such a scenario is beyond the semantic of intra-SEP interlinkage, therefore not supported!

Figure V.7 – Correct "TLS-L4" interlinkage configuration for release, but L4 incoming bearer release request first

The standard case of TLS release request first is depicted in Figure V.8: the TLS session release (2, 3) triggers the subsequent IP transport connection release (4, 6).

Figure V.8 – Correct "TLS-L4" interlinkage configuration for release – TLS incoming bearer release request

##### V.4.2.2.2	Outgoing direction

SEP T1(S1) initiates an outgoing L4 bearer release procedure (see Figure V.9). Such a L4-related stimuli may be received the partner SEP (1) (due to correspondent inter-SEP interlinkage settings) (but not from the MGC!). After successful TLS session release (3, 4), the MG immediately initiates an outgoing IP transport connection release (5).

Figure V.9 – Correct "TLS-L4" interlinkage configuration for release, but L4 stimuli first

Above scenario in Figure V.9 is feasible when using a correct inter-SEP interlinkage configuration by connecting source "TCP at partner SEP" to destination "TLS at T1(S1)".

Figure V.10 outlines the case of a TLS-related stimuli (here originated from MGC, but might be also from partner SEP T2(S1)):

Figure V.10 – Correct "TLS-L4" interlinkage configuration for release TLS stimuli

Appendix VI

TLS alert protocol from H.248 gateway perspective

(This appendix does not form an integral part of this Recommendation.)

## VI.1	Background

TLS [IETF RFC 5246] provides the specific protocol capability of an embedded alert information channel, defined as TLS alert protocol. The TLS alert protocol covers a variety of different alert services, which itself lead to "TLS alert"-specific handling in H.248 gateways.

## VI.2	H.248 gateway framework concerning TLS alert handling

The TLS alert protocol defines two fundamental alert categories:

- Closure alert; used in normal service scenarios ("TLS session release"), thus covered by the TLS basic session control package (see clause 8.6.5.1); and

- Error alerts, used in abnormal situations, subject of the TLS session maintenance package (see clause 11).

There are around 25 alert types within the error alert category. Furthermore, the TLS alert protocol defines two severity levels for further qualification of each alert type:

- Warning level: alert provides only a warning indication, the communication should/could continue; and

- Fatal level: continuation of communication not possible, the TLS session shall be released.

There are consequently three dimensions in structuring TLS alert activities: a) alert category, b) alert type and c) alert severity.

Further, H.248 media gateways may be involved in a variety of different TLS use cases (see clause 6) and interworking models (see clause 7), leading to the question concerning the likely location of the served user instance, responsible for processing the particular TLS alert information. There are three options in case of TLS protocol handling by a distributed architecture such as H.248 gateways:

- MG: the MG itself processes and terminates an incoming TLS alert or sents a TLS alert to the remote TLS endpoint, without any involvement of other remote control/management plane elements. Thus, it is a scenario with MG local TLS alert processing.

- MGC: either, the MGC may trigger the MG to sent a specific TLS alert to the remote TLS endpoint, e.g. due to security aspects, application logic, etc.; or the MGC would be notified by the MG by incoming TLS alert information (Note: which again may be mapped to an [ITU-T H.248.8] error code).

- NMS/EMS (here: element management system of MG): e.g., in case of particular TLS alerts resulting in fault or security management actions.

Above high level evaluation defines a framework for TLS alert handling by H.248 gateways.

## VI.3	TLS error alerts categorization

TLS error alerts are defined in detail by clause 7.2.2/[IETF RFC 5246]. The following observations could be made:

- roughly 70 % of the (ca. 25) error alerts are of fatal severity, i.e., shall trigger the release of the TLS session;

- there are two error alerts which may appear only as warnings; 
and

- a few error alerts are related to "older TLS versions".

## VI.4	Summary

The individual handling of specific TLS error alerts should be implementation specific and is out of scope of this Recommendation. However, as outlined in clause VI.2, a general mechanism is supported by this Recommendation (by the tlsm package, clause 11) which allows to notify the MGC about received TLS alert information by the MG, - for instance the MGC could be notified about the fatal severity in order to initiate the release of a TLS session -, and the MGC would be able to trigger the MG in sending specific TLS alerts (e.g., due to application specific requirements).

Appendix VII

TLS session resumption – Framework

(This appendix does not form an integral part of this Recommendation.)

## VII.1	Introduction

Support of TLS session resumption is principally feasible in network solutions with H.248 entities, but not straightforward due to the typical nature of communication services supported by H.248 profiles (e.g., given by characteristics of call-dependent gateway control activities, call-to-bearer mapping models, limited call holding times, etc.).

This Recommendation does not provide H.248 support for the control of TLS session resumption.

The purpose of this Appendix is to provide background information, example use cases and to identify typical operational constraints. H.248 support for the control of TLS session resumption is for further studies.

## VII.2	Brief summary of TLS session resumption

TLS session resumption is a feature allowing the reuse of a recently established TLS security session. This results in an abbreviated TLS handshake (instead of a full TLS handshake) for a new TLS session by avoiding the computationally costly calculation of the TLS master secret.

Hence, the principle benefit of TLS session resumption is twofold: a) reduced utilization of network resources as well as b) optimized TLS session setup times.

TLS session resumption is only applicable for TLS sessions between a given TLS server/client pair.

## VII.3	Example use cases

The notion of "resumable session" refers to a TLS security session for which (after the handshake has been completed) the security context has been cached. The notion of "resumed session" refers to TLS sessions that make use of the cached security context of resumable sessions, thus resumed sessions utilize an abbreviated handshake.

TLS session resumption may be applied in different directions:

- parallel usage: multiple TLS-enabled bearer connections in parallel (see clause VII.3.1); or

- serial usage: a single TLS-enabled bearer connection is interruped and resumed again (see clause VII.3.2).

NOTE to call control signalling in examples:

The SIP is used as an example call control protocol. There is always a single end-to-end call between the two UEs X and Y. Resumed TLS sessions belong to the same, original call. A single SIP call may be given by a SIP dialog (as identified by the SIP Call-ID header value). Whenever multiple "SIP session" instances appear in the following use case examples, then they belong to a single SIP call.

### VII.3.1	Use case: "File transfer session initiated while a chat session is active"

This use case is part of the so called Rich Communication Suite (RCS), see [b-GSMA-RCS.07]. Following quote provides a brief summary:

"3.5.2 Interaction with other RCS features

A 1-to-1 File Transfer is not linked to other services (for example CS-voice call or ongoing chat session) and can be used either during or outside of other communication sessions.

The procedure for any file transfer within an ongoing 1-to-1 chat session is implemented as a separate session in parallel with the ongoing 1-to-1 chat and therefore is the same as the procedure for initiating a separate session for File Transfer."

The use case presented focuses on the interface between UE-X and the MG-1 ("which may serve as an access gateway"). It can be characterized as follows:

- E2ae security is applied between UE-X and MG-1, i.e. the MSRP sessions between the UE-X and the MG-1 are secured by TLS.

- Initially, a single end-to-end MSRP chat session is established between two UEs.

- During the active chat session a file transfer session is established between the same participants.

- File transfer via MSRP is used (see [b-IETF RFC 5547]).

Figure VII.1 shows a simplified network configuration after the chat session between UE-X and UE-Y is established.

Figure VII.1 – MSRP-based instant messageing – Established chat session between two UEs

During the chat session both UEs decide to setup an additional session for a file transfer. Figure VII.2 shows the network configuration after the file transfer session is established.

Figure VII.2 – Parallel MSRP/TLS sessions for chat and file transfer between two UEs

In case the MSRP sessions from the UE to the MG are secured by means of using TLS (such as the e2ae security model) TLS session resumption may be used to setup the TLS session for the file transfer. Figure VII.3 illustrates the correspondent half-call model:

Figure VII.3 – End-to-access edge (e2ae) security model: potential TLS session resumption

To apply TLS session resumption at least the following conditions must be true:

C1: 	Both TLS sessions are established between the same TLS client/server pair.

C2: 	TLS session resumption is supported by the UE.

C3: 	TLS session resumption is supported by the MG (note: the support level concerning root only or non-root in addition is implementation specific).

C4: 	TLS client/server role of the resumed session is the same than for the previous session.

The abstracted SIP and H.248 signalling flow and TLS handshake is illustrated in Figure VII.4:

Figure VII.4 – Scenario for TLS session resumption for the file transfer

Figure VII.4 underlines that the TLS security context for the chat session is saved locally at UE-X and/or at the MG-1.

When the new TLS session for the file transfer is setup, the security context from the chat session is retrieved at the UE-X and/or MG-1, thus session resumption can be used for the abbreviated TLS handshake. After the TLS handshake for the file transfer is finished, two TLS sessions between UE-X and MG-1 are active in parallel.

### VII.3.2	Use case: "Resumption of an interrupted file transfer"

In the previous use case TLS session resumption was applied to resume a currently active TLS session, thus resulting into two established TLS sessions in parallel between the same endpoints.

In the use case presented here TLS session resumption is used to resume a session which has beenclosed. The use case can be characterized as follows:

- A 1-to-1 file transfer is established between UE-X and UE-Y

- E2ae security is applied between UE-X and MG-1, i.e. the MSRP sessions between the UE-X and the MG-1 are secured by TLS.

- The file transfer between UE-X and UE-Y fails for any reason, the media session as well as the signalling dialog is terminated.

- The file transfer is resumed by establishing a new signalling dialog and a new media session.

The scenario (see Figure VII.5) differs from the previous use case in the sense that the two TLS sessions are setup sequentially.

Figure VII.5 – Scenario for resuming an interrupted file transfer

## VII.4	Conclusions and guidelines

From MG perspective constraints apply to setup a resumable session and a resumed session.

Constraints for a resumable session:

- MG internal constraints:
The capability for TLS session resumption must be supported in the MG. An implementation that does not support caching of a TLS security context can still support session resumption on the server side if session tickets as specified in [b-IETF RFC 5077] are used. Therefore the capability to support session resumption might be different for server- and client-side implementation and it may depend on the supported method used for session resumption (session identifiers vs. session tickets).

- Constraints defined by network service control (via H.248 interface):
Usage of TLS session resumption may be dependent on security policies, service level agreements, communication service type, etc. The MGC may consequently need to disallow a session to be resumable (either call-individually or in general).

- Constraints enforced by the TLS protocol:
Cases are defined in the base TLS RFCs which explicitly forbid a session to be resumable.

Constraints for resumed sessions:

- MG internal constraints:
These are the same constraints as defined above for resumable sessions.

- Constraints defined by network service control (via H.248 interface):
As above, usage of TLS session resumption may be dependent on security policies, service level agreements, communication service type, etc. The MGC may consequently need to disable resumption.

- Constraints enforced by the TLS protocol:

- A resumable session must exist (cached session identifier available, session ticket available, resumable session not expired, certificate not expired, etc)

- Both TLS endpoints must support the same method for session resumption (session identifer vs. session tickets)

Bibliography

[b-ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[b-ITU-T H.248.WEBRTC]	Draft Recommendation ITU-T H.248.WEBRTC (2014), Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles.

[b-ITU-T Q.1950]	Recommendation ITU-T Q.1950 (12/2002), Bearer independent call bearer control protocol.

[b-ITU-T T.38]	Recommendation ITU-T (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[b-ITU-T V.153]	Recommendation ITU-T V.153 (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T Y.2704]	Recommendation ITU-T Y.2704 (01/2010), Security mechanisms and procedures for NGN.

[b-IETF RFC 3749]	IETF RFC 3749 (05/2004), Transport Layer Security Protocol Compression Methods

[b-IETF RFC 4145]	IETF RFC 4145 (09/2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[b-IETF RFC 4346]	IETF RFC 4346 (04/2006), The Transport Layer Security (TLS) Protocol Version 1.1.

[b-IETF RFC 4366]	IETF RFC 4366 (04/2006), Transport Layer Security (TLS) Extensions.

[b-IETF RFC 5077]	IETF RFC 5077 (01/2008), Transport Layer Security (TLS) Session Resumption without Server-Side State.

[b-IETF RFC 5547]	IETF RFC 5547 (05/2009), A Session Description Protocol (SDP) Offer/Answer Mechanism to Enable File Transfer.

[b-IETF RFC 6460]	IETF RFC 6460 (2012), Suite B Profile for Transport Layer Security (TLS).

[b-IETF tls-attacks]	IETF draft-sheffer-uta-tls-attacks-00 (2014), Summarizing Current Attacks on TLS and DTLS.
URL: http://tools.ietf.org/id/draft-sheffer-uta-tls-attacks-00.txt

[b-IETF tls-minimal]	IETF draft-ietf-lwig-tls-minimal (2014), A Hitchhiker's Guide to the (Datagram) Transport Layer Security Protocol for Smart Objects and Constrained Node Networks.
URL: http://tools.ietf.org/html/draft-ietf-lwig-tls-minimal-01

[b-ETSI TS 133 203]	ETSI TS 133 203 V11.2.0 (2012-11), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; 3G security; Access security for IP-based services (3GPP TS 33.203 version 11.2.0 Release 11).

[b-ETSI TS 133 310]	ETSI TS 133 310 V11.2.0 (2013-02), Universal Mobile Telecommunications System (UMTS); LTE; Network Domain Security (NDS); Authentication Framework (AF) (3GPP TS 33.310 version 11.2.0 Release 11).

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-OMA-TS-TLS-V1_0-20080902-A]	Open Mobile Alliance OMA-TS-TLS-V1_0-20080902-A (09/2008), OMA TLS Profile.

[b-GSMA-RCS.07]	GSM Association RCC.07 (09/2013), Rich Communication Suite 5.1 Advanced Communications Services and Client Specification, Version 4.0.

____________________

Defined as: "A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint." ([IETF RFC 6714]).