| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 231 (PLEN) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June - 11 July 2014 |
| TD |  |  |
| Source: | Editor H.248.93 |  |
| Title: | Consent: H.248.93 (ex H.248.DTLS) "Gateway control protocol: H.248 support for control of transport security using DTLS" (New) |  |

1.	Introduction

This document contains draft new H.248.93 (ex H.248.DTLS) "Gateway control protocol: H.248 support for control of transport security using DTLS" for Consent.

NOTE – A.5 justification in TD 210/Plen.

AAP Summary

This Recommendation provides information regarding Datagram Transport Layer Security (DTLS) protocol support by H.248 entities and focuses on the reuse of "H.248 TLS packages" (according to Recommendation ITU-T H.248.90) for DTLS. This Recommendation defines a H.248 package extension to the TLS capability negotiation package for the support of DTLS-SRTP sessions.

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 7 5 Conventions 8 5.1 Conventions used in signalling flows 8 5.2 DTLS endpoint notations 8 6 Use case descriptions 9 6.1 Use cases related to DTLS transport modes 9 6.2 Bearer connection network use cases with H.248 IP-IP gateways 9 6.2.1 High-level use case categories 9 6.2.2 Use case variations 11 6.3 Bearer connection network use cases with DTLS transport mode change 11 6.4 Bearer connection network use cases with multiparty services 11 7 Models 11 7.1 Network model from H.248 entity point of view 11 7.2 Bearer connection model 12 8 Basic session control package (for DTLS) 12 9 DTLS-specific stream endpoint interlinkage procedures 13 10 Capability negotiation package (for DTLS) 14 11 DTLS extended capabilities package 14 11.1 Properties 15 11.1.1 DTLS-SRTP protection profiles 15 11.1.2 Master Key Identifier usage 15 11.2 Events 16 11.3 Signals 16 11.4 Statistics 16 11.5 Error codes 16 11.6 Procedures 16 11.6.1 Extension versus base package 16 11.6.2 Principles of capability negotiations 16 11.6.3 DTLS protocol parameter "DTLS-SRTP protection profiles" 16 11.6.4 DTLS protocol parameter "Master Key Identifier" 16 12 Session maintenance package (for DTLS) 17 13 Traffic volume metrics package (for DTLS) 17 14 Package-less DTLS control 18 14.1 Related to DTLS authentication 18 I.1 Use cases for "application protocol agnostic DTLS handling" 19 I.2 Use cases for "DTLS-based transport security for facsimile packet relay service T.38" 20 I.3 Use cases for "WebRTC data traffic" 22 I.4 Use cases for "DTLS-based key exchange for SRTP" 24 I.4.1 Two considered key exchange options for SRTP 25 I.4.2 H.248 impact by DTLS-SRTP 26 II.1 Overview 26 II.2 Conventions 26 II.3 Establishment of DTLS security sessions 26 II.3.1 Successful establishment – Terminating side 26 II.3.2 Successful establishment – Originating side 27 II.3.3 Unsuccessful establishment 28 II.4 Release of DTLS security sessions 28 II.4.1 Successful release – Terminating side 28 II.4.2 Successful release – Originating side 29 II.4.3 Unsuccessful release 30

List of Tables

Page

Table 1 – Basic session control package (for DTLS) 13 Table 2 – DTLS-specific stream endpoint interlinkage procedures 13 Table 3 – Intra-SEP interlinkage options for DTLS 14 Table 4 – Capability negotiation package (for DTLS) 14 Table 5 – Session maintenance package (for DTLS) 17 Table 6 – Traffic volume metrics package (for DTLS) 18

List of Figures

Page

Figure 1 – Conventions for DTLS endpoint types 8 Figure 2 – DTLS transport modes 9 Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways 10 Figure 4 – Network model from H.248 entity point of view ("half call/bearer connection model") 11 Figure 5 – L4/IP protocol stack with DTLS protocol 12 Figure 6 – Two-termination context with a DTLS termination 12 Figure I.1 – Use cases for "application protocol agnostic DTLS handling" 20 Figure I.2 – Use cases for "DTLS-based transport security for facsimile packet relay service T.38" 22 Figure I.3 – Use cases for "WebRTC data traffic" 24 Figure I.4 – Media-path decoupled key exchange using SDP (via SIP and H.248) 25 Figure I.5 – Media-path coupled key exchange using DTLS 25 Figure II.1 – Successful establishment – Terminating side 27 Figure II.2 – Successful establishment – Originating side 28 Figure II.3 – Successful release – Terminating side 29 Figure II.4 – Successful release – Originating side 29

Draft new ITU-T Recommendation H.248.93 (ex H.248.DTLS)

Gateway control protocol: 
H.248 support for control of transport security using DTLS

Summary

Datagram Transport Layer Security (DTLS) is a session layer protocol for securing IP transport protocols. DTLS bearer plane traffic could be terminated or forwarded by ITU-T H.248 media gateways. DTLS is derived from the Transport Layer Security (TLS) protocol. This Recommendation provides information for DTLS support by H.248 entities with focus on the reuse of "H.248 TLS packages" (according to [ITU-T H.248.90]) for DTLS. This Recommendation defines a H.248 package extension to the TLS capability negotiation package for the support of DTLS-SRTP sessions.

Keywords

H.248, TLS, DTLS

# 1	Scope

The DTLS (Datagram Transport Layer Security) protocol [b-IETF RFC 4347] and [IETF RFC 6347] is derived and thus aligned with the TLS (Transport Layer Security) protocol [IETF RFC 5246]. There are consequently many commonalities between the control of DTLS bearers and TLS bearers in H.248 gateways.

H.248-controlled TLS bearers are subject of [ITU-T H.248.90] and [ITU-T H.248.91].

The purpose of this Recommendation is to define usage of [ITU-T H.248.90] for DTLS bearers.

In scope of this Recommendation are in particular

- description of DTLS specific use cases;

- modelling information;

- description of MG bearer plane differences between DTLS and TLS;

- usage of TLS-defined H.248 packages for DTLS bearer types; and

- an extension package for the specific application of DTLS-SRTP [IETF RFC 5764].

Appendix I provides a non-exhaustive list of example use cases for DTLS in two slightly different areas of applications:

- DTLS as "transport security" means for "DTLS-over-L4" or "L4-over-DTLS" IP bearer traffic; and

- DTLS as "key exchange" means for RTP/L4/IP bearer traffic using media security according the SRTP.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (07/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.91]	Recommendation ITU-T H.248.91 (07/2014), Gateway control protocol: Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles.

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T X.200]	Recommendation ITU-T X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 6347]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.2.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	DTLS transparent forwarding: MG packet forwarding behaviour with the characteristic of DTLS-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a DTLS-PDU flow.

NOTE 1 – A DTLS PDU relates to a DTLS message in [IETF RFC 5246].

NOTE 2 – Definition based on clause 3.1.1, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of DTLS message integrity in the context of "DTLS transparent forwarding". The MG might be DTLS aware; e.g. support of DTLS related statistics or event detection would not violate transparent forwarding behaviour.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DCCP | Datagram Congestion Control Protocol |
| --- | --- |
| DTLS | Datagram Transport Layer Security |
| IFP | Internet Facsimile Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MKI | (SRTP) Master Key Identifier |
| PSTN | Public Switched Telephone Network |
| RTP | Real-time Transport Protocol |
| SCTP | Stream Control Transmission Protocol |
| SDES | SDP security Descriptions |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SRTP | Secure RTP |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TPKT | (T.38) Transport Protocol Data Unit Packet |
| UDP | User Datagram Protocol |
| UDPTL | (Facsimile) UDP Transport Layer (protocol) |
| WebRTC | Web-based Real-Time Communication = Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

## 5.1	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| L4 Est.req L4 Est.ack | Abstracted (protocol-independent) representation for establishment requests/acknowledgements of new connection-oriented IP transport connections. |
| --- | --- |
| L4 Rel.req L4 Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing connection-oriented IP transport connections. |
| DTLS Est.req DTLS Est.ack | Abstracted (DTLS message/procedure independent) representation for establishment requests/acknowledgements of new DTLS security sessions. |
| DTLS Rel.req DTLS Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing DTLS security sessions. |

## 5.2	DTLS endpoint notations

The notion of endpoint represents different concepts, see Figure 1:

Figure 1 – Conventions for DTLS endpoint types

Usage in

- H.248 control: H.248 terminations/stream endpoint with DTLS processing are denoted as DTLS-enabled termination or stream endpoint (SEP) respectively;

- user plane (DTLS): a DTLS bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., DTLS protocol termination) plus a service access point (i.e., L4+ access).
Furthermore, DTLS is a client/server type of protocol, a "DTLS endpoint" provides thus either a client or server role.

The Stream Endpoints (SEP) subject of the packages and procedures of this Recommendation may act as a DTLS bearer connection endpoint or not:

- The basic session control package (for DTLS, see clause 8), the capability negotiation package (for DTLS, see clause 10), and the session maintenance package (for DTLS, see clause 12) are related to [ITU-T H.248.90], hence assumes a SEP that is a DTLS bearer connection endpoint.

- The traffic volume metrics package (for DTLS, see clause 13) (related to [ITU-T H.248.90]) is applicable both for SEPs that are DTLS bearer connection endpoints and SEPs which are not.

- The usage of the Stream endpoint interlinkage package [ITU-T H.248.92] for DTLS (see clause 9) assumes a SEP that is a DTLS bearer connection endpoint.

- The DTLS extended capabilities package (see clause 11) assumes a SEP that is a DTLS bearer connection endpoint.

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to DTLS transport modes

The DTLS protocol is independent of the underlying transport protocol stack. Multiple DTLS transport modes are possible (Figure 2).

Figure 2 – DTLS transport modes

This Recommendation is not dependent on any DTLS transport mode, but uses DTLS-over-UDP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-DTLS or different DTLS protocol stack variants, leading to various bearer network connection use cases (see Figure 3):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection without any applied transport security (abbreviated as "non-DTLS to non-DTLS");

- Use case #1.2: an end-to-end L4 connection traversing two network domains with different transport security policies ("DTLS", "no DTLS"). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as "DTLS to non-DTLS");

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end L4 connection with applied transport security (abbreviated as "DTLS to DTLS");

- Use case #1.4: there is end-to-end transport security, but different DTLS profiles (abbreviated as "DTLS to DTLS*");

Figure 3 – Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.2	Use case variations

#### 6.2.2.1	"DTLS-to-DTLS transparent forwarding"

This scenario is a variation of use case #1.3 and requires the MG behaviour of DTLS transparent forwarding according clause 3.2.1.

## 6.3	Bearer connection network use cases with DTLS transport mode change

The L4 transport protocol could change, e.g., an H.248 MG enabled for DTLS/UDP to DTLS/DCCP interworking.

## 6.4	Bearer connection network use cases with multiparty services

A multipart communication services leads to connection models with more than two H.248 terminations. Such gateway topologies provide normally application aware type of functions (e.g. as media server), thus DTLS sessions would be terminated by the MG. It results in a type of "DTLS to DTLS*" interworking, such as use case #1.4 (clause 6.2.1).

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 4) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a DTLS-capable IP host remote endpoint. Both DTLS endpoints span a network transport security domain (here DTLS domain).

NOTE  – There might be an additional network interface in case of dedicated, centralized network servers in the DTLS domain with respect to key distribution (such as key management systems). Such kinds of interfaces are out of scope of this Recommendation.

Figure 4 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of DTLS level  mode(s) of operation in the H.248 MG; and

- configuration and procedures of the DTLS/L4/IP protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

Figure 5 provides the L4/IP protocol stack with the suite of DTLS protocols and their sub-layer organization.

Figure 5 – L4/IP protocol stack with DTLS protocol

Figure 6 details the generic connection-model where a DTLS-enabled termination is connected to a single other termination (either DTLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 6 – Two-termination context with a DTLS termination

# 8	Basic session control package (for DTLS)

See Table 1.

Table 1 – Basic session control package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS basic session control package" (tlsbsc) according to [ITU-T H.248.90] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. The same state model applies, see Annex A. The DTLS protocol internal procedural differences are not visible from H.248 interface perspective, with regards to the basic blocking, establishment and release of a DTLS session. |
| Limitations, modifications, special considerations? | None. |
| Further comments? | DTLS supports additional protocol extensions (e.g.. sequence numbers, timers) in order to address unreliable L4 transport protocols. Thus, DTLS itself provides an assured communication service, without any impact on the H.248 interface. |

# 9	DTLS-specific stream endpoint interlinkage procedures

See Table 2.

Table 2 – DTLS-specific stream endpoint interlinkage procedures

| Reusable H.248 package? | Yes, the "Stream endpoint interlinkage package" (seplink) according to [ITU-T H.248.92] shall be basically used for DTLS, too. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | Yes. - DTLS transport modes (see clause 6.1): The interlinkage between DTLS and the underlying L4 protocol is limited, actually L4 protocol dependent (due to support of connectionless (UDP) and connection-oriented (SCTP, DCCP) L4 protocols). - DTLS position in IP protocol stacks: The DTLS layer could be located on top of an L4 transport protocol (as usual), but also vice versa according to existing DTLS applications. Table 3 indicates the principle intra-SEP interlinkage options. |
| Limitations, modifications, special considerations? | None. |
| Further comments? | None. |

Table 3 – Intra-SEP interlinkage options for DTLS

|  | Intra-SEP interlinkage options for DTLS: |  |  |
| --- | --- | --- | --- |
|  | No. | <source transport EP> | <interlinked tranport EP> |
|  | 1a | DTLS | lower layer EP |
|  | 1b | lower layer EP | DTLS |
|  | 2a | DTLS | upper layer EP |
|  | 2b | upper layer EP | DTLS |

# 10	Capability negotiation package (for DTLS)

See Table 4.

Table 4 – Capability negotiation package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS capability negotiation package" (tlscn) according to [ITU-T H.248.90] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No for native DTLS sessions. The DTLS protocol internal procedural differences are not visible from H.248 interface perspective. Yes for DTLS-SRTP sessions. There are DTLS protocol extensions in case of DTLS-SRTP [IETF RFC 5764] (see clause I.4.1), leading to additional H.248 elements for (related "DTLS-SRTP protection profiles" and "SRTP Master Key Index". See below clause 11. |
| Limitations, modifications, special considerations? | None. |
| Further comments? | None. |

# 11	DTLS extended capabilities package

|  | Package Name: | DTLS extended capabilities package |
| --- | --- | --- |
|  | Package ID: | dtlscn (0x011e) |
|  | Description: | Native DTLS sessions may be negotiated solely on basis of package tlscn (as base package) according to [ITU-T H.248.90]. DTLS sessions for the purpose of key exchange for SRTP sessions, so called DTLS-SRTP [IETF RFC 5764], need the consideration of additional (D)TLS protocol parameters. |
|  | Version: | 1 |
|  | Extends: | tlscn (0x0118) version 1 |

## 11.1	Properties

## 11.1.1	DTLS-SRTP protection profiles

|  | Property Name: | DTLS-SRTP protection profiles |
| --- | --- | --- |
|  | Property ID: | dspp (0x0009) |
|  | Description: | This property indicates the protection profiles for DTLS-SRTP, according to clause 4.1.2/ [IETF RFC 5764] "SRTP Protection Profiles", and their precedence (in decending order of preference) for the negotiation with the remote DTLS endpoint. |
|  | Type: | Sublist of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the SRTP protection profile in accordance to the DTLS-SRTP Protection Profiles registry of IANA (http://www.iana.org/assignments/srtp-protection/srtp-protection.xhtml#srtp-protection-1). The first value of the value pair as defined by the IANA registry shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The "0x" shall be omitted and if needed, a "0" is used for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned NOTE – A particular DTLS protocol version may define default value(s). Thus, an H.248 profile which defines a default version for DTLS could also specify default value(s) for this property. |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 11.1.2	Master Key Identifier usage

|  | Property Name: | Master Key Identifier usage |
| --- | --- | --- |
|  | Property ID: | mkiu (0x000a) |
|  | Description: | This property indicates the use or not of the DTLS-SRTP specific Master Key Identifier (MKI) (according to [IETF RFC 5764], clause 4.1.3 "srtp_mki value"). |
|  | Type: | Boolean |
|  | Possible values: | False MKI shall not be used True MKI shall be used |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

## 11.2	Events

None.

## 11.3	Signals

None.

## 11.4	Statistics

None.

## 11.5	Error codes

None.

## 11.6	Procedures

### 11.6.1	Extension versus base package

This extension package would typically be used in conjunction with the base package "tlscn" (clause 10/[ITU-T H.248.90]), however, it could be also applied exclusively when all base package elements would use a default or provisioned value.

### 11.6.2	Principles of capability negotiations

The same principles as of the base package (clauses 10.6.1 to 10.6.3 of [ITU-T H.248.90]) apply for the extension package.

### 11.6.3	DTLS protocol parameter "DTLS-SRTP protection profiles"

Property dspp defines an additional H.248 signalling element for the support of the DTLS protocol and shall be used in alignment with clause 10.6.6/[ITU-T H.248.90]. It has to be noted that there are no interactions with the base package properties because the DTLS protocol extension parameter "DTLS-SRTP protection profiles" is orthogonal to the (D)TLS protocol parameter "TLS Cipher Suites" (see clause 10.1.3/[ITU-T H.248.90]).

### 11.6.4	DTLS protocol parameter "Master Key Identifier"

#### 11.6.4.1	Usage for DTLS-SRTP bearers

Property mkiu defines an additional H.248 signalling element for the support of the Master Key Identifier (MKI) for DTLS-SRTP. The srtp_mki parameter is part of the DTLS UseSRTPData object (together with parameter SRTPProtectionProfiles, see clause 11.6.3), and the srtp_mki value contains the SRTP Master Key Identifier (MKI) value (if any) that the client will use for his SRTP packets.  If this field is of zero length, then no MKI will be used.

Usage of an MKI or not for a particular DTLS-SRTP session is not indicated at call control signalling (such as SIP/SDP). Signaling property mkiu at the H.248 interface from MGC to MG would be thus based on a MGC-local policy.

#### 11.6.4.2	Comment to "DTLS-SRTP vs SDES-SRTP"

The MKI for SRTP is transferred along the media-path in case of DTLS-SRTP, and transferred via the signalling path (SIP, H.248) in case of SDES-SRTP ([b-IETF RFC 4568] "SDP Security Descriptions for Media Streams").

Gateway control interface:

- SDES-SRTP uses [b-ITU-T H.248.77] in combination with the SDP "a=crypto:" attribute. The MKI in this case is a SDP attribute parameter.

- DTLS-SRTP uses this Recommendation (i.e., the the SDP "a=crypto:" attribute is not used).

- Hence, both methods are mutually exclusive, an H.248-controlled SRTP stream endpoint uses one option.

# 12	Session maintenance package (for DTLS)

See Table 5.

Table 5 – Session maintenance package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS session maintenance package" (tlsm) according to [ITU-T H.248.90] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. There might be different alerts, but the DTLS protocol internal procedural differences do not impact the H.248 package design. |
| Limitations, modifications, special considerations? | None. |
| Further comments? | None. |

# 13	Traffic volume metrics package (for DTLS)

See Table 6.

Table 6 – Traffic volume metrics package (for DTLS)

| Reusable H.248 package? | Yes, the "TLS traffic volume metrics package" (tlstv) according to [ITU-T H.248.90] shall be used for DTLS. |
| --- | --- |
| H.248: DTLS specific differences versus TLS? | No. The TLS data model could be reused for DTLS (see Annex B). |
| Limitations, modifications, special considerations? | None. |
| Further comments? | None. |

# 14	Package-less DTLS control

## 14.1	Related to DTLS authentication

The SDP "a=fingerprint:" attribute (according to [IETF RFC 4572]) shall be used according to clause 13.2/[ITU-T H.248.90].

Annex A

State modelling for 
DTLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

The same state model as for TLS applies for DTLS, see Annex A.3/[ITU-T H.248.90].

Annex B

DTLS protocol layer: Data model

(This annex forms an integral part of this Recommendation.)

The same data model as for TLS applies for DTLS, see Annex B/[ITU-T H.248.90].

Appendix I

Sample use cases of DTLS bearer encryption

(This appendix does not form an integral part of this Recommendation.)

This Appendix illustrates some network level scenarios that employ DTLS bearer encryption.

## I.1	Use cases for "application protocol agnostic DTLS handling"

The MG is unaware of the IP application protocol carried by DTLS packets. There are some basic interworking modes (see Figure I.1), from perspective of UDP and TCP based data transports:

- use case #I.1.1: MG provides interworking between "DTLS/UDP/IP" to "TCP/IP";

- use case #I.1.2: MG provides interworking between "DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.1.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

## I.2	Use cases for "DTLS-based transport security for facsimile packet relay service T.38"

The IP application protocol is given by T.38 [b-ITU-T T.38], which relates to the Internet Facsimile Protocol (IFP) plus an underlying application level framing protocol such as UDPTL (Facsimile UDP Transport Layer).

Background:

[b-ITU-T T.38] defines three transport modes (T.38 over UDPTL/UDP, RTP/UDP or TPKT/TCP), but does not provide any recommendation which mode should be selected when integrity and confidentiality protection is required.

Use case "T.38-over-UDPTL/DTLS/UDP" relates to the UDPTL transport mode and is characterized by following aspects:

- the MG type would follow either a PSTN-to-IP connection model, such as access or trunking gateways in PSTN emulation solutions, or an IP-to-IP connection model in case of T.38 interworking according [b-ITU-T V.153];

- such a network configuration relates, for both connection models, to the "TLS to non-TLS" interworking case (see clause 6.2.1);

- UDP transport implies DTLS.

There are two principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.2.1: MG provides interworking between "DTLS/UDP/IP" to "UDP/IP"; and

- use case #I.2.2: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode ("thus, similar as use case #I.1.3").

It should be noted that the MG is not aware of the IP application protocol, hence it is also not aware of the "UDPTL" protocol. Hence, use cases #I.2.2 and #I.1.3 should be identical from H.248 control perspective.

Figure I.1 – Use cases for "application protocol agnostic DTLS handling"

Figure I.2 – Use cases for "DTLS-based transport security 
for facsimile packet relay service T.38"

## I.3	Use cases for "WebRTC data traffic"

Background:

WebRTC relates to a multimedia application inclusive of a data component. The WebRTC data uses a "SCTP/DTLS/UDP" transport.

The WebRTC data traffic related DTLS use cases provide further characteristics in addition to the use cases in clauses I.1 and I.2:

- hierarchical protocol stack layering by using a secured tunnel (given by SCTP/DTLS);

- multiplexing model because a single DTLS security session could be shared by multiple WebRTC data channels and

- communication between various WebRTC endpoint types leads to UDP-to-UDP and UDP-to-TCP interworking support by H.248 MGs.

There are three principal use cases in case of end-to-end UDP based transport (see Figure I.2):

- use case #I.3.1: MG provides interworking between "SCTP/DTLS/UDP/IP" to "TCP/IP";

- use case #I.3.2: MG provides interworking between " SCTP/DTLS/UDP/IP" to "TLS/TCP/IP"; and

- use case #I.3.3: MG provides interworking between "(DTLS/)UDP/IP" to "(DTLS/)UDP/IP" in DTLS transparent forwarding mode.

It should be noted that the MG again is not aware of the application protocol itself. However, the main difference is the embedded "SCTP stream/SCTP association" layering in comparison to clause I.1.

Figure I.3 – Use cases for "WebRTC data traffic"

## I.4	Use cases for "DTLS-based key exchange for SRTP"

There are multiple key exchange options for the secure real-time transport protocol (SRTP), see [b-IETF RFC 7201] and [b-IETF RFC 7202]. One media-path coupled key exchange option is based on DTLS (see below sub-clause I.4.1.2). Another, non-DTLS-based option is already supported by H.248. Clause I.4.1.1 summarizes that pure signaling-path coupled key exchange option in order to contrast the DTLS-SRTP variant.

### I.4.1	Two considered key exchange options for SRTP

#### I.4.1.1	Media-path decoupled key exchange using SDP (via SIP and H.248)

Figure I.4 recalls again the protocol stack for SDES-based key exchange for SRTP (SDP security descriptions, see [b-IETF RFC 4568]), as in scope of [b-ITU-T H.248.77]:

Figure I.4 – Media-path decoupled key exchange using SDP (via SIP and H.248)

DTLS is not involved.

#### I.4.1.2	Media-path coupled key exchange using DTLS

Figure I.5 illustrates the alternative of a DTLS-based key exchange for SRTP according to [b-IETF RFC 5763] (framework) and [IETF RFC 5764] (protocol), called briefly "DTLS-SRTP":

Figure I.5 – Media-path coupled key exchange using DTLS

DTLS-SRTP uses DTLS in a limited way

- DTLS protocol suite: usage of the DTLS handshake, cipher change and alert protocols (inclusive DTLS record layer), without any application data transfer over DTLS at all; and

- time horizon: only during key derivation, rekeying phases and alert events.

The derivation process of the SRTP keys (using (D)TLS) is detailed in clause 4.2/[IETF RFC 5764].

### I.4.2	H.248 impact by DTLS-SRTP

DTLS-SRTP introduces some DTLS extensions for SRTP key establishment. The DTLS protocol extension parameters are defined in clause 4.1/[IETF RFC 5764]. These extension parameters are covered by the dtlscn package (see clause 11).

Appendix II

Signalling flows for 
basic DTLS session establishment and release

(This appendix does not form an integral part of this Recommendation.)

## II.1	Overview

The tlsbsc package (clause 8/[ITU-T H.248.89]) defines the basic TLS security session control with scope on support for establishment and release. Basic DTLS session control reuses the solution defined by that package.

## II.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (DTLS) is highlighted besides the H.248 interface.

Furthermore all figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local DTLS session endpoint.

It has to be noted that some DTLS specifics are not visible at the abstraction level considered, such as

- Bearer type indication (via SDP "m=" line) when creating a DTLS-enabled SEP / Termination; and

- DTLS timeouts and retransmissions (during e.g. establishment handshake procedures) (i.e., related to [IETF RFC 6347]/Figure 3 "DTLS Timeout and Retransmission State Machine").

NOTE – IP transport protocol: L4 bearer connection control procedures are optional, dependent on a connection-oriented IP transport protocol (see clause 6.1). Such procedures are indicated, but out of scope of this Appendix.

## II.3	Establishment of DTLS security sessions

### II.3.1	Successful establishment – Terminating side

See Figure II.1, termed as use case (E.1):

More details for this example:

- The MGC blocks the start of DTLS session establishment (1).

- Early incoming DTLS messages are discarded by the MG (2).

- The MGC unblocks the DTLS SEP (3).

- DTLS session establishment (5 to 7); MG acting as DTLS server.

- Optional notification of MGC concerning available DTLS session for application data transfer (8), if subscription to event by MGC (4).

### II.3.2	Successful establishment – Originating side

See Figure II.2, use case (E.2).

Figure II.1 – Successful establishment – Terminating side

Figure II.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause II.3.1

- DTLS session establishment (5 to 7), triggered by MGC (4); MG acting as DTLS client.

- Optional notification of MGC concerning available DTLS session for application data transfer (9), if subscription to event by MGC (8).

### II.3.3	Unsuccessful establishment

The DTLS security session negotiation handshake is not completed.

## II.4	Release of DTLS security sessions

### II.4.1	Successful release – Terminating side

See Figure II.3, termed as use case (R.1).

Figure II.3 – Successful release – Terminating side

More details for this example:

- MG receives an incoming DTLS close_notify alert (2), the indication for DTLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released DTLS session (4), if subscription to event by MGC (1).

### II.4.2	Successful release – Originating side

See Figure II.4, use cases (R.2) illustrates an outgoing DTLS session release:

Figure II.4 – Successful release – Originating side

More details for this example:

- MGC initiates the DTLS session release (1).

- MG sends an outgoing DTLS close_notify alert (2), the indication for DTLS session release towards remote DTLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released DTLS session (5), if subscription to event by MGC (4).

### II.4.3	Unsuccessful release

An unsuccessful DTLS session release procedure would imply that the MG remains in H.248 state Established and that the MGC could possibly suspect a still established DTLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (04/2006), Datagram Transport Layer Security.

[b-IETF RFC 4568]	IETF RFC 4568 (2006), Session Description Protocol (SDP) Security Descriptions for Media Streams.

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[b-IETF RFC 7201]	IETF RFC 7201 (04/2014), Options for Securing RTP Sessions.

[b-IETF RFC 7202]	IETF RFC 7202 (04/2014), Securing the RTP Framework: Why RTP Does Not Mandate a Single Media Security Solution.

[b-ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[b-ITU-T V.153]	Recommendation ITU-T V.153 (12/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services.

[b-ITU-T T.38]	Recommendation ITU-T T.38 (09/2010), Procedures for real-time Group 3 facsimile communication over IP networks, including its Amendment 1 (2014).

__________________