| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4596 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Seoul, 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.SHAPER |  |  |  |
| Title: | H.248.Shaper "Gateway control protocol: H.248 support for traffic shaping" (New): Input draft Ed. 0.3 |  |  |  |
| Purpose: | Meeting input (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's input draft ed. 0.3 for draft new H.248.Shaper "Gateway control protocol: H.248 support for traffic shaping".

# Changes versus Ed. 0.2 (TD 182 (WP 1/16) (2014-07)):

- no changes from Editor's side

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.SHAPER Ed. 0.1 | (Motivation C-473) / Input (C-474) to SG16 Sapporo m., June/July 2013 Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-C-0474 |
| H.248.SHAPER Ed. 0.2 | Output (TD 182/WP1) from SG16 Sapporo meeting, June/July 2013 Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-140630-TD-WP1-0182 |
| H.248.SHAPER Ed. 0.3 | Input (AVD-4596) to WP 1/16 South Korea meeting (2014-11) Location: |
| H.248.SHAPER Ed. 0.4 | Output (TD-xx) from WP 1/16 South Korea meeting (2014-11) Location: |
| H.248.SHAPER Ed. 0.5 |  |
| H.248.SHAPER Ed. 0.6 |  |
| H.248.SHAPER Ed. 0.7 |  |
| H.248.SHAPER Ed. 0.8 |  |
| H.248.SHAPER Ed. 0.9 |  |
| H.248.SHAPER Ed. 0.10 |  |
| H.248.SHAPER Ed. 0.11 |  |
| H.248.SHAPER Ed. 0.12 |  |
| H.248.SHAPER Ed. 0.13 |  |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 7 4 Abbreviations and acronyms 7 5 Conventions 8 6 Motivation 8 6.1 Background: QoS building blocks in packet networks 8 6.2 History I: B-ISDN (ATM) in the 1990 decade 9 6.3 History II: Legacy border (H.248 IP-to-IP) gateways in Next-Generation Networks 9 6.4 Evolving IP infrastructures 11 7 Use cases and models 11 7.1 Use case #1: Jitter reduction in network peering cases 11 7.2 Use case #2: Multiplexing 11 7.3 Use case #3: Different traffic levels of policing and shaping 12 7.4 Use case #4: Layered protocol architectures with hierarchical or/and tunneled stacks 12 7.5 Use case #5: "unknown" or/and "difficult" traffic characteristics 13 7.6 Use case #6: "non-supportive" user equipment 13 8 Models 13 8.1 Principle order of policer and shaper stages in the packet forwarding path 13 8.2 Simple queueing model 14 9 Traffic shaping package 15 10 Profile guidelines 15 10.1 Use of traffic policing for emulating traffic shaping 15

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 2 – Traffic & QoS models with different granularity 10 Figure 3 – Use case "reduction of packet delay variation" (at network peering level) 11 Figure 4 – Use case "Multiplexing" (multi-termination context) 12 Figure 5 – Use case "different traffic levels of policing and shaping" (dual-termination context, multi-stream-per-termination structures) 12 Figure 6 – ### provide an example traffic profile ### 13 Figure 7 – ### provide a network model ### 13 Figure 8 – Unidirectional packet processing pipeline – Staged policer (I) and shaper (II) 13 Figure 9 – Two-stage traffic handling based on (I) traffic policer and (II) traffic shaper (without consideration of possible, additional traffic filter stages at ingress & egress interfaces) 14 Figure 10 – Queuing model of two-stage policer / shaper mapped on H.248 Context model 15

Draft new Recommendation ITU-T H.248.xx (ex H.248.Shaper)

Gateway Control Protocol:
H.248 support for traffic shaping

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

Traffic shaping is a mechanism that alters the traffic characteristics of a stream of packets on a traffic flow to achieve a desired modification of those traffic characteristics, in order to achieve better network efficiency whilst meeting the QoS objectives or to ensure conformance at a subsequent interface. Traffic shaping must maintain packet sequence integrity on a connection-oriented traffic flow. Shaping modifies traffic characteristics of a packet flow with the consequence of increasing the mean packet transfer delay.

# 1	Scope

The traffic characteristics of an end-to-end packet flow are intentionally modified by the two QoS support functions of traffic policing (see [ITU-T H.248.53]) and traffic shaping. Both functions are complementary and orthogonal: each one could be applied individually or both could be applied in conjunction. The exclusive use of traffic policing is sufficient for the very majority of communication services and network architectural environments as in typical scope of H.248 gateways.

This Recommendation defines support for traffic shaping by H.248 gateways.

Examples of traffic shaping are

- peak packet rate reduction

- burst length limiting,

- reduction of packet delay variation (PDV)

by suitably spacing packets in time and queue service schemes.

{Editor's note 1 (2014-07): complete scope section.}

{Editor's note 2 (2014-07): the convention of "packet" needs to be decided – either a generic approach as H.248.53 (see clause 5), or by focusing on a) packets at the network layer only and b) just consideration of IP. For discussion..}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway control protocol: QoS support packages.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (02/2012), Gateway control protocol: Guidelines for packet-based streams.

[ITU-T H.248.82]	Recommendation ITU-T H.248.82 (03/2013), Gateway control protocol: Explicit congestion notification support.

[ITU-T H.248.86]	Recommendation ITU-T H.248.86 (01/2014), Gateway control protocol: H.248 Support for deep packet inspection.

[ITU-T G.799.1]	Recommendation ITU-T G.799.1 (05/2011), Signal processing functionality and performance of an IP-to-IP voice gateway optimized for the transport of voice and voiceband data.

[ITU-T I.371]	Recommendation ITU-T I.371 (2004), Traffic control and congestion control in B-ISDN.

[ITU-T I.378]	Recommendation ITU-T I.378 (2002), Traffic control and congestion control at the ATM Adaptation Layer type 2.

[ITU-T Y.1221]	Recommendation ITU-T Y.1221 (2002), Traffic control and congestion control in IP-based networks.

[ITU-T Y.1291]	Recommendation ITU-T Y.1291 (2004), An architectural framework for support of Quality of Service in packet networks.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	policer [ITU-T H.248.53]: A functional or physical entity that enforces pre-negotiated policies or contracts.

3.1.2	shaper [ITU-T H.248.53]: A functional or physical entity that modifies the characteristics of data leaving the entity.

3.1.3	scheduler [ITU-T H.248.53]: A functional or physical entity that controls which packets to select for transmission.

3.1.4	traffic policing [ITU-T Y.1291]: Traffic policing deals with the determination of whether the traffic being presented (i.e., the ingress H.248 stream in a H.248 ephemeral termination) is (on a hop-by-hop basis) compliant with pre-negotiated policies or contracts. Traffic policing is e.g., according to clause 7.1.3 of [ITU-T Y.1221] (called "parameter control") for IP, or clause 7.2.3 of [ITU-T I.371] (called "Usage Parameter Control (UPC) and Network Parameter Control (NPC)") for ATM.

3.1.5	traffic scheduling (queuing and scheduling) [ITU-T Y.1291]: This mechanism controls which packets to select for transmission on an outgoing link. Scheduling is e.g., further detailed for IP in clause 7.1.6 of [ITU-T Y.1221] on packet scheduling, for ATM in clause 7.2.6 of [ITU-T I.371] on cell scheduling, or for AAL2 in clause 7.2.5 of [ITU-T I.378] on packet scheduling.

3.1.6	traffic shaping [ITU-T Y.1291]: A mechanism that alters the traffic characteristics of the traffic leaving a node (i.e., the egress H.248 stream from an H.248 ephemeral termination). Traffic is shaped e.g., according to clause 7.1.5 of [ITU-T Y.1221] for IP, clause 7.2.7 of [ITU-T I.371] for ATM, or clause 7.2.4 of [ITU-T I.378] for AAL2.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AAL2 | ATM Adaptation Layer type 2 |
| --- | --- |
| ATM | Asynchronous Transfer Mode |
| B | Bucket size of a token bucket |
| Bp | Bucket size of peak token bucket |
| Bs | Bucket size of sustainable token bucket |
| GBRA | Generic Byte Rate Algorithm |
| GoS | Grade of Service |
| IETF | Internet Engineering Task Force |
| INI | Inter-Network Interface |
| IP | Internet Protocol |
| LD | Local Descriptor |
| M | Maximum allowed packet size |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| N | Size (in byte) of an IP packet |
| NPC | Network Parameter Control |
| OAM | Operations, Administration, Maintenance |
| PCI | Protocol Control Information |
| PDP | Policy Decision Point |
| PDU | Protocol Data Unit |
| PEP | Policy Enforcement Point |
| QoS | Quality of Service |
| R | Rate of a token bucket |
| Rp | Rate of peak token bucket |
| Rs | Rate of sustainable token bucket |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| SDP | Session Description Protocol |
| TB | Token Bucket |
| UNI | User Network Interface |
| UPC | Usage Parameter Control |

# 5	Conventions

None.

# 6	Motivation

## 6.1	Background: QoS building blocks in packet networks

FIXTHIS

NOTE – H.248.86 (DPI) allows the fine-granular classification of traffic beyond the usual 5-tuple based segregation at IP transport connection traffic level.

Figure 1 – See Figure 1/Y.1291 – Architectural framework for QoS support

## 6.2	History I: B-ISDN (ATM) in the 1990 decade

FIXTHIS … [description] …

- Aggregate bearer control package

- ATM Forum: VoAAL2oDSL

## 6.3	History II: Legacy border (H.248 IP-to-IP) gateways in Next-Generation Networks

FIXTHIS … [description] …

- coarse granular traffic models and QoS handling

Figure 2 – Traffic & QoS models with different granularity

{Editor's note  (2014-07): readability of Fig. 2 needs to be improved. Use labels and move text outside the figure ….}

## 6.4	Evolving IP infrastructures

FIXTHIS

# 7	Use cases and models

## 7.1	Use case #1: Jitter reduction in network peering cases

Two packet network domains of different operators are interconnected at core network level (Figure 3). Both core network domains differ significantly with respect to their Grade of Service (GoS) conditions. E.g., the observed packet delay variations (PDV) are considerably different (PDVA << PDVB). Such incoming traffic from B to A would basically increase the PDVA figure, i.e., negatively impact GoSA. A shaper in B to A direction would resolve that issue. A shaper in reverse direction is not required. There are also not any policers required due to the assumption that the traffic flows were already policed at the network edge from access-to-core network level.

Figure 3 – Use case "reduction of packet delay variation" (at network peering level)

This use case is also described in [ITU-T G.799.3] with limited scope on RTP traffic for voice services only.

Use case variations:

- #1.1: PDV elimination (or complete PDV reduction)

- #1.2: PDV adjustment (i.e., a limitation down to the level as currently represented by existing GoS conditions).

{Editor's note  (2014-07): … jitter reduction is also known as "reshaping" => clarify terminology.}

## 7.2	Use case #2: Multiplexing

FIXTHIS … [description] multi-termination context structures…

Figure 4 – Use case "Multiplexing" (multi-termination context)

Use case variations:

- #2.1: specific scenarios from  [H.248.STGROUP]

- #2.2: specific scenarios from  [H.248.RTPMUX]

## 7.3	Use case #3: Different traffic levels of policing and shaping

FIXTHIS … [description]  in dual-termination contexts…

Figure 5 – Use case "different traffic levels of policing and shaping" (dual-termination context, multi-stream-per-termination structures)

## 7.4	Use case #4: Layered protocol architectures with hierarchical or/and tunneled stacks

FIXTHIS … [description]   see e.g. specific SCTP transport modes in [H.248.SCTP], WEBRTC stack [H.248.WEBRTC]…

## 7.5	Use case #5: "unknown" or/and "difficult" traffic characteristics

FIXTHIS … [description] …

Figure 6 –  ### provide an example traffic profile ###

## 7.6	Use case #6: "non-supportive" user equipment

FIXTHIS … [description] …

Figure 7 –  ### provide a network model ###

# 8	Models

## 8.1	Principle order of policer and shaper stages in the packet forwarding path

FIXTHIS … [description] …

Figure 8 – Unidirectional packet processing pipeline – Staged policer (I) and shaper (II)

It may be noted that

- the traffic policer is linked to the ingress packet bearer interface and

- the traffic shaper is linked to the egress packet bearer interface

independent of the IP connection model (i.e., the number of IP-enabled terminations per context).

## 8.2	Simple queueing model

Figure 9 provides a high level queueing model, the functions of traffic policing and traffic shaping may be described as "policy rule based packet processing functions" (see [ITU-T H.248.79]).

It may be noted that

- the traffic policer

  - does not add "any" internal IP packet transfer delay,

  - but is a lossy function in principal (due to possible packet discard actions); and

- the traffic shaper

  - may increase the delay ("because this is the actual means to alter traffic characteristics such as packet rates, packet delay variations, packet burst sizes"),

  - but is a lossless function in principal (hence the need for a waiting queue).

Figure 9 – Two-stage traffic handling based on (I) traffic policer and (II) traffic shaper
(without consideration of possible, additional traffic filter stages at ingress & egress interfaces)

The traffic flow rates describe the time-independent relations under stationary conditions. There are following basic traffic flow rate relationships:

Stage "policer":

Ta(S1)=Ta(S1)-Discard	(Eq. 1)

Stage "shaper":

Tb(S1)=Tb(S1)	(Eq. 2)

And there would be

Tb(S1)=Ta(S1)	(Eq. 3)

under the condition of lossless internal IWF(s) (e.g., if there would not be any additional filtering stages with packet discard rule actions).

Figure 10 indicates the mapping of the queuing model to the H.248 context model in case of a (packet, packet) connection model:

Figure 10 – Queuing model of two-stage policer / shaper mapped on H.248 Context model

# 9	Traffic shaping package

FIXTHIS …
design critera…
* support at flow, stream and termination level
* one property for the minimum inter-departure time between subsequent packets
* one property for the maximum size of the waiting queue ("in order to bound the maximum delay")?

# 10	Profile guidelines

## 10.1	Use of traffic policing for emulating traffic shaping

FIXTHIS …

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title - Placeholder>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ETSI TR 183 068]	ETSI TR 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

_________________