| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4598 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.13 (Rel. 2): Terminology related to connection endpoints and H.248 stream endpoints |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes terminology related to "endpoint" notations.

1	Background

There were "endpoint notations" discussed and added to the Convention clause in some H.248.x-Recommendations recently approved (H.248.89, H.248.90, H.248.93) or still in work (H.248.SCTP).

The subject raised discussions arround the aspects of

- plane-specific endpoint representations,

- user plane endpoint: full protocol termination ("connection-endpoint") versus partial protocol termination ("relay-endpoint");

- abstraction level: modelling the entire protocol stack as endpoint versus the consideration of layer individual protocol endpoints ("as required for some H.248.x defined interworking functions"); and

- terminology baseline (X.200, Y.2011).

2	Conclusions

There are following observations:

- A comprehesive description of the terminology framework related to user plane "connection endpoints" versus control plane "H.248 stream endpoints" would be beneficial.

- Modelling: the monolithic endpoint object model is the default assumption in H.248.x-Recommendations, however, more detailled endpoint objects models (such as plane-specific representations) are also required in some cases, hence more information is solicited;

- Terms: majority of X.200 & Y.2011 is still perfectly reuseable in the "H.248 gateway" context, but additional terms might be needed (there's at least one new definition required, for the "(N)-relay-endpoint").

It is suggested to capture that information in a new Appendix for H.Sup.13.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against H.Sup.13 (Release 1; 07/2014).

#1:

[Begin Proposal]

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

## 1.1	Release 2

Release 2 adds further terms, in particular related to following areas:

- endpoint notations;

[End Proposed Correction]

#2:

[Begin Proposal]

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ITU-T X.200]	Recommendation ITU-T X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[ITU-T Y.2011]	Recommendation ITU-T Y.2011 (10/2004), General principles and general reference model for Next Generation Networks.

[End Proposed Correction]

#3:

[Begin Proposal]

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

3.1.1	(N)-association (clause 5.3.1.1/[ITU-T X.200]): A cooperative relationship among (N)-entity-invocations.

3.1.2	(N)-connection (clause 5.3.1.2/[ITU-T X.200]): An association requested by an (N1)-entity for the transfer of data betweeen two or more (N1)-entities. The association is established by the (N)-layer and provides explicit identification of a set of (N)-data-transmissions and agreement concerning the (N)-data-transmission services to be provided for the set.

3.1.3	(N)-connection-endpoint (clause 5.3.1.3/[ITU-T X.200]): A terminator at one end of an (N)-connection within an (N)-service-access-point.

3.1.4	(N)-protocol (clause 5.2.1.9/[ITU-T X.200]): A set of rules and formats (semantic and syntactic) which determines the communication behavior of (N)-entities in the performance of (N)-functions.

3.1.5	(N)-relay (clause 5.3.1.6/[ITU-T X.200]): An (N)-function by means of which an (N)-entity forwards data received from one peer-(N)-entity to another peer-(N)-entity.

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| (N)-relay-endpoint | A lightweight (N)-connection-endpoint, located in a network element within the end-to-end (N)-connection path. The (N)-relay endpoint provides only partial processing of the (N)-protocol (in contrast to the full terminator function of the (N)-connection-endpoint). NOTE 1. |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |
| NOTE 1 – Example: the [ITU-T H.248.84, ITU-T H.248.89] TCP proxy mode is based (TCP)-connection-endpoints (due to complete termination/origination of the TCP) and TCP relay and merge modes are based on (TCP)-relay-endpoints (due to the stateless, minimal involvement in TCP processing). |  |

[End Proposed Correction]

#4:

[Begin Proposal]

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| GCP | Gateway Control Protocol |
| --- | --- |
| C-EP | Connection Endpoint |
| EP | Endpoint |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| (N) | protocol layer N |
| R-EP | Relay Endpoint |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |

[End Proposed Correction]

#5:

[Begin Proposal]

Appendix #2#

User plane "connection endpoints" 
versus
control plane "H.248 stream endpoints"

## #2#.1	Introduction

The H.248 MG is processing (N)-connection user plane traffic, such as the termination of an (N)-connection protocol (leading to an (N)-connection-endpoint in the MG), forwarding of (N)-protocol-data-units, etc. The user plane is synonym to the terms bearer plane (from H.248 perspective) and data plane (from NGN perspective).

The top-level model of equating an (user plane) (N)-connection-endpoint with an (control plane) (H.248) stream endpoint (or (H.248) termination) is sufficient for the very majority of existing gateway control applications. The correspondent "H.248.x-series Recommendations" [ITU-T H.248.x] do not detail the endpoint model, using just the term (H.248) stream endpoint for both plane-specific objects.

Such an abstract endpoint object model is not sufficient for some advanced user plane interworking functions (IWF) (see Appendix #1#) as defined for H.248 MGs.

## #2#.2	Base terminology

Base terminology for protocols (such as H.248 gateway control protocol), network elements and network architectures (such the decomposed H.248 gateway) are (still) [ITU-T X.200] and [ITU-T Y.2011].

## #2#.3	Plane-specific endpoint objects – top-level model

### #2#.3.1	Plane-specific model

The top-level object model of MG resources behind an H.248 SEP / termination provides Figure #2#.1 from a plane-specific perspective:

Figure #2#.1 – Plane-specific endpoint objects in the H.248 MG – Top-level model

The (user plane) (N)-connection-endpoint object has representations in the

- control plane, given by the (H.248) stream endpoint object, and

- management plane, given by the managed object.

The management plane aspect is not further considered in this Supplement.

It should be noted that plane-specific object representations are often required when statefull entities are considered.

NOTE 1 – The expired work on an IETF SNMP MEGACO MIB illustrates the principle as such, from perspective of "protocol layer" management.

NOTE 2 – [ETSI 183 025], Annex B "Relation of the H.248 ServiceStates Property Model with Management Models" illustrates a specific example.

The control plane relates to the gateway control protocol H.248 and it's relation to the user plane object is the prime purpose of this Appendix.

### #2#.3.2	Mapping of endpoint objects between planes

There is always the assumption of a 1:1 mapping between a (user plane) (N)-connection-endpoint and a (control plane) (H.248) stream endpoint (or (H.248) termination). That is a valid generic assumption because (bearer) technology independent. The notion of "(bearer) technology" relates usually in H.248 to a particular user plane protocol, protocol stack segment or entire protocol stack.

Thus, there might be divergence from above 1:1 mapping in technology specific cases.

## #2#.4	Endpoint objects for layered protocol architectures

### #2#.4.1	(H.248) stream endpoint pairs – technology independent model for detailed protocol stacks

The primary purpose of H.248 MG is the support of interworking functions at various protocol layers (see Appendix #1#), typically below the application layer. Such functionality implies a more detailed consideration of multiple protocol layers, see Figure #2#.2:

Figure #2#.2 – H.248 SEPP – Technology independent model for detailed protocol stacks

It may be noted that Figure #2#.2 illustrates the example of a "relay function" at layer (N+1). There are other (N+1)-IWF types which require an (N+1)-connection-endpoints.

### #2#.4.2	(H.248) stream endpoint pairs – technology independent model for entire protocol stack

Whenever any detailed protocol layering is not required, then the abstracted concept of an (H.248 bearer)-connection is sufficient, see Figure #2#.3:

Figure #2#.3 – H.248 SEPP – Technology independent model for entire protocol stacks

There is a 1:1 mapping between a (user plane) (H.248 bearer)-connection-endpoint and a (control plane) (H.248) stream endpoint (or (H.248) termination). This is the basic model sufficient for many existing H.248.x Recommendations.

## #2#.5	Technology specific models – Examples

### #2#.5.1	Bearer protcol 'TCP'

#### #2#.5.1.1	SEPP in "TCP relay" or "TCP merge" mode

Figure #2#.4 illustrates the layered connection endpoint model for a "TCP-enabled H.248 SEPP" in TCP relay (or TCP merge) mode configuration:

Figure #2#.4 – Example – SEPP in "TCP relay" or "TCP merge" mode

Possible further lower layer Lx-IWFs (such as the (L3)-IWF for IPv4-to-IPv6 interworking) are not detailed in Figure #2#.4.

#### #2#.5.1.2	SEPP in "TCP proxy" mode

Figure #2#.5 illustrates the layered connection endpoint model for a "TCP-enabled H.248 SEPP" in TCP proxy mode configuration:

Figure #2#.5 – Example – SEPP in "TCP proxy" mode

Difference: there are (TCP)-connection-endpoints in TCP proxy mode due to the protocol termination/origination of TCP.

### #2#.5.2	Bearer protcol 'DTLS'

#### #2#.5.2.1	SEPP in end-to-access-edge transport security mode

The so-called end-to-access-edge transport security mode implies the protocol termination of DTLS in the H.248 MG. There will be consequently a (DTLS)-connection-endpoint, associated to the (H.248) stream endpoint (Figure #2#.6).

Figure #2#.6 – SEPP in end-to-access-edge transport security mode

It has to be noted that upper layers (such as the application layer) are omitted in Figure #2#.6.

#### #2#.5.2.2	SEPP in end-to-end transport security mode

The so-called end-to-end transport security mode implies support of "DTLS transparent forwarding" by the H.248 MG. The H.248 MG might be totally unaware (agnostic) of the (L4)-application ("DTLS traffic") or aware in "DTLS transparent forwarding" mode (e.g., support of DTLS-specific H.248 events or statistics are not excluded in this mode). Figure #2#.7 illustrates the example of a DTLS-aware SEPP configuration which would be modelled by (DTLS)-relay-endpoinst.

Figure #2#.7 – SEPP in end-to-end transport security mode – Example of a DTLS-aware MG

[End Proposed Correction]

_________________

See clauses 3.4 and 3.11 in [ITU-T Y.2011].

See e.g. [ITU-T H.248.89] for TCP: there are different variants of for the user plane object, - the (TCP)-connection-endpoint for TCP proxy mode with 12 states and the (TCP)-relay-endpoint for TCP relay/merge modes without any state; and the H.248 stream endpoint object with two or three states.