| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4611 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.13 (Rel. 2): Terminology related to interworking |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution suggests initial proposals for terminology related to interworking.

1	Background

H.248 gateways are inherently positioned as a network function for "interworking" aspects related to adjacent network segments, as fundamentally outlined by e.g. H.246 or Y.2011. On the other side, the general "interworking framework" (models = Y.1251; terminology = Y.1401) doesn't match exactly interworking scenarios as in scope of H.248 gateways, as well as lacking any lower levels of details as inherently required for decomposed network elements.

2	Discussion

The ITU database for terminology provides multiple entries for interworking related terms. We'd like to focus on the "recommended" definitions:

Interworking (IW) and interworking function (IWF):

Table 1 – Discussion of term "interworking"

| Term | Definition | Reference | Useful from H.248 perspective? |
| --- | --- | --- | --- |
| interworking | The term ''interworking'' is used to express interactions between networks, between end systems, or between parts thereof, with the aim of providing a functional entity capable of supporting an end-to-end communication. | ITU-T Y.1401 (02/2008) | Yes |
| interworking function | These functions are referred to in the interworking definition, which include the conversion between protocols and the mapping of one protocol to another. The functionality required between networks can be separated from the functionality, if any, required in end systems. | ITU-T Y.1401 (02/2008) | Yes |

Service interworking:

Table 2 – Discussion of term "service interworking"

| Term | Definition | Reference | Useful from H.248 perspective? |
| --- | --- | --- | --- |
| service interworking | Interworking between two dissimilar (unlike) networks directly without the benefit of an intermediary network. | ITU-T Y.1401 (02/2008) | useful as baseline, but needs further qualification due to the aspect of an "intermediary network" |
|  | In service interworking, the Interworking Function (IWF) of Figure 4 terminates the protocol used in network 1 and translates (i.e., mapping) the Protocol Control Information (PCI) to the PCI of the protocol used in network 2 for User, Control and Management Plane functions to the extent possible. In general, since not all functions may be supported in one or other of the networks, the translation of PCI may be partial or non-existent. However, this should not result in any loss of user data since the payload is not affected by PCI conversion at the service interworking IWF. | Y.1251 (02), 3.2 |  |

Network interworking:

Table 3 – Discussion of term "network interworking"

| Term | Definition | Reference | Useful from H.248 perspective? |
| --- | --- | --- | --- |
| network interworking | Interworking between two similar (like) networks via an intermediary network with dissimilar characteristics | ITU-T Y.1401 (02/2008) | useful as baseline, but needs further qualification due to the aspect of an "intermediary network" |
|  | In network interworking, the PCI of the protocol used in network 1 and network 2 and the payload information are transferred transparently by an IWF of Figure 4. Typically the IWF encapsulates (known as tunnelling in some specifications) the information which is transmitted by means of an adaptation function and transfers it transparently to the other network. | Y.1251 (02), 3.3 |  |

It was observed in some follow-up discussions at the last meeting that particularily terms "service interworking" and "network interworking" would need to be further qualified for usage in context with H.248 gateways.

3	Conclusions

The terminology related to interworking is basically required for other H.248 terminology, such as endpoint aspects (AVD-4598), media translation aspects (AVD-4612), B2BUA aspects (AVD-4599), apart from legacy wording such as "appication-agnostic", etc.

At least following aspects should be expressible in "H.248 interworking" cases:

- protocol layering, i.e., (N)-IWF, (N-1)-IWF, etc;

- layer (or stack) agnostic and layer (or stack) aware IWFs;

- distinction between the (protocol) endpoint function and interworking function (also from H.248 context/termination model point of view);

- IWFs located between homogeneous or heterogeneous protocol stacks; and

- plane specific IWF aspect (in order to separate the MG-level bearer plane IWFs (in scope of the H.248 GCP) from the MGC-level call control plane IWFs (out of scope of H.248).

4	Proposal

We'd like to propose a new Appendix for H.Sup.13 concerning the motivation and derivation of interworking related terms. The key terms would be listed in clause 3.2 in order to be referenceable by other ITU-T Recommendations.

Status: we've identified six H.248 specific interworking terms, which are useful and required already for past or ongoing H.248 work items.

It is proposed to accept the marked up changes below. Changes are marked up against H.Sup.13 (Release 1; 07/2014).

#1:

[Begin Proposal]

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

## 1.1	Release 2

Release 2 adds further terms, in particular related to following areas:

- interworking and interworking functions;

[End Proposed Correction]

#2:

[Begin Proposal]

# 2	References

[ITU-T H.246]	Recommendation ITU-T H.246 (05/2006), Interworking of H-series multimedia terminals with H-series multimedia terminals and voice/voiceband terminals on GSTN, ISDN and PLMN.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ITU-T Y.1251]	Recommendation ITU-T Y.1251 (08/2002), General architectural model for interworking.

[ITU-T Y.1401]	Recommendation ITU-T Y.1401 (02/2008), Principles of interworking.

[ITU-T Y.2011]	Recommendation ITU-T Y.2011 (10/2004), General principles and general reference model for Next Generation Networks.

[End Proposed Correction]

#3:

[Begin Proposal]

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

3.1.1	bearer interworking function (BIWF, [ITU-T Q.Sup31]): A functional entity which provides bearer control and media mapping/switching functions (MMSF) within the scope of a Serving Node (…). A BIWF contains one Bearer Control Nodal Function (…) and one or more MCF and MMSF, and is functionally equivalent to a Media Gateway that incorporates bearer control.

3.1.2	Interworking (IW, [ITU-T Y.1401]): the term ''interworking'' is used to express interactions between networks, between end systems, or between parts thereof, with the aim of providing a functional entity capable of supporting an end-to-end communication.

3.1.3	Interworking function (IWF, [ITU-T Y.1401]): These functions are referred to in the interworking definition, which include the conversion between protocols and the mapping of one protocol to another. The functionality required between networks can be separated from the functionality, if any, required in end systems.

3.1.4	Network interworking [ITU-T Y.1401]: Interworking between two similar (like) networks via an intermediary network with dissimilar characteristics.

3.1.5	Service interworking [ITU-T Y.1401]: Interworking between two dissimilar (unlike) networks directly without the benefit of an intermediary network.

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |

{Editor's note (2014-11): following terms are subject to addition to Table 1 (see Appendix #1#) as soon as stable and agreed:}:

| H.248 service interworking | the service interworking provided by H.248 gateways. |
| --- | --- |
|  | Further qualification: - call control plane H.248 service interworking: H.248 service interworking at H.248 MGC level between call control signalling protocols; - bearer plane H.248 service interworking: H.248 service interworking at H.248 MG level. |
| H.248 network interworking | the network interworking supported by one or multiple H.248 gateways (from end-to-end perspective). |
| heterogeous H.248 service interworking | H.248 service interworking between disimilar protocol stacks. |
| homogeneous H.248 service interworking | H.248 service interworking between similar protocol stacks. |
| Layer-(N)-agnostic H.248 service interworking | Bearer plane H.248 service interworking where the top-level (N-1)-IWF is unaware of protocol N and upper layers. |
| Layer-(N)-aware H.248 service interworking | Bearer plane H.248 service interworking where the top-level (N-1)-IWF is aware of protocol N and possibly also other upper layer protocols. |

[End Proposed Correction]

#4:

[Begin Proposal]

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| BIWF | Bearer Interworking Function |
| --- | --- |
| C-EP | Connection Endpoint |
| EP | Endpoint |
| GCP | Gateway Control Protocol |
| IW | Interworking |
| IWF | Interworking Function |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| (N) | protocol layer N |
| R-EP | Relay Endpoint |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |

[End Proposed Correction]

#5:

[Begin Proposal]

Appendix #1#

Detailing interworking terminology

## #1#.1	Introduction

Fundamental purpose of a gateway is support of interworking between different networks. If there isn't any need for any interworking functions in the end-to-end communciation path, then there isn't any reason to route the traffic over an H.248 gateway.

Interworking is primarily subject of some ITU-T Y.x-series of Recommendations (in case of NGNs), which define some high-level terminology. That terminology need to be detailed in order to meet the plethora and variety of interworking functions as defined by the "H.248.x-series Recommendations" [ITU-T H.248.x] for ITU-T H.248 gateways.

## #1#.2	Baseline of terminology for H.248 gateways

[ITU-T H.248.1] itself is silent about interworking aspects due to the obviousness for the particular network element type "gateway" (either decomposed or integrated). Interworking models and terminology are subject of [ITU-T Y.1251] and [ITU-T Y.1401]. The possible position of H.248 gateways in pre-NGN and NGN scenarios is e.g. indicated in [ITU-T H.246] and Appendix II/[ITU-T Y.2011].

## #1#.3	Problem with existing high level terminology

The traffic handled by an H.248 context is passing an interworking function in general (see Figure #1#.1):

Figure #1#.1 – H.248 context model and H.248 bearer interworking function
(in a typical dual-termination context)

Such a top-level IWF (as part of the H.248 bearer plane) needs to be further detailed and qualified. Such kind of decomposition is e.g., indicated by Figure II.2/[ITU-T Y.2011] ("but still too high level") and a basic IWF qualification is reflected by the two concepts of network interworking and service interworking. All that kind of interworking concepts are fully inline with purpose and scope of H.248 gateway deployment scenarios, however, both terms take an end-to-end perspective by consideration the concatenation of multiple network domains in series ("intermediary network"), whereas interworking by H.248 gateways is limited on the two adjacent interconnected networks only.

## #1#.4	A more detailed interworking terminology

### #1#.4.1	Starting point: "related principles" by [ITU-T Y.1251]

Below text in italics is quoted from clause 9 "Related principles"/[ITU-T Y.1251]:

In general, it can be stated that whenever any protocol stack difference exists between two networks, interworking is required to interconnect the two networks. This general principle is independent of the method used to achieve such interworking, i.e., irrespective of whether the service interworking method of 8.1 or the network interworking of 8.2 is used to effect such interworking.

Terminology aspect: introduction of "protocol stack" concepts, which includes the principle of "layered protocol architectures", i.e., the assignment of an IWF to a particular layer (abbreviated as (N)-IWF for layer N).

Any protocol discontinuity between N1 and N2, at any layer of their respective stacks, constitutes interworking. For example, if N1 were IP over a Local Area Network and N2 was IP over Frame Relay, interworking is taking place between the Local area Network and the Frame Relay network because of the layer 2 discontinuity, even though there is no discontinuity at layer 3 (the IP layer). Adaptation functions are required for IP over the LAN layer 2 protocol and for IP over the Frame Relay protocol.

Terminology aspect: the notion of IWF covers not a single function, rather a set of various capabilities (such as "adaptation"), thus a qualification of IWFs should be possible.

If layer N is the uppermost layer from the interworking standpoint, there must be at least one layer protocol discontinuity between the protocol stacks of networks N1 and N2 for their interconnection to be considered as an interworking case. This discontinuity may occur at layer N itself, or at any other layer N–j (for j less than N; j greater equal 1), or at several layers simultaneously including or excluding the layer N itself. It should also be noted that the number of layer protocols in each stack may be different.

Terminology aspect: a layer specific (N)-IWF may exist or not, but there is at least one (N)-IWF.

In general, mappings between features may span several layers. It cannot be assumed that mappings performed purely at a peer level are sufficient, or that each peer layer is independent of the underlying layer(s). For example, a particular feature at layer N in N1 may be required to be mapped to non-peer layer in N2. These considerations may apply particularly to QoS related features and/or aspects.

Terminology aspect: there might be interdependencies between multiple (N)-IWFs at different layers.

### #1#.4.2	General IWF model

Figure #1#.2 illustrates the general IWF model at the example of basic reference model (BRM) according to [ITU-T X.200]. The number of protocol layers (here seven) is not relevant because usage of H.248 media gateways is not limited by any specific layered protocol reference model.

Figure #1#.2 – General IWF model as part of a dual-termination relationship in an H.248 context

The two H.248 terminations T1 and T2 (see also Figure #1#.1) provide endpoint (EP) functionality per protocol layer, and there could be an IWF at each layer in principle.

### #1#.4.3	Real IWF models

It is obvious that real H.248 gateway deployments (as defined by H.248 profiles) does only consider a limited IWF model, primarily by focusing on a particular stack segment only and avoidance of any information related to protocols which are not subject of processing by the H.248 media gateway.

Figure #1#.3 illustrates an example of a network address translation (NAT) and port translation (PT) function for interworking between two IP network domains (with different IP address realms), plus the Ethernet (ET) priority adaptation at layer 2 (here: IP-over-Ethernet).

Figure #1#.3 – Real IWF model at the example of a network address translation (NAT) and port translation (PT) function for interworking between two IP network domains (with different IP address realms)

Some observations:

- layer 1 is actually out of scope of H.248 gateway control, hence not further considered any more;

- native protocol functions, such as IP packet fragmentation or L4 checksum calculation are rather subject of the (IP)-EP and (L4)-EP than the (N)-IWF;

- layer awareness: the H.248 MG is (L4+)-agnostic in above example.

Further, a distinction is required concerning full or partial protocol termination, which could be differentiated by qualifying an endpoint (EP) as either connection endpoint (C-EP) or relay endpoint (R-EP). The distinction is only relevant at the top protocol layer of interworking (because all lower layer endpoints are of type C-EP). Below two examples:

Example – basic IWF models for IP [ITU-T H.248.64]:

An H.248 IP-to-IP media gateway may support various types of IP interworking, see Figure #1#.4:

Figure #1#.4 – Example – basic IWF models for IP

The B2BIH mode implies a full termination of the IP, in contrast to IP router modes. The difference of an (IP)-C-IP and (IP)-R-IP has impact on some protocol functions, such as ICMP handling.

Next example – basic IWF models for TCP [ITU-T H.248.84, ITU-T H.248.89]:

There are three main modes of interworking known for H.248 TCP-to-TCP media gateways, see Figure #1#.5:

Figure #1#.5 – Example – basic IWF models for TCP

The TCP proxy mode mode implies a full termination of the TCP, in contrast to TCP relay and merge modes.

### #1#.4.4	Y.1401 interworking models using H.248 gateways as interworking units

[ITU-T Y.1401] describes the principles of service and network interworking, which are examined in this clause from perspective of H.248 gateways. Figure #1#.6 outlines case "service interworking", an H.248 gateway positioned between two network domains with different protocol layering (N1 and N2):

Figure #1#.6 – Y.1401 "Service Interworking" with H.248 gateway as interworking unit

The concept of service interworking results just in the termination of the two network specific protocol stacks A-over-B and X-over-Y by the H.248 MG from the local scope of an H.248 gateway.

In contrast, the illustration of the concept of network interworking needs to take a more global scope beyond a single H.248 gateway. Figure #1#.7 illustrates the network interworking case at the example of three network domains (based on two network technologies N1 and N2):

Figure #1#.7 – Y.1401 "Network Interworking" with H.248 gateway as interworking unit

Abstracted network protocol stack N1, "A-over-B" uses following semantic: the symbol 'A' represents the application related protocol layer(s), and symbol 'B' all lower layers (typically transport related). Network interworking is characterized by the fact that protocol (stack segment) 'A' is tunneled through an interim network (here N2). Tunneling relates to the complete encapsulation of 'A' by another protocol (here 'X'). Thus, the two H.248 MGs in Figure #1#.7 provide a (X)-EP function at the H.248 termination connected towards N2.

## #1#.5	Definitions

Given the interworking considerations and discussion of basic Y.1401 principles in previous clause, following terms may be now derived for usage in "H.248.x-series Recommendations" [ITU-T H.248.x]:

Firstly, service and network interworking in context of H.248 gateways:

| H.248 service interworking | the service interworking provided by H.248 gateways. |
| --- | --- |
|  | Further qualification: - call control plane H.248 service interworking: H.248 service interworking at H.248 MGC level between call control signalling protocols; - bearer plane H.248 service interworking: H.248 service interworking at H.248 MG level. |

| H.248 network interworking | the network interworking supported by one or multiple H.248 gateways (from end-to-end perspective). |
| --- | --- |

Example: end-to-end Group 3 facsimile across an interim IP network where two H.248 PSTN-IP media gateways provide the T.38 on-ramp and off-ramp gateway functions.

Secondly, H.248 service interworking should be further qualified due to the variety of protocol stack combinations:

| homogeneous H.248 service interworking | H.248 service interworking between similar protocol stacks. |
| --- | --- |

Example: IPv4-to-IPv6 interworking relates to similar protocol stacks, which covers the case of identical protocol stacks.

| heterogeous H.248 service interworking | H.248 service interworking between disimilar protocol stacks. |
| --- | --- |

Examples: interworking between circuit-switched and packet-switched networks, between IP and non-IP packet networks, between a connectionless IP transport protocol (UDP) and connection-oriented variants (TCP, SCTP), etc.

Thirdly, the combination of the legacy concepts of "media aware" / "media agnostic" and H.248 service interworking may be combined, leading to following general definitions:

| Layer-(N)-agnostic H.248 service interworking | Bearer plane H.248 service interworking where the top-level (N-1)-IWF is unaware of protocol N and upper layers. |
| --- | --- |
| Layer-(N)-aware H.248 service interworking | Bearer plane H.248 service interworking where the top-level (N-1)-IWF is aware of protocol N and possibly also other upper layer protocols. |

[End Proposed Correction]

_________________