| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4620 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Support of WebRTC Datachannel transport in H.TPS-SIG |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution discusses the support of DTLS in H.TPS-SIG.

2.	Discussion

The IETF CLUE protocol utilises the WebRTC Datachannel mechanism for the transport of messages. Datachannel utilizes an SCTP over DTLS transport. At the last Q5/16 meeting in Sapporo C.514r1 which proposed the support of SCTP for the transport of CLUE in H.TPS-SIG was accepted. However it was noted that the support of DTLS is an outstanding part of the solution and that the support of DTLS transports would need to be added.

In order for H.TPS-SIG capable endpoints to utilise the same transport mechanism for CLUE as SIP based endpoints it is proposed to update H.TPS-SIG to incorporate a WebRTC datachannel SCTP over DTLS transport mechanism by making a reference to H.245 Annex U “Support of WebRTC DataChannel.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 169 R3/WP1 from the July 2014 Q.5/16 Rapporteurs’ meeting in Sapporo.

Recommendation ITU-T H.TPS-SIG Signalling for telepresence-enabled conferencing

...

#### 7.2.2.1	SCTP Transport

As per [IETF CLUE DC] the CLUE protocol is transported in a SCTP stream over a DTLS connection. An RTCWEB data channel [IETF RTCWEB DC] is used to establish the CLUE data channels over this connection.

In order to use the same transport, a H.323 endpoint shall establish a WebRTC DataChannel using the procedures from Annex U/[ITU-T H.245]. Once the DTLS handshake has been completed, the device shall establish and use the SCTP association as defined by clause 3.3/[IETF CLUE DC]. It shall use the data channel establishment protocol [IETF DCEP] as per clause 3.2/[IETF CLUE DC] to initiate a bidirectional CLUE channel with its peer.

Once the CLUE channel is established between the peers, they shall perform CLUE signalling as per [IETF CLUE SIG].

_____________________