| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4627 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.TPS-AV: CLUE framework alignment |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes updates to H.TPS-AV to align the draft Recommendation with the current status of the CLUE Framework.

2.	Discussion

This Contribution proposes a number of updates to H.TPS-AV to align it with the current version of the CLUE framework IETF draft <draft-ietf-clue-framework-17>.  The current draft H.TPS-AV (TD 175r2/WP1) references draft-ietf-clue-framework-15. The IETF draft editor has noted that the changes between versions 15 to 16 are:

1. Remove Audio Channel Format attribute

2. Add Audio Capture Sensitivity Pattern attribute

3. Clarify audio spatial information regarding point of capture and point on line of capture. Area of capture does not apply to audio.

4. Update section 12 example for new treatment of audio spatial information.

5. Clean up wording of some definitions, and various places in section 5 and 10.

6. Remove individual encoding parameter paragraph from section 9.

7. Update Advertisement diagram.

8. Update Acknowledgements.

9. References to use cases and requirements now refer to RFCs.

10. Minor editorial changes.

Changes from 16 to 17:

1. Ticket #59 - rename Capture Scene Entry (CSE) to Capture Scene View (CSV)

2. Ticket #60 - rename Global CSE List to Global View List

3. Ticket #61 - Proposal for describing the coordinate system. Describe it better, without conflicts if cameras point in	 different directions.

4. Minor clarifications and improved wording for Synchronisation Identity, MCC, Simultaneous Transmission Set.

5. Add definitions for CLUE-capable device and CLUE-enabled	 call, taken from the signaling draft.

6. Update definitions of Capture Device, Media Consumer, Media Provider, Endpoint, MCU, MCC.

7. Replace "middle box" with "MCU".

8. Explicitly state there can also be Media Captures that are not included in a Capture Scene View.

9. Explicitly state "A single Encoding Group MAY refer to encodings for different media types."

10. In example 12.1.1 add axes and audio captures to the diagram, and describe placement of microphones.

11. Add references to data model and signaling drafts.

12. Split references into Normative and Informative sections. Add heading number for references section.

The Contributors have reviewed the changes and have identified the following impacts:

- To align with CLUE the Audio Channel Format parameter should be removed from clause 7.1.3.3.1/[ITU-T H.TPS-AV].

- As individual encoding information has been removed from CLUE it is proposed to remove the maxH264Mpbs parameter from clause 7.1.2.7.2/[ITU-T H.TPS-AV]. This is a codec specific parameter and is already handled in that coding specification.

- As updates have been made to the “point of capture” and “point on line of capture” regarding audio encoding it is proposed to update clause 7.1.3.2/[ITU-T H.TPS-AV] on audio capture spatial information to capture the updates and to remove the “capture area” parameter.

- Due to the addition of the Audio capture sensitivity pattern to CLUE introduce a new parameter for this in update clause 7.1.3.2/[ITU-T H.TPS-AV].

- To align with the new term “Capture Scene View” clause 7.1.1.3 should be updated from using “Capture Scene Entry” to “Capture Scene View”.

There is no specific H.TPS-AV attribute for a “Global View List” however it is not proposed to add an attribute because it is a CLUE protocol concept for tying Capture Scenes together. The CLUE concepts of “Capture Scene View”, “Signalling Transmission Set” are also not contained in H.TPS-AV.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 175r2/WP1 from the July 2014 Q.5/16 Rapporteurs’ meeting in Sapporo.

Draft new ITU-T H.TPS-AV

Audio/video parameters for telepresence systems

…

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.1361]	ITU-R Recommendation BT.1361 (02/1998), Worldwide unified colorimetry and related characteristics of future television and imaging systems.

[ITU-R BT.2020]	ITU-R Recommendation BT.2020 (08/2012), Parameter values for ultra-high definition television systems for production and international programme exchange.

[ITU-R BT.601]		ITU-R Recommendation BT.601-7 (03/2011), Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange.

[ITU-T G.122]	ITU-T Recommendation G.122 (03/1993), Influence of national systems on stability and talker echo in international connections.

[ITU-T H.241]	ITU-T Recommendation H.241 (02/2012), Extended video procedures and control signals for ITU-T H.300-series terminals.

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.264]	ITU-T Recommendation H.264 (01/2012), Advanced video coding for generic audiovisual services.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T P.300]	ITU-T Recommendation P.300 (11/2001), Transmission performance of group audio terminals (GATs).

[ITU-T P.501]	ITU-T Recommendation P.501 (01/2012), Test signals for use in telephonometry.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE DM]		IETF draft-ietf-clue-data-model-schema-03, An XML schema for the CLUE Data Model.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-16, Framework for Telepresence Multi-Streams.

…

7.1.1.2 Scene Description

| Identity: | Scene Description |
| --- | --- |
| Description: | This parameter provides a textual description of the overall scene. Multiple languages may be included in the description. |
| Format: | List of Strings. |
| Possible Values: | A list of <description text> and <language tag> may be provided. |
| Default: | - |
| Reference: | See the "Description" attribute in 7.3.1 of [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | Describing the scene using text in multiple languages may assist in the consumer being able to make a more informed decision about which scenes to choose in the event that multiple scenes are offered. |

7.1.1.3 Capture Scene View Description

| Identity: | Capture Scene View Description |
| --- | --- |
| Description: | This parameter provides a textual description of the captures that represent a scene. Multiple sets of one or more captures may represent a scene. This description provides text associated with each set. Multiple languages may be included in the description. |
| Format: | List of Strings. |
| Possible Values: | A list of <description text> and <language tag> may be provided. |
| Default: | - |
| Reference: | See the "Description" attribute in 7.3.2 of [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | Describing the set of captures representing a scene the scene using text in multiple languages may assist in the consumer being able to make a more informed decision about which set of captures to choose in the event that multiple set of captures are offered. |

…

7.1.2.7 Video capture encoding information

This clause describes parameters related to encoding information of a single video capture.

7.1.2.7.1	Maximum Video Bitrate

| Identity: | maxVideoBitrate |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single video encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "max-mbps" in [IETF RFC 6184] and "CustomMaxMBPS" in [ITU-T H.241] for further information. |
| Interface: | - |
| Supplementary Information: | - |

{Contributor’s note: It is proposed to remove this parameter as the CLUE framework does not discuss this parameter any more nor is it supported in SDP signalling.}

7.1.2.7.2	Maximum video resolution width

| Identity: | maxWidth |
| --- | --- |
| Description: | This parameter indicates the maximum video resolution width in pixels. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "horizontal image size" in [IETF RFC 6236] and "CustomPictureFormat" in [ITU-T H.245] for further information. |
| Interface: | - |
| Supplementary Information: | - |

7.1.2.7.3	Maximum video resolution height

| Identity: | maxHeight |
| --- | --- |
| Description: | This parameter indicates the maximum video resolution width in pixels. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "vertical image size" in [IETF RFC 6236] and "CustomPictureFormat" in [ITU-T H.245] for further information. |
| Interface: | - |
| Supplementary Information: | - |

7.1.2.7.4	Maximum video framerate

| Identity: | maxFrameRate |
| --- | --- |
| Description: | This parameter indicates the maximum video framerate. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "framerate" in [IETF RFC 4566] and "MaxFPS" in [ITU-T H.241] for further information. |
| Interface: | - |
| Supplementary Information: | - |

7.1.3	Audio parameters

This clause describes the telepresence parameters related to the audio capabilities.

7.1.3.1	Number of audio captures

| Identity: | NumAudioCapture. |
| --- | --- |
| Description: | This parameter indicates the number of audio captures. |
| Format: | Integer |
| Possible Values: | 0 upwards. |
| Default: | - |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | This parameter may be implicitly signalled as a result of describing multiple captures. For example each audio capture in [IETF CLUE FW] represents an audio stream. Therefore the total number of audio captures in a capture scene entry indicates the number of audio streams. |

7.1.3.2	Audio capture spatial information

This clause describes parameters related to spatial information of a single audio capture.

7.1.3.2.1 Audio capture point

The "Capture point" parameter in clause 7.1.2.6.2 applies to audio captures where it describes the nominal midpoint of the microphones.

7.1.3.2.2 Audio point on line of capture

The " Point on line of capture " parameter in clause 7.1.2.6.3 applies to audio captures where it describes axis of the microphones. It may be used with the sensitivity pattern to define a volume where sounds can be expected to be picked up.

7.1.3.2.3 Audio capture sensitivity pattern

| Identity: | AudioCapSensitivity |
| --- | --- |
| Description: | This parameter provides information regarding the nominal sensitivity pattern of the microphone used to capture the audio. |
| Format: | Enumeration |
| Possible Values: | omni, shotgun, cardioid, hyper-cardioid |
| Default: | None. |
| Reference: | See “Audio Capture Sensitivity Pattern” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

{Contributor’s note: It is proposed to add a new parameter for Audio Capture Sensitivity Pattern as this has been added to the CLUE framework. It is also proposed to provide updates to the Capture Point and Line on Capture Point parameters to take into consideration audio aspects. Capture Area is deleted as this no longer applies to audio.}

7.1.3.3	Audio capture encoding information

This clause describes parameters related to encoding information of a single audio capture.

{Contributor’s note: As the Audio Channel Format has been removed from CLUE it is proposed to be removed from the draft H.TPS-AV.}

7.1.3.3.1 Maximum Audio Bitrate

| Identity: | maxAudioBitrate |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single audio encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "bandwidth" in [IETF RFC 4566] and "maxBitRate" in [ITU-T H.245] for further information. |
| Interface: | - |
| Supplementary Information: | - |

…

### I.1.2	Visual parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Video Colour Gamut | Y | N | Signalled as part of the codec information, e.g. in H.264 as described by clause 7.1.2.1. |
| Video LumaBitDepth | Y | N | {Editor’s note: This is signalled as part of the codec payload information. It doesn’t appear to be signalled during session initation signalling, i.e. via SDP or H.245} |
| Video ChromaBitDepth | Y | N | {Editor’s note: This is signalled as part of the codec payload information. It doesn’t appear to be signalled during session initation signalling, i.e. via SDP or H.245} |
| Effective Resolution | N | N | Not signalled. |
| Number of video captures | Y | Y | Implicitly signalled through the actual number of advertised captures in CLUE. |
| Capture area | Y | Y | See "Area of Capture" in [IETF CLUE FW]. |
| Capture point | Y | Y | See "Point of Capture" in [IETF CLUE FW]. |
| Point on line of capture | Y | Y | See the "Point on line of Capture" attribute in [IETF CLUE FW]. |
| Maximum video bitrate | Y | Y | See “max-mbps” in [IETF RFC 6184] and “CustomMaxMBPS” in [ITU-T H.241]. |
| Maximum video resolution width | Y | N | See “horizontal image size” in [IETF RFC 6236] and “CustomPictureFormat” in [ITU-T H.245]. |
| Maximum video resolution height | Y | N | See “vertical image size” in [IETF RFC 6236] and “CustomPictureFormat” in [ITU-T H.245]. |
| Maximum video framerate | Y | N | See "framerate" in [IETF RFC 4566] and “MaxFPS” in [ITU-T H.241]. |

### I.1.3	Audio parameters

| Parameter |  | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- | --- |
| Number of audio captures |  | Y | Y | Implicitly signalled through the actual number of advertised captures in CLUE. |
| Audio Capture point | Y |  | Y | See "Point of Capture" in [IETF CLUE FW]. |
| Audio Point on line of capture | Y |  | Y | See the "Point on line of Capture" attribute in [IETF CLUE FW]. |
| Audio capture sensitivity pattern | Y |  | Y | See the "Audio Capture Sensitivity Pattern" attribute in [IETF CLUE FW]. |
| Maximum audio bitrate |  | Y | Y | See ”bandwidth” in [IETF RFC 4566] and “maxBitRate” in [ITU-T H.245]. |
| Audio Level |  | Y | N | {Editors’s note: Is there a need to send an “audio level” parameter at session initiation? No audio level in H.245 Audio level may be signalled via RTP using IETF RFC 6464.} |
| Sending Loudness Rating |  | N | N | Not signalled. |
| Weighted Microphone-Loudspeaker Coupling Loss |  | N | N | Not signalled. |

____________________