| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4644 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Update of SCTP transport in H.TPS-SIG |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution discusses the update of SCTP in H.TPS-SIG.

2.	Discussion

At the Sapporo meeting, a contribution C.515 about SCTP media transport in H.323 was introduced in Q2. Q2 experts reviewed it and accepted to add the support of SCTP to H.245. The output text can be found in TD 171R1/WP1. One of the changes is the addition of SCTP usage to the existing H.245 messages, which can be found in MediaTransportType and DataProtocolCapability of Annex A. Thus, three different usages of SCTP can be supported: SCTP, DTLS-SCTP, and SCTP-DTLS. Then in Annex U, the support of WebRTC DataChannel is introduced.

In section 7.2 of H.TPS-SIG, carriage of CLUE messages in H.323 systems is discussed. SCTP transport for CLUE messaging is described in sub-section 7.2.2.1. In this sub-section, DTLS/SCTP is used for the CLUE channel, whose procedures reference Annex U of [ITU-T H.245].

The current sub-section 7.2.2.1 does discuss how the use of a DTLS/SCTP stream is determined. The contributors believe it is worth including how the DTLS/SCTP stream is negotiated in this text.

Also, a few editorial changes are made to clarify the whole procedure with specific H.245 messages.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 169 R3/WP1 from the meeting on July 2014 Q.5/16 in Sapporo.

Recommendation ITU-T H.TPS-SIG Signalling for telepresence-enabled conferencing

...

#### 7.2.2.1	SCTP Transport

During capability exchange, an H.323 endpoint shall receive a TerminalCapabilitySet message with SCTP usage and parameters from the peer. In order to use CLUE over SCTP the choice of a DTLS/SCTP transport shall be negotiated (via TerminalCapabilitySet messaging) prior to the establishment of a data channel.

As per [IETF CLUE DC] the CLUE protocol is transported in a SCTP stream over a DTLS connection. An RTCWEB data channel [IETF RTCWEB DC] is used to establish the CLUE data channels over this connection.

In order to use the same transport, after the capability exchange, OpenLogicChannel messages are used to establish a bidirectional logical channel utilising an DTLS/SCTP transport with the negotiated capabilities using the procedures from Annex U/[ITU-T H.245]. Firstly, a DTLS connection is setup. Once the DTLS handshake has been completed, the device shall establish an SCTP association accordingly. It shall then use the data channel establishment protocol [IETF DCEP] as per clause 3.2/[IETF CLUE DC] to initiate a bidirectional CLUE channel with its peer.

Once the CLUE channel is established between the peers, they shall perform CLUE signalling over the negotiated DTLS-SCTP association as per [IETF CLUE SIG].

_________________