| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4652 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE |  |  |  |
| Title: | H.TPS-SIG: Basic call session setup |  |  |  |
| Purpose: | Proposal |  |  |  |

1	Introduction

This contribution proposes to add the description of basic telepresence call session setup flows between the endpoints.

2	Discussion

The telepresence endpoint should carry the CLUEControlCapability during call session setup procedure using GEF. The most direct basic CLUE call establishment procedure is that two endpoints carry the CLUEControlCapability in the support feature to indicate the support of CLUE signalling. if one endpoint can’t understand CLUEControlCapability feature in the support feature it should ignore it instead of release the call.

It’s not recommend for the initiator of a call to carry the CLUEControlCapability in the needed feature.  If the peer can’t support the CLUEControlCapability, the call will be released and unsuccessful.

# 3	Proposal

It is proposed to make these changes in H.TPS-SIG. Change is marked up against TD 169 R3/WP1 from the last meeting.

[Begin Proposal]

## 8.2	Call session setup

If two endpoints both support CLUE signalling the normal basic call session setup procedure should be followed as the figure below.

Figure ? Basic CLUE call establishment procedure

Telepresence endpoint A carries the CLUEControlCapability in supportedFeatures field in order to signal that it supports CLUE signalling. When telepresence endpoint B receives the setup message it knows the peer supports CLUE signalling.  Therefore B responses with Call proceeding message also carrying supported feature CLUEControlCapability for A to know it supports CLUE signalling too. After Alerting and Connect message a normal basic CLUE call is established.  The CLUEControlCapability feature also can be carried in Altering or Connect messages.

If one endpoint does not support the CLUEControlCapaility a non-CLUE traditional H.323 call session setup procedure should be followed as the figure below.

Figure ? Non-CLUE call establishment procedure

Telepresence endpoint A carries the CLUEControlCapability in supportedFeatures field in order to signal that it supports CLUE signalling. When telepresence endpoint B receives the setup message it knows the peer supports CLUE signalling. But B does not support CLUE signalling therefore B ignores THE CLUEControlCapability and responses with Call proceeding message carrying none supported feature for A to know it does not supports CLUE signalling. After Alerting and Connect message a Non-CLUE traditional H.323 call is established.

[End Proposal]

_________________