| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4655 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.ALTC: new work item "Alternate IP connectivity support by H.248 gateway control protocol" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a new work item for a new H.-series Supplement related to "Alternate IP connectivity support by H.248 gateway control protocol", as well as an initial draft.

Discussion

This contribution is driven by parallel discussions in 3GPP concerning the addition of ALTC support in 3GPP defined H.248 profile specifications. It looks like that the set of stage 2 requirements could be limited to the absolute minimum, which would allow to support the "simplest stage 3 solution" as possible. However, there might be H.248 profiles with additional ALTC requirements or/and consideration with respect to potential interactions with other mechanisms (beyond ICE and SDP offer/answer models). Therefore it might be beneficial to document the ALTC discussions in such a Supplement.

We could imagine following Supplement structure:

- network model for ALTC with focus on H.248 entities (clause 6);

- discussion of principle ALTC solution options from H.248 side (clause 7);

- typical H.248 signalling requirements related to ALTC (clause 8), i.e., the so-called stage 2 information for H.248 profiles;

- evaluation and discussion of signalling examples in order to challenge H.248 ALTC solutions from clause 7 (clause 9); and

- H.248 profile guidelines, i.e., stage 3 recommendations (clause 10).

The contribution proposes initial input material (based from ALTC discussions from the past 18 months).

____________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.ALTC Ed. 0.1 | Initial draft (AVD-4655) to WP 1/16 Seoul (KR) meeting (2014-11) Location: |
| H.Sup.ALTC Ed. 0.2 | Output (TD-##) from WP 1/16 Seoul (KR) meeting (2014-11) Location: |
| H.Sup.ALTC Ed. 0.3 |  |
| H.Sup.ALTC Ed. 0.4 |  |
| H.Sup.ALTC Ed. 0.5 |  |
| H.Sup.ALTC Ed. 0.6 |  |
| H.Sup.ALTC Ed. 0.7 |  |
| H.Sup.ALTC Ed. 0.8 |  |
| H.Sup.ALTC Ed. 0.9 |  |
| H.Sup.ALTC Ed. 0.10 |  |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Supplement 6 4 Abbreviations and acronyms 6 5 Conventions 6 6 Network model 6 7 Support of alternate IP connectivity: principle options from H.248 side 7 7.1 Solution options: basic H.248 context models and H.248 media descriptor elements 8 7.1.1 H.248 ALTC context model "M1": reservation of two H.248 contexts 8 7.1.2 H.248 ALTC context model "M2": reservation of two H.248 terminations 9 7.1.3 H.248 ALTC context model "M3": reservation of two H.248 stream endpoints 10 7.1.4 H.248 ALTC context model "M4": reservation of two H.248 media groups 11 7.1.5 H.248 ALTC context model "M5": reservation of a "dual-stack …" 11 7.2 Solution options: complementary H.248 resource usage indication information 11 7.2.1 H.248 signalling element "S1": SDP-based ALTC call type indication 11 7.2.2 H.248 signalling element "S2": Termination connection model package (H.248.51) 12 7.2.3 H.248 signalling element "S3": usage of media mixer resources (H.248.9) 12 7.2.4 H.248 signalling element "S4": Resource management package(s) (H.248.63) 12 7.2.5 H.248 signalling element "S5": SDP Offer/Answer model (H.248.80) 12 8 Requirements (from H.248 profile perspective) 12 8.1 Communication service: number of media components 12 8.2 Communication service: early end-to-end IP connectivity 12 8.3 Call control signaling: number of negotiation cycles 13 8.4 Gateway control signaling: resource management process in H.248 MG 13 8.5 Gateway control signaling: explicit ALTC "call" tagging 14 8.6 Gateway control signaling: specific resource components in H.248 MG 14 9 Discussion 14 9.1 Call control signalling: examples of SIP SDP offers 14 9.1.1 SIP SDP offer example 1: monomedia audio telephony 14 9.1.2 SIP SDP offer example 2: multimedia WebRTC call 15 9.2 Gateway control signalling: examples 15 9.2.1 Example H.248 signalling in case of "application aware IWF" 15 9.2.2 Example H.248 signalling in case of "application agnostic IWF" 20 10 Profile guidelines 23

List of Tables

Page

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1 17 Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2 18 Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request 19 Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply 20 Table 9.2.1.1/3 – Example command encoding – Step 3 – MGC request 21 Table 9.2.1.1/4 – Example command encoding – Step 4 – MG reply 21 Table 9.2.1.2/1 – Example command encoding – Step 1 – MGC request 21 Table 9.2.1.2/2 – Example command encoding – Step 2 – MG reply 21 Table 9.2.1.2/3 – Example command encoding – Step 3 – MGC request 21 Table 9.2.1.2/4 – Example command encoding – Step 4 – MG reply 21 Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request 22 Table 9.2.1.3/2 – Example command encoding – Step 2 – MG reply 22 Table 9.2.1.3/3 – Example command encoding – Step 3 – MGC request 22 Table 9.2.1.3/4 – Example command encoding – Step 4 – MG reply 22 Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request 22 Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply 23 Table 9.2.1.4/3 – Example command encoding – Step 3 – MGC request 23 Table 9.2.1.4/4 – Example command encoding – Step 4 – MG reply 23 Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request 23 Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply 23 Table 9.2.2.1/3 – Example command encoding – Step 3 – MGC request 23 Table 9.2.2.1/4 – Example command encoding – Step 4 – MG reply 24 Table 9.2.2.2/1 – Example command encoding – Step 1 – MGC request 24 Table 9.2.2.2/2 – Example command encoding – Step 2 – MG reply 24 Table 9.2.2.2/3 – Example command encoding – Step 3 – MGC request 24 Table 9.2.2.2/4 – Example command encoding – Step 4 – MG reply 24 Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request 25 Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply 25 Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request 25 Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply 25 Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request 25 Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply 25 Table 9.2.2.4/3 – Example command encoding – Step 3 – MGC request 26 Table 9.2.2.4/4 – Example command encoding – Step 4 – MG reply 26

List of Figures

Page

Figure 1 – Network model for alternate IP connectivity from H.248 perspective 10 Figure 2 – H.248 ALTC context model "M1": reservation of two H.248 contexts 11 Figure 3 – H.248 ALTC context model "M2a": reservation of two H.248 terminations 12 Figure 4 – H.248 ALTC context model "M2b": reservation of two H.248 terminations 13 Figure 5 – H.248 ALTC context model "M3": reservation of two H.248 stream endpoints 13 Figure 6 – H.248 ALTC context model "M4": reservation of two H.248 media groups 14

Draft New Supplement # to the ITU-T H-Series (H.Sup.ALTC)

Gateway control protocol: 
Alternate IP connectivity support by H.248 entities

Summary

<Mandatory material>

Keywords

ALTC

Introduction

The alternate IP connectivity capability results from transitioning scenarios in evolving IP version 4 networks towards IP version 6. The reality of IPv4-IPv6 coexistence introduces heterogeneous scenarios with combinations of IPv4 and IPv6 nodes, some of which are capable of supporting both IPv4 and IPv6 dual-stack (DS) and some of which are capable of supporting only IPv4 or only IPv6. In this context, H.248 entitities could be involved in alternate IP connectivity scenarios, at least when the H.248 media gateway support dual-stack (DS) capable IP interfaces.

# 1	Scope

The capability of alternate IP connectivity (ALTC) is basically supported by H.248 entities, without any need to extend the existing H.248 protocol capabilities. However, there are in principle multiple solutions possible how a particular H.248 profile could define ALTC support.

The purpose of this Supplement is to provide guidance on this topic. The Supplement is structured as follows:

- network model for ALTC with focus on H.248 entities (clause 6);

- discussion of principle ALTC solution options from H.248 side (clause 7);

- typical H.248 signalling requirements related to ALTC (clause 8), i.e., the so-called stage 2 information for H.248 profiles;

- evaluation and discussion of signalling examples in order to challenge H.248 ALTC solutions from clause 7 (clause 9); and

- H.248 profile guidelines, i.e., stage 3 recommendations (clause 10).

{Editor's note (2014-11): the document is positioned as Supplement because we don't expect any normative H.248 extensions, at least at this stage of the discussion …}

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (10/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (01/2014), Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248.

[IETF RFC 6947]	IETF RFC 6947 (05/2013), The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute.

[ETSI TS 183 018]	ETSI TS 183 018 V3.5.2 (01/2010), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.

[ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

# 3	Definitions

## 3.1	Terms defined elsewhere

None.

## 3.2	Terms defined in this Supplement

3.2.1	Alternate (IP) Connectivity: <definition> FIXTHIS

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| ALTC | Alternate (IP) Connectivity |
| --- | --- |
| B2BIH | Back-to-Back IP Host (mode according to [ITU-T H.248.64])] |
| DS | Dual-Stack (parallel support of V4 and V6) |
| GCP | Gateway Control Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NA(P)T-PT | Network Address (and Port) Translation plus (Internet) Protocol (Version) Translation |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| UA | (SIP) User Agent |
| UE | User Equipment |
| V4 | Internet Protocol version 4 |
| V6 | Internet Protocol version 6 |

# 5	Conventions

None.

# 6	Network model

There will be two interconnected IP network domains (labelled as X and Y in Figure 1) in case of ALTC support by H.248 gateways:

Figure 1 – Network model for alternate IP connectivity from H.248 perspective

It is sufficient to consider an end-to-end network configuration where at least one domain supports IPv4 and IPv6 connectivity and dualstack capable IP host entities (here domain Y). The SIP B2BUA entity receives a SDP offer with SDP "a=altc" qualified media descriptions from X or Y, dependent on the call establishment direction. The SIP level SDP offer/answer procedures are subject of [IETF RFC 6947] and out of scope of this Recommendation.

The H.248 IP-to-IP media gateway is most likely configured for a B2BIH mode, especially due to potential IP version translation in case of a final V4-to-V6 configuration. This is also known as NAT46 (of NAT64) mode. IP version translation is only possible in B2BIH mode (and not in IPR mode).

Therefore the assumption of B2BIH mode, despite the fact of scenarios with final V4-only or V6-only end-to-end IP connectivity.

# 7	Support of alternate IP connectivity: principle options from H.248 side

The SDP based ALTC qualification of media descriptions is firstly only required at the SIP-based call control level. H.248 gateway control signalling for ALTC support does not necessarily require such kind of SDP tagging due to alternative solutions concerning media plane IP resource reservations.

## 7.1	Solution options: basic H.248 context models and H.248 media descriptor elements

The illustrations take the example of a dualmedia call with audio and video over RTP, inclusive RTCP control flows.

## 7.1.1	H.248 ALTC context model "M1": reservation of two H.248 contexts

Figure 2 illustrates how such a model could look like. Termination TX serves the bearer traffic towards UE_X and is out of consideration in this Supplement. The bearer traffic for the preferred IP connection is assigned to Termination TY,P. Both Terminations (TX and TY,P) are created in Context Cz1. The alternate IP connection is assigned to Termination TY,A, which is created in a separate Context Cz2.

Figure 2 – H.248 ALTC context model "M1": reservation of two H.248 contexts

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) and replaced by Termination TY,A (via H.248 MOVe command); or

- Termination TY,A is deleted (via H.248 SUBtract command).

The 2nd Context Cz2 disappears again in both cases.

The motivation for the "two context" model (M1) might be the isolation of resources associated with the alternate IP connection, given the assumption of an estimated low probability that these resources will be finally used.

Model (M1) looks also fairly attractive from H.248 control perspective because Context Cz1 represents the usual context configuration, without any ALTC specific aspects.

### 7.1.2	H.248 ALTC context model "M2": reservation of two H.248 terminations

The alternate IP connection is assigned to a self-contained Termination TY,A in model M2, similar as in previous model M1. However, there is only a single context from the very beginning, see Figure 3:

Figure 3 – H.248 ALTC context model "M2a": reservation of two H.248 terminations

It is obvious that there are some context-internal resource management aspects during the temporary phase when both Terminations TY,P and TY,A exists:

- the IWF resources for media processing (here: audio transcoding IWF and video transparent forwarding IWF) might be already prepared and internally connected to the external IP interfaces …

- … raising the question of the internal H.248 stream topology (e.g., topologies [TX(S1), TY,P(S1), oneway] plus [TX(S1), TY,A(S1), oneway]).

Hence, there might be the option of shared resources (e.g., all three SEPs S1 are connected to the same audio transcoder entity), or separate resources (i.e., for "preferred" and "alternate" SEPs S1) are prepared. This is a remarkable aspect of model M2a, but should be implementation specific (to be confirmed), hence out of scope of this Supplement.

Purpose of model M2a is to serve the possible requirement of early media (see R-8.2/2 in clause 8). If such a requirements does not exist in an H.248 profile, then the consideration of a simpler model might be sufficient, see model M2b in Figure 4:

Figure 4 – H.248 ALTC context model "M2b": reservation of two H.248 terminations

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) or

- Termination TY,A is deleted (via H.248 SUBtract command).

The context internal cut-through occurs either "as soon as possible" (M2a) or with the final context configuration (M2b).

### 7.1.3	H.248 ALTC context model "M3": reservation of two H.248 stream endpoints

NOTE – Model M3 is conditional, limited on communication services related to single-stream-per-termination structures!

We consider the example of monomedia call (here: audio), resulting in a single H.248 stream (S1) for the preferred IP configurations (Figure 5). The alternate IP connection resources are defined via a separate H.248 stream descriptor (see SEP S2). Both SEPs S1 and S2 belong to the same Termination TY:

Figure 5 – H.248 ALTC context model "M3": reservation of two H.248 stream endpoints

When the final IP connection option is negotiated, when either

- SEP TY(S1) is deleted again (via H.248 SUBtract command) or

- SEP TY(S2) is deleted (via H.248 SUBtract command).

The interconnection of SEP TY(S1) with SEP TX(S1) is straightforward, and the interconnection of SEP TY(S2) with SEP TX(S1) may require capabilities as defined by [ITU-T H.248.STGROUP] (to be confirmed).

Model M3 looks attractive due to the simplicity of single terminations only. However, the context model does not provide a general purpose solution as mentioned.

### 7.1.4	H.248 ALTC context model "M4": reservation of two H.248 media groups

Another approach may be through the use of the H.248 LocalControl properties ReserveValue and ReserveGroup, which would allow two addresses to be indicated. The semantic of these LocalControl properties is that only one resource is used at a time. The unused one can be removed at a later date. There would be two H.248 media groups reserved (to be confirmed), see Figure 5:

Figure 6 – H.248 ALTC context model "M4": reservation of two H.248 media groups

Model M4 may result in quite extensive H.248 stream descriptors …

### 7.1.5	H.248 ALTC context model "M5": reservation of a "dual-stack …"

… model of a single resource component "dual-stack IP interface" (as kind of alias addresses) … for further studies …

## 7.2	Solution options: complementary H.248 resource usage indication information

The qualification of an H.248 context (or stream) for "ALTC awareness" might improve the MG level resource management.

### 7.2.1	H.248 signalling element "S1": SDP-based ALTC call type indication

It could be imagined that the occurrence of SDP attribute "a=altc:" in the H.248 Local and/or Remote descriptor will be assigned the semantic, that the MG could qualify associated resources of type "ALTC". E.g., resources which are only temporarily reserved, but finally not used and released immediately after call establishment completion.

FIXTHIS – check applicability

### 7.2.2	H.248 signalling element "S2": Termination connection model package (H.248.51)

FIXTHIS – check applicability

### 7.2.3	H.248 signalling element "S3": usage of media mixer resources (H.248.9)

FIXTHIS – check applicability

### 7.2.4	H.248 signalling element "S4": Resource management package(s) (H.248.63)

FIXTHIS – check applicability

### 7.2.5	H.248 signalling element "S5": SDP Offer/Answer model (H.248.80)

The SDP-based ALTC signalling method at SIP level is independent of the SDP offer/answer model (see clause 4.2.4/[IETF RFC 6947]), thus, the SDP attribute "a=altc:" may be used in the legacy SDP offer/answer (see clause 3.2.3.1/[ITU-T H.248.80]) and revised SDP offer/answer (see clause 3.2.3.2/[ITU-T H.248.80]) syntax.

# 8	Requirements (from H.248 profile perspective)

There are multiple solutions for ALTC support (see previous clause), but some of them are conditional dependent on typically H.248 profile dependent requirements. The purpose of this clause is to highlight such kind of capabilities.

## 8.1	Communication service: number of media components

This aspect affects normally the number of required H.248 streams (and therefore also the internal context topology).

R-8.1/1: Support of single media descriptions only per H.248 context ("monomedia call").

R-8.1/2: Support of multiple media descriptions per H.248 context ("multimedia call").

## 8.2	Communication service: early end-to-end IP connectivity

The ALTC feature is subject of the call establishment phase. There are options how fast the end-to-end bearer establishment is finalized.

R-8.2/1: End-to-end IP connectivity should be available with completion of the end-to-end call control signalling.

R-8.2/2: End-to-end IP connectivity should be available as soon as possible, i.e., the MG should internally cut-through the bearer path as soon as fully specified media and transport descriptions are available for individual traffic directions.

NOTE – Requirement R-8.1/2 is also known as "early media", e.g., the called communication endpoint could already sent media in backward direction to the calling communication endpoint when acknowledging an incoming communication request.

It has to be noted that the temporary phase between a partial H.248 context configuration ("which allows early media") and the final complete H.248 context configuration is typically lasting only a few tens or hundreds of milliseconds. Thus, support of requirement R-8.2/1 should be normally more than sufficient for call controlled communication services, inclusive the category of realtime services.

There are two follow-up requirements for the early media capability (R-8.2/2):

R-8.2/3: The MG should support unidirectional early media, from the terminating (altc) UE back to the originating UE.

R-8.2/4: The MG should support bidirectional early media between both UEs.

Above requirements impact the context internal model and topology, e.g. oneway or bothway topology, and may reduce the set context model options (as outlined in clause 7.1).

## 8.3	Call control signaling: number of negotiation cycles

The indication and negotiation of media configurations at call control signalling level could theoretically subject of multiple (SDP offer/answer) negotiation cycles. The number of cycles is usually only bounded by performance objectives concerning call establishment delays.

R-8.3/1: Network solutions aims for single cycle negotiations, i.e., there is only one SDP offer and one SDP answer at SIP level.

R-8.3/2: Network solutions allow multi cycle negotiations.

Normally, the number of H.248 resource reservation cycles is decoupled and independent from call control procedures, however, incoming SIP messages usually acting as direct stimuli for H.248 signalling actions.

## 8.4	Gateway control signaling: resource management process in H.248 MG

Following resource management models might be worth of consideration:

- model of resource "protocol stack":

- monolithic resource model, i.e., an entire SDP media description is modelled as a single MG resource;

- decomposed resource model by separating the protocol stack in a lower segment of network/transport resources and a upper segment of the actual media-related resources.

NOTE – The decomposed model (1b) is already anchored in the ETSI TISPAN Ia profile (see [ETSI TS 183 018], clause 5.17.1.11 "One-Stage and Two-Stage BGF Resource Reservation") by splitting the protocol stack in SDP "c="-line and SDP "m="-line resource partitions.

- state model of MG resources:

- two-state model, i.e., a particular resource is either yet released or completely reserved and allocated;

- three-state model by consideration of an explicit reservation state besides the final allocation state.

The three state model allows the pure logical handling of MG resources during the temporary phase of ALTC. SDP media descriptions which are only partially specified (or underspecified) result in the reservation of resources (at a logical level, hence such resources are not yet able to be used for real traffic). The finally completed SDP media specifications will then result in the actual allocation of MG resources.

It is obvious that there will be trade-off decisions between "early end-to-end connectivity and media path usage" versus "MG resource management complexity". It is not the purpose of this Supplement to evaluate this topic in more detail, but there might be at least following requirements postulated due to H.248 profiles according to [ETSI TS 183 018]:

R-8.4/1: The MGC-to-MG resource reservation process follows the one-stage model, i.e., available SDP "m="-line is already signalled together with "c="-line information.

R-8.4/2: The MGC-to-MG resource reservation process supports the two-stage model, i.e., the MGC would provide in a first phase IP connection resources only ("which would be here SDP "c="-line plus "a=altc:"-lines"), without yet media type / media format information. The "m="-line part would be then subjedt of the second phase in case of media aware type of IWF.

## 8.5	Gateway control signaling: explicit ALTC "call" tagging

The ALTC network service might be basically fully transparent to H.248 entities (e.g., in case of a lacking ALTC signalling element at the H.248 interface), however, the MG might support optimized resource management when knowing that a particular call contains "ALTC resource components". Resulting in following requirements:

R-8.5/1: ALTC could be transparent to the H.248 MG. The MG processes "ALTC context configuration" requests as regular communication services.

NOTE – This requirement could result in inefficient MG resource usage and decreased MG capacity.

R-8.5/2: Support of explicit ALTC context tagging (via a ContextAttribute, or Termination-level property, or SDP element, or …).

## 8.6	Gateway control signaling: specific resource components in H.248 MG

For further studies ("inserted due to email comments"):

R-8.6/1: ALTC should be supported by "conferencing resources".

R-8.6/2: …

# 9	Discussion

The various solution options from clause 7 are evaluated by considering example signalling scenarios.

### 9.1	Call control signalling: examples of SIP SDP offers

### 9.1.1	SIP SDP offer example 1: monomedia audio telephony

A typical SIP-level SDP offer is illustrated in Table 9.1.1:

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... c=IN IP4 <UE_A_IPv4_addr_data> m=audio <UE_A_port_audio> RTP/AVP <fmt> … ; SDP attributes for audio format description(s) a=altc:1 IP6 <UE_A_IPv6_addr_data> a=altc:2 IP4 <UE_A_IPv4_addr_data> … | The "altc" attribute is a media-level-only attribute and MUST NOT appear at the SDP session-level. The "c="-line is at session-level in example 1. |

### 9.1.2	SIP SDP offer example 2: multimedia WebRTC call

Consideration of a dualmedia WebRTC call with media components audio and video. A typical SIP-level SDP offer is illustrated in Table 9.1.2:

Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... a=group:BUNDLE audio video ; m=audio <UE_A_port_audio_rtp> RTP/SAVPF 103 104 0 8 106 105 13 126 c=IN IP4 <UE_A_IP_addr_audio_rtp> a=rtcp:<UE_A_port_audio_rtcp> IN IP4 <UE_A_IP@_audio_rtcp> a=mid:audio a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrecv a=altc:1 IP6 <UE_A_IPv6_addr_audio data> a=altc:2 IP4 <UE_A_IPv4_addr_audio data> ; m=video <UE_A_port_video_rtp> RTP/SAVPF 100 101 102 c=IN IP4 <UE_A_IP_addr_video_rtp> a=rtcp:<UE_A_port_video_rtcp> IN IP4 <UE_A_IP@_video_rtcp> a=mid:video a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrcv a=altc:1 IP6 <UE_A_IPv6_addr_video data> a=altc:2 IP4 <UE_A_IPv4_addr_video data> … | There are two media descriptions for audio and video. Both are grouped via bundling. The SIP UA provides a multihomed IP host capability (inclusive dual-stack IP interfaces) and wants to use different IP interfaces for each media component. Hence, there are SDP media-level "c="-lines (in contrast to example 1). There are consequently two pairs of "a=altc:"-lines, one per media description. Furthermore, RTCP control flows could be either transport multiplexed (via "a=rtcp-mux") or transport unmultiplexed by allocation of an explicit transport port (via "a=rtcp:"). |

The 2nd SDP offer example should be more suitable for the demonstration of ALTC support by H.248 entities due to variety of IP interfaces, alternatives, transport multiplexing and RTCP port allocation options.

It has to be noted that the typical ICE functionality (as part of WebRTC call control signalling) is not considered here. There would not be any added value, see see clause 4.2.3/[IETF RFC 6947] "Interaction with ICE".

## 9.2	Gateway control signalling: examples

(SIP) SDP Offer example 2 is used for below H.248 signalling examples. H.248 signalling illustrates only the "ALTC side" of an H.248 context.

## 9.2.1	Example H.248 signalling in case of "application aware IWF"

Thus, there would be fully specified media descriptions.

### 9.2.1.1	Usage of H.248 ALTC context model "M1" (two contexts)

The SIP-level SDP offer stimulates an H.248 transaction request from MGC to MG (Table 9.2.1.1/1). The MGC selects the two context model "M1" (as illustrated in Figure 2).

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.1/1 and Table 9.2.1.1/2:

Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=audio <UE_A_port_audio_rtp> RTP/SAVPF 103 104 0 8 106 105 13 126 c=IN IP4 <UE_A_IP_addr_audio_rtp> a=rtcp:<UE_A_port_audio_rtcp> IN IP4 <UE_A_IP@_audio_rtcp> a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrecv a=altc:1 IP6 <UE_A_IPv6_addr_audio data> a=altc:2 IP4 <UE_A_IPv4_addr_audio data>… }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=video <UE_A_port_video_rtp> RTP/SAVPF 100 101 102 c=IN IP4 <UE_A_IP_addr_video_rtp> a=rtcp:<UE_A_port_video_rtcp> IN IP4 <UE_A_IP@_video_rtcp> a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrcv a=altc:1 IP6 <UE_A_IPv6_addr_video data> a=altc:2 IP4 <UE_A_IPv4_addr_video data> }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } Transaction = 2 { Context = $ { ; Context CZ2 Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=audio <UE_A_port_audio_rtp> RTP/SAVPF 103 104 0 8 106 105 13 126 c=IN IP4 <UE_A_IP_addr_audio_rtp> a=rtcp:<UE_A_port_audio_rtcp> IN IP4 <UE_A_IP@_audio_rtcp> a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrecv a=altc:1 IP6 <UE_A_IPv6_addr_audio data> a=altc:2 IP4 <UE_A_IPv4_addr_audio data>… }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=video <UE_A_port_video_rtp> RTP/SAVPF 100 101 102 c=IN IP4 <UE_A_IP_addr_video_rtp> a=rtcp:<UE_A_port_video_rtcp> IN IP4 <UE_A_IP@_video_rtcp> a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrcv a=altc:1 IP6 <UE_A_IPv6_addr_video data> a=altc:2 IP4 <UE_A_IPv4_addr_video data> }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | The ADD.request command for Termination TX is omitted. … NOTE … this is just a quick copy & paste syntax, not yet checked for correctness, … also bundling / grouping was not considered … |

A possible example MG reply is indicated in Table 9.2.1.1/2:

Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.1/3 and Table 9.2.1.1/.4:

Table 9.2.1.1/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.1/4:

Table 9.2.1.1/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.1.2	Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.2/1 and Table 9.2.1.2/2:

Table 9.2.1.2/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.2/2:

Table 9.2.1.2/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.2/3 and Table 9.2.1.2/.4:

Table 9.2.1.2/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.2/4:

Table 9.2.1.2/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.1.3	Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.3/1 and Table 9.2.1.3/2:

Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.3/2:

Table 9.2.1.3/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.3/3 and Table 9.2.1.3/.4:

Table 9.2.1.3/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.3/4:

Table 9.2.1.3/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.1.4	Usage of H.248 ALTC context model "M3" (two media groups for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.4/1 and Table 9.2.1.4/2:

Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.4/2:

Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.4/3 and Table 9.2.1.4/.4:

Table 9.2.1.4/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.4/4:

Table 9.2.1.4/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

### 9.2.2	Example H.248 signalling in case of "application agnostic IWF"

#### 9.2.2.1	Usage of H.248 ALTC context model "M1" (two contexts)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.1/1 and Table 9.2.2.1/2:

Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.1/2:

Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.1/3 and Table 9.2.2.1/.4:

Table 9.2.2.1/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.1/4:

Table 9.2.2.1/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.2	Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.2/1 and Table 9.2.2.2/2:

Table 9.2.2.2/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.2/2:

Table 9.2.2.2/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.2/3 and Table 9.2.2.2/.4:

Table 9.2.2.2/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.2/4:

Table 9.2.2.2/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.3	Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.3/1 and Table 9.2.2.3/2:

Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/2:

Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.3/3 and Table 9.2.2.3/.4:

Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/4:

Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.4	Usage of H.248 ALTC context model "M3" (two media groups for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.4/1 and Table 9.2.2.4/2:

Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.4/2:

Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.4/3 and Table 9.2.2.4/.4:

Table 9.2.2.4/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.4/4:

Table 9.2.2.4/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

# 10	Profile guidelines

FIXTHIS

Appendix I

Example H.248 profile: 3GPP Iq profile

## I.1	Summary of stage 2 requirements

{Contributor's note (2014-11): there might be following stage 2 summary after a quick check:

…for further studies}

Bibliography

[b-ETSI TR 183 046]	ETSI TR 183 046 V3.3.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); SDP Interworking between Call/Session Control Protocols (SIP/SDP, RTSP/SDP; etc.) and the Gateway Control Protocol (H.248/SDP).

__________________

See e.g. Figure G.2 in [ETSI TR 183 068].