| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4658 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | H.TPS-AV: Parameter Interfaces |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Although interfaces are in the parameter template for H.TPS-AV, and interfaces are defined in H.420, none of the defined parameters include the interface.  Polycom proposes interfaces for each parameter.

# Introduction

As H.TPS-AV moves toward consent, it is important that the text be polished over the coming meetings in order to minimize last-minute adjustments.  Review of the current parameters reveals that no interfaces are included.  Clearly this needs to be corrected.

# Discussion

One option is to simply remove the interfaces from the template, and also delete them from all the existing parameter descriptions.  However, Polycom believes it is better to include the interface names, especially when the CLUE data channel will be used in some form to supplement H.245 signalling.

The current H.TPS-AV parameter template refers to the interface names used in section 9 of ITU-T Rec. H.420.  One potential problem with this approach is that the interfaces specified there do not provide a way to distinguish parameters carried in the CLUE signalling channel from parameters carried in telepresence extensions that might be added to the existing H.245 channel.  Similarly the media interface labels do not allow RTP, RTCP and in-band codec messages (e.g., H.264 SEI) to be distinguished.

A second potential problem is that the H.420 interfaces identify the two components being connected in the interface number.  For instance, C5 is between two telepresence endpoints, C2 is between the telepresence endpoint and an MCU, and C4 is between a telepresence endpoint and legacy endpoint.  A reference to “Interface C” is underspecified, and listing all the “C” interfaces that apply between telepresence devices seems burdensome.

Therefore, Polycom proposes instead to identify the parameter carriage more specifically. Parameters expected to be carried via CLUE use interface “CLUE”.  Existing video parameters like maxH264Mpbs use the interfaces “H.241” and “SDP”.  Parameters carried in H.264 SEI messages (such as LumaBitDepth) use the “interface” H.264 SEI.  We believe that this would be more useful than employing the interface labels in H.420.

Finally, we propose that parameters that are not signalled say that explicitly.

The interfaces that result are shown in Table 1.  Parameters for which signaling has not yet been proposed (but which likely need signalling) are listed as TBD.

| H.TPS-AV (current name) | Interface |
| --- | --- |
| CapturedMediaType | CLUE |
| Scene Description | CLUE |
| Capture Scene Description | CLUE |
| Language | CLUE |
| Priority | CLUE |
| Embedded text | CLUE |
| Related to | CLUE |
| Presentation | CLUE |
| PersonInfo | CLUE |
| PersonType | CLUE |
| Scene Information | CLUE |
| Capture Description | CLUE |
| AreaScale | CLUE |
| Mobility of Capture | CLUE |
| View | CLUE |
| maxGroupBitrate | CLUE |
| Max Capture Encodings | CLUE |
| ColourGamut | H.264 and H.265 SEI |
| LumaBitDepth | H.264 and H.265 SEI |
| ChromaBitDepth | H.264 and H.265 SEI |
| EffectiveResolution | Not Signalled |
| CapArea | CLUE |
| CapPoint | CLUE |
| PointonlineofCap | CLUE |
| maxVideoBitrate | H.241, SDP |
| maxH264Mpbs | H.241, SDP |
| maxWidth | H.241, SDP |
| maxHeight | H.241, SDP |
| maxFrameRate | H.241, SDP |
| AudioChannelFormat | H.245, SDP |
| maxAudioBitrate | H.245, SDP |
| AudioLevel | TBD |
| SendLoudnessRating | TBD |
| weightedMicrophoneLoudspeakerCouplingLoss | Not Signalled |
| EndtoEndVideoDelay | Not Signalled |
| EndtoEndAudioDelay | Not Signalled |
| AudioVideoSynchronization | Not Signalled |
| MultiContentCapture | CLUE |
| MccSources | CLUE |
| MaxMccSources | CLUE |
| MccPolicy | CLUE |
| MccSynch | CLUE |
| ReceiveLoudnessRating | TBD |
| Type | TBD |
| CRI | TBD |
| ColTemp | TBD |
| ReverberationTime | Not Signalled |
| AmbientNoise | Not Signalled |
| SoundInsulation | Not Signalled |

Table 1 - H.TPS-AV Proposed Interfaces

# Conclusion

Polycom proposes interfaces in Table 1 be used for the existing parameters.

Polycom also proposes that reference to section 9 of H.420 be removed from the parameter template at the start of section 7, and be replaced with an illustrative list (CLUE, SDP, H.241, H.245).

__________________