| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4671 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Cisco Systems, Inc. |  |  |  |
| Title: | Proposed revisions to H.245 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

During the last SG16 meeting, several changes were introduced to H.245 for DTLS support.  The new fields make use of text strings that, while workable, are not constructed in the same way as other H.245 constructs.  This contribution proposes changes to produce the same results, but using syntax that is more consistent with H.245’s other syntax.

# Previously Proposed Syntax

What was previously proposed for inclusion in H.245 is the following:

DTLSSecurityCapability ::= SEQUENCE

{

hashFunction			SEQUENCE OF IA5String ( SIZE (1..65535) ),

setupInformation		IA5String ( SIZE (1..65535) ) OPTIONAL,

connectionInformation	IA5String ( SIZE (1..65535) ) OPTIONAL,

fingerprint			IA5String ( SIZE (1..65535) ) OPTIONAL,

...

}

It is noted that there is no explanation of these fields currently in H.245, but we understand them to be parameters taken from RFC 5763.

# Alternative

The same fields could be represented in ASN.1 in such a way as to not introduce arbitrarily long string fields.  The following is proposed to replace the above, with some slightly modified element names from the current H.245 syntax:

SetupRole ::= CHOICE

{

active 			NULL,

passive 			NULL,

active-passive		NULL,

hold-connection	NULL, -- Does this even apply from RFC 4145?

...

}

ConnectionType ::= CHOICE

{

new	 			NULL,

existing 			NULL,

...

}

DTLSSecurityCapability ::= SEQUENCE

{

setup	      		SetupRole,

connection			ConnectionType OPTIONAL,

hash					OBJECT IDENTIFIER,

fingerprint			OCTET STRING ( SIZE (1..256) ) OPTIONAL,

...

}

Where the following object identifiers are defined for H.245 (to be added to Annex D):

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) sha1(0)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) sha224(1)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) sha256(2)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) sha384(3)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) sha512(4)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) md5(5)}

{itu-t(0) recommendation(0) h(8) 245 dtls-hash(2) md2(6)}

# Conclusion

The intent of these proposed changes it to keep the spirit of the additions, that being an encoding that works well for interworking, while at the same time having an encoding that is consistent with other H.323 / H.245 constructs.

__________________