| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4682 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.92 Release 2: additional support of "intra-Termination interlinkage" (seplink package version 2) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides a follow-up discussion on C.685 from the last meeting concerning an updated of H.248.92.

1	Discussion

The result of the last meeting was:

| C.685 | H.248.92 Release 2: additional support of "intra-Termination interlinkage" (seplink package version 2) | Alcatel-Lucent International |
| --- | --- | --- |

This Contribution was discussed. It was agreed not to start a H.248.92 Release 2 at this stage pending the identified of a real use case for the functionality.

Because C.685 itself did not provide a concrete use case, rather a general discussion only.

Concrete use case example:

There was a 3GPP CT4 meeting since our last Q.3/16 meeting, where we submitted use case material related to H.248 WebRTC gateways, see C4-150408:

URL: http://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_68bis_Bratislava/Docs/C4-150408.zip

That discussion paper provides one section about a comprehensive analysis concerning the usage of interlinkage in context of WebRTC gateway interworking functions, see:

Start of C4-150408 excerpt on stream interlinkage:

## 3.6 Use case #6: MG enhancement – additional usage of stream interlinkage

H.248.92 is applicable in principle whenever a protocol uses explicit bearer control procedures. That's basically the case for H.248 WebRTC gateway interworking functions, given by connection-oriented protocols DTLS, SCTP (and possibly DCEP) and TCP (if used as L4 protocol) at an "H.248 WebRTC termination", and TCP and TLS at the "H.248 non-WebRTC termination".

Table 6.1 provides an example list of potential enhancements using stream interlinkage with scope on termination T1 only:

Table 6.1 – Use case #6 – Examples for stream interlinkage within the H.248 WebRTC termination

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #6.0 | DTLS-to-SCTP interlinkage (establishment): - successful DTLS connection establishment triggers the SCTP association establishment procedure | - type "intra-SEP interlinkage" in case of a H.248 stream grouping model where both, the (DTLS)-connection-endpoint and (SCTP)-association-endpoint are part of the same H.248 SEP (e.g., see Figure 4.0, deaggregation SEP T1(S4)) - type "intra-Termination interlinkage" in case of a H.248 stream grouping model where the (DTLS)-connection-endpoint and (SCTP)-association-endpoint would be assigned to separate H.248 SEPs (e.g., see Figures 9.1 and 10.1) |
| #6.1 | SCTP-to-DTLS interlinkage (release): - successful SCTP association shutdown triggers the DTLS connection release procedure | - type "intra-SEP interlinkage" in case of a ….(see #6.0) - type "intra-Termination interlinkage" in case of a ….(see #6.0) |
| #6.2 | (SCTP-stream)-to-(SCTP)-association interlinkage (release): - precondition: only a single DC is still in use; - a SCTP Stream reset procedure (as WebRTC DC closure) triggers the shutdown of the SCTP association | - type "intra-Termination interlinkage" due to the H.248 stream grouping model where the (SCTP)-stream-endpoints and the (SCTP)-association-endpoint are assigned to separate H.248 SEPs (e.g., see Figure 4.1) |

Figure 6.1 depicts candidates of possible stream interlinkage usage within termination T1:

Figure 6.1: H.248 WebRTC gateway – H.248 Context model – 
Use case #6: Examples for stream interlinkage within the H.248 WebRTC termination

Table 6.2 provides an example list of potential enhancements using stream interlinkage between T1 and T2:

Table 6.2 – Use case #6 – Examples for stream interlinkage between terminations T1 & T2

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #6.4 | … (see Fig. 6.2) |  |
| to | … |  |
| #6.25 | … |  |

Figure 6.2 outlines candidates of possible stream interlinkage usage between termination T1 and T2:

Figure 6.2: H.248 WebRTC gateway – H.248 Context model – 
Use case #6: Examples for stream interlinkage between terminations T1 & T2

Some observations and conclusions:

- non-WebRTC interlinkage scenarios UCs #6.14 and #6.25 are already supported (as part of eMEDIASEC);

- interlinkage between DTLS and SCTP (UCs #6.0 and #6.1) depends on the handling of both protocol layers from H.248 perspective, i.e., as individual protocols or as an abstracted, monolithic object (sth like a H.248 "SCTP/DTLS" bearer connection endpoint);

- new interlinkage capability (called "intra-Termination interlinkage"), see e.g., UC #6.2, which requires an upversioned H.248 seplink package v2;

- the number of theoretical interlinkage cases is cut into halves when TLS is not used at termination T2;

- some interlinkage candidates are only applicable when the DTLS connection, SCTP associaten and WebRTC data channel would be established at the same point in time ("given by the nature of interlinkage which "immediately takes effect", and does not support delayed actions");

- thus, stream interlinkage is applicable as well for H.248 context configurations as required for WebRTC data applications, but the theoretical number of possible scenarios (as listed in Fig. 6.1 and 6.2) will end up in only a few cases.

End of C4-150408 excerpt on stream interlinkage:

2	Conclusions

3GPP C4-150408, clause 3.6 identifies three use cases related to intra-Termination interlinkage, - two of them are conditional, dependent on an extended stream grouping model, and one scenario which is already applicable for the "default WebRTC data channel stream grouping model".

We think that there is already sufficient justification (to extend H.248.92) from that WebRTC gateway example in particular.

Furthermore: a general justification is given by H.248.STGROUP, i.e., the new option of H.248 stream grouping models. There might be immediate cases of interlinkage within a single H.248 stream group in future. Intra-"stream group"-interlinkage is effectively the considered case of intra-Termination interlinkage.

3	Proposal

It is proposed to accept the proposal of previous C.685, see attachment..

|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 685 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2014 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 − CONTRIBUTION 685 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent |  |  |  |
| Title: |  | H.248.92 Release 2: additional support of "intra-Termination interlinkage" (seplink package version 2) |  |  |  |

Summary

An extension (called "intra-Termination interlinkage") for the H.248 seplink package is discussed. Additional support of "intra-Termination interlinkage" seems to be fairly straightforward and requires obviously just a small syntax change. There is neither an extension package or new package required, rather just a new package version seems to be sufficient. Thus, a work item for a next release of H.248.92 is requested.

# Discussion and conclusions

The new concept of H.248 stream grouping (as currently defined within work item H.248.STGROUP) allows essentially the partitioning of a protocol stack segment on multiple H.248 stream endpoints (SEP). Parallel contributions for H.248.STGROUP discuss such kind of stream grouping models, see C-677, C-678 and C-684.

Background:

The stream endpoint interlinkage package (seplink version 1), - as published by Recommendation ITU-T H.248.92 (10/2014) -, supports basically

- inter-SEP interlinkage ("the interlinked SEPs are located at different terminations") and

- intra-SEP interlinkage ("interlinkage within a single SEP at a specific termination").

What's missing is a 3rd conceptual interlinkage type:

- intra-Termination interlinkage ("interlinkage between different SEPs of a specific termination").

"Intra-Termination interlinkage" wasn't considered so far because a correspondent use case was not yet discussed and the work on H.248.STGROUP was just at the beginning.

There is a single termination in scope of interlinkage, both in case of intra-SEP (2) and intra-Termination (3) interlinkage. That's a common characteristic of the two interlinkage categories, but what's actually the difference? Both interlinkage capabilities differ in following aspects:

- intra-SEP interlinkage: 
 complete protocol stack (segment) in its entirety mapped on a single SEP,
 interlinkage between different protocol layers of that stack results consequently in "intra SEP interlinkage" scenarios;

- intra-Termination interlinkage: 
 complete protocol stack (segment) mapped on multiple SEPs ("which would be semantically grouped in case of H.248.STGROUP stream grouping"),
 interlinkage between different protocol layers of that stack could result in "inter SEP interlinkage limited to a single-termination" (abbreviated here as "intra-Termination" interlinkage) scenarios.

It may be observed that inter-Termination interlinkage isn't yet supported and would require syntactical (see below) and semantical changes.

The purpose of this contribution is to identify potential changes in H.248.92 (10/2014).

The start of a revision of H.248.92 is requested in case of support of such extensions.

# Technical discussion:

Parameter "interlinkedSEP" in property "linktopo" needs to be extended by adding a stream-level to the termination identifier. A simple and straightforward syntactical extension could be the following change:

linktopo = interlinkedSEP COLON

sourceTransportEP COLON

interlinkedTransportEP COLON

mode

interlinkedSEP         = TerminationID ["." StreamID]

sourceTransportEP      = proto

interlinkedTransportEP = proto

mode = modetypes *(COMMA modtypes)

modetypes = "*" / "est" / "rel"  ; * is all mode types

; est is Establishment

; rel is Release

Some examples:

Example 1 - intra-SEP interlinkage (as starting point) with seplink v1:

Table 1 – Example 1 "legacy" intra-SEP interlinkage

| H.248/SDP encoding | Comments: |
| --- | --- |
| Modify = ip/1/1/1 { Media { Stream = 1 { LocalControl { ... seplink/linktopo = ["ip/1/1/1:TCP:TLS:est"] }, ... } }} | - Identifier source SEP: TermID/StreamID = "ip/1/1/1"/"1" - Identifier interlinked SEP: TermID/StreamID = "ip/1/1/1"/"1" Identical, hence intra-SEP interlinkage. |

Example 2 - intra-termination but inter-SEP interlinkage:

Table 2 – Example 2 "intra-termination but inter-SEP interlinkage"

| H.248/SDP encoding | Comments: |
| --- | --- |
| Modify = ip/1/1/1 { Media { Stream = 1 { LocalControl { ... seplink/linktopo = ["ip/1/1/1.2:TCP:TLS:est"] }, ... }, ; -- end of stream 1 Stream = 2 { ... } }}} | - Identifier source SEP: TermID/StreamID = "ip/1/1/1"/"1" - Identifier interlinked SEP: TermID/StreamID = "ip/1/1/1"/"2" Different StreamIDs, but same termination, hence intra-termination interlinkage. |

Example 3 - inter-termination and inter-stream interlinkage:

Table 3 – Example 3 "inter-termination and inter-stream interlinkage"

| H.248/SDP encoding | Comments: |
| --- | --- |
| Modify = ip/1/1/1 { Media { Stream = 1 { LocalControl { ... seplink/linktopo = ["ip/1/1/2.2:TCP:TLS:est"] }, ... } }} | - Identifier source SEP: TermID/StreamID = "ip/1/1/1"/"1" - Identifier interlinked SEP: TermID/StreamID = "ip/1/1/2"/"2" Different StreamIDs and different terminations termination, hence inter-termination and inter-stream interlinkage. |

Conclusion: the proposed and evaluated syntax extension seems to satisfy the new requirement.

# Proposals

It is proposed to accept the marked up changes below.

#1:

[Begin Proposal]

Draft revised Recommendation ITU-T H.248.92

Gateway Control Protocol:
Stream endpoint interlinkage package

Summary

The interlinkage of stream endpoints (SEP) relates to the empowerment of the MG by the media gateway controller (MGC) to perform the interconnection of two protocol endpoints. The interlinkage relates to abstracted bearer control protocol procedures as mainly part of bearer establishment and release. The pair of protocol endpoints might be located in a single SEP or in different SEPs, leading to so-called intra-SEP and inter-SEP interlinkage configurations. This Recommendation defines an H.248 package which allows the MGC to signal such interlinkage configurations to the MG. The discussion of use cases, models and examples complements the description of the H.248 protocol extensions.

Revision 1 provides additional support of so-called "intra-termination interlinkage".

# 1	Scope

The H.248 media gateway bearer interface protocol stack may contain principally connectionless and connection-oriented protocols. The concerned protocol layer(s) from H.248 perspective, i.e., which are in scope of the H.248 Stream Descriptors, are typically a network layer (L3) and higher. Connection-oriented protocols are known from L2 onwards in general, and in case of IP from L4 and higher concerning with respect to basic reference models for layered protocol architectures.

Connection-oriented protocols are characterized by explicit bearer connection control procedures, related to the establishment, modification and/or release of a bearer connection. Normally, the MG as slave is triggered by the master MGC for a particular bearer connection control procedure, and then autonomously executing such a procedure at the bearer interface related to a particular H.248 termination (T) / stream endpoint (SEP).

The end-to-end bearer plane connectivity typically implies scenarios where multiple SEPs are interconnected in an H.248 context. SEP-individual bearer control procedures are then often interlinked; e.g. an incoming bearer connection establishment procedure (at one SEP) could lead to an outgoing bearer connection establishment procedure (at a partner SEP), - at the same or at a different protocol layer.

An MGC may wish to individually control each of these procedures (so called MGC-strictly controlled mode). However there are many use cases where the MGC could benefit from a delegation of "interlinkage work" down to the MG itself.

Such kind of behaviour is the purpose of this Recommendation.

An H.248 package, called the "stream endpoint interlinkage" package (text codepoint 'seplink'), is defined in a generic manner, which represents a flexible tool to enable a wide range of interlinkage options.

The supported interlinkage

- is limited to a single context,

- is direction dependent,

and allows the interlinkage

- of protocol endpoints, at the same or different layers, in different SEPs,

  - located at different terminations ("inter-SEP interlinkage"), or

  - located at the same termination ("intra-termination interlinkage") and

- of protocol endpoints at different layers within an individual SEP ("intra-SEP interlinkage"),

and

- allows multiple interlinkage configurations to be assigned to an individual SEP.

## 1.1	Applicability statements

The capability of "interlinkage" relates to the MG internal forwarding of protocol control information (PCI) between different SEPs and/or different protocol layers. A layered protocol architecture is thus associated with a SEP.

The interlinkage capability is limited to protocol stack segments, which may be unambiguously described via the mechanism in clause 7.1.1.

The defined interlinkage capability is generic, i.e., applicable for all kind of connection-oriented bearer protocols. However, the focus of the initial package version is tightly coupled with protocols related to IP.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.89]	Recommendation ITU-T H.248.89 (07/2014), Gateway control protocol: TCP support packages.

[ITU-T X.200]	Recommendation ITU-T X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	stream endpoint (SEP) [clause 3.2.10/ITU-T H.248.1]: relates to a stream associated to a Termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.2	stream endpoint pair (SEPP) [clause 3.2.11/ITU-T H.248.1]: two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there is no further associated SEP on a third Termination etc).

3.2.3	stream endpoint tuple (SEPT) [b-ITU-T H.Sup.13]: the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	interlinkage: the MGC empowered interconnection of two protocol endpoints. The two protocol endpoints are associated to a single stream endpoint (SEP) or different SEPs within the same context. The MG is enabled for autonomously forwarding of indicated stimuli from the source protocol endpoint to the destination endpoint. The stimuli relates to abstracted control information (and not any real Lx protocol control information).

3.2.2	horizontal interlinkage: synonym term for type "inter-SEP interlinkage".

3.2.3	vertical interlinkage: synonym term for types "intra-SEP interlinkage" and "intra-termination interlinkage".

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| EP | End Point |
| --- | --- |
| IP | Internet Protocol |
| L2, L3, L4 | (Protocol) Layer 2 / 3 / 4 |
| Lx | (Protocol) Layer x |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PCI | Protocol Control Information |
| S | Stream |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream End Point |
| SEPP | Stream End Point Pair |
| SEPT | Stream End Point Tuple |
| T | Termination |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

None.

# 6	Motivation, use cases and models

H.248 controlled Media Gateways may terminate multiple connection-oriented transport layer protocols, like TCP, TLS and SCTP.

This is outlined in the following Figure 1 where in a given H.248 context each Stream Endpoint (SEP) hosts at least one or more connection-oriented transport layer endpoints.

Figure 1 – Two-termination context with multiple connection-oriented protocols 
in the SEP-specific protocol stack (segments)

The interlinkage concept is constituted by the following characteristics:

- the interlinkage scope is limited to a single context;

- an interlinkage configuration is defined via an H.248 Property, which itself is linked to a particular stream endpoint/termination;

- a specific external, incoming bearer control procedure (e.g., establishment event) immediately triggers the execution of an external, outgoing bearer control procedure, at the same or another termination in the context;

- a single interlinkage configuration acts in an unidirectional manner; and

- multiple interlinkage configurations in parallel may be defined for a single SEP.

Thus, four principal interlinkage types may be differentiated (see also illustration in Appendix I.2):

- inter-SEP interlinkage (when source and interlinked transport endpoints are located at different terminations);

- intra-SEP interlinkage (when source and interlinked transport endpoints are located at the same termination at a single SEP);

- intra-termination interlinkage (when source and interlinked transport endpoints are located at the same termination at different SEPs);and

- multi-SEP interlinkage (when a single stimuli is used to trigger multiple outgoing bearer procedures).

The following Figure 2 outlines the various interlinkage options enabling an MG autonomous establishment/release of a connection/session-oriented partner transport protocol endpoint (again using the example of a stream endpoint pair (SEPP)):

Figure 2 – Principle interlinkage options in a two-termination context 
from the perspective of a single SEP (here: SEP-A)

Example interlinkage configurations as illustrated by the arrows in Figure 2 are:

- Intra-SEP interlinkage from lower to higher protocol layer: within one SEP a lower layer connection oriented transport protocol endpoint (EP) triggers an upper layer connection oriented transport protocol EP (NOTE 1).

- Intra-SEP interlinkage from higher to lower protocol layer: within one SEP an upper layer connection oriented transport protocol EP triggers a lower layer connection oriented transport protocol EP (NOTE 1).

- Inter-SEP interlinkage: Given a SEP pair SEP-A triggers the connection oriented transport protocol of the peer SEP-B. Either the same or a different transport protocol can be interlinked, thus for example a SEP-A TCP endpoint could trigger a SEP-B TCP endpoint, or a SEP-A TCP endpoint could trigger e.g. a SEP-B SCTP endpoint.

NOTE 1: This does not mean that a lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between them.The first three interlinkage configurations are part of SEP-A. When the 3rd interlinkage configuration (3) should be complemented by interlinkage in reverse direction, then following, fourth interlinkage configuration needs to be associated with SEP-B:

- Inter-SEP interlinkage: Given a SEP pair, SEP-A receives a trigger regarding a connection oriented transport protocol from its peer SEP-B. Either the same or different transport protocol can be interlinked, thus for example SEP-B TCP endpoint could trigger a SEP-A TCP endpoint, or a SEP-B TCP endpoint could trigger e.g. a SEP-A SCTP endpoint.

The principal limitations are as follows:

- Inter-SEP interlinkage (also known as "horizontal protocol layer interlinkage") is only permitted between SEPs belonging to the same H.248 stream.

- Intra-SEP interlinkage (also known as "vertical protocol layer interlinkage") is only permitted within a SEP, i.e. between its supported transport protocol layers.

- In inter-SEP horizontal protocol layer interlinkage only external, incoming procedures are permitted as stimuli.

- The triggered procedure is always an outgoing procedure.

- Establishment procedures within one SEP shall follow an upwards order, i.e. the initiation of an establishment procedure shall only start when the establishment procedure at the lower layer is completed.

- Release procedures within one SEP should generally follow a downwards order, i.e. the initiation of a release procedure should only start when the release procedure at the upper later is completed. However, depending on the protocol rules, it may be possible to omit an explicit release procedure at the upper layer altogether.

{Editor's note (02/2015 meeting): any limitations of intra-termination interlinkage?}

NOTE – The last two limitations are due to basic communication principles in layered protocol architectures (see [ITU-T X.200]: an (N+1)-layer is based on the service of the (N)-layer connection).

A H.248 context may contain more than two terminations where theseterminations host SEPs belonging to the same H.248 stream. In this case the SEP interlinkage capabilitiy needs to be able to address a dedicated stream endpoint tuple (SEPT).

For example, the following interlinkage scheme should be supported in case of an H.248 context containing three SEPs (same H.248 streamID).

Figure 3 – Interlinkage example for a three-termination context

Whereas SEPP (SEP-A, SEP-B) can trigger each other regarding the identified transport protocol endpoints, the SEP-A connection oriented transport protocol EP may trigger SEP-C connection oriented transport protocol EP in addition. However SEPP (SEP-B, SEP-C) does not have any interlinkage. SEP-C connection oriented transport protocol EP cannot trigger SEP-A connection oriented transport protocol EP in any way.

# 7	Stream endpoint interlinkage package

|  | Package Name: | Stream endpoint interlinkage package |
| --- | --- | --- |
|  | Package ID: | seplink (0x011b) |
|  | Description: | This package provides the functionality to interlink connection/session oriented transport protocol endpoints. If two transport protocol endpoints are interlinked the establishment and/or release of a connection/session at the source transport protocol endpoint will trigger an MG-autonomous establishment and/or release of the interlinked transport protocol endpoint. Package version 2 provides additional support of so-called "intra-termination interlinkage". |
|  | Version: | 2 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Interlinkage topology

|  | Property Name: | Interlinkage topology |
| --- | --- | --- |
|  | Property ID: | linktopo (0x0001) |
|  | Description: | This property defines the MG interlinkage behaviour regarding connection/session oriented transport endpoints. Interlinkage can be specified between the connection-oriented transport layers of a given SEP or between any connection-oriented transport layers of a given SEP pair. Interlinkage defines a undirectional information flow ("stimuli") from a "SEP-local transport EP" towards another "SEP-local or remote transport EP". |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string instance of the sub-list is coded according to the following ABNF syntax: linktopo = interlinkedSEP COLON sourceTransportEP COLON interlinkedTransportEP COLON mode interlinkedSEP = TerminationID ["." StreamID] sourceTransportEP = proto interlinkedTransportEP = proto mode = modetypes *(COMMA modtypes) modetypes = "*" / "est" / "rel" ; * is all mode types ; est is Establishment ; rel is Release TerminationID, COLON, COMMA are as per Annex B.2/[ITU-T H.248.1]. For binary usage of H.248.1 the binary TerminationID octets should be encoded using a hexadecimal octet coding (as per Annex B.3/[ITU-T H.248.1]. The wildcarding field shall be omitted. The only wildcarding allowed is the use of ALL or CHOOSE which shall be indicated using "*" and "$" respectively. Value proto is as per clause 9/[IETF RFC 4566], with the restriction that a single transport protocol value should be selected when the IANA proto codepoint represents a protocol stack segment (format e.g. "x/y/z"), rather than a single individual protocol layer (format "x"). Example (looking at the example of a hierarchical IANA proto codepoint): the MGC may select value "TLS" in case of an application-agnostic IANA proto codepoint "TCP/TLS" or application-aware IANA proto codepoint "TCP/TLS/MSRP". Where: interlinkedSEP: Is the TerminationID and optional StreamID of the interlinked SEP where outgoing transport protocol signalling is triggered. The source inter-linked SEP is the Termination where the linktopo property is set. This may be set to the same value as the source SEP TerminationID in which case it refers to Figure 2 Options 1 and 2. sourceTransportEP: Is the transport protocol endpoint which is the source of the signalling. interlinkedTransportEP: Is the transport protocol endpoint which sends the outgoing signalling. In Figure 2 options 1 and 2 the sourceTransportEP and interlinkedTransportEP shall be set to different transport protocols. mode: Specifies whether the interlinkage is related to connection/session establishment and/or release procedures. NOTE: Detailed agreements regarding the admitted <proto> values may be defined in an appropriate H.248 profile specification. Typically this is specified in the profile clauses on mandatory/optional support of SDP. |
|  | Default: | Empty list |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 7.2	Events

None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

### 7.5.1	Incorrect intra-SEP interlinkage

|  | Error Code #: | 488 |  |
| --- | --- | --- | --- |
|  | Name: | Incorrect stream endpoint interlinkage |  |
|  | Definition: | The protocol value indicated in the "source transport endpoint" (sourceTransportEP) or "interlinked transport endpoint" (interlinkedTransportEP) parameters in the Interlinkage topology property relates to a protocol which is unsuitable for interlinkage. E.g., all protocols with the characteristic of being "connectionless" (NOTE). Background information: There are the two "mode of communication" concepts (see [ITU-T X.200]) of "connection-oriented" and "connectionless". Connectionless protocols (such as UDP) inherently lack bearer connection control procedures, thus, a connectionless protocol (layer) acting as a) a source transport endpoint: would never generate any stimuli. Hence interlinkage would never be triggered; and b) an interlinked transport endpoint: would silently discard received stimuli because there are no procedures which could be triggered. NOTE – There are many protocols (beyond the basic reference model according to [ITU-T X.200]) with an unclear "connectionless / connection-oriented" classification, such as UDPTL or RTP (the associated "bearer control protocol" RTCP could be e.g., used for "bearer release"). |  |
|  | Error text in the Error Descriptor: |  | - |
|  | Comment: | None. |  |

## 7.6	Procedures

### 7.6.1	Configuration of "interlinkage topologies"

#### 7.6.1.1	SEP identification:

a) Source of stimuli:

The source SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which triggers autonomous procedures at another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID and StreamID are derived from the termination/stream the LocalControl descriptor that contains the seplink/linktopo property value. The name of the source SEP is thus implicitly given.

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "sourceTransportEP").

b) Sink of stimuli:

The destination SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which receives a trigger to start autonomous procedures from another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID is explicitly specified in the seplink/linktopo property value (parameter "interlinkedSEP").

- The StreamID is either

  - derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value; or

  - explicitly specified in parameter "interlinkedSEP".

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "interlinkedTransportEP").

#### 7.6.1.2	Traffic directions:

The seplink/linktopo property defines a unidirectional behavior, that is it specifies the interlinkage behavior of a source (identified by the TerminationID and StreamID the LocalControl descriptor is related to) and an interlinked SEP (identified by the TerminationID; same StreamID as source).

In case a bidirectional interlinkage between a SEPP/SEPT is expected, each SEP's Local Descriptor must contain a corresponding interlinkage topology.

#### 7.6.1.3	Wildcard usage:

The interlinked SEP can be specified using the ALL wildcard

"seplink/linktopo = ["*:TCP:TCP:est,rel"]".

In this case the specified interlinkage behaviour is valid to all matching SEPPs (i.e. only horizontal interlinkage, vertical interlinkage is specifically excluded, that is an ALL wildcard excludes its own TerminationID). In case a new SEP is added into the H.248 context later on, the ALL wildcarded interlinkage topology is applied as well to the new SEP.

The interlinked SEP can be specified using the CHOOSE wildcard

"seplink/linktopo = ["$:TCP:TCP:est,rel"]".

The CHOOSE wildcard matches the TerminationID that the MG assigns in the first Add Command that uses a CHOOSE wildcard in the same action. This allows to add a new termination/SEP, and to modify the existing interlinkage topologies (using a modify command) within the same action.

Transaction = 1 {

Context = 1 {

Add = ip/1/$/$ {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["ip/1/1/1:TCP:TCP,est,rel"] }}}}

Modify = ip/1/1/1 {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["$:TCP:TCP:est,rel"] }}}}

NOTE - In the above examples mode setting "est,rel" could be replaced by "*".

#### 7.6.1.4	Dynamic aspects

The exact point in time when an interlinkage stimulus should be generated by the source transport endpoint is not explicitly specified by this Recommendation, rather subject of implementations. This might be dependent on the real bearer connection protocol used, the overall protocol stack behind the SEP, the considered state modeling of a bearer connection endpoint, different handling between inter- and intra-SEP interlinkage, etc.

Appendix II provides some general guidelines, state models and examples by consideration of generic protocol layers.

### 7.6.2	Relation to H.248 Stream Mode:

A specified interlinkage topology is independent of the LocalControl Descriptor StreamMode property. The StreamMode property affects the application data flow rather than the transport protocol control information used to establish or close the corresponding transport connection.

### 7.6.3	Relation to H.248 Topology descriptor

The effectiveness or non-effectiveness of a specified horizontal interlinkage topology depends on the context level topology descriptor, i.e. whether the corresponding flow direction in the topology descriptor for a particular SEPP (or in case of wildcard ALL usage: for all matching SEPPs) has been enabled or not.

### 7.6.4	Error cases:

In case if a new interlinkage topology refers to an unknown termination identifier, the MG shall reject the command using Error #430 "Unknown TerminationID".

In case if a new interlinkage topology refers to a valid termination identifier but this termination doesn't contain an SEP for this specific stream identifier, then the MG shall reject this using Error #473 "Conflicting Property Values".

In case if a new interlinkage topology refers to a transport protocol not part of the SEP's "m="-line transport protocol, then the MG shall reject this using Error #472 "Required Information Missing".

In case a new interlinkage topology is specified refering to a transport protocol which either

- does not generate an interlinkage stimuli (at source side); or

- does not trigger a bearer control procedure (at sink side)

then the MG shall reject this using Error #488 "Incorrect stream endpoint interlinkage". Most likely the cause of lack of stimuli would be the specification of a connection-less protocol in the interlinkage configuration by the MGC.

### 7.6.5	Examples:

#### 7.6.5.1	Example of a "TLS/TCP to SCTP" connection model

i) Example of intra-SEP interlinkage:

The example has an SEP-A (T-A) that uses transport layers TLS/TCP. A TCP connection setup autonomously triggers a TLS session setup and a TLS session release autonomously triggers a release of the TCP connection.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:TCP:TLS:est","T-A:TLS:TCP:rel"]
} } } }

The resulting interlinkage is as follows (Figure 4):

Figure 4 – Example of a "TLS/TCP to SCTP" connection model 
– Vertical interlinkage configuration at SEP-A

ii) Example of inter-SEP interlinkage:

In addition SEP-A's transport layer TCP can be interlinked with SEP-B's transport layer SCTP (i.e. SEP-A's TCP connection establishment/release autonomously triggers SEP-B's SCTP connection establishment/release).

Note: SEP-A's TCP connection establishment/release can be either stimulated by using the corresponding H.248 TCP package capabilities ( e.g., [ITU-T H.248.89] properties tcpbcc/EstBNC (outgoing establishment procedure (TCP client role)) or tcpbcc/RelBNC) or by receiving a TCP connection establishment/release request from the far-end TCP endpoint.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:TCP:TLS:est", "T-A:TLS:TCP:rel",
                            "T-B:TCP:SCTP:est,rel"]
} } } }

Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP"

iii) Example of bidirectional inter-SEP interlinkage:

To make sure that SCTP connection establishment at SEP-B will also trigger SEP-A TCP connection establishment and that SCTP connection release at SEP-B will also trigger SEP-A TLS session release, the seplink/linktopo property has to be applied as follows in SEP-B's LocalControl Descriptor:

H.248 command request:

Modify = T-B {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:SCTP:TCP:est", "T-A:SCTP:TLS:rel"]
} } } }

Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS"

v) Example of intra-termination interlinkage:

{to be added}

Appendix I

Illustration of interlinkage capabilities

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates the interlinkage concept, the principal interlinkage types and indicates some use cases which are beyond current interlinkage capabilities.

## I.1	Illustration of concept

Figure I.1 summarizes the terminology used and the principal elements of the seplink/linktopo property.

Figure I.1 – Stream endpoint assigned property linktopo and their parameters

The linktopo property is always associated with a specific stream endpoint (here SEP "Ta/Si").

## I.2	Illustration of supported use cases

Three principal interlinkage types may be differentiated.

### I.2.1	Use case "Inter-SEP interlinkage"

The interlinkage across two terminations is called "inter-SEP interlinkage" (Figure I.2).

Figure I.2 – Use case "Inter-SEP interlinkage"

Table I.1 summarizes the principal interlinkage description:

Table I.1 – Interlinkage description – Use case "Inter-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" | The mode codepoint could est or rel. |

### I.2.2	Use case "Intra-SEP interlinkage"

There are two variants of intra-SEP interlinkage:

#### I.2.2.1	Externally driven "Intra-SEP interlinkage"

The interlinked transport endpoint may be located at the same SEP as the source transport endpoint, called "intra-SEP interlinkage" (Figure I.3).

Figure I.3 – Use case: Externally driven "Intra-SEP interlinkage"

Table I.2 summarizes the principal interlinkage description:

Table I.2 – Interlinkage description – Use case "Intra-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "EPn" (at Ta/Si) | "EPnn+1" (at Ta/Si) | "est" | Not possible for release. |

#### I.2.2.2	Internally driven "Intra-SEP interlinkage"

The following is the difference between the previous externally driven intra-SEP interlinkage case and the internally-driven one:

An MG-internal stimuli (see Figure I.4), - either originating in the MGC (0a) (i.e., related to H.248 signalling) or in the MG itself due to a previous interlinkage scenario (0b) -, triggers an external outgoing procedure (1). The internal stimuli (2) will be generated, as usual, when the source transport endpoint successfully transitions its state.

Figure I.4 – Use case: Internally driven "Intra-SEP interlinkage"

The interlinkage description is identical to the externally driven "Intra-SEP interlinkage" (i.e., according to Table I.2). .

### I.2.3	Use case "Multi-SEP interlinkages"

A single stimulus may be used to trigger multiple outgoing bearer procedures (of the same or different type). The interlinked transport endpoints may be located at the same or different SEP(s). Figure I.5 illustrates such an example.

Figure I.5 – Use case "Multi -SEP interlinkage"

The linktopo property "value" represents a list structure (Table I.3):

Table I.3 – Interlinkage description – Use case "Multi-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" | The mode codepoint could be est or rel. |
| "Tc" | "EPn+1" (at Ta/Si) | "EPn" (at Tc/Si) | "…" | ditto |

## I.3	Controversial use cases

### I.3.1	Use cases with "self-referential interlinkage"

The case of intra-SEP interlinkage with identical source transport and interlinked transport endpoints is called self-referential interlinkage.

Such an interlinkage seems to be controversial in general concerning the question which incoming bearer control procedures trigger immediate outgoing control procedures? What would be correspondent mode values?

There are no useful self-referential interlinkage scenarios identified for seplink package version 1.

### I.3.2	Use cases with "tunneled (IP) protocol stacks"

The usage of tunneling is e.g. a characteristic of many IP access network technologies.The notion of tunnel refers to hierarchical protocol stacks of type e.g. "X-over-IP-over-X-over-IP",

Such an interlinkage seems to be controversial in general concerning the unambiguous identification of source transport and/or interlinked transport endpoints due to the existing supported value range by seplink package version 1. E.g., the indication of "TCP" (in case of a TCP-over-TCP stack) would be ambiguous. Such scenarios are for further studies.

NOTE – Extended interlinkage according to seplink package version 2 supports this use case under the condition that "tunneled (IP) protocol stacks" would be mapped on multiple H.248 streams (e.g., using "stream grouping semantics" based on [b-ITU-T H.248.STGROUP] with separate SEPs for source transport and interlinked transport endpoints.

### I.3.3	Use cases with "extended (IP) protocol stacks"

Extended protocol stacks are also beyond the layered protocol architectures according to basic reference models, but the interlinkage is not necessarily between identical protocol types (as in above "tunneled scenarios").

The support of such use cases is already in scope of seplink package version 1, but conditional. The usage depends on the unambiguous identification of source transport and/or interlinked transport endpoints.

NOTE – Extended interlinkage according to seplink package version 2 allows unambiguous support of this use case under the same conditions as described in clause I.3.2.

Appendix II

Considerations about state modelling and dynamics

(This appendix does not form an integral part of this Recommendation)

## II.1	Purpose

This Appendix provides some background information concerning the MG-internal generation and forwarding of the stimuli for interlinkage. It serves to clarify principle differences between inter- and intra-SEP interlinkage and the timing of vertical H.248 signalling (for interlinkage configurations) versus horizontal bearer control procedures. Where there are discrepancies between this Appendix and this Recommendation, the semantics and procedures of the main body of this Recommendation take precedence over those described in this Appendix.

NOTE – To seplink package version 2: the case of "intra-termination interlinkage" is applicable whenever "intra-SEP interlinkage" specific information is provided in this Appendix.

## II.2	State modelling

### II.2.1	Used model for a single bearer connection endpoint

The seplink package version 1 supports two interlinkage modes, related to bearer connection establishment and release (see clause 7.1.1, property parameter mode). Such bearer connection endpoint behaviour could be modelled by (at least) two states (such as Idle and Established (synonym to Data transfer ready)). However, in order to address transient behaviour and unsuccessfull cases following (see Figure II.1) a four-state model is considered (with emphasis on the establishment procedure only):

Figure II.1 – State modelling – Bearer connection endpoint

NOTE 1 – The generic model is derived from [ITU-T X.200], e.g., consistent with layer 3 and 4 bearer connection endpoint modelling (see [b-ITU-T X.213], [b-ITU-T X.214]). The transient states S2 and S3 are dashed because they are not in scope of this Recommendation.

The protocol neutral primitives (request, confirm, indication, response) are according to [b-ITU-T X.210]. They allow the modelling of all kinds of procotol-specific bearer control procedures.

NOTE 2 – Example "TCP": the three-way handshake procedures for establishment: the first TCP SYN represents the CONNECT request / CONNECT indication information (for outgoing / incoming), and the last TCP ACK the the CONNECT confirm / CONNECT response information (for outgoing / incoming),

### II.2.2	Used model for two interlinked bearer connection endpoints

The two state machines of both bearer connection endpoints are coupled (see Figure II.2). Some state transitions of the interlinked endpoint are triggered (by interlinkage stimuli) by the source endpoint.

Figure II.2 – State modelling – Interlinked bearer connection endpoints

## II.3	Interlinkage behaviour

### II.3.1	Principle difference between intra- and inter-SEP interlinkage

There is a principle limitation in the intra-SEP intelinkage due to fundamental principles of layered protocol architectures (see clause 6). E.g., an (N+1)-layer service depends on the existence of an underlying (N)-layer service, thus, an (N+1)-layer establishment procedure may only started when the (N)-layer is already in state "Data transfer ready" (or Established, etc).

This leads to the following difference:

- intra-SEP interlinkage ("vertical interlinkage"): only serial execution of the two bearer control procedures at the SEP; and

- inter-SEP interlinkage ("horizontal interlinkage"): also parallel execution of the bearer control procedures at both SEPs possible.

This aspect impacts the point in time (or the position in the state machine) when the stimulus could be sent as the earliest possible from the source to the interlinked transport endpoint in case of lower to upper layer interlinkage.

### II.3.2	Timing of H.248 signalling and bearer control procedures

The MGC should be aware of the interlinkage behaviour before the bearer control procedure at the source transport endpoint begins. However, there might be cases of delayed H.248 signalling, i.e., the MG receives an H.248 command request with an interlinkage configuration when the bearer control procedure at the source transport endpoint is already in process or already completed. An interlinkage request received at that time shall trigger a bearer control procedure at the interlinked transport endpoint based on the bearer protocol state and/or transition. One bearer control state shall not trigger multiple bearer control procedures.

This aspect impacts the point in time (or the position in the state machine) when the stimulus is still be generated and sent from the source to the interlinked transport endpoint.

### II.3.3	State-driven interlinkage behaviour

A stimulus is principally generated at the source transport endpoint either due to a particular state transitioning (dynamic) or/and due to state value (static).

#### II.3.3.1	Intra-SEP interlinkage

##### II.3.3.1.1	Bearer connection establishment

Intra-SEP interlinkage for establishment implies protocol layer relation with an upper (N+…)-layer interlinked transport endpoint versus a (N)-layer source transport endpoint. The outgoing bearer connection control procedure at the interlinked transport endpoint is then either triggered by a "CONNECT response" (1) or a "CONNECT confirm" (2) state transition, or when the source transport endpoint is already in state "Data transfer ready" (3) (see Figure II.3).

Figure II.3 – Interlinkage behaviour – Stimuli for intra-SEP interlinkage 
– Bearer connection establishment

It should be noted that there is only a single stimulus (i.e., either dashed arrow 1, 2 or 3 in Figure II.3).

##### II.3.3.1.2	Bearer connection release

For further studies. It may be noted that a more detailed model (as outlined in Figure II.3) might be necessary. Particularly for protocols with additional transient states towards state "Idle".

#### II.3.3.2	Inter-SEP interlinkage

##### II.3.3.2.1	Bearer connection establishment

Inter-SEP interlinkage is unconditional with regards to protocol layer dependencies. Thus, the outgoing bearer connection control procedure at the interlinked transport endpoint could be as earliest as possible already started by a "CONNECT indication" (4) besides the intra-SEP interlinkage stimuli according to clause II.3.3.1.1 ("CONNECT response" (1) or "CONNECT confirm" (2) state transition, or state "Data transfer ready" (3)), see Figure II.4.

Figure II.4 – Interlinkage behaviour – Stimuli for intra-SEP interlinkage 
– Bearer connection establishment

It should be noted that there is only a single stimulus (i.e., either dashed arrow 1, 2, 3 or 4 in Figure II.4).

##### II.3.3.2.2	Bearer connection release

For further studies. It may be noted that a more detailed model (as outlined in Figure II.4) might be necessary. Particularly for protocols with additional transient states towards state "Idle".

## II.4	Illustration of temporary interlinkage behaviour

### II.4.1	Note to example interlinkage scenario

The timing of some example scenarios is illustrated. The exact point in time of the stimuli in the timing diagrams below is only roughly indicated in order not to overload the figures with too many details.

### II.4.2	Example #1: single establishment-release cycle, interlinked bearer connection follows

The interlinkage configuration is signalled in "Idle" with modes "est" and "rel". The state of the interlinked bearer connection follows the source transport endpoint (Figure II.5).

### II.4.3	Example #2: alternate establishment-release cycles, interlinked bearer connection follows

There are multiple, alternate state changes of the source transport endpoint in contrast to previous examples. The state of the interlinked bearer connection follows again the source transport endpoint (Figure II.6) because e.g. any subsequent repeated Idle-to-Established transition generates a further stimulus.

### II.4.4	Example #3: only establishment interlinked, no effect of subsequent stimuli

Interlinkage is limited to establishment only in this example. Possible subsequent stimuli do not affect the state of the interlinked transport endpoint (Figure II.7).

### II.4.5	Example #4: delayed interlinkage signalling

The interlinkage configuration is signalled behind the schedule, when the source transport endpoint completed already his bearer control procedures (in this example). A stimulus is still generated (Figure II.8).

Figure II.5 – Timing example #1 (single establishment-release cycle, interlinked bearer connection follows)

Figure II.6 – Timing example #2 (alternate establishment-release cycles, interlinked bearer connection follows)

Figure II.7 – Timing example #3 (only establishment interlinked, no effect of subsequent stimuli)

Figure II.8 – Timing example #4 (delayed interlinkage signalling)

Bibliography

[b-ITU-T H.Sup.13]	Supplement ITU-T H.Sup.13 (07/2014), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 1.

[b-ITU-T X.210]	Recommendation ITU-T X.210 (11/1993), Information technology – Open Systems Interconnection – Basic Reference Model: Conventions for the definition of OSI services.

[b-ITU-T X.213]	Recommendation ITU-T X.213 (10/2001), Information technology – Open Systems Interconnection – Network service definition.

[b-ITU-T X.214]	Recommendation ITU-T X.214 (11/1995), Information technology – Open Systems Interconnection – Transport service definition.

____________________