| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4687 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: clause L.1 – discussion and update proposal for "use case #1: alternate speech-text telephony" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal in order to progress use case #1 on alternate speech-text telephony.

1	Discussion and conclusions

There was a first technical discussion on this use case:

Last Q.3 report:

| C.676 (Rev.1) | H.248.STGROUP: model discussion (1) for use case "alternate speech-text telephony" | Alcatel-Lucent International |
| --- | --- | --- |

Comments:

- Has 3GPP defined stage 3 for this use case? Needs to be followed up.
[Reply]: concerned 3GPP stage 3 would be the 3GPP H.248 Mn profile, i.e., 3GPP 29.332. Only clause 10.2.3.5/29.332 provides explicit stage 3 information (see below). Unfortunately, the information is focusing on the SDP part only as well as principal functional behaviour, but lacking concrete H.248 call-dependent procedural information with comprehensive syntax.

- Could ReserveGroup/ReserveValue be used instead? 
[Reply]: This is already addressed in clause L.1.1.1: usage of ReserveGroup is conditionally feasible, at least for basic service support. However, features like component-level StreamMode settings or statistics require the usage of H.248 stream grouping.

- Question on why the audio stream is changed to a deaggregation stream? 
[Reply]: This is already addressed in clause L.1.1.5: in order to avoid any interactions with the stream configuration at PES side.

The use case as such is still valid, also due to the not yet discussed aspects of RTP topology impacts and the option of additional bundling (which could lead to two group semantics for a single stream group, hm …, see clause L.1.2).

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

Change proposal #1:

[Begin Proposed Correction]

Appendix L
Living List – H.248.STGROUP Possible use cases

(This appendix only contains working material.)

## L.1	Use case #1: alternate speech-text telephony

### L.1.1	Use case #1.1: alternate speech-text telephony in IP-to-IP gateways (without RTP bundling)

#### L.1.1.1	High-level description of the gateway interworking service

The alternate speech-text telephony communication (see e.g. [b-ITU-T F.703], [b-ITU-T F.790], [b-ETSI ETR 333], [b-ETSI EG 202 320]) is a regulated service, e.g., as defined for 3GPPs global text telephony (GTT) service ([b-ETSI TS 122 226], [b-ETSI TS 123 226]). That service has the peculiarities that at least one communication party [b-ITU-T AVA TR2] is using audio and text quasi in parallel, but only one media format at a particular point in time and also only in unidirectional manner (e.g., a mute-only person gets audio in receive only mode and text in send only mode, or deaf-only person a vice versa configuration). Figure L.1 illustrates combinations of interest in this Appendix, i.e., one termination with audio and embeed text as voiceband data (using [b-ITU-T V.18] modem), and one termination with separate stream endpoints for native VoIP (audio) and ToIP (text). Again, combinations when one communcation party could use audio or text in bidirectional manner is out of scope because effectively leading to an audio-only or text-only call. The H.248 context model in scope are combinations with unidirectional audio and text (Figure L.1).

This use case is hence a good example for demonstrating stream grouping with scope on

- stream aggregation: two unidirectional traffic flows (carried in bidirectional H.248 media streams) are combined to a single bidirectional one;

- non-applicability of the directionality token usage in property groupse (despite the mentioned fact of a unidirectional characteristic at traffic flow level);

- RTP topology related RTCP handling in the non-grouped and grouped stream model; and

- what happens in case of additional bundling (subject of use case #1.2 in clause L.1.2).

Figure L.1 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways 
– Traffic flow models

The basic question relates to the modeling of audio and text at the left hand side, e.g., whether both could be merged into a single H.248 stream? This is basically possible by usage of the H.248 ReserveGroup property and the definition of two H.248 media groups. There are deficiencies with this model, primarily that both media groups share all common stream endpoint characteristics such as traffic directionality configuration (StreamMode). Other constraints are stream-level statistics. Therefore an enhanced context model based on stream grouping is discussed.

The IP application protocol for H.248 stream endpoint TA(S1) is [b-ITU-T V.152], which relates to a dual mode VoIP / VBDoIP configuration (with an audio mode and VBD mode). The example media formats are

- V.152 audio mode: G.711 µ-law without silence suppression

- V.152 VBD mode: G.711 µ-law as VBD codec plus RTP packet redundancy in order to provide an assured transport service (see clause 6.3.2/[b-ITU-T V.152]).

#### L.1.1.2	Selected stream grouping model

See Figure L.2 with the usage of stream grouping at termination TC (there are two H.248 components streams (TC(S2)  and TC(S3)) are assigned to H.248 aggregation stream TC(S1)).

Figure L.2 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways 
– Context stream grouping model

One call leg is supposed to be located in an IMS (IP multimedia subsystem) domain, and the other call led in a PES (PSTN emulation subsystem) domain.

Desired MG behaviour:

- If SEP TC(S2) receives an AMR/RTP packet, then the AMR encoded voice sample is transcoded to a G.711 encoded voice sample that then SEP TA(S1) transmits an RTP packet with RTP "PT = 0".

- If SEP TC(S3) receives a T.140/RTP packet, then the encoded text is transformed into a corresponding series of baudot tones, which is G.711 encoded and packetized into a series of G.711/VBD/RTP packets (based on [b-ITUT-T V.152], which then are transmitted by SEP TA(S1) within RFC 2198 ("PT=102") RTP packets.

- If SEP TA(S1) receives a G.711/RTP packet with encoded voice sample, then this voice sample is AMR encoded and a corresponding AMR/RTP packet is transmitted by SEP TC(S2). Such an IWF relates to "audio transcoding",

- If SEP TA(S1) receives an RFC 2198 RTP packet ("PT=102"), then it's extracts the contained "PT=98" VBD payloads, then it interprets the VBD payload as a fraction of text part encoding baudot tone and tries to transform the baudot tone into the corresponding text characters (several VBD payloads may need to be concatenated before doing this), then it transforms these text characters into the corresponding T.140 format, which is then encoded and packetized into T.140/RTP packets as per RFC 4103, which then are transmitted by SEP TC(S3). Such an IWF relates to "text transcoding".

Thus, termination TC would have to "know" whether to use its H.248 stream S2 or its S3 for transmission of an RTP packet. This would depend on the payload type format of the RTP packet(s) TA(S1) receives and on the payload type(s) which the IWF uses when generating RTP packets. Hence, the MG is expected to aggregate streams according to the "FID" semantic.

Furthermore, termination TA implies the usage of a single H.248 stream in "dual mode" usage, which is inherently satisfied by V.152 (and its two state model of "audio mode" and "VBD mode").

#### L.1.1.3	Overall signalling scenario (PES-to-IMS call)

It has to be noted that the PES-to-IMS call establishment direction is the difficult one due to the uncertainity of additional text usage, leading to dynamic stream grouping aspects. On the contrary, the IMS-to-PES call establishment direction is straightforward from stream grouping perspective due to the explicit capabilities indications already in the first SDP offer sent from the IMS UE.

The following message sequence diagram (Figure L.3) shows the messages and message numbering scheme which is used by all example message flows in this use case, which are related to the PES network domain being the communication session originator.

Figure L.3 – Overall message sequence diagram for a call in PES to IMS direction

There is call control signalling between the PES domain and the MGC (1, 8), and the IMS domain and the MGC (4, 5, 9, 12). Such call control signalling would be SIP based, but the specific call control protocol syntax is out of scope here.

There is the basic assumption in above scenario that the context C1 is initially created and prepared for audio-only, - and extended to additional text support after a correspondent update from IMS side. The rationale behind is the expected distribution between "audio only" and "audio & text" calls (e.g., an estimated propability of 99.99 % of type "audio only").

#### L.1.1.4	Signaling steps

Notes:

- Some GTT independent SDP attributes are not shown in the following example messages. Such additional SDP attributes would typically be present in Local and Remote descriptors. Also, some GTT independent LocalControl descriptor properties are deliberately not shown.

- Events and Signals descriptors, which would potentially be used in some cases, are also not shown.

Step 1: PES call control signalling (e.g., based on SIP-I)

- request for a "speech" call

Steps 2 and 3: H.248 transaction request/reply {Create Ctxt, Add terminations "IP_A" and "IP_C"}

See Table L.1.1.

Table L.1.1 – Example H.248 signalling for steps 2 and 3 ("audio only configuration")

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| Transaction = 1 { Context = $ { Add = ip/$/$/$ { ; "PES termination" Media { Stream = 1 { LocalControl { Mode = SendReceive, ReservedValue = ON ; NOTE 1 }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced ; start of SDP-defined H.248 media group v=0 c=IN IP4 $ m=audio $ RTP/AVP 0 98 102 a=rtpmap:98 PCMU/8000 a=gpmd:98 vbd=yes a=rtpmap:102 red/8000 ; packet redun. a=fmtp:102 98/98/98/98 a=… }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP4 <PES_IP_addr_audio> m=audio <PES_port_audio> 0 98 a=rtpmap:98 PCMU/8000 a=gpmd:98 vbd=yes a=rtpmap:102 red/8000 ; packet redun. a=fmtp:102 98/98/98/98 a=… }}}}, Add = ip/$/$/$ { ; "IMS termination" Media { Stream = 1 { LocalControl { Mode = SendReceive, ReservedValue = ON ; NOTE 2 }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=audio $ RTP/AVP 103 0 13 100 a=rtpmap:100 telephone-event/8000 a=fmtp:100 0-15 a=rtpmap:103 AMR/8000/1 a=fmtp:103 mode-set=0,2,5,7 a=ptime:20 a=silenceSupp:on - - - - }}}}}} | Initial speech only configuration with media formats AMR at IMS side and G.711 µ-law at PES side. Hence, the MG is requested for an interworking function "audio transcoding". The PES termination is additional enabled for potential VBD, i.e., an - audio configuration 'PCMU' and - VBDoIP configuration with 'PCMU' as VBD codec. |
| Reply = 1 { …} | positive acknowledgement |
| NOTE 1 – The V.152 dual mode configuration implies a ReserveValue 'true' setting. NOTE 2 – The setting ReserveValue 'true' leads to an additional reservation of audio codec types "0" and "13", which could actually result in a transcoding-less context configuration of "PCMU-to-PCMU" interworking (tbc). The setting ReserveValue 'false' would enforce an audio transcoder "AMR-to-PCMU". |  |

Steps 4 and 5: SIP call control signalling – SDP offer/answer cycle

- request for a "audio-only" call with AMR as codec which is confirmed by IMS UE

Steps 6 and 7: H.248 transaction request/reply for completion of the initial audio-only context configuration

Step 8: PES call control signalling – positive response

The IMS terminal decides (e.g., based on configuration data) to change from audio-only to parallel usage of audio and text, leading to a SIP UPDATE or re-INVITE request (step 9).

Step 9: SIP call control signalling – SDP offer

- with an additional media description for a media type 'text'.

Steps 10 and 11: H.248 transaction request/reply {modification of termination "IP_C"}

See Table L.1.2.

Table L.1.2 – Example H.248 signalling for steps 10 and 11

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| Transaction = 3 { Context = <C1> { Modify = <IP_C> { ; NOTE 1 TerminationState { mgroup/groupse = ["FID 2 3"] }, Media { Stream = 1 { LocalControl { mgroup/stragg = [2, 3], ReservedValue = ON }, Local {}, Remote {} }, Stream = 2 { LocalControl { Mode = SendReceive, ReservedValue = ON }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=audio $ RTP/AVP 0 103 100 a=rtpmap:103 AMR/8000/1 a=rtpmap:100 telephone-event/8000 a=fmtp:103 mode-set=0,2,5,7; max-red=0 a=ptime:20 a=maxptime:30 a=silenceSupp:off - - - - }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 <IMS_IP_addr_audio> m=audio <IMS_port_audio> RTP/AVP 0 110 100 a=rtpmap:110 AMR/8000/1 a=rtpmap:100 telephone-event/8000 a=fmtp:110 mode-set=0,2,5,7 a=ptime:20 a=maxptime:30 a=silenceSupp:off - - - - } }, Stream = 3 { LocalControl { Mode = SendReceive, ReservedValue = OFF }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=text $ RTP/AVP 120 121 a=rtpmap:120 red/1000/1 a=fmtp:120 121/121/121 ; NOTE 2 a=rtpmap:121 t140/1000/1 }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 <IMS_IP_addr_text> m=text <IMS_port_text> 120 121 a=rtpmap:120 red/1000/1 a=fmtp:120 121/121/121 ; NOTE 2 a=rtpmap:121 t140/1000/1 } }}}}} | MGC adds TerminationState descriptor containing the mgroup/groupse property in order to request the MG to group the two new SEPs 2 and 3 with semantic "FID" as per RFC 5888. Stream S1: MGC changes this SEP to the aggregating SEP. This allows keeping SEP <IP_A>.S1 without any modifications. MGC sets RV = ON in order to avoid SEP 1 being deleted. MGC replaces Local and Remote with empty descriptors, as the aggregation SEP does not need any own Local or Remote descriptor content and especially does not have an own externally visible transport address. Stream S2: MGC creates SEP 2 which takes over the previous role of <IP_C>'s SEP 1 (audio media towards IMS). However, new SEP 2 is created with choose wildcards in the Local descriptor, as the MG does not support moving SEP 1's transport address to SEP 2. Stream S3: New SEP 3 for the new text media component towards IMS. |
| NOTE 1 - In Step (10) the MGC does not modify the PES side termination <IP_A>.Thus especially it's SEP 1 remains PCMU-based for audio and VBD signals. But due to the modification of <IP_C>'s SEP 1 to an SEP aggregating the new SEPs 2 and 3, and due to the new SEPs 2 and 3 describe audio and RTT text based media components, respectively, the MG now knows that it needs to insert an interworking function into the internal data path between <IP_A>'s SEP 1 on the one hand side and <IP_C>'s SEPs 2 and 3 on the other hand side. The IWF provides: - audio transcoding between AMR and PCMU; - text transcoding between RFC 4103 based RTT to text/baudot/PCMU/VBD; and - "mixing" of the two internal PCMU-based streams to PCMU/RTP/UDP/IP packets in PES direction. NOTE 2 – The example shows packet redundacy levels of two at SEP TC(S3) for native text-over-RTP and of three at SEP TA(S1) for text in VBD encoding (see Table L.1.1.). The higher redundancy might be justified by the different encoding schemes or/and different GoS conditions in the two IP domains. |  |
| Reply = 3 { …} | positive acknowledgement |

Step 12: SIP call control signalling – SDP answer

- completion of call establishment

#### L.1.1.5	Observations

Figure L.3 summarizes again the transition from the initial audio-only context to additional text support.

Figure L.3 – Use case #1.1: summary of the example evolution of the considered context stream grouping model

Steps 9 to 12 are only executed for the rare cases of additional text support. The H.248 "PES" termination would be then not affected by a "context reconfiguration".

The aspect of "RTP topology" [b-ITU-T H.248.88] should be carefully considered:

- Phase 1 "audio only" (steps 1 to 8):

- either B2BRE or RTPMT topology (due to enforced audio transcoding);

- Phase 2 "additional text" (steps 9 to 12):

- The RTP topology between the three external H.248 media streams looks like an "RTP mixer", but the overall communication topology relates still to an two-party call only, underlined by the fact that there are only two H.248 terminations and the H.248 topology descriptor is not used.

- The ultimate requirement for RTCP is the isolation of RTCP control flows to their RTP session legs only (from an end-to-end perspective). Thus, any incoming RTCP report needs to be fully terminated in its H.248 SEP. Such a behaviour is only guaranteed by a B2BRE topology configuration, which is therefore explicitly enforced on each stream and each traffic direction.

- The RTP topology (in the H.248 MG) should not change (tbc) by a transition from phase 1 to phase 2 from an end-to-end communication perspective. That's the reason why the B2BRE topology was already explicitly enforced in both phases.

{Editor's note (2015-02): the rtpt/rtptopo property is located at LD-/RD-level (and not at LCD level) (see H.248.88), which might be in conflict with the LD-/RD-embedded SDP lines for media description, hm?}

### L.1.2	Use case #1.2: alternate speech-text telephony in IP-to-IP gateways with RTP bundling

Use case #1.2 provides RTP bundling in the IMS domain in addition to use case #1.1.

For further studies:

- bundling: shared IP interface (network address only) between TC(S2) and TC(S3)
=> what kind of information needs to be added to TC(S1)  description?

- "groupse" semantic 'FID': what about LIP or BUNDLE?

{Editor's note (2015-06): there was not yet any technical discussion of use case #1.2.}

### L.1.3	Use case #1.3: additional performance monitoring

The transition from initial audio-only mode to "global text telephony" mode could demand for correspondent performance monitoring support (e.g., due to a regulated service).

In order to minimize cost, the number of H.248 statistics might be reduced by consideration of media stream specific characteristics. For instance by support of stream-level statistics.

a) Number of H.248 statistics: at least one performance metric is selected per media type (audio & text) in order to characterize the performance of the media stream.

b) Type of H.248 statistic: RTP packet loss for audio in order to get feedback about Grade of Service, and RTP payload traffic volume in octets for text as a measure for "characters". Furthermore, a performance metric such as information loss isn't really helpful for media type "text" due to enabled packet redundancy mode.

Hence, in order to reduce the cost of performance monitoring to the absolute minimum, following stream-level statistics might be considered ("the Statistics descriptors would be inserted in the H.248 transaction request in step 10"):

Statistics{  ; Stream 2 = audio

rtp/pl,     ; "RFC3550 packet loss" ingress

recrtcp/rpl ; "RFC3550 packet loss" egress

}

Statistics{  ; Stream 3 = text

rtpad/payloados, ; "T.140 text" octets sent

rtpad/payloador  ; "T.140 text" octets recv.

}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.Sup.13]	Supplement ITU-T H.Sup.13 (10/2015), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

[b-IANA group semantics]	IANA registry for "Semantics for the "group" SDP Attribute".
URL: http://www.iana.org/assignments/sdp-parameters/sdp-parameters.xhtml#sdp-parameters-13

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework.

[b-IETF CLUESIG]	IETF draft-ietf-clue-signaling (xx/xxxx), CLUE Signaling.

[b-IETF FLUTE]		IETF draft-ietf-rmt-flute-sdp-03 (09/2012), SDP Descriptors for FLUTE.

[b-ITU-T V.18]	Recommendation ITU-T V.18 (11/2000), Operational and interworking requirements for DCEs operating in the text telephone mode.

[b-ITU-T F.703]	Recommendation ITU-T F.703 (11/2000), Multimedia conversational services.

[b-ITU-T F.790]	Recommendation ITU-T F.790 (01/2007), Telecommunications accessibility guidelines for older persons and persons with disabilities.

[b-ITU-T AVA TR2]	Technical Report ITU-T FG AVA TR Version 1.0 (10/2013), Part 2: Vocabulary for ITU-T Focus Group on Audiovisual Media Accessibility (FG AVA).

[b-ETSI ETR 333]	ETSI ETR 333, Text Telephony; User requirements and Recommendations.

[b-ETSI EG 202 320]	ETSI EG 202 320 Version 1.2.1 (10/2005), Human Factors (HF); Duplex Universal Speech and Text (DUST) communications.

[b-ETSI TS 122 226]	ETSI  TS 122 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 1: Service description (3GPP TS 22.226 version 9.0.0 Release 9).

[b-ETSI TS 123 226]	ETSI  TS 123 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 2: Service description (3GPP TS 22.226 version 9.0.0 Release 9)

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Living List Bibliography

[b-ITU-T V.18]	Recommendation ITU-T V.18 (11/2000), Operational and interworking requirements for DCEs operating in the text telephone mode.

[b-ITU-T F.703]	Recommendation ITU-T F.703 (11/2000), Multimedia conversational services.

[b-ITU-T F.790]	Recommendation ITU-T F.790 (01/2007), Telecommunications accessibility guidelines for older persons and persons with disabilities.

[b-ITU-T AVA TR2]	Technical Report ITU-T FG AVA TR Version 1.0 (10/2013), Part 2: Vocabulary for ITU-T Focus Group on Audiovisual Media Accessibility (FG AVA).

[b-ETSI ETR 333]	ETSI ETR 333, Text Telephony; User requirements and Recommendations.

[b-ETSI EG 202 320]	ETSI EG 202 320 Version 1.2.1 (10/2005), Human Factors (HF); Duplex Universal Speech and Text (DUST) communications.

[b-ETSI TS 122 226]	ETSI  TS 122 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 1: Service description (3GPP TS 22.226 version 9.0.0 Release 9).

[b-ETSI TS 123 226]	ETSI  TS 123 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 2: Service description (3GPP TS 22.226 version 9.0.0 Release 9).

[b-IETF clue-prot]	IETF draft-ietf-clue-protocol (##/201#), CLUE protocol.

[b-IETF clue-sign]	IETF draft-ietf-clue-signaling (##/201#), CLUE signaling.
(=> SDP "a=group:CLUE")

[b-IETF clue-dc]	IETF draft-ietf-clue-datachannel (##/201#), CLUE Protocol Data Channel. 
(=> SDP-based vs DCEP-based DC control procedures")

[b-IETF RFC 6947]	IETF RFC 6947 (05/2013), The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute.

[End Proposed Correction]

____________________