| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4689 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: clause L.3 – discussion and update proposal for "use case #3: Telepresence media server" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal for clause L.3 related to "use case #3 "Telepresence media server".

1	Discussion

There are minor editorial and technical updates.

More important: there wasn't yet any comprehensive technical discussion about this use case at the last 2015-02 meeting. Thus, there are still some pending questions for clarification arround this use case, see in particular the electronic comments.

Technical feedback would be appreciated (e.g., comment [A5] is wondering whether we need a new event in the H.248.STROUP mgroup package or not).

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

Change proposal #1:

[Begin Proposed Correction]

## L.3	Use case #3: Telepresence media server

### L.3.1	Use case #3.1: Protocol stack for application control mapped on group of three streams

Telepresence essentially relates to a realtime multimedia conferencing service associated with a bearer-level control protocol, called CLUE (see [b-IETF clue-prot], [b-IETF clue-sign], [b-IETF clue-dc]).

The example focuses on a single termination, related to one conference party, of a context with usually multiple terminations. Only the stream modelling for the application control traffic, as processed at H.248 MG level, is in scope of this example. Thus, the usually numerous H.248 media streams for audio, video, image, text, data, etc of telepresence based conferencing services are not considered here. The mapping of application control on three, grouped H.248.SEPs is studied in this example, see Figure L.5:

Figure L.5 – Use case #3.1: Telepresence server – Protocol stack for application control mapped on group of three streams

Such a fine granular stream group mapping is motivated by following objectives or assumptions in this example:

- UDP is the primary IP transport protocol, but it cannot be excluded that detected NAT problems could lead to a replacement by TCP during the active conferencing phase.

- The SCTP association and DTLS session/connection are decoupled concerning the directionality of establishment.

- The SCTP association and DTLS session/connection are decoupled concerning the timeline of establishment.

- The DTLS protocol is profiled, primarily due to supported protocol versions and the explicit notification of possible error alerts.

- SCTP association establishment is conditional, dependent on successful DTLS authorization and established DTLS session.

- Thus, SCTP protocol procedures initially blocked. SCTP association establishment is rather triggered in a subsequent step, after confirmation of trusted user.

- Only dial-in conference parties are supported, hence DTLS server role expected, and H.248 SEP prepared for incoming DTLS session establishment procedure.

- SCTP protocol failures could lead to a shut-down of the SCTP association, but should not lead to the release of a DTLS session/connection, rather a re-establishment of an SCTP association.

- CLUE handling for telepresence support: relay of CLUE signalling information between MGC and MG requires a message backhauling service (based on [b-ITU-T H.248.78]), which is tight to a particular SEP here.

- Termination prepared for incoming CLUE protocol messages.

- There are two mechanisms for establishing a CLUE data channel (see [b-IETF clue-dc]): the Data Channel Establishment Protocol (DCEP) and the SDP-based "SCTP over DTLS" data channel negotiation mechanism. The discussed example is using the SDP approach.

- Performance monitoring: a few protocol specific statistics are requested, which result in stream-level statistics.

The considered context model uses three H.248 SEPs (see Figure L.5, "stream group" SG1 in termination T1). Table L.3.1 illustrates example H.248 signalling with regards to the initial creation of the context and termination T1:

Table L.3.1 – Example H.248 media descriptor – step 1 for initial DTLS establishment

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ ; stream group for application control traffic TerminationState { mgroup/groupse = ["CLUE 1 2 3"] ; NOTE 1 } ; CLUE control group ## Stream = 3 { ; IP transport stack LocalControl{ ipdc/realm = <realm_X_IPv6>, ; NOTE 2 Mode = SendRecv }, Local{ v=0 ; c=IN IP6 $ … m=application $ UDP – b= … ; bitrate of IP transport connection …}, Remote{ v=0 ; c=IN IP6 <network address> … m=application <L4_port> UDP – b= … ; bitrate of IP transport connection …}, Statistics{ ; IP layer specific statistics ipocs/ipos, ; NOTE 3 ipocs/ipor ; NOTE 3 } } /* end of Stream 3 */ Stream = 2 { ; DTLS session/connection LocalControl{ ; DTLS protocol profiling tlsbsc/bceb = "unblocked", ; NOTE 4 tlscn/tlsv = ["1.2" "1.0"], ; NOTE 4 tlscn/srsc = OFF, ; NOTE 5 Mode = SendRecv }, Local{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! ## m=application - DTLS – ; NOTE 6 a=fingerprint:$ (to be confirmed) }, Remote{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application - DTLS – a=fingerprint:SHA-256 <UE_A_DTLS_fingerprint> }, Events = 1234 { ; DTLS layer specific events tlsbsc/BNCChange [Type = Est} ; NOTE 7 tlsm/mgea ; NOTE 8 } Statistics{ ; DTLS layer specific statistics ## tlstv/rectmfrag, ; NOTE 9 ## tlstv/senttmfrag ; NOTE 9 } } /* end of Stream 2 */ Stream = 1 { ; CLUE & SCTP association LocalControl{ sctpbcc/bceb = "blocked", ; NOTE 10 sctpbcc/sctpid = <SCTP StreamID>, ; NOTE 11 Mode = Inactive }, Local{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:- }, Remote{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:<UE_A_port_SCTP> }, Events = 5678 { ; CLUE message backhauling mcbalg/det{ stream=1, ; NOTE 12 ## ehpf ="webrtc-datachannel", ## label="CLUE" } } Statistics{ ; SCTP specific statistics … } } /* end of Stream 1 */ ; end of stream group for application control ; streams for application data traffic ## Stream = 10 { ; audio stream 1 … etc, out of scope of this example | Example traffic flow configuration from Figure L.v1. Termination T1: - Group semantics: - Which streams: S1, S2 and S3 are grouped; - Semantics: "CLUE" (or "CLUE*") (to be confirmed) (NOTE 1) - Directionality: unspecified, hence default "bidirectional" at TerminationState level, however, finally profile at Stream level Stream endpoint T1(S3): - IP transport stack - UDP-based (but might be dynamically replaced by TCP in case of NAT issues) - IP address realm indication (NOTE 2) Stream endpoint T1(S2): - DTLS layer Stream endpoint T1(S1): - SCTP layer and CLUE protocol |
| ; other Terminations T2, … |  |
| NOTE 1 –The "grouping semantic" CLUE is defined/proposed for call control signaling, but not yet for H.248 gateway control. Following semantic is supposed her: the streams which are grouped constitute together the protocol stack as used for MG-level CLUE signalling. The distribution on multiple H.248 SEPs facilitates the control of protocol layer specific operations. NOTE 2 – IP domain indication is required. The control scope could be limited on SEP T1(S3) only. NOTE 3 – The IP layer traffic volume in octets is measured in order to get real data concerning a particular telepresence conference user configuration. The measurements are subject of fine-tuned bitrate estimations for traffic control of future telepresence calls. These H.248 statistics are tight to the "IP" SEP in order to avoid any ambiguities due to the overall, hierarchical protocol layering of "L4-over-L4" (here: SCTP/…/L4/IP"). NOTE 4 – DTLS protocol profiling: might be necessary when telepresence mandates any DTLS protocol version (which is the case in WebRTC, hence might be applicable here too). NOTE 5 – DTLS protocol profiling: furthermore, DTLS renegotiation is explicitly disabled due to multiple reasons there are security issues with this (D)TLS protocol procedure per se, e.g., if used at all in TLS, then only in association with RFC 5746; however the next TLS version (1.3) seems to deprecate that procedure at all. . The expected MG behavour should be the blocking of any DTLS handshake procedure after the initial, successful establishment of the DTLS connection. NOTE 6 – The SDP-based media description for the "DTLS" SEP requires only the indication of protocol "DTLS" and security information. NOTE 7 – Incoming DTLS session establishment request(s) are expected. NOTE 8 – DTLS alerts could lead to the automatic release of a DTLS connection (and loss of (SCTP)-association-endpoint and loss of (CLUE)-endpoint). The MGC wants to be notified about the particular DTLS alert type. NOTE 9 – The MGC observes the "DTLS control traffic" by enabling statistics related to "DTLS messages" (see Annex B and clause 12 in [b-ITU-T H.248.90]), which cover essentially all DTLS protocol data units without any SCTP data. NOTE 10 – Preparation of SCTP layer by initial blocking of any SCTP bearer control procedures (due to first confirmation of trusted user and successfully established DTLS connection inline with allowed DTLS protocol profile). NOTE 11 – SCTP StreamID handling: for further studies (dependent on SDP- vs Property-based indication). NOTE 12 – Handling of CLUE information for telepresence support. It may be noted that events parameter "stream=1" relates to an H.248 stream and not to an SCTP stream. |  |

When the remote user equipment answers, then the MGC will modify the configuration of context C1. There might be following actions:

Table L.3.2 – Example H.248 media descriptor – step 2 for completion

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| FIXTHIS… Stream = 3 { ; IP transport stack => no changes Stream = 2 { ; DTLS session/connection => no changes Stream = 1 { ; CLUE & SCTP association => SCTP association establishment procedure |  |

### L.3.2	Use case #3.2: Protocol stack for application control mapped on group of two streams

{Editor's note (2015-02): use case #3.1 illustrates the most granular model concerning the mapping on individual H.248 SEPs. Such a mapping is feasible (thanks to H.248.STGROUP) and beneficial for specific H.248 telepresence media server scenarios. However, there are network solutions without the need of a full-fledged usage of the outlined telepresence transport protocol stack, a simplified NAT traversal context, c etc. Such scenarios could lead to simplified stream grouping models. Such use cases are subject of this and the next sub-clause.}

This approach distributes the protocol stack for application control on two SEPs, which both are grouped together using stream grouping. The primary purpose is to separate SCTP and its upper layer protocol CLUE from the underyling tunneled transport stack, see Figure L.6:

Figure L.6 – Use case #3.2: Telepresence server – Protocol stack for application control mapped on group of two streams

### L.3.3	Use case #3.3: Protocol stack for application control mapped on a single stream

This model should be basically feasible and might be default for H.248 profiles with telepresence support. The model might be sufficient particularily when all the assumptions of use case #3.1 would be not relevant (i.e., in case of a reduced set of requirements).

{Editor's note: "proof of assumption still missing"}

The resulting context model is depicted in see Figure L.7, using a single stream for the entire protocol stack for application control:

Figure L.7 – Use case #3.3: Telepresence server – Protocol stack for application control mapped on a single stream

It has to be noted that use case #3.3 does not use any kind of stream grouping.

### L.3.4	Benefits of stream grouping for CLUE-based telepresence

Some observations related to the stream grouping based solutions #3.1 and #3.2:

- aggregation stream looks beneficial for additional services (here: H.248 statistics for performance monitoring support);

- isolation of individual bearer-connection-endpoints (here L4, DTLS, SCTP) in self-contained SEPs looks attractive from H.248 signalling side ("e.g., because subsequent modification operations could limit the signalling on information only affected within a H.248 media descriptor").

[End Proposed Correction]

____________________