| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4696 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.PAURES: Part II – Package review |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal for clause 8.

1	Introduction

The reader should firstly review parallel AVD-4695, which discusses Pause/Resume-signaling in a general context and provides updates for the clauses 1 to 7, as well as a new proposed Appendix related to detailed use case documentation.

This contribution focuses on the new proposed H.248 rempr package in clause 8.

2	Discussion

## 2.1	P/R-signalling protocol endpoints: MG roles as RTP receiver and RTP sender

Figure 1 illustrates the current design of the rempr package:

Figure 1 – H.248 rempr package – latest design status

We'd like to note following observations:

- the H.248 termination / stream-endpoint, enabled for P/R-signalling could provide the role of a) RTP sender, b) RTP receiver or c) both;

leading to two follow-up aspects:

- package name: "Remote Pause and Resume" might be misleading because there's also "Local Pause and Resume" support in our understanding.
=> alternative name might be "RTP stream pause and resume package" or "Pause and Resume support package"?

- parameter 'SSRC': the parameter value setting isn't straightforward due to the multiple role option (see discussion in more detail in clause 2.3).

## 2.2	P/R-signalling protocol endpoints: finite state machine modeling

AVD-4695 started to consideration of the state model behind RTP sender entities. The Events and Signals of the H.248 rempr package are directly associated to state transitioning of such state models. We'd like to add a new Appendix II which illustrates state models for RTP sender and RTP receiver side, as well as their relationship to the H.248 package elements.

We are convinced that such a complementary formal model would improve the H.248 package specification with regards to an unambiguous RTP behaviour.

## 2.3	Identification of RTP sender and RTP receiver entities via SSRC elements

The "SSRC parameter(s)" are the key elements within P/R-signalling in order to indentify the RTP sender and RTP receiver as signalling endpoints. Figure 2 recalls again the basic RTCP Feedback (FB) message structure as introduced by RFC 4585:

Figure 2 – generic RTCP FB Packet ("Feedback message") 
– Principal structure of the Protocol Data Unit

Thus, following RTCP codepoint tuples are in scope of H.248.PAURES:

- native P/R-signalling is based on {PT, FMT} = {205, TBA1};

- emulated P/R-signalling is based on {PT, FMT} = {205, 3 & 4}.

Figure 3 illustrates the RTCP FB message types for native P/R-signaling:

Figure 3 – RTCP FB Packets for Pause/Resume-signaling – Structure

And Figure 4 highlights the three SSRC fields as part of such a new RTCP FB message:

Figure 4 – RTCP FB Packets for Pause/Resume-signaling – SSRC related fields

We'd like to note following observations:

- the SSRC elements are crucial to the P/R-service;

- the number of SSRC fields between native RTCP FB P/R messages and RTCP FB TMMBx messages for an emulations approach is different ("3 to 2");

- thus, the rempr package information related to SSRC parameters needs to be carefully revisited which SSRC field(s) the MG exactly is requested to process;

- we'd like to add a new Appendix III "SSRC identifiers for Pause/Resume signaling endpoints" on this subject.

This contribution proposes some updates related to SSRC information from that perspective. However, the evaluation isn't yet complete and may require another review cycle ("due to the flexible usage and positioning of the package, which allows to address RTP receiver and RTP sender roles").

## 2.4	MGC knowledge about SSRC information?

We are not sure whether this aspect was already sufficiently discussed so far. We see following aspects in general:

- MGC call control signalling interface:

  - Incoming call request: 
i) explicit indication of P/R-signaling? (YES!, see clause 9/[IETF STREAM PAUSE] in case of SIP/SDP);
ii) explicit signaling of SSRC information related to RTP sender / RTP reciever entities? (OPEN);

  - Outgoing call request: 
i) explicit indication of P/R-signaling? (YES!, see clause 9/[IETF STREAM PAUSE] in case of SIP/SDP);
ii) explicit signaling of SSRC information related to RTP sender / RTP reciever entities? (OPEN);

=> actions for H.248.PAURES: 
[A1]: Clarification whether explicit SSRC information is part of call control signalling?

[A2]: Clarification in package procedures (clause 8.6) whether RFC 5576 SDP attributes for SSRC signalling are used or not?

- MGC gateway control signalling (H.248) interface:
There are multiple options of signalling of SSRC information, at least:

  - H.248.PAUSRES rempr package;

  - H.248.71 rtcpsdes package;

  - RFC 5576 SDP a=ssrc: attribute

=> actions for H.248.PAURES: 
[A3]: Clarification which H.248 tool is used for SSRC information signalling? (default of course is the rempr package assumption).

This contribution provides some updates from that perspective, but still misses the MGC call control signalling related evaluation. We've tried to start to resolve [A3] in parallel AVD-4697, which proposes a new clause for the main body (with a subclause on "x.4 Gathering SSRC values, relevant for P/R-signaling, from the IP media path").

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD 4678, the input draft for this Q.3/16 meeting (AVD-4678).

Change proposal #1:

[Begin Proposed Correction]

# 8	Remote pause and resume package

|  | Package name: | Remote Pause and Resume |
| --- | --- | --- |
|  | Package ID: | rempr (0x????) |
|  | Description: | This package allows a MGC to request that the MG issue a request to a remote endpoint to pause (and subsequently resume) the transmission of an RTP media stream. The H.248 media stream and RTP media flow is supposed to be used in bidirectional manner. Hence there are two RTP streams, one in incoming and one in outgoing direction. The package makes the assumption that the MG may provide P/R-signaling for both unidirectional RTP streams. Both P/R-signalling protocols are disjoint. The package description makes the assumption that the MG could provide the RTP receiver or/and the RTP sender function in parallel on a particular H.248 SEP enabled for support of this package. There are two slightly different RTP stream pause state models at RTP sender and RTP receiver side, see also Appendix II. # {Contributors note: the package provides also support for MG role as RTP sender, right? If yes, then we would get a "local pause and resume package" … or?} |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Autonomous Response

|  | Property name: | Autonomous Response |
| --- | --- | --- |
|  | Property ID: | ar (0x0001) |
|  | Description: | The property indicates whether the MG shall provide an autonomous response to PAUSE and RESUME requests (NOTE) or whether it should notify the MGC and await confirmation from the MGC on what action to take. NOTE – There are two options how such a stimuli could be received by the H.248 SEP: a) from a remote RTP endpoint or/and b) from another H.248 SEP within the context (see e.g. use case #3b in Figure I.2). {Contributors note: above text refers to the proposal by AVD-4695. The updated text reflects our understanding. However, we are not sure whether it was intended by the authors of this package.} |
|  | Type: | Boolean |
|  | Possible values: | On MG autonomous: The MG responds autonomously Off MGC controlled: The MG notifies the MGC and awaits a response |
|  | Default: | On (unless provisioned otherwise) |
|  | Defined in: | LocalControl or TerminationState |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	RTP Pause State

|  | Event name: | RTP Pause State |
| --- | --- | --- |
|  | Event ID: | rtpps (0x0001) |
|  | Description: | This event allows the MG to indicate when the state associated with an RTP stream Pause / Resume changes, i.e., the H.248 stream endpoint realizes the RTP sender function. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	State

|  | Parameter name: | State |
| --- | --- | --- |
|  | Parameter ID: | state (0x0001) |
|  | Description: | This parameter indicates which pause states the MGC wishes to be notified about. Based on the RTP pause state model according to clause 6/[IETF STREAM PAUSE] (see also Appendix II.2). |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | paused The MG has paused RTP media flow as a result of the receipt PAUSE request from a remote RTP endpoint. resumed The MG has resumed RTP media flow as a result of the receipt of a RESUME request from a remote RTP endpoint localPause The MG has paused RTP media flow based on a local decision. localResume The MG has resumed RTP media flow based on a local decision. |
|  | Default: | All (paused, resumed, localPause, localResume) |

##### 8.2.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 8.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Observed State

|  | Parameter name: | Observed State |
| --- | --- | --- |
|  | Parameter ID: | obstate (0x0001) |
|  | Description: | This parameter indicates the detected pause state. It is reported whenever there is a transition between the states. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | As per “states” clause 8.2.1.1.1 |
|  | Default: | None |

##### 8.2.1.2.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.2.2	Detect Pause / Resume Request

|  | Event name: | Detect Pause Resume Request |
| --- | --- | --- |
|  | Event ID: | dprreq (0x0002) |
|  | Description: | This event allows the MG to indicate when a PAUSE or RESUME request has been detected. Thus, the H.248 stream endpoint realizes the RTP sender function. |

#### 8.2.2.1	EventsDescriptor parameters

##### 8.2.2.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 8.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.2.2	ObservedEventsDescriptor parameters

##### 8.2.2.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained the detected request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.2.2.2.2	Request Type

|  | Parameter name: | Request Type |
| --- | --- | --- |
|  | Parameter ID: | reqt (0x0002) |
|  | Description: | This parameter indicates whether a PAUSE or a RESUME is requested for the RTP Stream. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “PAUSE” A PAUSE has been requested. “RESUME” A RESUME has been requested. |
|  | Default: | None |

##### 8.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.2.3	Detect Pause / Resume Result

|  | Event name: | Detect Pause Resume Result |
| --- | --- | --- |
|  | Event ID: | dprres (0x0003) |
|  | Description: | This event allows the MG to detect and indicate whether the remote RTP sender has: sent a PAUSED indication, a REFUSE indication or resumed RTP transmission in response to a request initiated by the MGC. It also detects a failure of the RTP sender to take action on a PAUSE or RESUME request. Thus, the H.248 stream endpoint realizes the RTP receiver function. |

#### 8.2.3.1	EventsDescriptor parameters

##### 8.2.3.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### 8.2.3.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the detected PAUSED or REFUSED indication. It may also contain the pauseID related to a failed PAUSE or RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.2.3.2.2	Response Type

|  | Parameter name: | Response Type |
| --- | --- | --- |
|  | Parameter ID: | rest (0x0002) |
|  | Description: | This parameter indicates whether a PAUSED or a REFUSED indication has been received. It also indicates whether a RESUME request as resulted in the flow of RTP packets from the RTP sender. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “paused” A PAUSED indication has been received. “refused” A REFUSED indication has been received. “resumed” The MG has detected a resumed RTP flow. “failed” No response from the RTP sender has been detected |
|  | Default: | None |

##### 8.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 8.3	Signals

### 8.3.1	Local Pause

|  | Signal name: | Local Pause |
| --- | --- | --- |
|  | Signal ID: | lpause (0x0001) |
|  | Description: | This signal indicates that the MG shall pause the RTP stream and send a PAUSED indication to the remote RTP receiver(s). Thus, the H.248 stream endpoint realizes the RTP sender function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.1.1	Additional parameters

##### 8.3.1.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID of the PAUSE to which the PAUSED indication relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. {Editor's note (2015-02): there are basically three sources of information at MGC level with regards to SSRC values: a) as detect and reported by the MG via events rtpps/ssrc or dpreq/ssrc; b) via SDP attributes in SIP signalling (tbc); or/and c) via H.248.71 "statistic" rtcpdes/rssrc. It is supposed that method (a) is used in context of this package.} |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.2	Local Resume

|  | Signal name: | Local Resume |
| --- | --- | --- |
|  | Signal ID: | lresume (0x0002) |
|  | Description: | This signal indicates that the MG shall resume transmission of the RTP stream. Thus, the H.248 stream endpoint realizes the RTP sender function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.2.1	Additional parameters

##### 8.3.2.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the RESUME relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.2.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.3	Refuse

|  | Signal name: | Refuse |
| --- | --- | --- |
|  | Signal ID: | refuse (0x0003) |
|  | Description: | This signal indicates that the MG shall not change the state of the RTP stream. If the MG had received a remote request to change the state, it should send a REFUSE indication to the remote end. Thus, the H.248 stream endpoint realizes the RTP sender function. (to be confirmed) |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.3.1	Additional parameters

##### 8.3.3.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the REFUSE relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.3.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the REFUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.4	Remote Pause

|  | Signal name: | Remote Pause |
| --- | --- | --- |
|  | Signal ID: | rpause (0x0004) |
|  | Description: | This signal indicates that the MG shall send a PAUSE request to the remote RTP sender. Thus, the H.248 stream endpoint realizes the RTP receiver function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.4.1	Additional parameters

##### 8.3.4.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the PAUSE request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.4.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.5	Remote Resume

|  | Signal name: | Remote Resume |
| --- | --- | --- |
|  | Signal ID: | rresume (0x0005) |
|  | Description: | This signal indicates that the MG shall send a RESUME request to the remote RTP sender. Thus, the H.248 stream endpoint realizes the RTP receiver function. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.5.1	Additional parameters

##### 8.3.5.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.5.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 8.4	Statistics

{Editor’s note: There may be a need for a statistic to indicate the ratio of inactivity (due to a pause) to activity. Contributions are solicited.}

None

## 8.5	Error Codes

None

## 8.6	Procedures

This Package allows the gateway to act in four main modes:

- MG autonomous handling of pause and resume requests from a remote RTP receiver where the MGC is notified of RTP stream state change. The MG acts in the RTP sender role.

- MGC controlled handling of pause and resume requests from a remote RTP receiver where the MGC is notified of pause and resume messages from a remote RTP sender and provides a response to the MG regarding the action to take. The MG acts in the RTP sender role.

- MGC initiated pause/resume requests towards a remote RTP receiver where the MGC receives the pause and resume request results from MG. The MG acts in the RTP receiver role.

- MGC initiated local pause/resume requests towards a remote RTP receiver where the MGC receives the pause and resume request results from MG. The MG acts in the RTP sender role.

The procedures for these modes are described in clauses 8.6.3 to 8.6.7. Clauses 8.6.1 and 8.6.2 provide general procedures these modes.

### 8.6.1	General - SDP parameter setting

On H.248 Streams where RTP stream Pause / Resume is supported, for all four(?) modes of operation the MGC shall set SDP media descriptions in the Local and/or Remote Descriptors indicating the support of “pause” codec control message (CCM) [IETF RFC 5104] as per clause 9/[IETF STREAM PAUSE]. This allows the MGC to indicate to the MG the support of MG initiated local pause and resume (as RTP sender and RTP receiver) without the need for H.248 Signals and Events.

{Editor's note (2015-06): the RTP stream pause and resume behaviour could be (partially) emulated via RTCP FB TMMBR & TMMBN message types, too. This option is in scope of /[IETF STREAM PAUSE], but not (yet) in H.248.PAURES. It should be explicitly mentioned in this Draft. => new clause 9 proposed … }

#### 8.6.1.1	Pause

The SDP attribute for “pause” allows a “config” parameter to be set against an "m="-line which indicates what Pause/Resume CCM messages it supports. This description is based on the "m="-line describing a bi-directional bearer however the Local and Remote Descriptor describes a unidirectional bearer. Rather than mapping the “config” value to a separate value for the Local and Remote Descriptor the same value shall be set on both these Descriptors.

### 8.6.2	General – Event and Signal Scope

Given that pause and resume relate to a single RTP stream, Signals and Events should be set in such a way that the affected RTP streams are unambiguously identified. Where a H.248 Stream contains a single RTP stream (i.e. a single SSRC) the MGC may omit the ssrc parameter when setting any Signals and Events in this package. Where a H.248 Stream contains multiple RTP streams the MGC shall include the ssrc parameter indicating the SSRC of the applicable RTP stream.

The H.248 ssrc parameter shall be mapped by the MG on the correspondent SSRC field in a RTCP Pause/Resume CCM message, dependent on the applied in RTP sender or RTP receiver role (see also Appendix III).

Where there are multiple H.248 Streams on a Termination the MGC shall set the StreamID related to the RTP Stream in the Events and Signals in this package.

### 8.6.3	MG autonomous handling of remote requests

The MGC controller may confer handling of RTP Pause/Resume requests to the MG by setting the “Autonomous Response” (rempr/ar) property to “on” indicating that the MG shall autonomously process and respond to any RTP pause/resume messages as defined by [IETF STREAM PAUSE]. If the MGC requires to be notified of any changes in RTP transmission state the MGC shall set the “RTP Pause State” (rempr/rtppps) event with the states that it requires to be informed about. The MGC shall also set the ‘pause’ parameter in the Local and Remote Descriptor as appropriate according to clause 8.6.1.

On receipt of a PAUSE request message the MG shall as RTP sender act according to clauses 5.2 and 5.3/[IETF STREAM PAUSE]. This may result in the MG pausing the RTP stream and the sending of a PAUSED indication or it sending a REFUSE message and continuing the transmission of the RTP stream. If requested when entering the PAUSED state the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has entered the “paused” state (obstate=“paused”).

On receipt of a RESUME request message the MG shall as RTP sender act according to clause 5.4 /[IETF STREAM PAUSE]. This may result in the MG resuming the RTP stream or the sending of a REFUSE message and the continuation of the paused state. If requested when resuming media flow the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“resumed”).

{Contributor's note (2015-06): above two paragraphs make the assumption that the specific P/R-signalling endpoint role is not pre-assigned, rather subject of received P/R-signalling … }

According to clause 5.3/[IETF STREAM PAUSE] an MG may pause an RTP Stream at any time. If the MG initiates a local pause and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been paused (obstate=“localPause”) due to a local decision.

According to clause 5.4/[IETF STREAM PAUSE] an MG may resume an RTP Stream at any time. If the MG initiates a local resume and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“localResume”) due to a local decision.

### 8.6.4	MGC reception of pause and resume requests

The MGC may require that it be responsible for the decision of whether or not to pause or resume an RTP stream. In this case it shall set the “Autonomous Response” (rempr/ar) property to “off” indicating to the MG that it shall defer the decision to pause/resume an RTP Stream to the MGC. The MGC shall also set the “Detect Pause/Resume Request” (rempr/dprreq) to request notification of pause or resume requests.

On receipt of a PAUSE request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a pause (reqt=”PAUSE”). The MGC shall take into account any topology (end-to-end RTP session topology, see clause 7 and MG-local internal RTP topology, see Appendix I), service or other local considerations to determine whether to grant the pause request. In order to grant the request the MGC shall send the “pause” (rempr/lpause) signal to the MG. On receipt of the signal the MG shall pause the RTP Stream and send a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.2 and 5.3/[IETF STREAM PAUSE].

On receipt of a RESUME request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a resume (reqt=”RESUME”). The MGC shall take into account any topology (see clause 7), service or other local considerations to determine whether to grant the resume request. In order to grant the request the MGC shall send the “resume” (rempr/lresume) signal to the MG. On receipt of the signal the MG shall resume the RTP Stream as per clause 5.4/[IETF STREAM PAUSE]. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.4/[IETF STREAM PAUSE].

The MG shall not send a Notify.req with “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent with a request type for a pause state that the RTP stream is already in. I.e. the MG shall not send a request type equal to pause if the RTP stream is already paused.

### 8.6.5	MGC initiation of remote pause and resume requests

The MGC may require that for a particular RTP stream that the remote RTP sender pauses RTP transmission. In order to request the MG to send a PAUSE request to the remote RTP sender, the MGC shall send the “Remote Pause” (rempr/rpause) Signal to the MG. On reception of the signal the MG shall initiated a PAUSE request on the applicable RTP stream. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSE request. The use of the pause identity allows the MGC and MG to correlate pause/resume requests and replies.

The MGC may require that for a particular RTP stream that the remote RTP sender resumes RTP transmission. In order to request the MG to send a RESUME request to the remote RTP sender, the MGC shall send the “Remote Resume” (rempr/rresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response from the remote RTP sender regarding MGC initiated PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/rpause signal with a pause identity, the MG arms the Event and sends a Notify.req to the MGC when it detects a PAUSED or REFUSED indication for the applicable pause identity. On reception of a rempr/rresume signal with a pause identity, the MG arms the Events and sends a Notify.req to the MGC when it detects a REFUSED indication or when it detects a resumed RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest). If after a suitable timeout period (see clause 4.6/[IETF STREAM PAUSE] the MG does not detect any response from the RTP sender (either by signaling pause CCM messages or via a modification in the sending of the RTP stream) the MG shall send a Notify.req containing the dprres ObservedEvent containing the Response Type indicating a failure.

### 8.6.6	MGC initiation of local pause and resume

The MGC may require that for a particular RTP stream that the MG as RTP sender pauses RTP transmission. In order to request the MG to pause RTP sending, the MGC shall send the “Local Pause” (rempr/lpause) Signal to the MG. On reception of the signal the MG shall initiate a PAUSE on the applicable RTP stream. The MG shall send a PAUSED indication to the RTP receivers. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSED indication.

The MGC may require that for a particular RTP stream that the MG as RTP sender resumes RTP transmission. In order to request the MG to resume RTP sending, the MGC shall send the “Local Resume” (rempr/lresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response of the MG to MGC initiated local PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/lpause signal with a pause identity, the MG sends a Notify.req to the MGC when it enters the local paused state. On reception of a rempr/lresume signal with a pause identity, the MG sends a Notify.req to the MGC when it resumes RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest).

### 8.6.7	MG initiation of remote pause and resume

{Editor’s note: MG autonomous initiation of remote pause and resume should be considered.}

### 8.6.8	Error handling

The MGC shall not send signals or set events that are incompatible with the SDP “config” CCM parameters. For example, the MGC shall not request the “rpause” signal on a Stream if it has not set the “config” parameter equal to 1,2 or 4. The table below indicates the allowed settings:

Table 2 – Allowed setting of “rempr” package signals and events

| Package “rempr” Signals | Valid “configs” | Comments |
| --- | --- | --- |
| Local Pause (lpause) | 1,2,3,5,6,8 |  |
| Local Resume (lresume) | 1,2,4,5 |  |
| Refuse (refuse) | 1,2,5 |  |
| Remote Pause (rpause) | 1,2,4 |  |
| Remote Resume (rresume) | 1,2,4 |  |
| Package “rempr” Events | Valid “configs” | Comments |
| RTP Pause State (rtpps) | 1,2,3,4,5,6,7,8 | As the event may detect in state it may be set against any configuration. |
| Detect Pause Resume Request (dprres) | 1,2,3,4,6,7 |  |
| Detect Pause Resume Result (dprres) | 1,3,5 |  |

If the MGC sends a Signal or Event without having also set the having set the ‘pause’ or ‘paused’ SDP parameter in a Local or Remote Descriptor then the MG shall respond with error 472 “Required Information Missing”. If the MGC sends a Signal or Event on a Stream where the ‘pause’ or ‘paused’ SDP parameter is set contravening Table 2, then the MG shall respond with error 473 “Conflicting Property Values".

# 9	Package-less alternative: partial emulation service using RTCP FB messages related to Temporary Maximum Media Stream Bit Rate signaling

{Editor’s note (2015-06): this option is indicated in [IETF STREAM PAUSE]. It would lead to an package-less approach because the current rempr package does not address the RTCP FB TMMBR / TMMBN message types. It's also a kind of emulation service because a) the original semantic of these RTCP FB message types was slightly different, and b) the solution does not address all requirements and design considerations. Contributions are solicited. }

# 10	Examples

{Editor’s note: TBD}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix II

State models for Pause/Resume signaling endpoints

(This appendix does not form an integral part of this Recommendation.)

## II.1	Introduction

Clause 6/[IETF STREAM PAUSE] defines the RTP pause state model at RTP sender side. This model is the normative framework for the H.248 Events and Signals as defined by the rempr package in clause 8.

Purpose of this Appendix is to illustrate and indicate the state transitions which are corelated with the H.248 signalling elements. In case of any discrepancies between this Appendix and clause 8, the package specification in clause 8 takes precedence.

## II.2	RTP pause state model at RTP sender side

Figure II.1 illustrates the RTP sender side state model:

Figure II.1 – RTP sender (RTP source): RTP Pause State model – extended model with additional indication of H.248 signalling elements

## II.2	RTP pause state model at RTP receiver side

Figure II.2 illustrates the RTP receiver side state model:

{Editor's note (2015-06): [IETF STREAM PAUSE] does not provide any information about a potential state model at RTP receiver side. Below figure reflects a proposal by Q.3/16. The state names are provisional. There might be also a chance to reduce the number of state from three to two. Contributions are solicited.}

Figure II.2 – RTP receiver (RTP sink): RTP Pause State model – extended model with additional indication of H.248 signalling elements

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix III

SSRC identifiers for Pause/Resume signaling endpoints

(This appendix does not form an integral part of this Recommendation.)

## III.1	SSRC information in RTCP FB "Pause/Resume" messages

Figure III.1 outlines the structure of such a RTCP FB message type:

Figure III.1 – RTCP FB packets for Pause/Resume-signaling – SSRC related fields

There are (at least) three SSRC identifiers in such a RTCP FB packet:

- SSRC packet sender: the SSRC of the RTP endpoint issueing a P/R-signalling message in RTP sender or RTP receiver role;

- SSRC media source: original semantic see [IETF RFC 5104], not used in P/R-signalling messages, set to value '0';  and

- SSRC target: the SSRC of the remote RTP endpoint acting as targeted P/R-signalling (i.e., the associated RTP sender or RTP receiver).

It could be concluded that the H.248 signalled ssrc parameter (as part of Event configuration, Event notification and Signal configuration) is either mapped on SSRC #1 (packet sender) or SSRC #3 (target), dependent whether the H.248 unidirectional media stream endpoint is acting in RTP sender or RTP receiver role.

## III.2	Example use case

Figure III.2 provides an example, based on use case #3b from Appendix I, extended to bidirectional support of P/R-signaling. The H.248 rempr package supports this case, which facilitates the understanding  the scope of the ssrc information in H.248 Signals and Events of this package. It has to be noted that the MG provides an [ITU-T H.248.88] B2BRE RTP topology. Other RTP topologies might lead to different SSRC assignments.

Figure III.2 – Example use case #3b (from Appendix I) 
– Extended for bidirectional P/R-signaling and indication of SSRCs

[End Proposed Correction]

____________________