| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4697 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.PAURES: Part III – Possible interactions with RTP topologies |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses possible relations and interactions with H.248.88 and H.248.71.

1	Introduction

The existing Draft contains Editor's notes to both H.248 Recommendations, i.e., open questions for clarification. This contribution provides an update proposal based on a first analysis.

2	Discussion

We need firstly to classify P/R-signalling as a particular RTCP service before we could conclude on possible interactions.

The next questions relates to the possible combinations of 
a) MG-local P/R-signalling handling (termination or forwarding decision?) and 
b) MG-local applied H.248.88 RTP topology.

The third complication results from two possible RTCP PDU structures: compound vs reduced-size RTCP packet types, which lead to additional considerations.

The results of our analysis and correspondent guidelines are proposed to be documented in a new clause 8 ("either before or after the H.248 rempr package clause").

Parallel AVD-4696 started to evaluate the SSRC aspect, where we still got an open question concerning H.248.71. It could be generally concluded that the H.248.71 rtcpsdes package and the H.248.PAURES rempr package could be applied together in an H.248 profile without any interactions. However, there's one use case where the H.248.71 rtcpsdes package migth be beneficial for P/R-signalling. See also new clause 8.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD 4678, the input draft for this Q.3/16 meeting (AVD-4678).

Change proposal #1:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/09), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.66]	Draft Recommendation ITU-T H.248.66 (xxxx), Gateway control protocol: Packages for RTSP and H.248 interworking.

[ITU-T H.248.STGROUP]	Recommendation ITU-T H.248.STGROUP (xxxx), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5104]	IETF RFC 5104 (2008), Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF).

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF BUNDLE]	draft-ietf-mmusic-sdp-bundle-negotiation-16 (xxxx), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF STREAM PAUSE]	IETF draft-ietf-avtext-rtp-stream-pause-05 (xxxx), RTP Stream Pause and Resume.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 6	Relation to other Recommendations in the ITU-T H.248 sub-series

Gateways utilising H.248 currently have several methods to pause the transmission of media.

[ITU-T H.248.1]: Contains the Local Control Descriptor ‘Streammode’ property that allows the MGC to effectively stop sending a RTP media stream (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the Streammode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

[ITU-T H.248.1]: The H.248 connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate a media stream from other input streams effectively stopping the flow of media. The MGC may then move the Termination/Stream back into a Context to resume media flow.

[ITU-T H.248.9]: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the media flow via a DTMF digit.

[ITU-T H.248.66]: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

[ITU-T H.248.71]: Contains the "RTCP source description package" which allows the MGC to request the MG to monitor, extract and report SSRC values from RTP streams. Clause 8 describes how that capability might be used together with RTP stream pause and resume services.

[ITU-T H.248.88]: Describes basically the RTCP handling by an H.248 MG in association with a particular enforced local RTP topology. Clause 8 provides more detailed information on this subject.

None of the above methods match the functionality supported by [IETF STREAM PAUSE]. [ITU-T H.248.1] and [ITU-T H.248.66] do not support user originated requests being terminated at the MG and [ITU-T H.248.9] and [ITU-T H.248.66] do not allow the pause of media streams not related to the playout of signals.

Pause/resume relates to real time media flow. Therefore the resumption of media flow is based on the media at the time of the resume request rather than at the pause point. [ITU-T H.248.66] assumes that the playout occurs from the pause point unless another point is indicated in the resumption request.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 7	Considerations

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP pause and resume. It lists a number of RTP topologies. A H.248 gateway may act as a media sender, media receiver and/or mixer.  As such the design criteria listed in clause 4/[IETF STREAM PAUSE] should be considered when implementing RTP pause/resume on gateways.

…

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 8	RTP topology dependent RTCP handling

## 8.1	Introduction

The H.248 stream endpoint(s) or/and context as such involved in RTP stream pause and resume services might be requested to support additional RTP / RTCP related services, leading to potential interactions which should be considered by H.248 profile designers. A brief summary of the problem statement may be found in clause 6/[ITU-T H.248.88]. There are basically three aspects:

- correct handling of RTCP FB "Pause/Resume" messages, dependent on the end-to-end use case of RTP stream pause and resume (see Appendix I) and dependent on the MG-internally enforced RTP topology (see clause 8.2);

- correct handling of compound RTCP packets (see clause 8.3);

- correct handling of SSRC values due to their usage as P/R-signalling endpoint identifiers as well as their occurrence at "multiple places" in SDP and H.248 signalling elements (see clause 8.4).

## 8.2	Interactions with H.248.88

[IETF STREAM PAUSE] defines a service, with RTCP-based signaling along the IP media path, which belongs to following [ITU-T H.248.88] category:

- RTP profile dependent RTCP service (clause 8.3.2/[ITU-T H.248.88]) because it is required to use RTP profiles RTP/AVPF or RTP/SAVPF;

- handling of RTCP FB "Pause/Resume" messages is dependent on the network use case:

  - local P/R-signalling endpoint: requires the local origination and termination of such RTCP FB messages (i.e., support of function F-6 in clause 7.6/[ITU-T H.248.88]);

  - transparent forwarding of P/R-signalling (see e.g. use case #4 in Appendix I): requires the unmodified forwarding of such RTCP FB messages (i.e., support of function F-4 in clause 7.6/[ITU-T H.248.88]); and

- such required RTCP handling behaviour should be consistent when RTP topologies "point-to-point" (aka B2BRE), "transport translator" or "mixer" would be enabled (i.e., a property setting of rtpt/rtptopo equal to 'PP', 'TR' or 'MX').

It remains the question how the MG knows how RTCP FB "Pause/Resume" messages needs to be processed, i.e., transparently forwarding or origination/termination mode? The solution would be typically:

- the H.248 rempr package would be part of the operated H.248 profile;

- the usage of rempr package elements (in H.248 command request) implies the origination/termination mode; and

- the omission of rempr package elements implies the transparent forwarding mode (i.e., the MG is not allowed to block such RTCP FB messages).

## 8.3	Interactions with compound and reduced-size RTCP packets

There are not any known issues with reduced-size RTCP packet structure, because they are either "totally" related to or not to P/R-signalling (disjoint semantics).

However, compound RTCP packets might demand for an enhanced processing because the compound RTCP packet might contain multiple RTCP packet types with different destination RTP endpoints. For instance, an RTCP XR packet or RTCP FB ECN packet might be destined for a next RTP node in the forwarding path whereas the RTCP FB "Pause/Resume" packet needs to be terminated locally. Hence, the MG might be required to rewrite a (compound) RTCP packet (i.e., support of function F-5 in clause 7.6/[ITU-T H.248.88]).

## 8.4	Gathering SSRC values, relevant for P/R-signaling, from the IP media path

## 8.4.1	Active call interactions with H.248.71

The RTCP source description package according to clause 6/[ITU-T H.248.71] allows the observation and extraction of SSRC values from remote and local RTP endpoints. The H.248.71 rtcpsdes package is typically applied for network wide perfomance monitoring services for RTP traffic, which are using RTCP XR packets for transporting measurement data (see [b-ITU-T H.248.48] and [b-ITU-T H.248.87]). There are basically two H.248 profile types:

- H.248 profile with RTP stream Pause and Resume service support, but without any additional RTCP XR based services; or

- H.248 profile with support of both RTCP based services.

The H.248.71 rtcpsdes package could be applied together with the rempr package in case 1, but is not required for the P/R-signalling service. SSRC information would be directly extracted via H.248 Events (of the rempr package) from RTCP FB Pause/Resume messages.

With respect to case 2, where both packages are in use: same usage as in case 1 during active call phase.

### 8.4.2	Pre-call usage of H.248.71

The MGC might need to know the local SSRC value (e.g., for call control signaling purposes) of the RTP endpoint in the MG, participating in RTP stream pause and resume service, already before any RTP and RTCP packets are transmitted. Hence, the H.248 Events (of the rempr package) would not work. The solution to such a requirement could be addressed by the H.248.71 rtcpsdes package (see clause 6.6.1/[ITU-T H.248.71]).

### 8.4.3	RTP topology "RTP source translator" (H.248.88)

An H.248 context enabled both for a) transparent forwarding of P/R-signalling and b) a RTP source tranlator topology (see clause 11.1/[ITU-T H.248.88] needs carefull considerations because the RTP source translator might modify the SSRC field which is used for RTP sender / RTP receiver identification in P/R-signalling. Such an interaction is conditionally, typically dependent on the used RTCP packet structure (see clause 8.3).

# 9	Remote pause and resume package

...

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[b-ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[End Proposed Correction]

____________________